Notes on CP/M Ver. 2.2 with Cbios rev.E.3.

There  are  several  features of Morrow Designs version  of  CP/M

Version  2.2,  Cbios revision E.3 with which users accustomed  to

single  density CP/M on 8 inch drives may not  be  familiar.   In

addition,  some  explanation is required for certain utility com#

mands  provided  by Morrow Designs.  These special  features  and

utilities are discussed below.

CBIOS

The CBIOS supplied with CP/M Version 2.2 has some unique features

which  allow  users to easily use floppy diskettes  of  different

densities  and/or  sector sizes.  The CBIOS will read  and  write

diskettes  formatted  in IBM 3740 single density format  and  IBM

system  34  double density format.  This compatibility  with  IBM

diskettes is only in the formatting information,  not in the file

structure.  This  means that the Disk Jockey 2D/B or Disk  Jockey

DMA  can read IBM diskettes,  but special software is  needed  to

read IBM files.  No change is necessary within the CBIOS to allow

users to read and/or write any of the IBM standard sector sizes.

The  CBIOS also supports single and  double-sided  drives.  Users

with double-sided drives must use double-sided diskettes in order

to  take  advantage  of the built-in  double-sided  features.  If

single-sided  diskettes  are used in a  double-sided  drive,  the

single-sided  parameters  of  the  CBIOS  will  take  precedence.

Likewise,  if a double-sided diskette is inserted and the  system

is  warm-booted  then the double-sided features will be  switched

on.  Eg. 3 ms. seeks and extended storage.

The  CBIOS requires approximately 5K of memory.   The source  for

the CBIOS is in the file CBIOS&.ASM,  and the source for the COLD

boot loaders is in the file ABOOT&.ASM.  The two files,  BIOS.COM

and  CBIOS.COM,  are examples of disk and terminal  I/O  routines

supplied by Digital Research.

MOVCPM

The  MOVCPM.COM feature of CP/M can be used to directly  relocate

the "cold boot" loader,  the CBIOS and CP/M. If modifications are

done  to any of the above,  the MOVCPM.COM program will  relocate

all but the modified portions correctly. Follow the procedures on

the  next  page if any of the above sections are modified.  If  a

relocated CP/M program is created using the command:

A>movcpm xx

(where xx is the system size or * for auto-sizing) the  relocated

CP/M will begin executing.  However, the relocated CP/M is highly

unstable.  In particular,  if a warm boot occurs,  the system may

not  re-boot  properly.  This  is  not a bug  in  the  MOVCPM.COM

program,  but  rather  a function of the original  and  relocated

CP/M's.  It  is recommended that the following procedure be  used

instead ([RET] means press the RETURN key):

A>movcpm xx *[RET]

A>sysgen[RET]

cold boot the system.

This will insure that the newly relocated CP/M is stable.

For  users  with the hard disk systems,  the  MOVCPM.COM  program

relocates the CBIOS and BOOT for the hard disk version,  not  for

the floppy version.

Assigning Density

Users familiar with the Lifeboat/Morrow Designs 2D CP/M Ver. 1.4,

who  are  accustomed  to  using  the  DENSITY.COM  program,  will

appreciate  the ability of the current programs to  automatically

determine  diskette  density.  There is no need  to  inform  CP/M

Version  2.2  ahead of time of the density of  a  diskette.  Just

insert  the  diskette,  the rest is taken care of.  Care must  be

taken,  however,  when removing or replacing a diskette which has

already been  accessed by CP/M.    A disk reset operation  should

be  done  either by performing a warm boot,  or by making a  BDOS

call to reset the disk system (see the CP/M interface guide).

As these new features have been added,  the size of the Cbios has

grown  to  the  point where the version for  the  double  density

controller can no longer fit on the first two tracks if formatted

in  single density.   This means that you can only boot up with a

diskette  that has been formatted with 1024 byte  double  density

sectors.

Formatting a Diskette - FORMT#

The command file FORMT#.COM will format,  or initialize, a floppy

diskette  in any IBM standard format for users with a Disk Jockey

2D/B floppy disk controller.  Typing

A>FORMT#[RET]

executes a program that inquires the sector size - 128, 256, 512,

or 1024 bytes - to be formatted,  and the drive - A, B, C, or D -

where  the  initialization will take place.  The drive  specified

reflects the floppy drives only, nothing else. Formatting drive A

formats  the  first  floppy  connected to  the  Disk  Jockey  2D.

Formatting a diskette will automatically write sector headers for

the selected sector size.

Again  note that if your floppies have been added to your  system

via the INSTALL program,  or if they are drives C:,  D: or above,

then  the  CP/M drive numbers have nothing to do with FORMT#  and

that drive A will always be the first floppy disk drive.

Formatting a Diskette - FORMATDJ

The command file FORMATDJ.COM formats and initializes both 5  1/4

and 8 inch diskettes for the DJDMA floppy disk controller.

This  program  is menu-driven and is therefore self  explanatory.

Simply enter

A>FORMATDJ[RET]

and follow the instructions.

Formatting the Hard Disk - FORMATHD and FORMATMW

If   it  is  desired  to  format  the  hard  disk,   the  program

FORMATHD.COM  (8 and 14 inch hard disks) or FORMATMW.COM  (5  1/4

inch hard disks) should be used.

CAUTION!

Use  caution with the hard disk format programs:   They erase the

data on the hard disk and it can not be recovered. If you have to

format  the  disk,  think about what is on the hard  disk  before

proceeding.

Reconfiguring a System

To reconfigure the CP/M system for your particular hardware,  you

need  the  MAC assembler (not included with system  software)  to

assemble the CBIOS&.ASM and ABOOT&.ASM files.

Many  parameters  of the Cbios can be modified  by  changing  the

value  of  certain  key  variables  which  are  used  during  the

assembly.  For  example,  the label MSIZE is used to  adjust  the

memory  size  of the reconfigured CP/M.   The following  list  of

equates  were pulled out of the Cbios to give you an idea of  the

options   available  in  the  CBIOS&.ASM  file.    Refer  to  the

CBIOS&.ASM file if you want to see a full explanation for some of

these equates.

List of Equates - CBIOS&.ASM

maxhd	equ	0	;Set to number of HDCA3 hard disk drives

maxmw	equ	1	;Set to number of HDDMA hard disks

maxfd	equ	0	;Set to number of 2D/B floppies

maxdm	equ	2	;Set to number of DJ DMA floppies 8"

maxmf	equ	2	;Set to number of DJ DMA floppies 5 1/4"

hdorder	equ	0	;Set the order of logical drives ELSE 0

mworder	equ	1	;   if not included

fdorder	equ	0

dmorder	equ	2

mforder	equ	3

;HDC3 controller disk drives. Set only 1

M26	equ	0	;Shugart SA4000

M20	equ	0	;Fujitsu M2302B

M10F	equ	0	;Fujitsu M2301B

M10M	equ	0	;Memorex

;HDDMA controller disk drives. Set only 1

cm5619	equ	0	;CMI CM-5619

st412	equ	1	;Seagate ST-412

st506	equ	0	;Seagate ST-506

Make a copy of the .ASM files,  then edit the CBIOS&.ASM file for

your  desired system configuration.   This involves changing  the

numbers  following  the equ portion of the file  to  reflect  the

number of drives,  their order within the system, the drive type,

etc.  Use the following as a guideline for assembling the files:

A>MAC CBIOS&[RET]

A>MAC ABOOT&[RET]

Once the CBIOS&.ASM and ABOOT&.ASM files have been edited and re-

assembled, the following procedure may be used to incorporate the

new drivers into CP/M:

Enter

A>TYPE ABOOT&.PRN[RET]

watch  for  the label "OFFSET" in the  listing,  and  record  its

value.

Enter

A>TYPE CBIOS&.PRN[RET]

watch  for  the label "OFFSETC" in the listing,  and  record  its

value.

If  the  BIOSLN  equate was modified,  then we  must  modify  the

MOVCPM.COM  program to create a CP/M system that matches the  new

Cbios's  length.   The  following  procedure  will  perform  this

modification.

Enter

A>DDT MOVCPM.COM[RET]

-S103[RET]

Location  103  contains  the  BIOSLN  value  for  the  MOVCPM

program.

Enter

0103 15 xx[RET]

0104 43 .[RET]

where xx is the first two digits of BIOSLN and press  RETURN.

For instance, for a BIOSLN of 1800, type in 18[RET].

then enter

-G0[RET]

to exit DDT.

Next enter

A>SAVE 53 MOVCPM.COM[RET]

to save the reconfigured MOVCPM program, and

A>MOVCPM xx *[RET]

where xx is the MSIZE value that you assembled the CBIOS&.ASM

file with, then

A>SAVE 48 CPMxx.COM[RET]

with xx as above.

Again enter	DDT, this time with the new CP/M system file:

A>DDT CPMxx.COM[RET]

The value of xx is as above.

Enter

-HEX[RET]

then

-R(value of OFFSETC)[RET]

Use  the OFFSETC value obtained from the second step in  this

procedure.   Be  sure to omit the parenthesis.    Continue by

entering

-IBIOS&.HEX[RET]

then

-R(value of OFFSET)[RET]

with  the OFFSET value obtained from the first step  in  this

procedure.  Finally, enter

-^C (control C)

to return to CP/M from DDT.

Now enter

A>SAVE 48 CPMxx.COM[RET]

with xx as above, and then

A>SYSGEN[RET]

SYSGEN returns

Morrow Designs SYSGEN version 4.1

Source drive name (or <RETURN> if in memory)

Answer by pressing the RETURN key.  (At this point the source

for  the new  system is in RAM memory,  not on the disk where

you want it).  The return:

Destination drive name (or <RETURN> to warm boot)

Enter  the destination drive - A through P.  Be careful  about

which drives (hard disk or floppy) are drives A,  B,  etc, as

this  step  destroys  the original CP/M  on  the  destination

drive.

To complete this process, press the RETURN key twice:

Destination on (drive):, type <RETURN> to warm boot)

Function complete

Destination drive name (or <RETURN> to warm boot)

Reset  the  system and do a "cold boot."  The  system  header

displays your new system size now.

SINGLE COMMAND

The  program SINGLE.COM is of interest mainly to those users  who

have  only a single drive system and a DJ 2D/B  controller.   Its

purpose  is  to  allow  these users to copy  CP/M  files  between

diskettes.  Users with dual drive systems can perform such copies

faster and more conveniently simply by using PIP.

SINGLE  operates by intercepting CP/M calls to disk drive  select

routines.   Rather than selecting a NEW drive, SINGLE prompts the

user  to  place an "A" or "B" diskette into Drive  A.   In  other

words,  SINGLE  turns Drive A into a logical dual  drive  system.

The  user,  however,  must keep track of which diskette  has been

designated the "A" diskette and which the "B" diskette.

As an example,  to copy a file from drive B to drive A,  give the

command:

A>SINGLE S[RET]

to  begin  operation  in  single mode (the "S"  argument  is  for

Start).  The program will respond that SINGLE has been installed,

and return to CP/M.   Any operation that involves changing drives

will now result in a prompt to place the "A" or "B" diskette into

Drive A.   In our example, the "B" diskette would be the one with

the  file which we wish to copy onto the  system  diskette.

One could now type:

A>PIP A:DEST.FILE=B:SOURCE.FILE[V] [RET]

Depending upon the length of the file to be transferred,  a  fair

number  of requests to place the "A" or "B" diskette into Drive A

will appear until the copy is completed.   An entire system  disk

could  also  be backed up this way.  First one would  format  the

destination disk. Next, the command:

A>PIP A:=B:*.*[RET]

would begin the copy.  This process will take many many swaps but

will  do  the job.  The number of diskette swaps depends  on  the

amount of memory which PIP has to use,  and on the amount of data

to be transferred.

In  general,  any CP/M command which can be used in regular  mode

can  be  used while in single mode.   There are  two  exceptions,

however.   The FORMT#.COM program,  because it makes direct calls

to  Disk  Jockey  2D/B firmware,  will not function  properly  in

single  mode,   and  MOVCPM.COM  because  of  memory   relocation

problems.

To exit single mode, type:

A>SINGLE E[RET]

This  will  return the user to the standard CP/M system (The  "E"

argument  is for End).   Typing SINGLE E when not in single  mode

will report an error and return without any change being made, as

will typing SINGLE S if already in single mode.

REGEN

The  REGEN.COM command regenerates CP/M single density  diskettes

which  have been formatted in such a way that it is  incompatible

with  the  normal operation of the 1791  controller  chip  (IMSAI

diskettes in particular have this problem).  This program will be

of  interest  to those users with the Morrow Designs Disk  Jockey

2D/B Controller only.   It is not intended for use with the  Disk

Jockey DMA controller.

The  REGEN  program is designed to be used with  diskettes  which

have  been  formatted incorrectly - not with diskettes that  have

been partially destroyed.  An example of a diskette that could be

used with the REGEN program is one that the Disk Jockey 1D  board

can  read,  but  the Disk Jockey 2D/B cannot (the DJ1D  does  not

utilize the 1791 controller chip).

Essentially, the REGEN program reads a complete track of data and

format information into memory,  filters out the formatting,  re-

formats  the track and then re-writes the data.   This process is

repeated  until  the whole diskette has  been  regenerated.   The

original  diskette is both the source and  destination  diskette,

and must therefore be write enabled.

The  final  result should be a single density  diskette  that  is

readable  both on the machine that originally produced it as well

as on the Disk Jockey 2D/B controller.

Simply  typing  REGEN  will invoke this program.  It  has  enough

prompts to explain its use.

BOOTHD

The program BOOTHD.COM contains a boot strap loader for the  HDCA

hard disk controller.  BOOTHD.COM could be programmed into a PROM

so  that  you  could boot directly onto the  hard  disks  without

having to bother with the floppies.

BOOTMW

The  program  BOOTMW.COM  contains a boot strap  loader  for  the

HDC/DMA hard disk controller.

The  Morrow  Designs MPZ80 CPU card has both  BOOTHD  and  BOOTMW

encoded  in its PROM.   Refer to the MPZ80 manual for the  proper

switch settings.

#############################################################################