FREIRQ.COM (VERSION 1.0)      "FREE IRQ"

Copyright (c) Stubbs Consult.

FREIRQ shows the status of IRQ's (Interrupt Request Lines) on your PC.

An IRQ is a hardware Interrupt Request Line that lets a hardware device tell

the CPU that an event needs the CPU's attention has happened.

Most of the AT bus pc's have 16 IRQ lines, prior to AT's the PC's only had

8 IRQ's. Some IRQ's are pre-determined on AT's.  The timer uses 0 and the

keyboard uses 1.

FREIRQ picks up where MSD (Microsoft Diag.) leaves off. MSD does not report

IRQ's assigned to devices that are not contained in MSD's internal list.

MSD will  tell you the name of the program controlling a given IRQ only if

you have not disabled STACKS in your CONFIG.SYS file.

To show IRQ information for devices controlled by TSRs that are

loaded in high memory, freirq must be invoked with LOADHIGH, thus

LH freirq. Note that when freirq is loaded high, it can still detect

TSRs loaded into low memory, so if you use high memory on your system,

it's always a good idea to load freirq high.

freirq options:

[LH] freirq [/t] [/u] [/h]

When the FREIRQ is loaded as a TSR with the /t command line option, your

system is constantly being monitored in the background for IRQ activity.

When a previously unassigned IRQ is enabled, its IRQ information is stored in

a 2-byte data file, freirq.DAT.

If you change your system configuration, by removing or rejumpering

a board, you must reinitialize the freirq.DAT file.  This

is done by executing the /i option  freirq /i. Regardless of whether

the TSR component is resident, you display IRQ usage in the same way--by

typing freirq with no switches.

When stacks are enabled, DOS intercepts hardware interrupts, and

when interrupts occur, DOS sets up a stack from its pool of stacks

before dispatching them to the original interrupt handler. This ensures

that stack space will not be exhausted during  hardware interrupt

servicing.  Figure 3 (below) shows the techniques  used to identify the

code segment of the DOS STACKS interrupt  handlers.  Also shown in

Figure 3 is the way freirq inspects the  code in those handlers as it

looks for the original interrupt  service routine (ISR) address.