D460

Purpose: Emulate a Data General terminal (using COM1 or COM2 serial ports)

Syntax:  D460 [keyfile]

Options:

/Baud=n Set the Baud rate

/Com2   Set the Coms port (use /Com1 to /Com4)

/Mono   Set mono screen, even if the computer seems to have CGA or better.

What It Does:

The D460 program lets your PC emulate Data General terminals. You can choose

the type of terminal (D2/6053 to D460)  via the "pop-up" menu available with

the ALT-0 key combination (hold down the ALT key, and press the 0 key found

in the main keyboard, not the numeric key cluster at the right.

The Alt-0 menu is required if you want to switch to COM2, change parity, etc.

You can exit the program either by pressing Alt-X, or choosing the Quit option

in the pop-up menu. You can change the baud rate either through that menu, or

at start-up time (e.g. D460/B=19200). The program buffers serial input, so you

should be able to work with speeds of 38400 baud, given the correct handshaking.

Key-files:

If you give a parameter to the program, (e.g. D460 C:\D460.KEY), you can set

several parameters (including key definitions).

These files are plain text, and always begin with a number, then a space, then

a string. The number determines what is to be set. The codes are:

1 to 9 : set the string returned by ALT-1 to ALT-9 (on the main keyboard, not

the numeric keypad). You can define up to 80 characters, and these

apply whether you are in ANSI mode of DG mode.  To insert a Carriage

Return in the string use "^M" (without the quotes!), and similarly ^J

for line feed, and so on for other control characters. This means you

cannot put a "^" in these key macros other than as the last character.

10:      set the string to be sent, as if typed, when the program starts up.

11 to 255: these program the key responses when in DG mode; up to 3 characters

per key. There is no way to program ^M and ^J and ^Z. The number is

the second byte of the PC key's keycode, e.g. 68 for unshifted F10.

Normally you don't need to change many of these; any key not defined

gets what you'd expect for a DG terminal.

-1 to -255: these define the key responses in ANSI mode. Again, you cannot

use the "^M" notation possible for the ALT-number keys, and you only

have 6 bytes possible.

800      Sets the tab stops, e.g. 800 -----+--------+------------+-----+-----+

900      Sets the printer name, e.g. 900 MYDG.LOG

999      Sets the comms port to use; you might have trouble with ports > 2.

1000     Sets the baud rate for COM1

1001     Sets the parity (Even, Odd, 0, 1, None) for COM1

1002     Sets the Databits (5 to 8) for COM1

1003     Sets the Stopbits (1 or 2) for COM1

1004     Sets the Handshaking for COM1 (4 enables Xon/Xoff)

2000     Sets the baud rate for COM2

2001     Sets the parity (Even, Odd, 0, 1, None) for COM2

2002     Sets the Databits (5 to 8) for COM2

2003     Sets the Stopbits (1 or 2) for COM2

2004     Sets the Handshaking for COM2 (4 enables Xon/Xoff)

3000     Sets the baud rate for COM3

3001     Sets the parity (Even, Odd, 0, 1, None) for COM3

3002     Sets the Databits (5 to 8) for COM3

3003     Sets the Stopbits (1 or 2) for COM3

3004     Sets the Handshaking for COM3 (4 enables Xon/Xoff)

4000     Sets the baud rate for COM4

4001     Sets the parity (Even, Odd, 0, 1, None) for COM4

4002     Sets the Databits (5 to 8) for COM4

4003     Sets the Stopbits (1 or 2) for COM4

4004     Sets the Handshaking for COM4 (4 enables Xon/Xoff)

Note:

This program is distributed for free. There might be bugs. The emulation of

the D460 terminal is far from complete, but the D210 mode (the program's

default mode), is pretty good. The source for the program is available at a

small charge to cover costs and keep me in chocolate fish for years.

If you know it doesn't implement some control code sequences properly, let

me know. I'll give a free copy of the upgrade to the first person who gives

the the details.  Oh! It is free anyway; I forgot about that! :-)

COPYRIGHT (C)1989,1991,1993 MARK AITCHISON:

You may use and distribute this program for free, without any modifications

(don't even change documentation, especially this message!). Commercial

distribution without the author's permission is expressly forbidden.

You can always obtain the latest version of this program (plus more!) from:

Software & Support, P.O.Box 12-292 Christchurch, New Zealand

(allow something reasonable for media, postage and packing).

Address any suggestions or bug reports to that address, or directly to me by

e-mail to:

M.Aitchison@csc.canterbury.ac.nz