MODEM64: DOCUMENTATION

BY CHRIS LAMPTON, 75275,1373

01/29/84

MODEM64 IS A SIMPLE TERMINAL

EMULATOR WITH XMODEM DOWNLOAD FACILI-

TIES, DESIGNED TO RUN UNDER COMMODORE

64 CP/M. IT WILL NOT RUN UNDER OTHER

VERSIONS OF CP/M; IN FACT, IT WILL ONLY

RUN UNDER THE CP/M PACKAGE OFFERED BY

COMMODORE BUSINESS MACHINES.

WHEN FIRST RUN, MODEM64 IS IN

NORMAL TERMINAL MODE. ASCII DATA RE-

CEIVED THROUGH THE RS-232 PORT IS DIS-

PLAYED ON THE SCREEN. DATA TYPED ON THE

KEYBOARD IS TRANSMITTED. PARITY IS

IGNORED. MOST STANDARD ASCII CONTROL

CODES ARE IMPLEMENTED. TO EXIT MODEM64

PRESS FUNCTION KEY F2 (SHIFT-F1).

XMODEM DOWNLOADS ARE INITIATED BY

PRESSING FUNCTION KEY F8 (SHIFT-F7).

USING MODEM64 WITH AN RCP/M

(REMOTE CP/M DATABASE)

THE STANDARD RCP/M COMMAND FOR

XMODEM TRANSMISSION IS:

XMODEM S FILENAME.EXT

IF THE SPECIFIED FILE IS AVAILABLE,

THE RCP/M WILL RESPOND WITH THE NUMBER

OF 128 BYTE BLOCKS IN THE FILE AND THE

APPROXIMATE DOWNLOAD TIME. IF YOU

SHOULD DECIDE AT THIS POINT THAT YOU

WISH TO ABORT THE DOWNLOAD, PRESS

CTRL-'X' TO SEND AN ASCII CANCEL

SIGNAL TO THE SENDING COMPUTER.

TO BEGIN THE DOWNLOAD, PRESS F8,

AND MODEM64 WILL AUTOMATICALLY

REQUEST THE RCP/M TO START TRANS-

MITTING. THE MESSAGE 'XMODEM TRANSMIS-

SION INITIATED' WILL APPEAR ON THE

SCREEN. IF FOR ANY REASON THE TRANS-

MISSION IS NOT RECEIVED, MODEM64 WILL

CONTINUE BROADCASTING THE REQUEST FOR

APPROXIMATELY 100 SECONDS BEFORE

ABORTING THE DOWNLOAD AND RETURNING TO

TERMINAL MODE. SHOULD THIS OCCUR, THE

MESSAGE 'BLOCK NOT RECEIVED, XMODEM

TRANSMISSION ABORTED' WILL BE DIS-

PLAYED. THE DOWNLOAD MAY BE MANUALLY

ABORTED BY PRESSING THE RUN-STOP KEY.

ONCE TRANSMISSION HAS BEGUN, THE

DOWNLOADED BLOCKS WILL BE AUTOMATICALLY

SAVED TO THE DISK IN CP/M FORMAT. AS

EACH BLOCK IS RECEIVED, THE MESSAGE

'BLOCK #NN RECEIVED' WILL BE DISPLAYED,

WHERE NN IS THE BLOCK NUMBER IN HEXA-

DECIMAL. IF MORE THAN 255 (FF HEX)

BLOCKS ARE TRANSMITTED, THE BLOCK

NUMBER WILL ROLL OVER TO 00. CHECKSUMS

ARE USED TO VERIFY THE ACCURACY OF THE

TRANSMITTED DATA. AT THE END OF TRANS-

MISSION, MODEM64 WILL INFORM THE

SENDING COMPUTER OF THE SUCCESSFUL

RECEIPT OF THE FILE AND WILL DISPLAY

THE MESSAGE 'XMODEM TRANSMISSION COM-

PLETED' BEFORE RETURNING TO TERMINAL

MODE.

FILES WILL BE SAVED TO THE DISK

UNDER THE FILENAME 'NEWFILE,' WITH THE

EXTENSION '.XMD'. FILES DOWNLOADED

DURING A SINGLE SESSION WILL BE SEQUEN-

TIALLY NUMBERED, WITH THE FIRST FILE

GIVEN THE NAME 'NEWFILE1.XMD,' THE

SECOND FILE THE NAME 'NEWFILE2.XMD,'

AND SO FORTH. LETTERS OF THE ALPHABET

WILL BE USED ONCE THE TEN NUMERALS ARE

EXHAUSTED. GIVEN THE OBVIOUS LIMITATION

ON FILE NAMES, IT IS RECOMMENDED THAT

YOU NOT DOWNLOAD MORE THAN 35 FILES IN

ONE SESSION, OR YOU MAY FIND UNTYPABLE

ASCII CHARACTERS IN THE NAMES.

IT IS ALSO RECOMMENDED THAT YOU

RENAME ALL DOWNLOADED FILES IMMEDIATELY

AFTER THE SESSION, USING THE CP/M REN

COMMAND, BECAUSE EXISTING NEWFILES ON

THE DISK WILL BE DELETED BY MODEM64

DURING LATER SESSIONS TO MAKE ROOM FOR

FRESHLY DOWNLOADED FILES WITH THE SAME

SEQUENCE NUMBERS.

IN THE EVENT THAT YOU ATTEMPT TO

DOWNLOAD TO A FULL DISK (OR A FULL DISK

DIRECTORY), AN ERROR MESSAGE WILL BE

PRINTED AND THE DOWNLOAD ABORTED. ANY

DATA DOWNLOADED TO THE CURRENT FILE

BEFORE THE DISK LIMIT WAS REACHED WILL

BE PRESERVED.

THIS IMPLEMENTATION OF XMODEM IS

NOT BULLET PROOF. IT IS POSSIBLE FOR

THE SENDING COMPUTER AND THE RECEIVING

COMPUTER TO FALL OUT OF SYNC AND NOT

RECOVER, THOUGH THIS IS NOT A VERY

LIKELY EVENT. SHOULD IT OCCUR, THE

DOWNLOAD WILL MORE THAN LIKELY ABORT

BY ITSELF. HOWEVER, SHOULD YOU NOTICE

AN UNUSUALLY LONG PAUSE BETWEEN BLOCKS

-- SAY 20 SECONDS OR MORE -- YOU SHOULD

ABORT MANUALLY WITH THE RUN-STOP KEY.

THE SENDING COMPUTER MAY CONTINUE

BROADCASTING DATA, BUT WILL NOTICE

WITHIN A FEW SECONDS THAT NO ACKNOW-

LEDGING SIGNAL IS BEING RECEIVED AND

WILL CANCEL THE DOWNLOAD. YOU MAY THEN

INITIATE THE DOWNLOAD AGAIN. THIS HAS

HAPPENED TO ME ONCE IN ROUGHLY 40

DOWNLOADS. SECURITY WILL BE TIGHTENED

IN SUBSEQUENT VERSIONS.

SUGGESTIONS FOR FUTURE IMPROVEMENTS

AND REPORTS OF CURRENT BUGS SHOULD

BE CONVEYED TO THE AUTHOR VIA COMPU-

SERVE INFORMATION SERVICE EMAIL OR THE

CIS COMMODORE-64 SIG.