Coleg Prifysgol Gogledd Cymru           University College of North Wales

Labordy Cyfrifiadurol                   Computing Laboratory

Document: ALGOL RANDOM ACCESS I/O       Date: May 1980  Author: I. G. Jones

Random access input, output in ALGOL on the DEC SYSTEM 10.

______ ______ ______ ______ __ _____ __ ___ ___ ______ ___

This document details  the  procedures  implemented  for  random  access

input,  output  from an algol program.The records produced being written

onto the disc in dump record mode.

Integer, real, boolean, and long real arrays may be used for  the  array

in the procedures DREAD and DWRITE.

PROCEDURE DOPEN(CH,LLI);

INTEGER CH;  STRING LLI;

This procedure is used for opening a file on channel CH of name LLI, and

must be the first procedure called.

PROCEDURE DWRITE(CH,N,B1);

INTEGER CH,N;

ARRAY B1;

The above procedure is used for writing ARRAY B1 as a block of  data  on

the  previously  opened  channel  CH, with words from 1 to N of array B1

where array B1 can be of any type except string.  If N is  <128  then  1

block  of  128  words  will be written, with words from N+1 to 128 being

zero, while if N>128 then enough blocks of 128 words are written  so  as

to  accomodate  N.  The last block being zero filled if N is not exactly

divisible by 128.  If the array B1 is  long  real,  2*N  words  will  be

written to the disc file.

PROCEDURE DCLOSE(CH);

INTEGER CH;

When this procedure is called, the file allocated to channel CH  is  put

onto  the disc if the file has been written or modified in the operation

following DOPEN.  If DCLOSE is omitted from your program  then  the  new

file is not put onto the disc.

PROCEDURE DREAD (CH,N,B1);

INTEGER CH,N;

ARRAY B1;

ALGOL RANDOM ACCESS I/O        Page 2

This procedure is complementary to DWRITE, and is used for  reading  the

array  B1  from 1 to N, from channel CH, where CH is allocated to a file

which is on disc via DOPEN.

PROCEDURE DRFIND(CH,N);

INTEGER CH,N;

PROCEDURE DRFIND positions the  file  on  channel  CH  at  block  N  for

reading.   If  this  procedure is not used, reading will take place in a

sequential manner from the beginning of the file.  The  file  must  have

been  opened  by  DOPEN before this procedure may be called.  Users must

take into account that a record may span more than  one  disc  block  in

calculating  N.   If  N is a block further than the end of the file, and

DREAD is called after DRFIND, then the last block on the  file  will  be

read,  and  no indication that N is greater than the number of blocks on

the file will be given.

PROCEDURE DWFIND(CH,N);

INTEGER CH,N;

This procedure is similar to DRFIND, except that it positions  the  file

for writing on channel CH at block N.

BOOLEAN PROCEDURE DNEW(CH);

INTEGER CH;

This is a boolean procedure which returns a value TRUE if  the  file  on

channel  CH  is  a  new file, and a FALSE value if one is opening an old

file.

PROCEDURE DFEOF(CH);

INTEGER CH;

This procedure positions the file on channel CH at the end of  the  file

for writing purposes.

METHOD OF USE        ______ __ ___

The procedures used in an algol program should be declared  as  EXTERNAL

in the program, and called up on execution as follows.

.EX <PROGRAM>,REL:DRANDM.REL/SEARCH

N.B.  Since people sometimes use small blocks of  data  which  they  may

require  to access randomly.Then it is suggested that the following type

ALGOL RANDOM ACCESS I/O        Page 3

of procedure should be used to buffer the  data  to  128  or  less  word

blocks.With a similar type of procedure being used for retrieval.

PROCEDURE BUFFEROUT(CH,N,A);

VALUE CH,N;

INTEGER CH,N;

INTEGER ARRAY A;

BEGIN

OWN INTEGER ARRAY BUFF[1:128];

OWN INTEGER POINTER;INTEGER J;

EXTERNAL PROCEDURE DWRITE;

FOR J:=1 UNTIL N DO

BUFF[J+POINTER]:=A[J];

POINTER:=POINTER+N;

IF POINTER =128-N THEN

BEGIN

DWRITE(CH,128,BUFF);POINTER:=0;

END

END BUFFER;

Thus when one does a random  access  the  block  will  not  be  directly

accessible,  but  can be obtained using a simple procedure.  Also due to

the fact that a string array holds the address of strings, and  not  the

actual strings it is not possible to directly store the strings on disc.

This may be got around by changing the string, and storing in an integer

array as exemplified by the following program.

BEGIN

INTEGER I,J,K,L,M;

STRING S;

INTEGER ARRAY LLE[1:10];

EXTERNAL PROCEDURE DOPEN,DCLOSE,DWRITE;

DOPEN(8,"LLINYN.BUF");

FOR I:=1 UNTIL 5 DO

BEGIN

READ(S);

LLE[1]:=LENGTH(S);L:=7;

IF LLE[1] >45 THEN LLE[1]:=45;

K:=1;M:=2;

FOR J:=1 UNTIL LLE[1] DO

BEGIN

SFIELD(LLE[M],K,L,S.[J]);

K:=K+L;

IF K > 30 THEN

BEGIN

K:=1;M:=M+1

END

END;

DWRITE(8,10,LLE);

END;

ALGOL RANDOM ACCESS I/O        Page 4

DCLOSE(8)

END;

If the strings are small in length, then better efficiency in  terms  of

disc  storage  could be obtained by using the above procedure BUFFEROUT.

With a similar program being used to retrieve the strings.   (The  first

element of the array in the example has been used for storing the number

of characters in the string.)

ERRORS      ______

If an error is detected,

I.E.  A negative number is given  as  the  block  number  in  DRFIND  or

DWFIND,  or  a  string array is given as the array parameter in DREAD or

DWRITE then an  error  message  is  typed  on  you'r  terminal.Also  the

erroneous  operation  is abandoned and you'r program terminated.

#