C	****PROGRAM: TALK8F.F4, APRIL 1969 ****

C

C

C	BY PETER LEMKIN AND ALAN CHAUVENET*

C	NATIONAL INSTITUTES OF HEALTH

C	BETHESDA, MD. 20014

C

C

C	*(NOW AT DUKE UNIV, DURHAM, N.C.)

C

C

C

C INTRODUCTION:

C -------------

C THE ASSEMBLY OF LARGE PROGRAMS FOR SMALL MACHINES SUCH AS

C THE PDP8 IS APT TO BE LABORIOUS, TIME CONSUMING AND ALMOST

C IMPOSSIBLE IS DONE ON THE SMALL MACHINE ITSELF. IN ADDITION,

C THE ABILITY FOR MANY USERS TO ASSEMBLE PDP8 PROGRAMS ON A PDP10

C COMPUTER USING PAL10 OR PAL12 LIGHTENS THE LOAD OF SOFTWARE

C DEVELOPMENT ON THE SMALLER MACHINE. TALK8F IS A PDP10

C UTILITY PROGRAM WHICH TRANSFORMS THE OUTPUT OF ASSEMBLERS

C SUCH AS PAL10 OR PAL12 INTO A FORM AMENABLE TO DATAPHONE

C COMMUNICATION.

C

C

C

C HARDWARE REQUIRED:

C --------------------

C PDP10 WITH DATAPHONE OR HARDWARE INTERFACE

C TO A PDP8 WITH A PT08 SERIAL/PARALLEL INTERFACE.

C

C

C

C DESCRIPTION:

C -----------

C THIS PROGRAM READS A PAL10 ".BIN"  OUTPUT FILE FROM THE DISK

C AND CONVERTS IT TO A FORM OF ASCII CHARACTERS CODE.

C THIS CODE CAN BE SENT OVER THE TELEPHONE LINES AND

C INTERPRETED BY THE PDP8 "TALK10" PROGRAM RUNNING ON THE

C PDP-8.(A PUNCHED PAPER TAPE MAY BE READ INTO THE PDP8 THROUGH

C "OCTMON", A PDP8 OCTAL DEBUGGER PROGRAM, DECUS 8-298, WHICH

C ACCEPTS THIS SPECIAL CODE FORMAT.)

C THE PAL10 BINARY OUTPUT FILE MUST BE  ON THE USER'S DSK AREA.

C A  ".DAT" FILE, NAMED BY THE USER, WILL BE GENERATED AND WRITTEN

C IN THE USER'S DISK AREA . HE MAY THEN PIP IT TO HIS TELETYPE (TTY).

C (I.E. TO HIS BUFFERED PDP8 TTY THROUGH THE PDP8 TALK10 PROGRAM WHICH

C IS INTERFACED TO THE PDP10 THROUGH THE PT08.).

C THE BEGINNING AND END OF THE ".DAT" FILE CONTAIN STRINGS OF A'S

C WHICH ARE USED TO ENTER AND LEAVE THE TALK10 "OCTLDR" MODE.

C IF THE USER IS TALKING TO THE PDP10 VIA TALK10, SENDING THE ".DAT"

C FILE WILL LOAD IT INTO THE PDP8 MEMORY.

C

C

C

C

C

C

C CODING OF THE PAL10 BINARY OUTPUT FILE:

C ---------------------------------------

C THE "FIELD" CODE IS CONVERTED TO A 4 DIGIT NUMBER FOLLOWED BY "F".

C THE "MEMORY LOCATION" CODE IS CONVERTED TO A 4 DIGIT OCTAL NUMBER

C FOLLOWED BY A "Z". THIS IS THE LOCATION WHERE THE NEXT DATUM WILL

C BE PUT. THE "DATA" CODE IS CONVERTED TO A 4 DIGIT OCTAL NUMBER

C FOLLOWED BY A CARRIAGE RETURN. THE LEADER TRAILER CODE IS USED TO

C SIGNAL TALK8F THAT THE BEGINNING AND END OF THE FILE IS REACHED.

C

C

C

C

C

C COMPILING THE SOURCES:

C -------------------------

C NOTE: (1) TO COMPILE TALK8F,  COMPILE TALK8F.F4 AND READPL.MAC .

C       (2) LOAD TALK8F.REL AND READPL.MAC TOGETHER.

C	(3) DO A SAVE DSK:TALK8F.SAV TO GET THE ".SAV" FILE .

C

C

C

C

C

C EXAMPLE OF TALK8F/TALK10 OPERATION

C ------------------------------------

C EXAMPLE: (IT IS ASSUMED THAT THE USER IS TALKING TO THE PDP10

C FROM THE PDP8 TTY <==> (PDP8 PROGRAM "TALK10") <==> PT08 <==> PDP10.

C	EXAMPLE:

C	--------

C	 (1) ASSEMBLE THE PDP8 SOURCE FILE PDP8.SRC USING PAL10

C	    .R PAL10

C	    *PDP8.BIN,PDP8.LST_PDP8.SRC

C	   *^C

C	   .

C	   (2) NOW CONVERT THE PDP8.BIN FILE TO THE ASCII CODE FILE.

C	   .RUN DSK:TALK8F.SAV

C	   INPUT FILE:PDP8

C	   EXTENSION?:BIN

C	   OUTPUT FILE:PDP8

C

C	   YOU MAY NOW PIP PDP8.DAT FROM YOUR DISK AREA.

C	   EXIT

C	   .

C

C	 (3) NOW TRANSMIT THE ASCII "PDP8.DAT" FILE TO THE PDP8 TO BE

C	    LOADED INTO THE PDP8 MEMORY BY THE TALK10 PROGRAM.

C	  .R PIP

C	  *TTY:_DSK:PDP8.DAT

C	  *

C	 (4) THE ".DAT" FILE CONTAINS A STRING OF A'S  IN THE BEGINNING

C	   AND END OF THE FILE. THESE ARE USED TO TURN ON AND OFF THE

C	   PDP8'S TALK10 PROGRAM LOADER SUBPROGRAM CALLED  "OCTLDR".

C	   AFTER THE REST OF THE   A'S HAVE BEEN TYPED OUT, THE PROGRAM

C	   IS NOW LOADED INTO THE   PDP8 MEMORY AND TALK10 IS

C	   IN BUFFERED TELETYPE MODE AGAIN.

C          NOTE: A PROGRAM MAY BE LOADED INTO ANY PDP8 FIELD AS TALK8F

C	   WILL DO FIELD CONVERSIONS FOR ANY FIELD AND TALK10 WILL LOAD

C	   INTO ANY FIELD. TALK10 DOES NOT PREVENT DESTROYING ITSELF

C	   BY LOADING INTO ITS OWN CORE IMAGE.

C

C

C

C

C

C

C

C

C

C	**************************************************************

C

C

IMPLICIT INTEGER(A-Z)

COMMON /ZEEP/  HOLDA(129),SAVE(2)

COMMON /S8BUF/ D1,D2,D3,HOLD(128)

DATA CHECK,SUB,SUBF/O77,O100,O300/

C SETUP OUTPUT TO DISK FILE

C	SETUP THE INPUT FILE NAME IN READPL

TYPE 1065

1065	FORMAT(' INPUT FILE:'$)

ACCEPT 1001,FNME

1001	FORMAT(1A5)

TYPE 1002

1002	FORMAT(' EXTENSION?:'$)

ACCEPT 1001,EXT

CALL CONVER(FNME)

CALL CONVER(EXT)

C	SET UP THE OUTPUT FILE...

TYPE 101

101	FORMAT(' OUTPUT FILE:'$)

ACCEPT 102,PETER

102	FORMAT(1A5)

CALL OFILE(1,PETER)

LEAD=0

ORIG=0

FLD=0

D3=50

D1=0

DO 5 I=1,128

5      HOLD(I)=-1

12     FORMAT(I1)

C SPECIAL INITIALIZATION FOR FIRST RECORD (SKIP LEADER)

SAVE(1)=-1

SAVE(2)=-1

KEY=85+1

J=0

C NOW GET THE NEXT RECORD FROM THE DSK:FILE WITH READPL

ARG=0

C ROUTINE READPL--THIS WILL FILL HOLD WITH 128 WORDS (ONE DISC RECORD)

200    CALL READPL(FNME,EXT,ARG)

PFLG=0

IF(ARG.EQ.3) GOTO 4400

ARG=1

HOLDA(2)=SAVE(1)

HOLDA(3)=SAVE(2)

DO 225 I=1,126

225    HOLDA(I+3)=HOLD(I)

SAVE(1)=HOLD(127)

SAVE(2)=HOLD(128)

SWITCH=0

C ALL THE ABOVE JUNK IS TO ENABLE ME TO MAKE SURE THAT THE

C CHECKSUM INFO DOES NOT GET SENT TO THE OUTPUT FILE.

C  NOW WE BEGIN PROCESSING THE RECORD

DO 600 I=KEY,129,1

C NOW IF WE ARE LOOKI5G AT SECOND OF A PAIR WE WANT TO JUMP AND

C BUMP THE INDEX BECAUSE WE'VE ALREADY PROCESSED THE PAIR

IF (SWITCH .EQ. 1) GO TO 598

C IF WE SEE TRAILER IN OUR IMMEDIATE FUTURE, WE ARE NOW LOOKING

C AT THE CHECKSUM, SO WE'RE DONE--JUMP OUT OF LOOP AND WRITE THE

C TRAILER AND CLOSE THE FILE

IF ((I.NE.1).AND. (HOLD (I-1) .EQ. 128)) GO TO 800

C IF WE ARE LOOKING AT ORIGIN OR FIELD, JUMP. OTHERWISE WE HAVE A PAIR

C WHICH CONSTITUTE THE NEXT PDP-8 INSTRUCTION

IF (HOLDA(I) .GT. CHECK ) GO TO 300

C NOW PROCESS THE INSTRUCTION PAIR

OUTP= HOLDA(I)*64 + HOLDA(I+1)

C AND WRITE IT OUT

227       IF (LEAD .EQ. 0) GO TO 250

IF ((ORIG .EQ. 1) .AND. (FLD .EQ. 0)) GO TO 280

IF ((ORIG .EQ. 1) .AND. (FLD .EQ. 2)) GO TO 285

IF ((ORIG .EQ. 0) .AND. (FLD .EQ. 1)) GO TO 290

IF ((ORIG .EQ. 2) .AND. (FLD .EQ. 1)) GO TO 295

230   WRITE(1,2000)OUTP

2000   FORMAT (O4)

GO TO 597

250    LEAD=1

IF((ORIG.EQ.1).AND.(FLD.EQ.0))WRITE(1,2001)OUTOR,OUTP

2001  FORMAT (40HAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA,O4,1HZ,O4)

IF((ORIG.EQ.1).AND.(FLD.EQ.2))WRITE(1,2002)OUTOR,OUTFL,OUTP

2002  FORMAT (40HAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA,O4,1HZ,O4,1HF,

1O4)

IF((ORIG.EQ.0).AND.(FLD.EQ.1))WRITE(1,2003)OUTFL,OUTP

2003  FORMAT (40HAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA,O4,1HF,O4)

IF((ORIG.EQ.2).AND.(FLD.EQ.1))WRITE(1,2004)OUTFL,OUTOR,OUTP

2004  FORMAT (40HAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA,O4,1HF,O4,1HZ,

1O4)

IF((ORIG.EQ.0).AND.(FLD.EQ.0)) GO TO 230

ORIG=0

FLD=0

GO TO 597

280    WRITE (1,2005)OUTOR,OUTP

2005   FORMAT (O4,1HZ,O4)

ORIG=0

GO TO 597

285    WRITE (1,2006)OUTOR,OUTFL,OUTP

2006   FORMAT (O4,1HZ,O4,1HF,O4)

ORIG=0

FLD=0

GO TO 597

290    WRITE (1,2007)OUTFL,OUTP

2007   FORMAT (O4,1HF,O4)

FLD=0

GO TO 597

295    WRITE (1,2008)OUTFL,OUTOR,OUTP

2008   FORMAT (O4,1HF,O4,1HZ,O4)

FLD=0

ORIG=0

GO TO 597

C COME HERE TO SEE WHETHER FIELD OR ORIGIN

300    TEST=HOLDA(I)/64

C IF ORIGIN GO TO 301, IF FIELD GO TO 500. IF 200--LEADER-TRAILER

C SHOULD HAVE BEEN CAUGHT ABOVE AND THERE IS AN ERROR

IF (TEST-2) 301,400,500

C COME HERE IF ORIGIN

301    OUTOR= HOLDA(I) -SUB

OUTOR = 64*OUTOR + HOLDA (I+1)

ORIG=FLD+1

GO TO 597

400    WRITE (5,4000)

4000   FORMAT (' ERROR IN PAL10 READ--UNDETECTED LEADER-TRAILER')

GO TO 900

C  COME HERE FOR FIELD DEFN

500    OUTFL = HOLDA(I) -SUBF

OUTFL = OUTFL/8

FLD=ORIG+1

GO TO 599

597    SWITCH=1

IF(I.NE.127) GOTO 599

PFLG=1

GOTO 601

598    SWITCH=0

599    CONTINUE

600    CONTINUE

C NOW GO BACK AND READ THE NEXT RECORD

601    KEY=2

IF(PFLG.EQ.0) GOTO 200

HOLDA(1)=HOLDA(129)

KEY=1

GOTO 200

C NOW WRITE OUT TRAILER AND CLOSE THE FILE

800    WRITE (1,1000)

1000   FORMAT ('AAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAAA')

801    WRITE (5,8000) PETER

8000   FORMAT (' YOU MAY NOW PIP ',1A5,'.DAT FROM YOUR DISK AREA.')

C COME DIRECTLY HERE WHEN L-T ERROR FOUND AT 400

900    END FILE 1

STOP

4400	TYPE 4401

4401	FORMAT(' ERROR IN DSK READ OF INPUT FILE (READPL) '//)

CALL EXIT

END

;*********TITLE READPL.MAC APRIL 1970 ***********

;	BY PETER LEMKIN AND ALAN CHAUVENET

;	NATIONAL INSTITUTES OF HEALTH

;	BETHESDA, MD. 20014

;

;	THIS PROGRAM READS THE BINARY IMAGE FILE PRODUCED BY

;	THE PAL10 ASSEMBLER. IT MUST BE LOADED WITH TALK8F.F4 .

;

;MACRO 10 PROGRAM FOR USE WITH TALK8F, LOADED WITH IT.

;

TITLE READPL

ENTRY READPL

EXTERNAL S8BUF

PASS=10

READPL: 0

MOVEM PASS,SPARE

MOVE PASS,@2(16)

TRNE PASS,1

JRST READ

MOVE PASS,@(16)

MOVEM PASS,INNAME

MOVE PASS,@1(16)

MOVEM PASS,INNAME+1

INIT 16,17

SIXBIT /DSK/

Z

JRST FAIL

LOOKUP 16,INNAME

JRST FAIL

READ:	INPUT 16,[IOWD 200,S8BUF+3

0]

STATZ 16,740000

JRST FAIL

MOVE PASS,SPARE

JRA 16,3(16)

FAIL:	MOVE PASS,THREE ;SET ARG =3 IF ERROR

MOVEM PASS,@2(16)

JRA 16,3(16) ;RETURN

INNAME: 0

0

0

0

THREE: 3

SPARE: 0

;TITLE CONVER

INTERN CONVER

CONVER: 0

MOVEM 17,REG     ;SAVE REG 17

MOVE 17,[XWD 0,SAVE] ;SET UP POINTER

BLT 17,SAVE+7

MOVEI 4,0

MOVEI 5,5

MOVE 2,@0(16)

;CONVERT TO SIXBIT

RET:  ROTC 2,1

MOVEI 3,0

ROTC 2,1

JUMPE 3,ONE

JUMPN 3,ZER

RC:   ROTC 2,5

ROT 3,36

ROTC 3,6

SOJN 5,RET

ROT 4,6

RETRN: MOVEM 4,@0(16)

MOVSI 17,SAVE

BLT 17,7

MOVE 17,REG

JRA 16,1(16)

ONE:  MOVEI 3,1

JUMPA RC

ZER:  MOVEI 3,0

JUMPA RC

REG:  Z

SAVE: BLOCK 10

END