DTCERT-DECTAPE CERTIFICATION

DTCERT demands: 1) a 1,2 job

2) a dedicated TD10-C

3) DECtapes write enabled and in remote status

DTCERT is a program used to certify and label DECtapes.  The

certification and labelling process is done to format and label

DECtapes for use on the DECsystem10.  DTCERT is run on all new DEC-

tapes at UCC to prepare them for distribution to, and use by

users of DECtapes on the DEC-10.

To run DTCERT, OFILE (the OPSER subjob that performs regular

DECtape processing) must be killed in order to give DTCERT

sole access to the TD10-C controller.  Type F-STOP to the operator

console (CTY) in order to stop OFILE from processing any fur-

ther requests.  When OFILE is finished with any DECtapes it is

currently processing, type :KILL F to the CTY.  This will clear

the assignment of DTA010 to OFILE and free the TD10-C controller

for use by DTCERT.

*** Remove all user tapes from the DECtapes drives before

proceeding.  This is essential because DTCERT is unable to dif-

ferentiate between a DECtape with data on it and an uncertified

tape.  Any DECtape hung write enabled and in remote status is

assumed to be ready to be certified.  If you have a users tape

hung and meeting these conditions, it may get certified and all

the user's data will be destroyed.

When you are ready to begin DECtape certification, at the 1,2

terminal type:

RU DTCERT

(the OPRDPY terminal is convenient to use as it is located

nearest to the DECtape drives.  DO NOT use OPSER subjobs to run

DTCERT-there are too many problems that arise when trying to terminate

DTCERT if you do)

The DTCERT program will walk you through the certification

and labelling process.  DTCERT will run the DECtapes through four

passes from end zone to end zone during certification.  If there

are any problems with any of these passes, DTCERT will return

to the beginning of Pass 1.  At all times follow the instructions

output by DTCERT.  If there are any difficulties, the DTCERT pro-

gram will tell you what to do.  An outline of what happens in

each pass and of what will be needed from the operator is pro-

vided below.

BEGINNING

After you have started DTCERT a message will be typed to the

1,2 terminal to set the WRTM switch when a DECtape is ready to

be certified.  Be sure you have the tape(s) you want to certify

write enabled and in remote status.  Use as little leader as

possible on the takeup reel.  DTCERT will check the DECtape drives

in sequence (0,1,2,3,) for tapes that are ready to be certified.

It will certify the first tape that it comes to that meets the

requirements of being write enabled and in remote status.  It will

continue to certify tapes that meet these requirements at the

turning on of the WRTM switch until the operator terminates the

DTCERT program.

PASS 1

Turn on (to the up position) the WRTM switch at the

top of the DECtape drives when the tape(s) you want to

certify are ready.  If the tape you are certifying dismounts

at this point, turn off the WRTM switch and remount the

tape.  Turn on the WRTM switch to restart the certification.

Pass 1 of DECtape certification will write an end zone at

the start of the DECtape, 578 blocks of data and another end

zone.  During this pass timing marks are written on the DEC-

tape.  When the pass is completed, a message to remount the

tape (if necessary) and turn off the WRTM switch will be

output.  This message will appear whether or not the DECtape

has come off the end.  If it has, remount it before turning

off the WRTM switch.  At the turning off of the WRTM switch,

Pass 2 will begin.

PASS 2

At the start of Pass 2 the DECtape being certified will

back up for three seconds to skip past the end zone just

written.  It will then skip forward to the beginning of the

end zone.  The DECtape will then reverse again writing

block numbers and data back to the first end zone on the

tape.  No message will be generated at the end of this pass.

PASS 3

The DECtape will reverse and read the block numbers and

data for confirmation of what has been written.  No message is

generated at the end of this pass.

PASS 4

The DECtape will reverse and zero all data areas back to

the first end zone.  At this point it will confirm that every-

thing has gone well with the certification by giving the

operator a "TAPE OK" message and ask the operator for the

tape label.  If this message is not ouput, recertify the

DECtape.

When DTCERT asks for the DECtape label, type in the ID that

appears on the external tape label.  This will insert the tape

label into the zeroed directory, seek block 100 and write the

directory on the tape.

Put the tape you have just certified in local status and

turn on the WRTM switch to certify another tape on another

drive.

When you have finished certifying and labelling DECtapes,

clear the DECtape drives of tapes and be sure the WRTM switch

is off. Control-C the DTCERT program twice and log off the

1,2 terminal.  Type :AUTO FILE to the CTY (operator console)

in order to restart OFILE processing.

###################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################