PDP-8 SIMULATOR

===============

--------

ABSTRACT

========

THE PDP-8 SIMULATOR HAS TWO MAIN PARTS, THE

FIRST ACCEPTS INPUT IN PAL III LANGUAGE AND ASSEMBLES IT, AND

THE SECOND EXECUTES THE INPUT PROGRAM.

AT EACH STAGE OF OPERATION, INPUT, ASSEMBLY OR EXECUTION

THE SIMULATED PDP-8 MEMORY CAN BE EXAMINED, IN OCTAL DIGITS.

THE SIMULATOR ASSEMBLES AND EXECUTES MANY OF THE PAL III

INSTRUCTIONS, BUT ONLY A PORTION OF THE INPUT-OUTPUT INSTRUCTIONS,

AND NONE OF THE MICROPROGRAMMING FACILITIES ARE AVAILABLE.

COMMUNICATION AND INTERACTION WITH THE SIMULATOR ARE DONE

THROUGH THE TELETYPE , WHICH IS ALSO THE ONLY PERIPHERAL DEVICE

HANDLED BY THE SIMULATOR.

PROGRAMS CAN BE STORED AND RECALLED USING THE

DISK OPERATIONS FACILITIES.

THIS PROGRAM WAS WRITTEN IN MACRO-10 ON THE BRANDEIS

UNIVERSITY PDP-10 COMPUTER.  WE WISH TO THANK THE COMPUTER STAFF

AND PROFFESOR MAX CHRETIEN FOR THEIR COOPERATION.

DESCRIPTION, PRINCIPLES OF OPERATION AND GENERAL FEATURES OF

PDP-8 SIMULATOR,

MACRO-10 PROGRAM, WRITTEN ON BRANDEIS' PDP-10 COMPUTER BY

ODED SHMUELI    '77

KWABENA AKUFO   '77

ADVISOR:  MAX CHRETIEN

PROF. OF PHYSICS

BRANDEIS UNIVERSITY

DATE: OCTOBER 1975

C O N T E N T S

= = = = = = = =

1.  BASIC PRINCIPLES

2.  USERS' VIEW (SIMPLIFIED)

3.  GENERAL FEATURES

4.  CONCLUSION

APPENDIX A.   SAMPLE PROGRAM IN PAL III

APPENDIX B.   GENERAL FLOWCHART

APPENDIX C.   PROCEDURAL USE OF SIMULATOR INCLUDING

(I) PROGRAM INPUT

(II) EXAMINATION OF LOCATIONS

(III)   WRITING ON, AND READING FROM DISK

(IV)   EXECUTION AND EXAMINATION OF RESULTS

(V)   MODIFICATION OF PROGRAM

(VI)   READING OF A STORED PROGRAM FROM DISK,

EXECUTING IT.

APPENDIX D.   AVAILABLE INSTRUCTIONS

APPENDIX E.   PROGRAM LISTING OF THE PDP-8 SIMULATOR

BASIC PRINCIPLES

THE PROGRAM IS MADE UP OF TWO MAIN PARTS.  THE FIRST

PART CONSISTS OF AN INPUT UNIT FOR PAL III PROGRAMS AND AN

ASSEMBLER.  THE OUTPUT OF THE ASSEMBLER IS IDENTICAL TO

THAT OF THE PDP-8 ASSEMBLER, AND IS STORED IN AN ARRAY

CALLED PDP-8.

THE SECOND PART OF THE PROGRAM, WHICH IS INDEPENDENT

OF THE FIRST, SIMULATES THE ACTUAL EXECUTION OF A PALL III

PROGRAM ON THE PDP-8.  THIS PART CAN HANDLE MEMORY REFERENCE

INSTRUCTIONS, OPERATE MICROINSTRUCTIONS, AND INPUT/OUTPUT

1

TRANSFER INSTRUCTIONS.   MICROPROGRAMMING AND AUTOINDEXING

FACILITIES ARE NOT AVAILABLE.  SINCE THE TELETYPE IS THE

ONLY MEDIUM OF COMMUNICATION BETWEEN THE USER AND THE

SIMULATOR, THE INTERRUPT FACILITY OF THE PDP-8 IS NOT NEEDED

AND THEREFORE NOT SIMULATED EXCEPT FOR THE TELETYPE, WHICH IS

AN ACTUAL MEDIUM OF COMMUNICATION.

USERS'  VIEW

A GOOD WAY TO DESCRIBE THE USE OF THIS SIMULATOR IS

THROUGH THE USE OF ILLUSTRATIONS FOLLOWED BY A GENERAL

DISCUSSION.

PROBLEM: WRITE A PROGRAM TO SUM THE NUMBERS 2,-3,10

REFER APPENDIX A FOR SAMPLE PROGRAM.

1.

REFER TO APPENDIX D FOR LIST OF INSTRUCTIONS AVAILABLE ON

THE SIMULATOR.

-2-

STEP 1.

IN THE MONITOR MODE TYPE

.EX PDP/M,TR.MAC

STEP 2.

THE USER WILL RECEIVE THE FOLLOWING MESSAGES AT THE

START OF THE EXECTION OF THE SIMULATOR PROGRAM:

.EX PDP/M.TR.MAC

LINK:   LOADING

[LNKXCT PDP EXECUTION]

TYPE 1,2,3,4,5,6,7,A,B,C,D

B: COMPILE, C OR D: EXECUTE, 3: COPY FROM DISK, 6: WRITE N DISK

4: TO START WRITING PROGRAM WITH * (DESTROYS OTHER PROGRAMS ON FILE)

5: START PROGRAM INPUT (PRESERVES OTHER PROGRAMS ON FILE

UPPER CASE 1(!): INTERRUPT FACILITY TO EXAMINE COMPILATION ANYWHERE

CENT

FOLLOWING THE WORD 'CENT' AT THE END OF THE ABOVE

MESSAGES, TYPE '4'.

ADDITIONAL MESSAGES WILL BE OUTPUT.

4INITIALIZATION WILL BE DONE NOW

PD8 TABLE NUMBERED

INPUT PROGRAM

AFTER THE MESSAGE 'INPUT PROGRAM', THE USER SHOULD

TYPE AN ASTERISK(*) AND WAIT FOR THE OUTPUT OF A BLANK.

THEN TYPE THE STARTING LOCATION OF THE PROGRAM AND

RETURN THE CARRIAGE.

THE USER GETS THE MESSAGE

* 200

LC VALUE (*) KNOWN IS IN LC

THE USER WILL THEN START INPUT OF THE PROGRAM IN THE FOLLOWING WAY:

(1) LABELS,IF THEY EXIST SHOULD ALWAYS CONSIST OF TWO

CHARACTERS.IF THE LINE CONTAINS NO LABEL , CONTROL I

SHOULD BE TYPED. IN EITHER CASE , THE USER SHOULD WAIT UNTIL BLANKS

ARE OUTPUT AND THEN RESUME INPUT BY TYPING EITHER A COLON(:) OR COMMA(,).

COLON(:)INDICATES THAT WHAT FOLLOWS IS A MNEMONICS.

COMMA(,)INDICATES A NUMBER TO FOLLOW.

(2) MNEMONICS SHOULD BE A VALID PAL III INSTRUCTION ,

OTHERWISE THE USER WILL RECEIVE AN ERROR MESSAGE AND WILL BE

ENCOURAGED TO TRY AGAIN.

(3) INDIRECT ADRESSING IS DONE BY TYPING 'I' AFTER

THE MNEMONICS.

(4) IF THERE IS AN OPERAND IT MUST BE TWO CHARACTERS

LONG. AT THE END OF INPUT OF THE OPERAND THE TELETYPE WILL

AUTOMATICALLY MOVE TO TO THE NEXT LINE TO ACCEPT FURTHER INPUT.

IN CASE THERE IS NO OPERAND , THE USER SHOULD TYPE A CARRIAGE RETURN.

NUMBERS ARE OCTAL AND SHOULD BE PRECEDED WITH EITHER A PLUS

OR A MINUS SIGN. IN THE CASE OF A MINUS , THE SIMULATOR

ACKNOWLEDGES RECEIPT BY OUTPUTTING "(MINUS)". AFTER A NUMBER ,

THE USER SHOULD FIRST TYPE ONE CARRIAGE RETURN TO INDICATE

THE END OF INPUT OF THE NUMBER AND THEN TYPE A SECOND ONE TO

RETURN TO THE NEXT LINE.

(NOTE: ZERO MUST BE PRECEDED BY A PLUS SIGN.)

-3-

COMPILATION OF PROGRAM STARTS BY TYPING UPPERCASE P(@) AT THE

BEGINING OF A NEW LINE.  THE USER RECEIVES THE FOLLOWING MESSAGE:

: DCA

: HLT

CC	,+43

D5	,-(MINUS)21

@

COMPILATION STARTS

!!! TO INTERRUPT, ANYTHING TO CONTINUE

IF THE USER TYPES ANYTHING OTHER THAN UPPERCASE 1(!),

THE USER GETS TO 'CENT'.(THE PROGRAM CONTROL CENTER)

TO EXECUTE, THE USER SHOULD TYPE 'C'.

THE SIMULATOR RESPONDS IN THE FOLLOWING MANNER:

,CENT

CSTART EXEC. AT LABEL:

THE USER HAS TO SUPPLY A LABEL WHICH SHOWS WHERE

EXECUTION IS TO BEGIN.

!!! TO INTERRUPT, ANYTHING TO CONTINUE/CENT

CSTART EXEC. AT LABEL: KK

AC AT END OF EXECUTION: 000000000000

! TO DEBUG ANYTHING TO CONTINUE

DEBUGGING FACILITIES ENABLE USER TO EXAMINE

LOCATIONS IN THE SIMULATED PDP-8.  AFTER EXECUTION, SHOULD THE

USER TYPE UPPERCASE 1(!) THE SIMULATOR RESPONDS AS

FOLLOWS:

!!!!!...INTERUPTION...!!!

INPUT LOW LC TO TRANSLATE

400

-5-

THE USER SHOULD TYPE THE OCTAL VALUE OF THE

LOCATION, AT WHICH EXAMINATION IS TO BEGIN, IN THE PDP-8

THE SIMULATOR RESPONDS BY TYPING:

UP LC TO TRANS.

400

USER SHOULD TYPE THE OCTAL VALUE OF THE LOCATION UP

TO WHEREVER EXAMINATION IS TO BE DONE.

THE COMPUTER WILL THEN GIVE AN OCTAL LISTING OF

THOSE LOCATIONS AND THEIR CONTENTS. (SEE APPENDIX C)

TO COME BACK TO CENT.  TYPE UPPERCASE L(\)

-6-

GENERAL FEATURES

LOOKING AT APPENDIX B, ONE WILL NOTICE THE DISK

OPERATIONS AND THE VARIETY OF POSSIBILITIES IN INPUT,

EXAMINATION, EXECUTION AND DEBUGGING OF A PROGRAM.  WE

SHALL DISCUSS CERTAIN IMPORTANT PROCEDURES.

THE USE OF STATE S1 AND S3

THE '4', SHOULD NOT BE IN MOST CASES TYPED

TWICE (AT CENT), SINCE ONE FEATURE OF STATE S1 IS TO ZERO

THE WHOLE SIMULATED PDP-8 MEMORY.  THUS SHOULD THE USER

WISH TO RETURN TO INPUT MODE, STATE S3, TO INPUT NEW PROGRAMS

OR TO CHANGE EXISTING ONES, A '5' SHOULD BE TYPED (AT CENT)

INSTEAD OF '4'.

IN GENERAL, '4' IS TYPED ONLY ONCE AT THE VERY

BEGINING OF THE USE OF THE SIMULATOR, AND AFTERWARDS '5' IS

USED TO RESUME INPUT.

READING AND WRITING ON DISK FILES

THE USER SHOULD NOTE THE FOLLOWING:

WHENEVER THE USER WRITES ONTO THE DISK FILE TO STORE

A PROGRAM, WHATEVER IS ON THE DISK FILE, IS REPLACED BY

THE WHOLE OF THE CURRENT SIMULATED PDP-8 MEMORY.

SIMILARLY, READING STORED PROGRAMS FROM DISK FILE

REPLACES ALL OF THE CURRENT SIMULATED PDP-8 MEMORY BY

WHATEVER IS ON THE DISK FILE.

THUS, IF THERE IS A PROGRAM ON THE DISK FILE WHICH THE

USER WISHES TO USE WITH OTHER PROGRAMS THAT ARE YET TO

BE INPUT, THE USER SHOULD EMPLOY THE FOLLOWING PROCEDURE,

-7-

(A)  COPY FROM DISK

(B)  TYPE THE NEW PROGRAMS OR MAKE CORRECTIONS TO

THE PREVIOUSLY DISK STORED PROGRAM.

DEBUGGING

=========

THE USER SHOULD BE AWARE THAT ONCE COMPILATION IS

DONE, THE MNEMONIC SOURCE PROGRAM IS NO LONGER STORED.

THUS DEBUGGING IS DONE IN CLOSE CONNECTION WITH THE PDP-8

MACHINE LANGUAGE.  USING UPPERCASE 1(!), THE USER CAN EXAMINE

ANY LOCATION IN THE SIMULATED PDP-8 MEMORY.

EXAMPLES ARE PROVIDED IN APPENDIX C.

CORRECTIONS TO PROGRAM

======================

SHOULD A NEED TO MAKE CORRECTIONS IN A PROGRAM ARISE,

THE USER TYPE '5' AT CENT.

AFTER RECEIVING THE MESSAGE, 'INPUT PROGRAM', THE

USER SHOULD TYPE * (LOCATION NUMBER) TO RECEIVE;

LC VALUE (*) KNOWN IS IN LC

THEN THE LOCATION CAN BE REPLACED SIMPLY BY TYPING THE NEW LINE.

REPLACEMENTS CAN BE DONE FOR ANY OTHER LOCATIO WITH

NO NEED TO GO BACK TO CENT.

CONSULT APPENDIX C FOR FURTHER DETAILS AND EXAMPES

-8-

CONCLUSION

A FINAL REMARK CONCERNS THE REACTION TIME OF THE

TELETYPE.  WHEN THE COMPUTER IS VERY BUSY, THE USER ADVISED TO TYPE

THE PROGRAM VERY SLOWLY AND ALWAYS TO USE A RELIABLE TELETYPE.

THE USE OF THE PRINTING TELETYPE IS PREFERRED TO THAT

OF THE CRT TELETYPE.

THE MOST IMPORTANT PART OF THIS PAPER IS APPENDIX B

AND THE USER IS ADVISED TO HAVE A COPY OF IT CLOSE BY

WHILE USING THE SIMULATOR PROGRAM.  WE ARE AWARE THAT USERS MIGHT

FIND IT DIFFICULT TO ADJUST TO A SYSTEM THAT HAS MNEMONICS INPUT

FACILITIES, MACHINE LANGUAGE DEBUGGING FACILITIES AND IN

WHICH COMPILING, LOADING, DEBUGGING AND EXECUTION ARE BASICALLY

DONE CONTINUOUSLY AND AT ANY DESIRED SEQUENCE.

NEVERTHELESS, WE FEEL IT IS A GOOD INTRODUCTION TO

BOTH MACHINE LANGUAGE AND SYSTEM PROGRAMMING.

THROUGH THIS PROJECT WE HAVE GAINED A NEW INSIGHT

INTO SOFTWARE DESIGN PROBLEMS, HOWEVER PRIMITIVE AND LIMITED

THE OPERATING SYSTEM THAT WE HAVE CREATED IS.

CIRCLE	: A CHARACTER TO TYPE WHEN IN CENT.

ELIPSE	: A CHARACTER TO TYPE TO SWITCH STATES

S                S  : BY TYPING C YOU GET FROM

STATE S   TO STATE  S

: INDICATES AUTOMATIC TRANSITION AFTER COMPLETION OF MISSION.

CENT: ANY TYPING IN THE 'CENT' STATE SHOULD BE DONE

FOLLOWING THE OUTPUT OF THE WORD 'CENT'.

! : IN CENT, S3 :TYPING UILL PRODUCE TRANSLATION

MESSAGES BEGINING "LOW LC TO TRANSLATE".

AT ALL OTHER STATES UPPERCASE 1(!) SHOULD BE TYPED ONLY WHEN

MESQAGE GIVES THE USER OPTION TO DO SO.

APPENDIX D

THE FOLLOWING ARE PAL III INSTRUCTIONS THAT CAN BE USED

ON THE SIMULATOR.

MEMORY REFERENCE INSTRUCTIONS

AND

TAD

ISZ

DCA

JMS

JMP	(JMP .-1 NOT ALLOWED)

GROUP 1 OPERATE MICROINSTRUCTIONS

NOP

CLA

CLL

CMA

CML

RAR

RAL

RTR

RTL

IAC

GROUP 2 OPERATE MICROINSTRUCTIONS

SMA

SZA

SPA

SNA

SNL

SZL

SKP

HLT

TELETYPE KEYBOARD READER

KCF

KSF

KCA

KRS

KRB

TELETYPE TELEPRINTER

TFL

TSF

TCF

TPC

TLS

00020			APPENDIX C

00040

00060

00080

00100	* 200

00200	LC VALUE (*) KNOWN IS IN LC

00300

00400	ST	: CLA

00500			: CLL

00580			: TAD RQ

00590			: DCA RG

00600			: TLS

00700			: TAD MC

00800			: DCA CT

00900	NX	: TAD I RG

01000			: JMS TY

01100			: ISZ RG

01200			: ISZ CT

01300			: JMP NX

01400			: HLT

01500	TY	,+0

01600	T2	: TSF

01700			: JMP T2

01800			: TLS

01900			: CLA

02000			: JMP I TY

02100	MC	,-(MINUS)15

02200	CT	,+0

02300	RG	,+0

02350	RQ	,+400

02400	*400

02500	LC VALUE (*) KNOWN IS IN LC

02600

02700	JJ	,+312

02800	OO	,+317

02900	HH	,+310

03000	N1	,+316

03100	B1	,+240

03200	A1	,+301

03300	N2	,+316

03400	DD	,+304

03500	B2	,+240

03600	MM	,+315

03700	A2	,+301

03800	RR	,+322

03900	YY	,+331

04000	@

04100	COMPILATION STARTS

04200

04300	!!! TO INTERRUPT, ANYTHING TO CONTINUE/CENT

04400	CSTART EXEC. AT LABEL: ST

04500

04550	JOHN AND MARY

04600	AC AT END OF EXECUTION : 000000000000

04700	!  TO DEBUG ANYTHING TO CONTINUE/END OF DEBUG

05000	UPPER CASE L(\) TO EXIT ANYTHING TO DEBUG AGAIN

05100	\CENT

05300	5INPUT PROGRAM

05400	* 221

05600	LC VALUE (*) KNOWN IS IN LC

05700

05800	MC	,-(MINUS)14

05900	*411

05980	LC VALUE (*) KNOWN IS IN LC

05990

06000	SS	,+323

06100	UU	,+325

06200	EE	,+305

06300	@

06400	COMPILATION STARTS

06500

06600	!!! TO INTERRUPT, ANYTHING TO CONTINUE!!!!...INTERUPTION...!!!

06700	INPUT LOW LC TO TRANSLATE

06800

06900	221

07100	UP LC TO TRANS.

07200

07300	221

07400

07500		000000000221     777777777764

07600

07650	CENT

07700	!!!!...INTERUPTION...!!!

07800	INPUT LOW LC TO TRANSLATE

07900

08000	411

08200	UP LC TO TRANS.

08300

08400	413

08500

08600		000000000411     000000000323

08650

08700		000000000412     000000000325

08750

08800		000000000413     000000000305

08850

08900	CENT

09000	CSTART EXEC. AT LABEL: ST

09100

09200	JOHN AND SUE

09300

09400	AC AT END OF EXECUTION: 000000000000

09500	!  TO DEBUG ANYTHING TO CONTINUEH END OF DEBUG

09600	UPPERCASE L(\) TO EXIT, ANYTHING TO DEBUG AGAIN\

09700	CENT

00100			APPENDIX A

00200

00300

00400

00500	* 200

00600	LC VALUE (*) KNOWN IS IN LC

00700

00800	BG	: CLA

00900			: CLL

01000			: TAD N1

01100			: TAD N2

01200			: TAD N3

01300			: DCA SM

01400			: HLT

01500	N1	,+2

01600	N2	,-(MINUS)3

01700	N3	,+10

01800	SM	,+0

01900	@

02000	COMPILATION STARTS

02100

02200	!!! TO INTERUPT, ANYTHING TO CONTINUE/CENT

02300	6CENT

02400	CSTART EXEC. AT LABEL: BG

02500

02600	AC AT END OF EXECUTION: 000000000000

02700	!  TO DEBUG ANYTHING TO CONTINUE!!!!...INTERUPTION...!!!

02800	INPUT LOW LC TO TRANSLATE

02900

03000	212

03100	UP LC TO TRANS.

03200

03300	212

03400

03500		000000000212     000000000007

03600

03700	END OF DEBUG

03800	UPPERCASE L(\) TO EXIT, ANYTHING TO DEBUG AGAIN

03900	\CENT

BOX 1537,

BRANDEIS UNIVERSITY,

WALTHAM, MASS. 02154

OCT. 24, 1975

DEAR SIRS,

ENCLOSED IS A PAPER DESCRIBING THE FEATURES OF

A PDP-8 SIMULATOR.

PLEASE, LET US KNOW WHETHER YOU ARE INTERESTED IN

THIS SIMULATOR PROGRAM.

SINCERELY YOURS

ODED SHMUELI

KWABENA AKUFO

DIGITAL EQUIPMENT CORPORATION

SOFTWARE DEPARTMENT

MAYNARD,MASSACHUSETTS 01754

###