THIS IS THE DOCUMENTATION FILE FOR TAPTST.MAC

1)  AUTHOR:	ANDY COPANAS - DEC,MAYNARD,MASS

2)  DATE SUBMITTED:	FEBRUARY 10,1971

3)  PURPOSE:

TAPTST IS DESIGNED TO GIVE A MAGTAPE A WORKOUT IN

ORDER TO TRY AND FIND ITS BAD REGIONS, SO THAT THE USER WILL

HAVE A FAIR IDEA OF ITS RELIABILITY.

THIS IS ACCOMPLISHED BY WRITING A BUFFERFUL OF 1'S

WHICH IS 4800 (DECIMAL) WORDS LONG.  THIS MEANS THAT THERE

WILL BE FEW INTER-RECORD GAPS, THUS TESTING MOST OF THE TAPE'S

SURFACE.  (4800 WDS = 3FT. OF TAPE AT 800 BPI)

4)  METHOD:

TAPTST WILL ASK THE USER FOR THE TYPE OF TEST THAT HE

DESIRES. THE THREE CHOICES ARE:  1)READ & VERIFY AT 556 BPI ONLY,

2)READ & VERIFY AT 800 BPI ONLY, OR 3)READ & VERIFY AT 556, THEN AT

800 BPI.

WHEN THE TEST HAS BEEN SET UP, TAPTST WILL PROCEED TO

FILL THE TAPE WITH ONES, AND THEN WILL REWIND THE TAPE AND

PERFORM A VERIFICATION OPERATION TO SEE IF ALL RECORDS

CONTAIN ONLY ONES IN THEM.  THE PROGRAM COUNTERS WILL KEEP

TRACK OF THE NUMBER OF RECORDS READ OR WRITTEN ON THE TAPE.

THERE ARE TWO TYPES OF ERRORS THAT MAY OCCUR, THE

PROGRAM MAY BE UNABLE TO READ OR WRITE THE RECORD DUE TO

DATA OR PARITY ERRORS, OR THE VERIFICATION OF A WORD WITH-

IN A RECORD MAY FAIL.  IT IS DESIRABLE TO KNOW ABOUT BOTH

KINDS OF FAILURES.

TAPTST DOESN'T USE THE AUTOMATIC ERROR-CORRECTION FEATURE OF THE

MONITOR (BIT 29 OF THE MAGTAPE STATUS BITS).  THIS ALLOWS TAPTST

TO PERFORM ERROR RECOVERY EVERY TIME DATA OR PARITY ERRORS

OCCUR.  WHEN, FOR THESE REASONS, THE RECORD CANNOT BE WRITTEN OR

READ, TAPTST WILL GIVE THE TAPE A MAXIMUM OF 10 (DECIMAL)

TRIES BEFORE IT GIVES UP.  IT WILL INDICATE THIS TO THE USER,

AND WILL SKIP OVER THE BAD RECORD AND CONTINUE OPERATION.

IF A RECORD IS SUCCESSFULLY READ, THEN ANY ERRORS IN

VERIFYING WILL BE PINPOINTED AS FOLLOWS:

1) THE INCORRECT DATA WORD WILL BE PRINTED (IN OCTAL)

2) RECORD # (IN OCTAL)

3) SIZE OF RECORD (SHOULD ALWAYS BE 11300 OCTAL WDS.,

EXCEPT FOR LAST RECORD)

4) OCTAL POSITION WITHIN THE RECORD (I.E. 1ST WD, 577TH WD.,ETC)

THIS SHOULD HELP THE USER TO GAIN A ROUGH IDEA OF WHICH AREA

ON THE TAPE IS BAD, AND HOW BAD IT REALLY IS.

5)  SAMPLE OPERATING INSTRUCTIONS: (AND SAMPLE ERROR MESSAGES)

.LOG

JOB 1 DEC DATA CENTER SYS#169 TTY2

#13,602

PASSWORD:

1555	09-FEB-71

.AS MTA1 TAPTST

MTA1 ASSIGNED

.EX TAPTST		;OR- RUN DSK(DTAN) TAPTST

MACRO: TAPTST

LOADING

LOADER 6K CORE

EXECUTION

DO YOU WANT ONE PASS AT 556 BPI? (Y OR C.R.)

DO YOU WANT ONE PASS AT 800 BPI? (Y OR C.R.)

DO YOU WANT 2 PASSES, ONE AT 556,

THEN AT 800? (Y OR C.R.)

Y

556 BPI

NOW WRITING TAPE

0005   ATTEMPTS TO READ/WRITE ON TAPE

RECORD NUMBER (IN OCTAL)    0057

0002   ATTEMPTS TO READ/WRITE ON TAPE

RECORD NUMBER (IN OCTAL)    0237

READ/WRITE FAILED AFTER 10 TRIES ON

RECORD NUMBER (IN OCTAL)    0301

NOW VERIFYING TAPE

READ/WRITE FAILED AFTER 10 TRIES ON

RECORD NUMBER (IN OCTAL)    0057

VERIFY ERROR ON DATA WORD   177777770000

RECORD NUMBER (IN OCTAL)    0237

RECORD SIZE (IN OCTAL) =   011300

POSITION WITHIN THIS RECORD =    000356

VERIFY ERROR ON DATA WORD   676574738899

RECORD NUMBER (IN OCTAL)    0237

RECORD SIZE (IN OCTAL) =   011300

POSITION WITHIN THIS RECORD =    011001

READ/WRITE FAILED AFTER 10 TRIES ON

RECORD NUMBER (IN OCTAL)    0301

800 BPI

NOW WRITING TAPE

READ/WRITE FAILED AFTER 10 TRIES ON

RECORD NUMBER (IN OCTAL)    1000

NOW VERIFYING TAPE

READ/WRITE FAILED AFTER 10 TRIES ON

RECORD NUMBER (IN OCTAL)    1000

VERIFY ERROR ON DATA WORD   777777777773

RECORD NUMBER (IN OCTAL)    1040

RECORD SIZE (IN OCTAL) =   011300

POSITION WITHIN THIS RECORD =    007742

DONE

EXIT

.(AT THIS POINT, THE USER COULD MOUNT A NEW TAPE

ON THE SAME DRIVE, TYPE "REEN", AND START OVER AGAIN.)

6)  EXECUTION TIME: ABOUT 50 MIN. (OF REAL TIME) FOR 2 PASSES

OF A 2400 FOOT REEL. (556 & 800 BPI)