1

/******PROGRAM PDIS.PAL,  1969******

/PETER LEMKIN

/NATIONAL INSTITUTES OF HEALTH

/BETHESDA, MD. 20014

/

/

/

/INTRODUCTION:

/------------

/PDIS IS A SUBROUTINE FOR THE LINC8 COMPUTER USED TO

/ACCESS THE LINC CPU'S POINT DISPLAY  HARDWARE FROM

/THE PDP8 CPU. THIS GREATLY FACILITATES INCORPORATING THE

/LINC DISPLAY IN PDP8 PROGRAMS.

/

/

/

/

/HARDWARE REQUIRED:

/------------------

/LINC8 COMPUTER WITH AT LEAST 4K OF MEMORY.

/PDIS WILL COMPILE WITH PAL.

/

/

/

/

/

/DESCRIPTION:

/----------------

/SUBROUTINE PDIS IS A POINT DISPLAY ROUTINE WHICH CAN BE

/USED FOR DISPLAYING ARBITRARY SETS OF (X,Y) COORDINATES ON

/THE LINC DISPLAY SCOPE. IT MAY BE CALLED FROM ANY FIELD. IT SHOULD

/BE NOTED THAT THE DISPLAY ROUTINE MUST BE RESIDENT IN THE PART OF

/MEMORY WHICH CORRESPONDS TO THE CURRENT LINC CPU MEMORY BANK.

/THE (X,Y)=(0,0) POSITION IS THE ENTER OF THE SCREEN. (+X,+Y)

/IS TO THE RIGHT AND UP RESPECTIVELY.

/CALLING SEQUENCE:

/     JMS PDIS

/     Y  /Y COORD -Y=-377, +Y=377

/     X  /X COORD -X=-377, +X=377

/     NORMAL RETURN

/&

0000		FIELD 0

4000		*4000  /CORRESPONDS TO BANK 2 IN THE LINC CPU.

6165		ISSP=6165

6141		ICON=6141

/

/

/

/

4000	0000	PDIS,	0

4001	7300		CLA CLL

4002	1600		TAD I PDIS /GET Y

4003	3253		DCA Y

4004	2200		ISZ PDIS

4005	1245		TAD PK377 /COMPUTE  THE REAL X NEEDED FOR THE DISPLAY

2

4006	1600		TAD I PDIS /GET X

4007	3252		DCA X

4010	6214		RDF /SET UP RETURN

4011	1242		TAD PKCIF

4012	3237		DCA PDISEX

4013	6201		CDF 00

4014	1644		TAD I PK4010 /SAVE MEMORY

4015	3243		DCA PKSAVE

4016	2200		ISZ PDIS /SET UP RETURN

/NOW GO GET SET TO LINC UP.

/CLEAR FLAGS

4017	1246		TAD PK11

4020	6141		ICON /CLEAR LINC HANGUPS

4021	7200	        CLA

4022	1241		TAD GOLINC

4023	6165		ISSP

4024	7200		CLA

4025	1247		TAD PK10 /SELECT

4026	6141		ICON

4027	1251		TAD PK2

4030	6141	MP,	 ICON /GO DISP

4031	7200		CLA

4032	1246		TAD PK11 /DESELECT THE LINK

4033	6141		ICON

4034	7200		CLA

4035	1243		TAD PKSAVE /RESTORE MEMORY

4036	3644		DCA I PK4010

4037	6202	PDISEX,	CIF 00

4040	5600		JMP I PDIS /RETURN

/*****CONSTANTS******

4041	4054	GOLINC,	 LINC

4042	6202	PKCIF,	CIF 00

4043	0000	PKSAVE,	0

4044	4010	PK4010,	4010

4045	0377	PK377,	377

4046	0011	PK11,	11

4047	0010	PK10,	10

4050	0007	PK7,  	7

4051	0002	PK2,	2

4052	0000	X,	0

4053	0000	Y,	0

/

/

/

/

/

/SUBROUTINE LINC

/IS A LINC CODE SUBROUTINE USED TO DISPLAY 1

/POINT  AT X AND Y. IT IS CALLED AND STARTED BY PDIS.

/

/&

4054	0050	LINC,	0050	/SET 10 ----- LINC CODE

4055	0052		X-4000

4056	1000		1000	/LDA IN LINC CODE

4057	0053		Y-4000

3

4060	0150		150	/DIS 10....DISPLAY CHANNEL 0

4061	0000		0000    /HLT IN LINC CODE

/RETURN TO PDP8 CPU

/CHANGE BACK MODE.

/.....................................................

$