WESTERN MICHIGAN UNIVERSITY

COMPUTER CENTER

LIBRARY PROGRAM #2.5.1

CALLING NAME:   ECAP

PREPARED BY:    DAVID C. SHULTZ AND RUSSELL R. BARR III

PROGRAMMED BY:  *

APPROVED BY:    JACK R. MEAGHER

DATE:		JULY 20, 1975  (VERION 3)

ELECTRONIC CIRCUIT ANALYSIS PROGRAM

TABLE OF CONTENTS

1.0  INTRODUCTION

2.0  ECAP INPUT LANGUAGE

3.0  RESPONSES TO QUESTIONS - INPUT? AND OUTPUT?

4.0  RESPONSES FOR READY

5.0  LIMITATIONS

6.0  EXAMPLE RUNS

1.0  INTRODUCITON

THIS VERSION OF ECAP WAS CREATED IN RESPONSE TO REQUESTS THAT ECAP  (WESTERN

MICHIGAN'S ORIGINAL COPY OF THE ELECTRONIC CIRCUIT ANALYSIS PROGRAM) BE MADE

MORE CONVENIENT IN AN INTERACTIVE ENVIRONMENT, INCLUDING ERROR RECOVERY, MODEL

EDITING AND INPUT/OUTPUT OPTIONS.  THESE FEATURES AND SOME OTHERS WERE

INCORPORATED INTO ECAP.

THE PROGRAM ECAP IS AN INTEGRATED SYSTEM OF PROGRAMS DEVELOPED  PRIMARILY TO

AID THE ELECTRICAL ENGINEER IN THE DESIGN AND ANALYSIS OF ELECTRONIC

CIRCUITS.  DC ANALYSIS, AC ANALYSIS AND TRANSIENT ANALYSIS CAPABILITIES ARE

AVAILABLE.

THE DC ANALYSIS PROGRAM OBTAINS THE STEADY-STATE SOLUTION OF LINEAR ELECTRICAL

NETWORKS AND PROVIDES THE WORST CASE ANALYSIS DEVIAION (STATISTICAL) ANALYSIS,

AND SENSITIVITY COEFFICIENT CALCULATIONS IF REQUESTED.  THIS PROGRAM PROVIDES

AN AUTOMATIC PARAMETER-MODIFICATION CAPABILITY.

THE AC ANALYSIS PROGRAM OBTAINS THE STEADY-STATE SOLUTION OF LINEAR ELECTRICAL

NETWORKS SUBJECT OF SINE-WAVE EXCITATION AT A FIXED FREQUENCY.  SINCE THIS

PROGRAM ALSO CONTAINS THE AUTOMATIC PARAMETER-MODIFICATION CAPABILITY,  IT IS

EASY TO OBTAIN FREQUENCY AND PHASE-RESPONSE SOLUTIONS.

--------------------

*ECAP (DECUS #10-34) WAS OBTAINED FROM THE DIGITAL EQUIPMENT USER'S

SOCIETY. SUBSTANTIAL ADAPITIVE PROGRAMMING WAS DONE  AT WESTERN MICHIGAN

UNIVERSITY BY SAM ANEMA AND RUSSELL BARR.  THIS VERSION OF ECAP CONTAINS

SUBSTATIAL REPROGRAMMING AND ERROR CORRECTION BY DAVID SCHULTZ AT WESTERN

MICHIGAN UNIVERSITY.

THE TRANSIENT ANALYSIS PROGRAM OBTAINS THE TIME-RESPONSE SOLUTION (VOLTAGES

AND CURRENTS) OF BOTH LINEAR AND NON-LINEAR ELECTRICAL NETWORKS.

COMPLETE DETAILS ABOUT THE USE OF THIS PROGRAM ARE GIVEN IN THE BOOK, "IBM

ELECTRONIC CIRCUIT ANALYSIS PROGRAM--TECHNIQUES AND APPLICATIONS" BY JENSEN

AND LEIBERMAN (PRENTICE-HALL 1968).  MANY EXAMPLES WITH ANSWERS ARE  GIVEN IN

THIS BOOK.

DIFFERENCES BETWEEN THE WMU ECAP AND ECAP DESCRIBED IN THE IBM MANUAL ARE DETAILED

IN SECTION 2.0 THROUGH 6.0.

NOTE: UNDERFLOW MESSAGES CAN BY CAUSED BY VERY LARGE RESISTONS BEING

INPUTTED.

2.0  ECAP INPUT LANGUAGE

GENERALLY THIS VERSION'S INPUT LANGUAGE IS THE SAME AS THAT OF THE IBM ECAP

WITH THE EXCEPTIONS NOTED IN THE FOLLOWING SUBSECTIONS.

2.1  INPUT/OUTPUT

THE ADDITION OF THE QUESTIONS INPUT? AND OUTPUT? GIVE THE USER MORE FREEDOM

OF ACTION WITH HIS INPUT AND OUTPUT.  BECAUSE OF THIS, IT IS NO LONGER

NECESSARY OR POSSIBLE TO USE THE AS DSK 5 AND AS DSK 30 COMMANDS WITH ECAP.

SEE SECITON 3.0 FOR MORE INFORMATION.

2.2  DC, AC, AND TR

THE DC (OR DC ANALYSIS), AC (OR AC ANALYSIS) AND TR (OR TRANSIENT ANALYSIS)

COMMANDS NOW ARE USED AS A COMMAND (STARTING IN COLUMN 1) AFTER THE MODEL IS

ENTERED, INSTEAD OF AS SOLUTION CONTROL LINES.  SEE SECTION 6.0 FOR EXAMPLES.

2.3  EXECUTE

THE EXECUTE SOLUTION CONTROL LINE IS NOW NEEDED ONLY AFTER MODIFY GROUPS AND

IS OPTIONAL AT OTHER TIMES.

2.4  SOLUTION CONTROL LINES

SOLUTION CONTROL LINES ARE THOSE THAT DO NOT START WITH A LETTER IN COLUMN 1.

THAT IS: FREQUENCY, SENSITIVITY, EXECUTE, ETC.  THESE LINES START WITH  BLANKS

OR A TAB(CONTROL-I) AND MAY BE PRECEEDED BY AN OPTIONAL NUMBER FOR EDITING

PURPOSES.  IF NO NUMBERS ARE USED, THE INTERNAL REPRESENTATION WILL START AT

10 AND INCREASE BY 10 FOR EACH SOLUTION CONTRL LINE.

2.5 MODIFY

THE MODIFY COMMAND GROUP IS THE SAME EXCEPT THAT LINES ADDED TO THE MODIFY

GROUP OUT OF SEQUENCE (AS IN EDITING) SHOULD BE PRECEEDED WITH A NUMERIC  LINE

NUMBER TO DIFFERENTIATE BETWEEN NORMAL LINES IN THE MODEL AND THOSE IN THE

MODIFY GROUP.  IT IS SUGGESTED THAT WHENEVER THE MODIFY COMMAND IS USED, ALL

SOLUTION CONTROL LINES BE PRECEEDED WITH A NUMBER TO INSURE THE PROPER ORDER

OF EACH STATEMENT WHEN EDITING IS DONE.

EXAMPLE:

B5  N(0),R=10,E=100

10  PRINT, VOLTAGES

20  MODIFY

30  B5    R=20(3)80

40  EXECUTE

3.0  RESPONSES TO QUESTIONS-INPUT? AND OUTPUT?

THIS SECTION EXPLAINS THE FOLLOWING TWO QUESTINS YOU WILL ENCOUNTER:

INPUT?  (TYPE HELP IF NEEDED)--

AND

OUTPUT?  (TYPE HELP IF NEEDED)--

RESPONSES TO THESE QUESTIONS DEFINE FROM WHERE THE USER WISHES TO READ HIS

INPUT MODEL (INPUT?) AND WHERE THE USER WISHES TO WRITE HIS OUTPUT DATA

OR SAVE HIS MODEL (OUTPUT?).  SEE NOTE 2 BELOW FOR OTHER INPUT OPTIONS.

THE PROPER RESPONSE TO EACH OF THESE QUESTIONS CONSISTS OF THREE BASIC PARTS:

A DEVICE, A FILENAME, AND A PROJECT-PROGRAMMER NUMBER.

THE GENERAL FORMAT FOR THESE THREE PARTS IS AS FOLLOW:

DEVICE:FILE.EXT[PROJ,PORG]

1) DEV:  ANY OF THE FOLLOWING DEVICES ARE APPROPRIATE WHERE INDICATED:

DEVICE LIST	DEFINITION		STATEMENT USE

TTY:		TERMINAL		INPUT OR OUTPUT

DSK:		DISK			INPUT OR OUTPUT

CDR:		CARD READER		INPUT ONLY

LPT:		LINE PRINTER		OUTPUT ONLY

DTA0:		DECTAPE 0		INPUT OR OUTPUT

DTA1:		DECTAPE 1		INPUT OR OUTPUT

DTA2:		DECTAPE 2		INPUT OR OUTPUT

DTA3:		DECTAPE 3		INPUT OR OUTPUT

DTA4:		DECTAPE 4		INPUT OR OUTPUT

DTA5:		DECTAPE 5		INPUT OR OUTPUT

DTA6:		DECTAPE 6		INPUT OR OUTPUT

DTA7:		DECTAPE 7		INPUT OR OUTPUT

MTA0:		MAGNETIC TAPE 0		INPUT OR OUTPUT

MTA1:		MAGNETIC TAPE 1		INPUT OR OUTPUT

INPUT MAY NOT BE DONE FROM THE LINE PRINTER NOR MAY OUTPUT GO THE CARD READER.

2) FILE.EXT IS THE NAME AND THE EXTENSION OF THE FILE TO BE USED.  THIS

PART OF THE SPECIFICATION IS USED ONLY IF DISK OR DECTAPE IS USED.

3)  [PR0J,PROG]  IF A DISK IS USED AND THE USER WISHES TO READ A FILE IN

ANOTHER PERSON'S DIRECTORY, HE MAY DO SO BY SPECIFYING THE PROJECT-PROGRAMMER

NUMBER OF THE DIRECTORY FROM WHICH HE WISHES TO READ.  THE PROJECT NUMBER AND

THE PROGRAMMER NUMBER MUST BE SEPARTED BY A COMMA AND ENCLOSED IN BRACKETS.

OUTPUT MUST GO TOYOUR OWN AREA.

EXAMPLE:

OUTPUT?    LPT:/2

INPUT?     DSK:DATA.DAT[71171,71026]

IN THE EXAMPLE, TWO COPIES OF THE OUTPUT ARE TO BE PRINTED BY THE HIGH SPEED

LINE PRINTER.  THE INPUT DATA IS A DISK FILE OF DATA.DAT IN USER

DIRECTORY [71171,71026].

DEFAULTS:

1)  IF NO DEVICE IS SPECFIED BUT A FILE NAME IS SPECIFIED THE DEFAULT DEVICE

WILL BE DSK:

2)  IF NO FILENAME IS SPECIFIED AND A DISK OR DECTAPE IS USED THE DEFAULT ON

INPUT WILL BE FORM INPUT.DAT: ON OUTPUT IT WILL BE OUTPT.DAT.

3)  IF THE PROGRAM IS RUN FROM THE TERMINAL AND NO SPECIFICAION IS GIVEN

(JUST A CARRIAGE RETURN)  BOTH INPUT AND OUTPUT DEVICES WILL BE THE TERMINAL.

4)  IF THE PROGRAM IS RUN THROUGH BATCH AND NO SPECIFICATION OS GIVEN( A

BLANK CARD) THE INPUT DEVICE WILL BE DEVICE CDR:  AND THE OUTPUT DEVICE

WILL BE LPT:

5)  IF NO PROJECT-PROGRAMMER NUMBER IS GIVEN, THE USER'S OWN NUMBER WILL

BE ASSUMED.

NOTE:	(1)  IF LPT:  IS USED AS AN OUTPUT DEVICE MULTIPLE COPIES MAY BE OBTAINED

BY SPECIFYING LPT:/N WHERE N REFERS TO THE NUMBER OF COPIES DESIRED.

(2)  THE FOLLOWING TWO OPTIONS ARE NOT APPLICABLE FOR THE FIRST TIME

INPUT? IS ENCOUNTERED, I.E., IT IS APPLICABLE ONLY WHEN THE PROGRAM

BRANCHES BACK TO INPUT?

(A)	SAME OPTION

UPON RETURNING TO INPUT? IF THE SAME DATA FIEL IS TO BE USED

AGAIN, SIMPLY ENTER SAME, OTHERWISE, EITHER USE THE FINISH

OPTION OR ENTER ANOTHER FILE NAME,ETC.

(B)	FINISH OPTION

WHEN AT AN INPUT?, THE USER MUST ENTER FINISH TO BRANCH OUT

OF THE PROGRAM.  FAILURE TO DO SO, SUCH AS USING ^C (CONTROL C)

MIGHT RESULT IN LOSING THE ENTIRE FILE.

4.0  RESPONSES FOR READY

ANY OF THE FOLLOWING COMMANDS MAY BE TYPED IN RESPONSE TO A READY OR IF ECAP

IS NOT SPECIFICALLY REQUESTING OTHER RESPONSE (SUCH AS: INPUT?). NONE  OF

THE COMMANDS IN THIS SECTION ARE PART OF ECAP'S INPUT LANGUAGE BUT INSTEAD

ARE TYPED AT THE LEFT MARGIN ON THE TERMINAL.

COMMAND			PURPOSE

DC			START DC ANALYSIS

AC			START AC ANALYSIS

TR			START TRANSIENT ANALYSIS

LIST			LIST CURRENT MODEL ON TERMINAL

REMOVE  NN		REMOVE SOLUTION CONTROL LINE "NN"

SAVE			STORE CURRENT MODEL

NEW			INPUT A NEW MODEL

EXIT			CLOSE OUTPUT FILES AND EXIT FROM ECAP

HELP			TYPE THIS TEXT

IN ADDITION, YOU MAY ALSO ENTER A NEW LINE TO THE MODEL OR RETYPE A LINE

PREVISOLY ENTERED.  SEE SECTION 6.0 FOR EXAMPLES OF THE USE OF SOME OF THESE

COMMANDS.

NOTES:

A)  IF THE SAVE COMMAND IS USED AND THE RESPONSE TO THE SUBSEQUENT OUTPUT?

IS LPT:, A LISTING OF THE MODEL MAY BE OBTAINED ON THE LINE PRINTER IMMEDIATELY

WITHOUT EXITING FROM THE PROGRAM.

B)  TO RE-EMPHASIZE:  ONLY SOLUTION CONTROL LINES MY BE REMOVED FROM THE MODEL.

ONCE A BRANCH OR A NODE IS ESTABLISHED, IT MAY NOT BE REMOVED.

5.0  LIMITATIONS

A MAXIMUM OF:

(A)  10 NUMBERED COMMENT LINES.

(B)  60 BRANCH LINES

(C)  5 MUTUAL INDUCTANCE LINES.

(D)  20 SWITHC LINES

(E)  10 DEPENDENT CURRENT SOURCE LINES.

(F)  25 SOLUTION CONTROL LINES

(G)  NUMBERS WITH MAGNITUDES GREATER THAN 1E36 OR LESS THAN

1E-36 MAY CAUSE OVERFLOWS OR UNDERFLOWS.

6.0  EXAMPLE RUNS

THE FOLLOWING EXAMPLES WERE OBTAINED FROM THE BOOK "IBM ELECTRONIC CIRCUIT

ANALYSIS PROGRAM--TECHNIQUES AND APPLICATIONS"  BY JENSEN AND LIEBERMAN.  THE

USER'S RESPONSES ARE UNDERLINED TO SEPARATE THEN FROM PROGRAM OUTPUT.

6.1  TERMINAL EXAMPLE #1

THIS EXAMPLE IS BASED ON THE SINGLE STAGE COMMAND EMITTER AMPLIFIER ON

PAGE 58 OF THE IBM ECAP MANUAL. IT IS ENTERED FROM THE TERMINAL. AN OMITTED

BRANCH (B6) IS ENTERED BEFORE ATTEMPTING TO START EXECUTION OF THE MODEL.

.R ECAP<CR>

WMU ELECTRONIC CIRCUIT ANALYSIS PROGRAM (VERSION 3)

INPUT? (TYPE HELP IF NEEDED)--TTY:<CR>

READY(TYPE HELP IF NEEDED)

C1	SINGLE STATE COMMON EMITTER AMPLIFIER<CR>

C2	PAGE 58 IBM ECAP MANUAL - DC ANALYSIS<CR>

B1	N(0,2),R=2E3,E=20<CR>

B2	N(0,1),R=6E3,E=20<CR>

B3	N(0,1),R=1000<CR>

B4	N(1,3),R=350,E=-.5<CR>

B5	N(3,0),R=500<CR>

T1	B(4,6),BETA=50<CR>

SENSITIVITIES<CR>

PRINT, SENSITIVITIES<CR>

B6	N(2,3),R=11.1E3<CR>

DC<CR>

OUTPUT? (TYPE HELP IF NEEDED)--TTY:<CR>

C1	SINGLE STATE COMMON EMITTER AMPLIFIER

C2	PAGE 58 IBM ECAP MANUAL - DC ANALYSIS

PARTIAL DERIVATIVES AND SENSITIVITIES OF NODE VOLTAGES

WITH RESPECT TO RESISTANCES

BRANCH   NODE       PARTIALS            SENSITIVITIES

1        1       -6.36672142D-06     -1.27334428E-04

1        2       -4.41292319D-03     -8.82584639E-02

1        3       -8.96646600D-06     -1.79329320E-04

2        1       -3.93767894D-04     -2.36260737E-02

2        2        1.51202726D-03      9.07216361E-02

2        3       -3.87278038D-04     -2.32366824E-02

3        1        2.30212613D-03      2.30212614E-02

3        2       -8.83992203D-03     -8.83992203E-02

3        3        2.26418382D-03      2.26418381E-02

4        1        2.84804864D-06      9.96817028E-06

4        2        2.70948545D-04      9.48319910E-04

4        3       -6.93984979D-05     -2.42894743E-04

5        1        1.69551441D-04      8.47757205E-04

5        2        1.67974570D-02      8.39872854E-02

5        3        2.38784946D-04      1.19392473E-03

6        1       -1.13133789D-06     -1.25578506E-04

6        2        9.01299184D-06      1.00044209E-03

6        3       -1.59330086D-06     -1.76856396E-04

WITH RESPECT TO BETAS

BETA     NODE       PARTIALS            SENSITIVITIES

1        1        1.16225512D-03      5.81127570E-04

1        2       -9.25929915D-03     -4.62964963E-03

1        3        1.63684263D-03      8.18421329E-04

WITH RESPECT TO VOLTAGE SOURCES

BRANCH   NODE       PARTIALS            SENSITIVITIES

1        1        1.42635039D-03      2.85270078E-04

1        2        9.88636742D-01      0.19772735

1        3        2.00877680D-03      4.01755362E-04

2        1        1.37314752D-01      2.74629507E-02

2        2       -5.27274195D-01     -0.10545484

2        3        1.35051610D-01      2.70103221E-02

4        1       -3.87967306D-02     -1.93983653E-04

4        2       -3.69091938D+00     -1.84545969E-02

4        3        9.45361271D-01      4.72680637E-03

READY

EXIT<CR>

END OF EXECUTION

CPU TIME: 0.72	ELAPSED TIME: 5.93

EXIT

6.2 TERMINAL EXAMPLE #2

THIS EXAMPLE IS BASED ON THE SINGLE STAGE COMMON EMITTER AMPLIFIER ON PAGE 91

OF THE IBM ECAP MANUAL WITH PARAMETER MODIFICAION ADDED TO VARY THE

RESISTANCE OF BRANCH B5 FROM 100 TO 700 OHMS IN STEPS OF 200.  NOTE THAT THERE

IS AN ERROR IN THE DISK FILE CONTAING THE MODEL (EC91.DAT)WHICH WILL BE

CORRECTED AFTER ECAP FINDS IT.  AFTER THE CORRECTED MODEL IS RUN SUCCESSFULLY

IT WILL BE SAVED ON DISK WITH THE NAME EC91.DAT AND THE PROGRAM WILL BE EXITED.

.TYPE EC91.DAT<CR>

C1	SINGLE STAGE COMMON EMITTER AMPLIFIER

C2	PAGE 91 - IBM ECAP MANUAL

B1	N(0,2),R=2000

B2	N(0,1),R=6000

B3	N(0,1),R=1000

B4	N(1,3),R=350

B5	N(3,0),R=100

B6	N(2,3),R=11.1E3

B7	N(5,1),C=5E-6

B8	N(3,0),C=60E-6

B9	N(2,4),C=5E-6

B10	N(4,0),R=10E3

B11	N(5,0),R=10E6,I=1

T1	V(4,6),BETA=50

FREQUENCY=1000

PRINT, VOLTAGES

MODIFY

40	B5	R=300(2)700

EXECUTE

.R ECAP<CR>

WMU ELECTRONIC CIRCUIT ANALYSIS PROGRAM (VERSION 3)

INPUT? (TYPE HELP IF NEEDED)--EC91.DAT<CR>

READY

AC<CR>

T1    V(4,6),BETA=50

^

***ERROR # 20   CHARACTER IS NOT "B,N,R,G,E,I,L,C"

%%%%   1  ERROR(S) WERE DETECTED, EXECUTION IMPOSSIBLE

READY

T1	B(4,6),BETA=50<CR>

AC<CR>

OUTPUT? (TYPE HELP IF NEEDED)--TTY:<CR>

C1	SINGLE STAGE COMMON EMITTER AMPLIFIER

C2	PAGE 91 - IBM ECAP MANUAL

FREQ=  1000.0000

NODE OR            NODE

BRANCH           VOLTAGE

---------------------------

1   MAG    2.6243E+02

1   PHA      -12.8952

2   MAG    5.1069E+04

2   PHA     -174.4507

3   MAG    8.3120E+01

3   PHA      -82.9012

4   MAG    5.1069E+04

4   PHA     -174.2683

5   MAG    2.7131E+02

5   PHA      -19.4620

R    =  0.30000000E+03

FREQ=  1000.0000

NODE OR            NODE

BRANCH           VOLTAGE

---------------------------

1   MAG    2.6149E+02

1   PHA      -12.9952

2   MAG    5.1156E+04

2   PHA     -174.4377

3   MAG    8.3288E+01

3   PHA      -83.9010

4   MAG    5.1156E+04

4   PHA     -174.2553

5   MAG    2.7044E+02

5   PHA      -19.5806

R    =  0.50000000E+03

FREQ=  1000.0000

NODE OR            NODE

BRANCH           VOLTAGE

---------------------------

1   MAG    2.6131E+02

1   PHA      -13.0145

2   MAG    5.1174E+04

2   PHA     -174.4355

3   MAG    8.3318E+01

3   PHA      -84.1015

4   MAG    5.1174E+04

4   PHA     -174.2531

5   MAG    2.7026E+02

5   PHA      -19.6037

R    =  0.70000000E+03

FREQ=  1000.0000

NODE OR            NODE

BRANCH           VOLTAGE

---------------------------

1   MAG    2.6123E+02

1   PHA      -13.0226

2   MAG    5.1182E+04

2   PHA     -174.4346

3   MAG    8.3331E+01

3   PHA      -84.1874

4   MAG    5.1181E+04

4   PHA     -174.2522

5   MAG    2.7018E+02

5   PHA      -19.6135

READY

EXIT<CR>

END OF EXECUTION

CPU TIME: 1.12	ELAPSED TIME: 8.05

EXIT

6.3 BATCH EXAMPLE

IN THE FOLLWOING BATCH JOB SETUP, EACH LINE REPRESENTS ONE CARD, EACH

CARD STARTING IN COLUMN 1.  DO NOT INCLUDE THE COMMENTS ON THE RIGHT.   SEE

COMPUTER CENTS USER'S GUIDE #7.

$JOB[#$##,###]				;###,### REPRESENTS THE USER'S PROJECT-

PROGRAMMER NUMBER.

$PASSWORD ####				;#### REPRESENT THE USER'S PASSWORD

.R ECAP					;  START ECAP.

[COMMANDS TO ECAP]

(EOF)					; END OF FILE AVAILABLE FROM COMPUTER

CENTER.