The WHO command does the following:  It allows any user, including

non-priviledged ones, to determine the following:

What tasks are running (not stopped) on CO:

Which TT's (or VT's (=virtual terminals)) are logged in,

What their login account is,

What active tasks have that TT as their TI,

Flags for many of those tasks to show their current state, in

the following priority:

/ = checkpointed

? = waiting for terminal input

* = I/O outstanding

# = waiting for an event flag

blank = no special flagged state (usually compute-bound)

It does not lock out MCR activity, preferring instead to occasionally

get the task state slightly stale.  A reminder of the meaning of the flag

characters may be gotten w/:   WHO /HElp

Questions or bugs to:

Jim Neeland

Hughes Research Labs

3011 Malibu Canyon Rd.

Malibu, CA  90265

(213) 456-6411 ext. 333

Installation Notes

==================

The WHO.OBJ file is probably usable as-is.  The WHO.TKB file

may have to be edited to change the UIC of RSX11M.STB.