The DSC tape format.

___________________

The blocks contain each a header of 8 words layed out as below. DSC

writes an initial type 40 block which contains the bootstrap and the

image name., followed by 1 or more blocks containing a description of

the index file, the information is the file sequence numbers which are

required to be generated into the new indexfile.

Files are dumped as a sequence of blocks,

a type 2 block of 2048 + 16 bytes.

This contains general information from the file header, and

should be sufficient to name the file if DSC (at least in RSX11M v3)

didnt fill the filename and UIC fields with a constant. This block

type is also used by RMSRST. The only word used by RSTI is the

count of blocks used. The last 8 bytes are PUKCAB?? where the ?? is a

block checksum.

a type 4 block of 512 + 16 bytes

This is a copy of the file header block from the indexfile.

DSC uses this to reconstruct a file on the output disk. It is not

present on the RMSBCK tapes.

several type 1 blocks lengths 2048 + 16 bytes

except the last which may be 512,1024,1536 or 2048 +16bytes

The file data blocks. The block # field in the 8 word header increments

by 4 between them.

Intermediate type 4 records indicate the start of new file headers

in the inputfile.

HEADER

------

word 1	# of data bytes in block, not counting header. 512,1024,1536,2048

word 2	block type code. 1,2,4,40

word 3	block number in file being dumped

word 4	probably 2nd word of block #. always zero

word 5	File ID of file being dumped

word 6	File seq # of file being dumped

words 7,8 spare

The block # is zero for a type 2 or type 4 block.

The data follows immediately the 8 byte header