TPC---TAPE POSITION CONTROL TASK

THE TAPE POSITION CONTROL TASK (TPC) IS AVAILABLE TO ALLOW CONTROL OF

CERTAIN TAPE FUNCTIONS FROM THE CONSOLE. THE TAPES TO BE USED MUST

BE MOUNTED FOREIGN (MOU MT:/CHA=[FOR]   OR MOU MT1:/CHA=[FOR])

FOR TPC TO DO ANYTHING. COMMANDS ARE IN FREE FORM AND SHOULD INCLUDE

NUMBERS WHERE REQUIRED. NUMBERS MAY BE SEPARATED FROM COMMANDS BY

SPACES OR COMMAS, AND ONLY THE FIRST LETTER OF A COMMAND IS

SIGNIFICANT.

COMMAND		EXAMPLE		FUNCTION

REWIND #	REWIND 1	REWIND MT1: OR MT0: (R 0 REWINDS MT0:)

ENDFILE #	ENDF 0		WRITE END-FILE MARK ON MT0: OR MT1:

SKIP UNIT,#	SKP 0,10	SKIP "#" RECORDS (+ OR -) ON UNIT (UNIT=0 OR 1)

FILE UNIT,#	FIL 0,2		SKIP "#" FILES (+ OR -) ON UNIT (0 OR 1)