EXTENDING RSX-11M BY USE OF THE ILLEGAL INSTRUCTION TRAP

Daniel B. Curtis

Fermi National Accelerator Laboratory

Bativia, Il.

ABSTRACT

This paper describes a method of extending the  RSX-11M  executive  by

using  illegal  instructions to initiate the execution of kernal code.

This method does not modify the executive.  Code, which is loaded into

the  system  pool,  intercepts  the  illegal  instruction trap.  If it

recognizes the illegal instruction that caused the  trap,  it  does  a

directive  save  ($DIRSV),  and  the service invoked executes as if it

were a directive.

This method was developed to allow safe, moderately  efficient  access

to  a  shared memory data base by multiple tasks.  Since the data base

linkages are manipulated in an executive process, there is no need for

locking the data base.

INTRODUCTION

In many control or  real-time  situations,  it  is  desirable  to  add

functionality  to the computer's executive.  This is usually done only

when the benefits outweigh the costs of implementing and maintaining a

non-standard  executive.   This  paper  describes  a  simple method of

expanding the capabilities of the executive.

We use the illegal instruction trap to enter the executive and perform

a  pseudo-directive  operation  (an  extension).   Thus, the task that

wishes to invoke one of the executive extensions  places  the  correct

parameters  on  the  stack  and issues a specific illegal instruction.

The illegal instruction trap occurs and  is  intercepted  by  routines

which  are  loadable  and  reside in the executive dynamic pool space.

These routines determine if the trap should be  handled  as  a  normal

illegal instruction trap or as a request for executive extensions.  If

the instruction  that  caused  the  trap  is  an  executive  extension

request,  the  parameters  on the stack are examined for legality.  If

legal,  the  request  is  dispatched;   otherwise,  depending  on  the

severity  of  the  error,  the  request  is either treated as a normal

illegal instruction or the carry is set and returned to the task.

EXTENDING RSX-11M                                               PAGE 2

NO EXECUTIVE MODIFICATIONS

This method is not intended to facilitate  modification  of  executive

code.   It only allows the addition of directive-like extensions.  The

less these extensions interact  with  the  executive,  the  safer  the

extensions  will  be  and  the  more likely they will survive the next

release of the executive.

Examples of Executive Extensions

Some of the extensions that could be provided are:

1.  Large array  handling - Allows  access  to  attached  commons

without using virtual space to map to the common.

2.  Dynamic Memory and Queue Operations - Allows  multiple  tasks

to  operate  on  shared  structures  by implementing critical

sections through invoking the executive.

For 2 above, other methods could be used.  Some of these are:

1.  Mapping to the PSW and  raising  the  processor  priority  to

disable interrupts.  This has some disadvantages but is fast.

2.  Having a separate task manage the operations.  This  is  very

slow but quite safe.

3.  Using a lock bit for the structures.  This  is  fast  but  is

vulnerable  to  task  failures  and  requires  the additional

overhead of blocking and unblocking task execution.

The illegal instruction method is much faster but otherwise similar to

the separate task method.

Very Large Array Handling- This extension gives a  task  access  to  a

shared  common  to  which  the  task is attached but not mapped.  This

access is not fast;  however, it can randomly access 32K or more words

of  common  without using virtual space.  In some cases<1> this method

could be more efficient than mapping to different parts of the array.

Managing Queues and Dynamic Memory- This extension serializes  dynamic

memory  and  queue  operations.   Therefore, it allows tasks to access

queues, lists, etc., in a safe manner  whether  the  access  is  in  a

shared common or in the task.

*******

<1>  Where the data access sequence in the common is random and  would

possibly require a remap to access each data point.

EXTENDING RSX-11M                                               PAGE 3

Microcode Emulation-  Many  of  the  extensions  could  be  viewed  as

specialized  PDP-11  instructions.  As such, they may be implemeted in

micro-code for the PDP-11 computers that have that option.  The use of

the  illegal  instruction  trap allows a degree of transparency across

different PDP-11 computers.

SUPPORT TASKS

The extensions need to be easy to correct  and  modify.   This  allows

simple creation and debugging of the extensions without requiring much

knowledge of the executive internals.  This support is provided by two

tasks  that  allow  the  extensions  to  be added and removed from the

system.

Install Extension Task

This task installs the extensions into the executive pool by inserting

a  PIC  shared  library task image.  The extensions must be built with

the restrictions that are mentioned below.

Legality checks are made on the library that is loaded;   the  library

is  inserted  into  the  executive  pool space;  if XDT is part of the

executive, it is slightly modified;  and the illegal instruction  trap

vector is pointed to the inserted library.

This library must be removed before  another  insertion  attempt  will

succeed.  This removal is performed by the Remove Extension Task.

Remove Extension Task

This task removes all traces of the inserted library.  It  resets  the

illegal  instruction  vector  to  its  original  form, removes the XDT

modification if necessary, and returns the pool space.

XDT Problems

The XDT modification mentioned above is required  because  XDT  resets

the  illegal  instruction  vector  each time it proceeds from a break.

Therefore,  the  modification  is  needed  if  XDT  is  invoked.   The

modification  points  XDT's  default illegal instruction vector to the

inserted library.   This  modification  is  removed  when  the  Remove

Extension  Task  is  executed.   The  modification does not affect the

operation of XDT<2>.

*******

<2>  All this means is that normal illegal  instructions  are  handled

essentially  the  same.   XDT is not informed of the occurence of

the special instruction that invokes the extension.

EXTENDING RSX-11M                                               PAGE 4

REQUIREMENTS FOR THE EXTENSIONS ADDED

The requirements for the extensions that are added to the system  pool

are:

1.  The routines must be written  in  position  independent  code

(PIC).

2.  The total amount of code added to the pool must be less  than

4K words.  This is an arbitrary limitation.

Another possible implementation is to  create  and  load  the

library  into  a region, and to provide code in the pool that

receives the trap and maps the region into the kernal.   This

would  be  implemented just like executive access to loadable

driver code.

3.  The routines are built by the task builder just  as  if  they

were part of a PIC sharable library.

HOW TO INTERFACE TO THE SYSTEM

The code executed upon an illegal instruction can  be  separated  into

two parts:

1.  code executed at interrupt level.

2.  code executed at fork level.

Each of these parts have normal and error exit procedures.

Interrupt Part- This part  is  entered  directly  from  the  interrupt

vector  processing.   First, the illegal instruction is checked to see

if it is recognized.  If not, execution passes to the  normal  illegal

instruction  code.   The  normal  illegal  instruction  operation then

completes.

If the instruction is  recognized,  a  JSR (R5),@#$DIRSV  is  executed

which  causes  a  system  fork  and  eventual  execution  of  the code

following the call.

Fork  Part-  If  an  error  is  detected  that  requires  an   illegal

instruction  error return, a JMP @#$ILINS+4 is executed.  This totally

bypasses the XDT check but  correctly  handles  most  operations.   An

argument  error  sets  the  saved processor status carry bit and exits

normally.

A normal exit simply returns parameters and executes a RETURN.

EXTENDING RSX-11M                                               PAGE 5

CURRENT STATUS

Currently, the Install and Remove Extension tasks  have  been  written

and  tested.   We  have implemented the large array access extensions.

They have been slightly tested on a working RSX-11M  V3.1  development

system without indication of any problems.

The dynamic memory and queueing extensions are under development.

Large Array Handling Calls

The following describes the calling sequence for the first version  of

the large array handling extension<3>.

The task must use  the  region  ID  as  a  parameter  in  the  calling

sequence.  If this ID is incorrect, the illegal instruction will cause

the task to receive an illegal instruction exception.

Usage (calling sequences)- The  instruction  that  starts  the  system

extension  has  the  octal  representation  210.  The arguments of the

instruction are on the stack.  There are the following types.

1.  TYPE 0 -- transfer word to user

2.  TYPE 1 -- transfer word from user

3.  TYPE 2 -- transfer block of words to user

4.  TYPE 3 -- transfer block of words from user

5.  TYPE 4 -- set a block to a user specified value

********

<3>  Later versions of this package will provide a method of passing a

pointer  to  the  argument  block  instead  of requiring that the

parameters themselves be passed on the stack.

EXTENDING RSX-11M                                               PAGE 6

Each of these types have the following information on the stack at the

time  of  the 210 instruction execution.  This information is required

for each type.  Each type may have other  information  placed  on  the

stack along with the required information.

10(SP) INDEX HIGH PART

6(SP) INDEX LOW PART

4(SP) REGION ID WORD OBTAINED FROM

ATTACH REGION DIRECTIVE

2(SP) TYPE CODE

(SP) RAD50 REPRESENTATION OF "ILL"

Where the INDEX is a 24-bit byte offset into the region.

For type 0:

no more information needed

For type 1:

12(SP) VALUE TO BE STORED IN THE REGION

For type 2:

14(SP) NUMBER OF BYTES TO TRANSFER

12(SP) VIRTUAL ADDRESS OF DESTINATION

For type 3:

14(SP) NUMBER OF BYTES TO TRANSFER

12(SP) VIRTUAL ADDRESS OF SOURCE

For type 4:

14(SP) NUMBER OF BYTES TO SET

12(SP) VALUE TO SET INTO AREA

All cases except TYPE 0 finish the operation with the  stack  cleared.

TYPE 0 returns on the stack the value obtained from the area.

Carry clear implies a successfully  completed  operation.   Carry  set

implies  an  error and the stack is not cleared.  The instruction will

produce an illegal instruction AST for the following:

1.  The  instruction  used  to  cause  the  trap  is  an  illegal

instruction other than "210".

2.  The  first  word  of  the  user's  stack  is  not  the  RAD50

representation of "ILL".

EXTENDING RSX-11M                                               PAGE 7

3.  The TYPE argument is out of range.

4.  The region specified does not exist or is not attached to the

task.

5.  The high part of the index is non-zero<4>.

The instruction will return carry set for the following:

1.  If the index is odd.

2.  If the index points outside the region.

3.  If the block transfer/set would attempt  access  outside  the

region.

4.  If the block transfer/set has a zero byte count.

ACKNOWLEDGMENT

Work performed under the auspices of the U.S. Department of Energy  by

the Fermi National Accelerator Laboratory.

REFERENCES

Software Support Group document numbers

34, 67, and 70.

Daniel B. Curtis MS 222

FERMILAB POB 500

Batavia, IL.  60510

*********

<4>  The current version only allows a 16-bit index.   Later  versions

will support the 24-bit index.