1.0     INTRODUCTION . . . . . . . . . . . . . . . . . . . . 3

1.1       Instructors  . . . . . . . . . . . . . . . . . . . 3

1.2       Seminar Goal . . . . . . . . . . . . . . . . . . . 4

1.3       Seminar Outline  . . . . . . . . . . . . . . . . . 4

1.4       Seminar Materials  . . . . . . . . . . . . . . . . 5

2.0     FOUNDATIONS  . . . . . . . . . . . . . . . . . . . . 6

2.1       What Is A Crash? . . . . . . . . . . . . . . . . . 6

2.2       Why Analyze Crashes? . . . . . . . . . . . . . . . 6

2.3       How Do Systems Crash?  . . . . . . . . . . . . . . 7

2.4       Key Points . . . . . . . . . . . . . . . . . . . . 7

3.0     CRASH MANAGEMENT . . . . . . . . . . . . . . . . . . 8

3.1       Preparation  . . . . . . . . . . . . . . . . . . . 8

3.1.1       Materials  . . . . . . . . . . . . . . . . . . . 8

3.1.2       Actions  . . . . . . . . . . . . . . . . . . .  10

3.2       How To Crash Your System . . . . . . . . . . . .  12

3.3       Record Crash To Media  . . . . . . . . . . . . .  13

3.4       Post-Crash Procedure . . . . . . . . . . . . . .  14

3.5       Practice And Train Personnel . . . . . . . . . .  15

4.0     BASIC CRASH ANALYSIS . . . . . . . . . . . . . . .  16

4.1       CDA Usage  . . . . . . . . . . . . . . . . . . .  16

4.1.1       Command Format . . . . . . . . . . . . . . . .  16

4.1.2       Switches . . . . . . . . . . . . . . . . . . .  17

4.2       Find The Obvious And The Unexpected  . . . . . .  19

4.3       System Stack Analysis  . . . . . . . . . . . . .  21

4.3.1       The Kernel Stack . . . . . . . . . . . . . . .  21

4.3.2       The Crash Module ($CRASH)  . . . . . . . . . .  22

4.3.3       What Happens When A Crash Occurs?  . . . . . .  23

4.3.3.1       Start At Location 40 . . . . . . . . . . . .  23

4.3.3.2       Stack Limit Violation  . . . . . . . . . . .  24

4.3.3.3       IOT - The CRASH Macro  . . . . . . . . . . .  26

4.3.3.4       SST's  . . . . . . . . . . . . . . . . . . .  28

4.3.3.4.1       Trap Through Location 4  . . . . . . . . .  30

4.3.3.4.2       Illegal Instruction Trap . . . . . . . . .  32

4.3.3.4.3       Breakpoint . . . . . . . . . . . . . . . .  33

4.3.3.4.4       Segment Fault  . . . . . . . . . . . . . .  34

4.3.3.5       TRAP's And EMT's . . . . . . . . . . . . . .  35

4.3.3.5.1       EMT  . . . . . . . . . . . . . . . . . . .  36

4.3.3.5.2       TRAP . . . . . . . . . . . . . . . . . . .  37

4.3.4       Tracing The Original Trap  . . . . . . . . . .  38

5.0     CRASH DETECTIVE  . . . . . . . . . . . . . . . . .  39

5.1       The CDA File . . . . . . . . . . . . . . . . . .  40

5.2       Tools  . . . . . . . . . . . . . . . . . . . . .  41

5.2.1       ZAP  . . . . . . . . . . . . . . . . . . . . .  41

5.2.2       BINCMP . . . . . . . . . . . . . . . . . . . .  43

5.2.3       Specials . . . . . . . . . . . . . . . . . . .  44

5.3       Tricks . . . . . . . . . . . . . . . . . . . . .  45

5.3.1       Branch-on-self . . . . . . . . . . . . . . . .  45

5.3.2       On-line Patches  . . . . . . . . . . . . . . .  46

5.3.3       Logic Analyzer . . . . . . . . . . . . . . . .  47

5.3.4       T-Bit Traps  . . . . . . . . . . . . . . . . .  48

5.3.5       Mousetraps . . . . . . . . . . . . . . . . . .  49

5.4       Mathematics Of RSX-11M . . . . . . . . . . . . .  51

Page 2

APPENDIX A      CONCATENATED EXECUTIVE ASSEMBLY

A.1     STEP0.CMD  . . . . . . . . . . . . . . . . . . . . A-3

A.2     STEP1.TEC  . . . . . . . . . . . . . . . . . . . . A-4

A.3     STEP2.TEC  . . . . . . . . . . . . . . . . . . . . A-5

A.4     MASTER.MAC . . . . . . . . . . . . . . . . . . . . A-6

A.5     C5TA.MAC . . . . . . . . . . . . . . . . . . . . . A-7

APPENDIX B      CRASH MANAGMENT COMMAND FILE

APPENDIX C      CDA AND RELATED PATCHES

C.1     LOWCR.COR  . . . . . . . . . . . . . . . . . . . . C-2

C.2     CRASH.COR  . . . . . . . . . . . . . . . . . . . . C-3

C.3     CDA.CMD  . . . . . . . . . . . . . . . . . . . . . C-8

C.4     CDACLQ.PAT . . . . . . . . . . . . . . . . . . . . C-9

C.5     LUDMP.COR  . . . . . . . . . . . . . . . . . . .  C-10

C.6     TDUMP.PAT  . . . . . . . . . . . . . . . . . . .  C-12

APPENDIX D      RSX-11M DATA STRUCTURES

APPENDIX D      DATA STRUCTURES

D.1     SYSCM  . . . . . . . . . . . . . . . . . . . . . . D-2

D.2     TCB  . . . . . . . . . . . . . . . . . . . . . . . D-8

D.3     OCB  . . . . . . . . . . . . . . . . . . . . . .  D-12

D.4     AST CONTROL BLOCK  . . . . . . . . . . . . . . .  D-13

D.5     SEND/RECEIVE PACKET  . . . . . . . . . . . . . .  D-15

D.6     RECEIVE-BY-REFERENCE PACKET  . . . . . . . . . .  D-16

D.7     GROUP GLOBAL EVENT FLAGS . . . . . . . . . . . .  D-17

D.8     TASK HEADER  . . . . . . . . . . . . . . . . . .  D-18

D.9     PCB  . . . . . . . . . . . . . . . . . . . . . .  D-22

D.10    ATTACHMENT DESCRIPTOR  . . . . . . . . . . . . .  D-25

D.11    CLOCK QUEUE  . . . . . . . . . . . . . . . . . .  D-26

D.12    I/O PACKET . . . . . . . . . . . . . . . . . . .  D-29

D.13    LCB  . . . . . . . . . . . . . . . . . . . . . .  D-31

D.14    ITB  . . . . . . . . . . . . . . . . . . . . . .  D-32

D.15    DCB  . . . . . . . . . . . . . . . . . . . . . .  D-35

D.16    UCB  . . . . . . . . . . . . . . . . . . . . . .  D-37

D.17    SCB  . . . . . . . . . . . . . . . . . . . . . .  D-45

D.18    MAPPING ASSIGNMENT BLOCK . . . . . . . . . . . .  D-47

D.19    MOUNTED VOLUME LIST  . . . . . . . . . . . . . .  D-48

D.20    F11ACP VCB . . . . . . . . . . . . . . . . . . .  D-49

D.21    F11ACP WINDOW  . . . . . . . . . . . . . . . . .  D-51

D.22    LOCK BLOCK LIST  . . . . . . . . . . . . . . . .  D-53

D.23    FCB  . . . . . . . . . . . . . . . . . . . . . .  D-54

D.24    MTAACP VCB . . . . . . . . . . . . . . . . . . .  D-56

D.25    EMB  . . . . . . . . . . . . . . . . . . . . . .  D-58

D.26    INDEX FILE FORMAT  . . . . . . . . . . . . . . .  D-61

D.27    HOME BLOCK . . . . . . . . . . . . . . . . . . .  D-62

D.28    FILE HEADER  . . . . . . . . . . . . . . . . . .  D-64

Page 3

D.29    DIRECTORY ENTRY  . . . . . . . . . . . . . . . .  D-67

D.30    FDB  . . . . . . . . . . . . . . . . . . . . . .  D-68

D.31    FNB  . . . . . . . . . . . . . . . . . . . . . .  D-70

D.32    DSD  . . . . . . . . . . . . . . . . . . . . . .  D-71

D.33    $$FSR1 . . . . . . . . . . . . . . . . . . . . .  D-72

D.34    $$FSR2 . . . . . . . . . . . . . . . . . . . . .  D-73

APPENDIX E      EXAMPLE CRASHES

E.1     CRAYZO . . . . . . . . . . . . . . . . . . . . . . E-1

E.2     SST CRASHES - CRAT04, CRAILI, CRABPT, CRASEG . . . E-1

E.3     CRATRP AND CRAEMT  . . . . . . . . . . . . . . . . E-1

E.4     CRAOSP . . . . . . . . . . . . . . . . . . . . . . E-1

E.5     ODD VECTOR CRASHES . . . . . . . . . . . . . . . . E-2

E.5.1     CRAOD4 . . . . . . . . . . . . . . . . . . . . . E-2

E.5.2     CRAIOS And CRAIOU  . . . . . . . . . . . . . . . E-2

E.5.3     CRAEVO . . . . . . . . . . . . . . . . . . . . . E-3

CRASH DUMP ANALYSIS

Seminar Workbook

May 17, 1981

Kenneth Johnson

Monsanto

Ralph Stamerjohn

Monsanto

1

Crash Dump Analysis Workbook

This page left intentionally blank.

2

Crash Dump Analysis Workbook

INTRODUCTION

1.0  INTRODUCTION

Welcome to the RSX-11M Crash Dump Analysis Seminar.  This  is  the  seminar

workbook,  which  follows  the  graphics used in our presentation.  The workbook

records the important points we will make, allowing participants  to  spend  the

day  thinking rather than writing.  Space has been provided for additional notes

to be made in the workbook itself.  Please feel free to  ask  questions  at  any

time during the day.

1.1  Instructors

Kenneth Johnson

Programmer/Analyst I

Monsanto Agricultural Products Co.

Mail Stop T1C

800 N. Lindbergh Blvd.

St. Louis, MO   63166

Ken is the system manager of a major laboratory data system which  includes

two  PDP 11/70's and over 20 PDP 11/03's.  He has analyzed over 50 crashes, most

related to the development, startup, and operation of that system.  He has  been

working in RSX-11M about 2 years.

Ralph Stamerjohn

Senior System Programming Specialist

Monsanto Agricultural Products Co.

Mail Stop T1A

800 N. Lindbergh Blvd.

St. Louis, MO   63166

Ralph has responsibilities for providing  system  consulting  to  the  MAPC

network.   A large part of this is trouble shooting system problems and crashes.

He has worked on RSX-11M for 6 years (since version  1.0),  and  has  long  lost

track of the number of crashes he has examined (at least 400).

3

Crash Dump Analysis Workbook

INTRODUCTION

1.2  Seminar Goal

"After the course, you will be  able  to  properly  handle  RSX-11M  system

crashes and understand the basic principles of crash analysis."

Specifically, participants will learn how systems crash, how  to  obtain  a

crash  dump,  the  material  needed  for  crash  analysis,  what is proper crash

management, how to use CDA, adn the basic steps of crash analysis.  A number  of

"tricks of the trade" will also be presented.

1.3  Seminar Outline

1.  Introduction

2.  Foundations

3.  Crash Management

4.  Basic Crash Analysis

** LUNCH **

5.  Crash Detective

6.  Case Studies

7.  Class Problems

4

Crash Dump Analysis Workbook

INTRODUCTION

1.4  Seminar Materials

The seminar materials are all contained in this binder.  The first part  of

the  binder is the workbook.  Reference materials, labeled as appendices, follow

the workbook itself.  Also, all material for this seminar has been submitted  to

the RSX/IAS SIG Tapes in UIC [346,101].

A.  Concatenated Executive Assembly

B.  Sample Crash Management Command File

C.  Patches to CDA and Crash Module

o  Patch to LOWCR for JMP $CRALT (Digital)

o  Patch to CRASH to correct user stack pointer (Digital)

o  Patch to CRASH for RK06, TS11, and TU58 (Digital)

o  Patch to CRASH to save location 4 (Monsanto)

o  Patch to CDA for clock time problem (Monsanto)

o  Patch to CDA on non-Files-11 headers (Digital)

o  Patch to CDA on pre-AST status (Monsanto)

D.  Toggle-In Crash Routines

E.  RSX-11M Data Structures

5

Crash Dump Analysis Workbook

FOUNDATIONS

2.0  FOUNDATIONS

2.1  What Is A Crash?

"A crash is defined  as  when  an  RSX-11M  system  becomes  unuseable  for

unexpected reasons and must be rebooted."

Examples of a crash include system halts, exhausted pool, system  in  crash

routine, break to XDT, system looping, F11ACP abort, or application fault.

2.2  Why Analyze Crashes?

o  Maintain or improve system reliability (frequency of failures).

o  Maintain or improve system availability (% uptime).

o  Diagnose hardware problems.

o  Improve system performance.

When properly used, crash analysis is a powerful development tool resulting

in reliable, available, and high-performance systems.

6

Crash Dump Analysis Workbook

FOUNDATIONS

2.3  How Do Systems Crash?

A system crashes when it reaches a state that the executive cannot reliably

handle.   The  final  state (physical error) is triggered by some event (logical

error) which occured in the past.  If the same series of  events  are  repeated,

the system will crash again.

A crash dump is a frozen copy of the system at the point  of  the  physical

error.   This  is  rarely the real cause of the crash.  The whole point of crash

analysis is to track backwards in time from the physical error  to  the  initial

logical error and correct that problem.

2.4  Key Points

o  Cost

Crash analysis is  costly,  particularly  in  terms  of  time.   It  is

difficult  to  judge  how long it will take to resolve a crash.  If the

crash is merely annoying, ignore it.  However  keep  in  mind  that  if

unsolved, the problem may occur again.

o  Skill

Anyone can analyze a crash.  However,  unless  you  work  with  crashes

regularly,  you  will  find  yourself  starting from scratch each time.

Don't be discouraged.

o  Serial Operations

One thing happens at time in computer;  things just happen  very  fast.

Keep  in mind that a sequential series of events occurred to cause your

crash.  Your job is to trace these events back to the real error.

o  Reproducibility

If you can reliably reproduce a problem, it is solved.

7

Crash Dump Analysis Workbook

CRASH MANAGEMENT

3.0  CRASH MANAGEMENT

"The purpose of crash management is to guarantee that you  have  everything

you need to determine and resolve the cause of the crash."

3.1  Preparation

One of the most important steps in  preventing  system  crashes  is  to  be

prepared  for  a crash to occur.  When your system crashes, you must be ready to

grab ALL of the pieces and put them in a safe place until you can eliminate  the

problem that broke it.

3.1.1  Materials

o  Executive symbol table file (RSX11M.STB)

The symbol table file is required by  the  CDA  program  to  find  data

strutures in the crash image.

o  Executive map and cross-reference

The executive map is used to translate a PC value into  a  location  in

the  listings.   The  cross-reference  is  useful  to  find out where a

routine is called or a variable modified.  Always cref the map!

o  Executive listings

Used to follow what happened prior to the crash.  A useful technique is

to  prepare a concatenated listing - see appendix A.  The STB file, the

map, and the listings are all outputs from SYSGEN and should be saved.

8

Crash Dump Analysis Workbook

CRASH MANAGEMENT

o  Other maps and listings

It is good to have maps and listings of all privileged tasks which  are

mapped to the executive and maps and listings of drivers.  When a crash

occurs, either the executive or one  of  these  processes  is  probably

executing.

o  Processor handbook and programming card

The card is useful for assembly and disassembly of  MACRO-11  by  hand.

The  processor handbook is necessary to understand what the hardware is

doing, particulary with  reference  to  traps,  error  conditions,  and

memory management.

o  RSX-11M manuals

Of particular interest is the "Guide to Writing an I/O Driver" and  the

CDA  manual.   The  CDA  manual  lists  macros  which  define  RSX data

structures.

o  RSX-11M Logic Manual

While this document is based on 3.1, it is still useful for the written

description  of  executive  modules, macro expansions for all executive

macros, and  nice  diagrams  of  the  linkage  of  the  executive  data

structures.

9

Crash Dump Analysis Workbook

CRASH MANAGEMENT

3.1.2  Actions

o  Sysgen crash support into system

The  following  questions  in  SYSGEN  relate  to  crash  support.   We

recommend  selecting  the  console  terminal  for  stack  dump  support

(question 45), CDA support for dump analysis (question 48), the console

terminal  for  crash  notification  (question  49), and the appropriate

device and CSR for the memory dump (questions 50 and 51).

* 45. System crash register and stack dump CSR [O R:0-177700 D:177564]:

* 48. Dump analysis:  A- PANIC  B- Crash (CDA) [S]:

* 49. Output notification device CSR [O R:160000-177700 D:177564]:

* 50. Memory dump device (ddu:) [S R:3-4]:

* 51. CSR address of dump device [O R:160000-177700 D:default]:

o  Set up permanent storage for crashes

You must be prepared to keep crashes until they are resolved or for the

life  of  the system;  this depends on how reliable the system must be,

of course.  You should have a designated location to store crashes.   A

UFD should be reserved for current crashes.  Old unresolved crashes may

be put on tape, but  should  be  kept  if  there  is  a  likelihood  of

recurrence.

10

Crash Dump Analysis Workbook

CRASH MANAGEMENT

o  Define naming convention for crashes

The crashes must be named so that they can be identified.  We recommend

a  naming  convention  based  on system name, date, and crash that day.

The  sample  crash  management  command  file  assigns  the   filenames

"namemmdda".   "name"  is  a four letter name for your system.  "mm" is

the current month;  "dd" is the  date;   both  numerical.   "a"  is  an

alphabetic sequence number.  This only allows 26 crashes per day.  When

you hit 27, go drinking.  Note that the use of the  command  file  also

guarantees that the .CDA file winds up in the correct UFD.

o  Define documentation procedure

A procedure for  documenting  the  crash  ensures  that  the  transient

information  associated with the crash is not lost.  A practice that we

like is to write the information on the initial  listing.   This  keeps

everything  in  one  package.   The  information  that  one  wants is a

description of what was happening to the system just before the  crash.

Emphasis  should  be  given  to  anything new or unusual - both acts of

ommision and commision.

11

Crash Dump Analysis Workbook

CRASH MANAGEMENT

3.2  How To Crash Your System

o  Already crashed

The simplest case is that the crash message has printed on the  console

and you only need to mount a scratch medium and proceed.

o  XDT>X

If the executive has broken to XDT, typing an "X" will cause  the  exec

to jump to the crash routine and print the crash message.  Now you have

the "already crashed case".

o  Running - odd clock vector

If you want to crash the system when it  is  still  running,  OPEn  the

clock vector (location 100 (8)) and deposit an odd number.  Your system

will crash at the next clock interrupt.

o  Halted - start at location 40

If the system is halted at an unknown location or if it  was  hung  and

you  halted  it,  then  you can get a dump by starting the processor at

location 40 (8).  This location contains the instruction "JMP $CRASH".

12

Crash Dump Analysis Workbook

CRASH MANAGEMENT

3.3  Record Crash To Media

o  Protect your disks

If you MUST use a disk as your dump media, be SURE that you  remove  or

write-protect  your  regular  disk  first.   $CRASH writes out over the

first n blocks of the device, which is a sure-fire method of destroying

the boot block and home block of a FILES-11 disk.

o  Preallocated media

It is a good idea to have a reserved tape (or disk) for taking crashes.

I  have a crash dump tape for each of my 11/70's.  You don't want to be

in the situation where you lose a crash dump just because  you  haven't

got a tape.

o  Toggle-in dump routines

In some cases, the $CRASH routine may be partly or totally destroyed by

whatever  crashed  your  system.   If only part of it is gone, get your

executive listings and toggle  it  back  in.   Alternatively,  see  the

handout on toggle-in dump routines.  These may also be used on a system

which does not have crash support but which you  would  like  to  crash

dump.

13

Crash Dump Analysis Workbook

CRASH MANAGEMENT

3.4  Post-Crash Procedure

After the crash, it is time to put the planning you did into action.

o  Read in crash to .CDA file and name

This is easy since you have a command file which  does  everything  for

you;  mount the tape and type @CRASHREAD.

o  Get initial listing (/ACT/HDR/PCB)

We recommend these switches for the intial listing;  the  reasons  will

be  discussed  in  the  Basic Crash Analysis section.  The command file

does this for you.

o  Record system and user activity

After you get the listing  you  write  down  what  happened.   THIS  IS

ESSENTIAL.   The  crash  dump will tell you only what the system looked

like at the time of the crash.  It does not provide any history.   This

you must get by recording what was going on.  Do not rely on the memory

of yourself and others.  It may be days before you look at the crash.

o  File for future reference.

At this point you can put the crash aside with the confidence that  the

steps  you  have  followed guarantee that you have the information that

you need.

14

Crash Dump Analysis Workbook

CRASH MANAGEMENT

3.5  Practice And Train Personnel

"We maintain that anyone with the knowledge and authority to boot a  system

should know how to get a crash dump."

It is required that you and your staff practice  crashing  the  system  and

getting  a  dump.  This ensures that you will get it right under the pressure of

the real thing.

15

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.0  BASIC CRASH ANALYSIS

4.1  CDA Usage

4.1.1  Command Format

list(/-SP),binary_file=symbol_table/STB,crash_input/sw1...

Note that CDA is smart enough to use the binary  file  for  analysis  which

considerably reduces analysis time.

16

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.1.2  Switches

o  /ALL

Logical or of all analysis switches

o  /ACT (/ATL) *

Active Task List       (TCB's)

o  /BL:n

Identify starting block of crash input device

o  /CLQ

Clock Queue

o  /DEV (/DCB, /SCB, /UCB all same as /DEV)

Dumps data structures for all devices on the system

o  /DUMP:a:b (/DMP:a:b)

Octal, Rad50, and ascii dump of memory.  Works for 22 bit addresses.

o  /EXIT:n

CDA exits after detecting n analysis errors

o  /HDR *

Task Headers

o  /LIMIT:n

Limits output to n pages

o  /KMR

*    Recommended for initial listing

17

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

Force standard values for kernel mapping registers

o  /MEMSIZ:n

CDA saves nK words of memory in a binary file

o  /NET

No function.  If you have Phase III Decnet, try NDA.

o  /PCB (/PAR) *

PCB's, attachment descriptors, and memory map

o  /POOL

Octal, Rad50, and Ascii dump of  pool.   This  includes  indication  of

used/free/preallocated.

o  /-SP

Don't spool the output

o  /STB

Identifies symbol table file

o  /STD (/TCB, /TAL)

Dump of TCB's

o  /-SYS

Suppress system information

o  /TASK:task:a:b

Dump relative to beginning of task

18

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.2  Find The Obvious And The Unexpected

Start by looking through the initial listing.  Get a picture  of  what  was

happening  in  the  system when the crash occurred.  This overview will help you

fit pieces together later on.

1.  CDA Error Messages

CDA will report analysis errors such as bad links.  These  are  usually

symptoms of the problem.

2.  Task Status

o  Active tasks that shouldn't be active.

Examples might be TKTN or PMD, plus some specific to your system.

o  Tasks which should be active but aren't.

Examples are NETACP, F11ACP, MCR..., ...MCR, or your application.

o  Tasks in unusual states.

Tasks with ABO, HLT, or RDN set should be looked at carefully.  Why

didn't  this  task  get cleaned up?  F11ACP with an ABO status is a

definite problem indiciator.

o  Tasks with outstanding I/O.

Does a task have too many or too few  outstanding  I/O's?   An  I/O

count  of  255  decimal  is a bad sign - this is realy a -1.  Also,

many crashes, particularly  hardware  related,  come  from  I/O  in

progress.

19

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

3.  Pool Exhausted

Why is all of your pool gone?  Often this is apparent from the  initial

listing.   Look  for  such  things  as the CDA pool link error message,

receive data  queue  which  exceed  the  CDA  list  count,  receive  by

reference  queues,  or a PCB list which exceeds the list count.  If you

can't tell where pool went, the /POOL option will  dump  all  of  pool.

See  the handout on RSX pool usage.  Note that a complete pool analysis

is time consuming - more on this in Crash Detective.

4.  Original Crash Notes

The original  crash  notes  should  be  examined  for  any  mention  of

something  new  or different.  Try to correlate the written description

with the CDA output -- how does this information fit together.

20

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.3  System Stack Analysis

4.3.1  The Kernel Stack

The Kernel Stack is a condensed history of your computer's  activity  since

it  was  last  executing  in  user  state.   Control  is  transferred to the RSX

executive from a task when the processor executes a trap instruction or when  an

interrupt  occurs.   The new PSW from the interupt vector in low core determines

which stack pointer is used to push the old PC and PSW.  Thus the PC and PS from

the user task are pushed onto the kernel stack.

The variable $STKDP (stack depth), located in SYSCM, is used to keep  track

of  the  depth  of  nested interrupts or traps on the stack.  $STKDP is 1 when a

task is executing at user state.  $STKDP is zero if a single  trap  or  interupt

has  occurred.   The stack depth is negative if more than one trap and interrupt

have occurred.

$STKDP is decremented in two routines, $DIRSV (directive save)  and  $INTSV

(interrupt  save).  These routines are called to save the state of the processor

when a trap or interrupt occurs.  The stack depth is incremented in code  common

to $DIRXT (directive exit) and $INTXT (interrupt exit).  The stack depth is also

incremented in SSTSR (SST routines)  if  a  trap  has  occurred  during  certain

directive  processing.   This  protects  the  executive  from a bad address in a

directive parameter block.

There are four instructions which cause traps in PDP-11's:  EMT, BPT, TRAP,

and  IOT.   The  EMT is used by tasks to request executive services.  EMT 377 is

used for executive directives.  EMT 376 is used by privileged programs to switch

to system state.

If XDT is built into your system, the BPT instruction is used similarly  to

it's use in ODT in user tasks.

The TRAP instruction is used at system state to  return  directive  status.

The macro DRSTS is what you see in the executive listings.  TRAP is also used by

XDT to print characters on the console.

The IOT instruction is used by RSX11-M to transfer  control  to  the  CRASH

module.  The CRASH macro is just an IOT.  The IOT trap handler checks the system

state and jumps to $CRASH if the stack depth is less than one.

In addition to the trap instructions, there are 3  other  traps  which  are

also handled by the executive as SST's.  These are the illegal instruction trap,

the segment fault or memory protect violation, and the trap through  location  4

or odd address trap.  The unexpected occurence of any of these 7 traps while the

processor is in Kernel state will crash your system.

21

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.3.2  The Crash Module ($CRASH)

The CRASH module has three basic functions:  print out the  crash  message,

save  the volatile registers on the crash stack, and dump the contents of memory

to the crash device.  The first two instructions after $CRASH pop the PC and  PS

from  the  stack,  on  the  assumption  that  the  transfer of control to $CRASH

occurred after an IOT or other trap.  If the machine was halted and 40G was used

to restart it, then that assumption is false and results in incorrect values for

the PS, PC, and SP on the crash dump output.  The entry point  $CRALT  is  right

after  PC  and  PS  are  popped,  so  PC and PS show as zero, while SP comes out

correct.

22

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.3.3  What Happens When A Crash Occurs?

This section describes how your system goes down, and what the  CDA  output

looks like afterwards.

Specifically, the events leading up to the more common types of crashes are

discussed here.

The important aspects of the CDA output are  the  contents  of  the  kernel

stack,  the  value  of  the stack depth variable ($STKDP), the PC and the kernel

stack pointer.  The aim is to determine the point of the "physical"  crash,  the

PC  of  the  first  trap  which  led  up to the transfer of control to the crash

routine.  The "real" or "logical" cause of the crash may or may not be the same.

4.3.3.1  Start At Location 40

The contents of the executive stack are unknown, as are the user and kernel

stack  pointer,  and the PC.  If you have incorporated the patch for $CRALT into

your system, the registers on the CDA output accurately reflect the registers at

the time that the system was halted.

23

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.3.3.2  Stack Limit Violation

The first instruction in the $TRP04 routine is a check on the depth of  the

kernel  stack.   If the stack pointer is less than 400, a jump to $CRASH occurs.

Thus as soon as the first yellow zone violation occurs, the  system  is  crashed

without  trying  to push anything else on the stack.  Recall that for most 11's,

if the stack limit register contents  are  0,  then  a  trap  will  occur  after

instruction  completion  if  the  stack pointer goes below 400.  The instruction

will be aborted if the stack pointer goes below 340.

24

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

|-----------------------------------------------|

|       PC FROM TRAP THROUGH 4                  | [1],[3]

|-----------------------------------------------|

|       PS FROM TRAP THROUGH 4                  |

|-----------------------------------------------|

|       UNKNOWN CONTENTS OF EXECUTIVE STACK     | LOCATION 376, [2]

/       ......                                  /

$STKDP is less than or equal to 0.

[1] This is the "BEFORE CRASH" PC shown on the CDA listing.

[2] This is the "AFTER CRASH" kernel SP shown on the CDA listing.

[3] This is the PC following the "physical"  cause  of  the  crash;   i.e.,  the

instruction which caused the stack limit trap.

25

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.3.3.3  IOT - The CRASH Macro

There are at least five places in the executive or other  kernel  processes

where  the  system  is  deliberately crashed.  This occurs when an unrecoverable

error is detected.

o  DRDSP - illegal TRAP or EMT instruction at system state

o  SSTSR - SST at system state

o  DRQIO - device mask work inconsistency

o  CORAL - pool allocation error

o  NSPSBC - DECNET error

The crashes in DRDSP and SSTSR result in a special stack structure and  will  be

discussed separately.

The I/O function code is checked against  the  function  mask  in  the  DCB

(device  control  block)  during QIO processing.  If it is a legal function then

the function code is used to generate an offset  into  a  dispatch  table.   The

system  is  crashed  if the resulting offset is less than zero.  The stack depth

would be zero, and the crash PC would be in DRQIO.

The symbol R$$DER is used to turn on some optional debugging code in  CORAL

(pool  allocation routines).  This optional code makes consistency checks during

allocation and deallocation of executive pool.  The system is crashed if any  of

these  checks  fail.   The  stack depth would be less than one, and the crash PC

would be in CORAL.

The DECNET module NSPSBC, part of the  NT.NSP  process  in  RSX-11M  DECNET

Phase II, contains a CRASH macro commented "Temporary debugging aid." The system

will crash if certain unusual error conditions are detected.

26

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

|-----------------------------------------------|

|       PC FROM IOT (CRASH MACRO)               | [1]

|-----------------------------------------------|

|       PS FROM IOT                             |

|-----------------------------------------------|

|       UNKNOWN EXECUTIVE STACK                 | [2]

/       ......                                  /

$STKDP is less than or equal to 0.

[1] This is the "BEFORE CRASH" PC shown on the CDA listing.

[2] This is the "AFTER CRASH" kernel SP shown on the CDA listing.

27

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.3.3.4  SST's

There are four synchronous system traps (SST) which crash the  system  when

they occur at system state.  These are:

o  Trap through location 4 (other than stack violations)

o  Illegal instruction trap

o  Breakpoint or trace trap

o  Segment fault (memory protect violation)

The SST at system state is probably the most common kind of  system  crash.   Of

these  four,  the trap through 4 is most frequent.  The appearence of the system

stack is similiar, though not identical, for these four crashes.

28

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

|-----------------------------------------------|

|       PC FROM IOT (CRASH MACRO) IN SSTSR      | [1]

|-----------------------------------------------|

|       PS PRIOR TO IOT                         |

|-----------------------------------------------|

|       NO. OF BYTES (n) FOR SST                | [2]

|-----------------------------------------------|

|       SST CODE                                |

|-----------------------------------------------|

/       OPTIONAL PARAMETERS (n-4 bytes)         /

|-----------------------------------------------|

|       RETURN ADDRESS TO $DIRSV COROUTINE      |

|-----------------------------------------------|

|       R0 AT TIME OF SST                       |

|-----------------------------------------------|

|       R1 ...                                  |

|-----------------------------------------------|

|       R2 ...                                  |

|-----------------------------------------------|

|       R3 ...                                  |

|-----------------------------------------------|

|       R4 ...                                  |

|-----------------------------------------------|

|       R5 ...                                  |

|-----------------------------------------------|

|       PC FROM SST                             | [3]

|-----------------------------------------------|

|       PS AT TIME OF SST                       |

|-----------------------------------------------|

/       UNKNOWN EXECUTIVE STACK                 /

/       ......                                  /

$STKDP is less than or equal to -1.

[1] This is the "BEFORE CRASH" PC shown on the CDA listing.

[2] This is the "AFTER CRASH" kernel SP shown on the CDA listing.

[3] This is the PC following the "physical" cause of the crash.

29

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.3.3.4.1  Trap Through Location 4

The trap through location 4 at kernel state is  the  most  common  type  of

crash  that you will see.  These traps are ususally from odd address faults, but

also include nonexistant memory, bus timeouts, and stack limit  traps.   When  a

trap  through  4  occurs, the first action in the trap handler is to check for a

stack limit violation.  That case was  discussed  earlier.   In  the  non  stack

violation  cases, the odd address code (zero) and the byte count are pushed onto

the stack, and the exec branches to the common SST exit routine (SSTXT).

The next processing checks if the SST occurred in protected  areas  of  the

executive.    These   are   directive  dispatching,  QIO  processing,  and  PLAS

processing.  If the SST occurs in one of those areas, and at stack depth 0, then

the system will clean up.  Otherwise, the system is crashed via a CRASH macro.

At this point the stack looks like the picture shown.   The  key  point  to

recognizing  this  crash is that the kernel stack pointer points to the 4,0 byte

count and SST code.  Above that on the stack are the return  to  DIRSV  and  the

saved  registers  and  PSW.   So  now  we  know  where the trap occurred and the

contents of the registers at the time.

30

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

|-----------------------------------------------|

|       PC FROM IOT (CRASH MACRO) IN SSTSR      | [1]

|-----------------------------------------------|

|       PS PRIOR TO IOT                         |

|-----------------------------------------------|

|       000004                                  | [2]

|-----------------------------------------------|

|       000000                                  |

|-----------------------------------------------|

|       RETURN ADDRESS FOR $DIRSV COROUTINE     |

|-----------------------------------------------|

|       R0 AT TIME OF TRAP THRU 4               |

|-----------------------------------------------|

|       R1 ...                                  |

|-----------------------------------------------|

|       R2 ...                                  |

|-----------------------------------------------|

|       R3 ...                                  |

|-----------------------------------------------|

|       R4 ...                                  |

|-----------------------------------------------|

|       R5 ...                                  |

|-----------------------------------------------|

|       PC FROM TRAP THRU 4                     | [3]

|-----------------------------------------------|

|       PS PRIOR TO TRAP THRU 4                 |

|-----------------------------------------------|

/       UNKNOWN CONTENTS OF EXECUTIVE STACK     /

/       ......                                  /

$STKDP is less than or equal to -1.

[1] This is the "BEFORE CRASH" PC shown by CDA.

[2] The "AFTER CRASH" kernel SP shown by CDA points here.

[3] This is the location of the "physical" cause of the crash.

31

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.3.3.4.2  Illegal Instruction Trap

|-----------------------------------------------|

|       PC FROM IOT (CRASH MACRO) IN SSTSR      | [1]

|-----------------------------------------------|

|       PS PRIOR TO IOT                         |

|-----------------------------------------------|

|       000004                                  | [2]

|-----------------------------------------------|

|       000010                                  |

|-----------------------------------------------|

|       RETURN ADDRESS TO $DIRSV COROUTINE      |

|-----------------------------------------------|

|       R0 PRIOR TO ILLEGAL INS. TRAP           |

|-----------------------------------------------|

|       R1 ...                                  |

|-----------------------------------------------|

|       R2 ...                                  |

|-----------------------------------------------|

|       R3 ...                                  |

|-----------------------------------------------|

|       R4 ...                                  |

|-----------------------------------------------|

|       R5 ...                                  |

|-----------------------------------------------|

|       PC FROM ILLEGAL INS. TRAP               | [3]

|-----------------------------------------------|

|       PS PRIOR TO ILLEGAL INS. TRAP           |

|-----------------------------------------------|

/       UNKNOWN EXECUTIVE STACK                 /

/       ......                                  /

$STKDP is less than or equal to -1.

[1] This is the "BEFORE CRASH" PC shown on the CDA listing.

[2] This is the "AFTER CRASH" kernel SP shown on the CDA listing.

[3] This is the PC following the "physical" cause of the crash.

32

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.3.3.4.3  Breakpoint

|-----------------------------------------------|

|       PC FROM IOT (CRASH MACRO) IN SSTSR      | [1]

|-----------------------------------------------|

|       PS PRIOR TO IOT                         |

|-----------------------------------------------|

|       000004                                  | [2]

|-----------------------------------------------|

|       000004                                  |

|-----------------------------------------------|

|       RETURN ADDRESS TO $DIRSV COROUTINE      |

|-----------------------------------------------|

|       R0 AT TIME OF BPT                       |

|-----------------------------------------------|

|       R1 ...                                  |

|-----------------------------------------------|

|       R2 ...                                  |

|-----------------------------------------------|

|       R3 ...                                  |

|-----------------------------------------------|

|       R4 ...                                  |

|-----------------------------------------------|

|       R5 ...                                  |

|-----------------------------------------------|

|       PC FROM BPT                             | [3]

|-----------------------------------------------|

|       PS AT TIME OF BPT                       |

|-----------------------------------------------|

/       UNKNOWN EXECUTIVE STACK                 /

/       ......                                  /

$STKDP is less than or equal to -1.

[1] This is the "BEFORE CRASH" PC shown on the CDA listing.

[2] This is the "AFTER CRASH" kernel SP shown on the CDA listing.

[3] This is the PC following the "physical" cause of the crash.

33

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.3.3.4.4  Segment Fault

|-----------------------------------------------|

|       PC FROM IOT (CRASH MACRO) IN SSTSR      | [1]

|-----------------------------------------------|

|       PS PRIOR TO IOT                         |

|-----------------------------------------------|

|       000012                                  | [2]

|-----------------------------------------------|

|       000002                                  |

|-----------------------------------------------|

|       SAVED SR1                               |

|-----------------------------------------------|

|       SAVED SR2                               |

|-----------------------------------------------|

|       SAVED SR3                               |

|-----------------------------------------------|

|       RETURN ADDRESS TO $DIRSV COROUTINE      |

|-----------------------------------------------|

|       R0 AT TIME OF SEGMENT FAULT             |

|-----------------------------------------------|

|       R1 ...                                  |

|-----------------------------------------------|

|       R2 ...                                  |

|-----------------------------------------------|

|       R3 ...                                  |

|-----------------------------------------------|

|       R4 ...                                  |

|-----------------------------------------------|

|       R5 ...                                  |

|-----------------------------------------------|

|       PC FROM SEGMENT FAULT                   | [3]

|-----------------------------------------------|

|       PS AT TIME OF SEGMENT FAULT             |

|-----------------------------------------------|

/       UNKNOWN EXECUTIVE STACK                 /

/       ......                                  /

$STKDP is less than or equal to -1.

[1] This is the "BEFORE CRASH" PC shown on the CDA listing.

[2] This is the "AFTER CRASH" kernel SP shown on the CDA listing.

[3] This is the PC following the "physical" cause of the crash.

34

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.3.3.5  TRAP's And EMT's

The appearence of the stack after a spurious TRAP  or  EMT  instruction  in

kernel  state  is  very similar to the SST stack.  The major diferences are that

the PC of the IOT is in DRDSP instead of SSTSR, and that there is no SST code or

SST  parameters.   The occurence of a spurious TRAP or EMT is fairly rare.  Note

that the stack depth for this type of TRAP crash is less than or  equal  to  -2.

Recall that a TRAP at stack depth 0 is used to return directive status.

35

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.3.3.5.1  EMT

|-----------------------------------------------|

|       PC FROM IOT (CRASH MACRO) IN DRDSP      | [1]

|-----------------------------------------------|

|       PS PRIOR TO IOT                         |

|-----------------------------------------------|

|       RETURN ADDRESS TO $DIRSV COROUTINE      | [2]

|-----------------------------------------------|

|       R0 AT TIME OF EMT                       |

|-----------------------------------------------|

|       R1 ...                                  |

|-----------------------------------------------|

|       R2 ...                                  |

|-----------------------------------------------|

|       R3 ...                                  |

|-----------------------------------------------|

|       R4 ...                                  |

|-----------------------------------------------|

|       R5 ...                                  |

|-----------------------------------------------|

|       PC FROM EMT                             | [3]

|-----------------------------------------------|

|       PS AT TIME OF EMT                       |

|-----------------------------------------------|

/       UNKNOWN EXECUTIVE STACK                 /

/       ......                                  /

$STKDP is less than or equal to -1.

[1] This is the "BEFORE CRASH" PC shown on the CDA listing.

[2] This is the "AFTER CRASH" kernel SP shown on the CDA listing.

[3] This is the PC following the "physical" cause of the crash.

36

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.3.3.5.2  TRAP

|-----------------------------------------------|

|       PC FROM IOT (CRASH MACRO) IN DRDSP      | [1]

|-----------------------------------------------|

|       PS PRIOR TO IOT                         |

|-----------------------------------------------|

|       RETURN ADDRESS TO $DIRSV COROUTINE      | [2]

|-----------------------------------------------|

|       R0 PRIOR TO TRAP                        |

|-----------------------------------------------|

|       R1 ...                                  |

|-----------------------------------------------|

|       R2 ...                                  |

|-----------------------------------------------|

|       R3 ...                                  |

|-----------------------------------------------|

|       R4 ...                                  |

|-----------------------------------------------|

|       R5 ...                                  |

|-----------------------------------------------|

|       PC FROM TRAP                            | [3]

|-----------------------------------------------|

|       PS PRIOR TO TRAP                        |

|-----------------------------------------------|

/       UNKNOWN EXECUTIVE STACK                 /

/       ......                                  /

$STKDP is less than or equal to -2.

[1] This is the "BEFORE CRASH" PC shown on the CDA listing.

[2] This is the "AFTER CRASH" kernel SP shown on the CDA listing.

[3] This is the PC following the "physical" cause of the crash.

37

Crash Dump Analysis Workbook

BASIC CRASH ANALYSIS

4.3.4  Tracing The Original Trap

1.  Get mapping.

The mapping at the time of the crash is determined from the APR's saved

by  $CRASH.   Determine  both the Kernel and User mapping.  Look to see

which tasks and processes were mapped.  Remember that RSX-11M can  only

see 64 KW at a time, 32 KW kernel and 32 KW user.

2.  Get registers.

Determine the contents of the registers at the physical point of crash.

That  is  easy  for an SST crash, since $DIRSV saved them on the stack.

For an IOT crash,  stack  limit  violation  or  "40G"  crash,  use  the

registers from the CDA listing.

3.  Study the code.

From the mapping and the our examination of the kernel  stack  we  know

what was executing at the physical point of crash.  Get the appropriate

maps and listings and study the code.  What was  going  on?   Since  we

know  the  registers  it  should be possible to have a fair idea of the

answer to that question.  If we are lucky, then at this point  we  have

resolved  the  crash.  That is, it turns out that the physical point of

crash was also the real point of crash.

38

Crash Dump Analysis Workbook

CRASH DETECTIVE

5.0  CRASH DETECTIVE

In this section we will discuss what to do if the basic analysis  fails  to

find  the problem.  There is no specific cookbook to use at this point.  Rather,

you dig into your bag of tricks and tools and use whatever  is  appropriate  for

the crash.

39

Crash Dump Analysis Workbook

CRASH DETECTIVE

5.1  The CDA File

Your basic source of information as a crash  detective  is  the  CDA  file.

There  is much more information in the file than reported by CDA.  It is crucial

you understand the format of this file so you can understand how to look at it.

A CDA file is an image of the system at the point a  failure  occurs.   The

following  diagrams  compare  the CDA file to the virgin system and saved system

images.

CDA                    Virgin                   Saved

|-----------|           |-----------|           |-----------|

Block 1 | Address 0 |           | Label  1  |           | Label  1  |

|--       --|           |--       --|           |--       --|

Block 2 |   1000    |           | Label  2  |           | Label  2  |

|--       --|           |--       --|           |--       --|

Block 3 |   2000    |           | Address 0 |           | Bootstrap |

|--       --|           |--       --|           |--       --|

Block 4 |   3000    |           |   1000    |           |   1000    |

A few numbers to remember are:

1 Block  = 1000(8) bytes

4 Blocks = 1KW

200(8) Blocks = 32KW

Also remember, files start with block 1.  Therefore, in a CDA file, address 0 is

[block 1, offset 0] and address 3114 is [block 4, offset 114].  The formulas for

caculating [block, offset] for an address are:

block   = (address/1000) + 1    (1000 is octal)

offset  = MOD (address/1000)    (1000 is octal)

40

Crash Dump Analysis Workbook

CRASH DETECTIVE

5.2  Tools

Besides CDA, there are other tools you can use on crashes.  In this section

we  will  discuss  two  specific programs, ZAP and BINCMP, and how you can write

simple Fortran programs to look at crashes.

5.2.1  ZAP

ZAP is a very powerful tool when looking  at  crashes.   It  will  let  you

examine  any  location  in  the  crash  dump.   In particular, ZAP is useful for

examining data structures not output by CDA, executive and task code related  to

the  crash,  and  general  poking  around inside the crash dump file.  Points to

remember about ZAP include:

o  ZAP commands are similar to ODT.

o  ZAP is documented in Chapter 19 of the Utilities Manual.

o  ZAP is invoked as follows:

>RUN $ZAP

ZAP>file specification[/SW]

or

>ZAP

ZAP>file specification[/sw]

/AB     Absolute image

/LI     List task mapping

/RO     Read-only access

o  Note, >ZAP file specification is not allowed.

o  When using ZAP on a CDA file, use /AB/RO switches.

41

Crash Dump Analysis Workbook

CRASH DETECTIVE

ZAP can only access 32KW from a given base address.  When looking at a  CDA

file,  it  is  useful  to  set  relocation  registers.  For example, 1:0;1R sets

relocation register 1 to the start of the executive  (address  0).   Assume  you

wanted  to  examine  location  123404  in F11ACP.  According to the CDA listing,

F11ACP is located at physical  address  345200.   The  following  commands  will

examine this address:

_346:200-120000;2R

_2,123404/

or

_346:200;2R

_2,3404/

42

Crash Dump Analysis Workbook

CRASH DETECTIVE

5.2.2  BINCMP

BINCMP is a program written at Monsanto to perform a  binary,  word-by-word

compare  of  two  files.  It was especially written to compare a CDA file to the

virgin or saved system.  This is useful for checking to see  if  code  has  been

modified.

BINCMP is written in F4P and is available on the Spring, 1979 DECUS tape in

[346,100].   It will also be in the crash submission on this symposium tape set.

The following example shows how to use BINCMP to compare a CDA file against  the

virgin  system  image.  In the example, locations 0 to 64230 (the executive) are

compared.

RUN $BINCMP

BINARY COMPARE - V01.01

OUTPUT FILE  [A]: TI:

INPUT FILE  1[A]: CRASH.CDA

INPUT FILE  2[A]: RSX11M.TSK

FILE  1 OFFSET [O]: 0

FILE  2 OFFSET [0]: 2000

ADDRESS OFFSET [O]: 0

START  ADDRESS [0]: 0

LAST   ADDRESS [0]: 64230

SEGMENT OUTPUT [Y/N]: Y

VALUE   OUTPUT [Y/N]: Y

A few notes about BINCMP.  All numbers are entered in octal and are  stored

in  I*4  variables.   If  segment  output  is  selected,  the  addresses  of all

differences will be listed.  If value output is selected, the actual values will

be listed.

When comparing a CDA file to a  virgin  system,  there  are  many  expected

differences.   The  system stack, SYSCM, and SYSTB will all change as the system

is executing.  Also, many of the executive modules have temporary  variables  in

them.

43

Crash Dump Analysis Workbook

CRASH DETECTIVE

5.2.3  Specials

Finally, it is not difficult to  write  you  own  programs  to  do  special

analysis  of  a CDA file.  CDA files, system images, and tasks are considered as

fixed length, 512 byte record files.

For example, your analysis shows that something  is  illegally  referencing

location  30  (EMT  trap) and storing a 102456 in it.  The following F4P program

will read the crash dump file and output the phyiscal address of all  occurances

of either "30" or "102456".  This will give you possible problem areas.

PROGRAM EXAMPL

C

C OPEN 'CRASH.CDA' AND SEARCH FOR ALL OCCURANCES OF EITHER

C "30" OR "102456".

C

C       DEFINE VARIABLES

C

INTEGER*2       IBLK    !BLOCK NUMBER

INTEGER*2       IWRD    !BLOCK OFFSET

INTEGER*4       IADR    !PHYSICAL ADDRESS

C

C       OPEN CRASH FILE FOR INPUT.

C

OPEN (UNIT=1,FILE='CRASH.CDA',TYPE='OLD',READONLY,

1               ACCESS='DIRECT',RECORDSIZE=128)

C

C       READ FILE SEQUENTIAL UNTIL EOF, CHECK FOR 4 OR 102456.

C

IBLK = 1

100    CONTINUE

READ (1'IBLK,END=999) IBUF

DO 200 IWRD = 1,256

IF (IBUF(IWRD).EQ."000030) GOTO 210

IF (IBUF(IWRD).EQ."102456) GOTO 210

GOTO 200

210            CONTINUE

IADR = (IBLK-1)*512+(IWRD*2)

WRITE  (5,1000) IADR,IBUF(IWRD)

200            CONTINUE

IBLK = IBLK+1

GOTO 100

C

C       CLOSE FILE AND EXIT.

C

999   CLOSE (UNIT=1)

STOP

C

C       FORMAT STATEMENTS.

C

1000   FORMAT (' ADDRESS ,O12,' HAS VALUE ',O6)

END

44

Crash Dump Analysis Workbook

CRASH DETECTIVE

5.3  Tricks

The following are tricks we use at Monsanto to solve problems.   There  are

probably  others.   In  general, we start using tricks when we get a handle on a

problem but cannot nail it down.

5.3.1  Branch-on-self

The branch-on-self instruction (777)  is  a  useful,  poor-man's  debugging

tool.   When executed, the executive or a program will patiently loop on itself.

This provides a quick and dirty way to stop the execution of something,  examine

various values, and continue execution.

Branch-on-self is most useful when you can reproduce a problem and need  to

trace  back to the source.  For example, you have a program which aborts because

a certain location has a bad value but you do not know who sets the  value  bad.

By  using  the OPEN command and branch-on-self, you can easily trace the program

by the following steps:

1.  Fix the program in core.

2.  Set a branch-on-self at an appropriate point.

3.  Run the program and cause the problem to reproduce.

4.  When the branch-on-self is  executed  (check  with  ATL),  examine  the

location  which  is  going  bad.   If  it is bad, you have narrowed the

problem to the code executed before the patch.

5.  Set the next 777 at the appropriate point.

6.  Restore the orginal contents of the previous patch.  The  program  will

resume execution and loop when it hits the new patch.

7.  Loop through steps 4, 5, and 6.

Some notes about this technique.  If working on a program,  make  sure  its

priority  is  less than MCR.  Otherwise, you will lock-up your system.  Also, if

using branch-on-self for executive, driver, or system state code, you will  have

to use the console to do the examines and deposits.

45

Crash Dump Analysis Workbook

CRASH DETECTIVE

5.3.2  On-line Patches

Often, to trap a problem, you will want to make a patch to  the  executive.

This  is  rather easy to do with the OPEN command.  One problem is finding space

to put the patch.  There are several possibilities:

o  The bottom of the kernel stack is almost never used.   The  stack  runs

from the top of the vector space (V$$CTR) to the label $STACK.  Usually

10 to 20 words at the bottom can be used for patches.   Note,  you  run

the risk of the kernel stack pushing down and wiping out the patch.

o  The stack space used by CRASH can be used for patches.  These locations

are  between  $CRPBF and $CRSBN.  This provides 173 words of space on a

mapped system.  Note, that the patch will be wiped out  if  the  system

crashes.

o  At Monsanto, we  always  include  a  module  in  our  executives  which

allocates a 64 word area.

o  On the SIG tapes in the crash seminar area  is  a  program  named  POL.

This program will takes the number of bytes and allocates a buffer from

pool of this size.  It reports the starting address  of  the  allocated

area.  This space can then be used for patches.

Some notes about this technique.  If patching code which may  be  executed,

either  use  the console switches or XDT (see loadable XDT on SIG tapes).  Also,

the patch will only remain in the system until it is rebooted.  If  needed,  use

ZAP to patch the system save file to make the patch permanent.

46

Crash Dump Analysis Workbook

CRASH DETECTIVE

5.3.3  Logic Analyzer

If  you  have  them,  logic  analyzers  can  be  an  invaluable  tool   for

troubleshooting  certain  types  of  bugs,  particularly  unknown  errors  which

modified known locations.  At Monsanto, we have the HP 1610 logic  analyzer  and

its  interface  boards for the Unibus and Q-bus.  By fitting the boards into the

backplane and programming the HP 1610, we can have it watch the  Unibus  address

and  data lines.  For example, if location 30 is being modified, we can have the

analyizer watch for a write cycle to address 30.  Then, by looking at the trace,

we can get the PC of the instruction which wrote to location 30.

47

Crash Dump Analysis Workbook

CRASH DETECTIVE

5.3.4  T-Bit Traps

The T-Bit traps provide another method of "watching" program execution.  It

is  possible  in  RSX-11M to trace-trap every instruction.  Consider the problem

from before, some unknown instruction is setting location 30 to a  102456.   The

following  steps will setup trace-trap to watch location 30 and crash the system

if it changes from its normal value (23242).

1.  Using OPEN, write a special service routine to examine location 30.  In

this  example,  we will using the bottom of the kernel stack.  Note the

use of the RTT to return from the T-Bit trap.  Also, the T-Bit  is  set

in  the  PS  of the trap.  This is in case the trace instruction turned

the T-Bit off.

>OPEN 400

000400/ 000000 022737           ;CMP #23242,@#30

000402/ 000000 023242           ;

000404/ 000000 000030           ;

000406/ 000000 000401           ;BEQ .+4

000410/ 000000 000000           ;HALT

000412/ 000000 052766           ;BIS #20,2(SP)

000414/ 000000 000020           ;

000416/ 000000 000002           ;

000420/ 000000 000006$          ;RTT

>

2.  Using OPEN, modify the T-Bit trap vector to point to your special code.

This will make any programs using the BPT trap useless (like ODT).

>OPEN 14

000014/ 021526 000400$          ;Set to special code

>

3.  Using OPEN, modify the PS of all vectors whose could should  be  traced

to  set the T-Bit.  For example, to trace directives, set the T-Bit for

the EMT trap.  Then, when that vector is used, the T-Bit trap  will  be

enabled  and  the  special code executed.  The system will halt when it

sees location 30 change from its normal value.

>OPEN 32

000032/ 000340 000360$          ;Set T-Bit on interrupt

>

48

Crash Dump Analysis Workbook

CRASH DETECTIVE

5.3.5  Mousetraps

Mousetraps are a powerful  concept  in  crash  analysis.   In  basic  crash

analysis,  the process is to work up the stack and backwards in time to the real

cause of the error.  However, basic analysis can only go so far back before  the

necessary tracks have been destroyed.  Here is where you use a mousetrap.

A mousetrap is a patch designed to detect an error condition closer to  the

real  cause  of  the error.  For example, your analysis shows the system crashed

because something  corrupted  location  4.   However,  the  real  error  occured

sometime  in  the  past  and  only  was detected when a user program odd-address

trapped.  What you would like to do is detect the  error  as  soon  as  possible

after location 4 is overwritten.

One method would be to write a dummy task that caused an  odd-address  trap

and continually run the task.  This is a mousetrap in its crudest form.  Another

technique would be to use the T-Bit trap and  continually  monitor  location  4.

However, this introduces a great deal of overhead on your system.

A better technique would be to place checks for  location  4  at  strategic

points in the system.  The structure of RSX-11M lends itself very well for this.

For example, all context switches into the executive  enter  and  exit  using  a

common  co-routine,  $DIRSV.   Similarly, all interrupts are processed using the

co-routine $INTSV.  By patching this code to monitor location 4, you can  narrow

the  error  down  to  the  current  task  or executive context.  If the error is

trapped on entering the executive, you know the current task is the culprit.  If

the  error  is trapped on exiting the executive, you know the problem occured in

the current context of the executive.

The variety and scope of possible mousetraps is limitless.  The list  below

has some of the types used at Monsanto to shoot problems.

o  SYSXT

The module SYSXT processes all changes in context for RSX-11M.  In this

module  are  the $DIRSV and $INTSV co-routines, fork processing and the

task context switch code.  If the problem is  executive  code  or  data

structure  corruption, this is a good module for adding mousetraps.  By

putting the trap here, you will at  least  catch  the  problem  in  the

current context it occured.

49

Crash Dump Analysis Workbook

CRASH DETECTIVE

o  SYSCM

Sometimes, analysis traces a problem to the executive  data  structures

in  SYSCM.   If  this is the case, a concatenated executive listing can

give you all references to the structure.  Then you can put  mousetraps

at all the appropriate points.  For example, analysis shows a bad value

is being put into $RQSCH.  By putting mousetraps at every point in  the

executive  which  writes  this value, we can find out the source of the

bad data.

o  Special Drivers

When a problem is I/O related, special drivers can be used to catch it.

For  example, at Monsanto we have an system where every three months or

so, block 2 in a certain file is corrupted.  To catch this problem,  we

have  put  a  special driver between the executive and the disk driver.

This driver watches all disk writes and when it sees an IO.WLB  to  the

block  in  question, it checks the data for validity.  This way, we can

catch the source of the problem in the context of the user task.

Special drivers for use as mousetraps are fairly easy.   The  technique

is  to  write  a  loadable  driver with a copy of the real drivers data

base.   The  mousetrap  driver  gets  the  I/O  packets,  performs  the

necessary checks, and then forwards the I/O packet onto the real driver

by queuing directly to the driver queue ($DRQRQ).  The mousetrap driver

is enabled by redirecting the real driver to it.

Mousetraps come in two forms.  The first watches for a problem and  crashes

the  system.  Another flavor is used to gather and record past history to aid in

tracking down a problem.  For example, to continue your analysis, you would like

to  see  what tasks where executing in the immediate past.  A mousetrap could be

added to store the TCB of the new task each time a context switch was made in  a

circular  buffer.   Then  when  the  system crashes, your could get the last 'n'

tasks which ran.  The same technique can be used to gather any historical  data.

The limit is only how much circular buffer you can make available.

50

Crash Dump Analysis Workbook

CRASH DETECTIVE

5.4  Mathematics Of RSX-11M

Your best tool  to  solve  crashes  is  your  intuition.   People  good  at

debugging  have  a  difficult  time explaining the thought process they use.  To

them it is mearly a matter of "the problem is  'x'  because  the  symptoms  were

'y'".   Because of this, many people consider crash analysis and debugging to be

a black art which is only to be practiced by believers in the occult.

Nothing could be further from the truth.  Crash analysis, if anything, is a

scientific  discipline.   A  computer  system  is a finite-state system which is

governed by a set of basic rules.  Crash analysis is the process of  determining

how  the  system reached one particular, undesirable state.  Skilled programmers

understand the rules of the  system.   This  allows  them  to  intuitively  draw

conclusions  that  others must slowly work out by hand.  An analogy can be drawn

between  the  study  of  RSX-11M  and  other,   more   traditional,   scientific

disciplines.   For  example,  in  physics,  knowledge of Newton's Laws of Motion

provides a basis for intuitively explaining how an object will react to a  given

force.   A similar understanding of the fundamental laws of RSX-11M will provide

insight into explaining how crashes occur.

The following are Stamerjohn's  Laws  of  RSX-11M.   Note,  the  lists  are

grouped  from most fundamental to most complex.  If a basic law is violated, the

nature of the beast is changed.

*  PDP-11 CPU

o  The CPU executed instructions correctly.

o  The CPU fault mechanisms worked correctly.

o  The CPU priority mechanism worked correctly.

o  The CPU memory management mechanism worked correctly.   This  means

that  at  any  given moment, an instruction can only address the 32

KW's currently addressed by the hardware (except for the  MTPI  and

MFPI instructions).

*  Peripherals

o  In general, assume peripherals work correctly,  including  whatever

fault  mechanisms are built into them.  However, certain classes of

faults will go undetected by both the hardware and RSX-11M:

*  Unexpected    interrupts    generally    go    undetected    by

Digital-written device drivers and cause executive and database

corruption.  A good rule  is  for  all  drivers  to  check  for

unexpected interrupts and dismiss them.

*  Both "HOT" and "COLD" bits in device  logic,  particularly  DMA

addresses  will go undetected by both hardware and software and

cause errors.

51

Crash Dump Analysis Workbook

CRASH DETECTIVE

o  Peripheral-caused crashes look strange.  However, remember that the

device  did  not  randomly pick how the system crashed.  Some error

occured in the device and from  that  point,  it  performed  as  it

should.   The question to answer for such errors is "What condition

will cause the device to generate the symptoms observed?" And  then

prove that is what happened.

*  Memory Mapping (20K option assumed)

o  The RSX-11M executive always maps the first  20  KW's  of  physical

memory  with  APR0-4  and  the  I/O page with APR7.  These mappings

never change.  This holds true even if  pool  does  not  extend  to

20KW.  This is how DECNET common executive works.

o  The excutive uses APR6 to map buffers and things.

o  Loadable drivers are always mapped with APR5.   APR6  is  free  for

their use.

o  Privilege tasks are mapped with APR5-6.  They are  also  mapped  to

the  executive  APR0-4.  They may use APR7 for code and not the I/O

page, but when switching to the executive, it will map APR7 to  the

I/O page.

*  RSX-11M Executive/User Interface

o  The interface between a user task and the  executive  is  always  a

trap.

o  All traps have a common entry and exit point inside the executive.

*  RSX-11M Executive Processing

o  The executive services all events serially, i.e.  the processing of

one event (an EMT) is completely finished before processing another

event  (I/O  completion).   Not  until  all  executive  events  are

processed  will  the  executive  return  to  the  user task.  Note,

finished in this case could mean queuing  the  processing  to  some

queue.

o  For the executive, the context of the current user  task  is  valid

only  to  the  point  where  processing  results in queuing to some

queue.  When processing resumes because of dequeuing, the state  of

the current user task must be assumed to have changed.

*  Pool/Data Structures

52

Crash Dump Analysis Workbook

CRASH DETECTIVE

o  Once  created,  a  data  structure  does  not  moved  until  it  is

destroyed.  However, be careful in assuming when that is.

o  Some data structures are a very static existence (the PCB for GEN).

Structures  in  the  class  include device DCB's, UCB's, and SCB's,

main partition PCB's and such.

o  Data structures associated with tasks are less  static  but  remain

typically  remain  valid  as  long  as  the  task does.  Note, task

headers are only valid as long as a task is in memory.

o  Once you run out of enough pool  to  create  a  task  header,  your

system is dead.

o  RSX-11M cannot run more than 140 tasks in memory at the same  time.

This  is  assuming you have 12KW of free pool and each task has has

mimumun data structures size.  To execute, a task must have a  TCB,

PCB,  and  task  header.   The  minimum size of these structures is

60(8) (TCB), 34(8) (PCB), and 144 (task  header)  for  a  total  of

260(8)  per  task.   With 60000(8) bytes of pool, this works out to

214(8) tasks or 140(10).  Note, in this senario, the tasks have  no

luns, send no messages, and in general, do nothing useful.

*  Resource Allocation

o  Each task in the system has  a  priority.   All  resources  in  the

system  are allocated on the basis of this priority, primiarily CPU

time  and  memory.   Tasks  at  the  same  priority  are  allocated

resources on the basis of first-in, first-out.

o  A task must be in core to execute.  Note, this includes aborting.

o  The entire purpose of the  task  loading  logic  is  to  bring  the

highest  priority  task  out  of  memory  into memory.  Because the

partition wait queue is priority ordered, the  logic  nevers  looks

beyond the first task in the partition wait queue, even if the task

cannot currently be brought into memory.

o  The task loading logic checkpoints tasks if  there  is  not  enough

free   memory   to   load   the  highest  priority  task  which  is

out-of-memory.  The checkpointing logic searches for the first task

in the partition which is lower in priority and would leave a large

enough hole.

o  Round-robin   scheduling   only   effects   CPU-time    allocation.

Therefore,  the  algorithm  only operates on tasks in memory and at

the same priority.  For such  tasks,  each  one  will  be  given  a

time-slice to execute in.  Note, if a higher priority task uses all

the available CPU time, nothing will be available for  other  tasks

to  round-robin  schedule  with.   There  is  no priority-promotion

associated with round-robin scheduling.

53

Crash Dump Analysis Workbook

CRASH DETECTIVE

o  Disk swapping only effects memory allocation.  The algorithm varies

the priority with which a task competes for memory, nothing else.

*  RSX-11M Task Scheduling

o  Unless a task is at priority 255, its execution may be  interrupted

at  any  point  in  time  and the context of the system switched to

another task.

o  Similarly, the exceution of a program will never be interrupted  by

a  program  of  lower  priority,  no  matter  how important the low

priority programs execution is.

o  The only exception to the above is when a  user  task  switches  to

kernal state.  Note, after the switch, the task is no longer a task

but part of the executive.

o  If your program does something to cause a  higher-priority  program

to  execute,  you  will  not execute the next instruction before it

executes.

o  If your program does something to cause a lower-priority program to

execute,  it  will  not  begin  to  execute  until  your  task does

something to put it in a wait state or exits.

54

APPENDIX A

CONCATENATED EXECUTIVE ASSEMBLY

One  of  the  most  useful  listings  to  have  for  crash  analysis  is  a

concatenated  listing  of  the  executive.   This  is  a  listing of all RSX-11M

executive modules and when done correctly, provides a listing which is an  exact

image  of  the  executive.   Also,  by  concatenating  and assembling the entire

executive, a cross-reference of all symbols can be obtained.

The  steps  for  producing  a  concatenated  executive  listing  are   very

straightforward.  The basic principles are as follows:

1.  Copy all executive modules loaded in your system into one  file.   Copy

the modules in the order they are shown loaded by the executive map.

2.  Edit the resulting  file.   Change  all  .TITLE  statements  to  .SBTTL

statements  and  comment  out  all .END statements except the last one.

Also, place a .DSABL LSB statement at the end of each executive module.

This will prevent local symbols from carrying over between modules.

3.  Assmble the resulting file and request a cref  for  the  listing.   Use

LB:[1,1]EXEMC/ML and the RSXMC.MAC file as prefix files.

The following pages list the command and source files used at  Monsanto  to

automate this process.  The following files are used:

1.  STEP0.CMD

This is the master command file.  It executes STEP1.TEC  to  produce  a

PIP  command  file  from  the  executive  task build file, executes the

resulting file (RSXPIP.CMD) to copy  the  executive  sources  into  one

file,  executes  STEP2.TEC to edit the concatentated file for assembly,

and finally, assembles the concatentated executive.

2.  STEP1.TEC

This is a TECO macro which inputs RSXBLD.CMD  and  outputs  RSXPIP.CMD.

The output file is a command file which copies the individual executive

sources into one file, RSX11M.MAC.

A-1

CONCATENATED EXECUTIVE ASSEMBLY

3.  STEP2.TEC

This is a TECO macro which edits RSX11M.MAC to change it into a  format

suitable for assembly.

4.  MASTER.MAC

This file is used as the first file in RSX11M.MAC.  It sets the correct

title and defines the executive macros so the various executive symbols

appear in the listing.

5.  C5TA.MAC

This is a dummy module for the C5TA routine  from  SYSLIB.OLB.   It  is

used  to  properly  space  the addresses so the resulting listing is an

exact image of the executive.

See STEP0.CMD for further details.  Note, the assembly produces two errors.

This is because Digital has used the same symboL (L.LGTH) for two different data

structures.

All the files shown on the previous pages will be on the RSX/IAS  SIG  tape

in [346,101].  For the TECO macros, a dollar sign ($) is an escape.

A-2

CONCATENATED EXECUTIVE ASSEMBLY

STEP0.CMD

A.1  STEP0.CMD

.ENABLE SUBSTITUTION

.SETS UIC <UIC>

;

; Build and Assemble Concantenated Executive.

;

; This command file creates and assembles a concantenated RSX-11M

; executive.  The command file requires the following files to be

; present in SY0:'UIC'.

;

;       STEP1.TEC       Phase 1 Teco command file

;       STEP2.TEC       Phase 2 Teco command file

;       MACRO.MAC       Initial concantenated source

;       C5TA.MAC        Fake C5TA source module

;

; In addition, the following files from the system generation must

; be present in SY0:'UIC'.

;

;       RSXBLD.CMD      Task Build command file

;       RSXMC.MAC       Configuration prefix file

;       SYSTB.MAC       Device table source file

;

; Finally, psuedo device IN: must be assigned to the disk containing

; the executive sources. The sources are assumed to be in [11,10].

; The executive library file is assumed to be in LB:[1,1]EXEMC.MLB.

; Also, the tasks TEC, PIP, MAC, and CRF must be installed. If all

; is correct, answer the next question 'YES'. Otherwise, the command

; file will pause and allow you complete the necessary setup.

;

.ASK FLAG CONTINUE WITH FILE

.IFF FLAG .PAUSE

;

; Create PIP command file from RSXBLD.CMD and copy executive.

;

TEC @STEP1.TEC

@RSXPIP.CMD

;

; Format RSX11M.MAC into correct form.

;

TEC @STEP2.TEC

;

; Assemble executive, getting list and cref.

;

MAC ,RSX11M/-SP/CR=LB0:[1,1]EXEMC/ML,SY0:'UIC'RSXMC/PA:1,RSX11M

;

; Waiting on CREF to finish.

;

.WAIT CRF...

;

; All done.

;

A-3

CONCATENATED EXECUTIVE ASSEMBLY

STEP1.TEC

A.2  STEP1.TEC

!---------------------------------------------------------------!

! Concantentated RSX-11M Executive - Step 1 Macro               !

!                                                               !

!       This macro inputs the RSXBLD.CMD file and produces a    !

!       PIP command file to append the necessary sources into   !

!       one file.                                               !

!---------------------------------------------------------------!

!---------------------------------------------------------------!

!       Open input and output files.                            !

!---------------------------------------------------------------!

ERRSXBLD.CMD$ HK Y

EWRSXPIP.CMD$

!---------------------------------------------------------------!

!       Strip garbage from RSXBLD.CMD.                          !

!---------------------------------------------------------------!

K S,[1,1]EXELIB/LB/SS$ -18D L .,ZK

!---------------------------------------------------------------!

!       Strip library names from RSXBLD.CMD.                    !

!---------------------------------------------------------------!

0J <S/LB:$; 0K>

!---------------------------------------------------------------!

!       Convert remaining names to one/line.                    !

!---------------------------------------------------------------!

0J <FS:$

$;>

!---------------------------------------------------------------!

!       Format module names into correct PIP commands.          !

!---------------------------------------------------------------!

0J <S

$; 2R I.MAC$ 0L IPIP RSX11M.MAC/AP=IN0:[11,10]$ L>

!---------------------------------------------------------------!

!       Process special cases of files, output, and exit.       !

!---------------------------------------------------------------!

0JI

PIP RSX11M.MAC/ME=SY0:MACRO.MAC

PIP RSX11M.MAC/AP=SY0:RSXMC.MAC

$0J SC5TA$ 0L FSIN0:[11,10]$SY0:$0J SSYSTB$ 0L FSIN0:[11,10]$SY0:$EX$$

A-4

CONCATENATED EXECUTIVE ASSEMBLY

STEP2.TEC

A.3  STEP2.TEC

!---------------------------------------------------------------!

! Concantentated RSX-11M Executive - Step 2 Macro               !

!                                                               !

!       This macro inputs the RSX11M.MAC file and produces a    !

!       file with .TITLE's and .END's changed.                  !

!---------------------------------------------------------------!

!---------------------------------------------------------------!

!       Open input and output files.                            !

!---------------------------------------------------------------!

EBRSX11M.MAC$ HK Y

!---------------------------------------------------------------!

!       Change RSXMC to function correctly.                     !

!---------------------------------------------------------------!

NS$$YDF$ 0KK FS.TITLE$.SBTTL$ NS$$YDF$ 0KK

!---------------------------------------------------------------!

!       Change .TITLE, .END pairs.                              !

!---------------------------------------------------------------!

<FN.TITLE$.SBTTL$; V 0L

I       .PAGE

$L N.END^N^EA$ 0L

I       .DSABL  LSB

.PSECT

;$ L>

!---------------------------------------------------------------!

!       Insert final .END and exit.                             !

!---------------------------------------------------------------!

.END    $INITL

$EX$$

A-5

CONCATENATED EXECUTIVE ASSEMBLY

MASTER.MAC

A.4  MASTER.MAC

.TITLE  RSX11M

.SBTTL  RSX11M

;

; PREFIX FILE FOR CONCANTENATED RSX-11M ASSEMBLY.

;

; THIS FILE DECLARES THE MACROS USED TO DEFINE EXECUTIVE

; TABLE STRUCTURES AND BITS. THIS IS DONE TO GET AS MANY

; NON-GLOBAL SYMBOLS AS POSSIBLE.

;

.MCALL  IOERR$

IOERR$

.MCALL  FILIO$

FILIO$

.MCALL  ABODF$

ABODF$

.MCALL  CLKDF$

CLKDF$

.MCALL  DCBDF$

DCBDF$

.MCALL  EMBDF$

EMBDF$

.MCALL  HDRDF$

HDRDF$

.MCALL  HWDDF$

HWDDF$

.MCALL  ITBDF$

ITBDF$

.MCALL  PCBDF$

PCBDF$  ,,SYSDEF

.MCALL  PKTDF$

PKTDF$

.MCALL  SCBDF$

SCBDF$

.MCALL  TCBDF$

TCBDF$

.MCALL  UCBDF$

UCBDF$

.PAGE

A-6

CONCATENATED EXECUTIVE ASSEMBLY

C5TA.MAC

A.5  C5TA.MAC

.TITLE  C5TA

.IDENT  /02/

;

; FAKE C5TA, USED TO CORRECT SPACE FILL EXECUTIVE.

;

$C5TA::                         ;REF. LABEL

.BLKB   110             ;SIZE OF MODULE

.END

A-7

APPENDIX B

CRASH MANAGMENT COMMAND FILE

The following is a command file that will read a crash dump  from  a  crash

device  (MM:   in this case), name it, and generate the initial listing.  A copy

of this file is on the SIG tape.  It is named CRASHREAD.CMD.

B-1

CRASH MANAGMENT COMMAND FILE

.; Command file to read crash dumps, name according to the

.; date and crash, and get initial CDA listing using the

.; options /ACT/HDR/PCB.

.;

.; The name is the form NAMEMMDDX, where NAME is the system

.; name, MM is the month, DD is the day, and X is the crash

.; of the day (A for the first, B for the second, etc.)

.;

SET /UIC=[1,17]

.ENABLE SUBSTITUTION

.SETS   $NAME   "XXXX"

.;

.; Get date and parse out month and day.

.;

.SETS   $DATE   <DATE>

.PARSE  $DATE   "--" $DD MM YR

.IF MM ="JAN" .SETS $MM "01"

.IF MM ="FEB" .SETS $MM "02"

.IF MM ="MAR" .SETS $MM "03"

.IF MM ="APR" .SETS $MM "04"

.IF MM ="MAY" .SETS $MM "05"

.IF MM ="JUN" .SETS $MM "06"

.IF MM ="JUL" .SETS $MM "07"

.IF MM ="AUG" .SETS $MM "08"

.IF MM ="SEP" .SETS $MM "09"

.IF MM ="OCT" .SETS $MM "10"

.IF MM ="NOV" .SETS $MM "11"

.IF MM ="DEC" .SETS $MM "12"

.;

.; Loop and find which crash sequence number to use.

.;

.SETN   $POS    0

.SETS   ALBET   "ABCDEFGHIJKLMNOPQRSTUVWXYZ"

.100:   .INC    $POS

.SETS   $X      ALBET[$POS:$POS]

.SETS   $FILE   $NAME+$MM+$DD+$X

.TESTFILE       '$FILE'.CDA

.IF     <FILERR> = 1 .GOTO 100

.;

.; Install necessary tasks, load driver, and get dump.

.;

.IFNINS ...LOA  INS $LOA

.IFNINS ...UNL  INS $UNL

.IFNINS ...CDA  INS $CDA

.IFNLOA MM:     LOA MM:/HIGH/PAR=GEN

.SETS   F       $FILE

CDA 'F'/-SP,'F'/MEMSIZ:'<MEMSIZ>'.=[1,54]/STB,MM:/PCB/ACT/HDR

.;

.; Clean-up from command file.

.;

UNL MM:

REM CDA

B-2

APPENDIX C

CDA AND RELATED PATCHES

This section has  patches  used  at  Monsanto  to  correct  errors  in  the

executive  crash dump routine and CDA itself.  In this section are the following

files:

o  LOWCR.COR

This file patches the jump to $CRASH to correct jump to  $CRALT.   This

prevents  the top two words on the stack from being popped off and used

as the PS and PC of the trap address.

o  CRASH.COR

This file patches CRASH for a variety of problems.  It also includes at

patch to correctly get the user stack pointer and save location 4.

o  CDA.CMD

This file applies the patches to CDA.

o  CDACLQ.PAT

This file patches CDA to report the correct time for  the  clock  queue

dump.

o  LUDMP.PAT

This file patches CDA to not  print  garbage  for  non-Files-11  window

block, like DECNET.

o  TDUMP.PAT

This file patches CDA to correctly output the pre-AST status bits.

C-1

CDA AND RELATED PATCHES

LOWCR.COR

C.1  LOWCR.COR

LOWCR.MAC/AU:72./-BF=LOWCR.DMC

\

-2,2

.IDENT  /09A/

-11,11

; VERSION 09A

-27

;

;               MSH089 -- PRESERVE ENTIRE STACK DURING FORCED CRASH

%

-80,,/;MSH089/

.IF DF  C$$RSH

JMP     $CRALT          ;JMP TO CRASH ROUTINE

.IFF

-81,,/;MSH089/

.ENDC

/

C-2

CDA AND RELATED PATCHES

CRASH.COR

C.2  CRASH.COR

CRASH.MAC/AU:72./-BF=CRASH.DMC

\

-2,2

.IDENT  /04.03G/

-25,25

; VERSION 4.03A

-43

;               MSH039 -- DON'T RELY ON NXM TRAPS TO STOP RL11

;                         MEMORY DUMPING ON Q-BUS MACHINE

;

;               MSH057 -- PREVENT TK06 ROUTINE FROM CORRUPTING THE

;                         DISK BY A NXM TRAP

;

;               MSH058 -- DO NOT ATTEMPT TO CLEAR R-ONLY REGISTERS

;

;               MSH059 -- FIX TS11 BUGS

;

;               MSH101 -- GET USER MODE SP

;

;               MSH113 -- GET UMR#1 FOR TS11 MEMORY MAPPING

;

;               MAP003 -- SAVE LOCATION 4

;

%

-64,,/;MAP003/

$CRTR4::.WORD   0               ;TRAP 04 VALUE

-184,,/;MAP003/

MOV     @#4,$CRTR4      ;SAVE TRAP 04 LOCATION

-256,,/;MSH101/

.IF DF  M$$MGE

MOV     #PMODE!PR7,PS   ;;;LOCK OUT INTERRUPTS AND SET

;;;PREVIOUS MODE=USER

.IFF

-257,,/;MSH101/

.ENDC

-303,,/;MAP003/

.IF DF  M$$WRD

MOV     $CRAR5,24(SP)   ;;;RESET CORRECT APR5 VALUE

.ENDC

-623,632,/;MSH057/

C-3

CDA AND RELATED PATCHES

CRASH.COR

.IFTF   ;M$$EXT

.IF DF  M$$MGE

MOV     #140000,R5      ;;;SET TO MAP THROUGH APR6

CLR     @#KISAR6        ;;;POINT TO FIRST BYTE OF 1K SEGMENT

.IFF    ;M$$MGE

CLR     R5              ;;;R5 TRACKS THE DUMP PROCESS

.ENDC   ;M$$MGE

.IFT    ;M$$EXT

50$:    CLR     4(R0)           ;;;START AT BEGINNING OF UNIBUS MAP

.IFF    ;M$$EXT

CLR     4(R0)           ;;;START OF BEGINNING OF MEMORY

50$:

.IFTF   ;M$$EXT

MOV     #-2000,2(R0)    ;;;WRITE 1K WORDS PER TRANSFER

-636,649,/;MSH057/

BMI     80$             ;;;IF MI ERROR

.IFT    ;M$$EXT

ADD     #4000,@#UBMPR   ;;;POINT TO NEXT 1K WORD BOUNDARY

ADC     @#UBMPR+2       ;;;DON'T FORGET THE CARRY

ADD     #40,@#KISAR6    ;;;ADJUST DUMP TRACKING POINTER

CMP     #170000,@#KISAR6;;;HAVE WE HIT THE I/O PAGE?

.IFF    ;M$$EXT

.IF DF  M$$MGE

ADD     #40,@#KISAR6    ;;;ADJUST DUMP TRACKING POINTER

CMP     #7600,@#KISAR6  ;;;HAVE WE HIT THE I/O PAGE?

.IFF    ;M$$MGE

ADD     #4000,R5        ;;;ADJUST DUMP TRACKING POINTER

CMP     #160000,R5      ;;;HAVE WE HIT THE I/O PAGE?

.ENDC   ;M$$MGE

.ENDC   ;M$$EXT

BEQ     $CRCMP          ;;;IF EQ YES, DUMP IS DONE

TST     (R5)            ;;;DOES FIRST WORD OF NEXT TRANSFER?

C-4

CDA AND RELATED PATCHES

CRASH.COR

BCC     50$             ;;;IF CC YES

BR      $CRCMP          ;;;IF CS NO, DUMP IS DONE

80$:

-670,,/;MSH039/

.IFF

.IF DF  M$$MGE

MOV     #124.*4,-(SP)   ;;;DUMP MAXIMUM 124.K WORDS

.IFF

MOV     #28.*4,-(SP)    ;;;DUMP MAXIMUM 28.K WORDS

.ENDC

-681,681,/;MSH039/

BMI     WERRX           ;;;IF MINUS I/O ERROR

-688,688,/;MSH039/

BMI     WERRX           ;;;IF MINUS I/O ERROR

-715,715,/;MSH039/

BMI     WERRX           ;;;IF MINUS I/O ERROR

-723,,/;MSH039/

DEC     (SP)            ;;;DECREMENT MAXIMUM BLOCK DUMP

BLE     72$             ;;;IF EQ, DUMP IS DONE

-750,750,/;MSH039/

BEQ     WERRX           ;;;IF EQ, I/O ERROR

72$:    TST     (SP)+           ;;;CLEAN STACK

BR      $CRCMP          ;;;DUMP DONE

WERRX:  TST     (SP)+           ;;;CLEAN STACK

-879,879,/;MSH058/

TST     2(R0)           ;;;RESET RBUF

-969,975,/;MSH059/

100$:   MOV     #<1$+12>,R5     ;;;WE NOW COMPUTE ADDRESS OF COMMAND

BIC     #3,R5           ;;;BUFFER. (NEEDS DOUBLE WORD BOUND)

MOV     #8.,-(R5)       ;;;LENGTH OF COMMAND PACKET ADDRESS

CLR     -(R5)           ;;;UPPER 2 BITS OF COMMAND PACKET

MOV     #2$,-(R5)       ;;;LOW ORDER 16 BITS

MOV     #100004,-(R5)   ;;;WANT TO WRITE CHARACTERISTICS

MOV     R5,R4           ;;;COPY POINTER TO COMMAND PACKET

;;;R4=R5 IS FLAG THAT TS11 IS

;;;BEING INITIALIZED (SEE 180$)

-978,978,/;MSH059/

-980,980,/;MSH059/

MOV     #142010,(R5)    ;;;REWIND AND CLEAR VOLUME CHECK

-982,982,/;MSH059/

-984,984,/;MSH113/

.IF DF  M$$EXT

MOV     #20000,(R5)+    ;;;POINT BUFFER TO SECOND UMR

C-5

CDA AND RELATED PATCHES

CRASH.COR

.IFF

CLR     (R5)+           ;;;START AT LOCATION 0

.ENDC

-986,986,/;MSH059/

MOV     #512.,(R5)      ;;;NUMBER OF BYTES IN A RECORD

-992,993,/;MSH113/

CLR     @#UBMPR+4       ;;;INITIALIZE UMR TO ADDRESS 0

CLR     @#UBMPR+6       ;;;

-994,994,/;MSH059/

-997,998,/;MSH059/

MOV     #<1920./4>,R3   ;;;MAX NUMBER OF 4K WORD BLOCKS

40$:    MOV     #<1024.*4*2/512.>,R2;;;MAX NUMBER OF 512 BYTE REC/BLK

-1002,1002,/;MSH059/

MOV     #<124.*4>,R2    ;;;MAX NUMBER OF 4K WORD BLOCKS

-1006,1006,/;MSH059/

50$:    MOV     #9.,R5          ;;;RETRY COUNT+1

CALL    200$            ;;;ISSUE WRITE AND WAIT

-1015,1017,/;MSH113/

ADD     #<1024.*4*2>,@#UBMPR+4;;;BUMP ADDRESS OF UMR BY

ADC     @#UBMPR+6       ;;; 4K WORDS

MOV     #20000,2(R4)    ;;;REINITIALIZE POINTER WITHIN 4K

;;; WINDOW TO START OF UMR 1

-1019,1029,/;MSH059/

BNE     40$             ;;;IF NE BLOCK LIMIT NOT REACHED

.IFTF

;

; NOTE THAT THE FOLLOWING BRANCH IS NEVER TAKEN IF 180$ IS FALLEN

; INTO (R5 IS NEVER EQUAL TO R4, BECAUSE R5 IS A RETRY COUNT AND

; R4 IS AN ADDRESS, UNLESS 180$ IS BRANCHED TO FROM BELOW).

;

180$:   CMP     R4,R5           ;;;IS TS11 BEING INITIALIZED

BEQ     280$            ;;;IE EQ YES, ERROR EXIT

MOV     #100011,(R4)    ;;;WANT TO FORMAT (WRITE TAPE MARK)

MOV     R4,(R1)         ;;;ISSUE COMMAND

190$:   TSTB    (R0)            ;;;SUBSYSTEM READY?

BPL     190$            ;;;IF PL NO

BR      $CRCMP          ;;;SUCCESSFUL TERMINATION

200$:                           ;;;REF LABEL

.IFT

MOV     R3,-(SP)        ;;;SAVE BLOCK COUNTER

.IFTF

210$:   MOV     R4,(R1)         ;;;ISSUE COMMAND

-1032,1032,/;MSH059/

MOV     (R0),R3         ;;;GET STATUS

C-6

CDA AND RELATED PATCHES

CRASH.COR

-1033,,/;MSH059/

.IFT

MOV     (SP)+,R3        ;;;RESTORE BLOCK COUNTER

.ENDC

BIC     #1000,(R4)      ;;;CLEAR WRITE DATA RETRY BIT

-1037,1056,/;MSH059/

BNE     180$            ;;;IS NE YES

BIT     #4000,R3        ;;;NONEXISTANT MEMORY

BNE     180$            ;;;IF NE YES

BIC     #^C<16>,R3      ;;;ISOLATE TERMINATION CODE

CMP     #12,R3          ;;;RECOVERABLE ERROR (TAPE NOT MOVE)?

BEQ     270$            ;;;IF EQ YES

CMP     #10,R3          ;;;RECOVERABLE ERROR (TAPE MOVE ONE)?

BNE     280$            ;;;IS NE NO, I/O ERROR

BIS     #1000,(R4)      ;;;CHANGE COMMAND TO SPACE REVERSE,

;;; ERASE, WRITE DATA

270$:   DEC     R5              ;;;EXHAUSTED RETRY COUNT?

BNE     210$            ;;;IF NE NO

280$:   TST     (SP)+           ;;;DISCARD RETURN ADDRESS

/

C-7

CDA AND RELATED PATCHES

CDA.CMD

C.3  CDA.CMD

;

; APPLY CDA PATCHES - REFERENCE TO SOFTWARE DISPATCH OR SPR.

;

;       CDACLQ          MONSANTO        013522  017371

;

PAT CDACLQ   .OBJ=CDACLQ    .DOB/CS:013522,CDACLQ   .POB/CS:017371

;

;       LUDMP           SPR 11-29693    006500  022302

;

PAT LUDMP    .OBJ=LUDMP     .DOB/CS:006500,LUDMP    .POB/CS:022302

;

;       TDUMP           MONSANTO        073153  012554

;

PAT TDUMP    .OBJ=TDUMP     .DOB/CS:073154,TDUMP    .POB/CS:012554

C-8

CDA AND RELATED PATCHES

CDACLQ.PAT

C.4  CDACLQ.PAT

.TITLE  CDACLQ

;

; MODIFICATION:

;

;       2.04A -- PRINT TIME CORRECTLY (MONSANTO PATCH)

;

.IDENT  /2.04A/

.PSECT

.BLK.   = .

.       = .BLK.+1030

CALL    PAT1                    ;CALL FIRST PATCH

NOP                             ;

NOP                             ;

.       = .BLK.+1136

CALL    PAT2                    ;CALL SECOND PATCH

.PSECT  PAT

PAT1:   MOV     R2,-(SP)                ;SAVE R2

MOV     $TKPS,R3                ;GET ADDRESS OF TICKS

TRAP    0                       ;GET TICS PER SECOND

MOV     R2,R0                   ; MOVE TO R0

MOV     (SP)+,R2                ;RESTORE R2

RETURN

PAT2:   CALL    $DDIV                   ;GET NUMBER OF HOURS

MOV     R0,R4                   ;REMANINDER IS MINUTES

RETURN

.END

C-9

CDA AND RELATED PATCHES

LUDMP.COR

C.5  LUDMP.COR

.TITLE  LUDMP

;

; MODIFICATIONS:

;

;       0104A -- CHECK FOR FILES-11 DEVICE BEFORE ACCESSING FCB

;

.IDENT  /0104A/

.PSECT

.BLK.   = .

.       = .BLK.+4

NOWIND:

.       = .BLK.+14

UCBADR:

.       = .BLK.+1036

CALL    PAT1                    ;CALL PATCH 1

BCS     10$                     ; IF CS, DONE WITH LUN

.       = .BLK.+1250

10$:

.       = .BLK.

.PSECT  PAT1

PAT1:

MOV     #NOWIND,R1              ;ASSUME NO WINDOW

MOV     R2,R4                   ;GET WINDOW ADDRESS

MOV     UCBADR,R5               ;GET UCB ADDRESS

BEQ     10$                     ; IF EQ, IT IS UNASSIGNED

5$:     MOV     R5,R3                   ;COPY UCB ADDRESS

ADD     #2,R3                   ;POINT TO REDIRECT POINTER

TRAP    0                       ;GET IT

CMP     R2,R5                   ;END OF REDIRECT CHAIN?

BEQ     7$                      ; IF EQ - YES

MOV     R2,R5                   ;GET NEXT UCB ADDRESS IN CHAIN

BR      5$                      ; AND LOOP

7$:     MOV     R5,UCBADR               ;SAVE UCB ADDRESS

MOV     R5,R3                   ;COPY UCB ADDRESS

ADD     #10,R3                  ;POINT TO DEVICE CHAR.

TRAP    0                       ;GET IT

BIC     #007727,R2              ;ISOLATE DESIRED BITS

CMP     #140010,R2              ;IS DEVICE NOT FILES-11

BNE     10$                     ;IF NE, YES, DONE WITH LUN

MOV     R5,R3                   ;COPY UCB ADDRESS

ADD     #5,R3                   ;POINT TO STATUS BYTE

C-10

CDA AND RELATED PATCHES

LUDMP.COR

TRAP    0                       ;GET IT

BITB    #140,R2                 ;IS DEV NOT MOUNTED OR FOREIGN

BNE     10$                     ; IF NE, YES, DONE WITH LUN

CLC                             ;SET FILES-11 FLAG

BR      20$                     ;

10$:    SEC                             ;SET NON FILES-11 FLAG

20$:    RETURN                          ;

.END

C-11

CDA AND RELATED PATCHES

TDUMP.PAT

C.6  TDUMP.PAT

.TITLE  TDUMP

;

; MODIFICATION:

;

;       0115A -- OUTPUT PRE-AST STATUS CORRECTLY (MONSANTO PATCH)

;

.IDENT  /0115A/

.PSECT

.BLK.   = .

.       = .BLK.+176

.RAD50  /STP/

.       = .BLK.+202

.RAD50  /SPN/

.       = .BLK.+206

.RAD50  /WFR/

.WORD   0

.       = .BLK.+4414

CALL    .GTSTS                  ;CALL CONVERSION (RAD50)

.END

C-12

APPENDIX D

RSX-11M DATA STRUCTURES

D-1

APPENDIX D

DATA STRUCTURES

This sections is a reference guide to the  RSX-11M  data  structures.   The

section  includes  information  on  the  system database (SYSCM), executive pool

structures, Files-11, task images, FCS, and DECNET (phase II).

Each data structure is diagramed and  all  defined  symbolics  are  listed.

Also,  there  is  a  brief  verbal  description  of  the  major  function of the

structure, how and when the structure is created and destroyed,  and  the  major

method  used  to address and link the structure.  Finally, there are list of all

references from the structure to others and all refernces in other structures to

this type of structure.

D-1

DATA STRUCTURES

SYSCM

D.1  SYSCM

The module SYSCM is the RSX-11M executive data base.  Included  in  it  are

all  of  the  listheads for the major executive data structures, the time-of-day

and date fields, and executive status words.  The  following  diagram  show  the

format of SYSCM for a mapped system with all conditionals set on.

Macro: none     CDA Switch: /SYS        Size: variable

$CMBEG

|===============================|

00      |      Current Task Header      | $HEADR

|-------------------------------|

02      | Cur Task Pri  |       0       |        $CURPR

|-------------------------------|

04      |                               | $COMEF

|---   Global Event Flags    ---|

06      |                               |

|-------------------------------|

10      |        System Baselevel       | $SYSID

|---                         ---|

12      |            (ASCII)            |

|-------------------------------|

14      |     Pointer to TKTN TCB       | $TKNPT

|-------------------------------|

16      |    Pointer to Shuffler TCB    | $SHFPT

|-------------------------------|

20      |  Address of Clock Count Reg.  | $CKCNT

|-------------------------------|

22      |   Address of Clock CSR Reg.   | $CKCSR

|-------------------------------|

24      |       Clock Load Count        | $CLLDC

|-------------------------------|

26      |     System UIC (/SYSUIC)      | $SYUIC

|-------------------------------|

30      |               0               |

|-------------------------------|

32      |       Top of Executive        | $EXSIZ

|-------------------------------|

34      |  Power Recovery Request Flag  | $PWRFL

|-------------------------------|

36      |  Task Waiting for SIG Event   | $SIGFL

|-------------------------------|

(continued on next page)

D-2

DATA STRUCTURES

SYSCM

SYSCM (continued)

|-------------------------------|

40      |          LCB Listhead         | $LOGHD

|-------------------------------|

42      |  MCR Command Buffer Listhead  | $MCRCB

|-------------------------------|

44      |           Lock Word           | $LSTLK

|-------------------------------|

46      |  Pool Allocation Constant (3) |

|-------------------------------|

50      |     First Free Pool Node      | $CRAVL

|-------------------------------|

52      |               0               |

|-------------------------------|

54      |     Active Task Listhead      | $ACTHD

|-------------------------------|

56      |     Absolute Time Counter     | $ABTIM

|-------------------------------|

60      |       Current Task TCB        | $TKTCB

|-------------------------------|

62      |     Schedule Request TCB      | $RQSCH

|-------------------------------|

64      |         Stack Depth           | $STKDP

|-------------------------------|

66      |        DCB Listhead           | $DEVHD

|-------------------------------|

70      |     Pointer to MCR TCB        | $MCRPT

M$$CLI >>>>>>>>>|-------------------------------|

v     72      |                               |

v             |---                         ---|

v     74      |                               |

v             |---   MCR Prompt String     ---|

v     76      |          (ASCIZ)              |

v             |---                         ---|

v     100     |                               |

>>>>>>>>>>>>>>|-------------------------------|

102     |  Pointer to Error Logger TCB  | $ERRPT

|-------------------------------|

104     | Checkpoint File PCB Listhead  | $CFLPT

|-------------------------------|

106     |    Clock Fork Block Link      |

|-------------------------------|

110     |     Clock Fork Block PC       |

|-------------------------------|

112     |      Clock Ticks Count        | $INTCT

|-------------------------------|

(continued on next page)

D-3

DATA STRUCTURES

SYSCM

SYSCM (continued)

|-------------------------------|

114     |                               | $FRKHF

|---   Fork Queue Listhead   ---|

116     |                               |

|-------------------------------|

120     |     Feature Test Masks        | $FMASK

|-------------------------------|

122     |  Parity Address Vector Table  | $PARPT

|-------------------------------|

124     |     Clock Queue Listhead      | $CLKHD

|-------------------------------|

126     |        CO: Output UCB         | $COPT

|-------------------------------|

130     |         PCB Listhead          | $PARHD

|-------------------------------|

132     |    Pointer to LDR... TCB      | $LDRPT

|-------------------------------|

134     |        TCB Listhead           | $TSKHD

|-------------------------------|

136     | Group Global Event Flags List | $GGEF

|-------------------------------|

140     |    Idle Patterm Flag/Count    | $IDLCT $IDLFL

|-------------------------------|

142     |        Idle Pattern           | $IDLPT

|-------------------------------|

144     |     FEB       |      MAR      | $DYPMN

|---            |            ---|

146     |     APR       |      MAY      |

|---            |            ---|

150     |     JUN       |      JUL      |

|---       Days/Month        ---|

152     |     AUG       |      SEP      |

|---            |            ---|

154     |     OCT       |      NOV      |

|---            |            ---|

156     |     DEC       |      JAN      |

|-------------------------------|

(continued on next page)

D-4

DATA STRUCTURES

SYSCM

SYSCM (continued)

|-------------------------------|

160     |                 1             | $BTMSK

|---                         ---|

162     |                 2             |

|---                         ---|

164     |                 4             |

|---                         ---|

166     |                10             |

|---                         ---|

170     |                20             |

|---                         ---|

172     |                40             |

|---                         ---|

174     |               100             |

|---                         ---|

176     |               200             |

|---                         ---|

200     |               400             |

|---                         ---|

202     |              1000             |

|---                         ---|

204     |              2000             |

|---                         ---|

206     |              4000             |

|---                         ---|

210     |             10000             |

|---                         ---|

212     |             20000             |

|---                         ---|

214     |             40000             |

|---                         ---|

216     |            100000             |

E$$DVC >>>>>>>>>|-------------------------------|

v     220     |                               | $ERRHD

v             |---      EMB Listhead       ---|

v     222     |                               |

v             |-------------------------------|

v     224     |    Limit on Resident Data     | $ERRLM

v             |-------------------------------|

v     226     |    Error Sequence Number      | $ERRSQ

v             |-------------------------------|

v     230     |  Pointer to Error File Block  | $ERRSV

v             |-------------------------------|

v     232     |    Size of Resident EMB's     | $ERRSZ

v             |-------------------------------|

v     234     |   Device I/O Active Bitmap    | $IOABM

>>>>>>>>>>>>>>|-------------------------------|

(continued on next page)

D-5

DATA STRUCTURES

SYSCM

SYSCM (continued)

|-------------------------------|

236     |  Size of Memory (32W blocks)  | $SYSIZ

|-------------------------------|

240     |  (high byte)  |  Disk Unit #  |

|               ----------------|

242     |      LBN of Saved Image       |

|-------------------------------|

244     |    Load Device Name (ASCII)   |

|-------------------------------|

246     |    Size of Load Image (32W)   |

|-------------------------------|

250     |         Years/Universe        |

|-------------------------------|

252     |          Months/Year          |

|-------------------------------|

254     |          Days/Month           |

|-------------------------------|

256     |          Hours/Day            |

|-------------------------------|

260     |          Minutes/Hour         |

|-------------------------------|

262     |         Seconds/Minute        |

|-------------------------------|

264     |          Ticks/Second         | $TKPS

|-------------------------------|

266     |        Year of Universe       |

|-------------------------------|

270     |         Month of Year         |

|-------------------------------|

272     |         Day of Month          |

|-------------------------------|

274     |         Hour of Day           |

|-------------------------------|

276     |        Minute of Hour         |

|-------------------------------|

300     |       Second of Minute        |

|-------------------------------|

302     |        Tick of Second         | $TTNS

Q$$OPT >>>>>>>>>|-------------------------------|

v     304     |  Pointer to Free I/O Packets  | $PKAVL

v             |-------------------------------|

v     306     |     Max #     |   Current #   | $PKNUM $PKMAX

>>>>>>>>>>>>>>|-------------------------------|

(continued on next page)

D-6

DATA STRUCTURES

SYSCM

SYSCM (continued)

E$$XPR >>>>>>>>>|-------------------------------|

v     310     |     Maximum Extend Size       | $MXEXT

M$$EXT >>>>>>>>>|-------------------------------|

v     312     |  Mapping Assignment Listhead  | $UMRHD

v             |-------------------------------|

v     314     | Address of First Assigned UMR |

v             |-------------------------------|

v     316     |  # Statically Allocated UMR's |

v             |-------------------------------|

v     320     |                               | $UMRWT

v             |--- UMR Wait Queue Listhead ---|

v     322     |                               |

T$$CPW >>>>>>>>>|-------------------------------|

v     324     |    Temp Storage for $GSPKT    | $TEMP2

P$$OFF >>>>>>>>>|-------------------------------|

v     326     |                               | $TEMP0

v             |--- Temp Storage for Spawn  ---|

v     330     |                               | $TEMP1

>>>>>>>>>>>>>>|-------------------------------|

332     |     Temp Storage of DIC       | $DICSV

|-------------------------------|

334     |   Mounted Device Listhead     | $MOULS

|-------------------------------|

346     |               | Context Dis.  | $CXDBL

|===============================|

350                                       $CMFIN

D-7

DATA STRUCTURES

TCB

D.2  TCB

The TCB (Task Control Block) is the primary task data structure.  TCB's are

always  located  in  system  pool.  They are created by INS and destroyed when a

task is removed.  Also, TCB's are created by MCR when using  the  RUN  $filename

command.

The TCB's are chained together using the T.TCBL word.  The chain header  is

$TSKHD which is located in SYSCM.  Also, the currently active tasks are threaded

together using the T.ACTL offset.  This chain header is $ACTHD in SYSCM.

TCB References:

o  (T.CPCB) Checkpoint PCB

o  (T.CPCB) Interrupt transfer blocks (connect-to-interrupt)

o  (T.RCVL) Send/Receive messages queued to task

o  (T.RCVL) I/O packets queued to ACP's

o  (T.ASTL) AST control clocks queued to task

o  (T.UCB ) UCB of TI:  terminal

o  (T.TCBL) TCB of next installed task

o  (T.LDV ) UCB of load device

o  (T.PCB ) PCB of task partition

o  (T.ACTL) TCB of next active task

o  (T.SAST) Specified AST control block list

o  (T.ATT ) Attachment descriptors for task

o  (T.RRFL) Receive-By-Reference messages queued to task

o  (T.OCBH) OCB chain for this task

References to TCB's

o  ($TKNPT) TKTN TCB address

o  ($SHFPT) Shuffler TCB address

o  ($ACTHD) First TCB in active task list

o  ($TKTCB) Current active TCB

o  ($RQSCH) TCB requesting scheduling

o  ($MCRPT) MCR TCB address

o  ($ERRPT) Error Logger TCB address

o  ($TSKHD) First TCB in installed task list

o  (O.PTCB) OCB, parent task TCB

o  (P.TCB ) PCB, owner task TCB

o  (A.TCB ) Attachment descriptor, attached task TCB

o  (C.TCB ) Clock queue, TCB of task to request is for

o  (I.TCB ) I/O packet, issueing task TCB

o  (X.TCB ) ITB, owning task TCB

o  (U.ACP ) UCB, ACP TCB address

D-8

DATA STRUCTURES

TCB

TCB

Macro: TCBDF$   CDA Switch: /ACT /STD   Size: 100 bytes (32 words) w/PLAS, OCB

72 bytes (28 words) w/PLAS only

56 bytes (23 words) no options

|===============================|

00      |       Utility Link Word       | T.LNK

|-------------------------------|

02      |   I/O Count   | Task Priority | T.PRI  T.IOC

|-------------------------------|

04      |  Checkpoint PCB/ITB Listhead  | T.CPCB

|-------------------------------|

06      |                               | T.NAM

|---     Task Name (RAD50)   ---|

10      |                               |

|-------------------------------|

12      |                               | T.RCVL

|---  Receive Queue Listhead ---|

14      |                               |

|-------------------------------|

16      |                               | T.ASTL

|---    AST Queue Listhead   ---|

20      |                               |

|-------------------------------|

22      |                               | T.EFLG

|---    Local Event Flags    ---|

24      |                               |

|-------------------------------|

26      |        TI: UCB Address        | T.UCB

|-------------------------------|

30      |  Link to Next Installed Task  | T.TCBL

|-------------------------------|

32      |    Status (Blocking Bits)     | T.STAT

|-------------------------------|

34      |     Status (Status Bits)      | T.ST2

|-------------------------------|

36      |    Status (Attribute Bits)    | T.ST3

|-------------------------------|

40      |  (High Byte)  | Default Pri.  | T.DPRI D.LBN

|               ----------------|

42      |   Block Number of Load Image  |

|-------------------------------|

44      |    Load Device UCB Address    | T.LDV

|-------------------------------|

46      |           PCB Address         | T.PCB

|-------------------------------|

50      |    Maximum Task Image Size    | T.MXSZ

|-------------------------------|

continued on next page

D-9

DATA STRUCTURES

TCB

TCB (continued)

|-------------------------------|

52      |   Link to Next Active Task    | T.ACTL

|-------------------------------|

54      |    Specified AST Listhead     | T.SAST

P$$LAS >>>>>>>>>|-------------------------------|

v     56      |     Attachment Descriptor     | T.ATT

v             |---                         ---|

v     60      |          Listhead             |

v             |-------------------------------|

v     62      |    Partition Offset to Task   | T.OFF

v             |-------------------------------|

v     64      |   SREF Count  |               |       T.SRCT

v             |-------------------------------|

v     66      |      Receive-by-Reference     | T.RRFL

v             |---                         ---|

v     70      |          Listhead             |

P$$OFF >>>>>>>>>|-------------------------------|

v     72      |    Offspring Control Block    | T.OCBH

v             |---                         ---|

v     74      |          Listhead             |

v             |-------------------------------|

v     76      |  Outstanding Offspring Count  | T.RDCT

>>>>>>>>>>>>>>|===============================|

100                                       T.LGTH

T.STS Bit Masks

TS.EXE  100000  Task not in execution                   (1=yes)

TS.RDN  40000   I/O rundown in progress                 (1=yes)

TS.MSG  20000   Abort message being output              (1=yes)

TS.NRP  10000   Task mapped to nonresident partition    (1=yes)

TS.RUN  4000    Task running on another processor       (1=yes)

TS.OUT  400     Task is out of memory                   (1=yes)

TS.CKP  200     Task is checkpointed                    (1=yes)

TS.CKR  100     Task is request to be checkpointed      (1=yes)

D-10

DATA STRUCTURES

TCB

T.ST2 Bit Masks

T2.AST  100000  AST in progress                         (1=yes)

T2.DST  40000   AST recognition disabled                (1=yes)

T2.CHK  20000   Task is not checkpointable              (1=yes)

T2.CKD  10000   Checkpointing disabled                  (1=yes)

T2.SEF  4000    Task stopped for event flags            (1=yes)

T2.FXD  2000    Task fixed in memory                    (1=yes)

T2.TIO  1000    Task is engaged in terminal I/O         (1=yes)

T2.CAF  400     Dynamic checkpoint allocation failed    (1=yes)

T2.HLT  200     Task is being halted (exiting)          (1=yes)

T2.ABO  100     Task is marked for abort                (1=yes)

40      Saved T2.STP during AST

T2.STP  20      Task stopped                            (1=yes)

10      Saved T2.SPN during AST

T2.SPN  4       Task suspended                          (1=yes)

2       Saved T2.WFR during AST

T2.WFR  1       Task is in waitfor state                (1=yes)

T.ST3 Bit Masks

T3.ACP  100000  Task is an ACP (/AC)                    (1=yes)

T3.PMD  40000   Dump task with PMD on abort             (0=yes)

T3.REM  20000   Remove task on exit                     (1=yes)

T3.PRV  10000   Task is privileged (/PR)                (1=yes)

T3.MCR  4000    Task requested as external MCR function (1=yes)

T3.SLV  2000    Task is a slave task (/SL)              (1=yes)

T3.CLI  1000    Task is a command line interpreter      (1=yes)

T3.RST  400     Task is restricted                      (1=yes)

T3.NSD  200     Task does not allow send data (/-SE)    (1=yes)

T3.CAL  100     Task has checkpoint space in task image (1=yes)

T3.ROV  40      Task has resident overlays              (1=yes)

T3.NET  20      Network protocol level

D-11

DATA STRUCTURES

OCB

D.3  OCB

OCB's (Offspring Control Blocks) are used for task  spawning.   An  OCB  is

created  whenever  a  parent  spawns or connects to a child task.  The OCB's are

destroyed whenever the parent or the child task exits.

OCB's are link to the spawned (child) task and not the parent.   All  OCB's

for a task are linked together using the O.LNK word.  The listhead pointer is in

the spawned task TCB at T.OCBH.

OCB References:

o  (O.LNK ) Next OCB for the task

o  (O.MCRL) MCR command line (from spawn directive)

o  (O.PTCB) Parent task TCB

References to OCB's:

o  (T.OCBH) TCB, listhead for OCB chain

Macro: PKTDF$   CDA Switch: none        Size: 34 bytes (14 words)

|===============================|

00      |        OCB Link Word          | O.LNK

|-------------------------------|

02      |   MCR Command Line Address    | O.MCRL

|-------------------------------|

04      |   Parent Task TCB Address     | O.PTCB

|-------------------------------|

06      |      Exit AST Address         | O.AST

|-------------------------------|

10      |     Exit Event Flag Number    | O.EFN

|-------------------------------|

12      |   Exit Block Virtual Address  | O.ESB

|-------------------------------|

14      |                               | O.STAT

|---                         ---|

16      |                               |

|---                         ---|

20      |                               |

|---                         ---|

22      |                               |

|---    Exit Status Buffer   ---|

24      |                               |

|---                         ---|

26      |                               |

|---                         ---|

30      |                               |

|---                         ---|

32      |                               |

|===============================|

34                                        O.LGTH

D-12

DATA STRUCTURES

AST CONTROL BLOCK

D.4  AST CONTROL BLOCK

AST Control Blocks are used to declare AST's to a task.  The control blocks

are  always preallocated and come from the system pool.  For the specified AST's

(Receive AST, Receive-By-Reference, FPP AST, and  Powerfail),  one  AST  control

block  is allocated and linked to the task through the T.SAST offset in the TCB.

For other AST's, the executive reuses the associated data structure as  the  AST

Control  Block.   For  QIO  AST's, this is the I/O Packet.  Similarly, mark time

AST's use the clock queue entry and spawned task AST's use  the  OCB.   Finally,

terminal  I/O  uses  a  special  structure  from the driver pool for unsolicited

character AST's.

AST's which are pending for a task are queued together.   The  listhead  is

T.ASTL  in  the TCB.  The specified AST's are queued together through the T.SAST

offset in the TCB.  Note, these are not pending.

AST Control Blocks do not reference other data structures.  They are linked

together using the first word.

The A.CBL word determines the type and length of  the  AST  Control  Block.

This word is coded as follows:

o  If A.CBL = 0, the AST Control Block is to be deallocated by the dequeue

subroutine  pointed  to A.DQSR and mapped via the APR5 value in A.KSR5.

This is current only  used  by  the  full-duplex  terminal  driver  for

unsolicited character AST's.

o  If the low byte of A.CBL is 0 and the high byte is non-zero,  then  the

AST  is  a  speciable  AST.  The high byte is the type code and the AST

Control Block as a length of C.LGTH (clock queue).

AS.FPA  1       FPP AST

AS.RCA  2       Receive AST

AS.RRA  3       Receive-by-Reference AST

AS.PFA  4       Powerfail AST

o  If the high byte of A.CBL is 0 and the low byte is positive,  then  the

low byte is the size of the AST Control Block.

o  If the high byte of A.CBL is 0 and the low byte is 200, the AST Control

Block is for buffered I/O completion and has a length of I.LGTH.

o  If the high byte of A.CBL is 0 and the low byte is 300, the AST Control

Block is an OCB and has a length of O.LGTH.

D-13

DATA STRUCTURES

AST CONTROL BLOCK

AST Control Block

Macro: PKTDF$   CDA Switch: /ACT        Size: variable (see A.CBL)

|===============================|

-4      |     Subroutine APR5 Bias      | A.KSR5

|-------------------------------|

-2      |   Dequeue Subroutine Address  | A.DQSR

|-------------------------------|

00      |     Link to Next AST Block    |

|-------------------------------|

02      | AST Control Block Length/Type | A.CBL

|-------------------------------|

04      |   # of Bytes for Task Stack   | A.BYT

|-------------------------------|

06      |        AST Trap Address       | A.AST

|-------------------------------|

10      |       # of AST Parameters     | A.NPR

|-------------------------------|

12      |       First AST Parameter     | A.PRM

|-------------------------------|

14                      .

.

. (Rest of Block)

.

.

D-14

DATA STRUCTURES

SEND/RECEIVE PACKET

D.5  SEND/RECEIVE PACKET

Send/Receive Packets are used to send messages  between  tasks.   They  are

created  in  system pool when a SEND$ directive is issued and destroyed when the

receiving task exits or receives them.

Send/Receive Packets  are  linked  together  thought  word  0  and  to  the

receiving task at T.RCVL in the TCB.

Macro: none     CDA Switch: /ACT        Size: 44 bytes (18 words)

|===============================|

00      |    Link to Next S/R Packet    |

|-------------------------------|

02      |                               |

|---     Sender Task Name    ---|

04      |                               |

|-------------------------------|

06      |                               |

|---                         ---|

10      |                               |

|---                         ---|

12      |                               |

|---                         ---|

14      |                               |

|---                         ---|

16      |                               |

|---                         ---|

20      |                               |

|---                         ---|

22      |      Send Data Message        |

|---                         ---|

24      |                               |

|---                         ---|

26      |                               |

|---                         ---|

30      |                               |

|---                         ---|

32      |                               |

|---                         ---|

34      |                               |

|---                         ---|

36      |                               |

|-------------------------------|

40      |    Sender TI: UCB Address     |

|-------------------------------|

42      |      Sender Current UIC       |

|===============================|

44                                        I.LGTH

D-15

DATA STRUCTURES

RECEIVE-BY-REFERENCE PACKET

D.6  RECEIVE-BY-REFERENCE PACKET

Receive-by-reference packets are used to send messages between tasks.  They

are  created  in system pool when a SREF$ directive is issued and destroyed when

the receiving task exits or receives them.

Receive-by-reference packets are linked together thought word 0 and to  the

receiving task at T.RRFL in the TCB.

Macro: none     CDA Switch: /ACT        Size: 44 bytes (18 words)

|===============================|

00      |  Link to Next R-by-R Packet   |

|-------------------------------|

02      |    TCB Address/0 if no EFN    |

|-------------------------------|

04      |   EFN Mask/Sender Task Name   |

|-------------------------------|

06      | EFN Address/Sender Task Name  |

|-------------------------------|

10      | Region Attachment Descriptor  |

|-------------------------------|

12      |        Region Offset          |

|-------------------------------|

14      |         Region Size           |

|-------------------------------|

16      |        Access Status          |

|-------------------------------|

20      |                               |

|---                         ---|

22      |                               |

|---                         ---|

24      |                               |

|---                         ---|

26      |                               |

|---  Optional Data Message  ---|

30      |                               |

|---                         ---|

32      |                               |

|---                         ---|

34      |                               |

|---                         ---|

36      |                               |

|-------------------------------|

40      |                               |

|-------------------------------|

42      |                               |

|===============================|

44                                        I.LGTH

D-16

DATA STRUCTURES

GROUP GLOBAL EVENT FLAGS

D.7  GROUP GLOBAL EVENT FLAGS

Group global event flags are an extension to the  event  flags  that  allow

each UIC group to have global event flags (65-96).  The is one control block for

each UIC group.  The control blocks are created and dstroyed using directives.

The group global event flags are linked together using offset  G.LNK.   The

listhead  pointer  is  $GGEF  in  module SYSCM.  They do not reference any other

structures.

Macro: PKTDF$   CDA Switch: none        Size: 12 bytes (5 words)

|===============================|

00      |      Link to Next Block       | G.LNK

|-------------------------------|

02      |    Status     |  Group Number | G.GRP  G.STAT

|-------------------------------|

04      |         Access Count          | G.CNT

|-------------------------------|

06      |                               | G.EFLG

|---       Event Flags       ---|

10      |                               |

|===============================|

12

D-17

DATA STRUCTURES

TASK HEADER

D.8  TASK HEADER

Task headers hold the initial and current context of a task.  Task  Headers

are always located in system pool and are created whenever a task is loaded into

memory.   The  task  headers  are  destroyed  whenever  the  task  exits  or  is

checkpointed.

Task headers are not linked together.  The address of the task  header  for

the task current executing in a partition is stored offset P.PCB of the PCB.

Task Header References:

o  (H.PFVA) Powerfail AST Control Block

o  (H.FPVA) FPP AST Control Block

o  (H.RCVA) Receive AST Control Block

o  (H.FCS ) FCS impure area

o  (H.FORT) Fortran impure area

o  (H.OVLY) Overlay impure area

o  (H.VEXT) Work area extension vector

o           UCB address of assigned devices

o           Window blocks

o  (W.BATT) Attachment decsriptors

References to Task Headers:

o  ($HEADR) Current task header

o  (P.HDR ) PCB, current task header

D-18

DATA STRUCTURES

TASK HEADER

Task Header

Macro: TCBDF$   CDA Switch: /HDR        Size: Variable (see H.HDLN)

|===============================|

00      |    Current Stack Pointer      | H.CSP

|-------------------------------|

02      |    Header Length (bytes)      | H.HDLN

|-------------------------------|

04      |     Event Flag Mask Word      | H.EFLM

|---                         ---|

06      |    Event Flag Mask Address    |

|-------------------------------|

10      |      Current Task UIC         | H.CUIC

|-------------------------------|

12      |      Default Task UIC         | H.DUIC

|-------------------------------|

14      |        Initial PS             | H.IPS

|-------------------------------|

16      |        Initial PC             | H.IPC

|-------------------------------|

20      |        Initial SP             | H.ISP

|-------------------------------|

22      |  Debugging SST Vector Address | H.ODVA

|-------------------------------|

24      |  Debugging SST Vector Length  | H.ODVL

|-------------------------------|

26      |    Task SST Vector Address    | H.TKVA

|-------------------------------|

30      |    Task SST Vector Length     | H.TKVL

|-------------------------------|

32      |  Powerfail AST Control Block  | H.PFVA

|-------------------------------|

34      |     FPP AST Control Block     | H.FPVA

|-------------------------------|

36      |   Receive AST Control Block   | H.RCVA

|-------------------------------|

40      |    Event Flag Save Address    | H.EFSV

|-------------------------------|

42      | Pointer to FPP/EAE Save Area  | H.FPSA

|-------------------------------|

44      |  Pointer to Start of Windows  | H.WND

|-------------------------------|

46      |  Task Directive Status ($DSW) | H.DSW

|-------------------------------|

50      |   FCS Impure Area Pointer     | H.FCS

|-------------------------------|

continued on next page

D-19

DATA STRUCTURES

TASK HEADER

Task Header (continued)

|-------------------------------|

52      |  Fortran OTS Impure Pointer   | H.FORT

|-------------------------------|

54      |  Overlay Impure Area Pointer  | H.OVLY

|-------------------------------|

56      |  Work Area Extension Pointer  | H.VEXT

|-------------------------------|

60      |  Mailbox LUN  | Swapping Pri. | H.SPRI H.NML

|-------------------------------|

62      |   Rec/Ref AST Control Block   | H.RRVA

|-------------------------------|

64      |                               |

|-------------------------------|

66      |                               |

|-------------------------------|

70      |                               |

|-------------------------------|

72      | Pointer to Header Guard Word  | H.GARD

|-------------------------------|

74      |       Number of LUN's         | H.NLUN

|-------------------------------|

76      |      LUN 1 UCB Address        | H.LUN

|-------------------------------|

100     |      LUN 1 Window Block     |L|

|-------------------------------|

.

.

. (Lun Table)

.

.

continued on next page

D-20

DATA STRUCTURES

TASK HEADER

Task Header (continued)

|-------------------------------|

|    Number of Window Blocks    |

|-------------------------------|

|          PCB Address          | W.BPCB

|-------------------------------|

|   Low Virtual Address Limit   | W.BLVR

|-------------------------------|

|   Hish Virtual Address Limit  | W.BHVR

|-------------------------------|

| Attachment Descriptor Address | W.BATT

|-------------------------------|

|  Size of Window (32W Blocks)  | W.BSIZ

|-------------------------------|

|  Physical Memory Offset (32W) | W.BOFF

|-------------------------------|

|  Number PDR's |   First PDR   | W.BFPD W.BNPD

|-------------------------------|

|    Contents of Last PDR       | W.BLPD

|-------------------------------|

.                 W.LGTH

.

. (Window Blocks)

.

.

|-------------------------------|

|          Current PS           |

|-------------------------------|

|          Current PC           |

|-------------------------------|

|          Current R5           |

|-------------------------------|

|          Current R4           |

|-------------------------------|

|          Current R3           |

|-------------------------------|

|          Current R2           |

|-------------------------------|

|          Current R1           |

|-------------------------------|

|          Current R0           |

|-------------------------------|

|      Header Guard Word        |

|===============================|

D-21

DATA STRUCTURES

PCB

D.9  PCB

PCB's (Partition Control Blocks) are the main RSX-11M  data  structure  for

defining memory.  The is one PCB for every entity loaded into memory (except for

the executive).  Empty space in a partition is indicated by the gap between  two

PCB's.   PCB's  are  created  in  many  areas, for example, by MCR when defining

partitions or by the executive when loading a task into a system partition.

All main PCB's are linked together using the P.LNK offset in the PCB.   The

chain  listhead  is  $PARHD  in  module  SYSCM.   Subpartition PCB's are chained

together using the P.SUB  offset.   The  chain  header  is  P.SUB  in  the  main

partition PCB.

PCB References:

o  (P.LNK ) Next main partition PCB

o  (P.SUB ) Next subpartition PCB

o  (P.MAIN) Main partition PCB for this subpartition

o  (P.WAIT) TCB's waiting for load into partition

o  (P.TCB ) TCB of task currently loaded in partition

o  (P.HDR ) Task Header of task currently loaded in partition

o  (P.ATT ) Attachment Descriptors for task attach to partition

References to PCB's

o  ($PARHD) PCB chain listhead

o  (T.PCB ) Task PCB address

o  (A.PCB ) Attachment descriptor, PCB of owning task

o  (D.PCB ) DCB, device driver PCB

D-22

DATA STRUCTURES

PCB

PCB

Macro: PCBDF$   CDA Switch: /PAR        Size: 42 bytes (17 words) w/  PLAS

34 bytes (14 words) w/o PLAS

|===============================|

00      |      Link to Next PCB         | P.LNK

|-------------------------------|

02      |   I/O Count   |   Priority    | P.PRI  P.IOC

|-------------------------------|

04      |                               | P.NAM

|---  Partition Name (RAD50) ---|

06      |                               |

|-------------------------------|

10      |  Pointer to Next Subpartition | P.SUB

|-------------------------------|

12      | Pointer to Main Partition PCB | P.MAIN

|-------------------------------|

14      |   Starting Physical Address   | P.REL

|-------------------------------|

16      |     Size of Partition (32W)   | P.SIZE

|-------------------------------|

20      | Partition Wait Queue Listhead | P.WAIT

|-------------------------------|

22      |     Partition Swap Size       | P.SWSZ

|-------------------------------|

24      |     Partition Busy Flags      | P.BUSY

|-------------------------------|

26      |   TCB Address of Owner Task   | P.TCB

|-------------------------------|

30      |    Partition Status Flags     | P.STAT

|-------------------------------|

32      |    Address of Task Header     | P.HDR

P$$LAS >>>>>>>>>|-------------------------------|

v     34      |       Protection Word         | P.PRO

v             |-------------------------------|

v     36      |     Attachment Descriptor     | P.ATT

v             |---                         ---|

v     40      |           Listhead            |

>>>>>>>>>>>>>>|===============================|

42                                        P.LGTH

D-23

DATA STRUCTURES

PCB

P.STAT Masks

PS.OUT  100000  Partition is out of memory              (1=yes)

PS.CKP  40000   Partition checkpoint in progress        (1=yes)

PS.CKR  20000   Partition checkpoint requested          (1=yes)

PS.CHK  10000   Partition is not checkpointable         (1=yes)

PS.FXD  4000    Partition is fixed                      (1=yes)

PS.PER  2000    Parity error in partition               (1=yes)

PS.LIO  1000    Partition marked by SHF for long I/O    (1=yes)

PS.NSF  400     Partition is non shuffleable            (1=yes)

PS.COM  200     Library or common block                 (1=yes)

PS.PIC  100     Position independent library or common  (1=yes)

PS.SYS  40      System-controlled partition             (1=yes)

PS.DRV  20      Device Driver loaded in partition       (1=yes)

PS.DEL  10      Delete partition when not attached      (1=yes)

PS.APR  7       Starting APR number

D-24

DATA STRUCTURES

ATTACHMENT DESCRIPTOR

D.10  ATTACHMENT DESCRIPTOR

Attachment descriptors are used to map partitions and the tasks  which  are

attached to them.  In particular, attachment descriptors map dynamic regions and

task which reference them using the PLAS directives.

Attachment descriptors are linked with two separate threads.  The first  is

all  attachment  descriptors  for the partition.  This thread uses offset A.PCBL

and the listhead is P.ATT in the PCB.  The second is all attachment  descriptors

for  a  task.   This  thread uses offset A.TCBL and the listhead is T.ATT in the

PCB.

Attachment Descriptor References:

o  (A.PCBL) Next attachment descriptor in the PCB chain

o  (A.TCB ) TCB address of owning task

o  (A.TCBL) Next attachment descriptor in the TCB chain

o  (A.PCB ) PCB address of owning task

References to Attachment Descriptors:

o  (T.ATT ) TCB, first attachment descriptor for task

o           Receive-by-reference, attachment descriptor for region

o  (W.BATT) Task header, window block attachment descriptor

o  (P.ATT ) PCB, first attachment descriptor for partition

Macro: PCBDF$   CDA Switch: /PAR        Size: 14 bytes (6 words)

|===============================|

00      |   PCB Attachment Thread Word  | A.PCBL

|-------------------------------|

02      |   I/O Count   |   Priority    | A.PRI  A.IOC

|-------------------------------|

04      |  TCB Address of Attached Task | A.TCB

|-------------------------------|

06      |   TCB Attachment Thread Word  | A.TCBL

|-------------------------------|

10      | Mapping Count |    Status     | A.STAT A.MPCT

|-------------------------------|

12      |  PCB Address of Attached Task | A.PCB

|===============================|

14                                        A.LGTH

A.STAT Masks

AS.DEL  10      Task has deleted access                 (1=yes)

AS.EXT  4       Task has extend access                  (1=yes)

AS.WRT  2       Task has write access                   (1=yes)

AS.RED  1       Task has read access                    (1=yes)

D-25

DATA STRUCTURES

CLOCK QUEUE

D.11  CLOCK QUEUE

The Clock Queue is used for all time-related  events.   The  queue  entries

take  a  variety  of  formats,  depending  on  the  type  of  entry:  mark-time,

single-shot scheduling request, periodic scheduling request, or internal  system

request.

All Clock Queue entries are threaded together using the C.LNK offset.   The

listhead  is  $CLKHD  in module SYSCM.  The entries are ordered by the time they

come due.

Clock Queue References:

o  (C.LNK ) Next Clock Queue entry

o  (C.TCB ) TCB address

References to Clock Queue:

o  ($CLKHD) Clock Queue listhead

D-26

DATA STRUCTURES

CLOCK QUEUE

Clock Queue (Mark Time)

Macro: CLKDF$   CDA Switch: /CLQ        Size: 20 bytes (8 words)

|===============================|

00      |     Clock Queue Link Word     | C.LNK

|-------------------------------|

02      |   Event Flag  |   Type (0)    | C.RQT  C.EFN

|-------------------------------|

04      |         TCB Address           | C.TCB

|-------------------------------|

06      |     Absolute Time Request     | C.TIM

|---                         ---|

10      |         Becomes Due           |

|-------------------------------|

12      |     Mark Time AST Address     | C.AST

|-------------------------------|

14      |     Event Flag Mask Word      | C.SRC

|-------------------------------|

16      |    Event Flag Mask Address    | C.DST

|===============================|

20                                        C.LGTH

Clock Queue (Single-Shot Scheduling)

Macro: CLKDF$   CDA Switch: /CLQ        Size: 20 bytes (8 words)

|===============================|

00      |     Clock Queue Link Word     | C.LNK

|-------------------------------|

02      |               |  Type (4)     | C.RQT  C.EFN

|-------------------------------|

04      |         TCB Address           | C.TCB

|-------------------------------|

06      |     Absolute Time Request     | C.TIM

|---                         ---|

10      |         Becomes Due           |

|-------------------------------|

12      |                               |

|-------------------------------|

14      |                               |

|-------------------------------|

16      |        Scheduling UIC         | C.UIC

|===============================|

20                                        C.LGTH

D-27

DATA STRUCTURES

CLOCK QUEUE

Clock Queue (Periodic Scheduling Request)

Macro: CLKDF$   CDA Switch: /CLQ        Size: 20 bytes (8 words)

|===============================|

00      |     Clock Queue Link Word     | C.LNK

|-------------------------------|

02      |               |  Type (2)     | C.RQT  C.EFN

|-------------------------------|

04      |         TCB Address           | C.TCB

|-------------------------------|

06      |     Absolute Time Request     | C.TIM

|---                         ---|

10      |         Becomes Due           |

|-------------------------------|

12      |                               | C.RSI

|---   Reschedule Interval   ---|

14      |                               |

|-------------------------------|

16      |        Scheduling UIC         | C.UIC

|===============================|

20                                        C.LGTH

Clock Queue (Internal)

Macro: CLKDF$   CDA Switch: /CLQ        Size: 20 bytes (8 words)

|===============================|

00      |     Clock Queue Link Word     | C.LNK

|-------------------------------|

02      |               |  Type (6/8)   | C.RQT  C.EFN

|-------------------------------|

04      |         TCB Address           | C.TCB

|-------------------------------|

06      |     Absolute Time Request     | C.TIM

|---                         ---|

10      |         Becomes Due           |

|-------------------------------|

12      |       Subroutine Address      | C.SUB

|-------------------------------|

14      |      APR5 Relocation Base     | C.AR5

|-------------------------------|

16      |                               |

|===============================|

20                                        C.LGTH

D-28

DATA STRUCTURES

I/O PACKET

D.12  I/O PACKET

I/O packets are used to move  the  context  of  an  I/O  request  from  the

executive  to  device drivers and ACP's.  An I/O packet is created in the system

pool whenever a task issues a QIO or QIOW directive and is  destroyed  when  the

I/O request is finished.

I/O packets are threaded together using  the  I.LNK  offset.   Packets  are

queued  to  devices  using  the  S.LHD offset of the SCB.  Packets are queued to

ACP's using the T.RCVL offset of the ACP TCB.

I/O Packet References:

o  (I.LNK ) Next I/O Packet in queue

o  (I.TCB ) TCB of issueing task

o  (I.LN2 ) Second LUN word in task header

o  (I.UCB ) UCB address of device

References to I/O Packets:

o  (T.RCVL) TCB, Pointer to first I/O packet in ACP queue

o  (S.LHD ) SCB, Pointer to first I/O packet in device queue

o  (S.PKT ) SCB, current I/O packet

D-29

DATA STRUCTURES

I/O PACKET

I/O Packet

Macro: PKTDF$   CDA Switch: /DEV        Size: 44 bytes (18 words)

|===============================|

00      |    Link to Next I/O Packet    | I.LNK

|-------------------------------|

02      |   Event Flag  |   Priority    | I.PRI  I.EFN

|-------------------------------|

04      | TCB Address of Issueing Task  | I.TCB

|-------------------------------|

06      |  Pointer to Second Lun Word   | I.LN2

|-------------------------------|

10      |          UCB Address          | I.UCB

|-------------------------------|

12      |       I/O Function Code       | I.FCN

|-------------------------------|

14      |   I/O Status Virtual Address  | I.IOSB

|-------------------------------|

16      |   I/O Status Relocation Bias  |

|-------------------------------|

20      |  I/O Status Relocation Offset |

|-------------------------------|

22      |  AST Service Virtual Address  | I.AST

|-------------------------------|

24      |                               | I.PRM

|---                         ---|

26      |                               |

|---                         ---|

30      |                               |

|---                         ---|

32      |                               |

|---     I/O Parameters      ---|

34      |                               |

|---                         ---|

36      |                               |

|---                         ---|

40      |                               |

|---                         ---|

42      |                               |

|===============================|

44                                        I.LGTH

D-30

DATA STRUCTURES

LCB

D.13  LCB

LCB's (Logical Device Assignment Control Block) are  used  to  map  logical

device names to physical device names.  LCB's are located in system pool and are

manipulated by the ASN command.

LCB's are threaded together using the offset L.LNK.   The  list  header  is

$LOGHD in module SYSCM.

LCB References:

o  (L.LNK ) Next LCB in chain

o  (L.UCB ) TI:  UCB

o  (L.ASG ) Phyical device UCB

References to LCB's:

o  ($LOGHD) LCB chain lisheader

Macro: LCBDF$   CDA Switch: none        Size: 12 bytes (5 words)

|===============================|

00      |      Link to Next LCB         | L.LNK

|-------------------------------|

02      |   Logical Device Name (ASCII) | L.NAM

|-------------------------------|

04      |  Global Flag  |  Logical Unit | L.UNIT L.TYPE

|-------------------------------|

06      |        TI: UCB Address        | L.UCB

|-------------------------------|

10      |      Physical Device UCB      | L.ASG

|===============================|

12                                        L.LGTH

D-31

DATA STRUCTURES

ITB

D.14  ITB

ITB's (Interrupt Transfer Blocks)  are  used  to  map  an  interrupt  to  a

loadable  driver  or  connected  task.   The  system  uses three types of ITB's:

single controller, multiple controller, and connect-to-interrupt.  The first two

are created by LOA when a loadable device driver is loaded.  The last is used by

the executive to service the connect-to-interrupt feature.

Only connect-to-interrupt ITB's are threaded together using  offset  X.LNK.

The listhead for all ITB's belonging to a task is T.CPCB in the task TCB.

ITB (Single Controller)

Macro: none     CDA Switch: none        Size: 34 bytes (14 words)

|===============================|

00      |                               |

|---     JSR R5,@#INTSV      ---|

02      |                               |

|-------------------------------|

04      |        Device Priority        |

|-------------------------------|

06      |                               |

|---    MOV @#APR5,-(SP)     ---|

10      |                               |

|-------------------------------|

12      |                               |

|---                         ---|

14      |     MOV #mapping,@#APR5       |

|---                         ---|

16      |                               |

|-------------------------------|

20      |                               |

|---  JSR PC,@#driver entry  ---|

22      |                               |

|-------------------------------|

24      |                               |

|---    MOV (SP)+,@#APR5     ---|

26      |                               |

|-------------------------------|

30      |            RTS PC             |

|-------------------------------|

32      |            Unused             |

|===============================|

34

D-32

DATA STRUCTURES

ITB

ITB (Multiple Controller)

Macro: none     CDA Switch: none        Size: 54 bytes (22 words)

|===============================|

00      |                               |

|---                         ---|

02      |        MOVB @#PS,TEMP         |

|---                         ---|

04      |                               |

|-------------------------------|

06      |                               |

|---     JSR R5,@#INTSV      ---|

10      |                               |

|-------------------------------|

12      |        Device Priority        |

|-------------------------------|

14      |                               |

|---      MOV TEMP,R4        ---|

16      |                               |

|-------------------------------|

20      |                               |

|---     BIC #177760,R4      ---|

22      |                               |

|-------------------------------|

24      |           ASL R4              |

|-------------------------------|

26      |                               |

|---    MOV @#APR5,-(SP)     ---|

30      |                               |

|-------------------------------|

32      |                               |

|---                         ---|

34      |     MOV #mapping,@#APR5       |

|---                         ---|

36      |                               |

|-------------------------------|

40      |                               |

|---  JSR PC,@#driver entry  ---|

42      |                               |

|-------------------------------|

44      |                               |

|---    MOV (SP)+,@#APR5     ---|

46      |                               |

|-------------------------------|

50      |            RTS PC             |

|-------------------------------|

52      |            TEMP:              |

|===============================|

54

D-33

DATA STRUCTURES

ITB

ITB (Connect-to-Interrupt)

Macro: ITBDF$   CDA Switch: none        Size: 50 bytes (20 words)

|===============================|

00      |        Link to Next ITB       | X.LNK

|-------------------------------|

02      |                               | X.JSR

|---      JSR R5,@#INTSC     ---|

04      |                               |

|-------------------------------|

06      |               |     PSW       | X.PSW

|-------------------------------|

10      |       ISR Entry Point         | X.ISR

|-------------------------------|

12      |       Fork Thread Word        | X.FORK

|-------------------------------|

14      |            Fork PC            |

|-------------------------------|

16      |            Fork R5            |

|-------------------------------|

20      |            Fork R4            |

|-------------------------------|

22      |   Relocation Base for APR5    | X.REL

|-------------------------------|

24      |  Enable/Disable Entry Point   | X.DSI

|-------------------------------|

26      |      TCB of Owning Task       | X.TCB

|-------------------------------|

30      |    AST Dequeue Subroutine     |

|-------------------------------|

32      |    AST Queue Thread Word      | X.AST

|-------------------------------|

34      |       AST Control Word        |

|-------------------------------|

36      |   # of Bytes for Task Stack   |

|-------------------------------|

40      |        AST Trap Address       |

|-------------------------------|

42      |       # of AST Parameters     |

|-------------------------------|

44      |        Vector Address         | X.VEC

|-------------------------------|

46      |        Saved Vector PC        | X.VPC

|===============================|

50                                        X.LEN

D-34

DATA STRUCTURES

DCB

D.15  DCB

DCB's (Device Control Block) are used to define generic devices.  The DCB's

are always found in executive space, either SYSTB for resident drivers or system

pool for loadable drivers with loadable databases.

DCB's are chained together using the D.LNK offset.  The listhead pointer is

$DEVHD in module SYSCM.

DCB References:

o  (D.LNK ) Next DCB in device chain

o  (D.UCB ) First UCB of device

o  (D.DSP ) Device driver dispatch table

o  (D.PCB ) Device driver PCB address

Refereences to DCB's

o  ($DEVHD) Pointer to first DCB in device chain

o  (U.DCB ) UCB, pointer to DCB for UCB

D-35

DATA STRUCTURES

DCB

DCB

Macro: DCBDF$   CDA Switch: /DEV        Size: 34 (14 words)

|===============================|

00      |       Link to Next DCB        | D.LNK

|-------------------------------|

02      |       First UCB Address       | D.UCB

|-------------------------------|

04      |         Device Name           | D.NAM

|-------------------------------|

06      |  High Unit #  |   Low Unit #  | D.UNIT

|-------------------------------|

10      |       Length of an UCB        | D.UCBL

|-------------------------------|

12      | Driver Dispatch Table Address | D.DSP

|-------------------------------|

14      |   Legal Function Mask (0-15)  | D.MSK

|-------------------------------|

16      |  Control Function Mask (0-15) |

|-------------------------------|

20      |  NOP'ed Function Mask (0-15)  |

|-------------------------------|

22      |    ACP Function Mask (0-15)   |

|-------------------------------|

24      |   Legal Function Mask (16-31) |

|-------------------------------|

26      | Control Function Mask (16-31) |

|-------------------------------|

30      |  NOP'ed Function Mask (16-31) |

|-------------------------------|

32      |    ACP Function Mask (16-31)  |

L$$DRV>>>>>>>>>>|-------------------------------|

v     34      |       Driver PCB Address      | D.PCB

>>>>>>>>>>>>>>|===============================|

D-36

DATA STRUCTURES

UCB

D.16  UCB

UCB's (Unit Control Block) are the  major  device-related  data  structure.

The  UCB  holds  all  information related to a device unit, there is one UCB for

each separate unit.

The UCB's belonging to a particular device (DCB) are contigious in  memory.

The offset D.UCB in the DCB points to the first UCB.

UCB References:

o  (U.CLI ) TCB of CLI

o  (U.OWN ) UCB of owning TI:

o  (U.DCB ) DCB of corresponding device

o  (U.RED ) UCB or redirected device

o  (U.SCB ) SCB of corresponding device controller

References to UCB's:

o  ($COPT ) UCB address of CO:  device

o  (T.UCB ) TCB, task TI:  UCB address

o  (T.LDV ) TCB, task load device UCB address

o           S/R message, sending task TI:  UCB address

o           Task header, UCB address of assigned devices

o  (I.UCB ) I/O packet, UCB address of destination device

o  (L.ASG ) LCB, UCB address of physical device

o  (D.UCB ) DCB, address of first UCB

o           Mounted Volume, TI:  UCB address

o           Mounted Volume, disk UCB address

o  (V.UCB ) MTAACP VCB, magtape UCB address

D-37

DATA STRUCTURES

UCB

UCB (standard)

Macro: UCBDF$   CDA Switch: /DEV        Size: variable, see D.UCBL

M$$MUP >>>>>>>>>|===============================|

v     -2      |   UCB Address of Owner TT:    | U.OWN

>>>>>>>>>>>>>>|-------------------------------|

00      |          DCB Address          | U.DCB

|-------------------------------|

02      |      Redirect UCB Address     | U.RED

|-------------------------------|

04      |  Unit Status  | Control Flags | U.CTL  U.STS

|-------------------------------|

06      |  Unit Status  |  Unit Number  | U.UNIT U.ST2

|-------------------------------|

10      |    Characteristics Word #1    | U.CW1

|-------------------------------|

12      |    Characteristics Word #2    | U.CW2

|-------------------------------|

14      |    Characteristics Word #3    | U.CW3

|-------------------------------|

16      |    Characteristics Word #4    | U.CW4

|-------------------------------|

20      |          SCB Address          | U.SCB

|-------------------------------|

22      |      TCB of Attached Task     | U.ATT

|-------------------------------|

24      |                               | U.BUF

|---      Buffer Address     ---|

26      |                               |

|-------------------------------|

30      |          Byte Count           | U.CNT

|===============================|

U.CTL Masks

UC.ALG  200     Byte alignment allowed                  (0=yes)

UC.NPR  100     Device is NPR device                    (1=yes)

UC.QUE  40      Call driver before queueing             (1=yes)

UC.PWF  20      Call driver always at powerfail         (1=yes)

UC.ATT  10      Call driver always at attach/detach     (1=yes)

UC.KIL  4       Call driver always at I/O kill          (1=yes)

UC.LGH  3       Transfer length mask

D-38

DATA STRUCTURES

UCB

U.STS Masks

US.BSY  200     Unit is busy                            (1=yes)

US.MNT  100     Unit is mounted                         (0=yes)

US.FOR  40      Unit is mounted as foreign              (1=yes)

US.MDM  20      Unit is marked for dismount             (1=yes)

US.PWF  10      Powerfail occured                       (1=yes)

4       Device specific

2       Device specific

1       Device specific

Card reader specific status bits

US.MDE  2       Unit is in 029 translation mode         (1=yes)

US.ABO  1       Unit is marked for abort if not ready   (1=yes)

Files-11 specific status bits

US.WCK  10      Write check enabled                     (1=yes)

US.SPU  2       Unit is spinning up                     (1=yes)

US.VV   1       Volume valid is set                     (1=yes)

KMC-11-LP specific status bits

US.KPF  1       KMC-11 powerfail interlock

Full-duplex terminal driver specific status bits

US.CRW  4       Unit is waiting for carrier             (1=yes)

US.DSB  2       Unit is disabled                        (1=yes)

US.OIU  1       Output interrupt is unexpected          (1=yes)

Half-dupluex terminal driver specific status bits

US.DSB  10      Unit is disabled                        (1=yes)

US.CRW  4       Unit is waiting for carrier             (1=yes)

US.ECH  2       Unit has no echo in progress            (1=yes)

US.OUT  1       Unit is expecting output interrupt      (1=yes)

LPS11 driver specific status bits

US.FRK  2       Fork in progress                        (1=yes)

US.SHR  1       Shareable function in progress          (1=yes)

Magtape driver specific status bits

US.LAB  4       Unit has labeled tape on it             (1=yes)

US.BSP  2       Internal backspace in progress          (1=yes)

D-39

DATA STRUCTURES

UCB

U.ST2 Masks

US.UMD  10      Unit attached for diagnostics           (1=yes)

US.PUB  4       Unit is public device                   (1=yes)

US.RED  2       Unit is redirectable                    (0=yes)

US.OFL  1       Unit is offline                         (1=yes)

U.CW1 Masks

DV.MNT  100000  Device is mountable                     (1=yes)

DV.F11  40000   Device is mountable as Files-11         (1=yes)

DV.COM  20000   Device is mountable as communications   (1=yes)

DV.PSE  10000   Pseudo device                           (1=yes)

DV.OSP  4000    Output spooled device                   (1=yes)

DV.ISP  2000    Input  spooled device                   (1=yes)

DV.SWL  1000    Unit is software write-locked           (1=yes)

DV.MBC  400     Unit is on Massbus controller           (1=yes)

DV.UMD  200     User-mode dianostics supported          (1=yes)

DV.MXD  100     Mixed-mode Massbus device               (1=yes)

DV.SQD  40      Sequential device                       (1=yes)

DV.SDI  20      Single-directory device                 (1=yes)

DV.DIR  10      Directory device                        (1=yes)

DV.TTY  4       Terminal-like device                    (1=yes)

DV.CCL  2       Carriage control device                 (1=yes)

DV.REC  1       Record oriented device                  (1=yes)

D-40

DATA STRUCTURES

UCB

UCB (mountable)

Macro: UCBDF$   CDA Switch: /DEV        Size: variable (36 as shown)

M$$MUP >>>>>>>>>|===============================|

v     -2      |   UCB Address of Owner TT:    | U.OWN

>>>>>>>>>>>>>>|-------------------------------|

00      |          DCB Address          | U.DCB

|-------------------------------|

02      |      Redirect UCB Address     | U.RED

|-------------------------------|

04      |  Unit Status  | Control Flags | U.CTL  U.STS

|-------------------------------|

06      |  Unit Status  |  Unit Number  | U.UNIT U.ST2

|-------------------------------|

10      |    Characteristics Word #1    | U.CW1

|-------------------------------|

12      |    Characteristics Word #2    | U.CW2

|-------------------------------|

14      |    Characteristics Word #3    | U.CW3

|-------------------------------|

16      |    Characteristics Word #4    | U.CW4

|-------------------------------|

20      |          SCB Address          | U.SCB

|-------------------------------|

22      |      TCB of Attached Task     | U.ATT

|-------------------------------|

24      |                               | U.BUF

|---      Buffer Address     ---|

26      |                               |

|-------------------------------|

30      |          Byte Count           | U.CNT

|-------------------------------|

32      |      Address of ACP TCB       | U.ACP

|-------------------------------|

34      |      Address of ACP VCB       | U.VCB

|===============================|

D-41

DATA STRUCTURES

UCB

UCB (terminals)

Macro: UCBDF$   CDA Switch: /DEV        Size: variable (64 as shown)

C$$CLI >>>>>>>>>|===============================|

v     -6      |       Address of CLI TCB      | U.CLI

M$$MUP >>>>>>>>>|-------------------------------|

v     -4      |           Login UIC           | U.LUIC

v             |-------------------------------|

v     -2      |   UCB Address of Owner TT:    | U.OWN

>>>>>>>>>>>>>>|-------------------------------|

00      |          DCB Address          | U.DCB

|-------------------------------|

02      |      Redirect UCB Address     | U.RED

|-------------------------------|

04      |  Unit Status  | Control Flags | U.CTL  U.STS

|-------------------------------|

06      |  Unit Status  |  Unit Number  | U.UNIT U.ST2

|-------------------------------|

10      |    Characteristics Word #1    | U.CW1

|-------------------------------|

12      |    Characteristics Word #2    | U.CW2

|-------------------------------|

14      |    Characteristics Word #3    | U.CW3

|-------------------------------|

16      |    Characteristics Word #4    | U.CW4

|-------------------------------|

20      |          SCB Address          | U.SCB

|-------------------------------|

22      |      TCB of Attached Task     | U.ATT

|-------------------------------|

24      |                               | U.BUF

|---      Buffer Address     ---|

26      |                               |

|-------------------------------|

30      |          Byte Count           | U.CNT

|-------------------------------|

continued on next page

D-42

DATA STRUCTURES

UCB

UCB (terminals) continued

|-------------------------------|

32      |    Pointer to UCB extension   | U.TUX

|-------------------------------|

34      |                               | U.TSTA

|---                         ---|

36      |      Terminal Status Words    |

|---                         ---|

40      |                               |

|-------------------------------|

42      | Type-ahead Status and Pointer | U.TTAB

|-------------------------------|

44      | Fork Request  |  Lines/Page   | U.TLPP U.TFRQ

|-------------------------------|

46      |     Fork List Link Word       | U.TFLK

|-------------------------------|

50      | Current V-Pos | Current H-Pos | U.TCHP U.TCVP

|-------------------------------|

52      |        Terminal UIC           | U.UIC

|-------------------------------|

54      |  Modem Timer  | Terminal Type | U.TTYP U.TMTI

|-------------------------------|

56      |        Controller Type        | U.CYTP

|===============================|

U.CW2 Masks (Terminals)

U2.DH1  100000  Unit is a multiplexor                   (1=yes)

U2.DJ1  40000   Unit is a DJ11                          (1=yes)

U2.RMT  20000   Unit is remote                          (1=yes)

U2.HFF  10000   Unit handles hardware formfeeds         (1=yes)

U2.NEC  4000    Do not echo solicited input             (1=yes)

U2.CRT  2000    Unit is a CRT                           (1=yes)

U2.ESC  1000    Unit generates escape sequences         (1=yes)

U2.LOG  400     User logged onto terminal               (0=yes)

U2.SLV  200     Unit is slaved terminal                 (1=yes)

U2.DZ1  100     Unit is a DZ11                          (1=yes)

U2.HLF  40      Unit is in hold-screen mode             (1=yes)

U2.AT.  20      MCR command AT. is being processed      (1=yes)

U2.PRV  10      Unit is privileged terminal             (1=yes)

U2.L3S  4       Unit is a LA30S terminal                (1=yes)

U2.VT5  2       Unit is a VT05B terminal                (1=yes)

U2.LWC  1       Lower case to upper case conversion     (1=yes)

D-43

DATA STRUCTURES

UCB

U.TSTA+0 Masks

S1.IBF  100000  Buffered input in progress              (1=yes)

S1.USE  20000   Unsolicited input in progress           (1=yes)

S1.CTS  10000   Output stopped by control-S             (1=yes)

S1.DSI  4000    Input processing disabled               (1=yes)

S1.DEC  2000    Defer echo of character in U.TECB       (1=yes)

S1.DPR  1000    Defer processing of character in U.TECB (1=yes)

S1.BEL  400     Bell pending                            (1=yes)

S1.IBY  200     Input busy                              (1=yes)

S1.OBT  100     Output busy                             (1=yes)

S1.CTO  40      Output stopped by control-O             (1=yes)

S1.RNE  20      Echo suppressed                         (1=yes)

S1.RAL  10      Read-all in progress                    (1=yes)

S1.ESC  4       Escape sequence in progress             (1=yes)

S1.RUN  2       Rubout sequence in progress             (1=yes)

S1.RST  1       Read/special terminators in progress    (1=yes)

U.TSTA+2 Masks

S2.FDX  100000  Line is in full-duplex mode             (1=yes)

S2.FLF  40000   Force linefeed before next echo         (1=yes)

S2.HFF  20000   Hardware formfeed present               (1=yes)

S2.HHT  10000   Hardware horizontal tab present         (1=yes)

S2.VFL  4000    Vertical fill requirement               (1=yes)

S2.HFL  3400    Horizontal fill requirement             (1=yes)

S2.IRQ  200     Input request in queue                  (1=yes)

S2.ORQ  100     Output request in queue                 (1=yes)

S2.SRQ  40      Special request in queue                (1=yes)

S2.BRQ  20      Break-through-write request in queue    (1=yes)

S2.CR   10      Trailing CR required on output          (1=yes)

S2.WRB  4       Low bit in S2.WRA pattern               (1=yes)

S2.WRA  2       Context for wrap-around                 (1=yes)

S2.ACR  1       Wrap-around (auto CR/LF) required       (1=yes)

U.TSTA+4 Masks

S3.PCU  100000  Position cursor                         (1=yes)

S3.DAO  40000   Last character has data overrun error   (1=yes)

S3.BCC  20000   Last character has framing error        (1=yes)

S3.VER  10000   Last character has parity error         (1=yes)

S3.WAL  4000    Terminal is in write-pass-all mode      (1=yes)

S3.RCU  400     Restore cursor                          (1=yes)

S3.8BC  200     Pass 8 bits on input                    (1=yes)

S3.TAB  100     Type-ahead buffer allocation requested  (1=yes)

S3.WES  40      Task wants escape sequences             (1=yes)

S3.RAL  10      Terminal is in read-pass-all mode       (1=yes)

D-44

DATA STRUCTURES

SCB

D.17  SCB

SCB's (Status Control Blocks) are the controlling structure  for  a  device

controller.  It contains pointers to the device and the I/O queue listhead.

SCB's have no common link.  Each  UCB  points  to  a  SCB,  however,  if  a

controller has multiple units, multiple UCB's will point to one SCB.

SCB References:

o  (S.BMSV) EMB pointer

o  (S.LHD ) I/O packet queue listhead

o  (S.PKT ) Current I/O packet

References to SCB's

o  (U.SCB ) UCB, address of associated SCB

D-45

DATA STRUCTURES

SCB

SCB

Macro: SCBDF$   CDA Switch: /DEV        Size: Normally 26 - add 6 bytes

for error handling and 14

for 22-bit NPR devices

E$$DVC >>>>>>>>>|===============================|

v     -6      | Offset t/reg  | No. Registers | S.RCNT S.ROFF

v             |-------------------------------|

v     -4      |        Pointer to EMB         | S.BMSV

v             |-------------------------------|

v     -2      |   Device I/O Active Bit Mask  | S.BMSK

>>>>>>>>>>>>>>|-------------------------------|

00      |                               | S.LHD

|---     I/O Queue Listhead  ---|

02      |                               |

|-------------------------------|

04      |    Vector/4   | Dev. Priority | S.PRI  S.PRI

|-------------------------------|

06      |  Initial TMO  |  Current TMO  | S.CTM  S.ITM

|-------------------------------|

10      |  Cont. Status |  Cont. Index  | S.STS  S.CON

|-------------------------------|

12      | Address of Device Status Reg. | S.CSR

|-------------------------------|

14      | Address of Current I/O Packet | S.PKT

|-------------------------------|

16      |             Fork PC           | S.FRK

|-------------------------------|

20      |             Fork R5           |

|-------------------------------|

22      |             Fork R4           |

L$$DRV>>>>>>>>>>|-------------------------------|

v     24      |  Fork Driver Relocation Base  |

M$$EXT>>>>>>>>>>|-------------------------------|

v     26      |                               | S.MPR

v             |---                         ---|

v     30      |                               |

v             |---                         ---|

v     32      |   Additional Storage for      |

v             |---                         ---|

v     34      |     22-bit NPR Devices        |

v             |---                         ---|

v     36      |                               |

v             |---                         ---|

v     40      |                               |

>>>>>>>>>>>>>>|===============================|

42

D-46

DATA STRUCTURES

MAPPING ASSIGNMENT BLOCK

D.18  MAPPING ASSIGNMENT BLOCK

Mapping Assignment Blocks are used  on  extended  memory  machines  (11/70,

11/44) to map UMR's to physical addresses.  The blocks come from system pool.

Mapping Assignment Blocks are threaded together  using  the  M.LNK  offset.

The list header is $UMRHD in module SYSCM.

Macro: SCBDF$   CDA Switch: none        Size: 14 bytes (6 words)

|===============================|

00      |          Link Word            | M.LNK

|-------------------------------|

02      | Address of First Assigned UMR | M.UMRA

|-------------------------------|

04      |  Number of UMR's Assigned * 4 | M.UMRN

|-------------------------------|

06      | Low 16 Bits Mapped by 1st UMR | M.UMVL

|-------------------------------|

10      | (High 6 Bits) | High 2 Bits   | M.UMVH M.BFVH

|               ----------------|

12      |     22-Bit Physical Address   | M.BFVL

|===============================|

14                                        M.LGTH

D-47

DATA STRUCTURES

MOUNTED VOLUME LIST

D.19  MOUNTED VOLUME LIST

The Mounted Volume List is used to  determine  which  terminals  own  (have

mounted)  a  disk  volume.   The  entries  are taken from system pool by MOU and

returned by DMO when the volume is dismounted.

The list entries are chained together using the  M.MXT  offset.   The  list

header is $MOULS in module SYSCM.

Mounted Volume References:

o  (Offset 0) Next list entry

o  (Offset 4) Disk UCB

o  (Offset 6) Terminal UCB

References to Mounted Volumes:

o  ($MOULS) List header

Macro: none     CDA Switch: none        Size: 10 bytes (4 words)

|===============================|

00      |    Pointer to Next Entry      |

|-------------------------------|

02      |            4001               |

|-------------------------------|

04      |          Disk UCB             |

|-------------------------------|

06      |       Owning Terminal UCB     |

|===============================|

10

D-48

DATA STRUCTURES

F11ACP VCB

D.20  F11ACP VCB

The F11ACP VCB (Volume Control Block) holds all information  related  to  a

mounted  Files-11 disk.  The system creates the VCB is pool whenever a volume is

first mounted and destroys it when the volume is finally dismounted.

VCB's are not chained together.  The pointer to the VCB  for  a  particular

disk is stored in offset U.VCB of the disk UCB.

F11ACP VCB References:

o  (V.IFWI) Index file window block

o  (V.FCB ) FCB chain

References to F11ACP VCB's:

o  (U.VCB ) UCB, address of VCB for mounted device

Macro: F11DF$   CDA Switch: /DEV        Size: 62 bytes (25 words)

|===============================|

00      |       Transaction Count       | V.TRCT

|-------------------------------|

02      |   Index File Window Address   | V.IFWI

|-------------------------------|

04      |                               | V.FCB

|---      FCB Listhead       ---|

06      |                               |

|-------------------------------|

10      | Index BM Size |  (high byte)  | V.IBLB V.IBSZ

|----------------               |

12      |   Index Bit Map 1st Block #   |

|-------------------------------|

14      |    Maximum Number of Files    | V.FMAX

|-------------------------------|

16      |  Cluster Size |  Window Size  | V.WISZ V.SBCL

|-------------------------------|

20      | Storage Bit Map Size (Blocks) | V.SBSZ

|-------------------------------|

22      |  Extend Size  | (high byte)   | V.SBLB V.FIEX

|----------------               |

24      |  Storage Bit Map 1st Block #  |

|-------------------------------|

26      |      Volume Owner's UIC       | V.VOWN

|-------------------------------|

30      |      Volume Protection        | V.VPRO

|-------------------------------|

continued on next page

D-49

DATA STRUCTURES

F11ACP VCB

F11ACP VCB (continued)

|-------------------------------|

32      |    Volume Characteristics     | V.VCHA

|-------------------------------|

34      |  Volume Def. File Protection  | V.FPRO

|-------------------------------|

36      |  Volume File Sequence Number  | V.VFSQ

|-------------------------------|

40      |  # LRU Slots  | (high byte)   | V.FRBK V.LRUC

|----------------               |

42      |     Number of Free Blocks     |

|-------------------------------|

44      |                               | V.LABL

|---                         ---|

46      |                               |

|---                         ---|

50      |                               |

|---   Volume Name (ASCII)   ---|

52      |                               |

|---                         ---|

54      |                               |

|---                         ---|

56      |                               |

|-------------------------------|

60      | Free BM Block |   Status      | V.STAT V.FFNU

|===============================|

62                                        V.LGTH

V.STAT Masks

VC.HLK  10      Hardware lock the volume on dismount

VC.SLK  4       Software lock the volume on dismount

VC.BMW  2       Storage bitmap is write accessed

VC.IFW  1       Index file is write accessed

D-50

DATA STRUCTURES

F11ACP WINDOW

D.21  F11ACP WINDOW

F11ACP Windows are used  to  map  virtual  blocks  in  a  file  to  logical

(physical) disk blocks.  The windows are maintained by F11ACP and are created in

system pool whenever a file is opened and destroyed when the file is closed.

The windows are not threaded together.  There is one window block for  each

open  file.   The  address of the window is stored in the second LUN word of the

associated LUN table entry in the task header.

F11ACP Window References:

o  (W.FCB ) FCB address of file

o  (W.LKL ) Lock list for file

References to F11ACP Windows:

o           Task header, second lun word

o  (L.WI1 ) Lock list, address of window block

D-51

DATA STRUCTURES

F11ACP WINDOW

F11ACP Window (2/4 Format)

Macro: F11DF$   CDA Switch: none        Size: variable

|===============================|

00      |  Access Flags |   # Entries   | W.CTL

|-------------------------------|

02      | Size of PTR's |  (high byte)  | W.VBN  W.SIZE

|----------------               |

04      |   First VBN Mapped by Window  |

|-------------------------------|

06      |          FCB Address          | W.FCB

|-------------------------------|

10      |   Address of Lock Block List  | W.LKL

|-------------------------------|

12      |   # of Blocks Mapped by PTR   | W.RTRV

|-------------------------------|

14      |                               |

|---     LBN Block Number    ---|

16      |                               |

|-------------------------------|

.

.

. More Pointer Triples

.

.

W.CTL Masks

WI.BPS  100000  Bypass interlock                        (1=yes)

WI.EXL  40000   Manual unlock desired                   (1=yes)

WI.PND  20000   Window turn pending                     (1=yes)

WI.DLK  10000   Deaccess lock enabled                   (1=yes)

WI.LCK  4000    Lock against shared access              (1=yes)

WI.EXT  2000    Extend allowed                          (1=yes)

WI.WRV  1000    Write virtual block allowed             (1=yes)

WI.RDV  400     Read virtual block allowed              (1=yes)

D-52

DATA STRUCTURES

LOCK BLOCK LIST

D.22  LOCK BLOCK LIST

The Lock Block List are data structures used to provide multiple writers to

the  same  file (RMS record locking).  The structures provide a means for a task

to lock a block(s) for its exclusive use over a period of time.

All locked blocks for a file are threaded together using offset L.LNK.  The

list  header  is  located  in  offset F.LKL of the FCB.  Also, each window block

associated with the file points to the lock block list using offset W.LKL.

Lock Block References:

o  (L.LNK ) Next list entry

o  (L.WI1 ) Window block of current lock owner

References to Lock Blocks:

o  (W.LKL ) Window block, start of lock list for file

o  (F.LKL ) FCB, start of lock list for file

Macro: F11DF$   CDA Switch: none        Size: 10 bytes (4 words)

|===============================|

00      |    Link to Next Lock Block    | L.LNK

|-------------------------------|

02      |    Address of Window Block    | L.WI1

|-------------------------------|

04      | # Blocks Lock |  (high byte)  | L.VB1  L.CNT

|----------------               |

06      |     Starting VBN Number       |

|-------------------------------|

10                                        L.LGTH

D-53

DATA STRUCTURES

FCB

D.23  FCB

FCB's (File Control Blocks) contain information about currently open files.

The  FCB's  are  maintained by F11ACP and are first allocated using its internal

pool and then overflow into the system pool.

All FCB's for files on a volume are linked together using the F.LINK  word.

The list header is V.FCB of the VCB.

FCB References:

o  (F.LINK) Link to next FCB in chain

o  (F.LKL ) Lock block list for file

References to FCB's

o  (V.FCB ) F11ACP VCB, FCB list header

D-54

DATA STRUCTURES

FCB

FCB

Macro: F11DF$   CDA Switch: /HDR        Size: 52 bytes (43 words)

|===============================|

00      |      FCB Chain Pointer        | F.LINK

|-------------------------------|

02      |        File Number            | F.FNUM

|-------------------------------|

04      |      Sequence Number          | F.FSEQ

|-------------------------------|

06      |   Segment #   |               |        F.FSQN

|-------------------------------|

10      |       File Owner UIC          | F.FOWN

|-------------------------------|

12      |       File Protection         | F.FPRO

|-------------------------------|

14      | System Chars. |  User Chars.  | F.UCHA F.SCHA

|-------------------------------|

16      |                               | F.HDLB

|---    File Header LBN      ---|

20      |                               |

|-------------------------------|

22      |    LBN of Virtual Block 1     | F.LBN

|---                         ---|

24      |  If Contiguous, Zero if Not   |

|-------------------------------|

26      |                               | F.SIZE

|---  Size of File in Blocks ---|

30      |                               |

|-------------------------------|

32      |    # Locks    |  # Accesses   | F.NACS F.NLCK

|-------------------------------|

34      |    Status     |   # Writers   | F.NWAC/F.STAT

|-------------------------------|

36      |   Directory EOF Block Number  | F.DREF

|-------------------------------|

40      |   1st Word of Directory Name  | F.DRNM

|-------------------------------|

42      |   Address of Extension FCB    | F.FEXT

|-------------------------------|

44      |                               | F.FVBN

|---   Starting VBN for FCB  ---|

46      |                               |

|-------------------------------|

50      |     Lock Block Listhead       | F.LKL

|===============================|

52                                        F.LGTH

D-55

DATA STRUCTURES

MTAACP VCB

D.24  MTAACP VCB

The MTAACP VCB (Volume Control Block) holds all information  related  to  a

mounted  magtape.   The  system  creates  the  VCB in pool whenever a magtape is

mounted and destroys it when the magtape is dismounted.

The VCB's are chain together using the V.NXT offset.  The chain  header  is

U.VCB in the magtape UCB.

Macro: MTADF$   CDA Switch: /HDR        Size: 110 bytes (36 words)

|===============================|

00      |      Transaction Count        | V.TCNT

|-------------------------------|

02      |     Pointer to Next VCB       | V.NXT

|-------------------------------|

04      |    Pointer to Mounted List    | V.MVL

|-------------------------------|

06      |   Pointer to Unmounted List   | V.UVL

|-------------------------------|

10      | TCB Address of Accessing Task | V.ATL

|-------------------------------|

12      |         UCB Address           | V.UCB

|-------------------------------|

14      |  Mount Mode   | Current Vol # | V.RVOL V.MOU

|-------------------------------|

16      |      Tape Characteristics     | V.TCHR

|-------------------------------|

20      |    Current File Sequence #    | V.SEQN

|-------------------------------|

22      |    Current File Section #     | V.SECN

|-------------------------------|

24      |     # TM's to Next HDR1       | V.TPOS

|-------------------------------|

26      |  TMO Counter  | Process Stat. | V.PSTA V.TIMO

|-------------------------------|

30      |                               | V.STS

|---                         ---|

32      |   Command Execution Status    |

|---                         ---|

34      |                               |

|-------------------------------|

36      |         Spare Word            | V.SPAR

|-------------------------------|

(continued on next page)

D-56

DATA STRUCTURES

MTAACP VCB

MTAACP VCB (continued)

|-------------------------------|

40      |        Block Length           | V.BLKI

|-------------------------------|

42      |        Record Length          | V.RECL

|-------------------------------|

44      |                               | V.FNAM

|---                         ---|

46      |         File Name             |

|---                         ---|

50      |                               |

|-------------------------------|

52      |         File Type             | V.FTYP

|-------------------------------|

54      |        File Version           | V.FVER

|-------------------------------|

56      |                               | V.CDAT

|---     Creation Date       ---|

60      |                               |

|-------------------------------|

62      |                               | V.EDAT

|---    Expriation Date      ---|

64      |                               |

|-------------------------------|

66      |         Block Count           | V.BLKC

|---                         ---|

70      |      for File Section         |

|-------------------------------|

72      |   CC Attr.    |  Record Type  | V.RTYP V.FATT

|-------------------------------|

74      |                               | V.WIND

|---                         ---|

76      |                               |

|---                         ---|

100     |                               |

|---        Null Window      ---|

102     |                               |

|---                         ---|

104     |                               |

|---                         ---|

106     |                               |

|===============================|

110                                       S.VSCB

D-57

DATA STRUCTURES

EMB

D.25  EMB

EMB's (Error Message Blocks) are created by the executive whenever an error

is  logged.   The  EMB's  are  created  in  pool  and queued to the error logger

(ERL...) using the $ERRHD in module SYSCM.  The system supports different  types

of EMB's for various error conditions.

EMB (Nonsense Interrupt)

Macro: EMBDF$   CDA Switch: /SYS        Size: 22 bytes (9 words)

|===============================|

00      |    Size of EMB in (bytes)     | E.SIZE

|-------------------------------|

02      | Error Subcode |   Error Code  | E.CODE E.SCDE

|-------------------------------|

04      |                               | E.TIME

|---                         ---|

06      |         Time of Error         |

|---                         ---|

10      |                               |

|-------------------------------|

12      |       Sequence Number         | E.SEQ

|-------------------------------|

14      |      Saved I/O Bitmap         | E.ABM

|-------------------------------|

16      | # Interrupts  | ID of Vector  | E.VCTR E.LOST

|-------------------------------|

20      |     PS before Interrupt       | E.OPS

|-------------------------------|

22      |     PC before Interrupt       | E.OPC

|===============================|

D-58

DATA STRUCTURES

EMB

EMB (Device Error)

Macro: EMBDF$   CDA Switch: /SYS        Size: variable (54 + device registers)

|===============================|

00      |    Size of EMB in (bytes)     | E.SIZE

|-------------------------------|

02      | Error Subcode |   Error Code  | E.CODE E.SCDE

|-------------------------------|

04      |                               | E.TIME

|---                         ---|

06      |         Time of Error         |

|---                         ---|

10      |                               |

|-------------------------------|

12      |       Sequence Number         | E.SEQ

|-------------------------------|

14      |      Saved I/O Bitmap         | E.ABM

|-------------------------------|

16      |         Retry Count           | E.RTRY

|-------------------------------|

20      |   I/O Count   | Task Priority |        E.IOC

|-------------------------------|

22      |                               | E.TASK

|---    Task Name (RAD50)    ---|

24      |                               |

|-------------------------------|

26      |       Partition Address       | E.PAR

|-------------------------------|

30      |           Task UIC            | E.UIC

|-------------------------------|

32      |      QIO Function Code        | E.FCN

|-------------------------------|

34      |                               | E.PRM

|---                         ---|

36      |                               |

|---                         ---|

40      |                               |

|---                         ---|

42      |        QIO Parameters         |

|---                         ---|

44      |                               |

|---                         ---|

46      |                               |

|---                         ---|

50      |                               |

|-------------------------------|

52      |               | # Dev. Regs.  | E.RCNT

|-------------------------------|

54      /       Device Registers        / E.REGS

/===============================/

D-59

DATA STRUCTURES

EMB

EMB (Driver Load/Unload)

Macro: EMBDF$   CDA Switch: /SYS        Size: 32 bytes (13 words)

|===============================|

00      |    Size of EMB in (bytes)     | E.SIZE

|-------------------------------|

02      | Error Subcode |   Error Code  | E.CODE E.SCDE

|-------------------------------|

04      |                               | E.TIME

|---                         ---|

06      |         Time of Error         |

|---                         ---|

10      |                               |

|-------------------------------|

12      |       Sequence Number         | E.SEQ

|-------------------------------|

14      |      Saved I/O Bitmap         | E.ABM

|-------------------------------|

16      |                               |

|-------------------------------|

20      |                               |

|-------------------------------|

22      |                               |

|-------------------------------|

24      |                               |

|-------------------------------|

26      |       Load/Unload Code        | E.WHY

|-------------------------------|

30      |     Device Name (ASCII)       | E.NAME

|===============================|

D-60

DATA STRUCTURES

INDEX FILE FORMAT

D.26  INDEX FILE FORMAT

VBN

|===============================|

|                               |

1       |        Bootstrap Block        |

|                               |

|-------------------------------|

|                               |

2       |          Home Block           |

|                               |

|-------------------------------|

|                               |

3       |                               |

|                               |

|---                         ---|

|       Index File Bit Map      |

|         "n" size in           |

|     Home Block (first word)   |

|---                         ---|

|                               |

|                               |

|                               |

|-------------------------------|

3+n+0   |       Index File Header       |

|          INDEXF.SYS           |

|            (1,1)              |

|-------------------------------|

3+n+1   |      Bit Map File Header      |

|          BITMAP.SYS           |

|            (2,2)              |

|-------------------------------|

3+n+2   |     Bad Block File Header     |

|          BADBLK.SYS           |

|            (3,3)              |

|-------------------------------|

3+n+3   |  Master File Directory Header |

|          000000.DIR           |

|            (4,4)              |

|-------------------------------|

3+n+4   |    Checkpoint File Header     |

|          CORIMG.SYS           |

|            (5,5)              |

|-------------------------------|

|                               |

3+n+5   |    User File Header #1        |

|            (6,x)              |

|-------------------------------|

.

.

D-61

DATA STRUCTURES

HOME BLOCK

D.27  HOME BLOCK

Macro: HMBOF$                           Size: 1000 bytes (256 words)

|===============================|

00      |    Index File Bitmap Size     | H.IBSZ

|-------------------------------|

02      |      Location of Index        | H.IBLB

|---                         ---|

04      |          Bit Map              |

|-------------------------------|

06      |  Maximum # of Files Allowed   | H.FMAX

|-------------------------------|

10      | Storage Bit-Map Cluster Size  | H.SBCL

|-------------------------------|

12      |       Disk-Device Type        | H.DVTY

|-------------------------------|

14      |       Structure Level         | H.VLEV

|-------------------------------|

16      /                               / H.VNAM

/      Volume Name (ASCII)      /

/                               /

|-------------------------------|

32      |                               |

|---        Reserved         ---|

34      |                               |

|-------------------------------|

36      |      Volume Owner's UIC       | H.VOWN

|-------------------------------|

40      |    Volume Protection Code     | H.VPRO

|-------------------------------|

42      |    Volume Characteristics     | H.VCHA

|-------------------------------|

44      |    Default File Protection    | H.DFPR

|-------------------------------|

(continued on next page)

D-62

DATA STRUCTURES

HOME BLOCK

Home Block (continued)

|-------------------------------|

46      |                               |

|---                         ---|

50      |           Reserved            |

|---                         ---|

52      |                               |

|-------------------------------|

54      | # Extend Blk. |  # RTV Ptrs.  | H.WISZ H.FIEX

|-------------------------------|

56      |               | # LRU Entries | H.LRUC

|               ----------------|

60      |                               |

|---                         ---|

62      |                               |

|---                         ---|

64      |           Reserved            |

|---                         ---|

66      |                               |

|---                         ---|

70      |                               |

|-------------------------------|

72      |   Checksum of words 00-70     | H.CHK1

|-------------------------------|

74      /                               / H.VDAT

/    Creation Date and Time     /

/                               /

/-------------------------------/

112     /                               /

/      Volume-Header Label      /

/          (not used)           /

/-------------------------------/

256     /                               /

/  System Specific Information  /

/         (not used)            /

/-------------------------------/

400     /                               /

/    Relative Volume Table      /

/         (not used)            /

|-------------------------------|

776     |  Checksum of words 000-774    | H.CK2

|===============================|

1000

D-63

DATA STRUCTURES

FILE HEADER

D.28  FILE HEADER

Macro: HDFOF$                           Size: 1000 bytes (256 words)

|===============================|

00      |       56      |       27      | H.IDOF H.MPOF

|-------------------------------|

02      |         File ID Number        | H.FNUM

|-------------------------------|

04      |     File Sequence Number      | H.FSEQ

|-------------------------------|

06      |    Structure/System Level     | H.FLEV

|-------------------------------|

10      |        File Owner UIC         | H.FOWN

|-------------------------------|

12      |       File Protection         | H.FPRO

|-------------------------------|

14      | System Chars. |  User Chars.  | H.UCHA H.SCHA

|-------------------------------|

16      /                               / H.UFAT

/     User File Attributes      /

/ (For FCS, 1st 7 words of FDB) /

/                               /

|-------------------------------|

56      |                               | I.FNAM

|---                         ---|

60      |       Filename (RAD50)        |

|---                         ---|

62      |                               |

|-------------------------------|

64      |       Filetype (RAD50)        | I.FTYP

|-------------------------------|

66      |        Version Number         | I.FVER

|-------------------------------|

70      |       Revision Number         | I.RVNO

|-------------------------------|

(continued on next page)

D-64

DATA STRUCTURES

FILE HEADER

File Header (continued)

|-------------------------------|

72      |                               | I.RCDT

|---                         ---|

74      |        Revision Date          |

|---                         ---|

76      |                               |

|---            ----------------|

100     |               |               | I.RVTI

|----------------            ---|

102     |                               |

|---    Revision Time        ---|

104     |                               |

|---            ----------------|

106     |               |               | I.CRDT

|----------------            ---|

110     |                               |

|---                         ---|

112     |       Creation Date           |

|---                         ---|

114     |                               |

|-------------------------------|

116     |                               | I.CRTI

|---                         ---|

120     |       Creation Time           |

|---                         ---|

122     |                               |

|-------------------------------|

124     |                               | I.EXDT

|---                         ---|

126     |       Expiration Date         |

|---                         ---|

130     |                               |

|----------------            ---|

132     |               |               |

|-------------------------------|

134     |               |  Ext. Seg. #  | M.ESQN

|-------------------------------|

136     |      Extension File ID #      | M.ERVN

|-------------------------------|

140     |   Extension File Sequence #   | M.EFNU

|-------------------------------|

142     |  RTV Size - Always 1,3 Format | M.CTSZ M.LNSZ

|-------------------------------|

144     |  Max # RTV's  |     # RTV's   | M.USE  M.MAX

|-------------------------------|

146     /                               / M.RTRV

/       Retrieval Pointers      /

/         (in 1,3 format)       /

/                               /

|-------------------------------|

D-65

DATA STRUCTURES

FILE HEADER

776     |   Checksum of Words 000-774   | H.CKSM

|===============================|

1000

D-66

DATA STRUCTURES

DIRECTORY ENTRY

D.29  DIRECTORY ENTRY

Macro: none                             Size: 20 bytes (8 words/32 block)

|===============================|

00      |        File ID Number         |

|-------------------------------|

02      |     File Sequence Number      |

|-------------------------------|

04      |               0               |

|-------------------------------|

06      |                               |

|---                         ---|

10      |       Filename (RAD50)        |

|---                         ---|

12      |                               |

|-------------------------------|

14      |       Filetype (RAD50)        |

|-------------------------------|

16      |        Version Number         |

|===============================|

20

D-67

DATA STRUCTURES

FDB

D.30  FDB

Macro: FDOF$L, FCSBT$                   Size: 102 bytes (33 words)

|===============================|

00      |  Record Attr. |  Record Type  | F.RTYP F.RATT

|-------------------------------|

02      |          Record Size          | F.RSIZ

|-------------------------------|

04      |                               | F.HIBK

|---  Highest VBN Allocated  ---|

06      |                               |

|-------------------------------|

10      |                               | F.EFBK

|---     EOF Block Number    ---|

12      |                               |

|-------------------------------|

14      |  First Free Byte in EOF Block | F.FFBY

|-------------------------------|

16      |  Device Chars | Record Access | F.RACC F.RCTL

|-------------------------------|

20      |  Block I/O Buffer Descriptor  | F.BKDS/F.NRBD

|---            or           ---|

22      | User-record Buffer Descriptor |

|-------------------------------|

24      | Block I/O Status & AST Addr.  | F.BKST/F.NRBD

|---            or              |

26      | Next-record Buffer Descriptor |

|-------------------------------|

30      | Buffer Size/Next Record Addr. | F.OVBS/F.NREC

|-------------------------------|

32      |       End-of-block Value      | F.EOBB

|-------------------------------|

34      | Create Quantity & Statistics  | F.CNTG/F.RCNM

|---            or           ---|

36      |          Record Number        | F.STBK

|-------------------------------|

40      |       Extend Quantity         | F.ALOC

|-------------------------------|

42      |  File Access  |  LUN Number   | F.LUN  F.FACC

|-------------------------------|

44      |   Dataset Descriptor Pointer  | F.DSPT

|-------------------------------|

46      |  Default Filename Block Addr. | F.DFNB

|-------------------------------|

50      |  Bookkeeping  |    EFN        | F.BKEF F.BKP1

|-------------------------------|

(continued on next page)

D-68

DATA STRUCTURES

FDB

FDB (continued)

|-------------------------------|

52      |          Error Word           | F.ERR

|-------------------------------|

54      |  Buffer Count | Mul. Buf. No. | F.MBCT F.MBC1

|-------------------------------|

56      |   Reserved    | Mul. Buf. Bit | F.MBFG F.BGBC

|-------------------------------|

60      |       Device Buffer Size      | F.VBSZ

|-------------------------------|

62      |       Block  Buffer Size      | F.BBSZ

|-------------------------------|

64      |                               | F.BKVB/F.VBN

|---       VBN Number        ---|

66      |                               |

|-------------------------------|

70      |    Block Buffer Descriptor    | F.BDB

|-------------------------------|

72      |        Spooling Device        | F.SPDV

|-------------------------------|

74      |   Reserved    | Spooling Unit | F.SPUN F.CHR

|-------------------------------|

76      |          Control Bits         | F.ACTL

|-------------------------------|

100     |         Sequence Number       | F.SEQN

|===============================|

102                Start of FNB           F.FNB

D-69

DATA STRUCTURES

FNB

D.31  FNB

Macro: NBOF$L, FCSBT$           Size: 36 bytes (15 words)

|===============================|

00      |                               | N.FID

|---                         ---|

02      |           File-ID             |

|---                         ---|

04      |                               |

|-------------------------------|

06      |                               | N.FNAM

|---                         ---|

10      |          Filename             |

|---                         ---|

12      |                               |

|-------------------------------|

14      |          File Type            | N.FTYP

|-------------------------------|

16      |           Version             | N.FVER

|-------------------------------|

20      |           Status              | N.STAT

|-------------------------------|

22      |          Next File            | N.NEXT

|-------------------------------|

24      |                               | N.DID

|---                         ---|

26      |         Directory ID          |

|---                         ---|

30      |                               |

|-------------------------------|

32      |         Device Name           | N.DVNM

|-------------------------------|

34      |         Unit Number           | N.UNIT

|===============================|

36

D-70

DATA STRUCTURES

DSD

D.32  DSD

Macro: FDSOF$                           Size: 14 bytes (6 words)

|===============================|

00      |     Device String Length      | N.DEVD

|-------------------------------|

02      |     Device String Address     |

|-------------------------------|

04      |   Directory String Length     | N.DIRD

|-------------------------------|

06      |   Directory String Address    |

|-------------------------------|

10      |    Filename String Length     | N.FNMD

|-------------------------------|

12      |    Filename String Address    |

|===============================|

14

D-71

DATA STRUCTURES

$$FSR1

D.33  $$FSR1

Macro: BDOFF$                           Size: variable

|===============================|

00      |                               |

|---    I/O Status Block     ---|

02      |                               |

|-------------------------------|

04      |                               | B.VBN

|---   Virtual Block Number  ---|

06      |                               |

|-------------------------------|

10      |      Address of Next Buffer   | B.NXBD

|-------------------------------|

12      | Buffer Status |     Empty     |        B.BFST

|-------------------------------|

14      |             Empty             |

|-------------------------------|

16      /       Start of I/O Buffer     / S.BFHD

/               .               /

/               .               /

/               .               /

D-72

DATA STRUCTURES

$$FSR2

D.34  $$FSR2

Macro: $FSROF                           Size: 104 bytes (34 words)

|===============================|

00      |                               |

|---   Allocation Listhead   ---|

02      |                               |

|-------------------------------|

04      |     First Address in FSR1     | A.BFSR

|-------------------------------|

06      |     Last  Address in FSR1     | A.EFSR

|-------------------------------|

10      |       File Owner UIC          | A.OWUI

|-------------------------------|

12      |   Default File Protection     | A.FIPR

|-------------------------------|

14      /                               / A.DPB

/   Scratch Area and QIO DPB    /

/                               /

|-------------------------------|

44      |                               | A.IOST

|--- Scratch I/O Status Area ---|

46      |                               |

|-------------------------------|

50      /                               / A.DFDR

/ Default Directory Information /

/                               /

|-------------------------------|

100     |    Default Buffer Count       | A.DFBC

|-------------------------------|

102     |      Default Task UIC         | A.DFUI

|===============================|

104

D-73

APPENDIX E

EXAMPLE CRASHES

Included on the SIG tape in the crash analysis area are a number  of  files

named CRAxxx.MAC.  These are the sources for programs which you can run to crash

your system and generate example crashes.  CRAASMALL.CMD is a  template  command

file  to  assemble  all  of  these.  CRABLDALL.CMD is a template command file to

build all of them.  These have all been tested and all  crashed  the  system  on

which  they  were  run.   These programs are intended for use by a knowledgeable

person for educational purposes;  the authors take no  responsibility  for  what

happens when your system crashes.

E.1  CRAYZO

This program causes a stack limit violation  by  pushing  onto  the  kernel

stack until the crash occurs.

E.2  SST CRASHES - CRAT04, CRAILI, CRABPT, CRASEG

These programs cause executive SST crashes.  This provides examples of  the

SST  crashes  discussed  in  Basic  Crash  Analysis.  Interpreting these crashes

should be straight-forward after reading the Basic Crash Analysis chapter.

E.3  CRATRP AND CRAEMT

These programs crash the system via a TRAP and  an  EMT  at  system  state,

respectively.  These are more examples for Basic Crash Analysis.

E.4  CRAOSP

This is the crash caused when the kernel stack pointer becomes  odd.   This

is  a Fatal Stack Error.  The stack pointer is repositioned to 4, and the PS and

PC are pushed onto the stack.  The processor  traps  through  4,  and  jumps  to

$CRASH  because of a stack limit violation.  The key to understanding this crash

E-1

EXAMPLE CRASHES

CRAOSP

is to note that the kernel stack pointer on the CDA output is  4,  and  to  then

examine low core.

E.5  ODD VECTOR CRASHES

There are two types of odd vector crashes.  If the odd vector is location 4

or  the  IOT  vector, then a repeating pattern occurs on the kernel stack.  This

occurs because the attempt to handle the trap results in  another  trap  of  the

same  type.   In  the  case  of odd location 4, this happens entirely within the

PDP-11 hardware.  In the case of the IOT vector, this depends on the  fact  that

the IOT is used in RSX to crash the system.

The second type of odd vector crash occurs if some  other  trap  vector  or

interrupt vector is odd.  The fatal aspect here is that there is one more PS, PC

combination on the stack than the executive knows about.  The crash occurs  when

the executive tries to clean up after the second trap.

A general rule is that if the stack has a  repeating  pattern,  or  if  the

stack  seems  hopelessly  confused  prior  to  the  crash,  examine low core for

corrupted vectors.

E.5.1  CRAOD4

This is the crash caused when location 4 is odd, and an  odd  address  trap

occurs.   When the processor tries to fetch the new PC, another odd address trap

occurs, causing a repeating pattern on the stack down to the yellow zone  limit.

When  the  yellow  zone  limit  is reached, the processor alternates between odd

address traps and stack limit violation traps.  The push onto the  stack  occurs

before the PC is incremented, so the PC's pushed by the two traps differ by two.

When the red zone is reached, the stack pointer is reset to 4, the PS and PC are

pushed on the stack, and the processor halts.  Note that the exact appearence of

the stack in the yellow zone may vary depending on the priority of different CPU

errors in your CPU model.

E.5.2  CRAIOS And CRAIOU

These are examples of the crashes which occur when the IOT vector  is  odd.

CRAIOS  is an IOT at system state;  CRAIOU is an IOT at user state.  Both have a

13 word repeating pattern on the kernel stack.  The crash routine is  eventually

reached through the stack limit violation code.

E-2

EXAMPLE CRASHES

ODD VECTOR CRASHES

E.5.3  CRAEVO

This crash occurs if the EMT vector is odd.  The EMT pushes the task PS and

PC onto the stack.  After the EMT, the processor is in mode [current,previous] =

[kernel,user].  The fact that the vector address is odd causes  an  odd  address

trap.   Now  the  mode  is  [kernel,kernel].  The executive only knows about one

trap, and now proceeds to handle the odd address trap as though it  occurred  at

user  state.   If the current task has no SST vectors, then the executive cleans

the stack and jumps to the routine $ABCTK (abort current task).  The  RETURN  at

the  end  of  that  routine  is intended to take us back to $DIRSV in this case.

However, the stack contents are shifted by two words because of the extra  trap.

The result is that the executive tries to return to the location in kernel space

which corresponds to the value of R1 before the original trap.  If R1 is odd, as

in  this  example, an odd address trap occurs and the system crashes right away.

(Note that R1 is odd when CRAEVO begins execution, with contents of 12621 -  CRA

in RAD-50.) If R1 is even, the results are unpredictable, but bound to be bad.

If  the  current  task  does  have  SST  vectors,  the  results  are   less

predictable.   Normally  the SST parameters would be pushed onto the task stack,

along with the PS and PC from the SST.  This just isn't going  to  work  out  in

this case.

A similar crash can occur if an  interrupt  vector  or  miscellaneous  trap

vector is odd.  Again the key to this crash is not to grind out what happened to

the stack, but to check the vectors in low core.

E-3