PROFILE V01.0

THE PROFILE PACKAGE IS A SET OF TWO MACRO MODULES AND THIS

DOCUMENT THAT WILL ALLOW THE USER TO DETERMINE HOW OFTEN INSTRUCTION

MEMORY LOCATIONS ARE EXECUTED AT RUN(CRAWL) TIME AND A SUMMARY OF WHAT

INSTRUCTIONS WERE EXECUTED. THE 'MEASURE' MODULE IS TASK BUILT INTO

THE USERS PROGRAM TO RECORD THE MEMORY LOCATIONS AND INSTRUCTIONS

EXECUTED. THEN THE 'EVALUATE' PROGRAM IS RUN ON THE DATA GATHERED

AND A LISTING OF THE RESULTS IS GENERATED.

MEASURE.MAC

THIS MODULE IS TASK BUILT INTO THE TASK TO BE MEASURED AND

MAY NEED MODIFICATION AT BOTH THE SOURCE CODE LEVEL AND AT THE

TASK BUILD LEVEL TO PROPERLY OPERATE WITH THE USERS PROGRAM.

THE SOURCE CODE CHANGES ARE AS FOLLOWS:

L.U.N. USEAGE.	THE MEASURE MODULE REQUIRES THAT TWO(2) DATA FILES

BE OPEN DURING THE TAKING OF DATA. THE SOURCE CODE

USES A DEFAULT OF LUN 1 AND LUN 2. IF THESE ARE IN

CONFLICT WITH THE USEAGE OF THE USER PROGRAM, THESE

CAN BE ALTERED BY CHANGEING THE SYMBOLS 'INSLUN'

AND 'LOCLUN' THAT ARE DEFINED AT THE BEGINING

OF THE SORCE CODE TO NONCONFLICTING VALUES.

EVENT FLAG USEAGE

DATA GENERATED BY THE MEASURE MODULE IS KEPT ON DISK

REQUIREING QIO'S TO RECOVER AND STORE BLOCKS OF

COUNTERS. SINCE THIS MUST BE A SYNCHRONOUS OPERATION,

QIOW$'S ARE USED SECIFYING AS DEFAULT EVENT FLAG

20(10). IF THIS CONFLICTS WITH THE USER PROGRAM,

THIS CAN BE MODIFIED BY CHANGEING THE SYMBOL

'EVNFLG' AT THE BEGINING OF THE SOURCE TO A

NON CONFLICTING ONE.

DUE TO THE ADDITIONAL I/O REQUIRED BY THE MEASUREMENT ROUTINE,

SEVERAL MODIFICATIONS MAY BE REQUIRED AT TASK BUILD TIME.

OVERLAYS	THE MEASUREMENT ROUTINE IS CALLED VIA THE T-BIT

TRAP AFTER THE EXECUTION OF EACH INSTRUCTION, SO

IT MUST RESIDE IN THE ROOT SEGMENT. ALSO IT DOES'NT

KNOW ABOUT OVERLAYS, SO THAT THE DATA ON LOCATION

USEAGE IN THE OVERLAY AREA IS THE SUM OF ALL THE

OVERLAYS USEAGE.

NUMBER OF ACTIVE FILES

THE TASK BUILDER SETS UP THE TASK WITH A DEFAULT

NUMBER OF ACTIVE FILES OF FOUR(4). SINCE THE MEASUREMENT

ROUTINE REQUIRES 2, THIS LEAVES ONLY 2 FOR THE USER PROGRAM.

THIS CAN BE EXTENDED BY 'ACTFIL=NNN' WHERE NNN ARE THE NUMBER

OF ACTIVE FILES REQUIRED BY BOTH THE MEASUREMENT ROUTINE

AND THE USER PROGRAM (SEE ALSO 'FCS I/O BUFFER SIZE')

DATA FILE DISK ASSIGNMENT

THE MEASUREMENT ROUTINE GENERATES 3 DATA FILES ON

THE DEFAULT DISK ASSIGNED TO THE LUN'S USED (DEFAULT

ARE 1 AND 2 BUT CAN BE CHANGED IN THE SOURCE).

TKB DEFAULT IS SY: BUT THE USER CAN MODIFY THIS AT

TKB TIME BY THE USE OF 'ASG=XXN:1:2'.

FCS I/O BUFFER SIZE ($$FSR1)

THE I/O REQUIREMENTS OF THE MEASUREMENT ROUTINE REQUIRE

THE ASSIGNMENT OF SPACE FOR TWO(2) I/O BUFFERS. IF THE

USER PROGRAM DOES NOT REQUIRE THIS AREA, THE MEASURE

ROUTINE HAS BUILT IN TO IT THE TWO(2) BUFFERS THAT

IT REQUIRES. IF THE USER PROGRAM DOES USE FCS I/O

BUFFERS AND THE TOTAL BUFFER AREA IN $$FSR1 IS NOT

SUFFICENT TO HANDLE ALL THE FILES THAT WILL BE OPEN

SIMULTAINIOUSLY (TWO(2) OF WHICH ARE BY THE MEASUREMENT

ROUTINE), THEN THE USER MUST EXTEND THEN SIZE OF THE

I/O BUFFER AREA AT TKB TIME BY THE USE OF

'EXTSCT=$$FSR1:XXXX'

WHERE 'XXXX' IS THE TOTAL SIZE OF THE I/O BUFFER AREA

IN OCTAL BYTES (THE REQUIREMENT FOR THE MEASUREMENT

ROUTINE IS 2040(8) BYTES) (SEE ALSO 'NUMBER OF ACTIVE FILES').

HIGHEST LUN USED

THE TASK BUILDER BY DEFAULT SETS THE HIGHEST LUN USEABLE

BY A TASK TO 6. IF THE USER TASK OR MODIFIED MEASURE

ROUTINE REQUIRES A LUN HIGHER THAN 6, USE 'UNITS=N'

WHERE 'N' IS THE HIHEST LUN USED.

USER PROGRAM INTERFACE TO THE MEASUREMENT ROUTINE

THE USER HAS FOUR(4) ENTRY POINTS THAT HE CAN CALL FROM

FORTRAN OR ASSEMBLY TO INTERFACE WITH THE MEASUREMENT ROUTINE.

CALL	INIEXE			(JSR	PC,INIEXE)

CALLED ONLY ONCE BY THE USER PROGRAM. THIS SUBROUTINE

CREATES AND INITIALIZES THE THREE(3) DATA FILES USED

IN THE PROFILE PACKAGE AND STARTS THE MEASUREMENT OF

INSTRUCTION LOCATION'S EXECUTED AND INSTRUCTION TYPE

EXECUTION SUMMARY (THE T-BIT TRAP IS ENABLED). THIS

MUST BE THE FIRST EXECUTION MODULE SUBROUTINE CALLED

AT RUN TIME.

CALL TRMEXE			(JSR	PC,TRMEXE)

CALLED ONLY ONCE BY THE USER PROGRAM. THIS SUBROUTINE

TURNS OFF THE T-BIT TRAP, WRITES OUT ALL BUFFERS STILL

IN MEMORY AND CLOSES ALL OPEN DATA FILES. THIS MUST BE

THE LAST EXECUTION MODULE SUBROUTINE CALLED AT RUN TIME.

CALL TROFF			(JSR	PC,TROFF)

CALLED AT RUN TIME IN THE USER PROGRAM TO DISABLE MEASUREMENT

OF PROGRAM ACTIVITY.

CALL TRON			(JSR	PC,TRON)

CALLED AT RUN TIME IN THE USER PROGRAM TO RE-ENABLE MEASURMENT

OF PROGRAM ACTIVITY.

FOR MOST USER PROGRAMS, THE ONLY REQUIREMENTS WILL BE TO DO

A 'CALL INIEXE' AS THE FIRST EXECUTEABLE INSTRUCTION OF THE PROGRAM,

AND A 'CALL TRMEXE' AS THE LAST.

THE MEASUREMENT ROUTINE CREATES 3 DATA FILES FOR THE EVALUATION

PROGRAM.

MEASURE.DAT	A ONE BLOCK LONG FILE THAT CONTAINS THE FOLLOWING INFORMATION

FOR THE EVALUATION PROGRAM:

TASK NAME (AS INSTALLED)

TASK START TIME

TASK START DATE

LOCTCTR.DAT	A 256(10) CONTIGUOUS BLOCK LONG FILE THAT CONTAINS I*4

COUNTERS, ONE FOR EACH VIRTUAL WORD ADDRESS IN THE USERS

PROGRAM. AT THE COMPLETION OF EXECUTION OF EACH INSTRUCTION,

THE T-BIT TRAP IS TAKEN, AND THE LOCATION OF THE NEXT

INSTRUCTION TO BE EXECUTED IS OBTAINED FROM THE STACK,

AND THE CORRESPONDING COUNTER FOR THAT LOCATION IS

INCREMENTED.

INSTCTR.DAT	A 512(10) CONTIGUOUS BLOCK  LONG FILE THAT CONTAINS I*4

COUNTERS, ONE FOR EACH OF THE POSSABLE 65536(10)

INSTRUCTION BIT PATTERNS. AT THE COMPLETION OF EACH

INSTRUCTION, THE T-BIT TRAP IS TAKEN, AND THE NEXT

INSTRUCTION TO BE EXECUTED IS OBTAINED AND THE

CORRESPONDING COUNTER FOR THAT BIT PATTERN IS INCREMENTED.

EXECUTION TIME/SPACE OVERHEAD

IT MUST BE NOTED, THAT EVEN WITH THE USE IN THE MEASUREMENT ROUTINE

OF QIO'S RATHER THAT GET'S AND PUT'S, AND THE BUFFERING OF FOUR(4) LOCATION

TABLES AND EIGHT(8) INSTRUCTION TABLES IN MEMORY TO REDUCE I/O, THE

EXECUTION SPEED OF THE USER PROGRAM IS SEVERELY DEGRADED. A SMALL FORTRAN

TEST PROGRAM SHOWED AN INCREASE OF EXECUTION TIME OF AS MUCH AS 130:1

OVER THE NON MEASURED EXECUTION TIME. THIS NUMBER IS SUPPLIED ONLY AS A

ROUGH GUIDE AND NOT AS AN EXACT MEASURE OF ITS EFFECT ON THE RUN( OR

IN THIS CASE CRAWL) TIME OF A USER PROGRAM.

THE MEMORY REQUIREMENTS OF THE MEASUREMENT MODULE ARE AS FOLOWS:

CODE/INTERNAL BUFFERS		3994(10) WORDS

$$FSR1 (FCS I/O BUFFERS)	 528(10) WORDS

$$FSR2				  34(10) WORDS

$$RESL (FCS/SYSLIB ROUTINES)	2508(10) WORDS

______________

7064(10) WORDS

NOTE THAT IF THE USER PROGRAM DOES ANY FILE STRUCTURED I/O, THE

FCS/SYSLIB ROUTINES IN $$RESL ARE USED BY BOTH THE USER PROGRAM

AND THE MEASUREMENT ROUTINE.

EVALUATION PROGRAM

THE EVALUATION PROGRAM TAKES THE 3 DATA FILES GENERATED

BY THE MEASUREMENT ROUTINE (DELETEING THEM WHEN DONE) AND GENERATES

A LIST FILE SHOWING THE USER PROGRAM RUN TIME EXECUTION PROFILE.

THE LIST FILE GENERATED HAS FOUR(4) PARTS:

PROGRAM INSTRUCTION LOCATION USEAGE COUNT BY LOCATION

THIS SECTION MAKES ONE PASS THROUGH THE LOCATION COUNTER DATA FILE

AND PRINTS OUT IN INCREASING LOCATION ADDRESS ALL INSTRUCTION ADDRESSES

WITH NON-ZERO USEAGE COUNT, GIVEING THE OCTAL ADDRESS AND THE DECIMAL

USAGE COUNT.

PROGRAM INSTRUCTION LOCATION USEAGE COUNT BY COUNT

THIS SECTION IS THE RESULT OF A SORT DONE ON THE DATA FROM THE

PREVIOUS SECTION. ALL ADDRESSES WITH NON-ZERO COUNTS ARE PRINTED OUT

IN ORDER OF DECREASING USEAGE COUNTS. THE FORMAT BEING THE SAME AS THE

FIRST SECTION OF THE PRINTOUT.

PROGRAM INSTRUCTION USEAGE TABLE

THIS SECTION PRINTS OUT INFORMATION ON ALL THE INSTRUCTIONS

EXECUTED BY THE USER PROGRAM. FOR EACH INSTRUCTION EXECUTED, THE FORMAT

IS AS FOLLOWS:

INSTRUCTION MNEMONIC		NUMBER OF TIMES USED(DECIMAL)

E.G.

EMT				1020

DEPENDING ON THE INSTRUCTION, ONE OR MORE OF THE FOLLOWING TABLES ARE

PRINTED OUT FOLLOWING THE INSTRUCTION NAME AND TOTAL COUNT:

REGESTER OR ACCUMULATOR ONLY MODE INSTRUCTION

R0	R1	R2	R3	R4	R5	R6	R7

COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT

FULL SOURCE AND/OR INSTRUCTION MODE INSTRUCTION

R0	R1	R2	R3	R4	R5	R6	R7

MODE  R		COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT

MODE (R)	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT

MODE (R)+	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT

MODE @(R)+	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT

MODE -(R)	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT

MODE @-(R)	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT

MODE X(R)	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT

MODE @X(R)	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT	COUNT

(ANY MODE WHERE NO REGESTER IS USED, IS NOT PRINTED OUT)

WHERE THE INSTRUCTION IS A FULL SOURCE AND DESTINATION REGESTER OPERAND

TWO(2) TABLES ARE GENERATED, ONE EACH FOR SOURCE AND DESTINATION.

PROGRAM SUMMARY

THE FINAL PART OF THE LISTING IS A SUMMARY OF THE ENTIRE USER

PROGRAM. THIS INCLUDES THE TOTAL NUMBER OF INSTRUCTIONS EXECUTED,

A PAIR OF TABLES LAIKE THOSE IN PART THREE OF THE LISTING SHOWING

FOR SOURCE AND DESINATION USEAGE THE CPU REGESTERS AND MODES OF USE,

AND (IF ANY ARE USED) THE SOURCE AND DESTINATION UESAGES OF ANY

FPU ACCUMULATORS.

RUNNING THE EVALUATION PROGRAM

NO USER INTERACTION IS REQUIRED IN THE OPERATION OF THE

EVALUATION PROGRAM ONCE THE COMMAND 'RUN EVALUATE' HAS BEEN GIVEN.

THE PROGRAM AUTOMATICALY LOOKS FOR THE THREE(3) DATA FILES GENERATED

BY THE MEASUREMENT ROUTINE (DELETEING THEM WHEN DONE), GENERATEING

A LIST FILE OF THE RESULTS ('EVALUATE.LST') AND SPOOLING IT WHEN

DONE.

BUILDING THE PROFILE PACKAGE

THE MEASURE MODULE (MODIFIED AS NEEDED BY THE USER TO BE

COMPATABLE WITH THE LUN AND EVENT FLAG USEAGE OF HIS PROGRAM) IS

ASSEMBLED

MAC MEASURE=MEASURE

IT CAN THEN BE TASK BUILT INTO THE USER TASK WITH ANY REQUIRED OPTIONS

AS OUTLINED EARLIER IN THIS DOCUMENT.

THE EVALUATION PROGRAM NEED ONLY BE BUILT ONCE.

MAC EVALUATE=EVALUATE

TKB EVALUATE/FP=EVALUATE

NOTE THE '/FP' AS PART OF THE PRINT OUT FORMATTING USES THE FPU.

FOR ANY QUESTIONS, BUG REPORTS, IMPROVEMENTS CONTACT

MICHAEL N. LEVINE

CODE 3513

NAVAL WEAPONS CENTER

CHINA LAKE

CALIFORNIA

93555

(714) 939-2417

AVN  437-2417