.TITLE	CVL - CHANGE VOLUME LABEL

.IDENT	/01.30/

;

; (PROGRAM IS USEFUL IN INCREASING NUMBER OF NAMES IN A FILE "MXF")

;

;

; AUTHOR:	H. L. COLEMAN

;		MILLIKEN AND COMPANY

;		P. O. BOX 1926

;		MAIL STOP M-103

;		SPARTANBURG, S. C. 29304

;		(803)573-2556

;

; DATE:		16-OCT-78

;

; MODIFIED:

;		J. DOWNWARD

;		KMS FUSION, INC

;		3941 RESEARCH PARK DR

;		ANN ARBOR, MICH. 48104

;		(313)769-8500

;

;		JGD01 -	ADDED SUPPORT FOR /VI, /LRU, /WINDOW, /MAXEXT, /MXF

;			SWITCHES.  DISPLAY VOLUME CREATION DATE.

;

;		JGD02 - ADDED SUPORT FOR /FPRO=[RWED,RWED,RWED] AS IN INI

;

; FUNCTION:	READ THE DISK LABEL BLOCK AND CHANGE THE LABEL TO THAT

;		SPECIFIED IN THE MCR COMMAND.  IF NO MCR COMMMAND IS

;		AVAILABLE, THE OPERATOR IS PROMPTED FOR COMMANDS.

;

; INPUT:

;

;		MCR COMMAND OF THE FORM:

;

;			CVL DDU:LABEL/SW1/SW2/SW3....

;		WHERE  SW1,..SWN CAN BE ANY OF

;		/VI 	- DISPLAY VOLUME INFORMATION BUT DON'T CHANGE ANYTHING

;		/MAXEXT = NN	- CHANGE THE DEFAULT FILE EXTENSION TO THE NEW VALUE NNN

;		/WINDOW = NN	- CHANGE THE DEFAULT NUMBER OF WINDOWS TO NNN

;		/LRU	= NN	- CHANGE THE DEFAULT LRU THE VOLUME WILL BE MOUNTED WITH

;		/MXF	= NNN	- CHANGE THE MAXIMUM NUMBER OF FILES ALLOWED ON A VOLUME

;				  (ONLY INCREASES UPWARD ARE ALLOWED)

;				  (NN IS IN DECIMAL IN ALL CASES)

;		/FPRO	= [RWED,RWED,RWED,RWED]   WHERE  RWED  DOES THE SAME THING AS IN INI

;				   AND THE SWITCH CHANGES THE VOLUME DEFAULT FILE PROTECTION

;				   FOR  [SYSTEM,OWNER,GROUP,WORLD] TO THE NEWLY SPECIFIED

;				   DEFAULT PROTECTION.

;		CREATION DATE	  AS AN ADDED BONUS, THE DATE THE DISK WAS CREATED IS

;				  DISPLAYED.  THIS MAY BE USEFUL IN DETERMINING HOW LONG

;				  IT HAS BEEN SINCE THE DISK WAS 'BADDED', FORMATTED, ETC.

;

; OUTPUT:

;

;		THE SPECIFIED DISK LABEL BLOCK IS MODIFIED.