[307,20]NEWFIXER.DOC

As distributed, DEC's version of FIXER (FXR...) has some problems

when attempting to recover from memory parity errors. The DEC version

sometimes fails to correctly place bounds around the bad memory, and

it totally ignores recovery from parity errors in read-only regions

when multiple tasks are mapped to the region.

This version of FIXER, modified and enhanced by Larry Baker and

Gary Maxwell, performs the following actions when a memory parity

error is encountered:

1.  Using information supplied by the Executive, FIXER locates

any partition flagged as having a parity error.

2.  If the partition is a read-only region, then the region

is "kicked out" of memory, which forces a new copy of the

region to be loaded. Only the task which caused the parity

error is aborted. Other tasks mapped to the region go along

their merry way, as soon as the region is reloaded.

3.  FIXER then tests all memory in the partition, starting at

the base, until it encounters a bad memory word. The the

scan continues from the top of the partition, until the

next bad cell is located. This defines the bounds of the

bad memory, and a fixed, non-shuffleable partition is created

around the bad memory, eliminating it from futher use.

The DEC version of FIXER would fail at this phase if the

parity error occured on a 32 word boundary.

To build FIXER, you must have the following files relating to

the target system:

LB:[sysuic]RSX11M.STB

LB:[1,1]EXELIB.OLB

where "sysuic" is the System UIC for the current running system. Note

that we cannot provide the source file, NEWFIXER.MAC. The modifications

are too extensive to create a SLP file, and by the rules of the SIG Tape

Copy facility, we cannot place the modified source file in our

submission, since FIXER is DEC proprietary software.

To build NEWFIXER, invoke the following command file:

>@NEWFIXER.CMD

and the task is built. Copy the task image to your system UIC, then

enter VMR to replace the DEC version:

VMR>REMOVE FXR...

VMR>INSTALL NEWFXR

VMR>^Z

>

If you wish to obtain a copy of the source file NEWFIXER.MAC, please

send us an affidavit asserting that you own an M-Plus source licence,

along with a tape or floppy. Be sure to specify the tape format you

desire. (Of course, you can always disassemble the object file!)

We would appreciate any comments, complaints, or suggestions!

Larry Baker

Gary Maxwell

U. S. Geological Survey

345 Middlefield Road

Mailstop 77

Menlo Park, California  94025