Sizing Communications Requirements

B. Z. Lederman

I.T.T. World Communications

New York, N.Y. 10004

Abstract

Traffic analysis is a method used by the telecommunications  industry  to

determine  the number of circuits or operators required to meet the demand for

service of their customers.  Although it  is  not  widely  known  outside  the

industry,  it  can  be  applied  to  a wide variety of applications where some

number of requests for service must compete for a limited  resource  servicing

the  requests.   This  paper  will  not  attempt to give a complete history of

traffic analysis, but will try to show the fundamentals involved,  demonstrate

the  types  of  problems which traffic analysis can solve, and show where such

analysis may be of use in determining the quantity  of  resource  required  to

meet the demand of those wishing to be served by that resource.

Briefly, traffic analysis started when  telephone  companies  were  faced

with  a  situation  such  as  in  Figure 1.  The problems faced are:  how many

trunks (telephone lines or circuits) are required between towns  A  and  B  to

keep  most  customers  happy  (not  too many fast busy signals*), and how many

operators are required  in  each  town  to  answer  requests  for  assistance,

telephone  numbers,  etc.?   Too  few lines, and customers will complain about

busy signals, too many and the telephone company wastes money and resources by

supplying  unused lines:  too few operators, and customers will complain about

waiting too long for assistance, too many and operators will sit  idle.   This

represents  two  types  of  systems:   the  first  is a "blocking", or "loss",

system, where requests are abandoned if not immediately satisfied;  the second

is  a "waiting", or "queueing", system, where requests wait until served (or a

given amount of time has elapsed).  These two types of  situations  occur  for

many  applications outside of the telephone industry, and it is the purpose of

this paper to demonstrate how the methods of traffic analysis may be used  for

these applications.

The methods of  answering  these  questions  were  largely  developed  by

A. K. Erlang,  and are based on a number of assumptions which must be true for

the answers to be true.  First, it is assumed that requests  for  service  (in

this case, persons in one town calling a person in the other town) are made at

random in time:  experience has shown that this is an accurate description  of

* The telephone  company  considers  it's  job  done  when  one  telephone  is

connected  to another:  if the desired telephone is in use, or no-one answers,

it is not their concern.

Introduction to traffic analysis.                                       Page 2

B. Z. Lederman I.T.T. World Communications

the  actions  of  large  numbers  of  persons  acting  separately  (the  exact

distribution  is a form of Poisson, or exponential distribution).  Next, it is

assumed that the answer desired is that which will  satisfy  the  majority  of

requests over a given period of time.  Traffic analysis deals with statistical

processes:  it describes what one can normally expect to see in the long  run,

and  not  necessarily  what will happen at any one particular instant in time,

and measurements are usually taken over a period of  time  (for  example,  how

busy  a  system is during a one hour period) rather than for instants in time.

This is, in fact, what one usually wants in designing a  system;   to  satisfy

the  normal  request  for  day-to-day  service, rather than an abnormally high

request rate which may occur once in 3 years.  The Erlang conditions produce a

subset  of  more  general  queueing  theory,  and  result  in  equations which

(complicated as they may appear) are easier to use than more general  queueing

theory formulas.

The loss system is easier to explain, so  an  example  of  this  type  of

application  will be given first, as shown in Figure 2.  The problem involvs a

computer with a number of telephone lines attached to modems:  persons wishing

to  use the computer must call in, and the telephone company will connect them

to one of the lines, unless all are busy.  The question  is,  how  many  lines

(plus the attached modem and line on the computer) are required?

Before the number of lines can be computed, it is necessary to  know  how

much  traffic  will  be carried by these lines.  Notice I did not say how many

people want to use the system:  by assuming the traffic  is  generated  by  an

infinite  source, the traffic can be measured in a way which is independant of

the number of requests.  The measurement which is needed is the  total  amount

of time the lines in this group are busy over a given time period.  The period

of time most often used is one hour, and this will be used  for  the  example,

though the entire process can be scaled to larger or smaller time intervals if

desired.  During the one hour period in  question  (which  need  not  coincide

exactly  to a clock hour, i.e.  the period could start at 14:37:52 rather than

at exactly 2:00 PM), the total amount of time each line in the group  is  busy

will  be  measured,  and  the  sum  total is the amount of trunk occupancy, or

traffic, received.  For example, if there was only one call, and it lasted two

minutes,  then the amount of traffic in that hour was 120 call connect seconds

(CS).  If there were two calls, one for 3 minutes and one for 5  minutes,  the

total  traffic  was  480 CS or 4.8 cent call seconds (CCS) or 4.8 hundred call

seconds (HCS).  If a call was in progress when the measurement  period  began,

or  is  in  progress when the measurement period ends, only the portion of the

call within the period of interest is included:  for example,  a  call  begins

during  the  measurement period, and lasts for 8 minutes, but it started 2 1/2

minutes before the end  of  the  measurement  period;   therefore  only  2 1/2

minutes are added to the total traffic:  see Figure 3 for some examples.  When

the period is over, all of the traffic is added up.  It can  be  expressed  in

call-connect  seconds,  or  cent call seconds (which is more common within the

telephone industry), but it is most often  expressed  in  Erlangs  (named  for

A. K. Erlang).   One  erlang  is  equal to one call connect-hour, or 1 traffic

unit (TU), or 36 HCS, or 36 CCS, or 3600 connect seconds (CS), measured over a

one hour period of time.  In this paper, all traffic is measured in Erlangs.

Introduction to traffic analysis.                                       Page 3

B. Z. Lederman I.T.T. World Communications

Once the amount of traffic is known, and the number of lines which are to

carry  this  traffic  is  known, the Erlang B formula is used to calculate the

amount of blocking, or loss, incurred.  The formula is:

N

_A_

N!

B = ------------------- = B(N,A)

_N_    i

\    _A_

/__   i!

i=0

Where   A = Traffic Presented in Erlangs

N = Number of Servers (trunks) (Must be > 0)

B = Blocking

The blocking B (also written as B(N,A) to  indicate  the  blocking  for  given

values  of  N  and A) can be a value between 0 and 1:  zero blocking means all

traffic goes through, a B of 1 means that all traffic is  blocked  (none  goes

through).   A  value  of  0.5  means  one  half,  or 50%, is blocked (50% goes

through).  Don't be scared off by the  formula:   the  traditional  method  of

solution  has been with books of tables containing values of B for given N and

A, but now that computers are generally available (I assume that most  persons

reading  this  paper  have access to one), the formula may be easily solved by

computer.  One might ask what the procedure is if  one  knows  the  amount  of

traffic  expected,  and  has  determined  an acceptable value for B, as in the

present example.  The formula above cannot be  used  to  solve  for  N,  given

values  of B and A, so one must solve iteratively, by either moving around the

table in the book, or, on a computer, entering the amount of traffic,  setting

the  value  of  N  to  1,  and solve for B;  if B is less than or equal to the

desired B, then the answer has been obtained:  if not, increase the value of N

by   one,  and  try  again.   Fortunately,  computers  don't  get  "bored"  by

repetition.

To return to the example, let us suppose that 5.3 erlangs of traffic were

measured  during  the  test  period.   One  may  wish to determine the cost of

providing various levels of service, thus:

Introduction to traffic analysis.                                       Page 4

B. Z. Lederman I.T.T. World Communications

Number of             Grade of

trunks    Blocking    Service

--------------------------------

1       84.13%

2       69.03%

3       54.95%

4       42.13%

5       30.87%

6       21.43%     1 in    5

-------------------------------

7       13.96%

8        8.46%

9        4.75%     1 in   20

10        2.45%

11        1.17%     1 in  100

12         .51%

-------------------------------

13         .21%

14         .08%     1 in 1000

15         .03%

The grade of service is a term  used  to  count  the  number  of  lost  calls,

generally  one-in-a-whole  number  (one  does not see it expressed as one lost

call in 57 2/3).  This term is not widely used, and the  values  above  (which

are   approximates)  are  given  only  to  familiarize  the  reader  with  the

terminology in use.  From the above list, one can then add up the cost of each

line  and  modem,  and  balance  it  against  the possible increase or loss in

revenue for each level of service.  This is where traffic analysis  ends:   it

only  provides  the  numbers  upon  which  a  decision  can be made as to what

constitutes an acceptable loss during the busy period.

For situations such as these, it is normal to measure the  traffic  to  a

group  of servers.  In the above case, it would have been necessary to measure

the traffic which  the  telephone  company  is  presenting  to  the  group  of

telephone lines.  While the company might be willing to do this (especially if

it wishes to demonstrate  that  the  customer  requires  more  lines),  it  is

sometimes necessary to measure the traffic from the point where it is received

(in this case, the computer).  This can be done iteratively by  measuring  the

traffic  received,  and calculating the amount of traffic which must have been

presented in order for the amount of traffic received to get through.  If  the

traffic  presented  to  the  group of lines is A, then the traffic lost is the

traffic times the blocking, or

Traffic lost = L = B * A

Similarly, the traffic received is the traffic presented minus the loss, or

Traffic received = R = ( 1 - B ) * A

Introduction to traffic analysis.                                       Page 5

B. Z. Lederman I.T.T. World Communications

One must work iteratively, which is time consuming when only books  of  tables

are  available,  but the use of the computer makes this much easier.  Once the

traffic presented has been calculated, the actual blocking is also obtained.

It has been said that one usually calculates  blocking  for  the  busiest

hour  of the day:  this is to determine the number of servers required for the

heaviest load.  One could also calculate the blocking for the busy  hour  from

the  point of view of determining just how bad things are going to be at their

worst.  If a day's traffic is measured for each hour and  plotted,  one  might

see  something  like  Figure 4.  This is a typical business day, where traffic

picks up in the morning as the work day begins, drops somewhat at  lunch,  and

peaks  in the afternoon.  One can simply take the peak at 5:00 PM (1700 hours)

as the worst case, or calculate the blocking  hour-by-hour  to  determine  how

often the system is blocking above the desired level.

It should also be noted that the formula given assumes an infinite number

of  sources generating the traffic:  this is why only the volume of traffic is

required, and not the number of users.  If the traffic is generated be a fixed

number  of  sources, the actual blocking will be slightly less than that given

by the formula.  This is only important if  the  number  of  sources  is  only

slightly  larger  than  the  number of servers, and, in any case, the blocking

calculated assuming an infinite source can be taken  as  a  worst  case,  thus

supplying a slight additional safety factor.  Of course, if one knows that the

traffic will be generated by a fixed small number of sources, that places  the

upper  limit  on  the number of servers required.  In the above example, if it

were known that the  5.3  erlangs  of  traffic  were  being  generated  by  10

customers  (there being only 10 customers allowed to call this facility), then

there is no reason to provide more than 10 telephone  lines,  and  the  actual

blocking  with  10 lines should be zero, not 2.45%.  Most applications are not

this simple, however, and traffic analysis allows  one  to  obtain  reasonable

answers  to  problems  where  the number of requests for service may not be as

well known.

When blocking is high, the system is considered to be  congested  as  the

requests  for service cannot be satisfied.  Whether this is bad or not depends

upon one's point of view.  In  the  above  example,  if  one  is  supplying  a

time-sharing  service,  and  is  competing with others supplying this service,

then the number of lines should be selected to provide low blocking, otherwise

potential  customers  will  be  frustrated  at  not being able to get into the

system, and may cease to use the service.  From  the  point  of  view  of  the

person  sypplying  the lines and modems, low blocking implies that some of the

lines will often be idle;  a situation of high blocking would insure that  the

lines  would  be  busy  most of the time.  Consider the data from the previous

example in this manner:

Introduction to traffic analysis.                                       Page 6

B. Z. Lederman I.T.T. World Communications

Number                           Time any

of                 Traffic       trunk

trunks  Blocking     Received     is busy

------------------------------------------

1      84.13%         0.84        84.1%

2      69.03%         1.64        82.1%

3      54.95%         2.39        79.6%

4      42.13%         3.07        76.7%

5      30.87%         3.66        73.3%

6      21.43%         4.16        69.3%

------------------------------------------

7      13.96%         4.56        65.1%

8       8.46%         4.85        60.6%

9       4.75%         5.05        56.1%

10       2.45%         5.17        51.7%

11       1.17%         5.24        47.6%

12        .51%         5.27        43.9%

------------------------------------------

13        .21%         5.29        40.7%

14        .08%         5.29        37.8%

15        .03%         5.29        35.3%

It can be seen that as blocking goes up,  the  percentage  of  time  that  the

trunks  in  use  will  be  busy  goes  up;  as blocking goes down, the traffic

received approaches the traffic presented, and the  percentage  of  time  that

each  trunk  is  busy decreases.  Note also that the amount of time all trunks

are busy is exactly equal to the blocking.  This must be  taken  into  account

when  deciding what level of service to provide, by accounting for the cost of

the server vs.  the amount of additional traffic (or the benefit of user's not

seeing  a  busy  condition) provided by additional servers.  Alternatively, if

traffic is highly peaked and one can  be  certain  that  the  requestors  will

continue to call until served, then a high level of blocking may be chosen for

the busy hour to force the traffic load to distribute over time, and result in

greater  utilization  of fewer circuits.  If we take the example presented but

now say that the callers are branch offices reporting their daily transactions

to  a  main office at the end of the day, and the calls are placed by computer

so there are no operators to become  annoyed,  then  there  is  no  reason  to

provide  enough circuits for all branches to be able to call in at 6:00 PM and

then have them idle for the rest of the night.  One  would  instead  choose  a

number  of trunks which might have a high blocking at 6:00 PM (in this case, 3

or 4 trunks), and then measure the traffic to see that blocking falls off to a

low  value  by  11:00 PM, so that there is a good probability that all offices

can report in before processing ends  for  the  day.   This  would  force  the

traffic  to  distribute  over  a longer period of time, and one would obtain a

greater utilization of the circuits, as they will be busy a greater percentage

of the time.

If, for a given use, you do choose a low value of blocking for  the  busy

hour,  then  you  must  also  be  prepared  to  demonstrate why the additional

Introduction to traffic analysis.                                       Page 7

B. Z. Lederman I.T.T. World Communications

circuits are required to provide the desired level of service  should  someone

unfamiliar  with traffic analysis look at the circuits during the day and find

some of them idle.  The greatest difficulty in introducing traffic analysis to

people  is  convincing them that the statistical approach is correct, and that

the extra circuits are required to reduce blocking,  even  if  at  times  they

appear unused.

Another approach is to calculate the number of trunks needed  now  for  a

given  amount  of  blocking,  and  then add a percentage to account for future

growth:  thus if one determined that 5% blocking is  acceptable,  requiring  9

trunks  now,  one might add a 10% margin (.9 additional trunks, rounded up) or

10 trunks, so that future growth would not raise the blocking too much.   This

gives  one  some  lead  time  in  ordering  extra  circuits  when traffic does

increase.

The accuracy of these results should be examined.  In the above examples,

the  tables  were  divided  into  3 sections.  For 1 to 6 trunks, the blocking

predicted is probably too low, as the Erlang B formula does  not  account  for

re-tries, and when blocking is great, the requestors will probably try several

times to receive service.  Once the blocking goes much over 20%,  the  formula

is  mathematically correct, but may not account for the real-life situation as

accurately as it does  conditions  of  moderate  or  low  blocking.   This  is

generally  not  a problem, however, because once the blocking goes over 20% it

is generally sufficient to know that the system is congested, and an error  of

a  few  percent  is not of vital importance.  For 7 through 12 trunks (for the

example given), the values should  give  an  accurate  representation  of  the

blocking  to  be  expected  in  a  real  system.  For more than 12 trunks, the

accuracy will depend upon the number of sources generating  the  traffic.   If

the source is large (an infinite source was assumed), the values will be good.

If a small number of requestors are generating the traffic,  then  the  actual

blocking  will be less than that predicted.  Generally, the infinite source is

assumed, as this gives the worst blocking which will be experienced,  and  the

worst  case  condition  is often appropriate in planning a system.  Unless the

actual number of sources is only slightly larger than the number  of  servers,

the  difference  in  blocking  will  be small.  In the case where only traffic

received is known, and the traffic presented is inferred, a larger  margin  of

error should be assumed unless the blocking is low.

A waiting, or queueing, system is one where  the  requestors  wait  until

served.   The  case  covered  by the Erlang formulas is a multi-server, single

First In First Out queue (see Figure 5).  Examples of this are  a  bank  where

customers enter at the end of a single waiting line and the person at the head

of the line goes to the next available teller, a printer queue where files are

entered  at  the back of the queue and the file at the front is printed on the

next available device (even if there is only one printer  serving  the  queue,

the  formulas  work),  a telephone operator queue where persons calling in are

placed at the end of a waiting list and the free operator takes the next entry

off  of  the  front  of  the  list,  a  resturant (without reservations) where

customers entering are placed on a list, and the first party is given the next

available  table,  a  disk  queue  where  requests go on a queue and the first

Introduction to traffic analysis.                                       Page 8

B. Z. Lederman I.T.T. World Communications

request gets the next available controller, etc.  This works only if no-one is

allowed  to "jump" the queue, if requests have different priorities (the queue

is not strictly FIFO), then different (and more complicated) queueing formulas

must be used.

In the place of the simple question "how many requests  are  turned  away

for  lack  of  servers" asked in a blocking system, a queueing system involves

more complicated questions such as "how many requests are made to wait", "what

is  the  length  of  time the average request waits", and "what is the maximum

time a request waits?".  The solution to the first is fairly  simple,  and  is

represented  by  the  symbol P>0, or the probability of waiting more than zero

amount of time.  Obtaining more  information  than  this  is  another  matter.

There are two times of interest;  Tw, which is the average waiting time of all

requests (delayed and non-delayed), and Tw', which is the average waiting time

of  only  those  calls  which  are required to wait (those that are not served

immediately).  It is wise to remind the reader at  this  point  that  in  this

paper,  average  and mean both refer to the simple arithmetic mean (add up all

values and divide by the number of values).  In addition,  one  will  want  to

calculate  the  probability  of  waiting  no more than a given amount of time,

expressed as P>T;  the probability that a request waits more than time T.   It

is  not  possible  to calculate the maximum waiting time (or rather, it is not

necessary;   the  maximum  is  infinity),  but  as  the  time  increases,  the

probability  that  a call will have to wait that amount of time decreases.  In

most cases, it is sufficent to calculate the average, and the time by which  a

given percentage of all requests will be serviced;  i.e.  one can specify that

99% of all requests  will  be  serviced  by  time  T.   This  percentage  will

naturally  vary  with the application, and is one of the quality factors which

must be decided arbitrarily by the person evaluating the application.  If  one

is  designing a query-answer system for a travel agency, one might wish to set

a maximum wait for 90% of the requests, whereas a similar system installed  in

a  hospital operating room might require 99.9% of the inquiries be answered by

a given time to satisfy the users of the system.

In the simple blocking system, it was  possible  to  obtain  an  accurate

answer  knowing  only  the  total amount of traffic, rather than the length of

each request and the number of requests.  In waiting systems, only P>0 can  be

determined  this  way.   To  determine  the  other  values more information is

required, such as the mean holding  time  of  a  request  and  the  number  of

requests,  or  the individual holding times for each request, which eventually

amount to the same thing.  Obviously,  the  time  has  come  to  explain  more

terminology.   The  holding  time  of  a  transaction (a call in the telephone

industry) is the total length of time a particular request  lasts  from  first

initiation  to  final completion.  For a telephone call, this is the time from

the moment you lift the telephone receiver to request service to the time  you

replace  it  (and  actually  a  little  more,  as the used circuits have to be

cleared out when you have finished).  It doesn't  matter  if  your  call  went

thorough  or  not;   all time is the holding time.  This is different from the

effective time (or elapsed time, or connect time, or billable time, which  are

all  different  names  for the same quantity), which is the time starting when

the person you called picks up their telephone, to the time one of  you  hangs

Introduction to traffic analysis.                                       Page 9

B. Z. Lederman I.T.T. World Communications

up,  which  is  the  time  for  which  you  are billed.  The difference at the

beginning of the call is the set-up time;  the time required to establish  the

connection.   For  a  call  which  does  not  go  through,  the set up time is

identical with the holding time.  For a call which is successful,  the  set-up

time  plus  the effective time should equal the holding time.  For a situation

where requests are waiting for service, the waiting time would be part of  the

set-up  time, while the service itself would be the effective time.  To use an

example which will  be  familiar  to  most  readers,  and  to  show  that  the

techniques  are  applicable  to  more  than just communications and computers,

consider a bank where customers wait on a single line, and the person  at  the

front  of  the  line  goes to the next available teller.  The wait on the line

would be the set-up time or time on queue, and the time spent with the  teller

would  be  the  effective  time.  The probability of a person walking into the

bank and finding a teller immediately  available  (i.e.   no  line  of  people

waiting) is one minus the probability of waiting or 1-(P>0).  The average wait

on line is Tw', the average delay of requests which are delayed.  The  average

holding  time  is  the  average  of  the  time  each person spends in the bank

including the time spent at the teller, and the time spent waiting on line, if

any.   The  value  which  is  usually  of  interest  is the probability of the

customer waiting on line more than a given amount of time (or  P>T,  for  some

acceptable value of T).

Some astute readers may have spotted what appears to be  a  contradiction

between the definition of holding time and the definition of blocking systems,

where a request for service must be satisfied immediately, else  it  is  lost.

In  real life, requests are not instantly gratified.  If one dials a telephone

number, one usually waits at least a few seconds for the call to  go  through.

In  practice,  if  this  time is counted as part of the holding time (as is in

fact proper, as the server is busy while the request is processed),  then  the

formulas yield the correct answers.

The probability of delay may be calculated directly from the data, or the

simple blocking formula, Erlang B, may be used as an intermediate calculation.

The probability of any delay, P>0, is also known as C or C(N,A),  and  may  be

calculated from B(N,A) by one of the following:

1                  B(N,A)

C(N,A) = ----------------- = ---------------------

N - A             A

1 + ------------    1 -  - * (1 - B(N,A))

A * B(N-1,A)         N

B = B(N,A) = the Erlang B blocking formula given before.

A = the amount of traffic presented, in Erlangs.

N = the number of servers.

Note that in the first formula, the value of B is for N - 1, one  less  server

___  ____

than  the number of servers in question.  One may also calculate directly from

the data by:

Introduction to traffic analysis.                                      Page 10

B. Z. Lederman I.T.T. World Communications

N

A

------------------

(N - 1)! * (N - A)

C(N,A) = ----------------------------- = P>0

N-1

---    i           N

\     A           A

/    --- + ------------------

---   i!   (N - 1)! * (N - A)

i=0

or

N

N     A

--- * ---

N-A    N!

C(N,A) = ----------------------- = P>0

N-1    i           N

----  A     N     A

\    --- + --- * ---

/___  i!   N-A    N!

i=0

All of the above should work out to the same value, but  it  is  important  to

note  that  the formulas are valid only for N greater than A.  If A is greater

than or equal to N, then C(N,A) is assumed to be 1, and the  delay  approaches

infinity,  as there will not be enough servers to ever get all of the requests

off of the queue.  The only way to obtain time-delay values  for  an  existing

case  where  A > N is to extend the time period under observation to include a

period of time where traffic drops off and  the  servers  catch  up  with  the

queue.   If  this is done, the average traffic presented will be less than the

number of servers, and the time delays can be calculated.

An example of the output of these formulas can be shown  using  the  same

value of traffic (5.3 Erlangs) as in the first example:

N     B       C

-----------------

6    .214    .70

7    .14     .40

8    .085    .216

9    .048    .110

10    .025    .052

11    .012    .023

12    .0051   .0091

13    .0021   .0035

14    .0008   .0013

15    .0003   .0004

Introduction to traffic analysis.                                      Page 11

B. Z. Lederman I.T.T. World Communications

The next step is to obtain the probability  of  waiting  more  than  some

given  amount  of  time  T.   This requires knowing a new item of data, U, the

reciprocal of the average holding time per call.  For example, if one adds  up

all  of  the  holding times for the period in question (from which one usually

obtains the amount of  traffic  presented),  and  divides  by  the  number  of

requests,  the mean holding time is obtained, and the reciprocal of this is U.

If there were 318 minutes of holding time observed during an  hour,  which  is

5.3  Erlangs,  and  this  amount  of  time  was the result of 127 requests for

service, the mean holding time  is  2.5  minutes  (318/127),  and  U  is  0.40

(1/2.5).   When  U  and  C(N,A)  are known, the probability of waiting for any

given T may be obtained from:

-(N - A)UT

P>T = C(N,A) e

It is important to remember that U and T must have the same dimensions;  if  U

is expressed in minutes, T must also be in minutes.  A is always in Erlangs.

One may also calculate Tw, the mean time of delay for all calls, and Tw',

the mean time of delay for delayed calls.

C(N,A)               1

Tw = ---------     Tw' = ---------

U*(N - A)          U*(N - A)

The only difference is that one  takes  into  account  the  portion  of  calls

delayed,  which  is  C(N,A).  Tw is therefore also referred to as the expected

delay, E(W), as it is the average delay for all calls.

As an example of how the delays may be calculated, take the data from the

previous  example  (5.3 Erlangs, U=0.40), and assume that there are 7 servers.

This makes C(N,A) = 0.40, Tw is 0.59 minutes, and Tw' is 1.47 minutes.   Using

the formula for P>T and various values of T, the following is obtained:

T    P>T         T    P>T

------------------------------

.1  .378        3.5  .055

.25 .347        4.0  .041

.5  .301        4.5  .031

.75 .261        5.0  .023

1.0  .227        6.0  .014

1.5  .171        7.0  .0075

2.0  .129        8.0  .0043

2.5  .097        9.0  .0024

3.0  .073       10.0  .0014

As one might expect, as time increases, the probability of waiting  that  much

time  decreases,  and  the  probability of being served by that amount of time

increases.  One can say, for example, that the  probability  of  waiting  more

than  2.5 minutes is .097 which means that just under 10% of the requests wait

Introduction to traffic analysis.                                      Page 12

B. Z. Lederman I.T.T. World Communications

more than 2.5  minutes.   The  expected  waiting  time  is  weighted  for  the

distribution  of  waiting  requests  and  requests  which do not wait (in this

example P>0 is 0.4, so fewer than half of all requests should have any  wait).

If  these  waiting times are too long, then one would chose a larger number of

servers (in this case, 8 or more), and calculate the P>0 and P>T times  again,

to  determine  if  the  waiting  times are now acceptable.  Alternatively, one

might construct a table as before, showing delay  probabilities  for  a  given

amount  of  traffic  (in  this  case, 5.3 Erlangs with U = 0.40) and different

numbers of servers N.  All times are in minutes.

N  P>0   P>.1  P>.5  P>2.5  Tw'   Tw

---------------------------------------

6  .70   .68   .62   .39   3.57  2.5

7  .40   .38   .30   .097  1.47  0.59

8  .216  .20   .14   .023   .93   .20

9  .110  .097  .059  .005   .68   .075

10  .052  .044  .024  .001   .53   .028

11  .023  .019  .009  .0002  .44   .010

It is now simple to balance the cost of the  number  of  servers  against  the

expected  delay,  or  the probability of a wait exceeding a desired limit.  In

this example, if one wanted 99% of all calls to be served  within  30  seconds

(0.5  minutes),  then  P>.5 must be less than 0.01, which requires 11 servers.

If one wanted the expected (average) delay to be 15 seconds or less, 8 servers

are required (0.2 minutes is 12 seconds).

One must also remember to put the results back into the  context  of  the

problem.  If this were a print queue problem, then the values obtained are the

delays before the request is served.  In this case, this is  the  delay  until

the  job  starts to print.  If the time until the job is finished and ready to

be torn off the printer is the desired time, then  the  average  holding  time

(the  average  length  of  a  print job) would have to be added to the average

delay to obtain the average wait for job completion.  It is  a  good  practice

whenever  solving  a  traffic  analysis  problem to refer back to the original

conditions periodically to ensure that the answers make sense when applied.

Hopefully, the reader will now have some idea of the  types  of  problems

which traffic analysis can address.  If a situation where a number of requests

compete for a limited resource can be described in a  manner  consistent  with

the  Erlang  models  presented, then it is reasonably easy to calculate values

for the number of requests which will be turned away, or the  length  of  time

requests will wait.  With these values, one may then make reasonable decisions

about the allocation of these resources, or the need to increase the number of

servers, and by what amount, to reach the desired level of service.

Bibliography

Staff, "Engineering  and  Operations  in  the  Bell  System",  Bell  Telephone

Laboratories, Inc.  Part 5

Introduction to traffic analysis.                                      Page 13

B. Z. Lederman I.T.T. World Communications

Kleinrock, Leonard, "Queueing Systems:  Volume 1 Theory", John Wiley and Sons.

Covers all types of Queues, very advanced mathamatically.

Kleinrock, Leonard, "Queueing Systems:  Volume 2 Computer Applications",  John

Wiley and Sons.  Contains ARPANET examples.

Gorney, Leonard, "Queueing Theory:  A Problem  Solving  Approach",  Petrocelli

Books.  Has programs in TRS-80 Level II Basic.

Zussman,  Ronald,  "Computer  Queuing  Analysis  on  a  Handheld  Calculator",

Computer Design, November 1977 pp.  85-94

-,  "Teletraffic  Engineering  Manual,  Tables  and  Diagrams  for   Switching

Systems", Standard Elektrik Lorenz AG, Stuttgart

-, "Telephone Traffic Theory - Tables and Charts", Siemens  Aktiengesellshaft,

Munich ISBN 3-8009-1163-9

The two works above are books of tables, with explanations of the formulas and

their uses.  They are published in english.

The following Fortran Function  solves  the  Erlang  B  Blocking  formula

without  requiring any factorials or large powers by re-arrainging the formula

into a single series.

FUNCTION ERLB(N,A)

C

C       To solve the Erlang B formula without

C        factorials or large powers. ERLB

C        returns a value between 0 and 1.

C

C	Permision to use this algorithm is granted

C		provided the author is credited.

C

C       B. Z. Lederman

C       I.T.T. World Communications, N.Y.

C

C     TEMP1 gradually decreases in value: if it

C           becomes so small as to no longer be

C           significant, quit early.

C

TEMP1 = 1.

TEMP2 = 1.

C

DO 220 I = N, 1, -1

TEMP3 = FLOAT(I)

TEMP1 = TEMP1 * (TEMP3 / A)

IF (TEMP1 .LE. 0.00001) GO TO 225

220   TEMP2 = TEMP2 + TEMP1

225   ERLB = 1. / TEMP2

RETURN

END