DEVELOPING AN RSX-11M ACP IN A HIGHER ORDER LANGUAGE

Carl T. Mickelson

Goodyear Aerospace Corporation

1210 Massillon Road

Akron, Ohio 44315

ABSTRACT

Developing  an  Ancillary  Control  Processor  (ACP)  in  a higher order

language for  RSX-11M  is  not  difficult.   This  paper  describes  the

development  of an ACP to control a complex permuting memory for a large

scale parallel processor.  The central portions of the ACP  are  written

in  FORTRAN.   The  program  was  originally  written  as an interactive

program  to  aid  in  the  development  and  de-bug  cycle.   Once   the

interactive  implementation  was  completed, the ACP form of the program

was implemented using the same object library that was  used  to  create

the   interactive   program.   The  paper  discusses  the  basic  design

considerations  that  permitted  this  approach  to  be  successful.   A

discussion of some of the implementation details, and how existing DECUS

ACP documentation was used is also included.

INTRODUCTION

In May 1983 Goodyear Aerospace Corporation (GAC) delivered the Massively

Parallel  Processor (MPP) to the NASA Goddard Space Flight Center (GSFC)

in Greenbelt, Maryland.  The  MPP  is  the  largest  and  most  powerful

parallel processor ever built.  The central computational element in the

MPP  is  a  square array of 16,384 bit-serial processing elements (PE's)

with orthogonal nearest-neighbor connectivity  (figure  1).   Performing

all  of its calculations using bit-serial techniques, the MPP is capable

of performing  in  excess  of  6  billion  8-bit  additions  per  second

(1,2,3,4).

Because  of  its parallel architecture, the "natural" memory access mode

for the MPP PE's is to retrieve the same bit of each of 16K operands  in

a   single   memory  cycle.   This  word-parallel  /  bit-serial  memory

organization is in-consistent with the normal bit-parallel / word-serial

memory organization of a conventional computer.  Since the data that  is

processed  by  the  MPP  array  is generally provided by a conventional,

word-serial host computer system, some way must be found to  re-organize

or  "corner-turn"  the  data that will be processed in the array.  It is

also highly desirable that this corner-turning  operation  be  performed

"automatically",   without   the  requirement  to  execute  any  special

re-formatting subroutines.

The solution to this problem is  provided  in  the  MPP  system  by  the

inclusion  of  a  hardware buffer or staging memory inserted in the data

path between the host computer and the array memory  (figure  2).   This

1

staging  memory,  which  can  be  expanded  to  64 megabytes, is used to

permute data to be processed in the array unit as it is moved to or from

the host computer into or out of the array memory.  The  staging  memory

converts  the  bit-parallel  / word-serial host data organization to the

word-parallel / bit-serial  organization  required  in  the  array  unit

(figures  3  and  4).   Which of the 2**29 permutations performed by the

stager is controlled by a set of  890  control  bytes  computed  from  a

description of the data organization provided by the user.

The  purpose  of  this paper is to describe how the calculation of these

control  bytes  is  accomplished  using  an  RSX-11M  Ancillary  Control

Processor  (ACP)  and  to present some of the design considerations that

were pertinent to the development of this ACP.

STAGER CONTROL SOFTWARE

The use of an ACP in this application to provide support for the staging

memory is justified for a number of reasons:

1. The calculation of the stager control constants is complex  and  best

placed in a single program in the system.  This provides the consistency

necessary  to  insure  proper  system  operation  for many different MPP

application programs.

2. The ACP  mechanism  provides  the  capability  to  service  user  I/O

requests  in  a  transparent manner.  All MPP I/O requests are routed to

the MPP device driver.  It is the driver's responsibility to involve the

staging memory ACP for those requests that require its support.

3. The ACP can be used to both create  and  to  manage  an  I/O  process

involving  data  transfers  through the MPP staging memory.  The process

creation or open function  is  used  to  compute  the  hardware  control

parameters  required  to permute data as it is moved through the stager.

These parameters are saved and subsequently used to program  the  stager

hardware  whenever  a staging memory data transfer function is requested

by an application program.  The  I/O  process  is  terminated,  and  the

parameters  deleted  when  a close function is issued by the application

program.

4. The ACP mechanism allows this software to be  developed  without  the

restrictions  that  apply  to  an RSX-11M device driver.  As an ACP, the

program has all the attributes of an RSX-11M task, and can be  overlayed

to  support the complex computations necessary to produce the parameters

needed to program the staging memory.

SOFTWARE DESIGN OBJECTIVES

A  number  of  design  objectives  were  identified  early  in  the  ACP

development  effort.   The  first  such  objective  was  to  develop the

parameter  calculation  algorithm  in  the  form   of   an   interactive

higher-order  language  program.  This type of development vehicle would

provide the tool necessary to easily debug and check out the algorithm.

2

A second objective was to use a single object library  to  support  both

the interactive form of the parameter calculation algorithm and the ACP.

Once the algorithm was developed and checked out, it was an objective to

use  the  same  object  modules  to construct the staging memory ACP for

on-line use with the MPP device driver.   Such  an  objective  could  be

achieved   if   individual  modules  could  be  driven  from  either  an

interactive  main  program  or  from  a  routine  providing  an  on-line

interface to the user's I/O request (figure 5).

If  these  objectives could be achieved, both forms of the program could

be supported from a single set  of  source  files,  easing  the  program

maintenance  effort  and  eliminating  the  necessity  of re-writing the

parameter calculation algorithm in assembly  language.   However,  these

benefits are not achievable without considering the environment in which

the ACP must execute.

All  interactive  FORTRAN  I/O must be eliminated in the ACP form of the

program.  The I/O support required by FORTRAN  would  make  the  overall

program  too large for an ACP.  Any error conditions detected by the ACP

should be returned to the user program issuing the I/O request by way of

the QIO I/O status block.  Error messages, issued directly to  the  user

should be minimized.

The  ACP  must  fit into 12 Kwords of virtual address space, including a

memory allocation bitmap for data buffers in the staging memory.

I/O processes  opened  by  the  ACP  must  maintain  all  the  necessary

parameters  and information needed to program stager permutations at run

time.  Multiple occurrences of such I/O processes should be able  to  be

created  and maintained simultaneously.  Because of the possibility of a

variable number of concurrently opened processes, the ACP data base  can

not  be pre-sized.  Therefore, the parameters needed to maintain the I/O

processes would be kept in Executive Pool.

Finally, to be easily used to support data transfers  to  and  from  the

stager,  the  I/O  process  information  must be accessible to a FORTRAN

subroutine in the ACP that programs and controls the stager hardware.

SOFTWARE DESIGN APPROACH

The first step in developing the MPP staging memory control software was

to identify all the parameters needed from  the  user  to  describe  the

required  data  permutation  to  be  performed  in the hardware.  In the

interactive form of the program, these parameters are read either from a

data file or from the user's terminal.  In the ACP form of the  program,

the same parameters are supplied in the user's data buffer identified in

an "open" function QIO.

The  required  parameters, organized in a defined data structure for the

"open" QIO, are used to initialize a FORTRAN common data  base  for  the

hardware control parameter calculations.  The interactive program uses a

prompt/response  subroutine to acquire the needed startup data.  The ACP

uses a small assembly language subroutine to transfer the  initial  data

3

from the user's QIO buffer into the FORTRAN common data base.

The  ACP  version  of the program allocates a "window block" (similar in

concept to the Files-11 ACP) in Executive Pool and uses  this  block  to

collect  the  data  that  will  be needed later to move data through the

staging memory.  The window block contains a pointer to a  second  block

of pool used to store the computed hardware control parameters.

Both  the  interactive and the ACP versions of the program calculate the

hardware control parameters required for  the  user's  data  permutation

using  the  low  level  subroutines  that  are part of the common object

module library.  These subroutines are implemented as FORTRAN  functions

that  return  a status flag and/or error code in the event of a detected

error condition.

The interactive program main module uses the error information to inform

the user directly, via FORTRAN I/O statements, about the error  and  its

cause.   The  ACP  version  of the program returns the error code to the

user program that issued the QIO via the upper, device  dependent  byte,

of  the  first  word  of  the  IOSB.   This design technique allowed all

FORTRAN I/O requirements to be satisfied in a top-level routine so  that

the low level routines were useable in both program versions.

Upon successful completion of the requested I/O process "open" function,

the interactive program produces two output files.  The first contains a

copy  of  the parameter calculation results in binary form.  The purpose

of this file will be described later.

The second file is a listing of the  calculation  results  that  can  be

interpreted  by  the user to determine the memory packing efficiency and

I/O transfer rate achievable for his specified permutation.   A  listing

file  option  permits  the  binary data to be presented in its entirety.

This feature was used during the  program  development  effort  to  help

debug the parameter calculation algorithm.

The  ACP  program  version  simply  stores the results of the successful

parameter calculation step in the data block  allocated  in  pool.   The

final "open" QIO step performed in the ACP is to build a FORTRAN calling

sequence  in the I/O process window block.  All the parameters necessary

to properly move data through the  staging  memory  are  identified  and

located  through  addresses  contained  in  this  calling  sequence.  By

passing the address of the calling  sequence  contained  in  the  window

blocks  of  different  I/O processes to a single FORTRAN subroutine, the

staging memory can be made to  perform  any  previously  specified  data

permutation  in response to a data transfer QIO request issued by a user

program.

An alternate "open" function is  also  available  that  permits  a  user

program  to  create  a  stager I/O process without need of executing the

parameter calculation algorithm at  run  time.   This  alternate  "open"

function  makes  use  of  the  binary  output  data file produced by the

interactive program.  The user program issuing such  a  "fast-open"  QIO

provides  a  data  buffer  to the ACP that contains a copy of the binary

4

data file specifying the desired permutation.  The ACP uses the data  to

build  the  same pool data structures that are constructed by the "open"

function described earlier.  Once these structures are  created,  it  is

not possible to determine how a particular I/O process was created.  The

data  structures  contained  in  Executive Pool are identical for either

case.

ACP IMPLEMENTATION APPROACH

After developing and debugging the interactive staging memory  parameter

calculation  program according to the design objectives and restrictions

outlined earlier, development of  the  ACP  version  was  started.   The

sample  ACP  package  (5)  was  used  as  a  starting point.  The manual

accompanying the prototype ACP is an excellent source of information  on

how an ACP interfaces with the RSX-11M QIO mechanism.

The  specific  type of ACP used for this application is characterized in

the ACP documentation as a user ACP.  This  ACP  type  was  selected  in

order  to  avoid  making changes to either the RSX module DRQIO, the QIO

directive processor, or to the MCR mount processor (MOU).

In order to support  the  ACP,  the  user  written  device  driver  that

processes  I/O requests for the MPP performs all parameter checks on all

I/O functions including those  that  will  be  dispatched  to  the  user

written  ACP.   These  parameter  checks  include the re-mapping of data

buffer addresses provided in QIO requests to the driver or the ACP.   In

order  for a data transfer to be accomplished properly, the user program

virtual buffer address must be  mapped  into  a  kernal  APR  6  virtual

address,  with  relocation  bias,  before  the QIO is queued to the ACP.

When the ACP processes such an I/O request, and  subsequently  routes  a

data  transfer  QIO to the device driver, the buffer address provided in

the QIO is actually the previously relocated address of the user program

data buffer.  Because of this, the device driver, in its  QIO  parameter

checking  code,  must  not  relocate  the  data buffer address in an ACP

issued data transfer QIO.

RSX-11M, by its design, requires that an ACP must be "known" or  mounted

to  a  device  in order that "virtual" I/O functions can be issued for a

device.  This mounting operation is the system step that establishes the

link between a particular device and the  ACP  that  supports  it.   The

tasks  that perform this function are the enable (ENA) and disable (DIS)

tasks provided in the sample ACP package.  These programs were  directly

applicable to this ACP development effort with only minor changes in the

default  ACP  name coded into the ENA source.  Reference 6 provides some

patches that should be applied to these programs before their use.

The basic  interface  with  RSX-11M  was  provided  by  the  sample  ACP

prototype.   This  module  serves as the ACP's main program.  The stager

control parameter calculation algorithm  developed  in  the  interactive

program  is used for I/O process creation in the ACP.  This algorithm is

called whenever an "open" QIO is issued to the ACP.   The  window  block

created  for each of these processes is linked to the second LUT word of

the issuing task's logical unit table entry for the LUN assigned to  the

5

staging memory.  This link is then used to access I/O process parameters

whenever  a data transfer function is issued to the ACP.  If such a link

does not exist, the ACP returns  an  error;  no  I/O  process  has  been

created.

The  ACP  is  implemented  as  an  overlaid  program,  written mostly in

FORTRAN.  The overlay structure is divided  primarily  between  the  I/O

process creation or open functions and the data transfer functions.

The  I/O  process  open  function  uses  an  assembly language module to

transfer the user's process description parameters  from  the  QIO  data

buffer.  These parameters are used to initialize the same FORTRAN common

region  that is filled in via user dialog in the interactive development

program.  The ACP  then  executes  the  hardware  parameter  calculation

algorithm  to  fill  in  the I/O process window block.  Error conditions

detected in the user's initial data are returned to the user program  as

an ACP error code.

After  completing  the hardware parameter calculation algorithm, the ACP

fills in a 14 element FORTRAN subroutine calling sequence in the process

window block.  This calling  sequence  is  used  by  the  data  transfer

function module to move data into and out of the staging memory.

Assembly  language  modules are used to allocate space in executive pool

for the parameter buffers referenced in the process window block.  These

routines are also used to create an I/O process from pre-computed,  user

supplied  hardware parameters from a saved binary data file as described

earlier.

A data transfer request causes the ACP to find  the  appropriate  window

block  linked to the second LUT word of the user program's assigned LUN.

The main ACP module then calls a FORTRAN subroutine  to  do  the  actual

data transfer using the calling sequence in the selected window block.

Even  though  the  ACP is overlaid, a properly written user program will

not suffer excessive I/O processing delays.  If  all  the  required  I/O

processes  are  created  and  then  used  repetitively,  the ACP overlay

structure will not need to be re-loaded between QIO requests.

Some special steps are performed at compile  and  task  build  times  to

minimize  the  size of the ACP.  First, the FORTRAN language modules are

complied with all traceback code supressed.   Second,  FORTRAN  run-time

error  messages are limited only to an announcement of the runtime error

number and the program counter where the error occurred.   This  limited

error  report  is  produced  by  task  building  the ACP with the F4POTS

library module $SHORT.  Finally, since the ACP is  primarily  a  FORTRAN

program  that performs no FORTRAN I/O, certain of the F4POTS I/O support

modules may be shortened by using selected module versions contained  in

the  F4PNIO  library.  This library can be created in UIC [1,1] from the

FORTRAN language distribution file F4PNIO.OBJ.  Details of  the  use  of

these  modules  can be found in the FORTRAN 4 Plus, or FORTRAN 77 User's

Guides.

6

DEBUGGING AIDES AND SURPRISES

Debugging an ACP has some interesting side-effects.  Since  an  ACP  has

all  the  attributes of a task, it can be debugged using ODT as a "user"

program.  Additionally, if the "debuggable" ACP is built  with  default,

rather than an elevated, priority, the debugging effort can generally be

accomplished  without  a  severe  impact  on  system  performance.   Two

complications arose in debugging this particular ACP because the program

was overlaid and it was written in FORTRAN, requiring the use of some of

the FORTRAN run-time support routines.

The first complication was eased significantly  by  making  use  of  the

information  contained in reference 7.  Knowing the techniques described

in this article, it should have been easy to start up the ACP and  begin

a  debugging  session.   Since  an ACP is not normally started up like a

user task, but rather by a request directive (RQST$) issued by  the  ACP

enable  task  (ENA),  the  ACP  normally  runs  with TT0: as its default

terminal device.  If the ACP is going  to  be  debugged,  and  TT0:  and

another  system  terminal  are  available,  it is best to start the user

program that will be using the ACP on the second terminal and  use  TT0:

to  enter  ODT  debug  commands.   If only one terminal is available, it

should be TT0:.  Debugging efforts can be successful if care is taken in

entering commands since the terminal is shared by both the user  program

and ODT.

However,  attempting  to  debug  the  ACP with ODT proved to be somewhat

challenging.  When the enable task (ENA) was used to initiate the ACP to

be debugged, it seemed that the program would not execute.   If  started

from a terminal with a RUN command, the ACP would properly prompt for an

ODT command and would run correctly when started with the "G" command.

As  a  result  of  a  telephone  conversation  with Ralph Stamerjohn, at

Monsanto, it was finally realized that the ACP, when started by the  ENA

task,  was  running  with  its terminal set to the CO: device.  When the

task was started, ODT would attempt to input a command from  CO:.   With

the  console  logger  (COT...)  active,  the  read  request  received an

end-of-file and the ACP exited!  If COT... was first  stopped  with  the

command  SET  /NOCOLOG,  the  ACP  built with ODT executed properly when

started with the ENA task.  Note that during  normal  system  operation,

using  an ACP without ODT, if the ACP should abort, the console log will

have a message containing information identifying the cause and location

of the error.

The second check-out complication encountered in developing the ACP  was

related  to  the  FORTRAN OTS interface.  The FORTRAN subroutine used by

the ACP to move data through  the  staging  memory  has  three  optional

variable length arrays passed to it when it is called with the parameter

list  built  in the I/O process window block.  Certain permutations that

have no need to use these arrays use an address value of -1 to  indicate

that the parameters are not defined.

When  five  calls were made to this subroutine for permutations that did

not use the optional parameters, the ACP would abort.  The  problem  was

7

traced to a FORTRAN OTS run-time check done on variable length passed to

a  subroutine.   Since the address -1 is an illegal PDP-11 word address,

the OTS would detect and count an array size error for each  unspecified

array parameter upon entering the subroutine.  When the task error count

was  exceeded, the ACP aborted.  The fix for this particular problem was

to call the ERRSET subroutine during  ACP  initialization  to  turn  off

error  logging and limit checking for this error condition.  In this way

the ACP would continue to execute correctly for  permutations  that  did

not require the optional parameters.

FURTHER INFORMATION

Selected  modules  from  the  ACP, together with the manuscript for this

paper are available on  the  RSX  Sig  Tape  for  the  Fall  1983  DECUS

Symposium.   The  modules included on the tape are limited only to those

top-level routines  that  characterize  both  the  interactive  and  ACP

program  forms.   The  missing modules are limited to those that perform

the actual stager hardware parameter calculations.   These  modules  are

peculiar  to  the ACP developed for the MPP.  They provide no additional

specific insights into the design of an ACP for implementation  using  a

higher order language.

REFERENCES

1.  Batcher,  Kenneth  E.,  "Massively Parallel Processor (MPP) System",

AIAA Computers in Aerospace Conference II, October 1979

2. Tsoras, John, "The Massively Parallel Processor (MPP), Innovation  in

High  Speed  Processors",  AIAA  Computers  in Aerospace Conference III,

October 1981

3. Gilmore, Paul  A.,  "The  Computer  MPP",  International  Society  of

Photogrammetry  and  Remote  Sensing  Commission  II Symposium, August -

September 1982

4. Burkley, John T. and Mickelson, Carl T., "MPP:  A  Case  Study  of  a

Highly  Parallel  System",  AIAA  Computers  in Aerospace Conference IV,

October 1983

5. Stamerjohn, Ralph, "Up Your ACP" (et al), DECUS Spring 1980 Symposium

RSX SIG Tape, UIC=[346,100]

6.  Stamerjohn,  Ralph,  "Bug  Fixes  for   Sample   ACP",   DECUS   RSX

Multi-Tasker, Vol 16, #2, August 1982

7.  Johnson,  Kenneth,  "Using  ODT  in  Overlaid  Programs",  DECUS RSX

Multi-Tasker, Vol 13, #2, August 1980

8. "MPP System User's Guide", Goodyear Aerospace Corporation,  GER-1714,

April 1983

FIGURE CAPTIONS

Figure 1 - MPP Array Unit

Figure 2 - MPP System Block Diagram

Figure 3 - MPP Staging Memory

Figure 4 - Stager Reformatting Example

8

Figure 5 - Stager Software Structure

9