CROSS

Ted Hess, Digital Equipment Corporation

author

Joseph M. Newcomer, Carnegie-Mellon University

documentation

19 February 1981

The  material  used  in  this  specification,  including  but  not limited to

construction times and operating speeds, is for  informational  purposes  only.

All  such material is subject to change without notice.  Consequently DEC makes

no claim and shall not be liable for its accuracy.

This document describes the operational procedures for  CROSS,  an  assembler

which  operates  on  the  DECsystem-10/20  systems, translating source code for

6502, 6800, 6805, 6809, 8080, 8085, MCS-48,  Z80,  1802,  3870/F8[future],  and

8008 micro-processors into binary files suitable for absolute load.

All of the appendices to this manual and the index were constructed by Joseph

M. Newcomer,  Carnegie-Mellon  University  Computer  Science  Department.   The

information for the appendix on object  formats  is  derived  from  information

contributed  by  Ted  Hess, but the current editor takes responsibility for any

inaccuracies which may have crept in during the transcription.

Additional work has been done on CROSS by Mark L. Sproul and Mel  J. Pleasant

at Rutgers University.

CROSS-Microcomputer cross assembler                     1

1. Introduction

CROSS  is  an  assembler  which operates on the DECsystem-10/20 and assembles

code for many currently available micro-processors. With the exception  of  the

variations  listed  in this manual, CROSS implements the features of the PDP-11

macro  assembler  for  RSX-11D.    No  attempt  was  made  to  provide   source

compatibility  between  CROSS  and  other micro- processor assemblers.  Rather,

CROSS has been designed to provide consistent, powerful features for all of the

mnemonics associated with each micro-processor.

Numerous people have worked on CROSS over the years; appropriate credits  are

recorded in the change log of the source.

While CROSS is designed to be language compatible with the MACRO-11 language,

its   PDP-10  operating  environment  necessitates  a  PDP-10  style  operating

procedure.

Expression evaluation has been updated to provide the following features:

- Default radix is decimal.

- @<expr> indicates octal radix, same as to ^O<expr>

- %<expr> indicates binary (6502/6800/6809 only), same as to ^B<expr>

- %nn indicates register value nn (all others)

- $<expr> indicates hexadecimal radix

- ^H<expr> indicates hexadecimal radix

- <expr>^ indicates that the evaluated expression is to be  divided  by

256. (high byte)

- <expr>\ indicates that the evaluated expression is to be taken modulo

256 (low byte).

The  assembler  makes  two  passes over the source, outputting the binary and

listing files during pass two and appending a symbol or cross  reference  table

to  the  listing file.  PDP-10 memory is dynamically allocated for large symbol

table storage and macro storage.

Introduction

CROSS-Microcomputer cross assembler                     2

2. Syntax differences

The  syntax  of  CROSS  is more compatible with that of MACRO-11 than that of

most manufacturer's assemblers.  This means  that  you  will  not  be  able  to

compile  straight  8080,  Z80, 6502 or 6800 code with this assembler.  However,

the transliterations are reasonably straightforward and are described for  each

machine  in  the  appendices.  In general, you must specify registers with a %-

directive.

The machine which differs most from the manufacturer's  specification  of  an

assembler  syntax  is  the Zilog Z80.  This is because CROSS implements the Z80

syntax as a superset of the 8080 syntax.   Consequently,  the  syntax  of  most

operations  follows  that  of  the  8080.    In  addition,  some  Z80 mnemonics

conflicted with 8080 mnemonics; the Z80 has been assigned new  mnemonics  where

these  conflicts  existed.  Any Z80 operation which has no 8080 counterpart has

also been assigned a mnemonic;  some  correspond  to  the  Zilog  specification

(wherever  this was possible) and most do not.  Consequently, a complete cross-

reference between the Zilog syntax  and  the  CROSS  transliteration  has  been

provided.

Syntax differences

CROSS-Microcomputer cross assembler                     3

3. Operating procedures

3.1. Loading CROSS

CROSS is run by typing the monitor command:

@CROSS                 ;tops20

.R CROSS                ;tops10

3.2. Initial dialogue

When  the assembler is ready to accept the user's command string, it types an

asterisk.  The user must now type the command string on the same line.

The general form of the command string is:

binary output, listing output = source input(s)

In addition, the command(s) may be stored in a file (default extension  .CCL)

and read in by specifying:

@filename

Each input/output specification consists of the group:

dev:filnam.Ext[proj.,prog.]/switch:arg

- dev is:

DSK             for disk

PTR             for paper tape reader

PTP             for high speed punch

TTY             for the user's terminal

LPT             for a queued line printer

- If not specified, the device is assumed to be the disk.  Other system

devices may also be used, depending upon the system configuation.

- filnam is the file name of the appropriate file.

- ext  is  the file name extension for that file.  If is not specified,

the assumptions are:

binary string:  .BIN (absolute)

listing string: .LST

source string:

.M05            Motorola 6805

Operation

CROSS-Microcomputer cross assembler                     4

.M48            Intel MCS-48 family

.M65            MOS Technology/Synertek/Rockwell 6502

.M68            Motorola 6800

.M69            Motorola 6809 family

.M80            8080/Z80/8085

.M88            8008, 8080 mnemonics

.M08            8008 , Intel mnemonics

.M18            CPD1802, COSMAC

.MF8            Fairchild F8/Mostek 3870 [future]

- [proj.,prog.]  is the project-programmer number assigned for the disk

area to be used.  If not specified, the user's local area is assumed.

Once specified, the number pair becomes the  new  default  assumption

for  the following files. (Use translate command on TOPS20).  At CMU,

the CMU PPN format is accepted.

- Switches are specified by preceding  each  with  a  slash  character.

Switch  arguments  are  preceded  by colons.  In addition to explicit

specifications of the switches, a default  set  of  switches  may  be

specified with a SWITCH.INI file (chapter 6, page 13).

/M05            source is Motorola 6805 code

/M48            source is Intel MCS-48 code

/M65            source is 6502 code

/M68            source is Motorola 6800 code

/M69            source is Motorola 6809 code

/M80            source is Intel 8080, Intel 8085 or Zilog Z80 code

/M88            source is 8008 using 8080 compatible mnemonics

/M08            source is Intel 8008 original Intel mnemonics

/M18            source is 1802 code (RCA COSMAC)

/MF8            source is Fairchild F8/Mostek 3870 [future]

/LI             simulate .LIST directive (see chapter 4).

/NL             simulate .NLIST directive (see chapter 4).

Operation

CROSS-Microcomputer cross assembler                     5

/EN             simulate .ENABL directive (see chapter 5).

/DS             simulate .DSABL directive (see chapter 5).

/CRF            append  cross  reference  table  to the listing files

(see chapter 4).

/PTP            binary output is ASCII coded hex.  This switch  takes

an  optional format type as a parameter, as /PTP:fmt.

The format types are:

COS             Uses RCA COSMAC format  (see  section

XV.1.1)

INT             Uses   Intel   Intellec  (CP/M  .HEX)

format.

KIM             Uses MOS Technology format with start

address in the last record (this  was

the  default  CROSS format for a long

time).

MOS             Uses  MOS  Technology   format   with

record count in last record.

MOT             Uses  Motorola  Exorcisor  format (S1

and S9 records)

If no argument is specified, the format is based upon

the  language  input  type  determined  by  the  file

extension:

Extension                  PTP Format

.M65                       MOS

.M68,.M69                  MOT

.M80,.M08,.M88,.M48        INT

.M18                       COS

.MF8                       KIM (future)

/N              suppress  error  messages and expanded summary to the

terminal.

/OCT            listing format in octal - hex is default

/I              output binary in image mode (one byte per 36 bit PDP-

10 word). This switch must be used to  copy  directly

to paper tape.

/P              pack binary output.  This is the default case and the

switch is not needed.

/CDR            simulate   card  reader  input.    This  is  actually

Operation

CROSS-Microcomputer cross assembler                     6

independent  of  the physical input device but causes

the assembler to ignore all characters  on  a  column

after the 72nd.

/SOL            sequence output lines (see chapter 4).

/GNS            generate new source (see chapter 4).

/NSQ            suppress listing of PDP-10 style seq. Numbers.

/EQ             equate  the  following  symbols  to  zero.  This is a

method for introducing conditional arguments  without

creating special parameter tapes.  "/EQ:sym1:sym2" is

the  equivalent of the source statements "sym1=0" and

"sym2=0".

/LEN            set the page length for the output listing.

/WID            set the page width for the output listing.

Operation

CROSS-Microcomputer cross assembler                     7

4. Listing control

CROSS has two categories of listing controls:  MACRO-11 compatible, which may

be  specified  either  in  source  statements  or  in the command string, and a

superset which is limited to the command string.

4.1. Source statement listing control

Listings are controlled at source statement level through   the  use  of  the

.LIST  and  .NLIST  directives.    These are complementary in that any function

which can be enabled by a .LIST can  be  disabled  by  a  .NLIST.    To  reduce

redundancy,  the following descriptions will emphasize the .LIST while implying

the .NLIST complement.

The following are the allowable arguments for the .LIST directive along  with

the default settings (li for .LIST and nl for .NLIST):

The  first  group  controls  intra-line listing.  Suppression of any of these

will cause the assembler to utilize the space by moving the  remainder  of  the

line over.

SEQ             list  the sequence numbers.  These numbers represent the source

line  numbers  and  are  replaced  by  the  nesting  level   in

parentheses for generated lines (macros, etc.).

LOC             list the value of the location counter.

BIN             list the binary code generated by the assembler.

SRC             list  the  source  statement.    This is included primarily for

completeness.

COM             list the comment field.

BEX             list binary code which requires more  than  one  listing  line.

suppression  of  this  is  particularly  useful in reducing the

length of listing required for long .ASCII strings.

The following arguments pertain to entire statements:

MB              list macro binary and call only - no source

MD              list macro definitions.

MC              list macro calls.

ME-nl           list macro expansions.  The default causes the expansion to  be

suppressed  in the interest of supply economy.  ME controls all

generated text (.MACROs, .REPTs, and .IRPs).

Caution - due to their dual nature, .REPTs  and  .IRPs  qualify

both  as  definitions  and  calls.  CROSS treats MD and MC as a

pair in these cases, suppressing the listing if either is set.

.LIST/.NLIST

CROSS-Microcomputer cross assembler                     8

CND             list conditionals.  Suppression causes unsatisfied conditionals

and all conditional directives (whether satisfied or not) to be

not listed.

SYM             append a symbol table to the listing file.

TTM             list  in  terminal  format  (one  column  of  binary to a line,

truncation after 72 columns).  Default setting is determined by

the output device specified:  enabled if the physical device is

TTY: and disabled (line printer mode) in all other cases.

TOC-nl          list a table of contents during pass one.  This consists of the

.SBTTL directives being listed at the beginning of the  listing

file.    Each  line consists of the line number and all text to

the right of the .SBTTL directive.

LD-nl           list  listing  directives.    Specifically,  this  causes   all

.LIST/.NLIST  directives  which have no arguments to be listed.

its primary use is in debugging listing control.

.LIST and .NLIST directives with no arguments perform an over- ride function.

They control a listing  "level"  which  is  decremented  for  each  .NLIST  and

incremented for each .LIST.  The level count is used as a three way switch:

1. Zero (initial condition):  no special action.

2. Less  than  zero:    unconditionally suppress the listing except for

lines containing errors.

3. Greater than zero:  list unconditionally.

.LIST/.NLIST are normally paired to return the level  count  to  zero.    the

.NLIST/.LIST  sequence  is  used  to suppress a range which is normally listed,

while the .LIST/.NLIST sequence is used to over-ride a range which is  normally

suppressed,  such  as  in  a  macro  expansion.    The level count allows macro

expansions to use the listing control and return it to its previous state.

4.2. Command string equivalents

The command string contains the equivalent of all the above .LIST and  .NLIST

functions as well as some that are not available at source level.  Indeed, many

of  the  source level arguments are of primary interest at command string level

but can be used in the source to modify default settings.

The command  string  equivalents  of  .LIST  and  .NLIST  are  /LI  and  /NL,

respectively.    Arguments  are  specified by preceding them with colons.  Thus

/NL:bex:cnd is the equivalent of .NLIST CND,BEX.

In all cases, command string switches override their source level equivalents

(as well  as  previous  occurrences  in  the  command  string).    Effectively,

specification of a function in a command string will cause all its source level

equivalents  (and  their  complements) to be ignored.  i.e. /LI:me would enable

the listing of macro expansions and cause all ".LIST/.NLIST ME"  directives  to

.LIST/.NLIST

CROSS-Microcomputer cross assembler                     9

be ignored.

/LI  (no  arguments) causes everything to be listed, while /NL suppresses the

listing of all but error lines, the  symbol  table,  and  the  cross  reference

tables.

4.3. Other listing controls

The  following swithces are specified in output side of the command string by

preceding the mnemonic by a slash:

/OCT            list in octal - default is hex

/N              suppress error messages to the terminal (unless it is also  the

listing device).

/CRF            append  a  cross  reference  table  to the listing file.  CROSS

accomplishes this in the following manner:

1. The listing file is  generated  in  the  normal  way

during  pass  two,  using  the name specified by the

user.

2. Simultaneously, a temporary file (xxxCRF.TMP,  where

xxx  is  the  job number) is created.  This contains

encoded cref information.

3. At the end of pass two, the temporary file  is  read

and  the  cref  information  is  integrated into the

symbol table.

4. The references are appended to the listing file.  if

insufficient core was available to build the  table,

the   process   is  repeated.    When  through,  the

temporary file is deleted.

The cross reference table format is as follows:

1. The symbol itself.

2. Its  symbol  table  value,  unless  .NLIST sym   was

specified.    Therefore  the  symbol table itself is

always pre-empted.

3. All references to the symbol, except  that  no  more

than  one  reference  to a given source line will be

listed.  symbols in generated text always  refer  to

the previous source line.

4. Defining occurrences are flagged with a "#".

5. Destructive  references  are  flagged  with  a  "*".

These are defined to be all symbols  encountered  in

.LIST/.NLIST

CROSS-Microcomputer cross assembler                    10

op-code fields which store into a memory location or

a register.

6. As  many  references  per  listing line as possible,

depending upon the "ttm" mode, are used.   This  can

be increased by suppressing "sym".

/SOL            Sequence output lines.  Listings are normally source oriented:

1. Sequence numbers reflect the source line number.

2. Macro  expansion  lines have their nesting level, in

parentheses, in the sequence number field.

3. Lines containing nothing but a  form  feed  are  not

listed   but   cause   the  sequence  number  to  be

incremented.

4. Page   numbers   reflect   physical   input    pages

(incremented  only  upon  encountering a form feed).

Pages forced by the assembler have a -n  to  reflect

an  extension  page.    i.e.    If the assembler had

encountered 3  form  feeds  and  was  on  its  third

listing  page  since the last one, it would be "page

4-2".

when /SOL is specified, listings are output oriented:

1. Listed lines have consecutive  numbers,  except  for

binary extensions.

2. Page numbers reflect the number of pages listed.

/GNS            Generate a new source.  This causes the listing file to take on

the appearance of a source file.  specifically, it:

1. Suppresses  the  listing  of  headers,  error flags,

summary blocks, and any other output over which  the

user otherwise would have no control.

2. Simulates a /NL:seq:loc:bin:bex:me.

3. therefore,  by itself, it recreates the source file.

Normal  usage  would  have  supplementary   switches

following  it.    i.e.  /gns/nl:md:mc  /li:me  would

replace macros, repeats, and irps by their generated

code,  /gns/nl:cnd  would  purge  conditionals,  and

/gns/li:seq  would be an expensive way of creating a

sequence numbered listing of the source file.

/LEN:num        Sets the page length to the specified number.  This allows  the

user  to  take  advantage  of  longer  page  formats  for  some

.LIST/.NLIST

CROSS-Microcomputer cross assembler                    11

printers, or to produce listings suitable for viewing a page at

a  time on a video terminal.  At CMU, the default length is 80,

for the Dover using SAILA8.  The normal default is 58.

/WID:num        Sets the page width to the specified number.  This  allows  the

user   to   take  advantage  of  wider  page  formats,  or  get

presentable listings on devices which  have  narrower  margins,

such as home microcomputer printers.  At CMU, the default width

is 120, for the Dover using SAILA8.  The normal default is 132.

.LIST/.NLIST

CROSS-Microcomputer cross assembler                    12

5. Enable/Disable control

As  an  alternative  to  the  proliferation  of  directives,  CROSS  uses one

complementary pair, .ENABL/.DSABL, with arguments.  Their       command  string

equivalents  are  /EN  and  /DS.    The relationship between command string and

source statement specification, as well as their argument specification, is the

same as in .LIST/.NLIST.

M85             allow 8085 opcodes (RIM/SIM) in .M80 files

Z80             allow Z80 opcodes in .M80 files

FPT             floating point  truncation.    ".ENABL FPT"  is  equivalent  to

".TRUNC" while ".DSABL FPT" is equivalent to ".ROUND".

ABS             absolute assembly

AMA             absolute memory addressing

NPP             ".ENABL NPP"  causes suppression of the output of the following

code, while ".DSABL NPP" causes its resumption.

ERF             error flags can  be  selectively  suppressed  (or  enabled)  by

specifying   ERF,  directly  followed  by  the  selected  error

characters.  "/ds:ERFzm" would cause "m" and "z" errors  to  be

ignored,  "/ds:ERF"  would  cause all errors to be ignored, and

"/en:ERFa"  would  cause"a"  flags  (argument  errors)  to   be

flagged.

LC              lower  case  to upper case translation of input; the default is

.ENABL LC so listings contain output in the same case as it was

input.

LSB             local symbol block

TIM             timing information - causes number of states to  be  output  on

the  listing.  Alas, for most machines this data is not entered

in the internal tables.

REG             default register names

.ENABL/.DSABL

CROSS-Microcomputer cross assembler                    13

6. User-defined switch defaults

CROSS  is  capable  of  accepting user-defined switch defaults.  Any switches

that are valid in a command string may be specified in a  special  file  called

SWITCH.INI.    When  CROSS  is  run,  it  will look for this file in the user's

directory. At CMU, the logical device HOM: is checked, and if a SWITCH.INI file

is found there, it will be used instead of the one on the aliased or  connected1

directory                                                                       .    If  the  file  is  not found, then standard defaults (as defined

elsewhere in this document) are assumed.  If SWITCH.INI is found, but no  CROSS

switches are found, default settings are assumed.

To  specify  defaults,  use the editor of your choice to create a file called

SWITCH.INI.  This file must contain a line of the following format:

CROSS/switch1/switch2...../switchn

continuation lines are permitted.  Specifically, a line containing a terminal

hyphen is assumed to be followed by a continuation line.

Of course CROSS does not consider the hyphen to be a part of any switch.

Note that this feature is a subset of the facility offered normally by use of

SWITCH.INI files.

1

At CMU on TOPS-20, the logical device HOM: can be  assigned  to  the  user's

login directory by the LOGIN.CMD file.

SWITCH.INI

CROSS-Microcomputer cross assembler                    14

7. Macro libraries

Macro libraries are used by the .MCALL directive.  As in Macro-11, the format

is:

.MCall arg1,arg2,...

where  the  arguments are macro names required for the program.  However, CROSS

recognizes that users may  wish  to  have  different  libraries  for  different

machines,  but  a single user may be using CROSS for several different machines

at the same time.  Therefore, instead of the simple  rule  that  SYSMAC.SML  is

used,  if  it  exists,  and  then  trying  SYS:SYSMAC.SML, CROSS uses language-

dependent libraries.  The extension of the library is the appropriate extension

for the language being compiled. Thus, if you have a file whose extension,  for

reasons known only to yourself, is ".M80" but which contains 6809 code, and you

have  overridden  the  default language selection with a /M69 switch, the macro

library will have extension ".M69".  The extensions are the default  extensions

discussed on page 3, and in the rules below are indicated by language.

The search rules are thus:

DSK:SYSMAC.SML

DSK:SYSMAC.language

SYS:SYSMAC.SML

SYS:SYSMAC.language

SWITCH.INI

CROSS-Microcomputer cross assembler                    15

I. General notation

Each section of an appendix describes one aspect of a given machine

I.1. Machine State

This  section defines the registers or other status of the machine, including

condition code bits.  This status is referred to in  the  descriptions  of  the

instructions,  and  thus  may  include  status  or  registers  which  cannot be

specified in the syntax to CROSS.

I.2. Notation

This  section  describes  the  notation  used  in  the  description  of   the

operations.  For example, a set of instructions might all use registers R1, P2,

C3  and  D4, or immediate data.  This collection might be called src-opnd, thus

reducing the number of cases which need to appear in the opcode  list.    These

names, and other notations, appear here.

I.3. Syntax

Here,  the actual CROSS representation details, particularly when they differ

from those of the manufacturer's standard assembler, are discussed.

The names of locations, registers, operands, etc. which correspond to machine

state which can be referred to are given here.

I.4. Opcodes

Here, the complete list of mnemonics  are  given.    This  list,  the  syntax

section, and the notation section should allow one to write actual CROSS code.

The comments in the opcode list describe the operation; while not intended to

replace  the  manufacturer's or other reference manual, they do provide a quick

check on what is happening.

The semantics are expressed either as comments, or as fragments  of  code  in

the Bliss language.  In Bliss, the "." operator means "contents of", and a name

stands for its "address".  Thus, a description such as

A = .A + 1

means that register (or location) A gets the contents of register (or location)

A plus 1.

Bit fields are indicated by <> pairs, of the form <position,size>.  Thus,

.A<1,4>

means  the contents of the register (or location) named by A<1,4>, i.e., bits 1

thru 5.  Bit 0 is the low order bit.

Multibyte and other  complex  fields  are  indicated  by  catenation,  the  |

operator; thus, the HL register pair of the 8080 is indicated by %H|%L.

Boolean  truth  and falsehood is determined by the value of the low-order bit

of the expression used as the boolean; thus 0, 2, 4, 6, etc.  are  all  "false"

SWITCH.INI

CROSS-Microcomputer cross assembler                    16

values.

The  following  table  gives the operation names used for the descriptions of

the instructions and explanations of their meanings.

and             bitwise and

ash             arithmetic shift, positive values shift left, negative values

shift right

geq             greater than or equal, signed, two's complement

gequ            greater than or equal, unsigned

gtr             greater than, signed, two's complement

gtru            greater than, unsigned

leq             less than or equal, signed, two's complement

lequ            less than or equal, unsigned

lss             less than, signed, two's complement

lsh             logical shift, positive values shift left, negative values

shift right.

lssu            less than, unsigned, two's complement

not             bitwise complement

or              bitwise or

rot             rotate, positive values are rotate left, negative values are

rotate right

xor             bitwise exclusive or

+               two's complement add

-               two's complement subtract

*               two's complement multiply

<=>             swap; the contents of the two locations named are swapped.

SWITCH.INI

CROSS-Microcomputer cross assembler                    17

II. 8080 syntax

II.1. Machine State

The registers for the 8080 are defined as:

- %A - the accumulator

- %B - register B

- %C - register C

- %D - register D

- %H - register H, high-order memory reference register

- %L - register L, low-order memory reference register

- %SP - the stack pointer register

- %PSW - the processor status, or F register

Register  pairs  are  written  in  the  description  of  each  instruction by

concatenating two names with a vertical bar, e.g., %H|%L is the 16-bit value in

the HL register pair.  Parentheses are  sometimes  added  for  visual  clarity,

e.g., ".(%H|%L)".  The flags in the status register are represented by:

- c - the carry bit

- p - the parity bit.  True if the last operation had odd parity, false

if even parity.

- z - the zero bit.  True if the last operation had a zero result.

- s - the sign bit.  True if the last operation had a negative result.

- ie - the interrupt enable flag.

In each description of an instruction that alters the condition codes, either

an  explicit  assignment  is  written  (e.g., "c=0") or the condition codes are

listed in curly brackets.  In this case,  it  is  implied  that  c  is  set  to

indicate  carry  or  borrow (arithmentic operations), z is set if the result is

zero, s is set if the result is negative, and p is set if the parity is even.

II.2. Notation

To reduce the size of the description, several notational conventions will be

established for desribing the operands:

- i - represents an immediate operand which is 8 bits wide.

- m - represents a memory operand which is 16 bits wide.

- M - represents an implied memory operand addressed  by  the  register

pair %H|%L.

- MEM - represents all of addressible memory.

- DEV - represents all addressible devices.

- v - a special 3-bit value used for the RST instruction

- opnd  -  a  general  operand, either register or implied memory.  For

registers, it will be expressed as %r for one of the 8-bit registers,

and for memory it will be expressed as M in  the  instruction  syntax

8080

CROSS-Microcomputer cross assembler                    18

and MEM[.(%H|%L)] in the description of the instruction.

- %rp  - a register pair from the set {%B|%C, %D|%E, %H|%L %SP}.  These

would be written, in the assembler, as %B, %D, %H or %SP.

- %rpn - a register pair from the set {%PSW, %A, %B|%C, %D|%E, %H%L}.

- Written as %PSW, %SP, %B, %D or %H.

II.3. Syntax

- %A,%B,%C,%D,%E,%H,%L - 8-bit registers

- %PSW,%SP,%B,%D,%H - where appropriate, 16-bit registers

- M - memory, addressed via %H|%L

- Immediate data is represented by the expression which generates it

II.4. Opcodes

The opcodes that are recognized for the 8080 are shown below.   The  function

is given in a BLISS-like notation (the dot operator means "contents of").

ACI i           %A = .%A + i + .c; {c,z,s,p}

ADC opnd        %A = .%A + opnd + .c

ADD opnd        %A = .%A + .opnd; {c,z,s,p}

ADI i           %A = .%A + i; {c,z,s,p}

ANA opnd        %A = .%A and .opnd; c = 0; {z,s,p}

ANI i           %A = .%A + i; c = 0; {z,s,p}

CALL m          MEM[.%SP-1], MEM[.%SP-1] = .pc; %SP = .%SP - 2; pc = m;

CC m            if .c then CALL m

CM m            if .s then CALL m

CMA             %A = not .%A; {note: c,z,s,p not changed}

CMC             c = not .c

CMP opnd        .%A - .opnd; {c,z,s,p}

CNC m           if not .c then CALL m

CNZ m           if not .z then CALL m

CP m            if not .s then CALL m

CPE m           if .p then CALL m

CPI i           .%A - i; {c,z,s,p}

CPO m           if not .p then CALL m

CZ m            if .z then CALL m

DAA             %A = DecimalAdjust(.%A); {c,z,s,p}

DAD %B          %H|%L = .(%H|%L) + .(%B|%C); {c}

DAD %D          %H|%L = .(%H|%L) + .(%D|%E); {c}

DAD %SP         %H|%L = .(%H|%L) + .%SP; {c}

DCR opnd        opnd = .opnd - 1; {z,s,p}

DCX %rp         %rp = .(%rp) - 1

DI              ie = 0

EI              ie= 1;

HLT

IN i            %A = .DEV[i];

INR opnd        opnd = .opnd + 1; {z,s,p}

8080

CROSS-Microcomputer cross assembler                    19

INX %rp         %rp = .(%rp) + 1;

JC m            if .c then pc = m;

JM m            if .s then pc = m;

JMP m           pc = m;

JNC m           if not .c then pc = m;

JNZ m           if not .z then pc = m;

JP m            if not .s then pc = m;

JPE m           if .p then pc = m;

JPO m           if not .p then pc = m;

JZ m            if .z then pc = m;

LDA m           %A = .MEM[m];

LDAX %B         %A = .MEM[.(%B|%C)];

LDAX %D         %A = .MEM[.(%D|%E)];

LHLD m          %H = .MEM[m]; %L = .MEM[m+1];

LXI %rp,m       %rp = m;

MOV %r1,%r2     %r1 = .%r2;

MOV m,%r        MEM[m] = .%r;

MOV %r,m        %r = .MEM[m];

MVI opnd,i      opnd = i;

NOP             ;

ORA opnd        %A = .%A or .opnd; c = 0; {z,s,p}

ORI i           {%A = .%A or i;}; c = 0; {z,s,p}

OUT i           DEV[i] = .%A;

PCHL            pc = .(%H|%L);

POP %rpn        %rpn = .MEM[.%SP], .MEM[.%SP + 1]; %SP = .%SP + 2;

PUSH %rpn       %SP = .%SP - 2; MEM[.%SP], MEM[.%SP + 1] = .%rpn;

RAL             c|%A = .(c|%A) lsh 1;

RAR             c|%A = .(c|%A) lsh -1;

RC              if .c then RET;

RET             pc = .MEM[.%SP], .MEM[.%SP+1]); %SP=.%SP + 2;

RLC             c = .%A<7,1>; %A = .%A lsh 1

RM              if .s then RET;

RNC             if not .c then RET;

RNZ             if not .z then RET;

RP              if not .s then RET;

RPE             if .p then RET;

RPO             if not .p then RET;

RRC             c = .%A<0,1>; %A = .%A lsh -1;

RST v           CALL v lsh 8;

RZ              if .z then RET;

SBB opnd        %A = .%A - .opnd - .c; {c,z,s,p}

SBI i           %A = .%A - i - .c; {c,z,s,p}

SHLD m          MEM[m] = .%H; MEM[m+1] = .%L;

SPHL            %sp = .(%H|%L);

STA m           MEM[m] = .%A;

STAX %B         MEM[.(%B|%C)] = .%A;

STAX %D         MEM[.(%D|%E)] = .%A;

STC             c = 1;

SUB opnd        %A = .%A - .opnd; {c,z,s,p}

SUI i           %A = .%A - i; {c,z,s,p}

XCHG            %D <=> %H; %E <=> %L;

XRA opnd        %A = .%A xor .opnd; c = 0; {z,s,p}

8080

CROSS-Microcomputer cross assembler                    20

XTHL            MEM[.%SP - 1] <=> %H; MEM[.%SP] <=> %L;

8080

CROSS-Microcomputer cross assembler                    21

III. Z80 syntax

III.1. Introduction

The  Z80  portion of CROSS is not intended to correspond to the Zilog syntax;

instead, it is treated as an extension of  the  8080  operations  described  in

section  II.    This means that new opcode designators have been invented where

appropriate.

III.2. Machine State

The Z80 registers are designated exactly like the 8080  registers,  with  the

following additions:

- %X - index register X (16 bits)

- %Y - index register Y (16 bits)

- iv - the interrupt vector base address (8 bits)

- ra - the dynamic RAM refresh address (8 bits)

III.3. Notation

The following notational extensions are made to the 8080 description:

- b - A bit position, 0-7, for bit operations

- opnd  -  In  addition to %r and M, indexed operands are permitted for

all instructions which use opnd; these are expressed to the assembler

as d(X)  or  d(Y),  where  d  is  an  8-bit  displacement.    In  the

description, they are interpreted as ".%X+d" or ".%Y+d".

- %rpx - Register pair including %X, i.e., {%B|%C, %D|%E, %X, %SP}

- %rpy - Register pair including %Y, i.e., {%B|%C, %D|%E, %Y, %SP}

III.4. Opcodes

The extended opcodes for the Z80 are given below.  Those which are underlined

are  mnemonics  which  are neither 8080 instructions nor those defined by Zilog

for the Z80; they have been invented for CROSS.    A  complete  transliteration

from Zilog assembler format to CROSS syntax is given in section III.5.

BIT b,opnd      z = not .opnd<b,1>

CCD             .%A - .MEM[.(%H|%L)]; (%H|%L) = .(%H|%L) - 1;

(%B|%C) = .(%B|%C) - 1; {z,s,p}

CCDR            until .%A eql .MEM[.(%H|%L)] or .(%B|%C) eql 0 do begin

.%A - .MEM[.(%H|%L)]; (%H|%L) = .(%H|%L) - 1;

(%B|%C) = .(%B|%C) - 1 end; {c,z,p}

CCI             .%A - .MEM[.(%H|%L)]; (%H|%L) = .(%H|%L) + 1;

(%B|%C) = .(%B|%C) - 1; {z,s,p}

CCIR            until .%A eql .MEM[.(%H|%L)] or .(%B|%C) eql 0 do begin

.%A - .MEM[.(%H|%L)]; (%H|%L) = .(%H|%L) + 1;

(%B|%C) = .(%B|%C) - 1 end; {c,z,p}

CNO             CPO;

Z80

CROSS-Microcomputer cross assembler                    22

CO              CPE;

DADC %rp        (%H|%L) = .(%H|%L) + .%rp + .c; {c,z,n,p}

DADX %rpy       %X = .%X + .%rpy; {c,n}

DADY %rpx       %Y = .%Y + .%rpx; {c,n}

DJNZ i          if .%B @z(#) 0 then pc = .pc + 2 + i

DSBC %rp        (%H|%L) = .(%H|%L) - .%rp - .c; n = 1; p = 0; {c,z}

EXAF            (%A|%PSW) <=> (%A'|%PSW');

EXX             (%B|%C) <=> (%B'|%C'); (%D|%E) <=> (%D'|%E'); (%H|%L) <=>

(%H'|%L')

IM0

im = 0;

IM1

im = 1;

IM2             im = 2;

IND

INDR

INI

INIR

INP

JMPR i          pc = .pc + 2 + i;

JNO m           JPO m;

JO m            JPE m;

JRC i           if .c then JR i;

JRNC i          if not .c then JR i;

JRNZ i          if not .z then JR i;

JRZ i           if .z then JR i;

LBCD m          (%B|%C) = .MEM[m];

LDAI            %A = .iv;

LDAR            %A = .ra;

LDD             MEM[.(%D|%E)] = .MEM[.(%H|%L)]; (%D|%E) = .(%D|%E) - 1;

(%H|%L) = .(%H|%L) - 1; (%B|%C) = .(%B|%C) - 1; n = 0;

LDDR            until .(%B|%C) eql 0 do begin MEM[.(%D|%E)] = .MEM[.(%H|%L)];

(%D|%E) = .(%D|%E) - 1; (%H|%L) = .(%H|%L) - 1;

(%B|%C) = .(%B|%C) - 1; end;

LDED m          (%D|%E) = .MEM[m];

LDI             .MEM[.(%D|%E)] = .MEM[.(%H|%L)]; (%D|%E) = .(%D|%E) + 1;

(%H|%L) = .(%H|%L) + 1; (%B|%C) = .(%B|%C) - 1; {p}

LDIR            until .(%B|%C) eql 0 do begin MEM[.(%D|%E)] = .MEM[.(%H|%L)];

(%D|%E) = .(%D|%E) + 1; (%H|%L) = .(%H|%L) + 1;

(%B|%C) = .(%B|%C) - 1; end;

LIXD m          %X = .MEM[m+1], .MEM[m];

LIYD m          %Y = .MEM[m+1], .MEM[m];

LSPD m          %SP = .MEM[m+1], .MEM[m];

NEG             %A = - .%A; p = 0; {c,z,s}

OUTD

OUTDR

OUTI

OUTIR

OUTP

PCIX            pc = .%X;

PCIY            pc = .%Y;

RALR

Z80

CROSS-Microcomputer cross assembler                    23

RARR

RES b,opnd      %opnd<b,1> = 0;

RETI            return from interrupt

RETN            return from nonmaskable interrupt

RLCR

RLD             decimal arithmetic

RNO             RPO;

RO              RPE;

RRCR

RRD             decimal arithmetic

SBCD m          MEM[m] = .(%B|%C);

SDED m          MEM[m] = .(%D|%E);

SET b,opnd      opnd<b,1> = 1;

SIXD m          MEM[m] = .%X;

SIYD m          MEM[m] = .%Y;

SLAR

SPIX            %SP = .%X;

SPIY            %SP = .%Y;

SRAR

SRLR

SSPD m          MEM[m] = .%SP;

STAI            iv = .%A;

STAR            ra = .%A;

XTIX            %X <=> MEM[.%SP];

XTIY            %Y <=> MEM[.%SP];

III.5. Z80 Transliterations

The  following  transliterations  are  necessary  because  of the differences

between the Zilog definition of the instructions and the CROSS syntax.  This is

indexed  alphabetically  by  the  Zilog  operations  in  order  to   facilitate

translation of Zilog assembly programs to CROSS.

Zilog           CROSS ADC i    ACI i

ADC (HL)        ADC M

ADC HL,rp       DADC %rp

ADC (IX+d)      ADC d(X)

ADC (IY+d)      ADC d(Y)

ADC r           ADC %r

ADD i           ADI i

ADD (HL)        ADD M

ADD HL,rp       DAD %rp

ADD (IX+d)      ADD d(X)

ADD (IY+d)      ADD d(Y)

ADD r           ADD %r

AND i           ANI i

AND (HL)        ANA M

AND (IX+d)      ANA d(X)

AND (IY+d)      ANA d(Y)

AND r           ANA %r

BIT b,(HL)      BIT b,M

BIT b,(IX+d)    BIT b,d(X)

BIT b,(IY+d)    BIT b,d(Y)

Z80 transliterations

CROSS-Microcomputer cross assembler                    24

CALL m          CALL m

CALL C,m        CC m

CALL M,m        CM m

CALL NC,m       CNC m

CALL NZ,m       CNZ m

CALL NZ,m       CNZ m

CALL P,m        CP m

CALL PE,m       CPE m

CALL PO,m       CPO m

CALL Z,m        CZ m

CCF             CMC

CP i            CPI i

CP (HL)         CMP M

CP (IX+d)       CMP d(X)

CP (IY+d)       CMP d(Y)

CP r            CMP %r

CPD             CCD

CPDR            CCDR

CPI             CCI

CPIR            CCIR

CPL             CMA

DAA             DAA

DEC (HL)        DCR M

DEC IX          DCX %X

DEC IY          DCX %Y

DEC (IX+d)      DCR d(X)

DEC (IY+d)      DCR d(Y)

DEC rp          DCX %rp

DEC r           DCR %r

DI              DI

DJNZ d          DJNZ d

EI              EI

EX AF,AF'       EXAF

EX DE,HL        XCHG

EX (SP),HL      XTHL

EX (SP),IX      XTIX

EX (SP),IY      XTIY

EXX             EXX

HALT            HLT

IM 0            IM0

IM 1            IM1

IM 2            IM2

IN A,i          IN i

IN r,(c)

INC (HL)        INR M

INC IX          INX %X

INC IY          INX %Y

INC (IX+d)      INR d(X)

INC (IY+d)      INR d(Y)

INC rp          INX %rp

IND             IND

INDR            INDR

Z80 transliterations

CROSS-Microcomputer cross assembler                    25

INI             INI

INIR            INIR

JP m            JMP m

JP M,m          JM m

JP NC,m         JNC m

JP NZ,m         JNZ m

JP NZ,m         JNZ m

JP P,m          JP m

JP PE,m         JPE m

JP PO,m         JPO m

JP Z,m          JZ m

JR d            JMPR d

JR M,m          JRM m

JR NC,m         JRNC m

JR NZ,m         JRNZ m

JR NZ,m         JRNZ m

JR P,m          JRP m

JR PE,m         JRPE m

JR PO,m         JRPO m

JR Z,m          JRZ m

LD A,(m)        LDA m

LD A,(BC)       LDAX %B

LD A,(DE)       LDAX %D

LD A,i          MVI %A,i

LD A,r          MOV %A,%r

LD (m),A        STA m

LD (m),BC       SBCD m

LD (m),DE       SDED m

LD (m),HL       SHLD m

LD (m),IX       SIXD m

LD (m),IY       SIYD m

LD (m),SP       SSPD m

LD (BC),A       STAX %B

LD (DE),A       STAX %D

LD HL,(m)       LHLD m

LD (HL),i       MVI i

LD (HL),r       MOV %r,M

LD I,A          STAI

LD IX,(m)       LIXD m

LD IX,m         LXI %X,m

LD (IX+d),i     MVI i,d(X)

LD (IX+d),r     MOV %r,d(X)

LD IY,(m)       LIYD m

LD IY,m         LXI %Y,m

LD (IY+d),i     MVI i,d(Y)

LD (IY+d),r     MOV %r,d(Y)

LD r,A          MOV %r,%A

LD r,i          MVI %r,i

LD r,(HL)       MOV %r,M

LD r,(IX+d)     MOV %r,d(X)

LD r,(IY+d)     MOV %r,d(Y)

LD r,r          MOV %r,%r

Z80 transliterations

CROSS-Microcomputer cross assembler                    26

LD BC,(m)       LBCD m

LD DE,(m)       LDED m

LD HL,(m)       LHLD m

LD SP,(m)       LSPD m

LD IX,(m)       LIXD m

LD IY,(m)       LIYD m

LD rp,m         LXI %rp,m

LD SP,HL        SPHL

LD SP,IX        SPIX

LD SP,IY        SPIY

LDD             LDD

LDDR            LDDR

LDI             LDI

LDIR            LDIR

NEG             NEG

NOP             NOP

OR i            ORI i

OR (HL)         ORA M

OR (IX+d)       ORA d(X)

OR (IY+d)       ORA d(Y)

OR r            ORA %r

OUT (i),r

OUT i,A         OUT i

OUTD            OUTD

OUTDR           OUTDR

OUTI            OUTI

OUTIR           OUTIR

POP IX          POP %X

POP IY          POP %Y

POP rp          POP %rp

PUSH IX         PUSH %X

PUSH IY         PUSH %Y

PUSH rp         PUSH %rp

RES b,(HL)      RES b,M

RES b,(IX+d)    RES b,d(X)

RES b,(IY+d)    RES b,d(Y)

RES b,r         RES b,%r

RET             RET

RET M           RM

RET NC          RNC

RET NZ          RNZ

RET NZ          RNZ

RET P           RP

RET PE          RPE

RET PO          RPO

RET Z           RZ

RETI            RETI

RETN            RETN

RL (HL)         RALR M

RL (IX+d)       RALR d(X)

RL (IY+d)       RALR d(Y)

RL r            RALR %r

Z80 transliterations

CROSS-Microcomputer cross assembler                    27

RLA             RAL

RLC (HL)        RLC M

RLC (IX+d)      RLC d(X)

RLC (IY+d)      RLC i<d>(Y)

RLC r           RLC %r

RLCA            RLC

RLD             RLD

RR (HL)         RARR M

RR (IX+d)       RARR d(X)

RR (IY+d)       RARR d(Y)

RR r            RARR %r

RRA             RAR

RRC (HL)        RRC M

RRC (IX+d)      RRC d(X)

RRC (IY+d)      RRC d(Y)

RRC r           RRC %r

RRCA            RRC

RRD             RRD

RST v           RST v

SBC i           SBI i

SBC (HL)        SBC M

SBC HL,rp       DSBC %rp

SBC (IX+d)      SBC d(X)

SBC (IY+d)      SBC d(Y)

SBC r           SBC %r

SCF             STC

SET b,(HL)      SET b,M

SET b,(IX+d)    SET b,d(X)

SET b,(IY+d)    SET b,d(Y)

SET b,r         SET b,%r

SLA (HL)        SLAR M

SLA (IX+d)      SLAR d(X)

SLA (IY+d)      SLAR d(Y)

SLA r           SLAR %r

SRA (HL)        SRAR M

SRA (IX+d)      SRAR d(X)

SRA (IY+d)      SRAR d(Y)

SRA r           SRAR %r

SRL (HL)        CB3E

SRL (IX+d)      DDCBdd3E

SRL (IY+d)      FDCBdd3E

SRL r           CB??

SUB i           SUI i

SUB (HL)        SUB M

SUB (IX+d)      SUB d(X)

SUB (IY+d)      SUB d(Y)

SUB r           SUB %r

XOR i           XRI i

XOR (HL)        XRA M

XOR (IX+d)      XRA d(X)

XOR (IY+d)      XRA d(Y)

XOR r           XRA %r

Z80 transliterations

CROSS-Microcomputer cross assembler                    28

IV. 8008

The 8008 is described by two different sets of opcodes.  The one set is based

upon  the  8080  style of coding, and is a subset of the operations.  The other

set is based upon the original set of 8008 mnemonics  as  described  by  Intel.

Rather  than  give  complete  descriptions  of  these  operations,  the user is

referred back to the 8080 descriptions.  What  this  section  will  give  is  a

listing  of the available operations and a cross reference to the original 8008

mnemonics.

Note that the 8008 is "software compatible" with the 8080; the same mnemonics

will produce different bit patterns for the 8008 and the 8080.

IV.1. 8080 subset

The instructions which are implemented for the 8008 are:

ACI     ADC     ADD     ADI     ANA     ANI     CALL    CC      CM

CMP     CNC     CNZ     CP      CPE     CPI     CPO     CZ      DCR

HLT     IN      INR     JC      JM      JMP     JNC     JNZ     JP

JPE     JPO     JZ      MOV     MVI     NOP     ORA     ORI     OUT

RAL     RAR     RC      RET     RLC     RM      RNC     RNZ     RP

RPE     RPO     RRC     RST     RZ      SBB     SBI     SUB     SUI

XRA     XRI

The set of original 8008 operations are defined as:

8080            Original 8008

ADC             ACA, ACB, ACC, ACD, ACE, ACH, ACL

ACI             ACI

ADD             ADA, ADB, ADC, ADD, ADE, ADH, ADL

ADI             ADI

CALL            CAL

CNC             CFC

CPO             CFP

CM              CFS

CNZ             CFZ

CMP             CPA, CPB, CPC, CPD, CPE, CPH, CPL

CPI             CPI

CC              CTC

CPE             CTP

CP              CTS

CZ              CTZ

DCR             DCB, DCC, DCD, DCE, DCH, DCL

HLT             HLT

INR             INB, INC, IND, INE, INH, INL

IN              INP

JNC             JFC

JNZ             JFZ

JPO             JFP

JM              JFS

JMP             JMP

JC              JTC

JPE             JTP

8008

CROSS-Microcomputer cross assembler                    29

JP              JTS

JZ              JTZ

MOV             LAA, LAB, LAC, LAD, LAE, LAH, LAL, LAM,

LBA, LBB, LBC, LBD, LBE, LBH, LBL, LBM,

LCA, LCB, LCC, LCD, LCE, LCH, LCL, LCM,

LDA, LDB, LDC, LDD, LDE, LDH, LDL, LDM,

LEA, LEB, LEC, LED, LEE, LEH, LEL, LEM,

LHA, LHB, LHC, LHD, LHE, LHH, LHL, LHM,

LLA, LLB, LLC, LLD, LLE, LLH, LLL, LLM

MVI             LAI, LBI, LCI, LDI, LEI, LHI, LLI, LMI

ANA             NDA, NDB, NDC, NDD, NDE, NDH, NDL, NDM

ANI             NDI

NOP             NOP

ORA             ORA, ORB, ORC, ORD, ORE, ORH, ORL, ORM

ORI             ORI

OUT             OUT

RAL,RAR         RAL, RAR

RET             RET

RNC             RFC

RPO             RFP

RP              RFS

RNZ             RFZ

RLC,RRC         RLC, RRC

SUB             SBA, SBB, SBC, SBD, SBE, SBH, SBL, SBM

SBI             SBI

SBC             SUA, SUB, SUC, SUD, SUE, SUH, SUL, SUM

SUI             SUI

XRA             XRA, XRB, XRC, XRD, XRE, XRH, XRL, XRM

8008

CROSS-Microcomputer cross assembler                    30

V. 8048

V.1. Machine State

- R0-R7 general registers R0 thru R7; R0 and R1 can be used as indirect

memory reference registers.

- Bus<0,8> Bus register port (latched output, unlatched input)

- MB - Temporary location which holds result of last SEL mb.

- MEM - Addressible memory

- P1<0,8>-P7<0,8> I/O or expansion ports P1 thru P7

- PSW<0,8>  Program  status  word.    The following bits in the PSW are

named:

* SP = PSW<0,3> Stack pointer

* RB = PSW<4,1> Register bank select

* f1 = PSW<4,1> Software flag F1 (same as RB)

* f0 = PSW<5,1> Software flag F0

* ac = PSW<6,1> Auxiliary Carry status bit

* c = PSW<7,1> Carry status bit

A<0,8> Accumulator

- S[0:7]<0,16> Stack

- T0 Timer/counter timeout flag; external pin T0

- T1 Counter decrement request; external pin T1

V.2. Notation

The operands are:

- A - accumulator

- acf0f1 - A, c, F0, F1

- addr - 12-bit address specifying a memory byte

- ai - analog input 0 or 1

- BUS - bus input-output register

- c - carry bit

8048

CROSS-Microcomputer cross assembler                    31

- data - 8-bit immediate data

- dev - an I/O device

- ep - expander port P4, P5, P6 or P7

- F0,F1 - status bits

- f - status bits F0 or F1

- I - interrupt signal

- TCNTI - timer or interrupt signal

- int - I or TCNTI

- MB0,MB1 - memory bank select

- mb - MB0, MB1

- P1-P7 - I/O or expansion ports

- port - I/O port P1 or P2

- R0-R7 - registers R0-R7

- r - register R0 or R1

- @r - memory location MEM[.r]

- rn - register R0-R7

- RB0,RB1 - Register bank select

- rb - RB0, RB1

- reg - A, R0-R7

- src-opnd - MEM[.r], data or .rn

V.3. Syntax

Immediate data is expressed as # followed by the data.

V.4. Opcodes

ADD A,src-opnd  A = .A + src-opnd;

ADDC A,src-opnd A = .A + src-opnd + .c;

ANL port,data   port = .port and data;

ANL A,src-opnd  A = .A and src-opnd;

ANL BUS,data    Bus = .Bus and data;

ANLD ep,A       ep = ep and .A<0,4>;

CALL addr       SP = .SP + 1; Stack[.SP]<0,16> = (.PC<0,8>|.PSW<4,4>|.PC<8,4>);

8048

CROSS-Microcomputer cross assembler                    32

PC<11,1> = .MB; PC<0,11> = addr;

CLR acf0f1      acf0f1 = 0;

CPL acf0f1      acf0f1 = not .acf0f1;

DA A

DEC reg         reg = .reg - 1;

DIS int

DJNZ rn,addr8

EN int

ENT0 CLK        Enable timer output on pin T0

IN A,port       A = port;

INC A,@r        MEM[.r] = .MEM[.r] + 1;

INS A,BUS       A = .Bus;

JBn addr8       if .A<n,1> then PC<0,8> = addr8;

JC addr8        if .c then PC<0,8> = addr8;

JFf addr8       if .f then PC<0,8> = addr8;

JMP addr        PC<11,1> = .MB; PC<0,11> = addr;

JMPP @A         PC<0,8> = .(.PC<8,4>|.A);

JNC addr8       if not .c then PC<0,8> = addr8;

JNI addr8       if not .i then PC<0,8> = addr8;

JNTn addr8      if not .tn then PC<0,8> = addr8;

JNZ addr8       if not .z then PC<0,8> = addr8;

JTF addr8

JTn             if .tn then PC<0,8> = addr8;

JZ addr8        if .z then PC<0,8> = addr8;

MOV A,@r        A = .MEM[.r];

MOV A,T         A = .T;

MOV @r,A         MEM[.r] = .A;

MOV @r,data     MEM[.reg] = data;

MOV reg,data    reg = data;

MOV A,rn        A = .rn;

MOV PSW,A       PSW = .A;

MOV T,A         T = .A;

MOV rn,A        rn = .A;

MOVD A,ep       A<0,4> = ep;

MOVD ep,A       ep = .A<0,4>;

MOVP A,@A       A = .MEM[(.PC<8,4>|.A)];

MOVP3 A,@A      A = .MEM[(3|.A)];

MOVX @r,A       MEM[.r] = .A;

MOVX A,@r       A = .MEM[.r];

NOP             ;

ORL port,data   port = .port or data;

ORL A,src-opnd  A = .A + src-opnd;

ORL BUS,data    Bus = .Bus or data;

ORLD ep,A       ep = .ep or .A;

OUTL port,A     port = .A;

OUTL BUS,A      Bus = .A;

RAD A           A = D;

RET             PC<0,12> = (.Stack[.SP]<0,4>|.Stack[.SP]<8,8>); SP = .SP - 1;

RETR            PC<0,12> = (.Stack[.SP]<0,4>|.Stack[.SP]<8,8>);

PSW<4,4> = .Stack[.SP]<4,4>; SP = .SP -1;

RL A            A = .A rot 1;

RLC A           (c|A) = .(c|A) rot 1;

8048

CROSS-Microcomputer cross assembler                    33

RR A            A = .A rot -1;

RRC A           (c|A) = .(c|A) rot -1;

SEL ai          select analog input ai

SEL mb          MB = mb;

SEL rb

STOP TCNT       Stop counter or timer mode

STRT CNT        Counter increments on falling edge of T1

STRT T          Counter increments every 480 system clock ticks

SWAP A          .A<0,4> <=> .A<4,4>;

XCH A,@r        A <=> MEM[.r];

XCH A,rn        A <=> rn;

XCHD A,@r       A<0,4> <=> MEM[.r]<0,4>;

XRL A,src-opnd  A = .A xor opnd;

8048

CROSS-Microcomputer cross assembler                    34

VI. 1802

VI.1. Machine State

D               The data register

DF              The "data flag" register, or carry

P               4-bit program counter pointer

X               4-bit data counter pointer

T               8-bit buffer for P and X

Q               Q-status output register

VI.2. Opcodes

The operation codes recognized for the 1802 are:

ADC             D = .mem[.R[.X]] + .D + .DF

ADCI            D = data8 + .D + .DF

ADD             D = .mem[.R[.X]] + .D

ADI             D = data8 + .D

AND

ANI

BDF             if .DF then R[P]<7,0> = adr8

BGE

BL

BM

BNF

BNQ

BNZ

BN1

BN2

BN3

BN4

BPZ

BQ

BR

BZ

B1

B2

B3

B4

DEC n           R[n] = .R[n] - 1;

DIS

GHI

GLO

IDL

INC n           R[n = .R[n] + 1;

INP

IRX

1802

CROSS-Microcomputer cross assembler                    35

LBDF

LBNF

LBNQ

LBNZ

LBQ

LBR

LBZ

LDA

LDI

LDN

LDX

LDXA

LSDF

LSIE

LSKP

LSNF

LSNQ

LSNZ

LSQ

LSZ

MARK

NBR

NLBR

NOP

OR

ORI

OUT

PHI

PLO

REQ

RET

RSHL

RSHR

SAV

SD

SDB

SDBI

SDI

SEP

SEQ

SEX

SHL

SHLC

SHR

SHRC

SKP

SM

SMB

SMBI

SMI

STR

STXD

1802

CROSS-Microcomputer cross assembler                    36

XOR

XRI

1802

CROSS-Microcomputer cross assembler                    37

VII. 6502

VII.1. Machine state

The machine state is represented by:

- c - The carry bit

- z - The zero bit, true if result was zero

- v - The overflow bit

- s - The sign bit, true if result was negative

- d - Decimal mode

- i - Interrupt disable bit; 0 => interrupts enabled

- b - Breakpoint flag; set by BRK instruction trap

The machine registers are:

- ac<0,8> - The single accumulator

- x<0,8> - index register x, preindexes indirect addresses

- y<0,8> - index register y, postindexes indirect addresses.

- sp<0,8> - stack pointer.

- psw<0,8> - processor status word.

VII.2. Notation

The operands are expressed as:

- opnd8 - 8 bit operand corresponding to one of the form:

* addr - Base page direct (8 bit address).

* addr,X - Base page indexed by x; MEM[addr + .x].

* (addr,X) - Indexed indirect by x; (.MEM[addr + .x]).

* (addr),Y - Indexed indirect by y; (.MEM[addr] + .y).

- opnd16 - Any of the following operands in the form:

* addr - Extended direct (16 bit address); MEM[addr].

* addr,X - Absolute indexed via x; MEM[addr + .x].

* addr,Y - Absolute indexed via y; MEM[addr + .y].

6502

CROSS-Microcomputer cross assembler                    38

- opnd - Either opnd8 or opnd16.

- i - Immediate data, expressed as #expression.

VII.3. Syntax

The syntax for operands is much like that specified by MOS Technology.

Immediate  operands  are indicated by preceding the operand with a hash mark,

#.

X- or Y- indexed operands are written as a suffix ",X" or ",Y".

VII.4. Opcodes

The opcodes recognized for the Mos Technology 6502 are:

ADC opnd        ac = .ac + .opnd + .c; {s,z,c,v}

ADC i           ac = .ac + i + .c; {s,z,c,v}

AND opnd        ac = .ac and .opnd; {s,z}

AND i           ac = .ac and i; {s,z}

ASL opnd        c|opnd = opnd lsh 1; {s,z,c}

BCC d           if not .c then pc = .pc + 1 + d;

BCS d           if .c then pc = .pc + 1 + d;

BEQ d           if .z then pc = .pc + 1 + d;

BGE d           if .c then pc = .pc + 1 + d; Synonym for BCS

BIT opnd        .ac and .opnd; s = (.ac and .opnd)<7,1>;

v = (.ac and .opnd)<6,1>; {z}

BLT d           if not .c then pc = .pc + 1 + d; Synonym for BCC

BMI d           if .s then pc = .pc + 1 + d;;

BNE d           if not .z then pc = .pc + 1 + d;;

BPL d           if not .s then pc = .pc + 1 + d;;

BRK             MEM[.sp] = .pc<0,8>; MEM[.sp + 1] = .pc<8,8>;

MEM[.sp + 2] = .psw; sp = .sp - 3; pc = $FFFFFFFE; i = 1;

b = 1;

BVC d           if not .v then pc = .pc + 1 + d;;

BVS d           if .v then pc = .pc + 1 + d;;

CLC             c = 0;

CLD             d = 0;

CLI             i = 0;

CLV             v = 0;

CMP opnd        .ac - .opnd; {s,z,c}

CPX opnd        .x - .opnd; {s,z,c}

CPY opnd        .y - opnd; {s,z,c}

DEC             ac = .ac - 1; {s,z}

DEX             x = .x - 1; {s,z}

DEY             y = .y - 1; {s,z}

EOR opnd        ac = .ac - .opnd

INC             ac = .ac + 1; {z,s}

INX             x = .x + 1;

INY             y = .y + 1;

JMP m           pc = m;

JSR m           MEM[.sp] = .pc<8,8>; MEM[.sp - 1] = .pc<0,8>; sp = .sp - 2;

pc = m;

6502

CROSS-Microcomputer cross assembler                    39

LDA opnd        ac = .opnd; {s,z}

LDX opnd        x = .opnd; {s,z}

LDY opnd        y = .opnd; {s,z}

LSR             ac|c = .ac|0 lsh -1;

NOP             ;

ORA opnd        ac = .ac or opnd; {s,z}

PHA             MEM[.sp] = .ac; sp = .sp - 1;

PHP             MEM[.sp] = .psw; sp = .sp - 1;

PLA             ac = .MEM[.sp + 1]; sp = .sp + 1;

PLP             psw = .MEM[.sp + 1]; sp = .sp + 1;

ROL opnd        c|.MEM[opnd] = c|.MEM[opnd] rot 1; {s,z,c}

ROR opnd        c|.MEM[opnd] = c|.MEM[opnd] rot -1; {s,z,c}

RTI             psw = .MEM[.sp + 1]; pc<0,8> = .MEM[.sp + 2];

pc<8,8> = .MEM[.sp + 3]; sp = .sp + 3; pc = .pc + 1;

RTS             pc<0,8> = .MEM[.sp + 1]; pc<8,8> = .MEM[.sp + 2]; sp = .sp + 2;

pc = .pc + 1;

SBC opnd        ac = .ac - .opnd - (not .c); {s,z,c,v}

SEC             c = 1;

SED             d = 1;

SEI             i = 1;

STA opnd        MEM[opnd] = .ac; {s,z}

STX opnd        MEM[opnd] = .x; {s,z}

STY opnd        MEM[opnd] = .y; {s,z}

TAX             a = .x;

TAY             a = .y;

TSX             sp = .x;

TXA             x = .a;

TXS             x = .sp;

TYA             y = .a;

6502

CROSS-Microcomputer cross assembler                    40

VIII. 6800

VIII.1. Machine State

The registers are:

- %A<0,8> - accumulator A

- %B<0,8> - accumulator B

- %X<0,16> - Index register

- sp<0,16> - Stack pointer

- pc<0,16> - The program counter

The flags are:

- c - Carry

- o - Overflow

- s - Sign

- z - Zero

- xc - Auxiliary carry

VIII.2. Notation

- mem - A memory address which can be one of

* Base page direct (.MEM[mem], mem 8 bits)

* Extended direct (MEM[mem], mem 16 bits)

* Indexed (MEM[mem+.%X], mem 8 bits)

- memd - A memory address which can be one one of

* Extended direct (MEM[memd], memd 16 bits)

* Indexed (MEM[memd+.%X], memd 8 bits)

- i - An immediate operand, written as #i

- i16 - A 16-bit immediate operand, written as #i16

- d - A displacement, usually written as an address; CROSS will compute

the  actual  displacement  between  the  current  instruction and the

destination address.

- opnd - A general operand; may be either mem or i.   When  interpreted

6800

CROSS-Microcomputer cross assembler                    41

as  a  value,  ".opnd"  may  represent either .MEM[mem] or i.  Please

excuse the somewhat sloppy notation; it makes  the  description  more

compact.

- sr - The status register which holds c,z,s,o,xc.

VIII.3. Syntax

Immediate operands are written with a # symbol, e.g.

ADDA #$F9       ; Adds hex F9 to A

Indexing is specified by writing ",X" after the operand.  The operand must be

a value which will fit in 8 bits.

foo==$10

ADDA  foo,X

VIII.4. Opcodes

ABA             %A = .%A + .%B; {c,z,s,o,xc}

ADCA opnd       %A = .%A + .opnd + .c; {c,z,s,o,xc}

ADCB opnd       %B = .%B + .opnd + .c; {c,z,s,o,xc}

ADDA opnd       %A = .%A + .opnd; {c,z,s,o,xc}

ADDB opnd       %B = .%b + .opnd; {c,z,s,o,xc}

ANDA opnd       %A = .%A + .opnd; o=0; {z,s}

ANDB opnd       %B = .%B + .opnd; o=0; {z,s}

ASL memd        c=.memd<7,1>; memd=.memd lsh 1; o=.s xor .c; {z,s}

ASLA            c=.%A<7,1>; %A = .%A lsh 1; o=.s xor .c; {z,s}

ASLB            c=.%B<7,1>; %B = .%B lsh 1; o=.s xor .c; {z,s}

ASR memd        c=.memd<0,1>; memd=.memd ash -1; o=.s xor .c; {z,s}

ASRA            c=.%A<0,1>; %A=.%A ash -1; o=.s xor .c; {z,s}

ASRB            c=.%B<0,1>; %B=.%B ash -1; o=.s xor .c; {z,s}

BCC d           if not .c then BRA d;

BCS d           if .c then BRA d;

BEQ d           if .z then BRA d;

BGE d           if not (.o xor .s) then BRA d;

BGT d           if not (.z or (.s xor .o)) then BRA d;

BHI d           if not (.c or .z) then BRA d;

BHS d           if not .c then BRA d; Synonym for BCC

BITA mem        .%A and .mem; o = 0; {z,s}

BITB mem        .b and .mem; o = 0; {z,s}

BLE d           if .z or (.s xor .o) then BRA d;

BLO d           if .c then BRA d; Synonym for BCS

BLS d           if .c or .z then BRA d;

BLT             if .s xor .o then BRA d;

BMI d           if .s then BRA d;

BNE d           if not .z then BRA d;

BPL d           if not .s then BRA d;

BR d            BRA d;

BRA d           pc = .pc + 2 + d;

BSR d           MEM[.sp] = .pc<0,8>; MEM[.sp - 1] = .pc<8,8>; sp = .sp - 2;

pc = .pc + d + 2;

6800

CROSS-Microcomputer cross assembler                    42

BVC d           if not .o then BRA d;

BVS d           if .o then BRA d;

CBA             .%A - .%B; {c,z,s,o}

CLC             c=0;

CLI             ie = 0;

CLR memd        memd=0; c=0; z=1; s=0; o=0;

CLRA            %A = 0; c=0; z=1; s=0; o=0;

CLRB            %B = 0; c=0; z=1; s=0; o=0;

CLV             o = 0;

CMPA opnd       .%A - .opnd; {c,z,s,o}

CMPB opnd       .%B - .opnd; {c,z,s,o}

COM memd        memd=~.memd; c=1; o=0; {s,z}

COMA            %A=~.%A; c=1; o=0; {s,z}

COMB            %B=~.%B; c=1; o=0; {s,z}

CPX memd        %X<0,8>-.(memd+1); %X<8,8>-.memd; {s,z,o}

DAA             decimal arithmetic

DEC memd        memd=.memd-1; {s,z,o}

DECA            %A = .%A - 1; {s,z,o}

DECB            %B = .%B - 1; {s,z,o}

DES             sp = .sp - 1;

DEX             %X = .%X - 1; {z}

EORA opnd       %A=.%A xor .opnd; o=0; {s,z}

EORB opnd       %B=.%B xor .opnd; o=0; {s,z}

INC memd        memd=.memd+1; {s,z,o}

INCA            %A = .%A + 1; {s,z,o}

INCB            %B = .%B + 1; {s,z,o}

INS             sp = .sp + 1;

INX             %x = .%X + 1; {z}

JMP mem         pc = mem;

JSR mem         MEM[.sp]=.pc<0,8>; MEM[.sp-1]=.pc<8,8>; sp=.sp-2; pc=mem;

LDAAmem         A = .mem; o = 0; {o,s,z}

LDABmem         B = .mem; o = 0; {o,s,z}

LDS mem         sp<8,8>  = .mem; sp<0,8> = .(mem + 1); o = 0; {z,s}

LDS i16         sp = i16; {s,z}

LDX opnd        %X<8,8> = .opnd; %X<0,8> = .(opnd + 1); o = 0; {z,s}

LSRA            A = .A lsh -1;

LSRB            B = .B lsh -1;

NEG memd        memd = 0 -.memd; {c,s,z,o}

NEGA            %A = 0 - .%A; {c,s,z,o}

NEGB            %B = 0 - .%B; {c,s,z,o}

NOP             ;

ORAA opnd       %A=.%A or .opnd; o=0; {s,z}

ORAB opnd       %B=.%B or .opnd; o=0; {s,z}

PSHA            Synonym for PSHS A

PSHB            Synonym for PSHS B

PULA            Synonym for PULS A

PULB            Synonym for PULS B

ROL opnd        c|opnd = .(c|opnd) rot 1;

ROLA            c|A = .(c|A) rot 1;

ROLB            c|B = .(c|B) rot 1;

ROR opnd        c|opnd = .(c|opnd) rot -1;

RORA            c|A = .(c|A) rot -1;

6800

CROSS-Microcomputer cross assembler                    43

RORB            c|B = .(c|B) rot -1;

RTI

RTS

SBA             %A = .%A - .%B; {c,z,s,o}

SBCA opnd       %A = %A - .opnd - .c; {c,z,s,o}

SBCB opnd       %B = .%B - .opnd - .c; {c,z,s,o}

SEC             c = 1;

SEI             ie = 1;

SEV             o = 1;

STAA opnd       mem = .%A;

STAB opnd       mem = .%B;

STS mem         mem = .sp<8,8>; (mem + 1) = .sp<0,8>; o=0; {s,z}

STX mem         mem = .%X<8,8>; (mem + 1) = .%X<0,8>; o=0; {s,z}

SUBA opnd       %A = .%A - .opnd; {c,s,z,o}

SUBB opnd       %B = .%B - .opnd; {c,s,z,o}

SWI             software interrupt;

TAB             %B = .%A; o=0; {s,z}

TAP             sr = .%A; {c,z,s,o,xc}

TBA             %A = .%B; o=0; {s,z}

TPA             %A = .sr

TST memd        c = 0; z = .memd eql 0; s = .memd < 0; o = 0;

TSTA            c = 0; z = .%A eql 0; s = .%A < 0; o = 0;

TSTB            c = 0; z = .%B eql 0; s = .%B < 0; o = 0;

TSX             %X = .sp + 1;

TXS             sp = .%X - 1;

WAI             MEM[.sp]=.pc<0,8>; MEM[.sp-1] = .pc<8,8>;

MEM[.sp-2] = .%X<0,8>; MEM[.sp-3] = .%X<8,8>; MEM[.sp-4] = .%A;

MEM[.sp-5] = .%B; MEM[.sp-6] = .sr; sp = .sp-7;

6800

CROSS-Microcomputer cross assembler                    44

IX. 6805

There is one known bug in the 6805 code.  During pass 1, an indexedaddress is

assumed  to  require  2  bytes if it is a forward reference.  During pass 2, if

CROSS determines that the address can be  represented  in  one  byte,  it  will

assign  it  as one byte.  All labels and other position-dependent symbols after

this point will  now  have  different  values  assigned  during  pass  2;  thus

generating a phase (P) error.  This may be circumvented by explicitly using a >

operator to force the address to always be two bytes.

IX.1. Machine State

A<0,8>          Accumulator

X<0,8>          Index register

PC<0,11>        Program counter

CC<0,5>         Condition codes

h               half-carry (CC<4,1>)

i               interrupt mask (CC<3,1>)

n               negative (sign) status (CC<2,1>)

z               zero status (CC<1,1>)

c               carry status (CC<0,1>)

SP<0,11>

Stack  pointer;  note  however  that  only SP<0,5> is variable;

SP<5,6> is constant at ^B000011.

il              External interrupt line.

IX.2. Notation

data8           Any immediate 8-bit data.

opnd            Any operand in the modes direct, extended, indexed (no offset),

indexed (8-bit offset), indexed (16-bit offset).

opnd8           Any operand in  the  modes  direct,  indexed  (no  offset),  or

indexed (8-bit offset).

label           Any  operand which can be converted by CROSS to an 8-bit offset

(7 bits plus sign).

IX.3. Syntax

Immediate operands are preceded by #.

A > symbol preceding an operand forces that operand to be two bytes.

6805

CROSS-Microcomputer cross assembler                    45

A < symbol preceding an operand forces that operand to be one byte.

Indexing is indicated by following the operand with (X).

IX.4. Opcodes

ADC data8       A = .A + data8 + .c; {h,n,z,c}

ADC opnd        A = .A + .opnd + .c; {h,n,z,c}

ADD data8       A = .A + data8; {h,n,z,c}

ADD opnd        A = .A + .opnd; {h,n,z,c}

AND data8       A = .A and data8; {n,z}

AND opnd        A = .A and .opnd; {n,z}

ASL opnd8       opnd8 = .opnd8 ash 1; {n,z,c}

ASLA            A = .A ash 1; {n,z,c}

ASLX            X = .X ash 1; {n,z,c}

ASR opnd8       opnd8 = .opnd8 ash -1; {n,z,c}

ASRA            A = .A ash -1; {n,z,c}

ASRX            X = .X ash -1; {n,z,c}

BCC label       if not .c then BRA label;

BCLRn opnd8     opnd8<n,1> = 0;

BCS label       if .c then BRA label;

BEQ label

BHCC label      if not .h then BRA label;

BHCS label      if .h then BRA label;

BHI label

BHS label       if not .c then BRA label;

BIH label       if .il then BRA label;

BIL label       if not .il then BRA label;

BIT label       {n,z}

BLO label       if .c then BRA label;

BLS label

BMC label       if not .i then BRA label;

BMS label       if .i then BRA label;

BNE label       if not .z then BRA label;

BPL label       if not .n then BRA label;

BRA label       PC = .PC + signextended(label);

BRN label       ;

BRCLR opnd8,label

BRSET opnd8,label

BSETn opnd      opnd<n,1> = 1;

CLC             c = 0; {c}

CLI             i = 0; {i}

CLR opnd8       opnd8 = 0; n = 0; z = 1; {n,z}

CLRA            A = 0; n = 0; z = 1; {n,z}

CLRX            X = 0; n = 0; z = 1; {n,z}

CMP data        .A - data; {n,z,c}

CMP opnd        .A - .opnd; {n,z,c}

COM opnd8       opnd8 = not .opnd8; c = 1; {n,z,c}

COMA            A = not .A; c = 1; {n,z,c}

COMX            X = not .X; c = 1; {n,z,c}

CPX data        .X - data; {n,z,c}

CPX opnd        .X - .opnd; {n,z,c}

6805

CROSS-Microcomputer cross assembler                    46

DEC opnd8       opnd8 = .opnd8 - 1 ; {n,z}

DECA            A = .A - 1; {n,z}

DECX            X = .X - 1; {n,z}

INC opnd8       opnd8 = .opnd8 + 1 ; {n,z}

INCA            A = .A + 1; {n,z}

INCX            X = .X + 1; {n,z}

JMP opnd

JSR opnd

LDA data        A = data; {n,z}

LDA opnd        A = .opnd; {n,z}

LDX data        X = data; {n,z}

LDX opnd        X = .opnd; {n,z}

LSL opnd8       opnd8 = .opnd8 lsh 1; {n,z,c}

LSLA            A = .A lsh 1; {n,z,c}

LSLX            X = .X lsh 1; {n,z,c}

LSR opnd8       opnd8 = .opnd8 lsh -1; n = 0; {n,z,c}

LSRA            A = .A lsh -1; n = 0; {n,z,c}

LSRX            X = .X lsh -1; n = 0; {n,z,c}

NEG opnd8       opnd8 = -.opnd8; {n,z,c}

NEGA            A = -.A; {n,z,c}

NEGX            X = -.X; {n,z,c}

NOP             ;

ORA data        A = .A or data; {n,z}

ORA opnd        A = .A or .opnd; {n,z}

ROL opnd8       (c|opnd8) = .(c|opnd8) rot 1; {n,z,c}

ROLA            (c|A) = .(c|A) rot 1; {n,z,c}

ROLX            (c|X) = .(c|X) rot 1; {n,z,c}

ROR opnd8       (c|opnd8) = .(c|opnd8) rot -1; {n,z,c}

RORA            (c|A) = .(c|A) rot -1; {n,z,c}

RORX            (c|X) = .(c|X) rot -1; {n,z,c}

RSP

RTI

RTS

SBC data        A = .A - data - .c; {n,z,c}

SBC opnd        A = .A - .opnd; {n,z,c}

SEC             c = 1; {c}

SEI             i = 1; {i}

STA opnd        opnd = .A; {n,z}

STOP

STX opnd        opnd = .X; {n,z}

SUB data        A = .A - data; {n,z,c}

SUB opnd        A = .A - .opnd; {n,z,c}

SWI

TAX             X = .A;

TST opnd8       .opnd8 - 0; {n,z}

TXA             A = .X;

6805

CROSS-Microcomputer cross assembler                    47

X. 6809

There is one known bug in the 6809 code.  During pass 1, a PC-relativeaddress

is  assumed to require 2 bytes if it is a forward reference.  During pass 2, if

CROSS determines that the address can be represented in one byte because it  is

less  than  256 locations ahead, it will assign it as one byte.  All labels and

other position-dependent symbols after  this  point  will  now  have  different

values  assigned during pass 2; thus generating a Phase (P) error.  This may be

circumvented by explicitly using a > operator to force the address to always be

two bytes.

X.1. Machine State

A<0,8>          Accumulator A

B<0,8>          Accumulator B

CC<0,8>         Condition codes

e               "entire"  flag  -  indicates  action   on   RTI

instruction

f               FIRQ interrupt request mask; 1 => disabled

h               Aux carry (half carry); BCD arithmetic

i               IRQ interrupt request mask; 1 => disabled

s               Sign bit (1 => negative)

z               Zero bit (1 => zero result)

v               Overflow bit

c               Carry bit

D<0,16>         A|B

DP<0,8>         Direct page register

PC<0,16>        program counter

S<0,16>         stack pointer register

U<0,16>         stack pointer register

X<0,16>         index register

Y<0,16>         index register

6809

CROSS-Microcomputer cross assembler                    48

X.2. Notation

- addr  -  any  of  the  permissible addressing modes, direct page, one

byte, two byte, indexed, indirect, etc.

- opnd8 - any one-byte operand, except immediate

- opnd16 - any two-byte operand, except immediate

- data8 - 8 bit immediate data

- data16 - 16 bit immediate data

- reglist - A list of register names from the set CC, A, B, DP, X, Y, U

or S, PC

X.3. Syntax

- A - accumulator A

- B - accumulator B

- CC - condition codes

- D - accumulator D (=A|B)

- DP - direct page register DP

- PC - Program counter

- PCR - Program counter relative addressing

- S - Stack pointer S

- U - Stack pointer U

- X - Index register X

- Y - Index register Y

Binary values are indicated by preceding the value with a % symbol.

Immediate operands are written with a # in front of the expression; the value

of the operand  or  an  explicit  size  specification,  either  with  <  or  >,

determines  if  it  is  a  data8  or  data16.  Those instructions which require

double-width immediate operands will force data16 values even if the value will

normally fit in a single byte.

An explicit one-byte address or operand, opnd8, is designated by a < symbol.

An explicit two-byte address or operand, opnd16, is designated by a > symbol.

6809

CROSS-Microcomputer cross assembler                    49

Indirect  operands  are  enclosed in [] symbols; an indirect operand can be a

one-byte or two-byte operand.  Indexed, autoincrement, and autodecrement  modes

are permitted.

Indexing  is indicated by suffixing a ",r" to the operand, for various values

of index registers (X, Y, S, U, PC, PCR).

Single auto-decrement of register operands is indicated by prefixing a single

- symbol; double auto-decrement by two such symbols: --.

Single auto-increment of register operands is indicated by suffixing a single

+ symbol; double auto-increment by two such symbols: ++.

Note that the form which omits the  comma  before  the  register  in  indexed

modes, which is used in the 6809 assembler, is not accepted by CROSS, e.g.,

6809                    CROSS

R+                      ,R+

-R                      ,-R

R                       ,R

[R]                     [,R]

<R                      <,R

<[R]                    <[,R]

>R                      >,R

>[R]                    >[,R]

X.4. Opcodes

.SETDP addr     Indicates to the assembler that the direct page register refers

to addr; any references to that page will be handled as direct

page references.

ABX             X = .X + .B; Unsigned add

ADCA opnd       A = .A + .opnd + .c; {h,n,z,v,c}

ADCA data       A = .A + data + .c; {h,n,z,v,c}

ADCB opnd       B = .B + .opnd + .c; {h,n,z,v,c}

ADCB data       B = .B + data + .c; {h,n,z,v,c}

ADDA opnd       A = .A + .opnd; {h,n,z,v,c}

ADDA data       A = .A + data; {h,n,z,v,c}

ADDB opnd       B = .B + opnd; {h,n,z,v,c}

ADDB data       B = .B + data; {h,n,z,v,c}

ADDD opnd       (A|B) = .(A|B) + .opnd; {n,z,v,c}

ADDD data       (A|B) = .(A|B) + data; {n,z,v,c}

ANDA opnd       A = .A and .opnd; v = 0; {n,z,v}

ANDA data       A = .A and data; v = 0; {n,z,v}

ANDB opnd       B = .B and opnd; v = 0; {n,z,v}

ANDA data       B = .B and data; v = 0; {n,z,v}

ANDCC data8     CC = .CC and data8; {f,h,i,n,z,v,c}

ASL opnd        opnd = .opnd ash 1; {h,n,z,v,c}

ASLA            A = .A ash 1; {h,n,z,v,c}

ASLB            B = .B ash 1; {h,n,z,v,c}

ASR opnd        opnd = .opnd ash -1; {h,n,z,v,c}

ASRA            A = .A ash -1; {h,n,z,v,c}

6809

CROSS-Microcomputer cross assembler                    50

ASRB            B = .B ash -1; {h,n,z,v,c}

BCC opnd        if not .c then BRA opnd;

BCS opnd        if .c then BRA opnd;

BEQ opnd        if .z then BRA opnd;

BGE opnd        if not (.n xor .v) then BRA opnd

BGT opnd        if not (.z or (.n xor .v)) then BRA opnd

BHI opnd        if not (.c xor .z) then BRA opnd

BHS opnd        if not .c then BRA opnd

BITA            .A and opnd; v = 0; {n,z,v}

BITB            .B and opnd; v = 0; {n,z,v}

BLE opnd        if .z or (.n xor .v) then BRA opnd

BLO opnd        if .c then BRA opnd

BLS opnd        if (.c or .z) then BRA opnd

BLT opnd        if (.n xor .v) then BRA opnd

BMI opnd        if .n then BRA opnd

BNE opnd        if not .z then BRA opnd

BPL opnd        if not .n then BRA opnd

BR

BRA

BRN             ;

BSR

BVC opnd        if not .v then BRA opnd;

BVS opnd        if .v then BRA opnd;

CLR opnd        opnd = 0; n = 0; z = 1; v = 0; c = 0; {n,z,v,c}

CLRA            A = 0; n = 0; z = 1; v = 0; c = 0; {n,z,v,c}

CLRB            B = 0; n = 0; z = 1; v = 0; c = 0; {n,z,v,c}

CMPA opnd       .A - .opnd; {h,n,z,v,c}

CMPB opnd       .B - .opnd; {h,n,z,v,c}

CMPB data       .B - data; {h,n,z,v,c}

CMPD opnd       .(A|B) - .opnd; {h,n,z,v,c}

CMPD data       .(A|B) - data; {h,n,z,v,c}

CMPS opnd       .S - .opnd; {n,z,v,c}

CMPS data       .S - data; {h,n,z,v,c}

CMPU opnd       .U - .opnd; {n,z,v,c}

CMPU data       .U - data; {h,n,z,v,c}

CMPX opnd       .X - .opnd; {n,z,v,c}

CMPX data       .X - data; {h,n,z,v,c}

CMPY opnd       .Y - .opnd; {n,z,v,c}

CMPY data       .Y - data; {h,n,z,v,c}

COM opnd        opnd = not .opnd; v = 0; c = 1; {n,z,v,c}

COMA            A = not .A; v = 0; c = 1; {n,z,v,c}

COMB            B = not .B; v = 0; c = 1; {n,z,v,c}

CPX

CWAI data8      e = 1; PSHS PC,U,Y,X,DP,B,A,CC; .CC and data8;} wait for

interrupt {f,h,i,n,z,v,c}

DAA             bcd arithmetic adjust {n,z,v,c}

DEC opnd        opnd = .opnd -1; {n,z,v}

DECA            A = .A - 1; {n,z,v}

DECB            B = .B - 1; {n,z,v}

EORA opnd       A = .A xor .opnd; v = 0; {n,z,v}

EORA data       A = .A xor data; v = 0; {n,z,v}

EORB opnd       B = .B xor .opnd; v = 0; {n,z,v}

6809

CROSS-Microcomputer cross assembler                    51

EORB data       B = .B xor data; v = 0; {n,z,v}

EXG r ,r        r  <=> r ; {h,f,i,n,z,v,c}

1  2        1      2

INC opnd        opnd = .opnd + 1; {n,z,v}

INCA            A = .A + 1; {n,z,v}

INCB            B = .B + 1; {n,z,v}

JMP opnd        PC = opnd;

JSR

LBCC opnd       if not .c then LBRA opnd;

LBCS opnd       if .c then LBRA opnd;

LBEQ opnd       if .z then LBRA opnd;

LBGE opnd       if not (.n xor .v) then LBRA opnd;

LBGT opnd       if not (.z or (.n xor .v)) then LBRA opnd;

LBHI opnd       if not (.c xor .z) then LBRA opnd;

LBHS opnd       if not .c then LBRA opnd;

LBLE opnd       if .z or (.n xor .v) then LBRA opnd;

LBLO opnd       if .c then LBRA opnd;

LBLS opnd       if (.c or .z) then LBRA opnd;

LBLT opnd       if (.n xor .v) then LBRA opnd;

LBMI opnd       if .n then LBRA opnd;

LBNE opnd       if not .z then LBRA opnd;

LBPL opnd       if not .n then LBRA opnd;

LBR

LBRA

LBRN            ;

LBSR

LBVC opnd       if not .v then LBRA opnd;

LBVS opnd       if .v then LBRA opnd;

LDA opnd        A = .opnd; v = 0; {n,z,v}

LDA data        A = data; v = 0; {n,z,v}

LDAA            Synonym for LDA

LDAB            Synonym for LDB

LDB opnd        B = .opnd; v = 0; {n,z,v}

LDB data        B = data; v = 0; {n,z,v}

LDD opnd        (A|B) = (.opnd|.(opnd+1)); v = 0; {n,z,v}

LDD data16      (A|B) = data16; v = 0; {n,z,v}

LDS opnd        S = (.opnd|.(opnd+1)); v = 0; {n,z,v}

LDS data16      S = data16; v = 0; {n,z,v}

LDU opnd        U = (.opnd|.(opnd+1)); v = 0; {n,z,v}

LDU data16      U = data16; v = 0; {n,z,v}

LDX opnd        X = (.opnd|.(opnd+1)); v = 0; {n,z,v}

LDX data16      X = data16; v = 0; {n,z,v}

LDY opnd        Y = (.opnd|.(opnd+1)); v = 0; {n,z,v}

LDY data16      Y = data16; v = 0; {n,z,v}

LEAS opnd       S = opnd;

LEAU opnd       U = opnd;

LEAX opnd       X = opnd; {z}

LEAY opnd       Y = opnd; {z}

LSL opnd        opnd = .opnd lsh 1; {h,n,z,v,c}

LSLA            A = .A lsh 1; {h,n,z,v,c}

LSLB            B = .B lsh 1; {h,n,z,v,c}

LSR opnd        opnd = .opnd lsh -1; n = 0; {n,z,c}

6809

CROSS-Microcomputer cross assembler                    52

LSRA            A = .A lsh -1; n = 0; {n,z,c}

LSRB            B = .B lsh -1; n = 0; {n,z,c}

MUL             (A|B) = .A * .B; {z,c}

NEG opnd        opnd = - .opnd; {h,n,z,v,c}

NEGA            A = - .A; {h,n,z,v,c}

NEGB            B = - .B; {h,n,z,v,c}

NOP             ;

ORA opnd        A = .A or .opnd; v = 0; {n,z,v}

ORA data        A = .A or data; v = 0; {n,z,v}

ORAA opnd       Synonym for ORA

ORAB opnd       Synonym for ORB

ORB opnd        B = .B or .opnd; v = 0; {n,z,v}

ORB data        B = .B or data; v = 0; {n,z,v}

ORCC data8      CC = .CC or data8; {f,h,i,n,z,v,c}

PSHS reglist    if PC in reglist then (S=.S-1; .S = .PC<0,8>; S=.S-1;

.S=.PC<8,8>);

if U in reglist then (S=.S-1; .S = .U<0,8>; S=.S-1;

.S=.U<8,8>);

if Y in reglist then (S=.S-1; .S = .Y<0,8>; S=.S-1;

.S=.Y<8,8>);

if X in reglist then (S=.S-1; .S = .X<0,8>; S=.S-1;

.S=.X<8,8>);

if DP in reglist then (S=.S-1; .S = .DP;)

if B in reglist then (S=.S-1; .S = .B<0,8>;)

if A in reglist then (S=.S-1; .S = .A;)

if CC in reglist then (S=.S-1; .S = .CC;)

PSHU reglist    if PC in reglist then (U=.U-1; .U = .PC<0,8>; U=.U-1;

.U=.PC<8,8>);

if S in reglist then (U=.U-1; .U = .S<0,8>; U=.U-1;

.U=.S<8,8>);

if Y in reglist then (U=.U-1; .U = .Y<0,8>; U=.U-1;

.U=.Y<8,8>);

if X in reglist then (U=.U-1; .U = .X<0,8>; U=.U-1;

.U=.X<8,8>);

if DP in reglist then (U=.U-1; .U = .DP;)

if B in reglist then (U=.U-1; .U = .B<0,8>;)

if A in reglist then (U=.U-1; .U = .A;)

if CC in reglist then (U=.U-1; .U = .CC;)

PULS reglist    if CC in reglist then (CC = .(.S); S=.S+1);

if A in reglist then (A = .(.S); S=.S+1);

if B in reglist then (B = .(.S); S=.S+1);

if DP in reglist then (DP = .(.S); S=.S+1);

if X in reglist then (X<8,8> = .(.S); S=.S+1;

X<0,8> = .(.S); S=.S+1);

if Y in reglist then (Y<8,8> = .(.S); S=.S+1;

Y<0,8> = .(.S); S=.S+1);

if U in reglist then (U<8,8> = .(.S); S=.S+1;

U<0,8> = .(.S); S=.S+1);

if PC in reglist then (PC<8,8>= .(.S); S=.S+1; PC<0,8> = .(.S);

S=.S+1);

PULU reglist    if CC in reglist then (CC = .(.U); U=.U+1);

if A in reglist then (A = .(.U); U=.U+1);

6809

CROSS-Microcomputer cross assembler                    53

if B in reglist then (B = .(.U); U=.U+1);

if DP in reglist then (DP = .(.U); U=.U+1);

if X in reglist then (X<8,8> = .(.U); U=.U+1;

X<0,8> = .(.U); U=.U+1);

if Y in reglist then (Y<8,8> = .(.U); U=.U+1;

Y<0,8> = .(.U); U=.U+1);

if S in reglist then (S<8,8> = .(.U); U=.U+1;

S<0,8> = .(.U); U=.U+1);

if PC in reglist then (PC<8,8>= .(.U); U=.U+1; PC<0,8> = .(.U);

S=.S+1);

ROL opnd        c|opnd = .(c|opnd) rot 1;

ROLA            c|A = .(c|A) rot 1;

ROLB            c|B = .(c|B) rot 1;

ROR opnd        c|opnd = .(c|opnd) rot 1;

RORA            c|A = .(c|A) rot -1;

RORB            c|B = .(c|B) rot -1;

RTI

RTS

SBCA opnd       A = .A - .opnd - .C; {n,z,v,c}

SBCA data       A = .A - data - .C; {n,z,v,c}

SBCB opnd       B = .B - .opnd - .C; {n,z,v,c}

SBCB data       B = .B - data - .C; {n,z,v,c}

SEX             sign extend B into A; v = 0; {n,z,v}

STA opnd        opnd = .A; v = 0; {n,z,v}

STAA opnd       Synonym for STA

STAB opnd       Synonym for STB

STB opnd        opnd = .B; v = 0; {n,z,v}

STD opnd        opnd|opnd+1 = .(A|B); v = 0; {n,z,v}

STS opnd        opnd|opnd+1 = .S; v = 0; {n,z,v}

STU opnd        opnd|opnd+1 = .U; v = 0; {n,z,v}

STX opnd        opnd|opnd+1 = .X; v = 0; {n,z,v}

STY opnd        opnd|opnd+1 = .Y; v = 0; {n,z,v}

SUBA opnd       A = .A - .opnd; {n,z,v,c}

SUBA data       A = .A - data; {n,z,v,c}

SUBB opnd       B = .B - .opnd; {n,z,v,c}

SUBB data       B = .B - data; {n,z,v,c}

SUBD opnd       (A|B) = .(A|B) - .opnd|.(opnd+1); {n,z,v,c}

SUBD data       (A|B) = .(A|B) - data; {n,z,v,c}

SWI             software interrupt PSHS PC,U,Y,X,DP,B,A,CC; e=1; i=1; f=1;

PC = ^HFFFA;

SWI2            software interrupt PSHS PC,U,Y,X,DP,B,A,CC; e=1; i=1; f=1;

PC = ^HFFF4;

SWI3            software interrupt PSHS PC,U,Y,X,DP,B,A,CC; e=1; i=1; f=1;

PC = ^HFFF2;

SYNC            synchronize

TFR r ,r        r  = r

1  2        2    1

TST opnd        .opnd - 0; v = 0; {n,z,v}

TSTA            .A - 0; v = 0; {n,z,v}

TSTB            .B - 0; v = 0; {n,z,v}

6809

CROSS-Microcomputer cross assembler                    54

XI. Compiler directives

The  following  compiler directives and their presumed meanings are described

briefly here.  This is only a  refresher  for  those  who  have  read  the  DEC

assembler description.

.ABS            Compile non-relocatable (absolute) code.

.ADDR           Generates  a  16-bit  address  with the bytes in swapped order,

i.e., <low> <high>.  Equivalent  of  a  compile-time  byte-swap

operation on a .WORD.

.ASCII          ASCII string delimited by paired markers.

.ASCIZ          ASCIZ  string  delimited  by  paired  markers and supplied with

terminal null byte.

.ASECT          Not implemented in CROSS.

.BLKB           Allocate space for a  number  of  bytes  as  specified  by  the

operand.

.BLKW           Allocate  space  for  a  number  of  words  as specified by the

operand.

.BYTE           Each expression supplied will be stored in a single byte.

.CSECT          Not implemented in CROSS.

.DEPHA          Dephase; leave .PHASE control area.

.DSABL          See chapter 5.

.ENABL          See chapter 5.

.END            End of program.

.ENDC           End of conditional; paired with .IF.

.ENDM           End of macro definition or repeat; paired  with  .MACRO,  .IRP,

.IRPC, .REPT.

.ENDR           End  of  repetition;  paired with .REPT; equivalent to .ENDM in

this context.

.EOT            End-of-tape; assembler will wait for next  tape  to  be  loaded

(paper tape hackers only).

.EQUIV          Equates two symbols:

.equiv defsym,anysym

The symbols may be user symbols, builtin symbols, or macros.

Compiler directives

CROSS-Microcomputer cross assembler                    55

.ERROR          Causes  an error message to be issued.  The optional expression

is evaluated and output if present.

label: .Print expr;text

.FLT2           ?

.FLT4           ?

.GLOBL          Not implemented in CROSS.

.IF

Enter a conditional.

.IFDF           .IFDF <symbol> true if symbol is defined.  See also .IFNDF.

.IFEQ           .IFEQ <expression> true if expression evaluates to zero.

.IFF            Begins body of conditional if enclosing condition was false.

.IFG            .IFG <expression> true if expression greater than zero.

.IFGE           .IFGE <expression> true if expression greater than or equal  to

zero.

.IFGT           Same as .IFG

.IFL            .IFL <expression> true if expression is negative.

.IFLE           .IFLE <expression> true if expression is negative or zero.

.IFLT           Same as .IFL.

.IFNDF          .IFNDF <symbol> true if symbol is undefined.

.IFNE           .IFNE <expression> true if expression is non-zero.

.IFNZ           Same as .IFNE

.IFT            Begins body of conditional if enclosing condition was true.

.IFTF           Text  from  here  to the next .IFT, .IFF or .IFTF or the end of

the conditional will be included independent of  the  value  of

the condition of the enclosing conditional.

.IFZ            Same as .IFE

.IIF condition, arg, stmnt

Immediate if; if condition is met, stmnt is emitted.

.IRP            Indefinite repeat of the body; Sucessive arguments from arglist

are assigned to sym.

Compiler directives

CROSS-Microcomputer cross assembler                    56

.IRP sym,arglist

body

.ENDR

.IRPC           Indefinite repeat of the body.  Successive characters of string

are assigned to sym.

.IRPC sym,string

body

.ENDR

.LIMIT

Not implemented in CROSS.

.LIST           Enable listing.

.LOCAL          Not implemented in CROSS.

.MACRO          Begin a macro definition

.MACRO name arglist

body

.ENDM

.MCALL          Call macros from the system library.  See chapter 7.

.MEXIT          Leave macro expansion

.NARG sym       The symbol is equated to the number of arguments to the macro

.NCHR sym,str   The  symbol  sym  is equated to the number of characters in the

delimited string str

.NLIST          Turn off listing

.NTYPE sym,arg  The symbol is equated to the addressing mode of the argument.

.PAGE           Force a page break

.PDP10          Irrelevant

.PHASE          Assemble code as if it were located according to  the  argument

of this directive.

.PRINT          Print the text which follows on the terminal.

label: .Print expr;text

.PSECT          Not implemented in CROSS.

.RADIX          Change the radix to the value specified to this directive.

Compiler directives

CROSS-Microcomputer cross assembler                    57

.RAD50          Each   argument  is  converted  to  a  radix50  representation,

according to the  PDP-11  radix50  conventions.    This  allows

packing  three  bytes of compressed data in a 16-bit word. (The

"50" is octal 50)

.REM            Include a remark

.REPT           (.REPT <n>) Repeat the body of  this  directive  the  indicated

number of times.

.ROUND          Perform  arithmetic  operations with rounding to integer result

(see also .TRUNC).

.SBTTL          Specify subtitle text.

.SETDP          (6809 only) Tells the assembler which page is  currently  being

addressed by the Direct Page register; see page 49.  .TITLE

Specify title text.

.TRUNC          Perform arithmetic operations with truncation to integer result

(see also .ROUND)

.WORD           Allocate  space  for  each  value  specified.   Each expression

evaluates to a word (16 bits).

Compiler directives

CROSS-Microcomputer cross assembler                    58

XII. Macro quick summary

?               When  specified  with a formal, will cause a compiler-generated

symbol to be provided  if  an  actual  is  not  given  in  that

position.

'               Internal concatenation character

\<expr>         When  used  as  an argument, evaluates <expr> before performing

the actual-formal match.

Macro summary

CROSS-Microcomputer cross assembler                    59

XIII. Assembler syntax quick summary

<...>           Encloses   any   expression   (the  assembler's  equivalent  of

parentheses).  Expressions are evaluated left-to-right  without

operator heirarchy.

+               Addition or unary plus

-               Subtraction or unary minus

*               Multiplication

/               Division

&               Logical and

!               Logical or

;               Begins comment field

'               Indicates single ASCII character as a term in expressions.

@               Indicates  octal  radix number; same as ^O prefix (not valid in

8048 mode, where it is used to indicate addressing modes).

^B              Indicates binary number

^C              Takes the unary complement ("not") of the following expression

^H              Indicates hexadecimal number

^O              Indicates octal number

=               Symbol assignment

.               Current location counter

$               Indicates hexadecimal number

%               Indicates binary number (6502/6800/6805/6809 mode only)

^               If a postfix operator, divides the expression by 256.

\               If a postfix operator, takes the  evaluated  expression  modulo

256 (low byte).

Assembler summary

CROSS-Microcomputer cross assembler                    60

XIV. Error codes

A               Addressing  error;  illegal  mode,  length  of  address, signed

offset, etc.

D               Doubly-defined symbol referenced.

E               .end directive not found (its presence is assumed).

I               Illegal character detected,  i.e.,  a  non-printing  character;

printed as ? in the listing file.

L               Line buffer overflow; input line too long.

M               Multiple definition of a label; remember that labels are unique

only to the first 6 characters.

N               Number  error;  a  digit  not  legal  in  the  given  radix has

appeared.

O               Opcode or directive error.

P               Phase error; a label has a different value between pass  1  and

pass  2.    Possible causes are strange conditional expansions,

phase conditionals, etc.  but note also that some  modes  which

generate  "short"  and  "long" addresses may generate different

lengths on different passes.  See the warnings on pages 44  and

47.

Q               Questionable  syntax.    There are missing instructions, or the

instruction scan was  not  completed,  or  a  comment  was  not

properly delimited.

R               Register error; improper use of a register operand.

T               Truncation error; a value has more than 16 bits of significance

in  a  16-bit context or more than 8 bits of significance in an

8-bit context.

U               Undefined symbol.  The undefined symbol is assigned a value  of

0.

Z               Instruction error. The instruction flagged may not be available

on  the  destination  processor.  This warning occurs when z-80

instructions are used without a .enable z80 directive.

Assembler summary

CROSS-Microcomputer cross assembler                    61

XV. Object format

XV.1. PTP object format

If  the /PTP switch is given (and it must be given with the output file name,

not as part of the input file specification), the object  format  is  an  ASCII

file.    The details are determined by either the default output format derived

from the input file extension, or by the argument to the /PTP switch; see  page

5.

XV.1.1. COSMAC format

XV.1.2. INT Format

This format is used by Intel, Mostek, and the CP/M operating system.

Leader:         One  character, a colon, indicating the start of a record.  All

characters preceding the colon should be ignored.

Byte count:     Two characters (ASCII-encoded hex)  indicating  the  number  of

data bytes in the record.  This byte count does not include the

leader  character,  byte  count,  or address information.  Each

data byte is  represent  by  two  characters,  this  is  not  a

character count.

Address:        Four  characters  (ASCII-encoded  hex)  indicating the starting

load address for the record.

Record Type:    Two characters, always 00, meaning an absolute block.  This  is

00  even  in  blocks  that contain just a starting address. The

starting address should be detected by  a  data  length  of  0,

rather  than  the  record  type.  In  this format, the starting

address is often given with record type 01. CROSS does  not  do

this.

Data:           Several pairs of ASCII-encoded hex bytes to be loaded.

Checksum:       Two  characters  of (ASCII-encoded hex) checksum.  The checksum

is the negated 8-bit sum  of  the  byte  count,  address  high,

address  low, and data bytes, after they have been converted to

binary.

XV.1.3. KIM Format

Leader:         One character, a semi-colon, indicating the start of a  record.

All characters preceding the semi-colon should be ignored.

Byte count:     Two  characters  (ASCII-encoded  hex)  indicating the number of

Object format

CROSS-Microcomputer cross assembler                    62

data bytes in the record.  This byte count does not include the

leader  character,  byte  count,  or address information.  Each

data byte is  represent  by  two  characters,  this  is  not  a

character count.

Address:        Four  characters  (ASCII-encoded  hex)  indicating the starting

load address for the record.

Data:           Several pairs of ASCII-encoded hex bytes to be loaded.

Checksum:       Four characters of (ASCII-encoded hex) checksum.  The  checksum

is  the  negated  16-bit  sum  of the byte count, address high,

address low, and data bytes, after they have been converted  to

binary.

XV.1.4. MOT Format

This  format uses the S1 and S9 records of the standard Motorola format.  The

S1 record is a data record, and consists of:

S1              The characters which indicate the record type.

Byte count      Two characters of byte  count  (ASCII-encoded  hex);  the  byte

count  is  the count of all following data bytes, including the

checksum.

Address         Four characters (ASCII-encoded hex) indicated the starting load

address for the record.

Data            Two characters of ASCII-encoded hex for each byte to be loaded.

Checksum        A two character (one byte, ASCII-encoded hex) checksum  of  the

binary  values of the byte count, address, and data bytes.  The

sum of all these bytes plus the checksum should equal FF  .

16

An S9 record is a transfer record, and consists of:

S9              The record type indicator

Byte count      Same as an S1 record.

Address         Four characters (two bytes  ASCII-encoded  hex);  the  transfer

address at which exection should begin; if there was no address

on the .end statement, the value 0 is assumed.

Checksum        Same as an S1 record.

XV.2. Binary file formats

Object format

CROSS-Microcomputer cross assembler                    63

XV.2.1. Packed binary file format

+---+---------------+---------------+---+---------------+---------------+

|///|  byte 2       |    byte 1     |///|    byte 4     |    byte 3     |

|///|               |               |///|               |               |

+---+---------------+---------------+---+---------------+---------------+

0 1 2 3 4 5 6 7 8 9 1 1 1 1 1 1 1 1 1 1 2 2 2 2 2 2 2 2 2 2 3 3 3 3 3 3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

2

This  is  the "packed" output mode .  The bytes must first be serialized, and

then interpreted according to the description given in section XV.2.3.

XV.2.2. Image binary file format

The image binary output form consists of one 8-bit byte in  the  low-order  8

bits  of a PDP-10 word.  Word 0 of the output file is the first byte, word 1 of

the output file is the second  byte,  etc.    The  bytes  must  be  interpreted

according to the description given in section XV.2.3.

XV.2.3. Object format

After  obtaining  a  serial  byte stream from either a packed or image binary

file, the bytes are interpreted as:

- some number (zero or more) of 0 bytes (discard)

- a byte with the value 1.  (This byte is counted in the checksum).

- a byte with the value 0.  (This byte is counted in the checksum,  but

of course it has no effect because it is 0).

- The  low-order  8  bits  of the byte count, which we will refer to as

BC .

L

- The high-order 8 bits of the byte count, which we will  refer  to  as

BC .  The byte count BC is thus computed by

H

BC = (BC  * 400 ) + BC

H      8      L

2

If  you  stare  at  this and squint your eyes a bit, you will see that it is

laid down as two PDP-11 words; the first word is in the left  PDP-10  halfword,

so  the  lower-addressed byte is to the right.  The second word is in the right

PDP-10 halfword.

Object format

CROSS-Microcomputer cross assembler                    64

- The  low-order 8 bits of the address where the data will go, which we

will refer to as A .  This byte is counted as part of BC.

L

- The high-order 8 bits of the address where the data will go, which we

will refer to as A .  This byte is counted as part of BC.   The  load

H

address A is computed as the byte count, i.e.,

A = (A  * 400 ) + A

H      8     L

- BC-6  bytes of data, in order, starting with the byte for the address

A, then A+1, A+2, etc.

- One byte of checksum, which is the complemented sum of all the bytes.

When all of the bytes starting at the constant 1  plus  the  checksum

are added, the result should be zero.

The  above  is  repeated  as often as required to load the data.  The last such

group has BC=6 and the starting address is A.  Thus, it contains no data bytes.

This is the only way to distinguish this "transfer  address"  record  from  any

other record.

Object format

CROSS-Microcomputer cross assembler                    65

XVI. Site-specific differences

The  CROSS program can be configured for various sites.  It is suggested that

if you wish to make site-specific changes, you make  them  in  a  copy  of  the

master  copy of the source, which currently resides at <MicroComputer>@Rutgers.

Use conditional compilation switches, and when you are done, contact Rutgers to

take your source  back.    In  this  way,  we  have  the  advantage  of  shared

maintenance.

Please  contact  Joe Newcomer (Newcomer@CMU-10A) for up-to-date documentation

sources.

The current site-dependent configuration switches are:

CMUASW          Uses CMU PPN format for CMU TOPS-10 systems

CMUSW           Looks for SWITCH.INI on device HOM: first.

Sets default page length and width for the Dover printer  using

SAILA8.

UTAHSW          Uses [4,156], <Micro>, as the PPN for .MCALL libraries.

Object format

CROSS-Microcomputer cross assembler                    66

Index

!   59

#, in cref   9

$<expr>   1, 59

%<expr>   1, 59

%nn, register   1

&   59

', concatenation   58, 59

*   59

*, in cref   9

+   59

-   59

., location counter   59

.ABS   54

.ADDR   54

.ASCII   54

.ASCIZ   54

.ASECT   54

.BLKB   54

.BLKW   54

.BYTE   54

.CSECT   54

.DEPHAse   54

.DSABL   5, 12, 54

.ENABL   4, 12, 54

.END   54

.ENDC   54

.ENDM   54

.ENDR   54

.EOT   54

.EQUIV   54

.ERROR   54

.FLT2   55

.FLT4   55

.GLOBL   55

.IF   55

.IFDF   55

.IFEQ   55

.IFF   55

.IFG   55

.IFGE   55

.IFGT   55

Index

CROSS-Microcomputer cross assembler                    67

.IFL   55

.IFLE   55

.IFLT   55

.IFNDF   55

.IFNE   55

.IFNZ   55

.IFT   55

.IFTF   55

.IFZ   55

.IIF   55

.IRP   55

.IRPC   56

.LIMIT   56

.LIST   4, 7, 8, 56

.LOCAL   56

.MACRO   56

.MCALL   14, 56

.MEXIT   56

.NARG   56

.NCHR   56

.NLIST   4, 7, 8, 56

.NTYPE   56

.PAGE   56

.PDP10   56

.PHASE   56

.PRINT   56

.PSECT   56

.RAD50   56

.RADIX   56

.REM   57

.REPT   57

.ROUND   12, 57

.SBTTL   57

.SETDP   49, 57

.TITLE   57

.TRUNC   12, 57

.WORD   57

/   59

/CDR   5

/CRF   5, 9

/DS   5, 12

/EN   4, 12

/EQ   6

/GNS   6, 10

/I   5

/LEN   6, 10

/LI   4, 8

/M05   4

/M08   4

/M18   4

/M48   4

Index

CROSS-Microcomputer cross assembler                    68

/M65   4

/M68   4

/M69   4

/M80   4

/M88   4

/MF8   4

/N   5, 9

/NL   4, 8

/NSQ   6

/OCT   5, 9

/P   5

/PTP   5

/SOL   6, 10

/WID   6, 11

1802   4, 34

3870   4

6502   4, 37

6800   4, 40

6805   4, 44

6809   4, 47

72, column   5

8008   4, 28

8048   4, 30

8080   4, 17

8085   4

;, comment delimiter   59

<...>   59

=   59

?, macro argument modifier   58

@, in command   3

@<expr>   1, 59, 1, 59

.ABS   54

ABS   12

Absolute memory addressing   12

.ADDR   54

AMA   12

And, assembler operator   59

.ASCII   54

.ASCIZ   54

.ASECT   54

Index

CROSS-Microcomputer cross assembler                    69

^B<expr>   1

<expr>\   1

\<expr>   58

^B<expr>   59

<expr>\   59

BEX, listing option   7

BIN

extension   3

listing option   7

Binary listing option   7

Binary radix   1

.BLKB   54

.BLKW   54

.BYTE   54

Byte

high   1

Low   1, 59

^C<expr>   59

Card reader simulation   5

Catenation   15

CCL file   3

/CDR   5

CMUSW   65

CND, listing option   7

COM, listing option   7

Command line   3

Comment

delimiter   59

listing option   7

Concatenation operator   58, 59

Conditional

compilation: .IF   55

listing option   7

Contents, listing option   8

COSMAC

COSMAC 1802   4, 34

Format   61

See also   1802

/CRF   5, 9

CRF.TMP file   9

Cross-reference   5

.CSECT   54

Current location counter   59

D<expr>   1, 59

Decimal radix   1

.DEPHAse   54

/DS   5, 12

.DSABL   5, 12, 54

DSK, device   3

Index

CROSS-Microcomputer cross assembler                    70

/EN   4, 12

.ENABL   4, 12, 54

.END   54

.ENDC   54

.ENDM   54

.ENDR   54

.EOT   54

/EQ   6

.EQUIV   54

ERF   12

.ERROR   54

Error flags, suppression   12

Error message suppression   5

Exorcisor Format   5, 62

$<expr>   1, 59

%<expr>   1, 59

@<expr>   1, 59

^B<expr>   1, 59

^C<expr>   59

^H<expr>   1, 59

^O<expr>   1, 59

<expr>\   1

\<expr>   58

<expr>\   59

<expr>^   1, 59

Extension

.BIN   3

.CCL   3

.LST   3

.M05   3

.M08   4

.M18   4

.M48   3

.M65   4

.M68   4

.M69   4

.M80   4

.M88   4

.MF8   4

.SML   14

F8   4

Fairchild

F8   4

Floating point truncation   12

.FLT2   55

.FLT4   55

Formats

COS   5, 61

COSMAC   5, 61

CP/M   5, 61

Exorcisor   5, 62

Index

CROSS-Microcomputer cross assembler                    71

Image binary   63

INT   5, 61

Intel   5, 61

Intellec   5, 61

KIM   5, 61

MOS   5

MOS Technology   5, 61

Mostek   61

MOT   5, 62

Motorola   5, 62

Packed binary   63

PTP   61

FPT   12

Generate new source   6, 10

.GLOBL   55

/GNS   6, 10

^H<expr>   1, 59

Hex output   5, 61, 62

Hexadecimal radix   1

High byte   1

/I   5

.IF   55

.IFDF   55

.IFEQ   55

.IFF   55

.IFG   55

.IFGE   55

.IFGT   55

.IFL   55

.IFLE   55

.IFLT   55

.IFNDF   55

.IFNE   55

.IFNZ   55

.IFT   55

.IFTF   55

.IFZ   55

.IIF   55

Image mode output   5

Intel

8008   4, 28

8048   30

8080   4, 17

8085   4

MCS-48   4, 30

.IRP   55

.IRPC   56

KIM

Index

CROSS-Microcomputer cross assembler                    72

Format   5, 61

See also   6502

LC   12

LD, listing option   8

/LEN   6, 10

/LI   4, 8

Libraries, macro   14

.LIMIT   56

Line numbers   6

.LIST   4, 7, 8, 56

Listing option

BEX   7

BIN   7

CND   7

COM   7

LD   8

LOC   7

MB   7

MC   7

MD   7

ME   7

SEQ   7

SRC   7

SYM   8

TOC   8

TTM   8

LOC, listing option   7

.LOCAL   56

Location counter   59

Low byte   1, 59

Lower case   12

LPT, device   3

LSB   12

LST, extension   3

M05

/M05 switch   4

extension   3

M08

/M08 switch   4

extension   4

M18

/M18 switch   4

extension   4

M48

/M48 switch   4

extension   3

M65

/M65 switch   4

extension   4

M68

Index

CROSS-Microcomputer cross assembler                    73

/M68 switch   4

extension   4

M69

/M69 switch   4

extension   4

M80

/M80 switch   4

extension   4

M85   12

M88

/M88 switch   4

extension   4

.MACRO   56

Macro call listing option   7

Macro definition   56

Macro libraries   14

Macro, concatenation   58, 59

MB, listing option   7

MC, listing option   7

.MCALL   14, 56

MCS-48   4, 30

MD, listing option   7

ME, listing option   7

Memory addressing, absolute   12

.MEXIT   56

MF8   4

extension   4

MOS Technology

6502   4, 37

Mostek

3870   4

Motorola

6800   4, 40

6805   4, 44

6809   4, 47

/N   5, 9

.NARG   56

.NCHR   56

New source, generation   6, 10

/NL   4, 8

.NLIST   4, 7, 8, 56

See also   ^C

/NPP   12

/NSQ   6

.NTYPE   56

^O<expr>   1, 59

See also   Formats

/OCT   5, 9

Octal radix   1, 5

Or, assembler operator   59

Index

CROSS-Microcomputer cross assembler                    74

Output format   5

/P   5

%<expr>   1, 59

Packed binary output   5

.PAGE   56

.PDP10   56

.PHASE   56

Phase error   47

.PRINT   56

.PSECT   56

/PTP   5

PTP, device   3

PTR, device   3

.RAD50   56

.RADIX   56

Radix

10   1

16   1

2   1

8   1, 5

binary   1

decimal   1

default   1

hexadecimal   1

octal   1

RCA

1802   4, 34

REG   12

Register %nn   1

.REM   57

.REPT   57

RIM, opcode   12

Rockwell

6502   37

.ROUND   12, 57

S1 record   62

S9 record   62

.SBTTL   57

SEQ, listing option   7

.SETDP   49, 57

SIM, opcode   12

Site-dependent

CMU   65

Utah   65

/SOL   6, 10

Source listing option   7

Source, new, generation   6, 10

SRC, listing option   7

Suppression of error flags   12

Index

CROSS-Microcomputer cross assembler                    75

Switch

CDR   5

CRF   5, 9

DS   5, 12

EN   4, 12

EQ   6

GNS   6, 10

I   5

LEN   6, 10

LI   4, 8

M05   4

M08   4

M18   4

M48   4

M65   4

M68   4

M69   4

M80   4

M88   4

N   5, 9

NL   4, 8

NPP   12

NSQ   6

OCT   5, 9

P   5

PTP   5

SOL   6, 10

WID   6, 11

SWITCH.INI   13

SYM, listing option   8

Symbol table listing option   8

Synertek

6502   37

SYSMAC.SML   14

Table of contents, listing option   8

Terminal format listing option   8

TIM   12

.TITLE   57

TOC, listing option   8

.TRUNC   12, 57

Truncation   12

TTM, listing option   8

TTY, device   3

<expr>^   1, 59

UTAHSW   65

/WID   6, 11

.WORD   57

Z80   4, 12, 21

Index

CROSS-Microcomputer cross assembler                    76

Z80 transliteration   23

Zilog

Z80   4, 21

\, macro argument modifier   58

\<expr>   58

<expr>\   59

<expr>^   1

^<expr>   59

^B<expr>   1, 59

^C<expr>   59

^H<expr>   1, 59

^O<expr>   1, 59

|   15

Index

CROSS-Microcomputer cross assembler                     i

Table of Contents

1. Introduction                                                               1

2. Syntax differences                                                         2

3. Operating procedures                                                       3

3.1. Loading CROSS                                                            3

3.2. Initial dialogue                                                         3

4. Listing control                                                            7

4.1. Source statement listing control                                         7

4.2. Command string equivalents                                               8

4.3. Other listing controls                                                   9

5. Enable/Disable control                                                    12

6. User-defined switch defaults                                              13

7. Macro libraries                                                           14

I. General notation                                                          15

I.1. Machine State                                                           15

I.2. Notation                                                                15

I.3. Syntax                                                                  15

I.4. Opcodes                                                                 15

II. 8080 syntax                                                              17

II.1. Machine State                                                          17

II.2. Notation                                                               17

II.3. Syntax                                                                 18

II.4. Opcodes                                                                18

III. Z80 syntax                                                              21

III.1. Introduction                                                          21

III.2. Machine State                                                         21

III.3. Notation                                                              21

III.4. Opcodes                                                               21

III.5. Z80 Transliterations                                                  23

IV. 8008                                                                     28

Index

CROSS-Microcomputer cross assembler                    ii

IV.1. 8080 subset                                                            28

V. 8048                                                                      30

V.1. Machine State                                                           30

V.2. Notation                                                                30

V.3. Syntax                                                                  31

V.4. Opcodes                                                                 31

VI. 1802                                                                     34

VI.1. Machine State                                                          34

VI.2. Opcodes                                                                34

VII. 6502                                                                    37

VII.1. Machine state                                                         37

VII.2. Notation                                                              37

VII.3. Syntax                                                                38

VII.4. Opcodes                                                               38

VIII. 6800                                                                   40

VIII.1. Machine State                                                        40

VIII.2. Notation                                                             40

VIII.3. Syntax                                                               41

VIII.4. Opcodes                                                              41

IX. 6805                                                                     44

IX.1. Machine State                                                          44

IX.2. Notation                                                               44

IX.3. Syntax                                                                 44

IX.4. Opcodes                                                                45

X. 6809                                                                      47

X.1. Machine State                                                           47

X.2. Notation                                                                48

X.3. Syntax                                                                  48

X.4. Opcodes                                                                 49

XI. Compiler directives                                                      54

XII. Macro quick summary                                                     58

XIII. Assembler syntax quick summary                                         59

Index

CROSS-Microcomputer cross assembler                   iii

XIV. Error codes                                                             60

XV. Object format                                                            61

XV.1. PTP object format                                                      61

XV.1.1. COSMAC format                                                   61

XV.1.2. INT Format                                                      61

XV.1.3. KIM Format                                                      61

XV.1.4. MOT Format                                                      62

XV.2. Binary file formats                                                    62

XV.2.1. Packed binary file format                                       63

XV.2.2. Image binary file format                                        63

XV.2.3. Object format                                                   63

XVI. Site-specific differences                                               65

Index                                                                        66

Index