8. IBM VM/CMS KERMIT

Author:     Daphne Tzoar, Columbia University

Version:    ( unnumbered )

Date:       February 1983

Written  in  IBM  370  assembly  language to run under VM/CMS on IBM 370-series

mainframes (System/370, 303x, 43xx, 308x, ...).  These are half duplex systems;

the communication line must "turn around" before any data can be  sent  to  it.

The  fact  that  a packet has been received from the IBM system is no guarantee

that it is ready for a reply.  Thus any Kermit talking to this system must wait

for the line turnaround character (XON) before transmitting the next character.

IBM systems talk to their terminals through a  communications  front  end  (IBM

3705,  3725, COMTEN 3670, etc).  These front ends generally insist on using the

8th bit of each character for parity.  This means that binary files (files con-

taining other than ordinary letters,  digits,  punctuation,  carriage  returns,

tabs, and so forth) can not be correctly sent or received by these systems with

Kermit (protocol version 1).

The  IBM  system  under  VM/CMS is unable to interrupt a read on its "console".

This means that the IBM version of Kermit cannot timeout.    The  only  way  to

"timeout"  CMS Kermit is from the other side -- typing a carriage return to the

micro's Kermit causing it to  retransmit  its  last  packet,  or  an  automatic

timeout  as  provided  by  Kermit-20.  For this reason, CMS  Kermit  waits  ten

seconds before sending its first packet when sending files from VM/CMS.    This

gives  the  user  sufficient  time  to return to the local Kermit and issue the

Receive command.  Otherwise, a protocol deadlock would arise  requiring  manual

intervention by the user.

Also, VM/CMS stores files as records rather byte streams.  VM/CMS Kermit has to

worry  about  assembling incoming data packets into records and stripping CRLFs

from incoming lines, and also appending CRLFs  to  --  and  stripping  trailing

blanks from -- outgoing records.

The VM/CMS file specification is in the form

FILENAME FILETYPE FILEMODE

(abbreviated  FN  FT FM).  FM is equivalent to a device specification on DEC or

microcomputer systems (FN FT FM would translate to  FM:FN.FT).    FILENAME  and

FILETYPE  are  at  most  8  characters in length, each, and FILEMODE at most 2.

When FILEMODE is omitted from a filespec,  the  user's  own  disk  is  assumed.

Kermit-CMS sends only FILENAME and FILETYPE, and converts the intervening blank

to  a  period  for compatibility with most other operating systems.  Kermit-CMS

Commands:

SEND fn ft [fm]

Send the specified file(s), using * or % as the wildcard characters  (*

will  match  any  number  of  characters  while  %  matches  only one).

Kermit-CMS assumes the file is located on the  A  disk,  and  sets  the

filemode  to A1.  If, however, the file is located on a different disk,

the filemode must be cited.  Also, note that  if  you  use  *  for  the

filemode,  Kermit-CMS  will send only the first file that matches.  Ex-

amples:

The  command  SEND CEN SPSS will send CEN SPSS A1.  To send the

same file located on your B disk, you must  specify:  SEND  CEN

SPSS  B.   SEND * FORTRAN will send all fortran files on your A

disk.  SEND ABC% EXEC will send all exec files with a four let-

ter filename beginning with ABC. If you have the file PLOT  SAS

on  your A disk and your B disk, SEND PLOT SAS * will send PLOT

SAS A1.

RECEIVE [fn ft [fm]]

Receive the file(s) sent from the other Kermit.  If a  file  specifica-

tion  is  not included, Kermit-CMS will use the name(s) provided by the

other Kermit.  Use the  file  specification  to  indicate  a  different

filename  or  a disk other than the A disk (in this case, the file name

and type must also be supplied or = = FM can be used.)  Examples:

To receive files using the filename(s) sent by the micro,  use:

RECEIVE.    To  save  a  file  under a different name, specify:

RECEIVE ABC FORTRAN.  To save the file under the same name  but

on  the  B  disk,  specify:  RECEIVE  ABC FORTRAN B, or RECEIVE

= = B.

SET parameter value

Set the parameter to the specified value.  Legal Set commands are:

RECFM option

Denotes the record format to be used when  creating  the  file.

Only fixed and variable length records are allowed, where vari-

able is the default.  Indicate the desired record format by ei-

ther an F (fixed) or a V (variable).

LRECL decimal-number

Indicates  the  logical  record length.  The default is 80, and

the maximum allowed is 256.

QUOTE decimal-number

The ASCII value of the control prefix character you wish to use

in place of the default (#).  It must be  a  single,  printable

character  from  among  the  following:  33-62,  96, or 123-126

(decimal).

END decimal-number

Indicates the ASCII value  of  the  end-of-line  character  you

choose  to send.  The default is CR (ASCII 13 decimal), but can

be set to any two digit number between 00 and 31 (decimal).

PAC decimal-number

Allows the user to specify the packet size the micro should use

when sending to Kermit-CMS.  The range is 26-94,  where  94  is

the default.

SHOW parameter

Displays  the current value of any variable that can be changed via the

SET command.

STATUS  Returns the status of the previous execution of Kermit-CMS.  Therefore,

STATUS will either display the message "Kermit completed successfully",

or the last error encountered prior to aborting.

CMS     Issues a CMS command from within Kermit-CMS.

CP      Issues a CP command from within Kermit-CMS.

?       Lists all legal Kermit-CMS commands.

This is a list of other salient facts about Kermit-CMS:

1. The commands are supplied with a help option, so a question mark can

be  typed  to  get the appropriate format or a list of options.  The

question mark, however, must  be  followed  by  a  carriage  return;

Kermit-CMS will respond and display the prompt again.  For instance,

SET ? will list all valid options for the SET command.

2. When  receiving  files,  if  the  record format is fixed, any record

longer than the logical record length will be  truncated.    If  the

record  format is variable, the record length can be as high as 256.

For sending files, the maximum record length is 256.

3. Before connecting to the IBM mainframe from other systems (like  the

various  microcomputer  and  PC  Kermits,  DEC-20  Kermit, etc), you

should set the IBM flag ON so that echoing, parity, and  handshaking

can be done the way the IBM system likes.

4. Note  that  "("  and  ")"  act as word separators on the input line.

Therefore, if you try to set the quote character to  "(*"  or  "*(",

for example, only the first character will be used.

5. Since  some  Kermits  do not send an error packet when they "abort",

Kermit-CMS does not always know the micro has stopped sending it in-

formation.  Therefore, when you connect back to the IBM,  Kermit-CMS

may  still be sending packets (they will appear on the screen).  The

user must hit a carriage return until Kermit-CMS has sent  the  max-

imum  number  of  packets  allowed  and  aborts.  The error message,

however, will not indicate that communication  stopped  because  the

micro  aborted,  but  rather  that  no start of header character was

found.

6. The minimum send packet size Kermit-CMS will allow is 26.   This  is

necessary  to  avoid an error while sending the filename or an error

packet.  If the micro tries to set the value to  be  less  than  26,

Kermit-CMS  will immediately abort with an error of "Bad send-packet

size."

7. While the IBM's communication front end processor translates all in-

coming characters from ASCII terminals to EBCDIC, Kermit-CMS  trans-

lates  the data it reads back to ASCII (characters not representable

in ASCII are replaced by a null).  Not only is  it  easier  to  work

with  ASCII  characters,  but it makes things more consistent throu-

ghout the many versions of Kermit.  When the packets are sent to the

micro, Kermit-CMS converts all data back to EBCDIC.   The  ASCII  to

EBCDIC translation table can be found in the Appendix.

8. If  a transfer becomes stuck, you can CONNECT back to the CMS system

and type a lot of carriage returns -- each one will cause KERMIT-CMS

to  retransmit the current packet, until the retransmission limit is

reached, and you will be back at "KERMIT-CMS>" command level.

9. Kermit-CMS supplies the micro and the user with numerous error  mes-

sages.    If  the  execution must be abnormally terminated, an error

packet is sent to the micro before Kermit-CMS stops.  The same  mes-

sage can be retrieved via the STATUS command when Kermit-CMS returns

and  displays the prompt.  If Kermit-CMS aborted because the maximum

amount of retries was exceeded (20 on initialization packets  and  5

on  others),  the  error  message will display the most recent error

(i.e. the last NAK Kermit-CMS encountered).  If execution stops  be-

cause  the  micro gave up, the error message will convey that to the

user, but it is the micro's responsibility to  pinpoint  the  error.

The messages Kermit-CMS gives are as follows:

"Bad send-packet size"

Sent when the micro attempts to set its receive buffer size to a

value  that  is  less  than 26 (the minimum that Kermit-CMS will

accept) or larger than 94, the maximum.  It will also  occur  if

Kermit-CMS  tries  to send a packet that is larger than the max-

imum specified.

"Bad message number"

This and the following messages flag inconsistencies in a Kermit

packet.

"Illegal packet type" -- This can be caused by sending  server  com-

mands.

"Unrecognized State"

"No SOH encountered"

"Bad Checksum"

"Bad character count"

"Micro sent a NAK"

"Lost a packet"

"Micro aborted"

The micro abnormally terminated the transfer.

"Illegal file name"

When  receiving  the name of the file from the micro, Kermit-CMS

expects it to be in the  format  'filename.filetype'.    If  the

filename,  filetype,  or  dot is missing, Kermit-CMS will reject

(NAK) the packet.  Also, if either the filename or filetype  ex-

ceeds eight characters, it will be truncated.

"Invalid lrecl"

Kermit-CMS  will  abort  on  any file-system error it encounters

when reading from the file it is to send.    It  can  only  send

files  with  variable or fixed length record formats, therefore,

Wylbur Edit or Packed format files will cause an error.

"Permanent I/O error"

This signifies a permanent I/O error that occured  when  reading

from an existing file.  Execution is aborted immediately.

"Disk is read-only"

Attempt to write on a read-only disk.

"Recfm conflict"

If  a  filename  conflict  arises,  Kermit-CMS  will  append the

received file to the existing one, provided the  record  formats

of  the  two  are  the same.  Otherwise, this error will cause a

halt of the execution.

"Disk is full"

Refers  to  any  error regarding limitations on a user's storage

space.  Most likely, it signifies that  the  receiving  disk  is

full,  but  the  error  can also mean that the maximum number of

files allowed has been reached, or virtual storage capacity  has

been exceeded, and so on.

"Err allocating space"

Kermit-CMS  keeps a table of all files it has sent to the micro,

allocating extra space if more than ten files are  sent  at  one

time.    If  there  is an error obtaining more space, Kermit-CMS

will abort with this error.

Work on VM/CMS Kermit continues.  Planned future enhancements include:

1. 8-bit quoting, to allow binary files to pass  through  communication

front ends that insist on using the 8th bit for parity.

2. Ability to act as a Kermit Server.

3. Ability  to SET LINE, so that Kermit-CMS can be used as a local Ker-

mit, connecting to a remote host over another communication port.

Index

End-Of-Line (EOL)   74

IBM   73

LRECL   74

Packet-Length   74

Quote   74

RECEIVE   74

RECFM   74

SEND   73

SET   74

SHOW   74

Timeout   73

VM/CMS   73

Table of Contents

8. IBM VM/CMS KERMIT                                                         73

Index                                                                        78