LA34 SPEED

----------

The program LA34SPEED.TSK will change the speed of the line attached to

TT2: (the printer port) to that requested in it.

It is distributed with the relevent value set to 5 (300 baud) and may

be changed with ZAP as follows:

>ZAP

ZAP>LA34SPEED.TSK/AB

_4:246/

5

_enter new value here

0	50

1	75

2	110

3	134.5

4	200

5	300

6	600

7	1200

10	1800

11	2000

12	2400

13	3600

14	4800

15	7200

16	9600

17	19200

_X

>

Then 	>RUN LA34SPEED

which will set the port speed.

The connection to the printer must have the following characteristics:

Null MODEM	(crossed pins 2 & 3)

Dataset ready asserted (or you will get PIP error -3 Device not ready)

(connect pins 5 and 6 together for this)

The connection of pins 7 and 8 together says that the thing is the

console terminal, and BREAK is then active. It is irrelevent to the printer!.