_#N_#A_#M_#E

cux - call up another system

_#S_#Y_#N_#O_#P_#S_#I_#S

cux   (for UNIX version)

cuv   (for VAX/VMS version)

cdc   (for CDC NOS version)

_#D_#E_#S_#C_#R_#I_#P_#T_#I_#O_#N

Cux allows communication with another system.  It  manages

an interactive conversation with possible transfer of text

files and i/o redirection.

The program requires two serial ports.  Input from tty and

the  other  port is managed by AST service routines.  This

provides for up to 1200 baud communication  through  a  DZ

asynchronous multiplexer.

To connect to another system, run cux and then dial up the

other  computer.   At  this  point, anything typed at your

terminal is transmitted to the remote system.  You are ef-

fectively talking with the other system.

Once a connection is made, lines beginning with  control-W

(^W) have special meanings.

Cux interprets the following:

^W.                 terminate the conversation.

^W^Z                terminate the conversation.

^W<[:]file          send the contents of 'file' to the re-

mote  system  as  though  typed at the

terminal.

^W>[>][:]file       divert output to the named file.

^W!                 escape temporarily to  enter  commands

to the local system.

^W%take from [to]   copy file 'from' (on the  remote  sys-

tem) to file 'to' on the local system.

If 'to' is omitted, the 'from' name is

used both places.

^W%put from [to]    copy file 'from' (on local system)  to

file  'to'  on  the remote system.  If

'to' is omitted, the  'from'  name  is

used both places.

^Wx                 toggle 'start'  and  'stop'  character

transmission   to  the  other  system.

(xon-xoff protocol observed)

^We                 toggle   the   echo   of    characters

transmitted   to  the  remote  system.

This can be used  to  override  silent

diversion.

PAGE 2

^W^W                send the character '^W'

^Wa                 abort 'put' or '<'

^Wh                 print any available help text

The receive process handles output diversions of the  fol-

lowing form:

^W>[>][:]file

for zero or more lines to be written to 'file' or

^W>

to terminate the diversion.

In any case, output is diverted (or appended, if  '>>'  is

used)  to  the  file.   If  ':'  is used, the diversion is

silent, i.e., it is written only to the file.  If  ':'  is

omitted,  output  is  written  both to the file and to the

standard output.  The trailing '^W' terminates the  diver-

sion.

Conditional compilation permits tailoring the program to a

particular  operating  system.   Refer to cu.h for further

details.

_#A_#U_#T_#H_#O_#R

Robert J.  Donovan

Graef Anhalt Schloemer & Associates

Milwaukee, Wi 53216

414-461-6900

_#B_#U_#G_#S

Because operating system  service  is  required  for  each

character entered and received, throughput may greatly de-

teriorate when the system is under heavy (or even  normal)

use.