KERMIT UCSD is a version of KERMIT which runs under the UCSD psystem.  Due to

the limitations of UCSD Pascal, there are several routines which must be

written in assembly language.  This version was implemented on a Terak 8510A,

which is a PDP 11/2.  Detailed below are the functions of the routines, as

well as information needed for linking.

The command parser is a separate compilation unit, which has the filename

PARSER.

KERMIT uses assembly routines for host computer and keyboard

receiving. The file RCVHANDLR.TEXT contains all the routines for handling

the characters received from the host, KBDHANDLR.TEXT contains all the

routines for handling characters received from the keyboard, and SENDB contains

the routine for sending a continuous break to an IBM mainframe.

To build KERMIT, first assemble PARSER, RCVHANDLR, KBDHANDLR & SENDB.

PARSER must then be linked into a library called PARSELIB.  Then compile

KERMIT then finally enter the following linker sequence:

Host file? KERMIT

Lib file? PARSELIB

Lib file? SENDB

Lib file? RCVHANDLR

Lib file? KBDHANDLR

Lib file? <cr>

Map name? <cr>

Output file? KERMIT.CODE

The order in which the library files are typed in makes a difference (for some

unknown reason), as does the '.CODE' extension on the output file name.

The following declarations are needed for KBDHANDLR and RCVHANDLR

CONST RCVQSIZE = <maximum number of queued characters>

TYPE QUEUE = RECORD    (* These are order-dependent !!! *)

QSIZE: INTEGER

INP: INTEGER

OUTP: INTEGER

MAXCHAR: INTEGER

DATA: PACKED ARRAY [0..RCVQSIZE] OF CHAR

END

VAR RCVQ: QUEUE     (* must be declared in outermost block *)

The routines RCVINIT and KBDINIT must be called at the beginning of the

program in order to set up interrupt vectors and handlers.  They are

declared as

PROCEDURE RCVINIT (VAR Q: QUEUE; SIZE:INTEGER) EXTERNAL

PROCEDURE KBDINIT (VAR Q: QUEUE; SIZE:INTEGER) EXTERNAL

and should be called appropriately.  RCVINIT takes care of setting other

system interrupt priorities so that keyboard and remote input override them.

KBDINIT only deals with the keyboard interupts, since others will be taken

care of by RCVINIT.

The routines RCVFINIT and KBDFINIT should be Mcalled at the end of the

program in order to restore interrupt vectors, handlers and priorities.  They

are declared as

PROCEDURE RCVFINIT; EXTERNAL;

PROCEDURE KBDFINIT; EXTERNAL;

and are called with no parameters.

The queue handlers run continuously as interrupt-driven tasks at high

priority.  As characters come in, they advance the queue INP pointer and

keep track of the maximum number of characters in the queues in the MAXCHAR

variables.  Queue overflow is indicated by MAXCHAR > QSIZE.

To send a break the routine SENDBRK is used, which sends a continuous

break and should be declared as

PROCEDURE SENDBRK; EXTERNAL;

KERMIT-UCSD is a version of KERMIT written primarily in UCSD Pascal

Version II.0.  There are four assembly language routines which manage special

keyboard and remote input interrupt handlers, and one more which sends a

continuous break signal.  The program was developed on a TERAK 8510A

microcomputer, which is a PDP-11/2.  With the rewriting of the above five

routines, plus two interrupt handlers, the program should be easily movable

to any UCSD psystem.

KERMIT-UCSD has the following limitations:

1) No wild card designations of file names.

2) No eight-bit file quoting.

3) No character repeat counts.

4) No '?' and <esc> at the end of a command line.

5) No server communications.

6) Sending and receiving cannot be done for anything but .TEXT files

(which contain a two-block header and space compression codes).

7) Instead of using a clock to time out when waiting for a packet,

KERMIT-UCSD has a limit on the number of times it will look for

a response before giving up.

The commands recognized by KERMIT-UCSD are listed below.

CONNECT     To make a "virtual terminal" connection to a remote

system. To break the connection and "escape" back to the

micro, type the escape sequence (CTRL-] C, that is Control

rightbracket followed immediately by the letter C.)

EXIT        To return to main psystem command level.

HELP        To get a list of KERMIT commands.  HELP can be followed by

any command, in which case the help will refer only to that

command.

RECEIVE     To accept a file from the remote system.

SEND        To send a file or group of files to the remote system.

Takes a filename as a parameter.

SET         To establish system-dependent parameters.  The SET options

are as follows:

DEBUG            To set debug mode ON or OFF (default is OFF).

END-OF-LINE      To change the character used at the end of

packets to something other than the default

of CR.  It must be a digit between 0 and 31.

ESCAPE           To change the escape sequence that lets you

return to the PC Kermit from the remote host.

The default is CTRL-] c.

FILE-WARNING     ON/OFF, default is OFF.  If ON, Kermit will

rename an incoming file so as not to

write over a file that currently exists with the

same name.

IBM              ON/OFF, default is OFF.  This flaMg should be

ON only when transfering files between the PC

and an IBM VM/CMS system.  It also causes the

parity to be set appropriately and activates

local echoing.

LOCAL-ECHO       ON/OFF, default is OFF.  This sets the duplex.

It should be ON when using the IBM and OFF for

the DEC-20.

PARITY           EVEN, ODD, MARK, SPACE, or NONE.  NONE is the

default but if the IBM flag is set, parity is

set to MARK.  This flag selects the parity for

outgoing and incoming characters during CONNECT

and file transfer to match the requirements of

SHOW        To see the values of parameters that can be modified

via the SET command.  SHOW ALL shows all parameters;  SHOW

followed by a parameter listed under SET will show the value

of only that parameter.

This version of KERMIT does not have a SET BAUD command because the baud rate

is set by hardware on the TERAK.