Included in this directory is a modified version of the

Fermilab CD: driver from the Fall, 1982, RSX SigTape.

The modifications are primarily to extend the driver

to support 11M-Plus; however, they are conditionalized

for 11M also, and provide the 11M user with the 22-bit

support that is lacking from the original CDDRV.

What is CDDRV?  It is a CPU-to-CPU communications package

which allows two CPU's (PDP11's, or a PDP-11 to a VAX, once

Fermilab's VAX version becomes available) to communicate

using a DR11W high-speed DMA interface at each end.

CDDRV is, in fact, really a message-switching system as well

as a driver.  Its means of determining data packet routing

is a user-assignable packet-type-code.  A task which is to

receive data must declare the packet type codes which it is

to accept; there being no conflicts with existing assignments,

the task then becomes sole "owner," or receiver, for packets

with those packet-type-codes.

The driver also implements an AST mechanism which gives

a receiver task an immediate indication when one

of its "owned" packet types is being transmitted.

This allows receiver tasks to "listen" for data without

actually having to have an I/O operation pending

until there is data to be received.

Everything needed to build the driver should be present in this

directory. The files should be copied to a privileged UIC,

then CDC.CMD should be executed to set up the conditionals file,

and CDDRV.MAK executed to build the driver.

CDC.CMD		command file to create conditionals file

CDPREF.MAC	prefix file

CDDRV.MAC	driver source code

CDTAB.MAC	database source code

CDDRV.MAK	command file to assemble & build driver

PN172.MEM	Original Fermilab documentation file,

the CDDRV portion of which should apply to

this version of CDDRV.

PN172.RNO,.RNT,.BTC  Various files used by Fermilab to generate

their PN172.MEM?

The changes were developed and tested on PDP-11/70's

and on an LSI-11/23, using MDB's DR11W interfaces, but

they should work on Unibus systems with any of the other

DR11W available. On a Q-bus system, however, the changes for

22-bit mode are specific to MDB's MLSI-DR11-W board.  This is due

to the fact that there is no standard as yet, other than MDB's,

for extending the DR11W device registers to 22 bits.

This version is relatively new, but so far appears to be reliable

after about two months of experimental use.  We would appreciate

hearing about problems, or better yet, fixes for problems you might

find.

For more information and for the related CDPACK subroutine package,

refer to the Fall, 1982, RSX Sig tape; or to the latest release by

Fermilab of their CD:-related software (look elsewhere on this tape

for a new release by Fermilab).