a

HGRAPH Documentation - January 30, 1984

Version 2.1

Written by

H. L. McMaken

Applied Mathematical Sciences

Ames Laboratory - USDOE

Iowa State University

All rights (and wrongs) reserved

The following documents the line  drawing  routines  developed

for  use  on  the  LSI  11/23.   These routines will work with

either the Houston Instruments DMP-3  or  DMP-29  pen  plotter

(HIPLOT), the Grinnell Systems GMR 271 image processing system

(GRINNELL), or the Visual Products 550 (VISUAL).   With  minor

modifications,   it   will   work  with  any  Tektronixs  4014

compatible graphics terminal.

PAGE 2

UPGRADES FROM VERSION 1.1

The major upgrade of V2.1 from V1.1 is its ability to plot  on

either  a  graphics terminal or a pen plotter by the choice of

the unit number in the call to INIPLT.  This allows a user  to

write  a  program  to interactively edit his/her plot until it

"looks right", then send the  edited  graph  directly  to  the

plotter  without  relinking  or  rerunning  the program.  This

amounts to a tremendous savings in time when  working  with  a

small system.  To facilitate this editing process, several new

routines have been added to control a graphics cursor.   These

routines will put the cursor at a given position, and read the

location of the cursor.  Also, a set  of  routines  have  been

added to generate legends for the plots.  These include legend

writing routines which can be run  interactively  for  dynamic

placement   of  the  legends.   All  routines  have  now  been

rewritten in FLECS for enhanced readability and modualarity.

PAGE 3

TABLE OF CONTENTS

I.    TERMINOLOGY  . . . . . . . . . . . . . . . . . .   1

II.   GETTING STARTED  . . . . . . . . . . . . . . . .   2

III.  INITIALIZING AND TERMINATING PLOTS . . . . . . .   4

INIPLT

ENDPLT

DMPPLT

IV.   TWO DIMENSIONAL ROUTINES . . . . . . . . . . . .   6

FRAME

WINDOW

SCALE

AXIS

DASHLN

LINE

V.    LEGEND WRITING ROUTINES  . . . . . . . . . . . .  12

INILGN

ENDLGN

WRILGN

CONLGN

RECLGN

VI.   AUXILIARY ROUTINES . . . . . . . . . . . . . . .  15

MOVETO

ISCRX

ISCRY

XWORLD

YWORLD

CURPOS

DRWREC

CIRCLE

ERASE

VII.  CONTROLLING PROGRAM INPUT AND OUTPUT . . . . . .  17

PLTOFF

PLTON

VIII. CHARACTER READ AND WRITE ROUTINES  . . . . . . .  18

PLTSTR

WRTSTR

REDSTR

GETSTR

PAGE 4

IX.   CURSOR CONTROL ROUTINES  . . . . . . . . . . . .  19

DEFWIN

FNDCUR

PUTCUR

CURSON

X.    SUPPORT ROUTINES . . . . . . . . . . . . . . . .  20

XAXIS

YAXIS

LABELX

LABELY

DRXTIC

DRYTIC

IFRMT

TRIML

TRIMSP

ALLOUT

FNDINT

MOV1ST

DRDASH

VUPORT

INSECT

XI.   MISCELLANEOUS ROUTINES . . . . . . . . . . . . .  22

PLOT

MARKER

COLTYP

PENDWN

PENUP

WRITCH

XII.  THREE DIMENSIONAL ROUTINES . . . . . . . . . . .  23

PLOT3D

XIII. RUNNING A PROGRAM  . . . . . . . . . . . . . . .  25

XIV.  DISPLAYING A DISK PLOT FILE  . . . . . . . . . .  26

APPENDIX A - DEVICE INDEPENDENT ROUTINES

ALLOUT . . . . . . . . . . . . . . . . . . . .  28

AXIS   . . . . . . . . . . . . . . . . . . . .  29

CURPOS . . . . . . . . . . . . . . . . . . . .  30

DASHLN . . . . . . . . . . . . . . . . . . . .  31

DRDASH . . . . . . . . . . . . . . . . . . . .  32

DRWREC . . . . . . . . . . . . . . . . . . . .  34

DRXTIC . . . . . . . . . . . . . . . . . . . .  35

DRYTIC . . . . . . . . . . . . . . . . . . . .  36

ENDLGN . . . . . . . . . . . . . . . . . . . .  37

FNDINT . . . . . . . . . . . . . . . . . . . .  38

FRAME  . . . . . . . . . . . . . . . . . . . .  39

IFRMT  . . . . . . . . . . . . . . . . . . . .  40

INILGN . . . . . . . . . . . . . . . . . . . .  41

ISCRX  . . . . . . . . . . . . . . . . . . . .  42

PAGE 5

ISCRY  . . . . . . . . . . . . . . . . . . . .  43

LABELX . . . . . . . . . . . . . . . . . . . .  44

LABELY . . . . . . . . . . . . . . . . . . . .  45

LINE   . . . . . . . . . . . . . . . . . . . .  46

MOVETO . . . . . . . . . . . . . . . . . . . .  47

MOV1ST . . . . . . . . . . . . . . . . . . . .  48

PLTSTR . . . . . . . . . . . . . . . . . . . .  49

RECLGN . . . . . . . . . . . . . . . . . . . .  50

SCALE  . . . . . . . . . . . . . . . . . . . .  51

TRIML  . . . . . . . . . . . . . . . . . . . .  52

TRIMSP . . . . . . . . . . . . . . . . . . . .  53

VUPORT . . . . . . . . . . . . . . . . . . . .  54

WINDOW . . . . . . . . . . . . . . . . . . . .  55

WRILGN . . . . . . . . . . . . . . . . . . . .  56

XAXIS  . . . . . . . . . . . . . . . . . . . .  57

XWORLD . . . . . . . . . . . . . . . . . . . .  58

YAXIS  . . . . . . . . . . . . . . . . . . . .  59

YWORLD . . . . . . . . . . . . . . . . . . . .  60

APPENDIX B - HIPLOT AND VISUAL ROUTINES

CIRCLE . . . . . . . . . . . . . . . . . . . .  62

COLTYP . . . . . . . . . . . . . . . . . . . .  63

CONLGN . . . . . . . . . . . . . . . . . . . .  64

CURSON . . . . . . . . . . . . . . . . . . . .  65

DEFWIN . . . . . . . . . . . . . . . . . . . .  66

DMPPLT . . . . . . . . . . . . . . . . . . . .  67

ENDPLT . . . . . . . . . . . . . . . . . . . .  68

ERASE  . . . . . . . . . . . . . . . . . . . .  69

FNDCUR . . . . . . . . . . . . . . . . . . . .  70

GETSTR . . . . . . . . . . . . . . . . . . . .  71

INIPLT . . . . . . . . . . . . . . . . . . . .  72

MARKER . . . . . . . . . . . . . . . . . . . .  74

PENDWN . . . . . . . . . . . . . . . . . . . .  76

PENUP  . . . . . . . . . . . . . . . . . . . .  77

PLOT   . . . . . . . . . . . . . . . . . . . .  78

PLTOFF . . . . . . . . . . . . . . . . . . . .  79

PLTON  . . . . . . . . . . . . . . . . . . . .  80

PUTCUR . . . . . . . . . . . . . . . . . . . .  81

REDSTR . . . . . . . . . . . . . . . . . . . .  82

WRITCH . . . . . . . . . . . . . . . . . . . .  83

WRTSTR . . . . . . . . . . . . . . . . . . . .  84

APPENDIX C - GRINNELL ROUTINES

CIRCLE . . . . . . . . . . . . . . . . . . . .  87

COLTYP . . . . . . . . . . . . . . . . . . . .  88

DMPPLT . . . . . . . . . . . . . . . . . . . .  89

ENDPLT . . . . . . . . . . . . . . . . . . . .  90

ERASE  . . . . . . . . . . . . . . . . . . . .  91

INIPLT . . . . . . . . . . . . . . . . . . . .  92

MARKER . . . . . . . . . . . . . . . . . . . .  93

PLOT   . . . . . . . . . . . . . . . . . . . .  94

PLTOFF . . . . . . . . . . . . . . . . . . . .  95

PLTON  . . . . . . . . . . . . . . . . . . . .  96

PAGE 6

WRTSTR . . . . . . . . . . . . . . . . . . . .  97

APPENDIX D - QUICK REFERENCE GUIDE . . . . . . . 101

INDEX  . . . . . . . . . . . . . . . . . . . . . 103

LIST OF FIGURES

PLOTTING DIAGRAM . . . . . . . . . . . . . . .   1

SIMPLE 2D PLOT . . . . . . . . . . . . . . . .   2

MULTIPLE FRAME PLOT  . . . . . . . . . . . . .  11

PLOT WITH LEGEND . . . . . . . . . . . . . . .  13

3D PLOTS . . . . . . . . . . . . . . . . . . .  24

I. TERMINOLOGY

In order to understand many of the routines included  in  this

document,  one  must  understand the terminology that is used.

The  reader   will   encounter   such   phrases   as   "screen

coordinates", "world coordinates", "nominal inches", "plotting

frame", "plotting window", and "plotting page".  To understand

these, consider the following diagram:

The plotting window contains  the  data  to  be  plotted  (the

dashed  line in the above).  This data is "clipped" so that it

will only appear inside the  plotting  window.   The  plotting

frame is a region outside of which no plotting may take place.

The plotting window and the labels for the plotting window are

contained  within  the plotting frame.  The plotting window is

defined relative to the lower left hand corner of the plotting

frame.   The  plotting  page is the maximum allowable plotting

area.  It is dependent on the device used.  For the HIPLOT  it

is  10.25 by 7.25 units, for the GRINNELL it is 10.22 by 10.22

units, and for the VISUAL it is 10.24  by  7.8  units.   These

units  are expressed in "nominal inches".  On the HIPLOT these

are actual inches.  Nominal inches provide a global coordinate

system independent of the given device.  World coordinates are

the coordinates  associated  with  the  data  to  be  plotted.

Screen  coordinates are the coordinates used internally by the

plotting device.

PAGE 2

II. GETTING STARTED.

For the casual user, there are several higher  level  routines

which  should  be  of  interest.   These are divided into four

catagories:    initializing   and   terminating   plots,   two

dimensional   plots,  three  dimensional  plots,  and  writing

legends.   For  more  advanced  users,  additional  categories

entitled  Auxiliary  Routines,  Controlling  Program Input and

Output, Character Read and Write Routines, and Cursor  Control

Routines are included.

To get started, consider the following simple example program.

It  first  generates  the  data to be plotted into the x and y

arrays.  The next  five  statements  initialize  the  plotting

device and plotting routines, plot the data, and terminate the

plot.  These five routines are in general  the  only  routines

one  needs  to  use to create a two-dimensional plot.  A brief

description  of  each  routine  follows.   A   more   detailed

description follows in subsequent sections.

DIMENSION X(41), Y(41)

C

DO 10 I = 1, 41

X(I) = .05 * ( I - 1 )

Y(I) = SIN( 3.141592654 * X(I) )

10    CONTINUE

C

CALL INIPLT( 99, 10., 7. )

CALL SCALE( 0., 2., -1., 1. )

CALL AXIS( .5, .5, 'x', 1, 2, 1, 'sin', 3, 2, 1 )

CALL LINE( X, Y, 41, 0, 0, 0, 0 )

CALL ENDPLT

C

END

The above produces the following plot.

PAGE 3

Subroutine INIPLT initializes the plotting device and plotting

routines.   The  arguments  indicate  that  the  data is to be

written to FTN99.DAT, and that the plotting page is  10  by  7

nominal  inches.   It  defines by default the plotting window.

(This default plotting window may be  changed  by  a  call  to

WINDOW).   The  plotting  window defined by INIPLT is 6 by 4.2

nominal inches  starting  at  location  (2.,1.4).   Subroutine

SCALE sets up the correspondence between the world coordinates

and the screen coordinates.  For this program, the  horizontal

axis  runs  from  0  units to 2 units, while the vertical axis

runs from -1 units to +1 units.  Subroutine AXIS draws  a  box

around the plotting window, draws tick marks every .5 units on

the horizontal and vertical axes, and labels these tick marks.

It  centers  a lower case 'x' under the horizontal axis, and a

lower case 'sin' to the left of the vertical axis.  Subroutine

LINE  connects  the  points  to  be  plotted by line segments.

Subroutine ENDPLT terminates the plot.

PAGE 4

III. INITIALIZING AND TERMINATING PLOTS.

The following statements initialize and terminate plotting:

SUBROUTINE INIPLT

SUBROUTINE DMPPLT

SUBROUTINE ENDPLT

To start the plot, the user must first initialize the plotting

routines.   This  requires  that  the  following  statement be

executed PRIOR to calling any other plotting routine:

SUBROUTINE INIPLT( iunit, xsize, ysize )

This routine initializes the plot.  The  variables  xsize  and

ysize  denote the size of the plotting page in nominal inches.

For the HIPLOT , this is the size of the paper.  HIPLOT has  a

maximum  length  of  10.25 inches and a height of 7.25 inches.

Setting xsize larger than 10.25, or ysize larger than 7.25 may

cause  the  limit  switches  to  be activated during the plot.

This  will  cause  unpredictable  results.   No  plotting   is

permitted  outside this area.  The maximum size of the plot on

the GRINNELL is 10.22 by 10.22.  The maximum size of the  plot

on  the  VISUAL  is  10.24  by 7.8.  For compatability between

devices, bounds  within  the  common  intersection  should  be

chosen.

The variable iunit is the  logical  unit  number  selected  to

contain  the plot commands.  When plotting on the GRINNELL the

variable iunit is ignored.  When plotting on either the HIPLOT

or  the  VISUAL,  setting iunit equal to 7 or 8 sends the plot

immediately to the terminal.  When  iunit  equals  7,  graphic

commands for the VISUAL are sent.  When iunit equals 8, INIPLT

enables the printer port on the terminal and DM/PL IV plotting

commands  for  the  HIPLOT  are sent.  Use iunit equal to 8 if

your plotter is attached to your printer port.  Setting  iunit

equal  to 9 sends the plot to the queued plotting device.  For

iunit equal to 10 to 99, a disk file with name  FTNnn.DAT,  nn

equal  to  10  to 99 is generated.  The disk file will contain

DM/PL IV plotter commands for the HIPLOT.  It is hoped that at

a  later  date  either  an  interpreter  for DM/PL into VISUAL

graphics commands will be  written,  or  a  suitable  metafile

structure  will  be  implemented.   At  present  there  is  no

transparent way of generating a disk file of  VISUAL  graphics

commands (it can however be done).

PAGE 5

To dump the buffer so that the entire plot may  be  seen,  use

the following routine:

SUBROUTINE DMPPLT

Subroutine DMPPLT is useful for maintaining  synchronized  I/O

for  interactive  plotting,  without  the  hassle of recalling

INIPLT and ENDPLT.

To terminate a plot, use the following routine:

SUBROUTINE ENDPLT

This routine terminates the plotting.  It  must  be  the  last

plotting  routine  called  if  the  entire  plot file is to be

viewed.  No other plotting routine may  follow  without  first

calling  INIPLT.   ENDPLT  will  disable the printer port when

appropriate and close the disk file.

PAGE 6

IV. TWO DIMENSIONAL PLOTS.

For two dimensional plots the  following  routines  should  be

used.

SUBROUTINE FRAME

SUBROUTINE WINDOW

SUBROUTINE SCALE

SUBROUTINE AXIS

SUBROUTINE DASHLN

SUBROUTINE LINE

These are described below.

SUBROUTINE FRAME( xminf, xmaxf, yminf, ymaxf )

This routine allows the user to select the plotting frame.  No

plotting  may  take  place outside of this region.  Subroutine

FRAME maps the origin  into  the  point  (xminf,yminf).   This

routine  is  useful  when producing multiple plots on a single

page.  In this case one would like to make sure that parts  of

one plot do not overwrite parts of other plots.  By specifying

limits, one can control where plotting can  take  place.   The

variables   xminf  and  xmaxf  are  the  minimum  and  maximum

horizontal dimensions in  inches  of  the  allowable  plotting

area,  and  the  variables yminf and ymaxf are the minimum and

maximum vertical dimensions.  Subroutine  FRAME  need  not  be

called  unless desired.  However, the first call to FRAME must

follow  INIPLT  and  preceed  calls  to  WINDOW   and   SCALE.

Subsequent calls to subroutine FRAME will automatically update

the transformation from world to screen coordinates using  the

arguments  of the last call to WINDOW and SCALE.  The position

of the plotting window will be in the same place  relative  to

the point (xminf,yminf) as the previous plot.  If FRAME is not

called, the origin is taken to be the lower left  hand  corner

of  the  plotting  page, and the plotting frame is taken to be

the plotting page.

SUBROUTINE WINDOW( xwin0, xwin1, ywin0, ywin1 )

This routine sets up the dimensions of  the  plotting  window.

The variables xwin0, xwin1, ywin0, and ywin1 are the bounds in

inches of this window.  The values  xwin0,  xwin1,  ywin0  and

ywin1  are  given  relative  to  the  last call to FRAME.  The

default values for xwin0 and ywin0 are 20% of their respective

page  size, and the default values for xwin1 and ywin1 are 80%

of their respective page size.

SUBROUTINE SCALE( xmin, xmax, ymin, ymax )

This routine sets up  the  correspondence  between  the  world

coordinates and the screen coordinates.  World coordinates are

PAGE 7

the coordinates  associated  with  the  data  to  be  plotted.

Screen  coordinates  are  the  coordinates use by the plotting

device.  The variables xmin and xmax  represent  respectively,

the minimum and maximum x-values to be plotted.  The variables

ymin and ymax represent respectively, the minimum and  maximum

y-values to be plotted.  The point (xmin,ymin) will be plotted

at (xwin0,ywin0).  The point (xmax,ymax) will  be  plotted  at

(xwin1,ywin1).   Values  falling  outside  this  range will be

clipped.  The default values for (xmin,ymin)  and  (xmax,ymax)

are  (0,0)  and  (1,1),  respectively.  A call to SCALE is not

needed after a call to WINDOW.  This  feature  is  useful  for

multiple plots in which the scaling of the x and y axes do not

change.  To draw a second plot on the same  page  (or  screen)

the user need only issue a call to WINDOW to move the plotting

window.   This  will  adjust  the  coordinate   transformation

between  world  and  screen  coordinates  so that the previous

values supplied by SCALE are  now  plotted  at  the  positions

indicated  by  the  arguments  of  the  call to WINDOW.  Note,

however, that merely changing WINDOW will not  guarantee  that

one plot will not overwrite another plot.  To insure this, the

user should use FRAME.

SUBROUTINE AXIS( xtic, ytic, ixtit, ixlen, ixsiz, ixfmt,

iytit, iylen, iysiz, iyfmt )

This routine draws the axes for two  dimensional  plots.   The

variables xtic and ytic are the distances in world coordinates

between tick marks on the x and y axes.  If xtic  or  ytic  is

zero, no tic marks are drawn on that axis.  The variable ixtit

is the label for the x-axis.  The variable ixlen is the number

of characters in ixtit.  The variable ixsiz is the size (value

between 1 and 5) of the x label.  For the HIPLOT the following

sizes are available:

For ixsiz = 1 the characters are 0.105 inch high.

For ixsiz = 2 the characters are 0.140 inch high.

For ixsiz = 3 the characters are 0.210 inch high.

For ixsiz = 4 the characters are 0.280 inch high.

For ixsiz = 5 the characters are 0.420 inch high.

For the VISUAL the following sizes are available:

For ixsiz = 1 the characters are 0.093 inch high.

For ixsiz = 2 the characters are 0.147 inch high.

For ixsiz = 3 the characters are 0.187 inch high.

For ixsiz = 4 the characters are 0.280 inch high.

For ixsiz = 5 the characters are 0.373 inch high.

PAGE 8

For the GRINNELL the following sizes are available:

For ixsiz = 1 the characters are 0.16 inch high.

For ixsiz = 2 the characters are 0.16 inch high.

For ixsiz = 3 the characters are 0.32 inch high.

For ixsiz = 4 the characters are 0.32 inch high.

For ixsiz = 5 the characters are 0.32 inch high.

If ixsiz is outside of this range, no labels are written.  The

variable ixfmt defines the format of the values on the x-axis.

For ixfmt = 0 to 3, the format is taken to be  F9.ixfmt.   For

ixfmt  =  4,  the format is G9.2.  The variables iytit, iylen,

iysiz and iyfmt are defined in a  corresponding  way  for  the

y-axis.

SUBROUTINE DASHLN( x, y, n, icol, isym, isize, inum, lintyp )

This subroutine will draw dashed lines of  a  user  controlled

color  through  the  n  world  coordinate  points  (x,y).  The

variables x and y are n-dimensional arrays.  The variable isym

is  the symbol to be plotted at the points.  The value of isym

must be between 0 and 5.

For isym = 0 a plus sign is plotted.

For isym = 1 a cross is plotted.

For isym = 2 a square is plotted.

For isym = 3 a circle is plotted.

For isym = 4 a triangle is plotted

For isym = 5 an hourglass is plotted.

The variable isize is the size of the symbol.   The  value  of

isize  must  be  between  1  and 5.  The size of the symbol is

shown below.

For isize = 1 the symbols are 0.06 inch high.

For isize = 2 the symbols are 0.08 inch high.

For isize = 3 the symbols are 0.12 inch high.

For isize = 4 the symbols are 0.16 inch high.

For isize = 5 the symbols are 0.24 inch high.

A symbol is  plotted  every  abs(inum)  points.   If  inum  is

negative,  only  symbols are plotted.  If any of the following

are out of range, only a line is drawn:  isym, isize, or inum.

The  variable  icol determines the color of the line.  For the

GRINNELL the folowing colors are available:

For icol = 0, a white line is drawn.

For icol = 1, a red line is drawn.

For icol = 2, a orange line is drawn.

For icol = 3, a yellow line is drawn.

For icol = 4, a green line is drawn.

For icol = 5, a blue line is drawn.

For icol = 6, a purple line is drawn.

PAGE 9

For the HIPLOT the  following  association  between  icol  and

color is suggested:

For icol = 0, a black line is drawn.

For icol = 1, a red line is drawn.

For icol = 2, a orange line is drawn.

For icol = 3, a lime-green line is drawn.

For icol = 4, a green line is drawn.

For icol = 5, a blue line is drawn.

For icol = 6, a lavender line is drawn.

For icol = 7, a brown line is drawn.

The parameter lintyp controls the type of dashed line drawn.

For lintyp = 0, a solid line is drawn.

For lintyp = 1, a dotted line is drawn.

For lintyp = 2 to 6, a dashed line of increasing

dash length is drawn.

For lintyp = 7, a dash-dot line is drawn.

For lintyp = 8, a dash-dot-dot line is drawn.

SUBROUTINE LINE( x, y, n, icol, isym, isize, inum )

This  routine  draws  a  solid  line  through  the   n   world

coordinates,  (x,y).   The  arguments  of LINE are the same as

described above for  DASHLN,  and  is  equivalent  to  calling

DASHLN with lintyp = 0.

The following details the relation between INIPLT,  FRAME  and

WINDOW.

Subroutine INIPLT  sets  up  the  maximum  size  of  the  plot

(plotting  page).   This  is  either  the physical size of the

paper for the HIPLOT (i.e., 10.25 x 7.25 inches), the size  of

the  GRINNELL  screen  (i.e., 10.22 x 10.22 nominal inches) or

the size of the VISUAL  screen  (i.e.,  10.24  x  7.8  nominal

inches).   Within  this  region  you  may  futher  divide  the

plotting page by framing certain portions of it.  Within  each

plotting  frame  you can specify a plotting window.  The lines

drawn as part of the plot of x and y coordinate pairs will  be

contained within the plotting window.  The labels for the axes

will be contained within the  plotting  frame.   The  plotting

frame  will  be  contained  within  the plotting page (see the

section on TERMINOLOGY).  As an example, the following program

will  divide  the  plotting page into four separate regions in

which a copy of a plot of sin(x) and cos(x) from 0 to 6.28  is

drawn.  The plot produced by this program is shown below.

PAGE 10

DIMENSION X(41), S(41), C(41)

C

C	CALCULATE X AND Y VALUES

C

DO 10 I = 1, 41

X(I) = .157 * ( I - 1 )

C(I) = COS( X(I) )

S(I) = SIN( X(I) )

10	CONTINUE

C

C	SET PHYSICAL PAGE SIZE TO BE 10 X 7 INCHES

C

CALL INIPLT( 99, 10., 7. )

C

C	SELECT LOWER LEFT HAND QUARTER AS PLOTTING FRAME

C

CALL FRAME ( 0., 5., 0., 3.5 )

CALL WINDOW( 1.5, 4.5, 1., 3. )

CALL SCALE( 0., 6.28, -1., 1. )

CALL AXIS ( 1., .5, 'x', 1, 2, 0, 'y(x)', 4, 2, 1 )

CALL DASHLN ( X, C, 41, 0, 0, 0, 0, 1 )

CALL DASHLN ( X, S, 41, 1, 0, 0, 0, 2 )

C

C	DRAW SECOND PLOT IN UPPER LEFT

C	NOTE THAT FRAME WILL AUTOMATICALLY ADJUST WINDOW AND SCALE

C	FOR YOU

C

CALL FRAME ( 0., 5., 3.5, 7. )

CALL AXIS ( 1., .5, 'x', 1, 2, 0, 'y(x)', 4, 2, 1 )

CALL DASHLN ( X, C, 41, 2, 0, 0, 0, 3 )

CALL DASHLN ( X, S, 41, 3, 0, 0, 0, 4 )

C

C	DRAW THIRD PLOT IN UPPER RIGHT

C

CALL FRAME ( 5., 10., 3.5, 7. )

CALL AXIS ( 1., .5, 'x', 1, 2, 0, 'y(x)', 4, 2, 1 )

CALL DASHLN ( X, C, 41, 4, 0, 0, 0, 5 )

CALL DASHLN ( X, S, 41, 5, 0, 0, 0, 6 )

C

C	DRAW FOURTH PLOT IN LOWER RIGHT

C

CALL FRAME ( 5., 10., 0., 3.5 )

CALL AXIS ( 1., .5, 'x', 1, 2, 0, 'y(x)', 4, 2, 1 )

CALL DASHLN ( X, C, 41, 6, 0, 0, 0, 7 )

CALL DASHLN ( X, S, 41, 6, 0, 0, 0, 8 )

C

C	TERMINATE PLOTTING

C

CALL ENDPLT

C

END

PAGE 11

xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

PAGE 12

V. LEGEND WRITING ROUTINES

It is often useful to label a  graph  so  that  a  person  can

identify  which  curve  on the graph corresponds to which data

set.  In labeling the graph, one would like to associate  with

each line type, symbol or color a short phrase which describes

what is meant by that particular curve.  Such a label is known

as  a  legend.   Legend  writing  is  in general good plotting

practice.  There are five routines  which  help  the  user  to

construct legends.  These are:

SUBROUTINE INILGN

SUBROUTINE ENDLGN

SUBROUTINE WRILGN

SUBROUTINE CONLGN

SUBROUTINE RECLGN

SUBROUTINE INILGN( xminl, xmaxl, yminl, ymaxl )

This routine initializes  the  legend  writing  routines.   It

should  be the first routine called before attempting to write

a legend.   It  saves  all  current  plotting  parameters  and

defines  a  legend  frame  whose lower left corner is given by

(xminl,yminl)  and  whose  upper  right  corner  is  given  by

(xmaxl,ymaxl).   The parameters xminl, xmaxl, yminl, and ymaxl

are given in nominal inches.

SUBROUTINE ENDLGN

This routine  terminates  the  legend  writing  routines.   It

restores  all  plotting  parameters  to their previous values.

This should be the last legend routine called.

SUBROUTINE WRILGN( title, n, icol, isym, inum, lintyp )

This routine writes the  legend.   The  argument  title  is  a

string of characters of length n which is a description of the

curve defined by the parameters icol, isym, inum  and  lintyp.

These  latter parameters have the same meaning as they do with

DASHLN.  The parameter icol defines the  color  of  the  line,

isym  is  the symbol to be plotted, lintyp is the type of line

to be plotted and inum determines  whether  a  line  is  drawn

connecting  the symbols.  When inum is positive, both the line

and the symbols are drawn.  The  symbols  are  placed  at  the

beginning  and  the end of the line.  When inum is negative, a

single symbol is drawn at the right endpoint.  In either case,

the  description  is placed directly to the right of the right

endpoint.  Multiple  calls  to  WRILGN  will  produce  legends

directly beneath each other.

The following program demonstrates the above routines.

PAGE 13

DIMENSION X(21), Y1(21), Y2(21), Y3(21)

C

C     GENERATE DATA

C

PI = 4 * ATAN(1.)

DO 10 I = 1, 21

X(I) = .05 * ( I - 1 )

Y1(I) = SIN( PI * X(I) )

Y2(I) = SIN( 2 * PI * X(I) )

Y3(I) = SIN( 3 * PI * X(I) )

10    CONTINUE

C

C     DRAW GRAPH

C

CALL INIPLT( 99, 10., 7. )

CALL WINDOW( 2., 8., 1., 5. )

CALL SCALE( 0., 1., -1., 1. )

CALL AXIS( .5, .5, 'X', 1, 2, 1, 'Y', 1, 2, 1 )

CALL DASHLN( X, Y1, 21, 0, 0, 0, 0, 8 )

CALL DASHLN( X, Y2, 21, 0, 3, 3, 5, 0 )

CALL DASHLN( X, Y3, 21, 0, 5, 3, -1, 0 )

C

C     CONSTRUCT LEGEND

C

CALL INILGN( 2., 6., 5., 7. )

CALL WRILGN( 'sin(pi x)', 9, 0, 0, 0, 8 )

CALL WRILGN( 'sin(2 pi x)', 11, 0, 3, 5, 0 )

CALL WRILGN( 'sin(3 pi x)', 11, 0, 5, -1, 0 )

CALL ENDLGN

C

C     TERMINATE PLOTTING

C

CALL ENDPLT

C

END

PAGE 14

It is often difficult to know beforehand where one would  like

to  place  the  legend.   To help the user in this process two

legend writing routines have  been  written  to  interactively

control legend writing.  These are

SUBROUTINE CONLGN( nl, title, n, icol, isym, inum, lintyp,

xstrt, xlen, ystrt )

SUBROUTINE RECLGN( nl, title, n, icol, isym, inum, lintyp,

xstrt, xlen, ystrt )

The routine CONLGN constructs the legend  interactively  on  a

graphics  terminal,  and  the  routine RECLGN reconstructs the

legend for any plotting device.  These routines were developed

to  be used with the HIPLOT and VISUAL routines.  The argument

nl is the number of curves to be labeled.  The argument  title

is  a  byte  array  of  user  defined length.  It should be of

sufficient length to  contain  all  the  labels  for  all  the

legends.   The  routine  CONLGN will read into title the label

typed on the screen.  If more than one  label  is  typed,  the

beginning  of  the  next  label  will be placed in the storage

location immediately following the last  label.   The  routine

RECLGN  will  write  title onto the plotting device defined by

iunit.  (See  the  section  on  Initializing  and  Terminating

Plots).  The argument n is an array of dimension nl containing

the length of the label.  The arguments icol, isym,  inum  and

lintyp  are  arrays  of dimension nl defining the curves to be

labeled.  The arguments xstrt, xlen, ystrt are real arrays  of

dimension  nl,  defining  the starting location of each sample

curve and its length.  These arguments are set by  CONLGN  and

used  in  RECLGN.   They  should  not be modified by the user.

When CONLGN is called it first turns on the cursor.  The  user

may  then position the cursor to the desired starting location

of the first legend.  The user then transmits the location  of

the  cursor  to  the  program  by striking the appropriate key

(such as the ENTER key on the VISUAL).   The  user  will  then

move  the  cursor  to  a second location horizontally and then

transmit the cursor location.  If the second  position  is  to

the  right  of  the  first  position, a line of that length is

drawn.  The type of line drawn is defined by the parameters of

the  argument  list.  If the second position is to the left of

the first position, a line is drawn starting  from  the  first

position  using  the last defined length, thereby allowing the

user to start a second column of legends of the same length as

the  first  column.  In either case, the user may then type in

the text used to describe that particular curve.  To construct

subsequent   legends  directly  beneath  the  current  legend,

transmit the cursor location without moving the cursor.   When

you  have  labeled your last curve, control will return to the

calling routine.  The  argument  list  will  contain  all  the

information  necessary  for  RECLGN  to reconstruct the legend

exactly as displayed on the graphics screen.

PAGE 15

VI. AUXILIARY ROUTINES.

The following routines allow the user to construct his/her own

specialized plotting routines by gaining access to lower level

routines with a minimum of overhead.

SUBROUTINE MOVETO

FUNCTION ISCRX

FUNCTION ISCRY

FUNCTION XWORLD

FUNCTION YWORLD

SUBROUTINE CURPOS

SUBROUTINE DRWREC

SUBROUTINE CIRCLE

SUBROUTINE ERASE

These are described below.

SUBROUTINE MOVETO( xwrld, ywrld, iud, lintyp )

Subroutine MOVETO will move from the current  cursor  position

to the world coordinate (xwrld,ywrld) only if it is inside the

plotting window.  Otherwise it will move to  the  intersection

of  the  line  segment  with the plotting window.  MOVETO will

draw a line only when iud is equal to one.  The type  of  line

drawn is determined by lintyp as described in DASHLN.

FUNCTION ISCRX( xwrld )

Function ISCRX converts the x  world  coordinate  into  the  x

screen coordinate.

FUNCTION ISCRY( ywrld )

Function ISCRY converts the y  world  coordinate  into  the  y

screen coordinate.

FUNCTION XWORLD( ix )

Function XWORLD converts the x  screen  coordinate  to  the  x

world coordinate.  It is the inverse of ISCRX.

FUNCTION YWORLD( iy )

Function YWORLD converts the y  screen  coordinate  to  the  y

world coordinate.  It is the inverse of ISCRY.

SUBROUTINE CURPOS( icurx, icury )

Subroutine CURPOS returns the current  screen  coordinates  in

icurx and icury.

PAGE 16

SUBROUTINE DRWREC( xmin, xmax, ymin, ymax, icol )

Subroutine DRWREC draws a rectangle of color icol, bounded  by

the world coordinates xmin, xmax, ymin and ymax.

SUBROUTINE CIRCLE( ix, iy, ir )

Subroutine CIRCLE draws a circle of radius ir with  center  at

(ix,iy).  The arguments are all in screen coordinates.

SUBROUTINE ERASE

Subroutine ERASE has been added to  the  software  package  in

order to erase the GRINNELL or VISUAL screen without reissuing

a call to INIPLT.  This has the advantage of not resetting all

of the internal saved parameters generated by calls to WINDOW,

SCALE, etc..  This routine has no  effect  for  plots  on  the

HIPLOT.

PAGE 17

VII. CONTROLLING PROGRAM INPUT AND OUTPUT

The user should  employ  the  following  routines  to  control

program  input  and  output so that it does not interfere with

plotting.

SUBROUTINE PLTOFF

Subroutine PLTOFF will disable the printer port when  plotting

on  the HIPLOT, or will enable the alphanumerics screen on the

VISUAL.  When used with PLTON, it allows  the  user  to  enter

data  to  the  program  while it is plotting data.  Innocently

typing in data while the printer port  is  enabled  can  cause

erratic  behavior  on  the HIPLOT.  A similar transgression on

the VISUAL results in overwriting the plot.

SUBROUTINE PLTON

Subroutine PLTON will enable the printer port when plotting on

the  HIPLOT,  or  will  enable the alphagraphics screen on the

VISUAL.

PAGE 18

VIII. CHARACTER READ AND WRITE ROUTINES

The  following  routines  control  reading  and   writing   of

character strings for plots.

SUBROUTINE PLTSTR

SUBROUTINE WRTSTR

SUBROUTINE REDSTR

SUBROUTINE GETSTR

These are described below.

SUBROUTINE PLTSTR( ix, iy, str, len, irot, isize )

Subroutine PLTSTR will write an alphanumeric string  'str'  of

length  'len' at screen location given by (ix,iy).  The height

of the string  is  determined  by  isize  (see  AXIS  for  the

relevant   sizes   of  the  characters).   The  argument  irot

determines the rotation of the  string.   For  irot  =  1  the

string  is  written  right side up, for irot = 2 the string is

rotated clockwise through 90 degrees, for irot = 3 the  string

is rotated 180 degrees, and for irot = 4 the string is rotated

through 270 degress.  Subroutine PLTSTR calls WRTSTR.

SUBROUTINE WRTSTR( str, len, irot, isize )

This routine writes a string at  the  current  location.   The

arguments   of  WRTSTR  are  the  same  as  the  corresponding

arguments for PLTSTR.

SUBROUTINE REDSTR( ix, iy, str, len, irot, isize )

This routine will read a  string  'str'  of  length  'len'  at

screen location given by (ix,iy).  The height of the string is

determined by isize, and the rotation is determined  by  irot.

REDSTR  will  read in both the current cursor location as well

as the character string.  The arguments irot and isize must be

supplied,  the rest of the arguments are determined by REDSTR.

After a call to REDSTR, the cursor will be positioned  at  the

begining  of the current string one line below.  The effect is

to mimic a  carriage  return  and  line  feed.   REDSTR  calls

GETSTR.

SUBROUTINE GETSTR( ix, iy, str, len, irot, isize )

This routine will read a string at  screen  location  (ix,iy).

The  arguments  ix,  iy,  irot  and  isize are supplied to the

routine, and str and len are returned.  The difference between

GETSTR  and  REDSTR  is that REDSTR allows you to position the

cursor to the desired starting of  the  string,  while  GETSTR

does not.

PAGE 19

IX. CURSOR CONTROL ROUTINES

SUBROUTINE DEFWIN

SUBROUTINE FNDCUR

SUBROUTINE PUTCUR

SUBROUTINE CURSON

The ability to control the position  of  the  cursor,  and  to

transmit  this  information  to  a running program is of great

help in interactively constructing a graph.  By cursor  it  is

meant  the full screen crosshair cursor on the VISUAL, not the

underscore cursor of the alphagraphics screen.  The  following

routines  are  the  lowest level routines which interface with

the cursor.  When the cursor is turned on, its position may be

controlled  by  the  small  arrows  on the numeric keypad.  To

transmit the current screen coordinate, the user should  press

the  ENTER  key  located next to the numeric keypad.  No other

key will transmit the cursor location.  Familiarity  with  the

cursor  positioning  keys  is  essential  to  properly use the

following routines.

SUBROUTINE DEFWIN( xmin, xmax, ymin, ymax )

Subroutine DEFWIN defines a window interactively on a graphics

screen.   The  arguments  xmin,  xmax,  ymin, and ymax are the

bounds in inches of this window.  DEFWIN uses  the  cursor  to

position  these  bounds.   DEFWIN can be used to interactively

control the plotting page, plotting frame, plotting window  or

legend window.

SUBROUTINE FNDCUR( ix, iy )

This routine reads in the current cursor location.   It  turns

on  the  cursor,  allows the user to position the cursor, then

accepts the current location when transmitted.  The  arguments

ix and iy are the screen coordinates of the cursor location.

SUBROUTINE PUTCUR( ix, iy )

This routine puts the cursor at the screen location  given  by

(ix,iy).  It does not turn the cursor on.

SUBROUTINE CURSON

This routine turns the cursor on.

PAGE 20

X. SUPPORT ROUTINES

The following routines are used by AXIS and MOVETO to  perform

specialized  fuctions.   They are supplied to the user so that

he/she may make modifications to perform nonstandard plotting.

For  instance,  a minor modification in LABELY can convert the

y-axis into log(y)-axis, so that the user can have a log plot.

A  similar  modification to LABELX will create a log-log plot.

Once you have made the change, linking with this  new  routine

will replace the default routine.  These routines are:

SUBROUTINE XAXIS

SUBROUTINE YAXIS

SUBROUTINE LABELX

SUBROUTINE LABELY

SUBROUTINE DRXTIC

SUBROUTINE DRYTIC

FUNCTION IFRMT

SUBROUTINE TRIML

SUBROUTINE TRIMSP

SUBROUTINE ALLOUT

SUBROUTINE FNDINT

SUBROUTINE MOV1ST

SUBROUTINE DRDASH

SUBROUTINE INSECT

The following nine routines are used by  AXIS.   Many  of  the

arguments have the same meaning as in the call to AXIS.

SUBROUTINE XAXIS( xtic, ixtit, ixlen, ixsiz, ixfmt )

SUBROUTINE YAXIS( ytic, iytit, iylen, iysiz, iyfmt )

These routines construct the x and y axes.

SUBROUTINE LABELX( xval, form, iform, is, iy, ichw, ixsiz )

SUBROUTINE LABELY( yval, form, iform, is, iy, ichw, iysiz )

These routines label a tic mark at the location (is,iy).   The

variables  xval  and yval are the values to be written at that

tic mark.  The argument ichw is the width of a character.

SUBROUTINE DRXTIC( ix, iy, lentic )

SUBROUTINE DRYTIC( ix, iy, lentic )

These routines draw a tic mark  of  length  lentic  at  screen

location (ix,iy).

FUNCTION IFRMT( form, ifmt )

This routine determines the format corresponding to the format

number ifmt.

PAGE 21

SUBROUTINE TRIML( label, n, len )

This routine trims leading blanks from  the  string  LABEL  of

length  N.  It returns in LEN the length of the trimmed string

and in  LABEL  the  string  left  justified  with  all  blanks

removed.

SUBROUTINE TRIMSP( title, len, ist, iend )

This routine trims off both leading and  trailing  blanks  for

the  centering  of axis labels.  The argument ist is the first

nonblank character of title, and iend  is  the  last  nonblank

character.

The following four routines are used in MOVETO.

SUBROUTINE ALLOUT( ix, iy, iud )

This routine handles the case  when  both  the  previous  data

point  and  the  current  data  point are outside the plotting

window.

SUBROUTINE FNDINT( ix0, iy0, ix, iy )

This routine finds the intersection with the  plotting  window

when  one  data  point  is outside the plotting window and one

data point is inside the plotting window.   The  arguments  ix

and iy may be reset by this routine.

SUBROUTINE MOV1ST( xwrld, ywrld, iud, lintyp )

Subroutine MOV1ST moves to the world coordinate (xwrld,ywrld).

The  argument  lintyp  refers  to  the  type  of  line  to  be

initialized (see DASHLN for details  on  the  types  of  lines

available).

SUBROUTINE DRDASH( ix, iy, iud )

This routine will draw a dashed line of the last defined  line

type  from  the  current  position  to  the  screen coordinate

(ix,iy).

SUBROUTINE VUPORT( xmin, xmax, ymin, ymax )

This routine is equivalent with SCALE.   It  is  included  for

compatibility with version 1.1.

SUBROUTINE INSECT( i1, j1, iplot, iplot0, x, y, nx, ny, l )

This routine computes the intersection point of  a  grid  line

which is partially hidden.  It is used by PLOT3D.

PAGE 22

XI. MISCELLANEOUS ROUTINES.

The following lower level  routines  have  been  included  for

completness.    These  routines  should  not  be  used  unless

absolutely  needed,  as  their  function  may  change  without

notice.

SUBROUTINE PLOT

SUBROUTINE MARKER

SUBROUTINE COLTYP

SUBROUTINE PENDWN

SUBROUTINE PENUP

SUBROUTINE WRITCH

SUBROUTINE PLOT( ix, iy, iud )

This routine moves to  the  screen  coordinates  (ix,iy).   If

iud=1, the pen is put down before moving, otherwise the pen is

put up before moving.  It checks to see if the point is within

the plotting page.

SUBROUTINE MARKER( isym, isize )

This routine places a marker at  the  current  position.   For

isym  = 0 a plus sign is drawn, for isym = 1 a cross is drawn,

for isym = 2 a square is drawn, for  isym  =  3  a  circle  is

drawn,  for  isym = 4 a triangle is drawn, and for isym = 5 an

hourglass is drawn.

SUBROUTINE COLTYP( icol )

This routine sets the line color.  It is inoperative  for  the

VISUAL and the single pen HIPLOT.

SUBROUTINE PENDWN

This routine puts the pen down so it will draw a visible line.

It is applicable only for HIPLOT and VISUAL.

SUBROUTINE PENUP

This routine puts the pen up so  no  line  is  drawn.   It  is

applicable only for HIPLOT and VISUAL.

SUBROUTINE WRITCH( ch, n )

This routine is used by the VISUAL and the  HIPLOT  to  output

characters  to  either  the  terminal  or  a  disk  file.  The

argument ch is a byte array of length n.

PAGE 23

XII. THREE DIMENSIONAL PLOTS.

Three dimensional plots with hidden line removal  are  handled

by the following routine:

SUBROUTINE PLOT3D( f,x,y,nx,ny,xw,yw,xv,yv,zv,line,scale )

The variable f is an nx by ny array containing the  values  of

the  surface to be plotted.  The variables x and y are also nx

by ny arrays.  They are used internally by PLOT3D for  storing

the x and y plot coordinates.  The variables nx and ny are the

dimensions of the f, x, and y arrays.  The variables xw and yw

are  the  dimensions of the plot in inches.  The variables xv,

yv and  zv  are  the  location  in  inches  of  the  point  of

observation  relative to the grid.  The grid is xw by yw.  The

variable line chooses which direction(s)  the  projected  grid

lines  are  drawn.  If line = 1, only lines in the x direction

are drawn.  If line = 2, only lines in  the  y  direction  are

drawn.   For  line  =  0,  lines are drawn in both the x and y

directions.  The variable scale is a scaling  factor  for  the

array  f.   For scale = 0, a suitable scaling factor is chosen

by PLOT3D.

A sample program is given below.

DIMENSION F(21, 11), X(21, 11), Y(21, 11)

C

PI = 3.141592654

TYPE 2

ACCEPT *, IUNIT

C

CALL INIPLT(IUNIT, 10., 7.)

C

DO 20 J = 1, 11

XX = .1 * (J - 1)

DO 10 I = 1, 21

YY = .1 * (I - 1)

F(I, J) = COS(PI * XX) * SIN(PI * YY)

10	     CONTINUE

20	CONTINUE

C

TYPE 1

ACCEPT *, LINE, XV, YV, ZV

C

CALL PLOT3D(F,X,Y,21,11,10.,5.,XV,YV,ZV,LINE,0.)

CALL ENDPLT

C

STOP

1	FORMAT('$INPUT LINE, XV, YV, ZV? ')

2	FORMAT('$INPUT IUNIT? ')

END

PAGE 24

With line = 0, XV = 20, YV = 30 and ZV = 40, the above program

produces the following plot.

With line = 0, XV = -10, YV = 30 and  ZV  =  40,  the  program

produces this plot.

PAGE 25

XIII. RUNNING A PROGRAM.

To compile, link and run a program on  the  11/23  which  will

plot  on the HIPLOT, the GRINNELL or the VISUAL, the following

sequence is used.  For the HIPLOT or VISUAL use,

FORT filespec

LINK filespec,SY:HGRAPH

RUN filespec

or

HGR filespec

For the GRINNELL use,

FORT filespec

LINK filespec,SY:GRPLOT,SY:GRLIB

RUN filespec

or

GRX filespec

The file SY:HGRAPH is an object library containing the  device

independent  plotting routines and the routines for the HIPLOT

and the VISUAL.   The  file  SY:GRPLOT  is  an  object  module

containing  the  device  dependent  plotting  routines for the

GRINNELL.  SY:GRLIB is an object library of routines  for  the

GRINNELL.  FLECS listings of the routines contained in each of

the  object  modules  are  located  on  the  system  disk   as

SY:name.FLS.

To compile, link and run a program on the VAX which will  plot

on the HIPLOT or VISUAL use the following sequence:

FORT filespec

LINK filespec,VAXLB::DRA1:[MCMAKEN.GRAPH]HGRAPH/L

RUN filespec

A  command  file  which  performs  the   above   sequence   is

VAXLB::DRA1:[MCMAKEN.GRAPH]HGR.COM.   When  using  the  HIPLOT

connected to a printer port,  REMEMBER  TO  SET  THE  TERMINAL

WIDTH  TO  132 (SET TERM/WID=132) before listing the plot file

on the terminal.

PAGE 26

XIV. DISPLAYING A DISK PLOT FILE.

To display a disk plot file containing the device instructions

for either the HIPLOT or the VISUAL, one should use one of the

following.  To display a plot on the VISUAL type,

TYPE filspec

To display a plot on the HIPLOT connected to  a  printer  port

type,

TYPLOT filspec on the 11/23,

or

@VAXLB::DRA1:[MCMAKEN.GRAPH]TYPLOT filspec on the VAX.

To display a plot on the queued HIPLOT device type,

HCOPY filspec

HIGO

As an example, suppose you generated a plot using  the  HIPLOT

software  with  iunit=99.   This  created  a  disk  file named

FTN99.DAT.  To display this file on the queued HIPLOT  plotter

type the following:

HCOPY FTN99.DAT

HIGO

Always make certain that you have both pen and paper installed

on the HIPLOT.

PAGE 27

APPENDIX A - DEVICE INDEPENDENT ROUTINES

SUBROUTINE ALLOUT

SUBROUTINE AXIS

SUBROUTINE CURPOS

SUBROUTINE DASHLN

SUBROUTINE DRDASH

SUBROUTINE DRWREC

SUBROUTINE DRXTIC

SUBROUTINE DRYTIC

SUBROUTINE ENDLGN

SUBROUTINE FNDINT

SUBROUTINE FRAME

FUNCTION IFRMT

SUBROUTINE INILGN

FUNCTION ISCRX

FUNCTION ISCRY

SUBROUTINE LABELX

SUBROUTINE LABELY

SUBROUTINE LINE

SUBROUTINE MOVETO

SUBROUTINE MOV1ST

SUBROUTINE PLTSTR

SUBROUTINE RECLGN

SUBROUTINE SCALE

SUBROUTINE TRIML

SUBROUTINE TRIMSP

SUBROUTINE VUPORT

SUBROUTINE WINDOW

SUBROUTINE WRILGN

SUBROUTINE XAXIS

FUNCTION XWORLD

SUBROUTINE YAXIS

FUNCTION YWORLD

PAGE 28

SUBROUTINE ALLOUT( IX, IY, IUD )

INTEGER ISCX(2), ISCY(2), IX1(2), IY1(2)

LOGICAL INSIDE, STARTNEW

COMMON /HWINDO/ SCRX,SCRY,IXORIG,IYORIG,IASCR,IBSCR,ICSCR,IDSCR

COMMON /HCURPO/ IXCUR, IYCUR

COMMON /CHKBON/ INSIDE, STARTNEW, LINNTP, LIN1, DASH

EQUIVALENCE ( ISCX(1), IASCR ), ( ISCX(2), IBSCR )

EQUIVALENCE ( ISCY(1), ICSCR ), ( ISCY(2), IDSCR )

C

INSECT = 0

IF ( IX .NE. IXCUR )

.  IXMAX = MAX0( IX, IXCUR )

.  IXMIN = MIN0( IX, IXCUR )

.  DO ( IS = 1, 2 )

.  .  IF( ISCX(IS) .GE. IXMIN .AND. ISCX(IS) .LE. IXMAX )

.  .  .  XSL = ( IY - IYCUR ) / FLOAT( IX - IXCUR )

.  .  .  IY0 = XSL * ( ISCX(IS) - IX ) + IY

.  .  .  IF ( IY0 .GE. ICSCR .AND. IY0 .LE. IDSCR )

.  .  .  .  INSECT = INSECT + 1

.  .  .  .  IX1(INSECT) = ISCX(IS)

.  .  .  .  IY1(INSECT) = IY0

.  .  .  ...FIN

.  .  ...FIN

.  ...FIN

...FIN

IF ( IY .NE. IYCUR .AND. INSECT .LT. 2 )

.  IYMAX = MAX0( IY, IYCUR )

.  IYMIN = MIN0( IY, IYCUR )

.  DO ( IS = 1, 2 )

.  .  IF( ISCY(IS) .GE. IYMIN .AND. ISCY(IS) .LE. IYMAX )

.  .  .  YSL = ( IX - IXCUR ) / FLOAT( IY - IYCUR )

.  .  .  IX0 = YSL * ( ISCY(IS) - IY ) + IX

.  .  .  IF ( IX0 .GE. IASCR .AND. IX0 .LE. IBSCR )

.  .  .  .  INSECT = INSECT + 1

.  .  .  .  IX1(INSECT) = IX0

.  .  .  .  IY1(INSECT) = ISCY(IS)

.  .  .  .  IF( INSECT.EQ.2.AND.IX1(1).EQ.IX1(2) ) INSECT=1

.  .  .  ...FIN

.  .  ...FIN

.  ...FIN

...FIN

IF ( INSECT .EQ. 2 )

.  IXCUR = IX1(1)

.  IYCUR = IY1(1)

.  CALL DRDASH ( IX1(1), IY1(1), 0 )

.  CALL DRDASH ( IX1(2), IY1(2), IUD )

...FIN

INSIDE = .FALSE.

C

RETURN

END

PAGE 29

SUBROUTINE AXIS( XTIC, YTIC, IXTIT, IXLEN, IXSIZ, IXFMT,

1                             IYTIT, IYLEN, IYSIZ, IYFMT )

BYTE IXTIT(IXLEN), IYTIT(IYLEN)

COMMON /WORLD/ XMINW, XMAXW, YMINW, YMAXW

C

CALL DRWREC( XMINW, XMAXW, YMINW, YMAXW, 0 )

CALL XAXIS( XTIC, IXTIT, IXLEN, IXSIZ, IXFMT )

CALL YAXIS( YTIC, IYTIT, IYLEN, IYSIZ, IYFMT )

C

RETURN

END

PAGE 30

SUBROUTINE CURPOS( ICURX, ICURY )

COMMON /HCURPO/ IXCUR, IYCUR

C

ICURX = IXCUR

ICURY = IYCUR

C

RETURN

END

PAGE 31

SUBROUTINE DASHLN( X, Y, N, ICOL, ISYM, ISIZE, INUM, LINTYP )

LOGICAL DRAWMARK, DRAWLINE, INSIDE, STARTNEW

DIMENSION X(N), Y(N)

COMMON /CHKBON/ INSIDE, STARTNEW, LINNTP, LIN1, DASH

C

DRAWMARK = ISIZE .GE. 1 .AND. ISIZE .LE. 5

DRAWMARK = DRAWMARK .AND. ISYM .GE. 0 .AND. ISYM .LE. 5

DRAWMARK = DRAWMARK .AND. INUM .NE. 0

DRAWLINE = INUM .GE. 0

C

IF ( DRAWLINE )

.  STARTNEW = .TRUE.

.  CALL COLTYP(ICOL)

.  CALL MOVETO(X(1), Y(1), 0, LINTYP)

.  DO ( I = 2, N ) CALL MOVETO(X(I), Y(I), 1, LINTYP)

...FIN

C

IF ( DRAWMARK )

.  STARTNEW = .TRUE.

.  ITEMP = IABS(INUM)

.  CALL COLTYP(ICOL)

.  DO ( I = 1, N, ITEMP )

.  .  CALL MOVETO(X(I), Y(I), 0, LINTYP)

.  .  IF ( INSIDE ) CALL MARKER(ISYM, ISIZE)

.  ...FIN

...FIN

C

RETURN

END

PAGE 32

SUBROUTINE DRDASH( IX, IY, IUD )

INTEGER LIND(11)

LOGICAL INSIDE, STARTNEW

REAL IDASH(11), ISPACE(11)

COMMON /HWINDO/ SCRX,SCRY,IXORIG,IYORIG,IASCR,IBSCR,ICSCR,IDSCR

COMMON /LINDAT/ IDASH, ISPACE

COMMON /CHKBON/ INSIDE, STARTNEW, LINNTP, LIN1, DASH

DATA LIND / 1, 2, 3, 4, 5, 6, 9, 10, 7, 11, 8 /

DATA IDASH/.02, .04, .08, .16, .24, .32, .16, .16, .04, .04, .04/

DATA ISPACE/.08, .08, .08, .08, .12, .16, .08, .08, .08, .08, .08/

DATA LINNTP, LIN1 / 0, 0 /

C

WHEN ( IUD .EQ. 1 .AND. LINNTP .NE. 0 )

.  DX = IX - X0

.  DY = IY - Y0

.  DS = SQRT( DX * DX + DY * DY )

.  IF ( DS .GT. 0. )

.  .  C = DX / DS

.  .  S = DY / DS

.  ...FIN

.  WHILE ( DS .GT. 0. )

.  .  WHEN ( DASH .GT. 0. )

.  .  .  DDS = AMIN1 ( DS, DASH )

.  .  .  DS = DS - DDS

.  .  .  DASH = DASH - DDS

.  .  .  IF( DASH .LE. 0. ) SPACE = SCRX * ISPACE(LIN1)

.  .  .  X0 = X0 + DDS * C

.  .  .  Y0 = Y0 + DDS * S

.  .  .  IX0 = X0 + .5

.  .  .  IY0 = Y0 + .5

.  .  .  CALL PLOT( IX0, IY0, 1 )

.  .  ...FIN

.  .  ELSE

.  .  .  DDS = AMIN1 ( DS, SPACE )

.  .  .  DS = DS - DDS

.  .  .  SPACE = SPACE - DDS

.  .  .  IF( SPACE .LE. 0. )

.  .  .  .  LIN1 = LIND(LIN1)

.  .  .  .  DASH = SCRX * IDASH(LIN1)

.  .  .  ...FIN

.  .  .  X0 = X0 + DDS * C

.  .  .  Y0 = Y0 + DDS * S

.  .  .  IX0 = X0 + .5

.  .  .  IY0 = Y0 + .5

.  .  .  CALL PLOT( IX0, IY0, 0 )

.  .  ...FIN

.  ...FIN

...FIN

ELSE

.  CALL PLOT( IX, IY, IUD )

...FIN

X0 = IX

Y0 = IY

PAGE 33

C

RETURN

END

PAGE 34

SUBROUTINE DRWREC( XMIN, XMAX, YMIN, YMAX, ICOL )

C

CALL COLTYP(ICOL)

CALL MOVETO(XMIN, YMIN, 0, 0)

CALL MOVETO(XMAX, YMIN, 1, 0)

CALL MOVETO(XMAX, YMAX, 1, 0)

CALL MOVETO(XMIN, YMAX, 1, 0)

CALL MOVETO(XMIN, YMIN, 1, 0)

C

RETURN

END

PAGE 35

SUBROUTINE DRXTIC( IX, IY, LENTIC )

C

CALL PLOT(IX, IY, 0)

CALL PLOT(IX, IY + LENTIC, 1)

C

RETURN

END

PAGE 36

SUBROUTINE DRYTIC( IX, IY, LENTIC )

C

CALL PLOT(IX, IY, 0)

CALL PLOT(IX + LENTIC, IY, 1)

C

RETURN

END

PAGE 37

SUBROUTINE ENDLGN

COMMON /HFRAME/ I1(4)

COMMON /HSCALE/ X1(4)

COMMON /HWINDO/ SCRX, SCRY, I2(6)

COMMON /WORLD/ X2(4)

COMMON /SAVPAR/ J1(4), Y1(4), J2(6), Y2(4)

C

DO ( I = 1, 6 ) I2(I) = J2(I)

DO ( I = 1, 4 )

.  I1(I) = J1(I)

.  X1(I) = Y1(I)

.  X2(I) = Y2(I)

...FIN

C

RETURN

END

PAGE 38

SUBROUTINE FNDINT( IX0, IY0, IX, IY )

LOGICAL INX, INY

COMMON /HWINDO/ SCRX,SCRY,IXORIG,IYORIG,IASCR,IBSCR,ICSCR,IDSCR

C

INX = IX .GE. IASCR .AND. IX .LE. IBSCR

INY = IY .GE. ICSCR .AND. IY .LE. IDSCR

IF ( .NOT. INX )

.  XSL = (IY - IY0) / FLOAT(IX - IX0)

.  WHEN ( IX .GT. IBSCR ) IX = IBSCR

.  ELSE IX = IASCR

.  IY = XSL * (IX - IX0) + IY0

.  INY = IY .GE. ICSCR .AND. IY .LE. IDSCR

...FIN

IF ( .NOT. INY )

.  YSL = (IX - IX0) / FLOAT(IY - IY0)

.  WHEN( IY .GT. IDSCR ) IY = IDSCR

.  ELSE IY = ICSCR

.  IX = YSL * (IY - IY0) + IX0

...FIN

C

RETURN

END

PAGE 39

SUBROUTINE FRAME( XMINF, XMAXF, YMINF, YMAXF )

COMMON /PAGSIZ/ XBOND, YBOND

COMMON /HFRAME/ IXMINF, IXMAXF, IYMINF, IYMAXF

COMMON /HSCALE/ XSLOPE, XCONST, YSLOPE, YCONST

COMMON /HWINDO/ SCRX,SCRY,IXORIG,IYORIG,IASCR,IBSCR,ICSCR,IDSCR

C

XN = AMAX1 ( 0., XMINF )

XX = AMIN1 ( XBOND, XMAXF )

YN = AMAX1 ( 0., YMINF )

YX = AMIN1 ( YBOND, YMAXF )

IXMINF = SCRX * XN

IXMAXF = SCRX * XX

IYMINF = SCRY * YN

IYMAXF = SCRY * YX

C

IDX = IXMINF - IXORIG

IDY = IYMINF - IYORIG

IXORIG = IXMINF

IYORIG = IYMINF

C

C      MOVE WINDOW INTO FRAME

C

IASCR = IASCR + IDX

IBSCR = IBSCR + IDX

ICSCR = ICSCR + IDY

IDSCR = IDSCR + IDY

XCONST = XCONST + IDX

YCONST = YCONST + IDY

C

RETURN

END

PAGE 40

FUNCTION IFRMT( FORM, IFMT )

INTEGER FMT(2) , EMT, NUM(5), FORM(3)

DATA FMT / '(F', '9.' /

DATA EMT / '(G' /

DATA NUM / '0)', '1)', '2)', '3)', '2)' /

C

IFORM = MAX0( 0, IFMT )

FORM(1) = FMT(1)

FORM(2) = FMT(2)

FORM(3) = NUM( MIN0( 5 , IFORM + 1) )

IF ( IFORM .GT. 3 ) FORM(1) = EMT

IFRMT = IFORM

C

RETURN

END

PAGE 41

SUBROUTINE INILGN( XMIN, XMAX, YMIN, YMAX )

COMMON /HFRAME/ I1(4)

COMMON /HSCALE/ X1(4)

COMMON /HWINDO/ SCRX, SCRY, I2(6)

COMMON /WORLD/ X2(4)

COMMON /CHRSIZ/ ICHW(5), ICHH(5)

COMMON /SAVPAR/ J1(4), Y1(4), J2(6), Y2(4)

COMMON /LGND/ YSTRT, DELY, DELX, X(2)

C

DO ( I = 1, 6 ) J2(I) = I2(I)

DO ( I = 1, 4 )

.  J1(I) = I1(I)

.  Y1(I) = X1(I)

.  Y2(I) = X2(I)

...FIN

C

CALL FRAME( XMIN, XMAX, YMIN, YMAX )

CALL WINDOW( 0., XMAX - XMIN, 0., YMAX - YMIN )

CALL VUPORT( 0., 1., 0., 1. )

DELY = YWORLD( ICHH(2) ) - YWORLD(0)

DELX = XWORLD( ICHW(2) ) - XWORLD(0)

YSTRT = .99 - DELY

X(1) = .05

X(2) = .2

C

RETURN

END

PAGE 42

FUNCTION ISCRX( XWRLD )

COMMON /HSCALE/ XSLOPE, XCONST, YSLOPE, YCONST

C

X = AMIN1( XWRLD * XSLOPE + XCONST, 32767. )

X = AMAX1 ( X, -32767. )

ISCRX = IFIX( X + .5 )

C

RETURN

END

PAGE 43

FUNCTION ISCRY( YWRLD )

COMMON /HSCALE/ XSLOPE, XCONST, YSLOPE, YCONST

C

Y = AMIN1 ( YWRLD * YSLOPE + YCONST, 32767. )

Y = AMAX1 ( Y, -32767. )

ISCRY = IFIX ( Y + .5 )

C

RETURN

END

PAGE 44

SUBROUTINE LABELX( XVAL, FORM, IFORM, IS, IY, ICHW, IXSIZ )

BYTE LABEL(9)

INTEGER FORM(3)

C

IF ( IXSIZ .NE. 0 )

.  ENCODE ( 9 , FORM, LABEL ) XVAL

.  CALL TRIML(LABEL, 9, LEN)

.  IF ( IFORM .EQ. 0 ) LEN = LEN - 1

.  IXL = LEN * ICHW

.  IX = IS - IXL / 2

.  CALL PLTSTR(IX, IY, LABEL , LEN , 1 , IXSIZ)

...FIN

C

RETURN

END

PAGE 45

SUBROUTINE LABELY( YVAL, FORM, IFORM, IS, IY, ICHW, IYSIZ )

BYTE LABEL(9)

INTEGER FORM(3)

COMMON /YLABEL/ IXMIN

C

IF ( IYSIZ .NE. 0 )

.  ENCODE ( 9 , FORM, LABEL ) YVAL

.  CALL TRIML ( LABEL, 9, LEN )

.  IF ( IFORM .EQ. 0 ) LEN = LEN - 1

.  IX = IS - ( LEN + 1 ) * ICHW

.  IXMIN = MIN0( IXMIN, IX )

.  CALL PLTSTR(IX, IY, LABEL , LEN , 1 , IYSIZ)

...FIN

C

RETURN

END

PAGE 46

SUBROUTINE LINE( X, Y, N, ICOL, ISYM, ISIZE, INUM )

DIMENSION X(N), Y(N)

C

CALL DASHLN(X, Y, N, ICOL, ISYM, ISIZE, INUM, 0 )

C

RETURN

END

PAGE 47

SUBROUTINE MOVETO( XWRLD, YWRLD, IUD, LINTYP )

LOGICAL CURRENTINSIDE, INX, INY, PREVIOUSINSIDE, STARTNEW

COMMON /HWINDO/ SCRX,SCRY,IXORIG,IYORIG,IASCR,IBSCR,ICSCR,IDSCR

COMMON /HCURPO/ IXCUR, IYCUR

COMMON /CHKBON/ PREVIOUSINSIDE, STARTNEW, LINNTP, LIN1, DASH

C

WHEN ( STARTNEW .OR. LINNTP .NE. LINTYP )

.  CALL MOV1ST(XWRLD, YWRLD, IUD, LINTYP)

.  STARTNEW = .FALSE.

...FIN

ELSE

.  IXC = ISCRX( XWRLD )

.  IYC = ISCRY( YWRLD )

.  IX = IXC

.  IY = IYC

.  INX = IX .GE. IASCR .AND. IX .LE. IBSCR

.  INY = IY .GE. ICSCR .AND. IY .LE. IDSCR

.  CURRENTINSIDE = INX .AND. INY

.  WHEN ( CURRENTINSIDE .OR. PREVIOUSINSIDE )

.  .  IF ( .NOT. ( CURRENTINSIDE .AND. PREVIOUSINSIDE ) )

.  .  .  IX0 = IXCUR

.  .  .  IY0 = IYCUR

.  .  .  WHEN ( PREVIOUSINSIDE )

.  .  .  .  PREVIOUSINSIDE = .FALSE.

.  .  .  .  CALL FNDINT( IX0, IY0, IX, IY )

.  .  .  ...FIN

.  .  .  ELSE

.  .  .  .  PREVIOUSINSIDE = .TRUE.

.  .  .  .  CALL FNDINT( IX, IY, IX0, IY0 )

.  .  .  .  IXCUR = IX0

.  .  .  .  IYCUR = IY0

.  .  .  .  CALL DRDASH(IX0, IY0, 0)

.  .  .  ...FIN

.  .  ...FIN

.  .  CALL DRDASH(IX, IY, IUD)

.  ...FIN

.  ELSE

.  .  CALL ALLOUT( IX, IY, IUD )

.  ...FIN

.  IXCUR = IXC

.  IYCUR = IYC

...FIN

C

RETURN

END

PAGE 48

SUBROUTINE MOV1ST( XWRLD, YWRLD, IUD, LINTYP )

LOGICAL INSIDE, INX, INY, STARTNEW

COMMON /HWINDO/ SCRX,SCRY,IXORIG,IYORIG,IASCR,IBSCR,ICSCR,IDSCR

COMMON /HCURPO/ IXCUR, IYCUR

COMMON /CHKBON/ INSIDE, STARTNEW, LINNTP, LIN1, DASH

COMMON /LINDAT/ RDASH(11), RSPACE(11)

C

IX = ISCRX( XWRLD )

IY = ISCRY( YWRLD )

INX = IX .GE. IASCR .AND. IX .LE. IBSCR

INY = IY .GE. ICSCR .AND. IY .LE. IDSCR

INSIDE = INX .AND. INY

WHEN ( LINTYP .GT. 0 )

.  LINNTP = MIN0( MAX0( LINTYP, 1 ), 8 )

.  DASH = SCRX * RDASH(LINNTP)

...FIN

ELSE

.  LINNTP = 0

...FIN

LIN1 = LINNTP

IF ( INSIDE ) CALL DRDASH( IX, IY, IUD )

IXCUR = IX

IYCUR = IY

C

RETURN

END

PAGE 49

SUBROUTINE PLTSTR( IX, IY, STR, LEN, IROT, ISIZE )

BYTE STR(LEN)

LOGICAL INX, INY

COMMON /HFRAME/ IXMINF, IXMAXF, IYMINF, IYMAXF

C

INX = IX .GE. IXMINF .AND. IX .LE. IXMAXF

INY = IY .GE. IYMINF .AND. IY .LE. IYMAXF

IF ( INX .AND. INY )

.  CALL PLOT ( IX, IY, 0 )

.  CALL WRTSTR ( STR, LEN, IROT, ISIZE )

...FIN

C

RETURN

END

PAGE 50

SUBROUTINE RECLGN( NL, TITLE, N, ICOL, ISYM, INUM, LINTYP,

1                   XSTRT, XLEN, YSTRT )

BYTE TITLE(1)

DIMENSION XSTRT(NL), XLEN(NL), YSTRT(NL)

INTEGER N(NL), ICOL(NL), ISYM(NL), INUM(NL), LINTYP(NL)

COMMON /PAGSIZ/ XBOND, YBOND

COMMON /LGND/ YSTART, DELY, DELX, X(2)

C

CALL INILGN( 0., XBOND, 0., YBOND )

IST = 1

DO  ( I = 1, NL )

.  X(1) = XSTRT(I)

.  X(2) = XSTRT(I) + XLEN(I)

.  YSTART = YSTRT(I)

.  CALL WRILGN(TITLE(IST),N(I),ICOL(I),ISYM(I),INUM(I),LINTYP(I))

.  IST = IST + N(I)

...FIN

CALL ENDLGN

C

RETURN

END

PAGE 51

SUBROUTINE SCALE( XMIN, XMAX, YMIN, YMAX )

COMMON /HSCALE/ XSLOPE, XCONST, YSLOPE, YCONST

COMMON /HWINDO/ SCRX,SCRY,IXORIG,IYORIG,IASCR,IBSCR,ICSCR,IDSCR

COMMON /WORLD/ XMINW, XMAXW, YMINW, YMAXW

C

XMINW = XMIN

XMAXW = XMAX

YMINW = YMIN

YMAXW = YMAX

C

XSLOPE = (IBSCR - IASCR) / (XMAX - XMIN)

YSLOPE = (IDSCR - ICSCR) / (YMAX - YMIN)

XCONST = - XSLOPE * XMIN + FLOAT(IASCR)

YCONST = - YSLOPE * YMIN + FLOAT(ICSCR)

C

RETURN

END

PAGE 52

SUBROUTINE TRIML( LABEL, N, LEN )

BYTE LABEL(N)

C

I = 1

WHILE ( I .LT. N .AND. LABEL(I) .EQ. ' ' ) I = I + 1

LEN = 0

DO ( J = I, N )

.  LEN = LEN + 1

.  LABEL(LEN) = LABEL(J)

...FIN

C

RETURN

END

PAGE 53

SUBROUTINE TRIMSP( TITLE, LEN, IST, IEND )

BYTE TITLE(LEN)

C

IST = 1

IEND = LEN

WHILE ( IST .LT. LEN .AND. TITLE(IST) .EQ. ' ' ) IST = IST + 1

WHILE ( IEND .GT. IST .AND. TITLE(IEND) .EQ. ' ' ) IEND = IEND - 1

C

RETURN

END

PAGE 54

SUBROUTINE VUPORT( XMIN, XMAX, YMIN, YMAX )

C

CALL SCALE(XMIN, XMAX, YMIN, YMAX)

C

RETURN

END

PAGE 55

SUBROUTINE WINDOW( XWIN0, XWIN1, YWIN0, YWIN1 )

COMMON /HWINDO/ SCRX,SCRY,IXORIG,IYORIG,IASCR,IBSCR,ICSCR,IDSCR

COMMON /WORLD/ XMINW, XMAXW, YMINW, YMAXW

C

IASCR = SCRX * XWIN0 + IXORIG

IBSCR = SCRX * XWIN1 + IXORIG

ICSCR = SCRY * YWIN0 + IYORIG

IDSCR = SCRY * YWIN1 + IYORIG

C

CALL SCALE( XMINW, XMAXW, YMINW, YMAXW )

C

RETURN

END

PAGE 56

SUBROUTINE WRILGN( TITLE, N, ICOL, ISYM, INUM, LINTYP )

BYTE TITLE(N)

DIMENSION X(2), Y(2)

COMMON /LGND/ YSTRT, DELY, DELX, XX(2)

DATA XX / .05, .2 /

C

Y(1) = YSTRT

Y(2) = YSTRT

X(2) = XX(2)

CONDITIONAL

.  ( INUM .LT. 0 )

.  .  NUM = -2

.  .  X(1) = XX(2)

.  ...FIN

.  ( INUM .GT. 0 )

.  .  NUM = 1

.  .  X(1) = XX(1)

.  ...FIN

.  ( INUM .EQ. 0 )

.  .  NUM = 0

.  .  X(1) = XX(1)

.  ...FIN

...FIN

CALL DASHLN( X, Y, 2, ICOL, ISYM, 2, NUM, LINTYP )

IX = ISCRX( X(2) + DELX )

IY = ISCRY( YSTRT - .5 * DELY )

CALL PLTSTR( IX, IY, TITLE, N, 1, 2 )

YSTRT = YSTRT - DELY

C

RETURN

END

PAGE 57

SUBROUTINE XAXIS( XTIC, IXTIT, IXLEN, IXSIZ, IXFMT )

BYTE IXTIT(IXLEN)

INTEGER FORM(3)

COMMON /HWINDO/ SCRX,SCRY,IXORIG,IYORIG,IASCR,IBSCR,ICSCR,IDSCR

COMMON /WORLD/ XMINW, XMAXW, YMINW, YMAXW

COMMON /CHRSIZ/ ICHW(5), ICHH(5)

C

WHEN ( IXSIZ .GE. 1 .AND. IXSIZ .LE. 5 ) IXS = IXSIZ

ELSE                                     IXS = 0

IF ( XTIC .NE. 0. )

.  IS = IASCR

.  IY = ICSCR - ICHH(MAX0( IXS, 1 )) * 2

.  LENTIC = SCRX / 4

.  XVAL = XMINW

.  IFORM = IFRMT( FORM, IXFMT )

.  XMAX = AMAX1( XMINW, XMAXW )

.  XMIN = AMIN1( XMINW, XMAXW )

.  IW = ICHW(MAX0(IXS,1))

.  WHILE ( XVAL .LE. XMAX .AND. XVAL .GE. XMIN )

.  .  WHEN ( XVAL .EQ. XMINW )

.  .  .  CALL LABELX( XVAL, FORM, IFORM, IS, IY, IW, IXS )

.  .  ...FIN

.  .  ELSE

.  .  .  IS = ISCRX( XVAL )

.  .  .  IF( IS .NE. IBSCR ) CALL DRXTIC(IS, ICSCR, LENTIC)

.  .  .  CALL LABELX( XVAL, FORM, IFORM, IS, IY, IW, IXS )

.  .  ...FIN

.  .  XVAL = XVAL + XTIC

.  ...FIN

...FIN

C

C      CENTER HORIZONTAL TITLE AND PLOT

C

IF( IXS .NE. 0 )

.  CALL TRIMSP( IXTIT, IXLEN, IST, IEND )

.  IXLN = IEND + 1 - IST

.  IXL = IXLN * ICHW(IXS)

.  IX = (IASCR + IBSCR - IXL) / 2

.  IY = ICSCR - 4 * ICHH(IXS)

.  CALL PLTSTR(IX, IY, IXTIT(IST), IXLN, 1, IXS)

...FIN

C

RETURN

END

PAGE 58

FUNCTION XWORLD( IX )

COMMON /HSCALE/ XSLOPE, XCONST, YSLOPE, YCONST

C

XWORLD = ( IX - XCONST ) / XSLOPE

C

RETURN

END

PAGE 59

SUBROUTINE YAXIS( YTIC, IYTIT, IYLEN, IYSIZ, IYFMT )

BYTE IYTIT(IYLEN)

INTEGER FORM(3)

COMMON /HWINDO/ SCRX,SCRY,IXORIG,IYORIG,IASCR,IBSCR,ICSCR,IDSCR

COMMON /WORLD/ XMINW, XMAXW, YMINW, YMAXW

COMMON /CHRSIZ/ ICHW(5), ICHH(5)

COMMON /YLABEL/ IXMIN

C

WHEN ( IYSIZ .GE. 1 .AND. IYSIZ .LE. 5 ) IYS = IYSIZ

ELSE                                     IYS = 0

IXMIN = IASCR

IF ( YTIC .NE. 0. )

.  IX = IASCR

.  IS = ICSCR

.  LENTIC = SCRY / 4

.  IFORM = IFRMT( FORM, IYFMT )

.  YVAL = YMINW

.  IHGT = ICHH(MAX0(IYS,1)) / 2

.  IY = IS - IHGT

.  IW = ICHW(MAX0(IYS,1))

.  YMAX = AMAX1( YMINW, YMAXW )

.  YMIN = AMIN1( YMINW, YMAXW )

.  WHILE ( YVAL .LE. YMAX .AND. YVAL .GE. YMIN )

.  .  WHEN ( YVAL .EQ. YMINW )

.  .  .  CALL LABELY( YVAL, FORM, IFORM, IASCR, IY, IW, IYS )

.  .  ...FIN

.  .  ELSE

.  .  .  IS = ISCRY( YVAL )

.  .  .  IY = IS - IHGT

.  .  .  IF( IDSCR .NE. IS ) CALL DRYTIC( IASCR, IS, LENTIC )

.  .  .  CALL LABELY( YVAL, FORM, IFORM, IASCR, IY, IW, IYS )

.  .  ...FIN

.  .  YVAL = YVAL + YTIC

.  ...FIN

...FIN

C

C      CENTER VERTICAL TITLE AND PLOT

C

IF ( IYS .NE. 0 )

.  CALL TRIMSP( IYTIT, IYLEN, IST, IEND )

.  IYLN = IEND + 1 - IST

.  IX = IXMIN - ICHH(IYS)

.  IYL = IYLN * ICHW(IYS)

.  IY = (ICSCR + IDSCR - IYL) / 2

.  CALL PLTSTR(IX, IY, IYTIT(IST), IYLN, 4, IYS)

...FIN

C

RETURN

END

PAGE 60

FUNCTION YWORLD( IY )

COMMON /HSCALE/ XSLOPE, XCONST, YSLOPE, YCONST

C

YWORLD = ( IY - YCONST ) / YSLOPE

C

RETURN

END

PAGE 61

APPENDIX B - HIPLOT AND VISUAL ROUTINES

SUBROUTINE CIRCLE

SUBROUTINE COLTYP

SUBROUTINE CONLGN

SUBROUTINE CURSON

SUBROUTINE DEFWIN

SUBROUTINE DMPPLT

SUBROUTINE ENDPLT

SUBROUTINE ERASE

SUBROUTINE FNDCUR

SUBROUTINE GETSTR

SUBROUTINE INIPLT

SUBROUTINE MARKER

SUBROUTINE PENDWN

SUBROUTINE PENUP

SUBROUTINE PLOT

SUBROUTINE PLTOFF

SUBROUTINE PLTON

SUBROUTINE PUTCUR

SUBROUTINE REDSTR

SUBROUTINE WRITCH

SUBROUTINE WRTSTR

PAGE 62

SUBROUTINE CIRCLE( IX, IY, IR )

BYTE MES(16)

LOGICAL VISUAL

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

1     FORMAT( 'CC', I4, ',', I4, I5 )

2     FORMAT( A1, '/', I3, ';', I3, ';', I2, ';0A' )

C

WHEN ( VISUAL )

.  CALL PENDWN

.  ENCODE( 15, 2, MES ) 27, IX, IY, IR

.  DO ( I = 1, 15 ) IF( MES(I) .EQ. ' ' ) MES(I) = '0'

.  CALL WRITCH( MES, 15 )

...FIN

ELSE

.  ENCODE( 16, 1, MES ) IX, IY, IR

.  CALL WRITCH( MES, 16 )

...FIN

C

RETURN

END

PAGE 63

SUBROUTINE COLTYP( ICOL )

BYTE MES(2)

LOGICAL VISUAL

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

C

IF( .NOT. VISUAL )

.  ITYPE = ICOL + 1

.  ITYPE = MIN0(ITYPE, 8)

.  ITYPE = MAX0(ITYPE, 1)

.  MES(1) = 'P'

.  MES(2) = ITYPE + 48

.  CALL WRITCH(MES, 2)

...FIN

C

RETURN

END

PAGE 64

SUBROUTINE CONLGN( NL, TITLE, N, ICOL, ISYM, INUM, LINTYP,

1                   XSTRT, XLEN, YSTRT )

BYTE TITLE(1)

DIMENSION XSTRT(NL), XLEN(NL), YSTRT(NL)

INTEGER N(NL), ICOL(NL), ISYM(NL), INUM(NL), LINTYP(NL)

LOGICAL VISUAL

COMMON /LGND/ YSTART, DELY, DELX, X(2)

COMMON /PAGSIZ/ XBOND, YBOND

COMMON /CRSPOS/ IXX, IYY

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

DATA IXX, IYY, XST, XLN / -100, -100, -1., .15 /

C

IF( VISUAL )

.  CALL INILGN( 0., XBOND, 0., YBOND )

.  CALL DRWREC( 0., 1., 0., 1., 0 )

.  X(2) = XWORLD(IXX) - DELX

.  X(1) = X(2) - XLN

.  YSTART = YWORLD(IYY) + .5 * DELY

.  IST = 1

.  DO ( I = 1, NL )

.  .  XLEN(I) = X(2) - X(1)

.  .  XSTRT(I) = X(1)

.  .  YSTRT(I) = YSTART

.  .  CALL FNDCUR( IX, IY )

.  .  IF( IX .NE. IXX .OR. IY .NE. IYY )

.  .  .  XSTRT(I) = XWORLD(IX)

.  .  .  YSTRT(I) = YWORLD(IY)

.  .  .  CALL FNDCUR( IX, IY )

.  .  .  XLEN(I) = XWORLD(IX) - XSTRT(I)

.  .  .  IF( XLEN(I) .LT. 0. ) XLEN(I) = XLN

.  .  ...FIN

.  .  X(1) = XSTRT(I)

.  .  X(2) = XLEN(I) + XSTRT(I)

.  .  YSTART = YSTRT(I)

.  .  CALL WRILGN('', 0, ICOL(I), ISYM(I), INUM(I), LINTYP(I))

.  .  CALL CURPOS( IX, IY )

.  .  CALL GETSTR( IX, IY, TITLE(IST), N(I), 1, 2 )

.  .  IST = IST + N(I)

.  .  XST = XSTRT(I)

.  .  XLN = XLEN(I)

.  ...FIN

.  CALL ENDLGN

...FIN

C

RETURN

END

PAGE 65

SUBROUTINE CURSON

BYTE ESC, FF, SUB, ENQ, CHAR

COMMON /HBYTE/ ESC, FF, SUB, ENQ, CHAR

C

CALL WRITCH(ESC, 1)

CALL WRITCH(SUB, 1)

CALL DMPPLT

C

RETURN

END

PAGE 66

SUBROUTINE DEFWIN( XMIN, XMAX, YMIN, YMAX )

BYTE ESC, FF, SUB, ENQ, CHAR

INTEGER IX, IY, JX, JY, B

LOGICAL VISUAL

COMMON /HBYTE/ ESC, FF, SUB, ENQ, CHAR

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

C

IF( VISUAL )

.  CALL FNDCUR(IX, IY)

.  CALL FNDCUR(JX, JY)

.  IXN = MIN0( IX, JX )

.  IXX = MAX0( IX, JX )

.  IYN = MIN0( IY, JY )

.  IYX = MAX0( IY, JY )

.  XMIN = IXN / 75.

.  XMAX = IXX / 75.

.  YMIN = IYN / 75.

.  YMAX = IYX / 75.

...FIN

C

RETURN

END

PAGE 67

SUBROUTINE DMPPLT

BYTE VTBUFR(128)

INTEGER VTPOS, VTBUFL

LOGICAL VISUAL

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

COMMON /VTBUF/ VTBUFR, VTPOS, VTBUFL

1     FORMAT( 1X, 128A1 )

2     FORMAT( '+', 128( A1, $ ) )

C

WHEN ( VISUAL ) WRITE(LUNPLT, 2) ( VTBUFR(I), I = 1, VTPOS )

ELSE            WRITE(LUNPLT, 1) ( VTBUFR(I), I = 1, VTPOS )

VTPOS = 0

C

RETURN

END

PAGE 68

SUBROUTINE ENDPLT

BYTE MES(12)

LOGICAL VISUAL

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

DATA MES /'P','0','E','F','3','0','0','0',',','0',' ','@'/

C

WHEN ( VISUAL )

.  CALL PLTOFF

.  CALL DMPPLT

...FIN

ELSE

.  CALL WRITCH(MES, 12 )

.  IF( LUNPLT .EQ. ITTNUM ) CALL PLTOFF

.  CALL DMPPLT

...FIN

IF ( LUNPLT .NE. ITTNUM ) CLOSE( UNIT = LUNPLT )

C

RETURN

END

PAGE 69

SUBROUTINE ERASE

BYTE MES(2)

LOGICAL VISUAL

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

DATA MES / 27, 12 /

C

IF( VISUAL ) CALL WRITCH(MES, 2)

C

RETURN

END

PAGE 70

SUBROUTINE FNDCUR( IX, IY )

BYTE ESC, FF, SUB, ENQ, CHAR

COMMON /HBYTE/ ESC, FF, SUB, ENQ, CHAR

BYTE HIGHX, LOWX, HIGHY, LOWY

1     FORMAT( 5A1 )

C

CALL CURSON

READ(5, 1) HIGHX, LOWX, HIGHY, LOWY, CHAR

IF ( CHAR .NE. 8 )

.  HIGHX = LOWX

.  LOWX = HIGHY

.  HIGHY = LOWY

.  LOWY = CHAR

...FIN

IHIGHX = HIGHX .AND. 31

ILOWX = LOWX .AND. 31

IHIGHY = HIGHY .AND. 31

ILOWY = LOWY .AND. 31

IX = IHIGHX * 32 + ILOWX

IY = IHIGHY * 32 + ILOWY

C

RETURN

END

PAGE 71

SUBROUTINE GETSTR( IX, IY, STR, LEN, IROT, ISIZE )

BYTE STR(80)

LOGICAL VISUAL

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

COMMON /CHRSIZ/ ICHW(5), ICHH(5)

1     FORMAT( Q, 80A1)

C

IF( VISUAL )

.  CALL PLTSTR( IX, IY, '', 0, IROT, ISIZE )

.  CALL DMPPLT

.  READ(5,1) LEN, (STR(J), J = 1, LEN )

.  ISZ = MIN0( 5, MAX0( 1, ISIZE ) )

.  ID = ICHH(2)

.  IXX = IX + ID * ( IABS( MOD( IROT - 1, 4 ) - 1 ) - 1 )

.  IYY = IY + ID * ( 1 - IABS( MOD( IROT - 1, 4 ) - 2 ) )

.  CALL PUTCUR( IXX, IYY )

...FIN

C

RETURN

END

PAGE 72

SUBROUTINE INIPLT( IUNIT, XSIZE, YSIZE )

BYTE ESC, FF, SUB, ENQ, CHAR, MES(10), TRANS(2), VTBUFR(128)

INTEGER VTPOS, VTBUFL

INTEGER VCHRWIDTH(5), VCHRHEIGHT(5)

INTEGER HCHRWIDTH(5), HCHRHEIGHT(5)

LOGICAL VISUAL

COMMON /PAGSIZ/ XBOND, YBOND

COMMON /HFRAME/ IXMINF, IXMAXF, IYMINF, IYMAXF

COMMON /HSCALE/ XSLOPE, XCONST, YSLOPE, YCONST

COMMON /HWINDO/ SCRX,SCRY,IXORIG,IYORIG,IASCR,IBSCR,ICSCR,IDSCR

COMMON /WORLD/ XMINW, XMAXW, YMINW, YMAXW

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

COMMON /CHRSIZ/ ICHW(5), ICHH(5)

COMMON /PENPOS/ IUPDWN

COMMON /HBYTE/ ESC, FF, SUB, ENQ, CHAR

COMMON /VTBUF/ VTBUFR, VTPOS, VTBUFL

DATA MES / ';', ':', 'E', 'H', 'H', 'O', 'U', 'A', 'L', '0' /

DATA FF , ENQ, SUB, ESC, TRANS / 12, 5, 26, 27, 29, 77 /

DATA VCHRWIDTH, VCHRHEIGHT / 7,10,20,30,40, 10,15,30,45,60 /

DATA HCHRWIDTH, HCHRHEIGHT / 18,24,36,48,72, 24,36,54,72,108 /

DATA IASCR, IBSCR, ICSCR, IDSCR, IXORIG, IYORIG, XCONST, YCONST

1    / 6 * 0, 2 * 0. /

C

ITTNUM = 7

LUNPLT = IUNIT

VISUAL = LUNPLT .EQ. ITTNUM

VTBUFL = 128

VTPOS = 0

IUPDWN = 0

WHEN ( VISUAL )

.  CALL WRITCH(TRANS, 2)

.  CALL ERASE

.  SCRX = 75.

.  SCRY = 75.

.  DO ( I = 1, 5 )

.  .  ICHW(I) = VCHRWIDTH(I)

.  .  ICHH(I) = VCHRHEIGHT(I)

.  ...FIN

...FIN

ELSE

.  IF ( IUNIT .EQ. 8 )

.  .  LUNPLT = ITTNUM

.  .  CALL PLTON

.  ...FIN

.  CALL WRITCH(MES, 10 )

.  SCRX = 200.

.  SCRY = 200.

.  DO ( I = 1, 5 )

.  .  ICHW(I) = HCHRWIDTH(I)

.  .  ICHH(I) = HCHRHEIGHT(I)

.  ...FIN

...FIN

XBOND = XSIZE

PAGE 73

YBOND = YSIZE

XMINW = 0.

XMAXW = 1.

YMINW = 0.

YMAXW = 1.

CALL FRAME( 0., XSIZE, 0., YSIZE )

CALL WINDOW( .2 * XSIZE, .8 * XSIZE, .2 * YSIZE, .8 * YSIZE )

C

RETURN

END

PAGE 74

SUBROUTINE MARKER( ISYM, ISIZE )

BYTE MES(4)

INTEGER MARKSIZE(5)

LOGICAL VISUAL

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

DATA MARKSIZE / 2, 3, 4, 6, 9 /

1     FORMAT('M ', 2I1)

C

ISZ = MAX0( MIN0(ISIZE, 5), 1 )

MRK = MIN0( MAX0(ISYM, 0), 5 )

WHEN ( VISUAL )

.  IW = MARKSIZE(ISZ)

.  CALL CURPOS( IX, IY )

.  SELECT ( MRK )

.  .  (0)  !  PLUS SIGN

.  .  .  CALL PLOT( IX-IW, IY, 0 )

.  .  .  CALL PLOT( IX+IW, IY, 1 )

.  .  .  CALL PLOT( IX, IY-IW, 0 )

.  .  .  CALL PLOT( IX, IY+IW, 1 )

.  .  ...FIN

.  .  (1)  !  CROSS

.  .  .  CALL PLOT( IX-IW, IY-IW, 0 )

.  .  .  CALL PLOT( IX+IW, IY+IW, 1 )

.  .  .  CALL PLOT( IX+IW, IY-IW, 0 )

.  .  .  CALL PLOT( IX-IW, IY+IW, 1 )

.  .  ...FIN

.  .  (2)  !  SQUARE

.  .  .  CALL PLOT( IX-IW, IY-IW, 0 )

.  .  .  CALL PLOT( IX+IW, IY-IW, 1 )

.  .  .  CALL PLOT( IX+IW, IY+IW, 1 )

.  .  .  CALL PLOT( IX-IW, IY+IW, 1 )

.  .  .  CALL PLOT( IX-IW, IY-IW, 1 )

.  .  ...FIN

.  .  (3)  !  CIRCLE

.  .  .  CALL CIRCLE( IX, IY, IW )

.  .  ...FIN

.  .  (4)  !  TRIANGLE

.  .  .  CALL PLOT( IX-IW, IY-IW, 0 )

.  .  .  CALL PLOT( IX+IW, IY-IW, 1 )

.  .  .  CALL PLOT( IX, IY+IW, 1 )

.  .  .  CALL PLOT( IX-IW, IY-IW, 1 )

.  .  ...FIN

.  .  (5)  !  HOURGLASS

.  .  .  CALL PLOT( IX-IW, IY-IW, 0 )

.  .  .  CALL PLOT( IX+IW, IY+IW, 1 )

.  .  .  CALL PLOT( IX-IW, IY+IW, 1 )

.  .  .  CALL PLOT( IX+IW, IY-IW, 1 )

.  .  .  CALL PLOT( IX-IW, IY-IW, 1 )

.  .  ...FIN

.  ...FIN

...FIN

ELSE

.  ENCODE( 4, 1, MES ) ( ISZ + 2 ) / 2, MRK

PAGE 75

.  IF ( MOD( ISZ, 2 ) .EQ. 1 )

.  .  MES(2) = MES(3)

.  .  MES(3) = '+'

.  ...FIN

.  CALL WRITCH(MES, 4)

...FIN

C

RETURN

END

PAGE 76

SUBROUTINE PENDWN

BYTE MESH(2), MESV(5)

LOGICAL VISUAL

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

COMMON /PENPOS/ IUPDWN

DATA MESH, MESV / 'D', ' ', 27, '/', '0', 'd', 29 /

C

WHEN ( VISUAL )

.  CALL WRITCH( MESV, 5 )

...FIN

ELSE

.  IF( IUPDWN .EQ. 0 ) CALL WRITCH(MESH, 2)

.  IUPDWN = 1

...FIN

C

RETURN

END

PAGE 77

SUBROUTINE PENUP

BYTE MESH(2), MESV(5)

LOGICAL VISUAL

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

COMMON /PENPOS/ IUPDWN

DATA MESH, MESV / 'U', ' ', 27, '/', '1', 'd', 28 /

C

WHEN ( VISUAL )

.  CALL WRITCH( MESV, 5 )

...FIN

ELSE

.  IF( IUPDWN .EQ. 1 ) CALL WRITCH(MESH, 2)

.  IUPDWN = 0

...FIN

C

RETURN

END

PAGE 78

SUBROUTINE PLOT( IX, IY, IUD )

BYTE LOWY, HIGHY, LOWX, HIGHX, YL, YH, XL, XH

BYTE MES(10)

LOGICAL VISUAL

COMMON /HFRAME/ IXMINF, IXMAXF, IYMINF, IYMAXF

COMMON /HCURPO/ IXCUR, IYCUR

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

DATA YL, YH, XL, XH / 96, 32, 64, 32 /

1     FORMAT( I4, ',', I4, 1X )

C

IX0 = IX

IY0 = IY

C

C      CHECK TO SEE IF IN BOUNDS

C

CONDITIONAL

.  ( IX0 .LT. IXMINF ) IX0 = IXMINF

.  ( IX0 .GT. IXMAXF ) IX0 = IXMAXF

...FIN

CONDITIONAL

.  ( IY0 .LT. IYMINF ) IY0 = IYMINF

.  ( IY0 .GT. IYMAXF ) IY0 = IYMAXF

...FIN

WHEN (IUD .EQ. 1) CALL PENDWN

ELSE CALL PENUP

C

WHEN ( .NOT. VISUAL )

.  ENCODE( 10, 1, MES ) IX0, IY0

.  CALL WRITCH(MES, 10)

...FIN

ELSE

.  MES(2) = MOD( IYCUR, 32 ) .OR. YL

.  MES(1) = ( IYCUR / 32 ) .OR. YH

.  MES(4) = MOD( IXCUR, 32 ) .OR. XL

.  MES(3) = ( IXCUR / 32 ) .OR. XH

.  MES(6) = MOD( IY0, 32 ) .OR. YL

.  MES(5) = ( IY0 / 32 ) .OR. YH

.  MES(8) = MOD( IX0, 32 ) .OR. XL

.  MES(7) = ( IX0 / 32 ) .OR. XH

.  CALL WRITCH(MES, 8)

...FIN

IXCUR = IX0

IYCUR = IY0

C

RETURN

END

PAGE 79

SUBROUTINE PLTOFF

BYTE MES(6)

LOGICAL VISUAL

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

DATA MES/ 27, '[', '?', '6', 'i', ' '/

C

WHEN ( .NOT. VISUAL )

.  CALL WRITCH(MES,6)

...FIN

ELSE

.  CALL WRITCH(24, 1)

...FIN

C

RETURN

END

PAGE 80

SUBROUTINE PLTON

BYTE MES(6)

LOGICAL VISUAL

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

DATA MES/ 27, '[', '?', '7', 'i', ' '/

C

WHEN ( .NOT. VISUAL )

.  CALL WRITCH(MES,6)

...FIN

ELSE

.  CALL WRITCH(31,1)

...FIN

C

RETURN

END

PAGE 81

SUBROUTINE PUTCUR( IX, IY)

BYTE ESC, FF, SUB, ENQ, CHAR

LOGICAL VISUAL

COMMON /HBYTE/ ESC, FF, SUB, ENQ, CHAR

COMMON /CRSPOS/ IXX, IYY

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

C

IF( VISUAL )

.  CALL PLOT( IX, IY, 0 )

.  CALL WRITCH( ESC, 1 )

.  CALL WRITCH( '/', 1 )

.  CALL WRITCH( 'f', 1 )

.  CALL WRITCH( ' ', 1 )

.  IXX = IX

.  IYY = IY

...FIN

C

RETURN

END

PAGE 82

SUBROUTINE REDSTR( IX, IY, STR, LEN, IROT, ISIZE )

BYTE STR(1)

LOGICAL VISUAL

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

C

IF( VISUAL )

.  CALL FNDCUR( IX, IY )

.  CALL GETSTR( IX, IY, STR, LEN, IROT, ISIZE )

...FIN

C

RETURN

END

PAGE 83

SUBROUTINE WRITCH( CH, N )

BYTE VTBUFR(128), CH(N)

INTEGER VTPOS, VTBUFL

LOGICAL VISUAL

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

COMMON /VTBUF/ VTBUFR, VTPOS, VTBUFL

C

IF( VTPOS + N .GT. VTBUFL ) CALL DMPPLT

DO ( I = 1, N )

.  VTPOS = VTPOS + 1

.  VTBUFR(VTPOS) = CH(I)

...FIN

IF ( VTPOS .GE. VTBUFL ) CALL DMPPLT

C

RETURN

END

PAGE 84

SUBROUTINE WRTSTR( STR, LEN, IROT, ISIZE )

BYTE STR(LEN), MES(93), IPLS(2), INUM(5)

LOGICAL VISUAL, INX, INY

COMMON /HFRAME/ IXMINF, IXMAXF, IYMINF, IYMAXF

COMMON /HCURPO/ IXCUR, IYCUR

COMMON /DEVICE/ VISUAL, LUNPLT, ITTNUM

COMMON /CHRSIZ/ ICHW(5), ICHH(5)

DATA INUM, IPLS / ':', '0', '1', '2', '3', ' ', '+' /

1     FORMAT('$Warning  string ', 100A1)

2     FORMAT('  is out of bounds and will not be plotted')

3     FORMAT( 'S', 2I1, A1 )

C

ISZ = MIN0( MAX0( ISIZE, 1 ), 5 )

IX = ICHW(ISZ) * LEN

IY = ICHH(ISZ)

IRT = MOD(IROT - 1, 4) + 1

IRRT = MOD( 12 - 2 * IRT , 8 )

ISX = IABS(IRT - 3) - 1

ISY = IABS(IRT - 2) - 1

IXL = ISX * IX - ISY * IY + IXCUR

IYL = ISX * IY + ISY * IX + IYCUR

INX = IXL .GE. IXMINF .AND. IXL .LE. IXMAXF

INY = IYL .GE. IYMINF .AND. IYL .LE. IYMAXF

WHEN ( INX .AND. INY )

.  IXCUR = IXL + ISY * IY

.  IYCUR = IYL - ISX * IY

.  MINLEN = MIN0( LEN, 80 )

.  WHEN ( .NOT. VISUAL )

.  .  ENCODE(4,3,MES) IRT, (ISZ+2)/2, IPLS(MOD(ISZ,2)+1)

.  .  DO ( I = 1, MINLEN ) MES(I + 4 ) = STR(I)

.  .  MES( MINLEN + 5 ) = 95

.  .  NUMCH = MINLEN + 5

.  ...FIN

.  ELSE

.  .  MES(1) = 27

.  .  MES(2) = 47

.  .  MES(3) = 48

.  .  MES(4) = 100

.  .  MES(5) = 31

.  .  MES(6) = 27

.  .  WHEN ( ISZ .EQ. 1 ) MES(7) = '0'

.  .  ELSE                MES(7) = '9'

.  .  MES(8) = 27

.  .  MES(9) = INUM(ISZ)

.  .  MES(10) = 27

.  .  MES(11) = 47

.  .  MES(12) = IRRT + 48

.  .  MES(13) = 101

.  .  DO ( I = 1, MINLEN ) MES(I + 13) = STR(I)

.  .  NUMCH = MINLEN + 13

.  ...FIN

.  CALL WRITCH(MES, NUMCH)

...FIN

PAGE 85

ELSE

.  WHEN ( LUNPLT .EQ. ITTNUM )

.  .  CALL PLTOFF

.  .  CALL DMPPLT

.  .  TYPE 1, (STR(I), I = 1, LEN)

.  .  TYPE 2

.  .  CALL PLTON

.  ...FIN

.  ELSE

.  .  TYPE 1, (STR(I), I = 1, LEN)

.  .  TYPE 2

.  ...FIN

...FIN

C

RETURN

END

PAGE 86

APPENDIX C - GRINNELL ROUTINES

SUBROUTINE CIRCLE

SUBROUTINE COLTYP

SUBROUTINE DMPPLT

SUBROUTINE ENDPLT

SUBROUTINE ERASE

SUBROUTINE INIPLT

SUBROUTINE MARKER

SUBROUTINE PLOT

SUBROUTINE PLTOFF

SUBROUTINE PLTON

SUBROUTINE WRTSTR

PAGE 87

SUBROUTINE CIRCLE( IX, IY, IR )

PIN = .3141592654

CALL PLOT( IX+IR, IY, 0 )

R = IR + .5

DO 10 I = 1, 20

IXS = IX + R * COS( PIN * I )

IYS = IY + R * SIN( PIN * I )

CALL PLOT( IXS, IYS, 1 )

10    CONTINUE

C

RETURN

END

PAGE 88

SUBROUTINE COLTYP (ITYP)

C

C     THIS SUBROUTINE SETS THE LINE TYPE

C

COMMON/COLOR/ISC

C

ITYPE = MIN0(ITYP, 8)

ITYPE = MAX0(ITYPE, 0)

ISC = ITYPE + 1

RETURN

END

PAGE 89

SUBROUTINE DMPPLT

CALL GRSBFD

RETURN

END

PAGE 90

SUBROUTINE ENDPLT

CALL GRSEND

RETURN

END

PAGE 91

SUBROUTINE ERASE

C

C     ERASES THE ENTIRE SCREEN AND SETS IT TO THE BACKGROUND COLOR

C

CALL GRFER("7777, "7777, 0)

RETURN

END

PAGE 92

SUBROUTINE INIPLT(IUNIT,X,Y)

COMMON /PAGSIZ/ XBOND, YBOND

COMMON /HFRAME/ IXMINF, IXMAXF, IYMINF, IYMAXF

COMMON /HSCALE/ XSLOPE, XCONST, YSLOPE, YCONST

COMMON /HWINDO/ SCRX,SCRY,IXORIG,IYORIG,IASCR,IBSCR,ICSCR,IDSCR

COMMON /WORLD/ XMINW, XMAXW, YMINW, YMAXW

COMMON /CHRSIZ/ ICHW(5), ICHH(5)

DATA IASCR, IBSCR, ICSCR, IDSCR, IXORIG, IYORIG, XCONST, YCONST

1    / 6 * 0, 2 * 0. /

INTEGER DATA(8)

COMMON/GRBUFR/IBUFL,IBUF(1000)

DATA DATA,IBUFL/512,4095,15,111,255,240,3840,2309,1000/

C

CALL GRSINI

CALL GRFER("7777,"7777,0)

CALL GRLWR(1,DATA,0,8,0)

C

SCRX = 50.

SCRY = 50.

C

ICHW(1) = 7

ICHW(2) = 7

ICHW(3) = 14

ICHW(4) = 14

ICHW(5) = 14

ICHH(1) = 9

ICHH(2) = 9

ICHH(3) = 18

ICHH(4) = 18

ICHH(5) = 18

C

XBOND = X

YBOND = Y

XMINW = 0.

XMAXW = 1.

YMINW = 0.

YMAXW = 1.

C

CALL FRAME( 0., X, 0., Y )

CALL WINDOW( .2 * X, .8 * X, .2 * Y, .8 * Y )

C

RETURN

END

PAGE 93

SUBROUTINE MARKER( ISYM, ISIZE )

INTEGER MARK(5)

DATA MARK / 1, 2, 3, 4, 6 /

C

ISZ = MAX0( MIN0(ISIZE, 5), 1 )

MRK = MIN0( MAX0(ISYM, 0), 5 )

IW = MARK(ISZ)

CALL CURPOS( IX, IY )

IF( MRK .NE. 0. ) GOTO 100

CALL PLOT( IX-IW, IY, 0 )

CALL PLOT( IX+IW, IY, 1 )

CALL PLOT( IX, IY-IW, 0 )

CALL PLOT( IX, IY+IW, 1 )

RETURN

100   IF( MRK .NE. 1 ) GOTO 200

CALL PLOT( IX-IW, IY-IW, 0 )

CALL PLOT( IX+IW, IY+IW, 1 )

CALL PLOT( IX+IW, IY-IW, 0 )

CALL PLOT( IX-IW, IY+IW, 1 )

RETURN

200   IF( MRK .NE. 2 ) GOTO 300

CALL PLOT( IX-IW, IY-IW, 0 )

CALL PLOT( IX+IW, IY-IW, 1 )

CALL PLOT( IX+IW, IY+IW, 1 )

CALL PLOT( IX-IW, IY+IW, 1 )

CALL PLOT( IX-IW, IY-IW, 1 )

RETURN

300   IF( MRK .NE. 3 ) GOTO 400

CALL CIRCLE( IX, IY, IW )

RETURN

400   IF( MRK .NE. 4 ) GOTO 500

CALL PLOT( IX-IW, IY-IW, 0 )

CALL PLOT( IX+IW, IY-IW, 1 )

CALL PLOT( IX, IY+IW, 1 )

CALL PLOT( IX-IW, IY-IW, 1 )

RETURN

500   CALL PLOT( IX-IW, IY-IW, 0 )

CALL PLOT( IX+IW, IY+IW, 1 )

CALL PLOT( IX-IW, IY+IW, 1 )

CALL PLOT( IX+IW, IY-IW, 1 )

CALL PLOT( IX-IW, IY-IW, 1 )

C

RETURN

END

PAGE 94

SUBROUTINE PLOT(IXO ,IYO ,IP)

COMMON /HFRAME/ IXMINF, IXMAXF, IYMINF, IYMAXF

COMMON /HCURPO/ IXCUR, IYCUR

C

COMMON/COLOR/ISC

INTEGER IDATA(4)

IX = IXO

IY = IYO

C

C     CHECK TO SEE IF IN BOUNDS

C

IF( IX .LT. IXMINF ) IX = IXMINF

IF( IX .GT. IXMAXF ) IX = IXMAXF

IF( IY .LT. IYMINF ) IY = IYMINF

IF( IY .GT. IYMAXF ) IY = IYMAXF

C

C     SAVE CURRENT COORDINATES

C

200   IXCUR = IX

IYCUR = IY

C

C     PLOT POINT

C

IDATA(3)=IX

IDATA(4)=IY

IF(IP.EQ.1)CALL GRFVL(1,ISC,0,1,IDATA,2)

IDATA(1)=IX

IDATA(2)=IY

RETURN

END

PAGE 95

SUBROUTINE PLTOFF

RETURN

END

PAGE 96

SUBROUTINE PLTON

RETURN

END

PAGE 97

SUBROUTINE WRTSTR(STR, LEN, IROT, ISIZE)

C

C     SUBROUTINE "WRITE STRING"

C     LEN IS THE LENGTH OF THE STRING

C     IROT DETERMINES THE ROTATION OF THE STRING (1 - 4), 1 IS

C     RIGHT SIDE UP, 2 IS ROTATED 90 DEGREES CLOCKWISE, ETC.

C     STRING IS CHECKED TO SEE IF IN IS IN BOUNDS

C

COMMON /HFRAME/ IXMINF, IXMAXF, IYMINF, IYMAXF

COMMON /HCURPO/ IXCUR, IYCUR

COMMON /CHRSIZ/ ICHW(5), ICHH(5)

C

BYTE STR(LEN)

BYTE CHARB(886)

INTEGER CHAR(223), LENGTH(223), MATRIX(8)

DATA CHARB/

1 "370,"220,"160," 20,"340,"210,"210,"210,"370,"210,"210,"160,

1 "360,"210,"210,"360,"200,"200,"200,"360,"210,"210,"360,"210,

1 "210,"360,"170,"200,"200,"200,"170,"160,"210,"200,"200,"200,

1 "210,"160,"170,"210,"210,"170," 10," 10," 10,"360,"210,"210,

1 "210,"210,"210,"360,"170,"200,"370,"210,"160,"370,"200,"200,

1 "360,"200,"200,"370,"100,"100,"100,"340,"100,"110," 60,"200,

1 "200,"200,"360,"200,"200,"370,"160,"210," 10,"170,"210,"210,

1 "170,"160,"210,"270,"200,"200,"210,"160,"210,"210,"210,"360,

1 "200,"200,"200,"210,"210,"210,"370,"210,"210,"210,"160," 40,

1 " 40,"140," 00," 40,"370," 40," 40," 40," 40," 40,"370,"140,

1 "220," 20," 20," 20," 20," 00," 20,"140,"220," 20," 20," 20,

1 " 20," 70,"210,"220,"340,"240,"220,"200,"200,"210,"220,"240,

1 "300,"240,"220,"210,"340,"100,"100,"100,"100,"100,"300,"370,

1 "200,"200,"200,"200,"200,"200,"210,"250,"250,"370,"220,"210,

1 "210,"210,"250,"250,"330,"210,"210,"210,"210,"310,"260,"210,

1 "230,"230,"250,"310,"310,"210,"160,"210,"210,"210,"160,"160,

1 "210,"210,"210,"210,"210,"160,"200,"200,"200,"360,"210,"210,

1 "360,"200,"200,"200,"360,"210,"210,"360," 10," 10," 10,"170,

1 "210,"210,"170,"150,"220,"250,"210,"210,"210,"160,"200,"200,

1 "200,"310,"260,"210,"220,"240,"360,"210,"210,"360,"360," 10,

1 "160,"200,"170,"160,"210," 10,"160,"200,"210,"160," 60,"110,

1 "100,"100,"340,"100,"100," 40," 40," 40," 40," 40," 40,"370,

1 "170,"210,"210,"210,"210,"160,"210,"210,"210,"210,"210,"210,

1 " 40,"120,"210,"210,"210," 40," 40,"120,"120,"210,"210,"210,

1 "210,"330,"250,"210,"210,"210,"330,"250,"250,"210,"210,"210,

1 "210,"120," 40,"120,"210,"210,"210,"120," 40,"120,"210,"210,

1 "160,"210," 10,"170,"210,"210,"210," 40," 40," 40," 40,"120,

1 "210,"210,"370,"100," 40," 20,"370,"370,"200,"100," 40," 20,

1 " 10,"370,"370,"210,"210,"120,"120," 40," 40," 40," 00," 40,

1 " 40," 40," 40," 40,"120,"120,"120,"120,"120,"370,"120,"370,

1 "120,"120," 40,"360," 50,"160,"240,"170," 40,"210,"200,"100,

1 " 40," 20," 10,"210,"170,"220,"250,"140,"120,"120," 40,"100,

1 " 40," 40," 40,"100,"100,"100,"100,"100,"100,"100," 40,"100,

1 " 40," 40," 40," 40," 40," 40," 40,"100,"120," 40,"370," 40,

1 "120," 40," 40,"370," 40," 40,"100," 40," 40,"370," 40,"200,

1 "100," 40," 20," 10,"160,"210,"310,"250,"230,"210,"160,"160,

1 " 40," 40," 40," 40,"140," 40,"370,"100," 40," 20," 10,"210,

PAGE 98

1 "160,"160,"210," 10," 60," 10,"210,"160," 20," 20,"370,"220,

1 "120," 60," 20,"160,"210," 10," 10,"360,"200,"370,"160,"210,

1 "210,"360,"200,"210,"160,"100,"100,"100," 40," 20," 10,"370,

1 "160,"210,"210,"160,"210,"210,"160,"160,"210," 10,"170,"210,

1 "210,"160," 40," 00," 00," 40,"100," 40," 40," 00," 40," 20,

1 " 40,"100," 40," 20,"370," 00," 00,"370,"100," 40," 20," 40,

1 "100," 40," 00," 40," 20," 10,"210,"160,"170,"200,"270,"250,

1 "270,"210,"160,"160,"100,"100,"100,"100,"100,"100,"100,"160,

1 " 10," 20," 40,"100,"200,"160," 20," 20," 20," 20," 20," 20,

1 " 20,"160,"210,"120," 40,"377," 40,"100,"100," 70," 20,"174,

1 "222,"222,"222,"174," 20," 70,"314,"110,"110,"204,"204,"204,

1 "170,"374,"100," 40," 20," 10," 20," 40,"100,"374,"340,"100,

1 "100,"100,"100,"100,"100,"104,"374,"356,"104,"104," 50," 50,

1 " 20," 20,"204,"374,"204," 00,"170," 00,"204,"374,"204," 70,

1 " 20," 20," 70,"124,"124,"124,"222," 70," 70," 20," 20," 20,

1 " 20," 50," 50,"252,"104,"356,"104,"104,"104,"104,"104,"104,

1 "104,"376,"170,"204,"200,"200,"160,"200,"204,"170,"170,"204,

1 "204,"204,"264,"204,"204,"170,"140,"220,"220," 20," 20," 20,

1 " 20," 20," 20," 20," 20," 20," 20," 20," 20," 20," 20," 20,

1 " 20," 20," 20," 20," 20," 20," 20," 20," 20," 22," 22," 14,

1 " 20," 20," 20," 20," 20," 20," 20," 20," 20,"154,"220,"220,

1 "220,"220,"150,"200,"200,"360,"210,"210,"360,"210,"210,"160,

1 "140," 20,"160,"200,"200,"160,"100,"100," 70,"100,"160,"210,

1 "210,"160," 40,"100,"100," 70,"160,"200,"200,"340,"200,"200,

1 "160,"100,"100,"160,"250,"250,"160," 20," 20," 40," 40," 20,

1 " 20," 30,"224,"144," 04," 04," 04," 44," 44," 44,"244,"130,

1 "140,"220,"200,"100,"100," 40,"104,"110," 50," 60,"250,"144,

1 "210,"210,"110,"120," 60," 40," 40,"100,"200,"200,"350,"220,

1 "220,"220,"220,"220,"300,"240,"220,"110,"110,"310,"140,"220,

1 "210,"210,"110," 60," 40," 50," 50," 50,"250,"170,"140,"220,

1 "220,"360,"220,"220,"140,"200,"200,"200,"340,"220,"220,"220,

1 "140,"160,"210,"210,"210,"210,"174," 40," 40," 40," 40,"240,

1 "170," 30," 44," 44," 44,"244,"144,"154,"272,"222,"222,"202,

1 "104,"114," 52," 20," 20,"250,"144," 40," 40,"160,"250,"250,

1 "250," 40," 40,"140,"220," 20,"140,"200,"200,"100,"170,"200,

1 " 30," 40," 40," 40,"100," 40," 40," 40," 30,"300," 40," 40,

1 " 40," 20," 40," 40," 40,"300," 20,"250,"100,"  0

1 /

DATA CHAR/

1 30 * 886,

1 886,886,346,353,356,363,370,377,384,387,396,405,410,415,418,

1 419,420,425,432,439,446,453,460,467,474,481,488,495,499,504,

1 509,513,518,525,006,020,032,046,058,072,086,100,113,128,142,

1 156,168,180,192,206,220,232,244,258,270,282,294,306,320,332,

1 532,541,546,555,558,886,001,013,027,039,053,065,079,093,107,

1 120,135,149,163,175,187,199,213,227,239,251,265,277,289,301,

1 313,327,865,685,874,883,886,886,886,886,886,886,886,886,886,

1 15 * 886,

1 886,886,886,886,886,886,886,886,886,886,886,886,603,339,639,

1 562,587,886,886,655,886,596,886,886,886,630,647,886,578,886,

1 621,886,571,886,612,886,886,886,886,886,886,559,694,700,709,

1 719,727,734,742,749,757,886,763,769,777,785,791,797,803,810,

1 818,824,830,886,836,842,848,856,886,886,886,886,886

PAGE 99

1 /

DATA LENGTH/

1 4122,4122,4122,-3094,3*4122,-10256,12*4122,-3102,9*4122,

1 4122,4122,4218,4158,4218,4218,4218,4218,4158,4248,

1 4248,4187,4187,4154,4125,4122,4187,4218,4218,4218,

1 4218,4218,4218,4218,4218,4218,4218,4171,4186,4187,

1 4171,4187,4218,4218,4218,4218,4218,4218,4218,4218,

1 4218,4218,4218,4218,4218,4218,4218,4218,4218,4218,

1 4218,4218,4218,4218,4218,4218,4218,4218,4218,4218,

1 4248,4187,4248,4156,4122,4122,4186,4218,4186,4218,

1 4186,4218,4216,4218,4202,4232,4218,4218,4186,4186,

1 4186,4216,4216,4186,4186,4218,4186,4186,4186,4186,

1 4216,4186,4248,4248,4248,4156,4122,4122,4122,4122,

1 4122,4122,4122,4122,4122,4122,4122,4122,4122,4122,

1 20*4122,

1 4122,4122,4250,4218,4234,4250,4250,4122,4122,4576,

1 4122,4218,4122,4122,4122,4250,4234,4122,4250,4122,

1 4250,4122,4218,4122,4250,4122,4122,4122,4122,4122,

1 4122,4158,4202,4248,4264,4234,4218,4234,4218,4232,

1 4202,0026,4202,4234,4232,4202,4202,4202,4218,4232,

1 4202,4202,4202,4122,4202,4202,4234,4248,4122,4122,

1 4122,4122,4122

1 /

C

C     CHECK TO SEE IF IN BOUNDS

C

ISZ = MAX0(ISIZE, 1)

ISZ = MIN0(ISZ, 5)

ID = 0

IF ( ISZ .GE. 3 ) ID = 1

LEN1 = LEN

DO 5 I = 1, LEN

IF ( STR(I) .EQ. 96 ) LEN1 = LEN1 - 1

IF ( STR(I) .LE. 31 ) LEN1 = LEN1 - 1

IF ( STR(I) .EQ. 8 ) LEN1 = LEN1 - 1

5     CONTINUE

IXW = ICHW(ISZ) * LEN1

IYW = ICHH(ISZ)

IRT = MOD(MAX0(IROT - 1, 0), 4) + 1

ISX = IABS(IRT - 3) - 1

ISY = IABS(IRT - 2) - 1

IXL = ISX * IXW - ISY * IYW + IXCUR

IYL = ISX * IYW + ISY * IXW + IYCUR

IF( IXL .LT. IXMINF .OR. IXL .GT. IXMAXF ) GOTO 100

IF( IYL .LT. IYMINF .OR. IYL .GT. IYMAXF ) GOTO 100

C

C     PLOT STRING

C

IX = IXCUR

IY = IYCUR

IESC = 0

LNX = MOD ( ( ID + 1 ) * ISX + 1024, 1024 )

LNY = MOD ( ( ID + 1 ) * ISY + 1024, 1024 )

PAGE 100

DO 20 I = 1 , LEN

NCHAR = STR(I)

NCHAR = NCHAR .AND. "377

NCHAR = NCHAR + IESC

IESC = 96

IF ( NCHAR .EQ. 96 ) GOTO 20

IESC = 0

N = LENGTH( NCHAR )

IOFF = ( MOD( IABS(N) , 16 ) - 10 ) * ( ID + 1 )

IDX = N / 16

N = MOD( IABS(IDX) , 64 )

ISTR = CHAR( NCHAR )

JJ = 0

IXL = IX - IOFF * ISY

IYL = IY + IOFF * ISX

DO 15 II = 1, N

NCHAR = CHARB(II + ISTR - 1) .AND. "377

DO 10 J = 1, 8

NTWO = 2 ** ( 8 - J )

MATRIX(J) = NCHAR / NTWO

NCHAR = MOD ( NCHAR , NTWO )

10       CONTINUE

CALL GRWDW(MATRIX,IXL,IYL,LNX,LNY,8,ID,ID,1,0)

IXL = IXL - ISY * ( ID + 1 )

IYL = IYL + ISX * ( ID + 1 )

15       CONTINUE

INC = ( 3 + IDX / 64 ) * ( ID + 1 )

C

IF ( INC .NE. 0 ) GOTO 17

IX = IX - IOFF * ISY

IY = IY + IOFF * ISX

C

17       IX = IX + INC * ISX

IY = IY + INC * ISY

20    CONTINUE

IXCUR = IX + ISY * IYW

IYCUR = IY - ISX * IYW

RETURN

C

C     STRING IS OUT OF BOUNDS

C

100   TYPE 1, (STR(I), I = 1, LEN)

TYPE 2

C

RETURN

1     FORMAT('$WARNING  string ', 100a1)

2     FORMAT('  is out of bounds and will not be plotted')

END

PAGE 101

APPENDIX D - QUICK REFERENCE GUIDE

SUBROUTINE ALLOUT( ix, iy, iud )

SUBROUTINE AXIS( xtic, ytic, ixtit, ixlen, ixsiz, ixfmt,

iytit, iylen, iysiz, iyfmt )

SUBROUTINE CIRCLE( ix, iy, ir )

SUBROUTINE COLTYP( icol )

SUBROUTINE CONLGN( nl, title, n, icol, isym, inum, lintyp,

xstrt, xlen, ystrt )

SUBROUTINE CURPOS( icurx, icury )

SUBROUTINE CURSON

SUBROUTINE DASHLN( x, y, n, icol, isym, isize, inum, lintyp )

SUBROUTINE DEFWIN( xmin, xmax, ymin, ymax )

SUBROUTINE DMPPLT

SUBROUTINE DRDASH( ix, iy, iud )

SUBROUTINE DRWREC( xmin, xmax, ymin, ymax, icol )

SUBROUTINE DRXTIC( ix, iy, lentic )

SUBROUTINE DRYTIC( ix, iy, lentic )

SUBROUTINE ENDLGN

SUBROUTINE ENDPLT

SUBROUTINE ERASE

SUBROUTINE FNDCUR( ix, iy )

SUBROUTINE FNDINT( ix0, iy0, ix, iy )

SUBROUTINE FRAME( xminf, xmaxf, yminf, ymaxf )

SUBROUTINE GETSTR( ix, iy, str, len, irot, isize )

FUNCTION IFRMT( form, ifmt )

FUNCTION ISCRX( xwrld )

FUNCTION ISCRY( ywrld )

SUBROUTINE INILGN( xmin, xmax, ymin, ymax )

SUBROUTINE INIPLT( iunit, xsize, ysize )

SUBROUTINE INSECT( i1, j1, iplot, iplot0, x, y, nx, ny, l )

SUBROUTINE LABELX( xval, form, iform, is, iy, ichw, ixsiz )

SUBROUTINE LABELY( yval, form, iform, is, iy, ichw, iysiz )

SUBROUTINE LINE( x, y, n, icol, isym, isize, inum )

SUBROUTINE MARKER( isym, isize )

SUBROUTINE MOVETO( xwrld, ywrld, iud, lintyp )

SUBROUTINE MOV1ST( xwrld, ywrld, iud, lintyp )

SUBROUTINE PENDWN

SUBROUTINE PENUP

SUBROUTINE PLOT( ix, iy, iud )

SUBROUTINE PLOT3D( f, x, y, nx, ny, xw, yw, xv, yv, zv, l, s )

SUBROUTINE PLTOFF

SUBROUTINE PLTON

PAGE 102

SUBROUTINE PLTSTR( ix, iy, str, len, irot, isize )

SUBROUTINE PUTCUR( ix, iy )

SUBROUTINE RECLGN( nl, title, n, icol, isym, inum, lintyp,

xstrt, xlen, ystrt )

SUBROUTINE REDSTR( ix, iy, str, len, irot, isize )

SUBROUTINE SCALE( xmin, xmax, ymin, ymax )

SUBROUTINE TRIML( label, n, len )

SUBROUTINE TRIMSP( title, len, ist, iend )

SUBROUTINE VUPORT( xmin, xmax, ymin, ymax )

SUBROUTINE WINDOW( xwin0, xwin1, ywin0, ywin1 )

SUBROUTINE WRILGN( title, n, icol, isym, inum, lintyp )

SUBROUTINE WRITCH( ch, n )

SUBROUTINE WRTSTR( str, len, irot, isize )

FUNCTION XWORLD( ix )

SUBROUTINE XAXIS( xtic, ixtit, ixlen, ixsiz, ixfmt )

FUNCTION YWORLD( iy )

SUBROUTINE YAXIS( ytic, iytit, iylen, iysiz, iyfmt )

PAGE 103

INDEX

ALLOUT . . . . . . . . . . . . . . . 21, 28

AXIS . . . . . . . . . . . . . . . . 2, 3, 7, 10, 13, 18, 20, 29

CIRCLE . . . . . . . . . . . . . . . 16, 62, 87

COLTYP . . . . . . . . . . . . . . . 22, 63, 88

CONLGN . . . . . . . . . . . . . . . 14, 64

coordinates, global  . . . . . . . . 1

coordinates, screen  . . . . . . . . 1

coordinates, world . . . . . . . . . 1

CURPOS . . . . . . . . . . . . . . . 15, 30

CURSON . . . . . . . . . . . . . . . 19, 65

cursor . . . . . . . . . . . . . . . 19

DASHLN . . . . . . . . . . . . . . . 9, 10, 12, 13, 15, 21, 31

DEFWIN . . . . . . . . . . . . . . . 19, 66

DMPPLT . . . . . . . . . . . . . . . 5, 67, 89

DRDASH . . . . . . . . . . . . . . . 21, 32

DRWREC . . . . . . . . . . . . . . . 16, 34

DRXTIC . . . . . . . . . . . . . . . 20, 35

DRYTIC . . . . . . . . . . . . . . . 20, 36

ENDLGN . . . . . . . . . . . . . . . 12, 13, 37

ENDPLT . . . . . . . . . . . . . . . 2, 3, 5, 10, 13, 23, 68, 90

ERASE  . . . . . . . . . . . . . . . 16, 69, 91

examples . . . . . . . . . . . . . . 2, 10, 13, 23

FNDCUR . . . . . . . . . . . . . . . 19, 70

FNDINT . . . . . . . . . . . . . . . 21, 38

FRAME  . . . . . . . . . . . . . . . 6, 9, 10, 39

GETSTR . . . . . . . . . . . . . . . 18, 71

GRINNELL . . . . . . . . . . . . . . 4, 8, 9, 16, 25

GRLIB  . . . . . . . . . . . . . . . 25

GRPLOT . . . . . . . . . . . . . . . 25

GRX  . . . . . . . . . . . . . . . . 25

HGRAPH . . . . . . . . . . . . . . . 25

HIPLOT . . . . . . . . . . . . . . . 4, 7, 9, 14, 16, 17, 22, 25, 26

HIX  . . . . . . . . . . . . . . . . 25

icol . . . . . . . . . . . . . . . . 8

IFRMT  . . . . . . . . . . . . . . . 20, 40

INILGN . . . . . . . . . . . . . . . 12, 13, 41

INIPLT . . . . . . . . . . . . . . . 2, 3, 4, 5, 6, 9, 10, 13, 16, 23, 72, 92

initializing plots . . . . . . . . . 4

INSECT . . . . . . . . . . . . . . . 21

inum . . . . . . . . . . . . . . . . 8

ISCRX  . . . . . . . . . . . . . . . 15, 42

ISCRY  . . . . . . . . . . . . . . . 15, 43

PAGE 104

isize  . . . . . . . . . . . . . . . 8

isym . . . . . . . . . . . . . . . . 8

iunit  . . . . . . . . . . . . . . . 4

ixfmt  . . . . . . . . . . . . . . . 8

ixlen  . . . . . . . . . . . . . . . 7

ixsiz  . . . . . . . . . . . . . . . 7

ixtit  . . . . . . . . . . . . . . . 7

iyfmt  . . . . . . . . . . . . . . . 8

iylen  . . . . . . . . . . . . . . . 8

iysiz  . . . . . . . . . . . . . . . 8

iytit  . . . . . . . . . . . . . . . 8

LABELX . . . . . . . . . . . . . . . 20, 44

LABELY . . . . . . . . . . . . . . . 20, 45

legend   . . . . . . . . . . . . . . 12

LINE . . . . . . . . . . . . . . . . 2, 3, 9, 46

lintyp . . . . . . . . . . . . . . . 9

MARKER . . . . . . . . . . . . . . . 22, 74, 93

MOV1ST . . . . . . . . . . . . . . . 21, 48

MOVETO . . . . . . . . . . . . . . . 15, 47

nominal inch . . . . . . . . . . . . 1

PENDWN . . . . . . . . . . . . . . . 22, 76

PENUP  . . . . . . . . . . . . . . . 22, 77

PLOT . . . . . . . . . . . . . . . . 22, 78, 94

PLOT3D . . . . . . . . . . . . . . . 21, 23

plotting frame . . . . . . . . . . . 1

plotting page  . . . . . . . . . . . 1

plotting window  . . . . . . . . . . 1

PLTOFF . . . . . . . . . . . . . . . 17, 79, 95

PLTON  . . . . . . . . . . . . . . . 17, 80, 96

PLTSTR . . . . . . . . . . . . . . . 18, 49

PUTCUR . . . . . . . . . . . . . . . 19, 81

RECLGN . . . . . . . . . . . . . . . 14, 50

REDSTR . . . . . . . . . . . . . . . 18, 82

running a program  . . . . . . . . . 25

SCALE  . . . . . . . . . . . . . . . 2, 3, 6, 7, 10, 13, 16, 21, 51

terminating plots  . . . . . . . . . 4

three dimensional plots  . . . . . . 23

TRIML  . . . . . . . . . . . . . . . 21, 52

TRIMSP . . . . . . . . . . . . . . . 21, 53

VISUAL . . . . . . . . . . . . . . . 4, 7, 9, 14, 16, 17, 22, 25, 26

VIX  . . . . . . . . . . . . . . . . 25

VUPORT . . . . . . . . . . . . . . . 21, 54

WINDOW . . . . . . . . . . . . . . . 6, 7, 9, 10, 13, 16, 55

WRILGN . . . . . . . . . . . . . . . 12, 13, 56

PAGE 105

WRITCH . . . . . . . . . . . . . . . 22, 83

WRTSTR . . . . . . . . . . . . . . . 18, 84, 97

XAXIS  . . . . . . . . . . . . . . . 20, 57

xmax . . . . . . . . . . . . . . . . 6

xmaxf  . . . . . . . . . . . . . . . 6

xmin . . . . . . . . . . . . . . . . 6

xminf  . . . . . . . . . . . . . . . 6

xsize  . . . . . . . . . . . . . . . 4

xtic . . . . . . . . . . . . . . . . 7

xwin0  . . . . . . . . . . . . . . . 6

xwin1  . . . . . . . . . . . . . . . 6

XWORLD . . . . . . . . . . . . . . . 15, 58

YAXIS  . . . . . . . . . . . . . . . 20, 59

ymax . . . . . . . . . . . . . . . . 6

ymaxf  . . . . . . . . . . . . . . . 6

ymin . . . . . . . . . . . . . . . . 6

yminf  . . . . . . . . . . . . . . . 6

ysize  . . . . . . . . . . . . . . . 4

ytic . . . . . . . . . . . . . . . . 7

ywin0  . . . . . . . . . . . . . . . 6

ywin1  . . . . . . . . . . . . . . . 6

YWORLD . . . . . . . . . . . . . . . 15, 60