DEC USER SOCIETY (DECUS) SYMPOSIUM PRESENTATION

#

_______________________________________________

I/D SPACE APL LANGUAGE INTERPRETER ON RSX-11M-PLUS

#

__________________________________________________

1985 SPRING DECUS U.S. SYMPOSIUM

NEW ORLEANS, LOUSIANA

MAY 27-31, 1985

BY BOB AWDE JR.

GENERAL MILLS, INC.

MINNEAPOLIS, MINNESOTA

PRESENTATION OUTLINE

#

____________________

1. PROBLEM STATEMENT AND SOLUTION

2. HARDWARE AND SOFTWARE REQUIREMENTS

3. DEFINITION OF A I/D SPACE TASK

4. DEFINITION OF SUPERVISOR MODE LIBRARIES

5. RE-BUILDING APL USING I/D SPACE AND

SUPERVISOR-MODE LIBRARIES

6. COMPARISONS BETWEEN THE ORIGINAL VERSION

OF APL AND THE I/D SPACE VERSION

7. PROBLEMS ENCOUNTERED IN THE CONVERSION

PROCESS

8. APL CHARACTER SET FOR VT200 SERIES TERMINALS

9. PROGRAM AVAILABILIITY

10. QUESTIONS AND ANSWERS

THE PROBLEM:

#

____________

WE HAVE DEVELOPED A SET OF TECHNICAL PROGRAMS THAT COULD

BE EFFICIENTLY PROGRAMMED IN APL. HOWEVER, THE SEVERE WORKSPACE

LIMITATIONS OF APL-11 RESTRICTED US FROM IMPLEMENTING THESE

PROGRAMS IN APL.

THE SOLUTION:

#

_____________

AFTER SUCCESSFULLY UPGRADING OUR OPERATING SYSTEM FROM

RSX-11M TO RSX-11M-PLUS AND OUR CPU FROM A PDP-11/23 TO A PDP-11/73,

THE POSSIBILITY EXISTED TO RE-BUILD APL-11 TO USE BOTH INSTRUCTION/

DATA (I/D) SPACE AND SUPERVISOR-MODE LIBRARIES.

GENERAL HARDWARE AND SOFTWARE REQUIREMENTS:

#

___________________________________________

THE FOLLOWING HARDWARE SUPPORTS BOTH I/D SPACE AND

SUPERVISOR-MODE LIBRARIES:

1. PDP-11/70

2. PDP-11/44

3. PDP-11/73

4. PDP-11/84

THE FOLLOWING SOFTWARE SUPPORTS BOTH I/D SPACE AND

SUPERVISOR-MODE LIBRARIES:

1. RSX-11M-PLUS V2.1

2. RSTS/E V9.0 ?

DEFINITION  OF AN I/D SPACE TASK:

#

_________________________________

ON ALL PDP-11 CPUS WITH MEMORY MANAGEMENT, THERE ARE

EIGHT ADDRESS PAGE REGISTERS (APR'S) CAPABLE OF ADDRESSING UP

TO 64KB OF VIRTUAL ADDRESS SPACE.

ON THE CPUS MENTIONED ABOVE (PDP-11/44, PDP-11/70S,

PDP-11/84), THERE EXISTS ANOTHER SET OF EIGHT DATA PAGE

REGISTERS (DPRS) THAT CAN MAP UP TO 64KB OF VIRTUAL DATA SPACE.

THE BENEFIT OF HAVING DPRS IS THAT PROGRAMS COULD BE POTENTIALLY

TWICE AS LARGE AS A NORMAL PROGRAM RUNNING ON OTHER PDP-11S. HOWEVER,

THE I-SPACE APRS CAN ONLY MAP INSTRUCTIONS WHILE THE D-SPACE DPRS CAN

ONLY MAP DATA.

THESE TWO CONFIGURATIONS ARE DIAGRAMMED BELOW:

ALL PDP-11S			PDP-11/44,70S,84

#

___________                        ________________

ALL SPACE			       I-SPACE          D-SPACE

________			       _______          _______

APR0 |________|			APR0  |_______|   DPR0 |_______|

APR1 |________|			APR1  |_______|   DPR1 |_______|

APR2 |________|			APR2  |_______|   DPR2 |_______|

APR3 |________|			APR3  |_______|   DPR3 |_______|

APR4 |________|			APR4  |_______|   DPR4 |_______|

APR5 |________|			APR5  |_______|   DPR5 |_______|

APR6 |________|			APR6  |_______|   DPR6 |_______|

APR7 |________|			APR7  |_______|   DPR7 |_______|

DEFINITION OF SUPERVISOR-MODE LIBRARIES

#

_______________________________________

ON ALL PDP-11S WITH MEMORY MANAGEMENT, BOTH KERNAL AND USER

MODE ARE SUPPORTED. USER MODE CONSISTS OF A FULL SET OF CPU REGISTERS

AND ADDRESS PAGE REGISTERS (APRS) DEDICATED TO RUNNING USER PROGRAMS.

KERNAL MODE CONSISTS OF ANOTHER FULL SET OF CPU REGISTERS AND APRS

DEDICATED TO RUNNING THE OPERATING SYSTEM AND PRIVILEGED PROGRAMS.

THE USE OF SEPARATE SETS OF CPU REGISTERS AND APRS REDUCES THE TIME

IT TAKES TO PERFORM A CONTEXT SWITCH (SWITCHING BETWEEN USER PROGRAMS

AND THE OPERATING SYSTEM). THIS CONFIGURATION IS DIAGRAMMED BELOW:

ALL PDP-11S

#

___________

USER MODE			      KERNAL MODE

________			        _______

R0 |________|			   R0  |_______|

R1 |________|			   R1  |_______|

R2 |________|			   R2  |_______|

R3 |________|			   R3  |_______|

R4 |________|			   R4  |_______|

R5 |________|			   R5  |_______|

R6 |________|			   R6  |_______|

R7 |________|			   R7  |_______|

________			        _______

APR0 |________|			 APR0  |_______|

APR1 |________|			 APR1  |_______|

APR2 |________|			 APR2  |_______|

APR3 |________|			 APR3  |_______|

APR4 |________|			 APR4  |_______|

APR5 |________|			 APR5  |_______|

APR6 |________|			 APR6  |_______|

APR7 |________|			 APR7  |_______|

ON THE PDP-11/44, 70S, AND 84, THERE EXISTS AN ADDITIONAL MODE

CALLED SUPERVISOR-MODE. RSX-11M-PLUS SUPPORTS MEMORY RESIDENT

LIBRARIES THAT BE CALLED IN SUPERVISOR MODE. THE BENEFIT IS THAT

PROGRAMS CAN POTENTIALLY BE 64KB LARGER THAN CONVENTIONAL PDP-11

PROGRAMS. SUPERVISOR-MODE LIBRARIES ARE CALLED USING A SPECIAL

INSTRUCTION SUPPORTED ON THESE PDP-11S (CALL SUPERVISOR MODE -

CSM). THE SUPERVISOR MODE SUPPORT CONFIGURATION IS DIAGRAMMED

BELOW:

USER MODE	        SUPERVISOR MODE        KERNAL MODE

________  	    _______		  _______

R0 |________|	       R0  |_______|	     R0  |_______|

R1 |________|	       R1  |_______|         R1  |_______|

R2 |________|        R2  |_______|         R2  |_______|

R3 |________|        R3  |_______|         R3  |_______|

R4 |________|        R4  |_______|         R4  |_______|

R5 |________|        R5  |_______|         R5  |_______|

R6 |________|        R6  |_______|         R6  |_______|

R7 |________|        R7  |_______|         R7  |_______|

________	            _______               _______

APR0 |________|	     APR0  |_______|	   APR0  |_______|

APR1 |________|	     APR1  |_______|	   APR1  |_______|

APR2 |________|	     APR2  |_______|	   APR2  |_______|

APR3 |________|	     APR3  |_______|	   APR3  |_______|

APR4 |________|	     APR4  |_______|	   APR4  |_______|

APR5 |________|	     APR5  |_______|	   APR5  |_______|

APR6 |________|	     APR6  |_______|	   APR6  |_______|

APR7 |________|	     APR7  |_______|	   APR7  |_______|

________	            _______               _______

DPR0 |________|	     DPR0  |_______|	   DPR0  |_______|

DPR1 |________|	     DPR1  |_______|	   DPR1  |_______|

DPR2 |________|	     DPR2  |_______|	   DPR2  |_______|

DPR3 |________|	     DPR3  |_______|	   DPR3  |_______|

DPR4 |________|	     DPR4  |_______|	   DPR4  |_______|

DPR5 |________|	     DPR5  |_______|	   DPR5  |_______|

DPR6 |________|	     DPR6  |_______|	   DPR6  |_______|

DPR7 |________|	     DPR7  |_______|	   DPR7  |_______|

RE-BUILDING APL-11 TO USE I/D SPACE AND SUPERVISOR-MODE LIBRARIES:

#

__________________________________________________________________

FORTUNATELY, THE APL WORKSPACE IS A DATA STRUCTURE AND THUS

OCCUPIES THE D-SPACE PORTION OF AN I/D SPACE TASK. THUS THE WOKSPACE

CAN EXPAND TO OCCUPY JUST ABOUT THE ENTIRE DATA SPACE AVAILABLE.

THE USE OF THE FILE CONTROL SERVICES (FCS) SUPERVISOR-MODE

LIBRARY MADE IT POSSIBLE TO RE-BUILD APL-11 WITHOUT OVERLAYING THE

INSTRUCTION SPACE (I-SPACE) PORTION OF THE TASK. THE ONLY OVERLAYING

THAT OCCURS IS IN D-SPACE WHERE ALL THE ERROR MESSAGE TEXT WERE LEFT

OVERLAYED IN ORDER TO MAXIMIZE THE SIZE OF THE WORKSPACE.

THE FOLLOWING TASK BUILD COMMAND FILE WAS USED TO RE-BUILD

APL-11:

APL/FP/CP/ID,APL/-WI/SH/-SP=APL/MP    ; /ID IS I/D SPACE TKB

;				      ; SWITCH

;

SUPLIB=FCSFSL	; LINK TO FCS SUPERVISOR-MODE LIBRARY

TASK=...APL	; OTHER OPTIONS ARE AS

STACK=256	; ORIGINALLY DEFINED

UNITS=15

ASG=SY0:5:6:13:14,TI0:15

ACTFIL=0

/

APL.ODL WAS MODIFIED TO ELIMINATE ALL I-SPACE OVERLAYS

AND REFERENCES TO SYSLIB WERE COMMENTED OUT SINCE THE

FCS SUPERVISOR-MODE LIBRARY WAS BEING USED FOR THESE

ROUTINES.

INSTALLATION INSTRUCTIONS TO MAXIMIZE THE WORKSPACE

SIZE ARE AS FOLLOWS:

MCR>INS APL/INC=64240

COMPARISONS BETWEEN THE ORIGINAL VERSION AND THE I/D SPACE VERSION:

#

___________________________________________________________________

ORIGINAL    I/D SPACE        PERCENT

CRITERIA	 	APL-11	     APL-11           CHANGE

#

________                ________    _________        _______

WORKSPACE SIZE (BYTES)	32,688	     53,548	      63.8 % INCREASE

DISK SPACE USED (BLOCKS)	   158		119	      24.7 % DECREASE

SPEED				            SLIGHTLY	   DUE TO NO OVERLAYS

FASTER

MEMORY REQUIREMENTS			    ABOUT 2 TIMES  DUE TO I/D SPACE USE

MORE MEMORY

CONSUMED

MAIN PROBLEM ENCOUNTERED DURING THE CONVERSION PROCESS:

#

_______________________________________________________

APL ESTABLISHES A LINKED LIST OF CORE ALLOCATION BLOCKS TO KEEP

TRACK OF MEMORY UTILIZATION WITHIN A WORKSPACE. ONE WORD WITHIN

THE BLOCK DEFINES THE SIZE OF THE BLOCK IN WORDS AND WHETHER THE

BLOCK IS CURRENTLY IN USE. A DIAGRAM OF AN UNUSED  CORE ALLOCATION

BLOCK LOOKS LIKE THIS:

**********

* LENGTH *	LENGTH (IN WORDS) OF THE BLOCK

**********

* FLINK  *	POINTER TO NEXT BLOCK IN LIST

**********

* BLINK  *	POINTER TO PREVIOUS BLOCK IN LIST

**********

*        *

* UNUSED *	AVAILABLE FOR APL USE

*        *

**********

* LENGTH *	LENGTH OF BLOCK (SAME AS LENGTH ABOVE)

**********

WHEN A BLOCK OF CORE IS IN USE, THE LENGTH IS SET

NEGATIVE (I.E. MOST SIGNIFICANT BIT IS SET). ALSO, THE LENGHT

VALUE IS CONVERTED TO BYTES (BY SHIFTING LEFT) AND THEN CONVERTED

BACK TO WORDS (BY SHIFTING RIGHT) MANY TIMES DURING THE EXECUTION

OF THE APL-11 INTERPRETER.

WHEN THE I/D SPACE VERSION OF APL-11 WAS CREATED, THERE

EXISTED THE POSSIBILITY THAT BIT 14 (NOTE: NUMBERING FROM 0-15)

COULD BE SET DUE TO THE INCREASE IN SIZE OF WORKSPACE. THIS

SITUATION COULD NOT HAPPEN IN THE ORIGINAL APL-11 INTERPRETER.

AS A CONSEQUENCE OF CONVERTING TO BYTES, BIT 15 COULD

SOMETIMES BECOME SET PARTICULARLY WHEN THE WORKSPACE WAS VIRTUALLY

EMPTY. THE CONVERSION BACK TO WORDS CREATED THE FOLLOWING

PROBLEM:

THE DEC PROGRAMMERS WHO IMPLEMENTED APL OPTED

TO USE THE ASR INSTRUCTION TO CONVERT FROM BYTES

TO WORDS. UNFORTUNATELY, THE ASR INSTRUCTION

LEAVES BIT 15 UNCHANGED AND THUS MISTAKENLY

COULD INDICATE THAT THE CORE BLOCK IS IN USE

WHEN IN REALITY IT IS FREE FOR USE.

TO CORRECT THIS PROBLEM, ALL USES OF ASR

INSTRUCTIONS WERE CHANGED TO ROR INSTRUCTIONS

WHICH ROTATE THE CARRY BIT INTO BIT 15. IT

SHOULD BE NOTED THAT THIS WAS A DIFFICULT

PROBLEM TO DETECT BUT, ON THE OTHER HAND,

WAS FAIRLY EASY TO CORRECT.

APL CHARACTER SET FOR THE VT220 FAMILY OF TERMINALS

#

___________________________________________________

THE VT200 FAMILY OF TERMINALS SUPPORTS DOWN-LINE LOADABLE

CHARACTER SETS. THE MAIN OBSTACLE OF USING THIS FEATURE IS THAT THE

CODE SEQUENCE USED TO GENERATE CUSTOM CHARACTER SETS IS DIFFICULT TO

GENERATE FROM SCRATCH. TO OVERCOME THIS DIFFICULTY, I HAVE WRITTEN

A PROGRAM CALLED COMPOSE THAT CAN BE USED TO EASILY DEVELOP CUSTOM

CHARACTER SETS. THE PROGRAM DISPLAYS A CHARACTER CELL THAT YOU USE

TO FORM YOUR CUSTOM CHARACTER BY TURNING OFF AND ON PIXELS WITHIN THE

DISPLAYED CHARACTER CELL. AFTER THE CUSTOM CHARACTER SET (UP TO 95

INDIVIDUAL CHARACTERS) HAS BEEN DEFINED, THE PROGRAM STORE THE

CHARACTER SET IN A FORTRAN DIRECT ACCESS FILE. THE PROGRAM ALSO WILL

GENERATE THE CODE SEQUENCE NEEDED TO GENERATE THE CUSTOM CHARACTER

SET ON THE VT220 FAMILY OF TERMINALS.

I USED THE PROGRAM TO GENERATE THE APL CHARACTER SET. IT

SHOULD BE POSSIBLE TO USE THE CODE SEQUENCE LISTED BELOW TO GENERATE

THE APL CHARACTER SET ON NON-RSX OPERATING SYSTEMS SIMPLY BY TYPING

THE FILE AT YOUR TERMINAL PROVIDED THAT THE VT220 IS CONFIGURED TO

BE A VT220 AND PERMIT USER DEFINED KEYS.

THERE IS ONE RESTRICTION THAT APPLIES. THE RESTRICTION IS THAT

THE OVERSTRUCK APL CHARACTERS WILL BE DISPLAYED AS ONE OF THE TWO

CHARACTERS INVOLVED IN THE OVERSTRIKE. FORTUNATELY, YOU CAN ENTER

OVERSTRUCK CHARACTERS IN THE NORMAL FASHION ON THE VT220 AND THEY CAN

BE PRINTED ON APL TERMINALS SUCH AS THE LA12 OR THE LA120 WITH THE APL

CHARACTER SET OPTION.

CODE SEQUENCE USED TO GENERATE THE APL CHARACTER SET ON THE VT220

#

__________________________________________________________________

<ESC>P1;1;1{1

CAC?CAC?/????????;??ACw???/??A@????;??OgC???/????@???;??GSa???/AAAAAAA?;

ggggggg?/????????;???CgO??/???@????;?AAA}???/?AAAB???;ACGOGCA?/????????;

_OGCGO_?/????????;gggwkig?/?A@?????;OOOSOOO?/???@????;??__????/?A@?????;

OOO{OOO?/???@????;????????/??BB????;??_OGCA?/A@??????;wCAAACw?/?@AAA@??;

?GC}????/?AABAA??;CaaQQSG?/BAAAAAA?;AAAQQYe?/@AAAAA@?;_ogc}__?/????B???;

]QIIIIq?/@AAAAA@?;wcQQQQc?/?@AAAA@?;AAAaQIE?/?A@?????;kQQQQQk?/@AAAAA@?;

KQQQQKw?/@AAAA@??;??wCA???/???@A???;??}AAAA?/??BAAAA?;??ee????/?A@?????;

?CgOgC??/?@???@??;??KK????/??BB????;ACGO_???/?????@A?;AAAAAAA?/????????;

wCCCwCC?/?@@@?@@?;???{????/AAABAAA?;?wCCCw??/?@???@??;}???????/BAAAA???;

?gSSSS??/??@@@@??;????????/CCCCCCC?;KScCcSK?/???@????;?_OGO_??/BAAAAAB?;

??Cw????/????@???;?OgggO??/????????;??GEA???/????????;}AAAAA}?/BAAAAAB?;

???}????/???B????;CCC{CCC?/???@????;wCCSCCw?/?@@@@@??;OSwOwSO?/?@???@??;

CAAqIIC?/???A????;??wcccW?/?A@?????;}AAAA???/B???????;_OGO_OG?/????????;

O_?}?_O?/??@B@???;?{???{??/??@@@???;wC_O_Cw?/?@???@??;?GGGGGo?/?@@@@@??;

OGC}CGO?/???B????;?oGGGGG?/??@@@@@?;OwSOOOO?/??@?????;{OOOOO??/@???????;

OOOOSwO?/????@???;??aSG???/AAAAAAA?;OOOOOOO?/????????;OgCACgO?/??@A@???;

ogcacgo?/B?????B?;}QQQQQk?/BAAAAA@?;{AAAAAC?/@AAAAA@?;}AAAACw?/BAAAA@??;

}QQQAAA?/BAAAAAA?;}QQQAAA?/B???????;wCAAaac?/?@AAAA@?;}OOOOO}?/B?????B?;

?AA}AA??/?AABAA??;???A}A??/@AAA@???;}OOgCA??/B???@A??;}???????/BAAAAA??;

}CGOGC}?/B?????B?;}CGO_?}?/B????@B?;{AAAAA{?/@AAAAA@?;}QQQQQK?/B???????;

{AAAa{??/@AAAA@A?;}QQQqQK?/B????@A?;KQQQQQc?/@AAAAA@?;AAA}AAA?/???B????;

}?????}?/@AAAAA@?;EW_?_WE?/??@A@???;}?_O_?}?/B@???@B?;ACgOgCA?/A@???@A?;

ACGoGCA?/???B????;AAaQIEA?/ABAAAAA?;?OOkAAA?/???@AAA?;?OOOOO{?/??????@?;

AAAkOO??/AAA@????;GSS}SS_?/?@@B@@??

<ESC>\

<ESC>)B

<ESC>)1

<ESC>[2J

<ESC>[H

^N

!"#$%&'()*+,-./0123456789:;<=>?

@ABCDEFGHIJKLMNOPQRSTUVWXYZ[\]^_

`abcdefghijklmnopqrstuvwxyz{|}~

<ESC>#3 !"#$%&'()*+,-./0123456789:;<=>?

<ESC>#4 !"#$%&'()*+,-./0123456789:;<=>?

<ESC>#3@ABCDEFGHIJKLMNOPQRSTUVWXYZ[\]^_

<ESC>#4@ABCDEFGHIJKLMNOPQRSTUVWXYZ[\]^_

<ESC>#3`abcdefghijklmnopqrstuvwxyz{|}~

<ESC>#4`abcdefghijklmnopqrstuvwxyz{|}~

^O

PROGRAM AVAILABILITY

#

____________________

THE I/D SPACE APL INTERPRETER HAS BEEN SUBMITTED TO

THE RSX SIG TAPE FOR THIS SYMPOSIUM. IT MAY BE SUBMITTED TO

THE DECUS LIBRARY SOMETIME IN THE FUTURE.

THE COMPOSE PROGRAM AND APL CHARACTER SET CODE SEQUENCE

LISTED ABOVE HAS ALSO BEEN SUBMITTED TO THE RSX SIG TAPE FOR THIS

SYMPOSIUM. YOU MAY ALSO ORDER THEM FROM THE DECUS LIBRARY (DECUS

PROGRAM NO. 11-760).

QUESTIONS AND ANSWERS

#

_____________________