F I G E D T

-----------

FIGURE EDITOR

USER INFORMATION

KEYBOARD COMMANDS:

Oxy   -  Set display origin to x,y.

Pxy   -  Move cursor position to x,y.

Ifile -  Include file at cursor position.

Ftext -  Fill box with text.

Ctext -  Insert centered text.

E     -  Exit and Save file.

Q     -  Quit and do not save file.

W     -  Wide screen (132 char.)

N     -  Narrow screen (80 char.)

Function Keys:

GOLD: Provides shifted functions on the other function

keys.

INSERT TEXT: Allows direct type-in of text at the current

cursor position.Terminates with the RETURN key.

UP, DOWN, LEFT, RIGHT: Move cursor in that direction. May be preceeded

with a decimal repetition if desired. If preceeded

by GOLD, moves to the screen margin.

SET, reset: Set or remove a select cursor. One or two may

be set.

Others: see keypad drawing.

+---------+---------+---------+---------+

|         |         |         | INSERT  |

|  GOLD   |         | NEXT    |  TEXT   |

|         |         |   WORD  |         |

+---------+---------+---------+---------+

|    ^  *3|       *1|       *2|         |

|    |    |  DRAW   |  AREA   |         |

|   UP    |  LINE   |  COPY   |         |

+---------+---------+---------+---------+

|    |  *3|       *1|       *2|         |

|    V    |  DRAW   |  AREA   |         |

|  DOWN   |   BOX   |  MOVE   |         |

+---------+---------+---------+---------+

|   <-- *3|   --> *3|       *1| INSERT  |

|  LEFT   |  RIGHT  |  AREA   | COLUMN  |

|         |         |  CLEAR  |         |

+---------+---------+---------+         +

|    INSERT LINE    | SELECT  | delete  |

|                   |         | column  |

|    delete line    |  reset  |         |

+---------+---------+---------+---------+

Notes: *1. Operates from select location to

current cursor location.

*2. Operates from area bounded by two

select locations to area bounded by

double cursor.

*3. Preceed with GOLD to move to the appropriate

screen margin.

*** Most function keys may be prefixed with a decimal

repetition entry.

*** Functions activated by GOLD are in lower case.

+---------+---------+---------+---------+

|    ^    |    |    |         |         |

|    |    |    |    |  <---   |   --->  |

|    |    |    V    |         |         |

+---------+---------+---------+---------+

Arrows are not used in this version of FIGEDT.