Sort-11 V3.0 performance                                                 Page  1

B. Z. Lederman

The  following  is  some  of  the  data  I  obtained when testing some

variants of Sort-11 V2.0 and V3.0.  Statistics were obtained  from  Sort-11

itself,  through  System  Accounting, SRMLOG (a system measurment program),

and with SPM-11.

The first test shown is sorting a file containing 20,000 records of 64

bytes (2579 blocks), each record being a string of random numbers (the file

is  in  "random"  order),  sorted  with four keys (1.8:57.8:9.8:17.8).  The

different lines of data are for differnt numbers of work files:  the  first

is  with  a large number of work files (5 for V2.0, 7 for V3.0), the second

is with the default number of work files, and the third (when  present)  is

for 3 work files.  The three letters at the left is the task name that that

version was installed under, which will be seen in other tables.

Data from SPM                System

Elapsed Time    CPU     DB1 F11ACP Over-  Accounting

Version  Type              sec.    sec.    QIO   QIO   lays  Directives

-----------------------------------------------------------------------

srt V2.0 as distributed     228    106    3149    31   88    9579

299    132    3687    89  199   11242

srr V2.0 non-overlayed      237    116    3061    32         3266

resident library   306    141    3464   114         3954

srs V2.0 overlayed          218    112    2413    35    7    2614

resident library   288    136    2780   122    7    3315

sr3 V3.0 as distributed     228    144    2241    86  110    2697

221    143    1832    72   88    2242

256    160    1971    51   64    2329

sr4 V3.0 resident library   211    145    1531    99   19    1947

211    146    1351    87   19    1740

239    161    1420    77   19    1736

sri V3.0 I/D space, non-    208    144    1398   101         1787

overlayed,         209    146    1247   114         1626

resident library   238    161    1298   104         1652

srj V3.0 as above, larger   200    142    1176    96         1568

work space         204    143    1042   115         1432

231    159    1086   131         1490

When  the  test  is  run  on  an  idle  system  (Sort  was the only task of

significance running at the time), there does not  appear  to  be  a  great

amount  of  difference  beween  versions.   It  can be seen that use of the

resident libraries reduces overlays, and reduces the number of  disk  I/O's

(there  is  more task image space for work area, so less disk work space is

required), but task elapsed time isn't much improved.  In the case of V2.0,

building  the task non-overlayed is worse than the overlayed version:  this

is because only a few overlays are needed (once the RMS overlays are  taken

out),  and  because less task work area is available so more disk work area

is used.  Of the three, the overlayed task linked  to  the  RMS  supervisor

Sort-11 V3.0 performance                                                 Page  2

B. Z. Lederman

mode  library  places  the  least  load  on the system, but this has little

effect on an idle system.  Comparing V3.0 with V2.0 as  distributed,  there

is  a  considerable  reduction  in  the number of directives and QIO's, but

little improvement elsewhere.  Linking to the RMS resident library produces

similar  benefits as were obtained with V2.0, and the resulting task is now

running elapsed times comparable to V2.0 (note that the  actual  percentage

difference  between  all  of  the  tasks  is  small).  With V3.0 however, a

further improvement is obtained by building the task non-overlayed,  as  it

may  be built as an I-and-D space task:  this yields a net increase in task

work space, rather than the loss that occurred with V2.0  which  cannot  be

built  I-and-D  space.   (There  is  so much more code with V3.0, primarily

command parsing, that it cannot reasonably be built  non-overlayed  without

making  it  I-and-D  space.) Now, the task has both increased internal work

space and also has no disk overlays, and this can be  seen  from  the  even

smaller  numbers of QIO's and directives issued.  Still, on an idle system,

the net elapsed time is not changed much, due to  waiting  times  for  disk

I/O.   It  might also be noted that V3.0 seems to be a little smarter about

choosing the best number of work files, and in using them effectively.

The  above test was repeated, but with 10 other programs running which

were created to load the CPU and disk to create an environment  similar  to

what  we  have  on our production systems.  The load from these programs is

constant and predictable, however, so that testing the  different  versions

of  Sort would be valid.  The data file was reduced to 15,000 records (1875

blocks).

Data from SPM

Elapsed Time    CPU     DB1 F11ACP Over-

Version  Type              sec.    sec.    QIO   QIO   lays

------------------------------------------------------------

srt V2.0 as distributed     332    108    2732   129   193

srr V2.0 non-overlayed      345    127    2532   137

srs V2.0 overlayed, reslib  304    119    1868   124

sr3 V3.0 as distributed     254    132    1328    81    88

sri V3.0 I/D space          242    133     867   115

srj V3.0 as above, larger   238    131     696   127

Now that there are other programs on the system competing for CPU time, and

more importantly, disk I/O, the improvement obtained is more obvious.   Use

of the resident library has improved V2.0 elapsed time by almost 10%;  V3.0

is now somewhat faster than V2.0, and a small but  significant  improvement

can  be  seen  in  the  non-overlayed versions of V3.0 over the distributed

version.  By reducing the number of QIOs, there  is  less  competition  for

disk  access,  and  reducing the number of directives issued means less CPU

time is taken up by the operating system, and allows  the  program  to  get

more  work done within it's time slice.  Increasing the internal work space

also allows the program to waste less time and disk resource by not  having

to reference the disk work files as often.

Sort-11 V3.0 performance                                                 Page  3

B. Z. Lederman

Note  that  all of the testing was done with RECORD sorts:  I have not

tested other types of sorts, as we do not  use  them  much  here.   I  have

tested  the different versions with different data files:  with small (less

than 3000 record) files, there is less  difference  between  the  different

versions.  With very small files (less than 100 records), V3.0 is usually a

little slower:  apparently it takes longer to parse out the commands (there

are  more  of them, especially with specification files) and initialize the

task than V2.0 did, but usually the difference is very small, and can  only

be  seen  by  measuring  it  (it  does  not appear to a person sitting at a

terminal that the sort took longer).  We have settled on V3.0 linked to the

RMS  library,  but  overlayed:   we  would  like to go to the I-and-D space

version, but there are other problems (like the inability to install it  in

VMR) that prevent us from doing so at this time.