Programs for testing the speed of CPUs                                   Page  1

B. Z. Lederman

We have applications which run on PDP-11/70s, and until recently there

was no other PDP-11 which was  as  fast.   With  the  introduction  of  the

PDP-11/84,  it  was  suggested  that applications could be moved to the new

machine, but we wanted some method of verifying the speed of the 11/84, and

there were no published instruction times.  I therefore wrote some programs

which will measure the speed at which a CPU will execute instructions  with

reasonable  accuracy:   it will at least give a method of comparing one CPU

to another.

The  method  I  use is quite simple:  a single instruction is executed

many times (generally a minimum of 1,000,000 executions), and the time this

takes  is measured.  Because the largest single program segment on a PDP-11

is about 32,000 words, it is neccessary to use a loop:  a long sequence  of

instructions is repeated as many times as needed to get the total number of

executions wanted.  There is also the effect of cache:  a short  loop  will

fit  entirely into cache and will execute at the fastest possible speed;  a

longer loop will not all fit into cache, and the effect  of  the  speed  of

main  memory  can  be seen.  Finally, it is neccessary to test the speed of

many different instructions (one at a time) to get an adequate profile of a

given CPU's performance.

The  program  S1D.MAC  implements  this type of test.  Although I have

tried to make it as straight-forward as possible, it should  be  remembered

that  this  is basically a diagnostic program, and that it does some things

that I normally wouldn't allow in a production program:   specifically,  it

treats  instruction  space  as  data,  is self-modifying, and there are two

tables which must be kept in alignment manually.  Sometimes  this  sort  of

thing  is neccessary in a diagnostic program.  While modifications would be

fairly easy for a programmer with some experience, I don't  suggest  making

any  changes  unless  you  are  fairly  certain  you know what to do.  (The

program is not priviledged, however, so the worst that can  happen  is  the

program will abort:  it won't crash your system.)

There  are  two global equates at the beginning of the program:  $NREP

is set to the number of instructions wanted in the loop, and $NPAS  is  the

number  of times the loop is executed.  Any combination which will multiply

to a large even number will do, such as 1,000 and  1,000;   3125  and  320;

2500  and  400,  or  any  other  suitable  combination.  Having the numbers

multiply out to 1,000,000 or 10,000,000 isn't absolutely neccessary, but it

makes  it  easier to read out the times:  at 1,000,000 repititions the time

measured in seconds is the instruction time in micro-seconds.  Because  the

loop  has  to  have  3 instructions to form the loop repeat logic, there is

another global called $NREAL which is $NREP minus 3.  As long as  the  loop

is reasonably long (several hundred instructions or more) the effect of the

loop repeat instructions will be negligible.  I considered making them task

build time options, but it would have made the program far too complicated.

Don't be supprised if the program takes more time to assemble when $NREP is

large, as it takes more time to fill the ISTRT area.

There are two tables which must match:  the first starts at MNTAB, and

is a list of mnemonics which print out when you run the program to tell you

Programs for testing the speed of CPUs                                   Page  2

B. Z. Lederman

what  instruction is being tested.  This must match the set of instructions

starting at TABLE if you want to know what is going on.   The  instructions

were  placed in their present order after running the program many times so

the data comes out in what I feel is  the  most  useful  form,  though  the

instructions  could  be in any order.  All of the instructions are position

independant, as they have to be moved by the  program  into  the  execution

area:   testing  other addressing modes would be difficult as the addresses

would all have to be adjusted,  and  I'm  not  sure  that  the  information

obtained  would significantly improve the comparisons between machines.  In

this program, all instructions are one word long:  another  program  called

S2D.MAC is identical except that two word instructions are tested (actually

instructions with  addressing  modes  that  cause  the  instruction  to  be

followed by one word of address information or literal data), and similarly

for S3D.MAC which tests three  word  instructions.   Using  three  seperate

programs makes things very much easier (for me, the programmer).  There are

also a few special cases:  the BRANCH instructions are set to always branch

to  the  next  instruction, which allows them to be tested even though they

are relocated within the program.  The have been selected so that one  will

always branch and the other will never branch:  on many machines there will

be a difference in execution speed for these  two  conditions.   The  other

special  case is the Jump to Subroutine instruction:  for this to be tested

there must be a Return from Subroutine instruction (which  is  in  a  fixed

location),  and  the  time  of  execution  really  is for both instructions

together.  As most subroutines end with a return, this should  be  a  valid

test  as  long  as  one  remembers  that  the  times  obtained  are for two

instructions.  The data  which  is  used  in  the  instructions  is  either

absolute  references  or  loaded into registers:  the data for the Multiply

and Divide instructions  may  not  be  the  best  possible  test  of  these

instructions,   but   it  does  work.   Auto-increment  and  Auto-decrement

addressing modes are difficult to test as the loop is  at  least  1,000,000

instructions  long,  and any continuous test would push the register out of

addressing range:  the only way out I  could  see  was  to  have  the  same

instruction  do  a decrement and increment together, so only two addressing

locations are used, and this can be seen in the MOV -(SP), (SP)+ test.

It  should  be  noted  that  these  programs  include  Floating  Point

Processor instructions:  though it would be easy  to  include  an  assembly

condition  to  include  these  only when wanted, I haven't been able to get

around to doing that yet.  Persons wishing to  test  machines  which  don't

have  FPP  should conditionalize or comment out the FPP instructions:  this

would also apply to anyone who does not have EIS  instructions  (MUL,  DIV,

etc.).   The  programs do not test FIS or CIS instructions, because none of

the machines I have access to have these instructions.

In  the  main body of the program there is space reserved beginning at

ISTRT:  this is initially filled with  No  Operate  instructions,  and  the

program  fills  this  space  with one of the instructions from the TABLE in

order to test it's execution time.  The system directive GTIM$  (Get  Time)

is  used  just  before the start of the loop and just after the loop to see

how long it took:  as system time is delivered in 0.1 second intervals,  it

is  neccessary  to  make the test loop long enough to be greater than this.

Programs for testing the speed of CPUs                                   Page  3

B. Z. Lederman

The fastest 11/70 and 11/84 instructions are about 0.3 micro-seconds  long,

so  if  a  1,000,000 instruction loop is used, the measured times are about

0.3 seconds, only a little above the measurment granularity.   Though  this

is  enough  for an approximate comparison, running a 10,000,000 instruction

test brings the shortest times up to about 3.0 seconds, and gives one  more

significant  digit  of  data.  Running tests longer than this should not be

neccessary.   For  slower  machines  (such  as  the  PRO-350  and   11/23),

10,000,000  instruction  loops  are  nice  but a little slow, and 1,000,000

instruction loops are probably adequate if you don't have  much  test  time

available.

The  use  of  the  system  directive to measure time brings up several

important factors in running these tests, of which the first is  the  amout

of  time  it takes to execute the GETIM$ directive.  I measured this on our

11/70 by having a program issue several GETTIM$ directives sequentially and

measuring  the  time  each  directive  took  to  execute with SPM-11:  this

package from DEC hooks into the executive  and  measures  time  in  0.00001

second  intervals  (100,000  per  second).   The  GETIM$  took about 0.0005

seconds (500 micro-seconds) to execute, which is  so  much  less  than  the

minimum resolution of 0.1 second that it can be ignored if the loop is long

enough to register at least 0.1 seconds.  A related point is that when  the

test  program  is  running, it should be the only program running, and at a

high enough priority that it will "grab" the CPU more or  less  exclusively

(the  CPU  will still have to service the system clock if you want the test

times).  I have run the tests on otherwise idle systems, and at a  priority

of 151 or 152 which is high enough to be above just about everything except

MCR:  I want the test to be below MCR in case I want to  abort  it.   While

this  might  make  the instructions test a little slow, it is still a valid

comparison between different machines, and when I have  compared  the  test

results  obtained  with  the  published  instruction times in the processor

handbooks, I have obtained quite good matches, indicating that the  program

is  testing  accurately.   Under these conditions, it should be possible to

run the test either on RSX-11M-Plus (as I did), or RSX-11M, or possibly  on

RSX-11S  and  obtain  valid results.  The basic program would probably also

run on IAS, but it might be neccessary to change the calls  to  the  system

library  that  format  the output messages and the QIOs if they are not the

same on IAS.

I  have  only  been  able  to  test 3 machines:  the 11/70, 11/84P and

PRO-350.  One of the reasons I am placing these programs on the DECUS  tape

is  the hope that other users with other machines will run the programs and

publish the results, so the user community will  have  additional  data  to

make  comparisons  between  models of PDP-11s when choosing a new system or

contemplating an upgrade path.  Also, by testing more machines, it will  be

possible  to  compare  the times with the published speeds in the processor

handbooks, and obtain additional verification  that  the  speed  tests  are

accurate  (or  not,  though I have reasonable evidence now that the test is

correct).  I would be very interested in seeing the  results  of  tests  on

other  CPUs,  or  suggestions on other instructions which might be added to

the test set.

Programs for testing the speed of CPUs                                   Page  4

B. Z. Lederman

I  have  also started on a version which will run on the VAX family of

machines:  the preliminary program is included  here  as  S3V.MAR,  and  it

tests  a  limited  set  of  3  byte instructions.  It will be neccessary to

expand the number of instructions tested by this program,  and  to  produce

programs  which  test  instructions which are different lenghts in the same

manner as the three PDP-11 programs.  This program works, but  I  have  not

yet  had an opportunity to run it as the only program on the system or at a

high priority, so the times I have  so  far  are  not  valid  and  are  not

included in this submission.  Also, I have not yet been able to investigate

the effect of longer or shorter loops on data  cacheing  or  page  boundery

crossing.  The program GETTIM.MAR is used to test the length of time needed

to do a $GETTIMS, which is  used  to  time  the  loop:   it  does  this  by

measuring the time which elapses while issuing 1000 directives.  This works

out  to  0.00012  seconds  on  a  11/750  (120  micro-seconds),  which   is

supprisingly  less than the RSX system:  I suspect that this is because the

system call is simpler, and that the directive itself has less work  to  do

as  it  simply returns the 8 byte system time register.  The program prints

out the 8 byte time values as I was also using it  to  test  that  my  time

subtraction  logic was correct before using it in the S3V program.  Because

the system clock keeps time in smaller intervals, the instruction times  on

the  750  come  out  in  0.01  second  intervals,  though I would still run

instruction loops of at least 1,000,000 for best accuracy.   I  would  like

very  much  to hear from other users who can suggest a good instruction set

for the VAX tests, and who obtain test results.

B. Z. Lederman

2572 E. 22nd St.

Brooklyn, N.Y. 11235

(212) 250-2300 7:30 AM - 3:30 PM Eastern time

or DCS (Lederman)