DEUNA Driver                 Corporate SPR           SPR Form No. 600789

XEDRV Problem                No. __________                  Page 1 of 7

Carl T. Mickelson, Goodyear Aerospace Corp.

Akron, Ohio 44315 (216) 796-2388

The DEUNA device driver distributed as part of RSX11M V4.1 Update D

has  a  serious  logic  error when issued a QIO to do a DEUNA diagnostic

function without a related transmit or receive data transfer.  The logic

error can cause the DEUNA to be  left  in  an  indeterminate  state;  we

experienced  the inability to transmit any packets after using this type

of diagnostic function QIO (i.e. with no data transfer).

Two scenarios are presented below that explain how such  a  QIO  is

erroneously  handled.   After these scenarios are discussed, patches are

presented that correct the errors, and return a success status for  such

a diagnostic function.  It is shown that the changes fix only the errors

and  do not affect how a QIO combining both data transfer and diagnostic

functions is performed.

This problem has been reported to the Telephone Support  Center  as

call # M850422151.

Attachments:

Copies of sections of XEDRV that pertain to the error analysis.

DEUNA Driver                 Corporate SPR           SPR Form No. 600789

XEDRV Problem                No. __________                  Page 2 of 7

Carl T. Mickelson, Goodyear Aerospace Corp.

Akron, Ohio 44315 (216) 796-2388

Scenario 1 - Setting the Auxiliary

Characteristics Buffer Length to ZERO

The DEUNA driver documentation included in the RSX11M V4.1 Update D

Release  Notes  states  in  section  11.6.6 on page 85, that to do DEUNA

diagnostic functions without doing a data transfer at the same time, one

should code the  length  of  the  characteristics  buffer  (Transmit  or

Receive  QIO parameter P4) to zero (0).  Doing this will cause IE.SPC to

be returned as the QIO status, but the documentation indicates that  any

diagnostics  buffer  (P5  and  P6)  present  will  be  processed.  THESE

STATEMENTS ARE INCORRECT.  What  actually  happens  follows:  IE.SPC  is

returned,  as  stated,  but  the  driver  does NOT re-map the diagnostic

buffer, thereby making the user's buffer inaccessible to  the  Executive

when  the  QIO  is  executed.   However,  the driver ATTEMPTS TO USE the

un-mapped buffer, and can leave the DEUNA in an indeterminate  state  as

experienced at this site.

Referring  to  the  attached listings, observe the following, using

IO.XMT (transmit a packet) as an example:

1.  UC.QUE is set in the device UCB so that the I/O  request  calls  the

driver  to  do  parameter  checking and buffer mapping using the issuing

task's memory mapping context.

2.  XEINI is where the driver is entered, and  after  some  initial  I/O

packet  processing  and  error  checking,  a  dispatch  table is used to

transfer control to XEXMT (XERCV for a receive (IO.RCV) QIO).

3.  XEXMT calls  BFCHK  to  validate  and  remap  the  user's  auxiliary

characteristics  and  diagnostic function buffers into Executive address

space.

4.  Examining BFCHK shows that if the characteristics buffer LENGTH (now

in I.PRM+10) in the I/O packet contains a zero, BFCHK branches  to  20$,

setting  the IE.SPC QIO status, and returns to XEXMT with the C bit set.

Note here that no examination has been made to test for the presence  of

or  to  re-map  a diagnostic function buffer (P5 and P6, now in I.PRM+12

and I.PRM+14).

5.  Continuing in XEXMT, the set C bit causes a branch to 70$, and hence

to XMTDN.

6.  At XMTDN (transmit  data  transfer  done),  the  driver  queues  the

un-mapped  diagnostic  function  buffer to the pending function queue in

the driver (if I.PRM+12 (P5) is non-zero).  This is a major error, since

the driver does not have the information available to properly access to

user's data buffer.  The result can place the DEUNA in an  indeterminate

state,  or  can  cause  the driver to hang, preventing completion of the

diagnostic function QIO.  Both of these cases have  been  seen  at  this

site.

DEUNA Driver                 Corporate SPR           SPR Form No. 600789

XEDRV Problem                No. __________                  Page 3 of 7

Carl T. Mickelson, Goodyear Aerospace Corp.

Akron, Ohio 44315 (216) 796-2388

The examination of BFCHK made to analyse scenario 1 above indicated

a second possible scenario for performing no-data diagnostic functions.

Scenario 2 - Setting the Auxiliary

Characteristics Buffer Address to ZERO

Setting the auxiliary buffer address parameter (P3) to zero changes

the execution path thru BFCHK.  Now the test of auxiliary buffer address

(I.PRM+6)  equal  to  0 upon entry to BFCHK causes a branch to 40$ where

the length field is cleared and BFCHK continues at 10$ to  validate  any

diagnostic  function  buffer found to be present.  Successful completion

of these operations causes a return to XEXMT with C clear, and an  error

status in R0 equal to zero (propagated out of CKCHR, where R0 is used to

check the proper formatting of the characteristics buffer).

Back  in  XEXMT upon return from BFCHK, the BCS falls through (C is

clear) to the test of the auxiliary  buffer  address  (I.PRM+6).   Since

this  parameter  is  zero, the branch to 50$ takes place, where an error

status of IE.BAD is returned,  and  the  QIO  is  terminated.   This  is

certainly not the desired result, but it is an improvement over scenario

1, since all of these type of diagnostic function QIO's are now at least

guaranteed to finish, leaving the DEUNA alone.

It  should  be  noted  that a similar problem exists in the receive

function code as well.

Clearly, the documentation in the Update D release  notes  (section

11.6.6),  while  well  intentioned,  is  erroneous.  Further, the driver

itself has a logic error  that  prevents  non-data  transfer  diagnostic

functions  from  completing whichever way is used to code the absence of

the auxiliary characteristics buffer.  A sort of "XE_Catch 22".

Attention now turns to correcting the driver logic errors  so  that

these types of diagnostic functions can complete successfully.

DEUNA Driver                 Corporate SPR           SPR Form No. 600789

XEDRV Problem                No. __________                  Page 4 of 7

Carl T. Mickelson, Goodyear Aerospace Corp.

Akron, Ohio 44315 (216) 796-2388

Driver and Documentation Changes

Needed to Fix Errors

Following are the changes (*) necessary to correct the driver code,

and  to  fix  the  driver  documentation  to properly perform diagnostic

functions without an associated data transfer.

Change Number 1

In XEXMT:, change to read:

CALL    BFCHK                   ; Validate optional characteristics

; .. buffers

*       BCS     60$                     ; If CS, invalid

TST     I.PRM+6(R3)             ; Do we have an AUX buffer?

*       BEQ     70$                     ; If EQ, none present

.

.  < code through 70$ remains as is>

.

At 70$:, change to read:

70$:    CLR     R1                      ; Clear status word 2

*       MOV     #IE.SPC&377,R0          ; Assume no DIAG buffer

*       TST     I.PRM+12(R3)            ; Do we have a DIAG buffer?

*       BEQ     60$                     ; If EQ, neither AUX nor DIAG, error

*       MOV     #IS.SUC,R0              ; Else, indicate success

BR      XMTDN                   ; Do xmit done, user wants diag info

Changing  the  BCS  target  will  reject  improper  buffers,  while

changing  the  BEQ  target  will  permit a properly re-mapped diagnostic

function buffer to be queued to the driver  and  subsequently  executed.

Insertion  of  the  code  following  70$  tests  to  see if a diagnostic

function buffer is provided when the auxiliary characteristics  function

buffer   is  known  to  be  missing  (only  path  to  70$  is  from  TST

I.PRM+6(R3)).  Should both buffers be missing,  IE.SPC  is  returned  to

status,  otherwise  IS.SUC  is  returned, indicating that the diagnostic

buffer was processed.  The status of each individual diagnostic function

is returned in the diagnostic buffer function status words (C.STAT).

Change Number 2

In XERCV:, change to read:

CALL    BFCHK                   ; Validate optional characteristics

; .. buffers

*       BCS     20$                     ; If CS, invalid

*       TST     I.PRM+6(R3)             ; Do we have an AUX buffer?

*       BEQ     50$                     ; If EQ, none present

DEUNA Driver                 Corporate SPR           SPR Form No. 600789

XEDRV Problem                No. __________                  Page 5 of 7

Carl T. Mickelson, Goodyear Aerospace Corp.

Akron, Ohio 44315 (216) 796-2388

.

.  < code through 50$ remains as is>

.

At 50$:, change to read:

* 50$:  MOV     #IE.SPC&377,R0          ; Assume no DIAG buffer

*       TST     I.PRM+12(R3)            ; Do we have a DIAG buffer?

*       BEQ     20$                     ; If EQ, neither AUX nor DIAG, error

*       MOV     #IS.SUC,I.PRM+12(R3)    ; Else, indicate success

CLR     I.PRM+4(R3)             ; Save status words

As before, changing the BCS target will  reject  improper  buffers,

while  adding  the  I.PRM+6  test  and  the  following BEQ will permit a

properly re-mapped diagnostic function buffer to be queued to the driver

and subsequently executed.  Adding the code at 50$ tests  to  see  if  a

diagnostic    function   buffer   is   provided   when   the   auxiliary

characteristics function buffer is known to be missing (only path to 50$

is from TST I.PRM+6(R3)).  Should both buffers  be  missing,  IE.SPC  is

returned  to  status,  otherwise IS.SUC is returned, indicating that the

diagnostic  buffer  was  processed.   The  status  of  each   individual

diagnostic function is returned in the diagnostic buffer function status

words (C.STAT).

Change Number 3

Change  the  driver  documentation  regarding  how to do diagnostic

functions without an associated data transfer.  The  modified  paragraph

11.6.6 from the Update D Release Notes should read as follows:

"11.6.6  Diagnostic  Functions  Without Data Transfer - To do diagnostic

functions without data transfer specify  all  the  parameters  correctly

with  the  exception  of  the  ADDRESS  of the auxiliary characteristics

buffer.  The auxiliary characteristics buffer address  must  be  set  to

ZERO.   This  address  value causes the driver to skip processing of the

user data buffer specified in the IO.XMT or IO.RCV  QIO.   However,  the

driver  will  process the diagnostic buffer, if present.  IS.SUC will be

returned as status for the call if a  diagnostic  buffer  is  processed,

IE.SPC  is  returned if neither the auxiliary nor diagnostic buffers are

present."

DEUNA Driver                 Corporate SPR           SPR Form No. 600789

XEDRV Problem                No. __________                  Page 6 of 7

Carl T. Mickelson, Goodyear Aerospace Corp.

Akron, Ohio 44315 (216) 796-2388

Validity of the Changes

The question of whether these changes will affect the execution  of

QIO's  that  perform  both data transfer and diagnostic functions in the

same parameter set needs to be addressed on two fronts.

First, since there are no changes made  in  the  buffer  validation

functions  BFCHK  and CKCHR, these routines perform as before.  In fact,

the change recommended in using the  address  of  the  auxiliary  buffer

equal  to zero simply allows the correct logic of BFCHK to be brought to

bear in validating a diagnostic buffer when no auxiliary characteristics

buffer is  provided.   This  buffer  validation  and  re-mapping  is  an

absolute necessity for diagnostic functions to work properly!

Secondly,  the  changes  recommended  for  XEXMT and XERCV must not

modify the program flow through either of  these  routines  for  a  dual

function   QIO   (transmit   or  receive,  plus  diagnostic  functions).

Examining the revised program flow in XEXMT  and  XERCV,  after  calling

BFCHK,  reveals  that  the  driver now properly rejects a QIO with a bad

diagnostics buffer when no auxiliary  buffer  is  provided,  instead  of

trying  to  make  use  of  an  unmapped  user  buffer as in the original

release.  Further, the changes made to test if both buffers are  missing

from the QIO properly returns an error code for no buffers specified and

a success code when the diagnostic buffer is processed.  This represents

improved  performance compared to the documentation in section 11.6.6 of

the release notes.  The user now KNOWS positively if his parameters  are

good  or  bad;  there  is  no  reason to guess about diagnostic function

success.

When  both  the  auxiliary  and  diagnostic  function  buffers  are

provided, the revisions presented here do not change the processing of a

properly formulated QIO request.

DEUNA Driver                 Corporate SPR           SPR Form No. 600789

XEDRV Problem                No. __________                  Page 7 of 7

Carl T. Mickelson, Goodyear Aerospace Corp.

Akron, Ohio 44315 (216) 796-2388

Additional DEUNA Comments

There  are a few additional comments that need to be made regarding

the  DEUNA  driver  and  micro-code  implementations  and   the   driver

documentation as presented in section 11 of the Update D Release notes.

1.  The Ethernet standard allows a minimum of 46. and a maximum of 1500.

data  bytes  in  an  Ethernet  packet.   These  limits  should  be fully

supported by the driver and micro-code in the DEUNA.  The current device

driver limits the maximum packet data length to 1498. to insure there is

room for the software padding word  inserted  by  the  driver  when  the

requested  protocol  specifies  padding in the transfer.  For those user

specified protocols for which  padding  is  not  requested,  the  driver

should  permit  a full 1500. bytes of data in the users message.  As the

driver currently is implemented, the 1498. byte limit is enforced on the

user's data buffer without regard to protocol padding status.   Further,

this  limit  is  imposed  upon  the user's auxiliary characteristics and

diagnostic function buffers as well.  The need for this  test  on  these

buffers is questionable.

2.   The micro-code in the DEUNA itself does not permit minimum Ethernet

packets to be received.  The minimum size packet recognized by the DEUNA

is 64. bytes of data PLUS 18. bytes  for  destination  and  source  node

addresses,   protocol   type,  and  frame  check  sequence.   This  size

restriction should be 64. bytes INCLUDING the 18. bytes  for  addresses,

protocol,  and  frame check.  Such a minimum size restriction would then

be consistent with the defined Ethernet  standard,  and  would  allow  a

minimum  data  field  of  46.  bytes within the transmitted packet.  The

DEUNA micro-code should be modified to support this  minimum  size,  and

the section of driver documentation (para. 11.6.7) that addresses buffer

size restrictions should be modified to reflect this change.

3.   Finally,  the  section of the driver documentation (para. 11.5, and

Table 2 Valid  Function  Codes)  that  pertains  to  issuing  diagnostic

functions  should  include  comments  that  certain  of these functions,

particularly those that modify the operational state of the DEUNA,  will

not be performed unless the task issuing the QIO is privileged.