LOADABLE DEVICE DRIVER DATA BASES

IN RSX11M SYSGEN

Carl T. Mickelson

Goodyear Aerospace Corporation

Akron, Ohio 44315

ABSTRACT

________In order to support multiple and differing configurations of peripherals

on different computer systems, a method for modifying RSX11M SYSGEN is

discussed that allows device drivers to be built with loadable, rather

than resident, device data bases.  System tailoring is done at VMR-time

when the particular devices for each configuration are included in the

bootable system image.  The method allows the addition of new devices

without a new SYSGEN.

INTRODUCTION

What do you do when your OEM employer's business is growing and he

acquires a room full of new computer systems for a variety of different

customers?  Each system has a different configuration of discs and

terminal interfaces and you are expected to provide a consistent

software development environment on all these different systems so that

user's may build programs on any development system and move them to any

test stand system to check out your product hardware.  The users don't

want to be confronted with different operating procedures on each

machine and would like to move even privileged tasks, when possible,

from system to system without re-building them.  In addition, you don't

want to be confronted with performing a new system generation for every

new system as it arrives.

It would be much more convenient to maintain a system generated for each

generic processor (11/34, 11/24, 11/40, 11/70) and memory configuration

and simply build an operating system for a new computer by selecting an

existing pre-generated system and specifying what devices are in the

configuration at VMR time.  Further, the tailoring of the terminal

driver for different systems raises an additional complication for

systems having the same processor but differing terminal I/O

controllers.

This paper discusses a method of achieving this degree of

hardware/software independence for RSX11M, and discusses how this goal

to provide a common operating environment was achieved.  It should be

noted that privileged tasks that map over the Executive can still only

be ported between systems with the same or similar processors and the

same memory address range (18 bit or 22 bit).  This represents an

improvement over "vanilla" RSX11M since device configuration is no

longer a consideration.

ISSUES NEEDING CONSIDERATION

The principal method needed to achieve the goals stated above is to

modify the RSX11M SYSGEN procedure to make all DEC supplied device

drivers look like user-written drivers, each task-built containing its

own loadable device data base structure.  By doing this, device data

structures are no longer resident in the Executive task image

(RSX11M.TSK), and individual devices can be configured into a system by

loading their data bases, together with their driver code at VMR time.

In fact, the only device drivers that need to be present in the bootable

system image are the boot device and terminal device drivers.  All

others can be loaded into GEN with a /HIGH switch in the system startup

file.  The ability to add additional devices is also enhanced, since a

SYSGEN can now be avoided.  There are, however, a number of issues that

must be addressed in order for this technique to be successful.

First, all link time references to I/O data structure symbols must be

eliminated, since the I/O data structures are no longer resident in the

Executive.  Without doing this, the link time references to .TT0 and

.CO0 in the Executive (SYSCM) and any privileged tasks such as the Pool

Monitoring Task (PMT) and the Console Output Task (COT) will produce

undefined references when they are task-built.

This requirement is satisfied by adding some code to the Executive

module INITL.MAC to walk down the device DCB chain when the virgin

executive is initially started to locate .TT0, the console terminal UCB

address, and .CO0, the console output UCB address.  Once found, these

addresses are loaded into the required fields of SYSCM, and RSX11M

continues as if the data bases were resident.  Similar code is added to

PMT and COT at their entry points, and the references to these symbols

are made indirectly to local copies of the addresses.  This isolates

these privileged tasks from the actual locations of the data structures.

Second, the Executive module SYSTB.MAC, generated by SYSGEN during the

device configuration phase, must be split into individual files

(xxTAB.MAC), one for each type of device selected for inclusion in the

SYSGEN process.  The only data structures remaining in SYSTB are the

pseudo-device structures for SY:, LB:, etc, and the clock queue

list-head.  The device data base files must later be assembled with the

RSX11M driver sources, so additional commands must be added to

RSXDRVASM.CMD.  Also, the driver task build command file, RSXDRVBLD.CMD,

must be modified to task build each driver with its own local data base.

Finally, to support different configurations of terminal interfaces on

systems with similar processors, a method must be found to build

multiple versions of the terminal driver.  Each version of the driver

must have a loadable data base structured for that particular system so

that the driver for the proper terminal configuration can be loaded at

VMR time.  It is assumed that the full-duplex terminal driver is the

driver of choice for this effort as it provides full system

functionality for the user.

DEVELOPING THE CHANGES

Once it is understood what must be done, a method must be developed to

automatically apply the changes when a system is to be generated.  This

method should be applied in two parts.

The first part applies the needed changes to SYSCM and the privileged

tasks to eliminate the link time references discussed earlier.  These

changes are required for the SYSGEN changes to be successful, but they

can stand alone so that the tasks do not directly access I/O structures

at link-time.

The second part of the modification package should apply the changes to

the SYSGEN procedure files, and should only be applied if the first part

changes are made.  These modifications amend the SYSGEN procedure to

achieve the functionality described earlier.

In order the develop the changes, a number of utilities are necessary,

among them the standard utilities SLP, PIP, MAC, TKB, etc, and a DECUS

supplied disassembler DOB.  This utility is needed to disassemble

certain modules, delivered only in object format so that they may be

modified in source form, re-assembled and re-inserted into their

appropriate object library.

A trial SYSGEN was performed, stopping at all end of section and end of

execution breakpoints to study the state of the files produced by

SYSGEN.  The SYSGEN command files themselves were examined to determine

where changes had to be made and a set of command procedure (.CMD) and

source update (.SLP) files were developed to apply the changes

automatically.  The specific modifications made to the SYSGEN procedure

are discussed below.

SYSGEN PROCEDURE CHANGES

The default option for all device drivers is set loadable, and this

specific question is eliminated from the SYSGEN dialogue.  The need for

this is self-evident since the desired goal is to make a system with

loadable device data bases.  Also, the user is given the opportunity to

include the Executive routines $PTWRD and $GTWRD regardless of whether

user-written device drivers are to be included during the system

generation.  This is done to insure that these routines are available if

a user-written driver will be incorporated at some future time.

Additional questions are added to the procedure to allow the user to

establish an identifier (xxx) for each specific version of the terminal

driver and system executive configuration being simultaneously generated

for a common processor type.  This identifier is appended to the

terminal driver task and symbol table file names, and to the Executive

symbol table and bootable system file names to designate the system

configuration pertinent to each set of files.

A tailored system VMR command file (SYSVMRxxx.CMD) is generated for each

individual system configuration, and the user is given the opportunity

to specify the device drivers that should be included in the system

image prior to the first bootstrap.  The balance of the VMR file is

tailored to install only those privileged tasks needed for support of

the devices loaded into the system image.  Thus, if DUDRV is loaded, RCT

is installed, and queue manager support is included only if LPDRV has

been loaded.

Finally, just before VMR is invoked to create a bootable image, the user

is permitted to select the version of the system and hence TTDRV version

to be used to create the bootable system image for the SYSGEN host

system.

The modified procedure, when completed through Phase II creates a set of

virgin system (RSX11Mxxx.SYS) files that can be moved to another disc

medium and together with the drivers and privileged tasks in [1,54],

libraries in [1,1] and other system directories such as [1,2] can be

used to VMR another system for a different hardware configuration.  The

resulting system can be booted on the new system configuration and has

the same Executive and resident library characteristics as the

originally VMR'ed system.

USING THE MODIFICATION KIT

The modifications described here are performed by a set of command

procedures that will be available on the RSX SIG tape from the New

Orleans DECUS.  They should be used in accordance with the following

procedure.

First copy the RSX11M baseline distribution kit to scratch media as

described in the RSX11M SYSGEN reference manual.  Then apply DEC's

distributed Update D patch procedure to recreate the RSX11M baseline kit

against which these modifications are performed.  If the Able Computer

Technology ENABLE Memory Management Unit (and Cache) is to be installed

in your host system, apply the Able supplied patches next.  Remember

that any system built with the ENABLE hardware support cannot be moved

to a system without the same hardware.  This is a restriction of the

Able patches, and not the changes discussed here.  Finally, apply the

procedure described in this paper by invoking the DECUSMODS.CMD command

procedure.

This command procedure supports all three types of RSX11M distribution

kit and will ask the user to specify which kit is in use.  The procedure

will then ask the user to re-build the indirect command processor (ICP)

to enlarge its symbol table if a very large system configuration is

being generated.  This step should be taken, and the new ICP installed

if the SYSGEN being performed will generate a system with a very large

number of devices.

DECUSMODS should be re-invoked to apply the modifications described

here.  A set of hexadecimal modifications can be installed by this

procedure if the user wishes.  These hex modifications are discussed in

the companion paper "RSX11M Hexadecimal Command Line Numerics" presented

at this DECUS symposium.  The next set of changes that can be applied

are the first part or I/O symbol linkage changes discussed earlier.

These must be selected if the loadable driver data base changes are to

be applied.

Each set of changes will be applied in sequence as the user selected.

The procedures are designed to verify the modified files against

expected results, but due to DECUS restrictions about copyrighted source

code content on SIG tape submissions, not all the required files can be

included.  This should pose no problems, so long as Update D of V4.1 is

used as the baseline for the procedure.  After the modifications are

performed, the normal SYSGEN procedure is initiated as described in the

SYSGEN manual.

DIFFERENCES IN SYSGEN PROCEDURE

The following procedural changes should be observed when performing a

SYSGEN with these changes applied.

If the system is autoconfigured, and the autoconfigure results are not

overridden and amended, the resulting system will be restricted to

supporting only the devices found by the autoconfigure program.  This

defeats a major purpose of using these changes, that is to generate a

single system capable of supporting many different hardware

configurations.

If autoconfigure is suppressed or overridden, when SYSGEN asks for the

number of controllers of each device type in the system, the user should

specify the controllers that comprise the union of all controllers in

all the systems to be supported.  Thus if three systems are to be

supported on three different systems disc types, the controllers for

each disc should be specified in the hardware configuration section.

The number of each controller type to be generated should be specified

as the maximum across all the systems to be supported.  By following

these two rules, SYSGEN will build all the device drivers necessary to

support all the systems and will generate data bases to support the

largest hardware configuration.  Smaller systems will indicate off-line

status for those devices not present at boot-time.

The same rules pertain to specifying the number of terminal controllers

YL, YH, YZ, etc during the system hardware configuration.  Later during

the procedure, where each system's terminal driver is configured, the

procedure will permit the user to specify the actual number of each

controller type supported by a given system configuration.  This is

where each TTDRV data base is reduced in size to support only the

specific terminal interfaces present in a system.

A word of caution applies here however.  If the maximum number of any

specific terminal interface class (YL, YH, YZ, etc) is greater than one,

the minimum number that should be specified during this tailoring phase

is either 0 (none) or 2 controllers for that class.  This is

particularly important for the YL class of single line controller.  If

this rule is not followed, the driver will assemble for a

multi-controller system configuration, but the LOAd processor will

create an Interrupt Transfer Block (ITB) in POOL to support only a

single controller.  If this should happen, the generated Executive will

CRASH ON THE FIRST KEYSTROKE after the virgin Executive is bootstrapped!

The SYSGEN procedure continues normally, configuring Executive, terminal

driver and other system features.  During the device configuration

phase, where CSR and vector addresses are specified for each device

controller, when the terminal devices are being configured, the modified

procedure asks for the system identifier discussed earlier.  The

procedure then asks for the number of each type of terminal controller

in the system.  The procedure creates a terminals data base containing

the currently specified subset of controllers and asks if another driver

data base is to be built.  If another is to be built, a new identifier

and controller set is requested and the new data base is created.  This

process is continued until no more data bases are requested.

During this section, if the first system identifier specified is the

NULL string, a standard single configuration system is built, including

support for loadable data base device drivers.  A non-null string causes

a tailored terminals data base (TTTABxxx.MAC) to be created for the

system.

Phase I of SYSGEN continues normally, assembling the Executive, drivers

and individual driver data bases and preparing for Phase II task

building.  Chaining to Phase II continues the SYSGEN procedure where the

system libraries, Executive and privileged tasks are built.  It is

during this phase that the modified procedure asks the user to

re-specify the list of system identifiers used to designate the

different terminal driver configurations.  This list is used to create a

virgin system boot file for each individual system configuration.  In

addition, the user is given the opportunity to specify a list of device

drivers to be installed in each system.  This list of drivers is used to

tailor a SYSVMR file for each system configuration.  At the end of this

stage, the user is asked to specify the system identifier for the system

on which the SYSGEN is being performed.  The proper version of the

terminal driver is selected for for inclusion in the host's new system,

and the system VMR process is executed to configure the new operating

system for the host machine.  When Phase II of SYSGEN ends, the new

Executive is booted and saved with the bootstrap block written in the

usual manner.

When the system re-boots itself, SYSGEN Phase III can be performed to

re-build any needed utilities to use RESLIB, ANSLIB or other system

features.  After completing Phase III, and purging the system disc, the

system is ready to be transported to another hardware configuration.

MOVING TO ANOTHER HARDWARE CONFIGURATION

To move the newly generated system to another hardware configuration,

the following minimum set of directories and files should be copied to a

new system disc device, ([1,1], [1,2], [1,54], [11,10]RSXMC.MAC).

Directory [1,1] contains the system object and macro libraries; [1,2]

contains the help files and standard startup procedure files; and [1,54]

contains the system privileged tasks and utilities.  RSXMC.MAC is

included to provide definitions of the supported system features so that

privileged tasks needing this information can be assembled correctly.

The correct version of the terminal driver task image and symbol table

should be copied from TTDRVxxx.* to TTDRV.* and VMR executed using

SYSVMRxxx.CMD to configure the bootable Executive for the new system.

The new system image RSX11Mxxx.SYS sholud then be bootstrapped and saved

with bootblock to create a system disc that is bootable on the new

configuration.

This new system has exactly the same Executive, resident libraries,

privileged tasks, and utilities as the original system.  Further,

neither system has suffered a loss of dynamic storage or POOL space due

to un-necessary device data structures resident in the Executive.  Also,

the terminal driver has been tailored to support the specific kind and

number of controllers on each hardware configuration.

ADDING A NEW DEVICE

Adding a new device to a system built with these changes is a relatively

easy job.  There are a number of different cases that need to be

considered, but the job can typically be accomplished without the need

of another full system generation.

If a device is to be added on an existing controller, the procedure is

as follows.  First, the data base source file xxTAB.MAC is edited,

adding a new UCB structure to the file.  The new UCB is added to the end

of the existing UCB's in the file and is linked to the DCB and SCB

already present.  The highest unit number field of the DCB is

incremented to enable the new unit.

The revised file is re-assembled with RSXMC.MAC and EXEMC.MLB to produce

an object file for the new data base.  The driver task and symbol table

are re-created by task-building the new data base with the driver code

object file.  The new driver is placed in service by simply LOAding it

at system startup if it was not installed by VMR.  Otherwise, a copy of

the virgin Executive, RSX11M.TSK, must be reinitialized with VMR and

re-booted to install the new driver.

Installing new devices on a new controller is a similar procedure,

except that a new SCB together with the UCB(s) must be added to the data

base.  Additional information on performing these steps can be found in

the Guide to Writing a Device Driver in the sections describing how the

I/O data structures are organized.

If a new type of device is to be added to the system and the system was

originally generated anticipating the device's inclusion in the

configuration, this task simply entails LOAding a driver that is already

available.  If a pre-built driver is not available, the data structure

for a similar type device could be used as a model for the new device.

However, this approach is difficult to accomplish successfully except in

the simplest of cases for straight forward devices.  The approach breaks

down if the device UCB is extended to include device specific fields.

In this case, the driver should be built during the SYSGEN procedure.

CONCLUSIONS AND COMMENTS

The techniques discussed here and embodied in the files of the

modification kit permit multiple configuration systems to be generated

simultaneously, with fully loadable device drivers and data structures.

This permits a running system to be expanded easily and can eliminate

the need to do an additional SYSGEN to make simple system configuration

changes.

These techniques have been successfully used to generate systems

supporting PDP 11/34, PDP 11/24, and PDP 11/40 UNIBUS processors.

Systems with both 18 and 22 bit address support have been built from big

disk, RK06/RK07 and RL02 distribution kits.  The following cautions and

caveats are applicable when considering using these changes:

1. Controller CSR and vector addresses must be consistently set across

all systems.  SYSGEN'ing different types of controllers at the same

address is acceptable as long as their device drivers are not LOAded

into the same system image during VMR processing or system operation.

2. Support for multiple terminal driver configurations is provided for

the full-duplex terminal driver only.

3. The changes apply to Update D of RSX11M V4.1.  Proper application to

other updates of V4.1 is not guaranteed.  Since Update D added DEUNA

driver support to RSX, and changes are made in building its data base,

earlier Update kits are known to be incompatible with the changes.

4. Laboratory devices (A/D and D/A converters) and the IC series devices

have not been addressed.  Since the systems at the author's site do not

use these types of interfaces, no changes have been designed for these

devices during SYSGEN.

5. Any layered product that expects to find some I/O symbols defined in

the Executive symbol table will not link successfully.  This applies

only to layered products that are built as privileged tasks as they are

the only ones with the potential to access these I/O structures

directly.

6. The compilers in use at the author's site have been built and used

without modification.  DECnet, however, has not yet been added to these

systems, so it is not known if the changes will effect the NETGEN

procedure.  Inclusion of DECnet in these systems is planned for the near

future.

7. While the changes made to the SYSGEN procedure do not depend upon the

processors being UNIBUS based, no Q-bus machine configurations have been

generated.  It is not known if these changes will properly support a

Q-bus based system.

8. Last, but not least, it must be remembered that systems generated

using this procedure are transportable only between multiple hardware

configurations that share the following characteristics:

a. same or similar processors; if built for EIS support, all

processors must support EIS.

b. have memory management hardware; must be mapped systems to provide

the support necessary for loadable device drivers.

c. same memory address range; if built for 22 bit extended memory,

all systems must support 22 bit addressing.

CAVEAT

The kit of files comprising the SYSGEN changes described here are

supplied for information purposes only.  Use of these changes at any

PDP-11/RSX11M site is entirely at the risk of the using organization.

Neither the author, nor Goodyear Aerospace Corporation, nor DECUS, nor

the RSX SIG is responsible if these changes do not perform successfully

in any particular system.