; - FILE FNDFIL.DOC

;

Fndfil is a program that searches all file headers on a disk trying

to locate a logical block within a file.  This can be useful when the

Error Logger indicates disk errors.  The error log reports certain errors by

logical block number (LBN).  The error log report of what LBN's are bad does

not lend itself to easily resolving these bad spots on the disk.  This

program helps you to identify what files these LBN's are in.  It additionally

computes the virtual block number within the identified file.  Once identified,

individual blocks of corrupted files can hopefully be restored or recreated

from backups and thus eliminating the cause of the system errors.

Please direct comments and suggestions to:

Barry Zion

The Federal Reserve Bank of N.Y.

95 Maiden Lane  -  7th floor annex

New York, N.Y. 10045

tel: (212) 791-5585

To run the program:

>run fndfil

DEVICE AND UNIT TO SEARCH ON:ddn:

DECIMAL LOGICAL BLOCK TO SEARCH FOR:nnnnnn

where ddn: is a device such as DB1:

and nnnnnn is an LBN in the range 1 - max

where max is the highest LBN for the particular disk

Fndfil outputs a header message indicating the 24-bit LBN it is

searching in octal.  Then if any file header's retrieval pointers are mapping

the LBN various information pertaining to the header are displayed.  Once the

program finishes you may wish to verify via a directory command that  the file

is indeed accessible.  One note of importance is that the FILE ID field

displayed by fndfil may be different from that displayed by a "PIP/FU" command.

This is because the map area of the file header could not contain enough

retrieval pointers to map the entire file.  An extension header was necessary

to map all blocks the file.  This occcurs when  a file composed of

more than 102 discontinuous segments or more than 26,112 blocks.  Another

way of accessing the file is to use DMP and dump the entire file header as

follows:

>DMP TI:=ddn:[uic]filnam/HD/BL:0:0

Substitute in the file name(s) output by fndfil and check the various

file header fields.

The indication that an extension header is in use is a non-zero

M.EFNU and M.EFSQ field in the map area.  The values in these fields will

be the file id of the extension header.

FNDFIL will continue searching all file headers even if a file

is found.  This is extra insurance in case of a severely corrupt file system

in which a block is allocated to more than one file.  When all file headers

are exhausted a file header summary is displayed showing in use and free file

headers as follows:

SEARCH OF ALL FILE HEADERS COMPLETED ON DR2:

FILE HEADER SUMMARY

SCANNED IN USE FILE HEADERS :   3090

SCANNED  FREE  FILE HEADERS :   3173

FILE HEADERS YET TO BE ALLOCATED :   1836

------------------------------------------

MAX FILE HEADERS FROM HOME BLOCK :   8099

FNDFIL CAN BE BUILT BY INVOKING @FNDFILBLD.CMD

This command file will assemble and task build all modules of fndfil.

And when you're ready to fix all the bugs I so cleverly and unknowingly left

in the program you can use @FNDFILODT.CMD.  This will create a task FNDFILODT

that includes ODT.

*******************************************************************************

R M S    f a n s    t a k e    n o t e

As an extra added bonus for all of you RMS fans out there I have

included FABERR.MAC and RABERR.MAC.  These two routines can be used as error

routines for all of your RMS calls involving FAB's and RAB's.  When called

they will translate all of the fields of the associated RMS block (FAB or RAB)

into intelligible english.  Sorry, I didn't bother with the STS or STV values.

You will still have to look them up in the RMS macro ref manual Table A-2 or

use online help "HELP RMS ERROR".

*******************************************************************************

To test FNDFIL use the command file @tstfndfil.cmd.  This command

file will DMP the file header of the newly created FNDFIL.TSK.  You can then

use this data as input to the program.  This will see if fndfil can at least

find itself!  Enter the LBN of the first (or any other) block of FNDFIL.TSK as

specified by the dmp.  A sample DMP follows:

Dump of DB1:[301,53]FNDFIL.TSK;2 - File ID 17576,5,0

File header

HEADER AREA

H.IDOF  	027

H.MPOF		056

H.FNUM,

H.FSEQ		(17576,5)

H.FLEV		401

H.FOWN		[301,053]

H.FPRO		[RWED,RWED,RWED,R]

H.UCHA		200 =  UC.CON

H.SCHA		000 =

H.UFAT

F.RTYP	001 = R.FIX

F.RATT	000 =

F.RSIZ	1000 = 512.

F.HIBK	H:0 L:000333 = 219.

F.EFBK	H:0 L:000334 = 220.

F.FFBY	0 = 0.

(REST)

000000 000000 000000 000000 000000 000000 000000 000000

000000

IDENTIFICATION AREA

I.FNAM,

I.FTYP,

I.FVER		FNDFIL   .TSK;2

I.RVNO		1

I.RVDT		26-MAR-85

I.RVTI		14:44:37

I.CRDT		26-MAR-85

I.CRTI		14:43:16

I.EXDT		--

MAP AREA

M.ESQN		000

M.ERVN		000

M.EFNU,

M.EFSQ		(0,0)

M.CTSZ		001

M.LBSZ		003

M.USE		002 = 2.

M.MAX		314 = 204.

M.RTRV				*********************************

SIZE	LBN			*********************************

219.	H:### L:###### = NNNNNN.<<<----- enter this as the LBN

CHECKSUM				*********************************

H.CKSM		020145		*********************************