Chapter 1

Console Device

This chapter describes how you do console (keyboard  and  screen)

input  and  output on the Amiga.  The console device acts like an

enhanced ASCII terminal.  It obeys  many  of  the  standard  ANSI

sequences  as  well as additional special sequences unique to the

Amiga.

1.1.  INTRODUCTION

Console I/O is tied closely to the Amiga Intuition  interface;  a

console  must  be  tied to a window that is already opened.  From

the Window data structure, the console device determines how many

characters it can display on a line and how many lines of text it

can display in a window without clipping at any edge.

You can open the console device many times,  if  you  wish.   The

result  of  each  open  call is a new console unit.  AmigaDOS and

Intuition see to it that only one window is the currently  active

one,  and its console, if any, is the only one (with a few excep-

tions) that receives notification of input events, such as  keys-

trokes.   Later  in  this chapter you'll see that other Intuition

events can be sensed by the console device as well.

NOTE:  For this entire chapter the characters  "<CSI>"  represent

the  Control  Sequence Introducer.  For output you may either use

the two-character sequence "<Esc>[" or  the  one-byte  value  $9B

(hex).  For input you will receive $9B's.

1.2.  SYSTEM FUNCTIONS

The various system functions, such  as  DoIO(),  SendIO(),  Abor-

tIO(),  CheckIO()  and  so on operate normally.  The only caveats

are that the CMD_WRITE may cause the caller to  wait  internally,

even with SendIO(), and a task waiting on response from a console

is at the user's whim.  If a user never  reselects  that  window,

and the console response provides the only wakeup-call, that task

may well sleep indefinitely.

November 17, 1985

- 2 -

1.3.  CONSOLE I/O

The console device may be thought of as a  kind  of  a  terminal.

You  send  a  character  stream  to the console device.  You also

receive a  character  stream  from  the  console  device.   These

streams may be characters or special sequences.

Console character screen output (as compared to  console  command

sequence  transmission) outputs all standard printable characters

(character values hex 20 thru 7E and A0 thru FF) normally.   Many

control  characters  such  as BACKSPACE and RETURN are translated

into their exact ANSI equivalent actions.  The line-feed  charac-

ter  is a bit different, in that it can be translated into a new-

line character.  The net effect is that the cursor moves  to  the

first column of the next line whenever a <LF> is displayed.  This

is set via the mode control sequences  discussed  under  "Control

Sequences for Screen Output".

If you read from the console  device,  the  keyboard  inputs  are

preprocessed  for you.  You will get the ASCII characters such as

"B".  Most normal text gathering programs will read from the con-

sole  device  in this manner.  Special programs like word proces-

sors and music keyboard programs will use raw  input.   Keys  are

converted via the keymap associated with the unit.

The sections below deal with the following topics:

o   Setting up for  console  I/O  (creating  an  I/O  request

structure)

o   Writing to the console to control its behavior

o   Reading from the console

o   Closing down a console device

This section shows you how to set up for console I/O.

1.4.  CREATING AN I/O REQUEST

Console I/O, like other devices, requires that you create an  I/O

request  message that you pass to the console device for process-

ing.  The message contains the command, as well as a  data  area.

In  the  data  area,  for a write, there will be a pointer to the

stream of information you wish to write to the  console.   For  a

read,  this  data  pointer shows where the console is to copy the

data it has for you.  There is also a length field that says  how

many  characters (maximum) are to be copied either from or to the

console device.

November 17, 1985

- 3 -

Here is a program fragment that can be used to create the message

block that you use for console communications.

For writing to the console:

struct IOStdReq *consoleWriteMsg;       /* I/O request block pointer */

struct Port *consoleWritePort;          /* a port at which to receive replies*/

consoleWritePort = CreatePort("mycon.write",0);

if(consoleWritePort == 0) exit(100);    /* error in createport */

consoleWriteMsg =  CreateStdIO(consoleWritePort);

if(consoleWriteMsg == 0) exit(200);     /* error in createstdio */

For reading from the console:

struct IOStdReq *consoleReadMsg;

/* I/O request block pointer */

struct Port *consoleReadPort;

/* a port at which to receive replies */

consoleReadPort = CreatePort("mycon.read",0);

if(consoleReadPort == 0) exit(300);     /* error in createport */

consoleReadMsg =  CreateStdIO(consoleReadPort);

if(consoleReadMsg == 0) exit(400);      /* error in createstdio */

These fragments show two messages and ports being  set  up.   You

would  use  this  if you want to have a read command continuously

queued up while using a separate message with its associated port

to send control command sequences to the console.

In addition, if you want to queue up  multiple  commands  to  the

console,  you  may wish to create multiple messages (but probably

just one port for receiving replied messages from the device).

1.5.  OPENING A CONSOLE DEVICE

For other devices, you normally use OpenDevice() to pass an unin-

itialized  IORequest  block to the device.  For a console device,

this is slightly different.  You must have initialized two fields

in  the  request  block;  namely, the data pointer and the length

field.  Here is a subroutine that can be used to open  a  console

device  (attach  it  to  an  existing  window).   It assumes that

intuition.library is already open, a window has also been opened,

and this new console is to be attached to the open window.

November 17, 1985

- 4 -

/* this function returns a value of 0 if the console

* device opened correctly and a nonzero value (the error

* returned from OpenDevice) if there was an error.

*/

OpenConsole(request,window)

struct IOStdReq *request;

struct Window *window;

{

/* Open a console device */

request->io_Data = (char *) window;

request->io_Length = sizeof(*window);

return(OpenDevice("console.device", 0, request, 0));

}

To perform console I/O, you fill in fields  of  the  console  I/O

standard request, and pass this block to the console device using

one of the normal I/O functions.  When  the  console  device  has

completed the action, the device returns the message block to the

port you have designated within the message itself.  The function

CreateStdIO()  initializes  the message to contain the address of

the ReplyPort.

Here are subroutines that uses the IOStdReq created  above.  Note

that  the  IOStdReq  itself contains a pointer to the device with

which it is communicating so a single function  can  be  used  to

communicate with multiple consoles.

November 17, 1985

- 5 -

/* output a single character to a specified console */

ConPutChar(request,character)

struct IOStdReq *request;

char character;

{

request->io_Command = CMD_WRITE;

request->io_Data = &character;

request->io_Length = 1;

DoIO(request);

return;

}

/* output a stream of known length to a console */

ConWrite(request,string,length)

struct IOStdReq *request;

char *string;

int length;

{

request->io_Command = CMD_WRITE;

request->io_Data = string;

request->io_Length = length;

DoIO(request);

return;

}

/* output a NULL-terminated string of characters to a console */

ConPutStr(request,string)

struct IOStdReq *request;

char *string;

{

request->io_Command = CMD_WRITE;

request->io_Data = string;

request->io_Length = -1;        /* tells console to end when it

* sees a terminating zero on

* the string. */

DoIO(request);

return;

}

1.6.  CONTROL SEQUENCES FOR SCREEN OUTPUT

Here is a list of the functions that the console device supports,

along with the character stream that you must send to the console

to produce the effect.  Where the function table indicates multi-

ple  characters, it is more efficient to use the ConWrite() func-

tion rather than ConPutChar() since it  avoids  the  overhead  of

transfering  the  message  block multiple times.  The table below

uses the second form of <CSI>, that is,  the  hex  value  9B,  to

November 17, 1985

- 6 -

minimize  the number of characters to be transmitted to produce a

function.

In the table below, if an item is enclosed in square brackets, it

is  an optional item and may be omitted.  For example, for INSERT

[N] CHARACTERS, the value for N or M is shown as  optional.   The

console  device  responds  to such optional items by treating the

value of N as if it is not specified.

The value of N or M is always a decimal  number,  having  one  or

more ASCII digits to express its value.

Table 1-1: Console Control Sequences

COMMAND SEQUENCE OF         CHARACTERS         (in hexadecimal)

BACKSPACE (move left one column)            08

LINE FEED  (move down one text line         0A

as specified by the mode

function below)

VERTICAL TAB (move up one text  line)       0B

FORM FEED (clear the console's screen)      0C

CARRIAGE RETURN (move to first column)      0D

SHIFT IN   (undo SHIFT OUT)                 OE

SHIFT OUT  (set MSB of each character       0F

before displaying)

ESC (escape; can be part of the control     1B

sequence introducer)

CSI (control sequence introducer)           ESC [

RESET TO INITIAL STATE                      9B 63

INSERT [N] CHARACTERS                       9B [N] 40

(Inserts one or more spaces, shifting the

remainder of the line to the right.)

CURSOR UP [N]  CHARACTER POSITIONS          9B [N] 41

(default = 1)

CURSOR DOWN [N] CHARACTER POSITIONS         9B [N] 42

(default = 1)

CURSOR FORWARD [N] CHARACTER POSITIONS      9B [N] 43

(default = 1)

CURSOR BACKWARD [N] CHARACTER               9B [N] 44

POSITIONS (default = 1)

CURSOR NEXT LINE  [N]  (to column 1)        9B [N] 45

CURSOR PRECEDING LINE [N]                   9B [N] 46

(to column 1)

MOVE CURSOR TO ROW; COLUMN                  9B [N] [3B  N] 48

where N is row, M is column, and

semicolon (hex 3B) must be present

as a separator, or if row is left

out, so the console device can tell

that the number after the semicolon

actually represents the column  number.

ERASE TO END OF DISPLAY                     9B 4A

ERASE TO END OF LINE                        9B 4B

November 17, 1985

- 7 -

INSERT LINE (above the line containing      9B 4C

the cursor)

DELETE LINE (remove current line, move      9B 4D

all lines up one position to fill

gap, blank bottom line)

DELETE CHARACTER [N] (that cursor is        9B [N] 50

sitting on and to the right if

[N] is  specified)

SCROLL UP [N] LINES (Remove line(s) from    9B [N] 53

top of screen, move all other lines

down, blanks [N]  bottom  lines).

SCROLL  DOWN  [N]  LINES (Remove line(s)    9B [N] 54

from bottom of screen, move all

other lines up, blanks [N]  top  lines).

SET MODE (cause LINEFEED to respond as      9B 32 30 68

RETURN-LINEFEED)

RESET MODE (cause LINEFEED to respond       9B 32 30 6C

only as LINEFEED)

DEVICE STATUS REPORT (cause console to      9B 6E

insert into your read-stream a CURSOR

POSITION REPORT; see "Reading from

the  Console"  for  more  info).

SELECT GRAPHIC RENDITION See note below.    [S];[F];[B]

(select text style, foreground color,

background color)

NOTE:  For SELECT GRAPHIC RENDITION, any number of

parameters, in any order, are valid.  They are

separated by semicolons.

<style> =

0       plain text

1       bold-face

3       italic

4       underscore

7       inverse-video

<fg> =

30 - 37 selecting system colors 0-7 for foreground

transmitted as two ASCII characters

<bg> =

40 - 47 selecting system colors 0-7 for background

transmitted as two ASCII characters

For example,  to select  bold-face,  with  color  3  as

foreground   and   color  0  as  background,  send  the

sequence:

9B 31 3B 33 33 3B 34 30 6D

November 17, 1985

- 8 -

representing the ASCII sequence:

"<CSI>1;33;40m"

where <CSI> is the control  sequence  introducer,  here

used as the single-character value 9B hex.

The following are not ANSI standard sequences; they are

private Amiga sequences.

In the four command descriptions that  follow,  length,

width,  and  offset  are comprised of one or more ASCII

digits, defining a decimal value.

COMMAND SEQUENCE OF CHARACTERS  (in hexadecimal)

SET PAGE LENGTH (in character raster lines,

causes console to recalculate,

using current font, how many text       9B <length> 74

lines will fit on the page.

SET LINE LENGTH (in character positions,

using current font, how many characters

should be placed on each line).         9B <width> 75

SET LEFT OFFSET (in raster columns, how far

from the left of the window

should the text begin).                 9B <offset> 78

SET TOP OFFSET (in raster lines, how far

from the top of the window's

RastPort should the topmost

line of the character begin).           9B <offset> 79

NOTE:   The console normally  handles  the  above  four

functions  automatically.   To  allow it to do so again

after setting your own values, you can send the func-

tion without a parameter.

COMMAND SEQUENCE OF CHARACTERS  (in hexadecimal)

SET RAW EVENTS-see the separate topic

"Selecting Raw Input Events"

below for more details.

RESET RAW EVENTS-see "Selecting Raw Input Events" below.

November 17, 1985

- 9 -

SET CURSOR RENDITION - make the cursor

visible or invisible:

invisible:    9B 30 70

visible:      9B 70

WINDOW STATUS REQUEST - ask the console

device to tell you the current

bounds of the window, in

upper and lower row and column

character positions.

(User may have resized or repositioned

it).  See "Window Bounds Report" below.    9B 71

Examples

Move cursor right by 1:

Character string equivalents:   <CSI>C or <CSI>1C

Numeric (hex) equivalents:      9B 43 9B 31 43

Move cursor right by 20:

Character string equivalent:    <CSI>20C

Numeric (hex) equivalent:       9B 32 30 43

Move cursor to upper left corner (home):

Character string equivalents:

<CSI>H  or

<CSI>1;1H or

<CSI>;1H or

<CSI>1;H

Numeric (hex) equivalents:

9B 48

9B 31 3B 31 48

9B 3B 31 48

9B 31 3B 48

Move cursor to the forth column of the first line of the window:

November 17, 1985

- 10 -

Character string equivalents:

<CSI>1;4H or

<CSI>;4H

Numeric (hex) equivalents:

9B 31 3B 34 48

9B 3B 34 48

Clear the screen:

Character string equivalents:

<FF> or CTRL-L  {clear screen character} or

<CSI>H<CSI>J    {home and clear to end of screen} or

Numeric (hex) equivalents:

0C

9B 48 9B 4A

Reading input from the console device returns an ANSI 3.64  stan-

dard  byte  stream.   This  stream  may contain normal characters

and/or RAW input event information.  You may also  request  other

RAW  input  events  using the SET RAW EVENTS and RESET RAW EVENTS

control sequences discussed below.  See "Selection of  Raw  Input

Events".

The following subroutines are useful for setting up  for  console

reads.  Only a single-character-at-a-time version is shown here.

NOTE:  This example does not illustrate the fact that  a  request

for  more  than  one character can be satisfied by only one, thus

requiring you to look at io_actual.

/* queue up a read request to a console, show where to

* put the character when ready to be returned.  Most

* efficient if this is called right after console is

* opened */

QueueRead(request,whereto)

struct IOStdReq *request;

char *whereto;

{

request->io_Command = CMD_READ;

request->io_Data = whereto;

request->io_Length = 1;

SendIO(request);

return;

}

November 17, 1985

- 11 -

/* see if there is a character to read.  If none, don't wait,

* come back with a value of -1 */

int

ConMayGetChar(consolePort,request,whereto)

struct Port *consolePort

struct IOStdReq *request;

char *whereto;

{

register temp;

if ( GetMsg(consolePort) == NULL ) return(-1);

temp = *whereto;

QueueRead(request,whereto);

return(temp);

}

/* go and get a character; put the task to sleep if

* there isn't one present */

UBYTE

ConGetChar(consolePort,request,whereto)

struct IOStdReq *request;

struct Port *consolePort;

char *whereto;

{

register temp;

while((GetMsg(consolePort) == NULL)) WaitPort(consolePort);

temp = *whereto;        /* get the character */

QueueRead(request,whereto);

return(temp);

}

For the most part, keys whose keycaps are labeled with ANSI stan-

dard  characters  will  ordinarily be translated into their ASCII

equivalent character by the console device through the use of its

keymap.  A separate section in this chapter has been dedicated to

the method used to establish a keymap and the internal  organiza-

tion of the keymap.

For keys other than  those  with  normal  ASCII  equivalents,  an

escape sequence is generated and inserted into your input stream.

For example, in the default state (no raw input events  selected)

the function and arrow keys will cause the following sequences to

be inserted in the input stream:

November 17, 1985

- 12 -

Table 1-2: Special Key Report Sequences

center box; c c l.

KEY     UNSHIFTED SENDS SHIFTED SENDS

F1      <CSI>0~ <CSI>10~                 F2      <CSI>1~ <CSI>11~

F3      <CSI>2~ <CSI>12~                 F4      <CSI>3~ <CSI>13~

F5      <CSI>4~ <CSI>14~                 F6      <CSI>5~ <CSI>15~

F7      <CSI>6~ <CSI>16~                 F8      <CSI>7~ <CSI>17~

F9      <CSI>8~ <CSI>18~                 F10     <CSI>9~ <CSI>19~

HELP    <CSI>?~ <CSI>?~  (same)

Arrow keys:

Up      <CSI>A  <CSI>T~                   Down    <CSI>B  <CSI>S~

Left    <CSI>C  <CSI> A~ (notice the space)

Right   <CSI>D  <CSI> @~ (notice the space)

If you have sent the DEVICE STATUS REPORT command  sequence,  the

console  device  returns a cursor position report into your input

stream.  It takes the form:

<CSI><row>;<column>R

For example, if the cursor is at column 40, and row 12, here  are

the ASCII values you receive in a stream:

9B 34 30 3B 31 32 52

A user may have either moved or resized the window to which  your

console  is bound.  By issuing a WINDOW STATUS REPORT to the con-

sole, you can read the current position and  size  in  the  input

stream.  This window bounds report takes the following form:

<CSI>1;1;<bottom margin>;<right margin>r

Note that the top and left margins are always 11 for  the  Amiga.

The  bottom  and right margins give you the window row and column

dimensions as well.  For a window that holds  20  lines  with  60

characters  per line, you will receive the following in the input

stream:

9B 31 3B 31 3B 32 30 3B 36 30 73

November 17, 1985

- 13 -

If the keyboard information, including "cooked"  keystrokes  does

not  give  you  enough  information  about  input  events you can

request additional information from the console driver.

The command to SET RAW EVENTS is formatted as:

"<CSI>[event-types-separated-by-semicolons]{"

If, for example, you need to know when each key  is  pressed  and

released you would request "RAW keyboard input".  This is done by

writing "<CSI>1{" to the console.  In a  single  SET  RAW  EVENTS

request,  you  can  ask  the  console to setup for multiple event

types at one time.  You must send  multiple  numeric  parameters,

separating  them  by  semicolons  (;).   Example:  ask for gadget

pressed, gadget released and  close  gadget  events   -    write:

"<CSI>7;8;11{" (all as ASCII characters, without the quotes).

You can reset, that is, delete from reporting, one or more of the

raw  input  event types by using the RESET RAW EVENTS command, in

the same manner as the SET RAW EVENTS was used to establish  them

in the first place.  This command stream is formatted as:

<CSI>[event-types-separated-by-semicolons]}

So, for example, you could reset all of the  events  set  in  the

above    example    by   transmitting   the   command   sequence:

"<CSI>7;8;11}" Here is a list of the valid raw input event types:

Table 1-3: Raw Input Event Types

REQUEST NUMBER  DESCRIPTION

0       nop     Used internally.

1       RAW     keyboard input. Intuition swallows all by the select

button.

2       RAW mouse input

3       Event Sent whenever your window is made active.

4       Pointer position

5       (unused)

6       Timer

7       Gadget pressed

8       Gadget released

9       Requester activity

10      Menu numbers

11      Close Gadget

12      Window resized

13      Window refreshed

14      Preferences changed (not yet implemented)

15      Disk removed

16      Disk inserted

November 17, 1985

- 14 -

1.7.  COMPLEX INPUT EVENT REPORTS

If you select any of these events you will start to get  informa-

tion about the events in the following form:

<CSI><class>;<subclass>;<keycode>;<qualifiers>;<x>;<y>;

<seconds>;<microseconds>|

where:

<CSI>

is a one-byte field.  It  is  the  "control  sequence  intro-

ducer", 9B in hex.

<class>

is the RAW input event type, from the above table.

<subclass>

is usually 0.  If the mouse is moved to the right controller,

this would be 1.

<keycode>

indicates which key number was pressed (see  figure  4-1  and

table  4-5).   This field can also be used for mouse informa-

tion.

<qualifiers>

indicates the state of the keyboard and system.  The  qualif-

iers are defined as follows:

November 17, 1985

- 15 -

Table 1-4: Input Event Qualifiers

BIT     MASK    KEY

0       0001    left   shift

1       0002    right    shift

2       0004    capslock  *Associated keycode is

special; see below.

3       0008    control

4       0010    left ALT

5       0020    right  ALT

6       0040    left Amiga key pressed

7       0080    right Amiga key pressed

8       0100    numeric pad

9       0200    repeat

10      0400    interrupt  Not currently used.

11      0800    multi broadcast  This window (active one)

or all windows.

12      1000    left mouse button

13      2000    right mouse button

14      4000    middle mouse button  (Not available on

standard  mouse.)

15      8000    relative mouse  Indicates mouse coordinates

are relative, not absolute.

The CAPS LOCK key is handled in a special manner.  It  only  gen-

erates  a  keycode  when  it is pressed, not when it is released.

However the up/down bit (80 hex) is still used and reported.   If

pressing  the caps lock key causes the LED to light then key code

62 (caps lock pressed) is sent.  If pressing the  caps  lock  key

extinguishes  the  LED  then key code 190 (caps lock released) is

sent.  In effect, the keyboard reports  this  key  as  held  down

until it is struck again.

The <x> and <y> fields are filled by some classes with an  Intui-

tion address:  x<<16+y.

The <seconds> and <microseconds> fields contain the  system  time

stamp  taken  at  the  time the event occurred.  These values are

stored as long-words by the system.

With RAW keyboard input selected, keys will no  longer  return  a

simple  one-character  "A" to "Z" but will rather return raw key-

code reports of the form:

<CSI>1;0;<keycode>;<qualifiers>;0;0;<seconds>;<microseconds>|

For example, if the user pressed and released the  "B"  key  with

the  left  shift  and  right  Amiga  keys  also pressed you might

receive the following data:

November 17, 1985

- 16 -

<CSI>1;0;35;129;0;0;23987;99|

<CSI>1;0;163;129;0;0;24003;18|

The <keycode> field is an ASCII decimal  value  representing  the

key pressed or released.  Adding 128 to the pressed key code will

result in the released keycode.   Figure  4-1  lets  you  convert

quickly  from  a  key  to its keycode. The tables let you convert

quickly from a keycode to a key.

Figure 1-1: The Amiga Keyboard, Showing Keycodes in Hex

The default values given correspond to:

1)  The values the console device will return when these keys

are pressed, and

2)  The Keycaps as shipped with the  standard  American  key-

board.

November 17, 1985

- 17 -

Table 1-5: System Default Console Key Mapping

RAW     UNSHIFTED       SHIFTED

KEY     KEYCAP  DEFAULT DEFAULT NUMBER  LEGEND  VALUE   VALUE

00      ` ~     ` (Accent grave)        ~  (tilde)

01      1    !     1       !

02      2    @     2       @

03      3    #     3       #

04      4    $     4       $

05      5    %     5       %

06      6    ^     6       ^

07      7    &     7       &

08      8    *     8       *

09      9    (     9       (

0A      0  )     0       )

0B      - _     -          (Hyphen)      _   (Underscore)

0C      =    +     =       +

0D      \\     |    \\      |

0E       (undefined)

0F      0       0       0 (Numeric pad)

10      Q       q       Q

11      W       w       W

12      E       e       E

13      R       r       R

14      T       t       T

15      Y       y       Y

16      U       u       U

17      I       i       I

18      O       o       O

19      P       p       P

1A      [{     [       {

1B      ]           }     ]       }

1C              (undefined)

1D      1       1       1  (Numeric pad)

1E      2       2       2   (Numeric   pad)

1F      3       3       3 (Numeric pad)

20      A       a       A           21      S       s       S

22      D       d       D           23      F       f       F

24      G       g       G           25      H       h       H

26      J       j       J           27      K       k       K

28      L       l       L

29      ; :     ;       :

2A      '       "     ' (single quote)    "

2B              (RESERVED)      (RESERVED)

2C              (undefined)

2D      4       4       4  (Numeric pad)

2E      5       5       5   (Numeric   pad)

2F      6       6       6 (Numeric pad)

30              (RESERVED)      (RESERVED)

31      Z       z       Z

32      X       x       X

33      C       c       C

34      V       v       V

35      B       b       B

36      N       n       N

37      M       m       M

38      ,  <     ,  (comma)       <

39      .  >     .  (period)      >

3A      / ?     /       ?

3B              (undefined)

3C      .       .       . (Numeric pad)

3D      7       7       7 (Numeric pad)

3E      8       8       8 (Numeric pad)

3F      9       9       9 (Numeric pad)

40      (Space        bar)     20      20

41      BACKSPACE      08      08

42      TAB     09      09

43      ENTER   0D      0D   (Numeric pad)

44      RETURN  0D      0D

45      ESC     1B      1B

46      DEL     7F      7F

47              (undefined)

November 17, 1985

- 18 -

48              (undefined)

49              (undefined)

4A      -       -       -  (Numeric Pad)

4B              (undefined)

4C      Up Arrow        <CSI>A~ <CSI>T~

4D      Down Arrow      <CSI>B~ <CSI>S~

4E      Forward Arrow   <CSI>C~ <CSI> A~         [1]

4F      Backward Arrow  <CSI>D~ <CSI> @~

_________________________

[1] In shifted Forward Arrow and Backward arrow, note

blank space after <CSI>.

November 17, 1985

- 19 -

RAW             UNSHIFTED       SHIFTED

KEY     KEYCAP  DEFAULT DEFAULT NUMBER  LEGEND  VALUE   VALUE

50      F1      <CSI>0~ <CSI>10~

51      F2      <CSI>1~ <CSI>11~

52      F3      <CSI>2~ <CSI>12~

53      F4      <CSI>3~ <CSI>13~

54      F5      <CSI>4~ <CSI>14~

55      F6      <CSI>5~ <CSI>15~

56      F7      <CSI>6~ <CSI>16~

57      F8      <CSI>7~ <CSI>17~

58      F9      <CSI>8~ <CSI>18~

59      F10     <CSI>9~ <CSI>19~

5A              (undefined)

5B              (undefined)

5C              (undefined)

5D              (undefined)

5E              (undefined)

5F      HELP    <CSI>?~ <CSI>?~

November 17, 1985

- 20 -

KEY     KEYCAP NUMBER  LEGEND

60      SHIFT (left of space bar)

61      SHIFT (right of space bar)

62      CAPS LOCK

63      CTRL

64      (Left) ALT

65      (Right) ALT

66      Amiga   (left of space bar)       Close Amiga

67      Amiga (right of space bar)      Open Amiga

68      Left  mouse  button       Inputs  are  only  for  the

one (not converted)    mouse connected to Intuition

69      Right mouse button        currently "gameport"

one.                (not converted)

6A      Middle mouse button           (not converted)

6B      (undefined)

6C      (undefined)

6D      (undefined)

6E      (undefined)

6F      (undefined)

November 17, 1985

- 21 -

center; l10 l.  RAW KEY NUMBER  FUNCTION

70-7F   (undefined)

80-F8   Up transition (release or unpress key of one

of the above keys).  80 for 00, F8 for 7F.

F9      Last keycode was bad           (was sent in order  to

resynchronize).

FA      Keyboard buffer overflow.

FB      (undefined, reserved for keyboard processor

catastrophe)

FC      Keyboard selftest failed.

FD      Power-up key stream start.  Keys pressed or

stuck at power-up will be sent between FD and FE.

FE      Power-up key stream end.

FF      (undefined, reserved)

FF      Mouse event, movement only, no button change.

(not converted)

Notes about the preceding table:

1)  "(undefined)" indicates that the current keyboard  design

should  not  generate this number.  If you are using Set-

KeyMap() to change the key  map  the  entries  for  these

numbers must still be included.

2)  "(not converted)" refers to  mouse  button  events.   You

must  use  the  sequence  "<CSI>2{" to inform the console

driver that you wish to receive mouse  events;  otherwise

these will not be transmitted.

3)  "(RESERVED)" Indicates  that  these  keycodes  have  been

reserved for non-US keyboards.  The "2B" code key will be

between the double-quote(") and RETURN  keys.   The  "30"

code key will be between the SHIFT and "Z" keys.

1.8.  KEYMAPPING

The Amiga has the capability  of  mapping  the  keyboard  in  any

manner  that  you  wish.   In other computers, this capability is

normally provided through the use of  "keyboard  enhancers".   In

the  Amiga,  however,  the  capability is already present and the

vectors that control the remapping are user accessible.

The functions called GetKeyMap() and SetKeyMap() each deal with a

set  of  8 longword pointers, known as the KeyMap data structure.

The KeyMap data structure is shown below.

November 17, 1985

- 22 -

struct KeyMap {

APTR km_LoKeyMapTypes;

APTR km_LoKeyMap;

APTR km_LoCapsable;

APTR km_LoRepeatable;

APTR km_HiKeyMapTypes;

APTR km_HiKeyMap;

APTR km_HiCapsable;

APTR km_HiRepeatable;

};

GetKeyMap() returns a pointer to this table of pointers,  showing

where  in  memory  each of the tables representing the keymapping

may be found.

As a prelude to the following material, note that the Amiga  key-

board  transmits  raw key information to the computer in the form

of a key position and a transition.  Figure 4-1 shows a  physical

layout of the keys and the hexadecimal number that is transmitted

to the system when a key is pressed.  When the key  is  released,

its  value,  plus hexadecimal 80, is transmitted to the computer.

The key mapping described herein refers to the  translation  from

this raw key transmission into console device output to the user.

The low key map provides translation of the key values  from  hex

00-3F;  the  high  key map provides translation of key value from

hex 40-67.

Raw output from the keyboard for the LoKeyMap  does  not  include

the  space bar, TAB, ALT, CTRL, arrow keys and several other keys

(shown in HiKeyMap, below).

Figure 1-2: Low Key Map Translation Table

November 17, 1985

- 23 -

Table 1-6: High Key Map Hex Values

40      Space

41      Backspace

42      Tab

43      Enter

44      Return

45      Escape

46      Delete

4A      Numeric Pad

4C      Cursor Up

4D      Cursor  Down

4E      Cursor  Forward

4F      Cursor Backward

50-59   Function keys F1-F10

5F      Help

60      Left  Shift

61      Right   Shift

62      Caps   Lock

63      Control

64      Left  Alt

65      Right Alt

66      Left Amiga

67      Right Amiga

The keymap table for the low and high keymaps consists of  4-byte

entries,  one  per hex keycode.  These entries are interpreted in

one of two possible ways:

a.  as 4 separate bytes, specifying how  the  key  is  to  be

interpreted when pressed:

o   alone

o   with one qualifier

o   with another qualifier

o   with both qualifiers

where a qualifier is one of three possible keys:

o   CTRL (control)

o   ALT

o   SHIFT

b.  as a longword containing the address of a string descrip-

tor,  where  a  string of hex digits is to be output when

this key is pressed.  If a string is to  be  output,  any

combination  of qualifiers may affect the string that may

be transmitted.

NOTE:   The keymap table must begin aligned on a  word  boundary.

Each  entry  is  4-bytes long, thereby maintaining word alignment

throughout the table.   This  is  necessary  since  some  of  the

entries  may  be  longword addresses and must be aligned properly

for the 68000.

As you may have noticed, there are three possible qualifiers, but

only  a  4-byte  space  in the table for each key.  This does not

allow space to describe what the computer should output  for  all

possible  combinations  of qualifiers.  This problem is solved by

November 17, 1985

- 24 -

only allowing all three qualifiers to affect the  output  at  the

same time in string mode.  Here is how that works.

For "vanilla" keys, such as the alphabetic keys, use the 4  bytes

to  represent  the  data  output  for the key alone, shifted key,

ALT'ed key, and shifted-and-ALT'ed key.  Then for  the  CTRL-key-

plus-vanilla-key,  use the code for the key alone with bits 6 and

5 set to 0.

For other keys, such as the return key or escape key, the qualif-

iers  specified in the keytypes table (up to two) are the qualif-

iers used to establish the response to the key.  This is done  as

follows.   In  the  keytypes table, the values listed for the key

types are  those  listed  for  the  qualifiers  in  keymap.h  and

keymap.i.  Specifically, these qualifier equates are:

KC_NOQUAL       0x00

KCF_SHIFT       0x01

KCF_ALT         0x02

KCF_CONTROL     0x04

KC_VANILLA      0x07

KCF_DOWNUP      0x08

KCF_STRING      0x40

As shown above, the qualifiers for  the  various  types  of  keys

occupy specific bit positions in the key types control byte.

A keymap table entry looks like this, in assembly code:

SOME_KEY:

DC.B    VALUE_1, VALUE_2, VALUE_3, VALUE_4

Here is how you interpret the keymap for various combinations  of

the qualifier bits:

November 17, 1985

- 25 -

Table 1-7: Keymap Qualifier Bits

If  Keytype is:   Then value in this position in the

keytable is out put when the key is

pressed along with:

KC_NOQUAL       -       -       -       alone

KCF_SHIFT       -       -       SHIFT   alone

KCF_ALT -       -       ALT     alone

KCF_CONTROL     -       -       CTRL    alone

KCF_ALT+KCF_SHIFT       SHIFT+ALT       ALT     SHIFT   alone

KCF_CONTROL+KCF_ALT     CTRL+ALT        CTRL    ALT     alone

KCF_CONTROL+KCF_SHIFT   CTRL+SHIFT      CTRL    SHIFT   alone

KC_VANILLA      SHIFT+ALT       ALT     SHIFT   alone*

*   special case-CTRL key, when pressed with one of  the  al-

phabet  keys  and  certain others, is to output key-alone

value with the bits 6 and 5 set to zero.

The vectors named km_LoKeyTypes  and  km_HiKeyTypes  contain  one

byte  per raw key code.  This byte defines the entry type that is

made in the key table by a set of bit positions.

Possible key types are:

o   any of the qualifier groupings noted above, or

o   KCF_STRING +   any  combination  of  KCF_SHIFT,  KCF_ALT,

KCF_CONTROL  (or KC_NOQUAL) if the result of pressing the

key is to be a  stream  of  bytes  (and  key-with-one-or-

more-qualifiers is to be one or more alternate streams of

bytes).

Any key can be made to output up to 8 unique byte streams

if KCF_STRING is set in its keytype.  The only limitation

is that the total length of all of the  strings  assigned

to  a  key  be  within  the "jump range" of a single byte

increment.  See the "String-Output  Keys"  section  below

for more information.

The low keytype table covers the raw keycodes from hex 00-3F, and

contains  one byte per keycode.  Therefore this table contains 64

(decimal) bytes.  The high keytype table covers the raw  keycodes

from hex 40-67, and contains 38 (decimal) bytes.

When a key is to output a string, the keymap table  contains  the

address  of a string descriptor in place of a 4-byte mapping of a

key as shown above.  Here is a partial table for a new  high  key

map  table which contains only three entries thus far.  The first

November 17, 1985

- 26 -

two are for the space bar and the backspace key; the third is for

the  tab  key, which is to output a string that says "[TAB]".  An

alternate string, "[SHIFTED-TAB]", is also to be  output  when  a

shifted TAB key is pressed.

Table 1-8: Composing an Alternate Key Map

newHiMapTypes:

DC.B        KCF_ALT,KC_NOQUAL,

DC.B        KCF_STRING+KCF_SHIFT,         ...     ;(more)

newHiMap:

DC.B        0,0,$A0,$20     ;space bar, and ALT-space bar

DC.B        0,0,0,$08       ;BACKSPACE key only

DC.L        newkey42        ;new  definition  for  string  to

;output for TAB key         ...     ;(more)

newkey42:

DC.B        new42ue   -    new42us       ;length    of    the

;unshifted string

DC.B        new42us  -  newkey42

;number of bytes from start of

;string descriptor to start of

;this string

DC.B        new42se - new42ss   ;length of  the shifted string

DC.B        new42ss  -  newkey42

;number of bytes  from  start of

;string descriptor to start of

;this string

new42us:

DC.B        '[TAB]'

new42ue:

new42ss:

DC.B        '[SHIFTED-TAB]'

new42se:

The new high map table points to the string descriptor at address

newkey42.   The  new  high map types table says that there is one

qualifier, which means that there are  two  strings  in  the  key

string descriptor.

Each string in the descriptor takes two bytes in this part of the

table;  the  first  byte  is the length of the string, the second

byte is the distance from the start  of  the  descriptor  to  the

start  of  the  string.  Therefore, a single string (KCF_STRING +

KC_NOQUAL) takes 2 bytes of string descriptor.  If there  is  one

qualifier,  4  bytes  of  descriptor  are used.  If there are two

qualifiers, 8 bytes of descriptor are used.  If there are 3 qual-

ifiers,  16  bytes  of  descriptor  are  used.  All strings start

immediately following the string  descriptor  in  that  they  are

November 17, 1985

- 27 -

accessed  as single byte offsets from the start of the descriptor

itself.  Therefore, the distance from the start of the descriptor

to  the  last string in the set (the one that uses the entire set

of specified qualifiers) must  start  within  255  bytes  of  the

descriptor address.

Since the length of the string is contained in a single byte, the

length  of any single string must be 255 bytes or less while also

meeting the "reach" requirement.

The length of a keymap containing string descriptors and  strings

is  variable  and  depends  on the number and size of the strings

that you provide.

The vectors called km_LoCapsable and km_HiCapsable point  to  the

first  byte  in  an  8-byte  table that contains more information

about the keytable entries.

Specifically, if the CAPS LOCK key has been pressed the CAPS LOCK

LED  is  on),  and  if  there is a bit on in that position in the

capsable map, then this key will be treated as though the  shift-

key  is  now  currently  pressed. For example, in the default key

mapping, the alphabetic keys are "capsable" but  the  punctuation

keys  are not.  This allows you to set the shift-lock, just as on

a normal typewriter,  and  get  all  capital  letters.   However,

unlike  a normal typewriter, you need not go out of shift-lock to

correctly type the punctuation symbols or numeric keys.

In the table, the bits that control  this  feature  are  numbered

from  the lowest bit in the byte, and from the lowest memory byte

address to the highest.  For example, the bit representing  caps-

able  status  for  the key that transmits raw code 00 is bit 0 in

byte 0; for the key that transmits raw code 08 it's bit 0 in byte

1,  and so on.

There are 64 bits (8-bytes) in each of the two capsable tables.

For both the low and high key maps there is an 8-byte table  that

provides  one  bit per possible raw key code.  This bit indicates

whether or not the specified key should repeat at the rate set by

the  Preferences  program.  The bit positions correspond to those

specified in the capsable bit table.

If there is a 1 in a specific position, the key can repeat.   The

vectors that point to these tables are called km_LoRepeatable and

km_HiRepeatable.

In the default low key map, all of the keys are  treated  in  the

same manner.  That is:

o   When pressed alone, they transmit the ASCII equivalent of

November 17, 1985

- 28 -

the unshifted key.

o   When shifted, they translate the ASCII equivalent of  the

shifted value when printed on the keycap.

o   When "ALT'ed" (pressed  along  with  an  ALT  key),  they

transmit  the alone-value with the high bit of a byte set

(value plus hex 80).

When shifted and ALT'ed, they transmit the  shifted-value

plus hex 80.

In this table, the bytes that describe the data to be transmitted

are positioned as the example for the "A" key shown here:

center; l l l  l.

key_A   DC.B    ('A')+$80       ;shifted and ALT'ed

DC.B    ('a')+$80       ;ALT'ed only

DC.B    ('A')   ;shifted only

DC.B    ('a')   ;not shifted or ALT'ed.

In addition to the response to the key alone, shifted, ALT'ed and

shifted-and-ALT'ed,  the  default low keymap also responds to the

key combination of "CTRL + key" by stripping off bits 6 and 5  of

the  generated  data  byte.   For example, CTRL + A generates the

translated keycode 01 (61 with bits 6 and 5 set to 0).

All keys in the low key map are mapped to their ASCII equivalents

as noted in the low key map key table shown above.

Since the low key table contains 4 bytes per key,  and  describes

the  keys (raw codes) from hex 00-3F, there are 64 times 4 or 256

bytes in this table.

Most of the keys in the high key map generate strings rather than

single  character mapping. The following keys map characters with

no qualifier, along with their byte mapping:

KEY     GENERATES VALUE:

BACKSP  $08 ENTER   $0D DEL     $7F

November 17, 1985

- 29 -

The following keys map characters and use a single qualifier:

KEY     GENERATES   VALUE:        IF   USED    WITH    QUALIFIER,

GENERATES VALUE:

SPACE   $20     $A0 (qualifier = ALT)

RETURN  $0D     $0A  (qualifier = CONTROL)

ESC     $1B     $9B (qualifier = ALT)

numeric pad '-' $2D     $FF (qualifier = ALT)

The following keys generate strings:

KEY     GENERATES    VALUE:        IF    USED    WITH    <SHIFT>,

GENERATES VALUE:

TAB   $09     $9B, followed by 'Z'

cursor:

UP    $9B, followed by 'A'    $9B, followed by 'T'

DOWN  $9B, followed by 'B'    $9B, followed by 'S'

FWD   $9B,  followed  by   'C'    $9B,   followed   by   '   ',

followed by '@'

BACKWD  $9B, followed by 'D'    $9B,  followed  by  '  ',

followed by 'A'

function keys:

F1    $9B, followed by '0~'   $9B, followed by '10~'

F2    $9B, followed by '1~'   $9B, followed by '11~'

F3    $9B, followed by '2~'   $9B, followed by '12~'

F4    $9B, followed by '3~'   $9B, followed by '13~'

F5    $9B, followed by '4~'   $9B, followed by '14~'

F6    $9B, followed by '5~'   $9B, followed by '15~'

F7    $9B, followed by '6~'   $9B, followed by '16~'

F8    $9B, followed by '7~'   $9B, followed by '17~'

F9    $9B, followed by '8~'   $9B, followed by '18~'

F10   $9B, followed by '9~'   $9B, followed by '19~'

HELP    $9B, followed by '?~'   (no qualifier used)

November 17, 1985

- 30 -

1.9.  CLOSING A CONSOLE DEVICE

When you have finished using a console, it must be closed so that

the memory areas it utilized may be returned to the system memory

manager.  Here is a sequence that you can use to close a  console

device:

CloseDevice(requestBlock);

Note that you should also delete the messages and  ports  associ-

ated with this console after the console has been closed:

DeleteStdIO(consoleWriteMsg);

DeleteStdIO(consoleReadMsg);

DeletePort(consoleWritePort);

DeletePort(consoleReadPort);

If you have finished with the window used for the console device,

you can now close it.

November 17, 1985

#####################################