FILE CALLS SUMMARY

==================

High Common

-----------

FCSERR	(see explanation under low common)

High Routines

-------------

call OPEN. (fil$,acc$,channel%,recordsize%)	fil$ is full or partial spec e.g.

"SY0:[123,321]WITTENOOM.ASS;12345"

or "FRED"

acc$ =	"R" for read only

"W" for write (=create new version of file)

"M" for modify (=update but no extend)

"U" forupdate (=normal access mode)

channel% = user assigned number in range 20:30 inclusive

(channel 20% uses LUN 20, 21% uses LUN 21, etc.)

recordsize% = record size in bytes

call OPENA. (Filename$,Channel%)		Append mode to a text file - create file if it doesn't exist

call OPENU. (Filename$,Channel%,Record.Size%)	Update mode to a record file - create file if it doesn't exist

call CLOSE. (channel%)				channel% as for OPEN.

call GET. (channel%,buffer$,lnmt%)		channel% as for OPEN.

buffer$ is destination of read & must be recordsize%(of OPEN.) bytes long

lmnt% is element (record) number to read

lowest lnmt% is 1%

call PUT. (channel%,buffer$,lnmt%)		channel% as for OPEN.

buffer$ is destination of write & must be recordsize%(of OPEN.) bytes long

lmnt% is element (record) number to write

lowest lnmt% is 1%

call GET.N (channel%,buffer$)			as for GET. but reads NEXT element

call PUT.N (channel%,buffer$)			as for PUT. but writes NEXT element

call GETS. (channel%,buffer$,lmnt%,number%)	as for GETs but gets number% elements starting at #lmnt%

call PUTS. (channel%,buffer$,lmnt%,number%)	as for PUTs but puts number% elements starting at #lmnt%

call PUT.A (channel%,buffer$)			as for PUT. but writes just after last current record: PUT.Append

call FCSNOB (channel%,numberofbytes)		returns the number of bytes in the file open on channel%

call FCSNOR (channel%,numberofrecords)		returns the number of records in the file open on channel%

Low Common

----------

FCSERR	ERR1%	error code from FCS (0%=OK)

ERR2%	error code from FCS: directive status code

FCSCA1	INI%		=-1 after FCSINI called (root it)

FCSCA2	FDB$(10%)=96%	FDBs for channels #20%:#30% (luns 20.:30.) (root it)

FCSCA3	FDBADR%(11%)	[0%]=junk (root it)

[i%]=address of FDB$(i%-1%)

FCSCA4	A.$		OPEN$ Mode Bytes (root it)

FCSCA5	A(1%),A$=1%	local scratch for FCSDIE (does not need to live in root)

FCSCA6	WHERE$=4%	where we got last input record using INOUT. (root it)

FCSCA7	FCSWHO%		which channel% INOUT. last used (root it)

Low Routines

------------

FCSINI						macro - read it

FCSOPN by ref (fdbadr%,lun%,acc%,fil${,recz%})	macro - read it

FCSCLS by ref (fdbadr%)				macro - read it

FCSGET by ref (fdbadr%{,buf${,lolmnt%{,hilmnt%}}}})macro - read it

FCSPUT by ref (fdbadr%{,buf${,lolmnt%{,hilmnt%}}}})macro - read it

FCSTEL by ref (fdbadr%,where$)			macro - read it

FCSSEK by ref (fdbadr%,where$)			macro - read it

C.	(C%)					checks C% is legal channel #

L.	(L%)					checks L% is legal record number

N.	(N%)					checks N% is >0%

FCSFDB						not executable: initialises FDBADR% of F.C.S.

Sequential Files

----------------

To open a sequential (typable) file just use recordsize%=0%.

Sequential files have variable record sizes (each record may have a different size).

So you must always say how long a record is on output & find out how long a record is on input.

Output

------

call BZSET. (channel%,buffersize%) to say how long an output record is.

Then call PUT.N (channel%,buffer$) to output the first buffersize% bytes of buffer$.

Then check FDBERR to see if it worked.

Input

-----

call BZSET. (channel%,buffersize%) to set the maximum input line length allowed.

Then call GET.N (channel%,buffer$) which will input the next record (line) to buffer$.

If the line is longer than buffersize% then an error is noted in FDBERR and only

the first buffersize% characters of the line appear in buffer$.

Then call BZENQ. (channel%,buffersize1%,buffersize2%).

This will return buffersize1% & buffersize2%.

buffersize2% will be the number of characters input to buffer$, and this is

normally the record length.

Naturally, the first buffersize2% characters of buffer$ are used and the

rest of buffer$ is unchanged.

Names

-----

BZSET. stands for Buffer siZe SET.

BZENQ. stands for Buffer siZe ENQuire.

High-Level Routines

-------------------

call INPUT. (channel%,buffer$,size%)	Inputs a record to buffer$.

returns size% = actual record length got

rest of buffer$ is unchanged

call INOUT. (channel%,buffer$,size%)	as for INPUT. but remembers where we got

record from for OPRIN. to use

call PRINT. (channel%,buffer$)		outputs a record from buffer$

call OPRIN. (buffer$)			as for PRINT. but overprints last record read by INOUT.

uses same channel% as last INOUT. call

call PRINL. (channel%,buffer$,size%)	outputs a record that is the first size% bytes of buffer$

Task-Building

-------------

Specify new subroutines (or new main)

-------------------------------------

just type:

MCR>#BUILD

this will build your .ODL & .CMD files

Build new .TSK file

-------------------

MCR>@progname

where progname is the main-program-name that you told #BUILD

[end: FCSCALLS.DOC]