PROLOD release notes

--------------------

PROLOD  is a  system server  task which  is called  via the  POSSUM

library   to   load  device   drivers.    The   task  image   is   named

LB:[ZZSYS]PROLOD.TSK and it is automatically  installed by the system at

boot time.  The installed task name is $LOAD.

Since PROLOD was not ready for release at the time P/OS version 2.0

was  shipped, a  PROLOD  server has  been provided  with  the file  name

PROLODV2.TSK.  For  P/OS version 2.0  or 2.0A  ONLY, you must  copy this

file to LB:[ZZSYS]PROLOD.TSK to use  loadable drivers.  For P/OS version

3.0 and later, the file is  already included on the distribution kit and

will be automatically installed with the rest of P/OS.

There is a minor bug with  the PROLODV2.TSK distributed on the P/OS

V3.0  PRODCL2 floppy;  it  is  corrected in  the  task  image with  this

distribution,  and  an  indirect  command  file for  ZAP  to  apply  the

correction to the PROLODV2.TSK distributed on  a P/OS version 3.0 kit is

also included.  The problem is  that the documented feature of assigning

the logical name PROLOD$MSG the equivalence of 0 to cause PROLOD to type

diagnostic  messages on  your terminal  will  not work  on 2.0  systems.

Analysis of the problem shows that it is either a documentation error in

the TLOG$  series of macros,  or a bug  in the TLOG$  macros themselves,

take your  pick.  The bottom line  is that if  you code the source  of a

program using  any of the TLOG$  macros (TLOG$, TLOG$C, or  TLOG$S), and

you place  a zero in  the third field,  which is currently  reserved, as

opposed  to  just  leaving  the  field blank,  the  DPB  generated  when

assembling on  a 3.0 system  will be 10. words  instead of the  9. words

generated on a 2.0 system.  The DPB will be regarded as invalid on a 2.0

system,  and  the  request  will  always fail  with  the  error  IE.SDP,

indicating invalid DIC or DPB size.  PROLOD does not check for the cause

of  the failure;  it  just assumes  the translation  failed  due to  the

absence of  the logical, and skips  typing the output message.   The ZAP

patch  corrects the  DPB to  a 2.0-compatible  DPB, and  then everything

works fine.   By the  way, if you  desire your code  to work  on pre-3.0

systems, the fix currently is to leave  the third field of a TLOG$ macro

blank  instead of  placing  a zero  in  it.   This causes  a  DPB to  be

generated that will work on all versions of P/OS.

There are  several errors  in the  version 2.0  Tool Kit  "Guide to

Writing an I/O Driver and Advanced Programmer Notes" which could confuse

the driver developer.

1    The  illustration  of the  KRB  in  figures  4-13 and  4-15  is

incorrect.  The  first two  words shown  are K.ICSR  and K.SLT.

These two words are actually located between K.PRM and K.VCT as

follows:

+-------------------------------+

K.PRM          |   Driver dependent storage    |

+-------------------------------+

K.ICSR         |   interrupt controller CSR    |

+-------------------------------+

K.SLT          |   reserved    | slot number   |

+-------------------------------+

K.VCT/K.CON    |   vector/4    | contrl. index |

+-------------------------------+

.              |               .               |

.              /               .               /

.              /               .               /

.              |               .               |

+-------------------------------+

2    Many of the labels, such as $DAT, described in the manual imply

the  the device  name is  optional.  It  is not  optional.  The

label in  the example  mentioned should be  of the  form $xxDAT

where "xx" is the logical device name associated with the first

DCB in the driver's database (D.NAM).

3    The "dev"  argument in  the GTPKT$, DDT$,  and INTSV$  macro is

described as being optional.  It is not.

4    The order  of the  last two DDT$  macro arguments  are reversed

(p4-5).  The correct format is:

DDT$    dev,nctrlr,iny,inx,ucbsv,new,buf,opt

5    The sample driver  BMDRV has no "#" characters due  to the text

preparation method  used.  Also, the following  labels (and any

references to them) are incorrect:

Documented as:                Should be:

-------------                 ---------

$DAT                          $BMDAT

$DCB                          $BMDCB

$END                          $BMEND

6    The description  of the PROLOD  call is in error.   The correct

description and format of the argument list follow:

STATUS,REQUEST,FNAM,FNAMSZ

where,

STATUS         The address of an eight word status block

REQUEST        The address of a word containing a request value

corresponding to the  operation to be performed.

The defined decimal values are:

1. = Load   a   driver   and  bring  online  all

associated controllers and units.

2. = Unload a driver after successfully bringing

all   associated   controllers  and   units

offline.

FNAM           The  address of  a word  which contains  the two

ASCII  characters from  which the  actual driver

task and  symbol table filenames may  be derived

by  PROLOD.  The  derived  name is  of the  form

LB:[ZZSYS]xxDRV.TSK   and   LB:[ZZSYS]xxDRV.STB.

Both files must be  present to successfully load

a driver.

FNAMSZ         The  size of  the aforementioned  two bytes  (in

bytes).

7    The  CTB  definition  description   was  not  documented.   The

following macro excerpt describes the CTB:

;+

; CONTROLLER TABLE (CTB)

;

; THE CONTROLLER TABLE IS A CONTROL BLOCK THAT CONTAINS A VECTOR

; OF KRB ADDRESSES.  THIS VECTOR MAY BE ADDRESSED BY THE CONTROLLER

; INDEX TAKEN FROM THE INTERRUPT PS BY $INTSI.

;

;-

.ASECT

.=177754

L.CLK:'L' .BLKW 8.              ;START OF CLOCK BLOCK (CLK BLK IS

;OPTIONAL, AND DRIVER DEPENDENT.

;ALLOCATION OF THESE 8 WORDS IS NOT

;REQUIRED IN THE DRIVER'S DATABASE

;UNLESS USED BY THE DRIVER ITSELF)

L.DID:'L' .BLKW 1               ;HARDWARE DEVICE ID (WORD ALLOCATION

;ALWAYS REQUIRED. MAY BE 0.)

L.ICB:'L' .BLKW 1               ;ICB CHAIN FOR THIS CTB

L.LNK:'L' .BLKW 1               ;CTB LINK WORD

L.NAM:'L' .BLKW 1               ;GENERIC CONTROLLER NAME (ASCII)

L.DCB:'L' .BLKW 1               ;DCB ADDRESS OF THIS DEVICE

L.NUM:'L' .BLKB 1               ;NUMBER OF KRB ADDRESSES IN TABLE

L.STS:'L' .BLKB 1               ;CTB STATUS BYTE

L.KRB:'L' .BLKW 1               ;START OF KRB ADDRESSES.

;

; NOTE: THE SYMBOL $XXCTB:: IS DEFINED  FOR EACH CTB, WHERE THE

; SYMBOL IS NOT THE START OF THE CTB, BUT INSTEAD THE START OF

; THE KRB TABLE AT THE END OF THE CTB (L.KRB). THE SYMBOL XXCTB (NO"$")

; IS GENERATED BY THE DDT$ MACRO AND IS USED TO IDENTIFY THE

; WORD IN THE DRIVER WHICH CONTAINS THE ADDRESS OF THE CONTROLLER'S

; CTB IN PRIMARY POOL. XXCTB IS REFERENCE BY THE CODE GENERATED IN THE

; INTSV$ MACRO WHEN DETERMINING THE UCB ADDRESS.

;

;+

; CONTROLLER TABLE STATUS BYTE BIT DEFINITIONS

;-

LS.CLK='B'1             ;CLOCK BLOCK AT TOP OF CTB (1=YES)

LS.MDC='B'2             ;MULTIDRIVER CTB. (1=YES)

LS.CBL='B'4             ;CLOCK BLK LINKED INTO CLK Q (1=YES)

LS.CIN='B'10            ;CONT. USE COMMON INT TABLE (1=YES)

LS.NET'B'=20            ;THIS IS DECNET DEVICE.

;ICB LISTHEAD IN K.PRM, L.DCB INVALID (1=YES)

Developer suggestions:

1 - Before attempting  to load your  device driver, be  sure to

load and  run XDT, so that  you are prepared to  handle any

problems which may arise.

2 - PROLOD attempts to return a meaningful error message to the

developer if and only if the logical name "PROLOD$MSG" with

the equivalence  name "0" is defined.   Multiple errors may

be detected  in the course  of loading and  sanity checking

the driver and it's database.

3 - If  your driver  fails  to load,  correct  the problem  and

reboot.

4 - Do not depend  on any registers being  preserved by $IODON.

In particular  this means  R5 may under  some circumstances

(overlapped I/O completion) be trashed.

The error codes that PROLOD returns as server dependent errors are:

0.      Illegal request format

- 1.      File not a valid driver task image

- 2.      Privileged command

- 3.      Inconsistent argument length

- 4.      Illegal request function code

- 5.      Illegal unit name format specified

- 6.      Specified unit not found

- 7.      Illegal controller name specified

- 8.      Specified controller not found

- 9.      Failed to offline device

- 10.     I/O error on input file

- 11.     Failed to bring device online

- 12.     Controller already online

- 13.     Unit already online

- 14.     File  has illegal STB format

- 15.     Device not found in system

- 16.     Illegal device name specified

- 17.     Database not found in system or driver

- 27.     Partition/region  not in system

- 33.     Device not mounted

- 34.     File not contiguous

- 35.     Open failure on file

- 40.     Task image I/O error in file

- 46.     Partition  too small

- 50.     Illegal driver task APR usage

- 51.     Partition/region  is a common

- 60.     Driver already resident

- 61.     Driver being loaded or unloaded

- 62.     Insufficient pool space

- 63.     Loadable driver support not in system

- 64.     Driver not loaded

- 65.     Driver cannot be unloaded, still online

- 66.     Device  is attached, busy, online and/or mounted

- 68.     Invalid driver data base at offset  in file

- 69.     Driver built with wrong executive STB file

- 70.     Warning - KRB  interrupt vector too high

- 72.     Warning - KRB  interrupt vector in use

- 73.     Symbol is undefined in file

- 74.     Symbol is doubly defined by file

- 75.     Illegal value for symbol in file

- 76.     Driver dispatch table is inconsistent

- 77.     CTB is not supported by driver -- not loaded

- 78.     Cannot load/unload a pseudo device

- 79.     Too many symbols of the form in file

- 80.     CTB does not exist

- 81.     DCB table for CTB is full

- 82.     KRB table of CTB will not accept KRB

- 83.     KRB not in loadable data base

- 84.     CTB name is a duplicate

- 85.     Warning - loadable driver larger than 4K

- 86.     Partition/region is not a common

- 87.     KRB is not offline

- 88.     Illegal use of partition or region

The  following  macro  program   illustrates  a  PROLOD  call.   Another

technique for calling PROLOD is the DCL command "LOAD" on version 3.0 or

later systems.

.TITLE  LOAD - Example PROLOD call

.mcall  qiow$s,exit$s   ;request system macros

;

; This task requests PROLOD to load the driver XXDRV from LB:[ZZSYS]

;

start:  mov     #loaarg,r5      ;get args for sample call to PROLOD

call    prolod          ;load the driver

;print status block always

mov     #stat,r2        ;point to status args

mov     #fmt,r1         ;output format

mov     #outbf,r0       ;output buffer

call    $edmsg          ;format it

qiow$s  #io.wlb,#5,#5,,,,<#outbf,r1> ;print it

exit$s                  ;exit

;

; local data

;

loaarg: .word   4               ;four arguments (simple load call)

.word   stat            ;pointer to eight word status block

.word   rqst            ;pointer to PROLOD request

.word   fnm             ;pointer to filename size

.word   fnmsiz          ;pointer to size (in bytes) of fnm

stat:   .blkw   8.              ;eight word status block

fnmsiz: .word   fnmsz           ;word containing size of fnm

rqst:   .word   1               ;load (and online all) request

fnm:    .ascii  /XX/            ;driver name

;implied filename "LB:[ZZSYS]XXDRV.TSK"

; AND "LB:[ZZSYS]XXDRV.STB"

fnmsz=.-fnm                     ;name extension as this can't be spec'd)

outbf:  .blkb   132.            ;output buffer for status display

fmt:    .asciz  /%Nload request status: %N%8P/ ;output buffer format string

.even

.end    start