;*********

;        *

; BASIC2 *

;        *

;*********

;

;

;	STOP00 - STOP AND END STATEMENTS

;	REGISTERS USED - R0,R1,R2,R3,R4

;

;	RUN00 - START PROGRAM EXECUTION

;	REGISTERS USED - R5

;

;	GOSB00 - GOSUB STATEMENT

; GOSB00 - GOSUB STATEMENT, PUSH CLASS 1 ITEM ON STACK THEN DO A GOTO

;	REGISTERS USED - R0,R1,R2,R3,R4

;

;	GOTO00 - GOTO STATEMENT

;

; GOTO00 - GOTO STATEMENT - RESET LINENO TO NEW EXECUTION POINT

;	REGISTERS USED - R0,R1,R2,R3,R4

;

;	RES00 - RESTORE STATEMENT

; RES00 - RESTORE STATEMENT - CLEAR THE DATA POINTER

;	REGISTERS USED - NONE

;

;	RET00 - RETURN STATEMENT

; RET00 - RETURN STATEMENT - FIND LAST GOSUB ITEM IN LIST,

;	GET ITS LINE NUMBER, PLACE IN LINENO, AND DELETE THE ITEM.

;	REGISTERS USED - R0,R1,R2,R3,R4,R5

;

;

; STFIND: STATEMENT FIND ROUTINE TO POSITION OURSELVES TO PROPER STATEMENT

; IN A LINE (USED FOR RETURN AND NEXT STATEMENTS)

; ON ENTRY:

;	R0 HAS LINE NUMBER

;	R5 HAS LINE HEADER

;	R1 POSITIONED TO START OF LINE

;	R2 HAS COUNT OF STATEMENTS TO SKIP

;

; ON EXIT:

;	R0 -> LINENO

;	R5 -> LINEHD

;	R1 AT START OF R2+1 STATEMENT

;	R2 UNDEFINED

;

;

;	DIM00 - DIM STATEMENT

; DIM STATEMENT - GENERATE DIMENSIONED TABLE ENTRY

;	REGISTERS USED - R0,R1,R2,R3,R4.

;

; SUBROUTINE STRGLN FOR USE WITH DIM STATEMENT

; ON ENTRY:

;	R1 POINTS JUST PAST '$'

; ON EXIT:

;	R1 POINTS JUST PAST LEN INFO [IF THERE]

;	R0 HAS LEN (+ FOR FIXED, - FOR VAR)

;	'V' SET IF SYNTAX ERROR

; OTHER REGISTERS USED: R2

;

; SUBROUTINE SUBSUB TO RETURN SUBSCRIPT VALUES FOR DIM

; ON ENTRY:

;	R1 POINTS TO START OF INFO [OPEN PAREN]

; ON EXIT:

;	R1 POINTS PAST END OF SUBSCRIPT INFO [PAST CLOSE PAREN]

;	R3 HAS FIRST SUBSCRIPT VALUE

;	R4 HAS 2ND SUBSCRIPT VALUE (0 IF NONE SPEC'D)

;	'V' SET IF SYNTAX ERROR

; OTHER REGISTERS USED: R2,R0

;

;

; SUBROUTINE SUBONE

; TO GET SUBSCRIPTS VIA SUBSUB AND CHECK

; THEM FOR IMMEDIATE (NON-VIRTUAL) USE.

; ON ENTRY:

;	R1 = TEXT PTR

; ON EXIT:

;	R1 POINTS PAST CLOSE PAREN

;	R3 HAS 1ST SUBSCRIPT

;	R4 HAS 2ND SUBSCRIPT

;	R0 HAS PACKED SUBSCRIPTS

;	'V' SET IF ERROR

; OTHER REGISTERS USED: R2

;

;	DEF00 - DEFINE FUNCTION STATEMENT

; DEFINE STATEMENTS ARE DONE HERE - ONE TABLE ENTRY IS MADE, CONFLICTS

;	ARE NOT CHECKED.  REGISTERS USED - R1,R2,R4.

;

;	EVAL00 - EVALUATE AN ARITHMETIC EXPRESSION

;

; EVAL - EVAL00, EVALUATE AN ARITHMETIC EXPRESSION.  UPON ENTRY, R1

;	POINTS TO THE CURRENT TEXT POSITION.  ON EXIT AC0 CONTAINS

;	THE NUMERIC VALUE OF THE EXPRESSION.  REGISTERS USED - ALL.

;	V-BIT WILL BE SET IF EXPRESSION TERMINATED BY ')', CLEAR OTHERWISE.

;	ON V SET, R1 POINTS PAST CLOSE PAREN

;	ON V CLEAR, R1 POINTS TO TERMINATING CHAR

;

;	GTP00 - GET AN OPERAND

;

; GETOP - GTP00, GET AN OPERAND.

; ON ENTRY:

;	R1 POINTS TO START OF OPERAND

;

; ON EXIT:

;	R1 POINTS TO NEXT CHAR AFTER SCAN ENDED

;	AC0 HAS VALUE OF OPERAND

;

; OTHER REGISTERS USED: POTENTIALLY ALL

;

; NOTE: THIS ROUTINE MUST BE RE-ENTRANT SINCE IT MAY RE-ENTER ITSELF

;	VIA CALLS TO "EVAL".

;	LEN00 - LENGTH OF STRING FUNCTION

;	FNSET - PARAM SETUP FOR USER FCN

;

; FNSET - SET UP PARAMS FOR FUNCTION CALL

; RETURN ADDRESS IS PULLED FROM STACK AND STACK IS USED

; TO STORE INFO ON ALREADY DEFINED DUMMY PARAMS. ENTRIES

; FOR DUMMY PARAMS ARE PUT INTO USER AREA AND NAMES FOR

; PREVIOUSLY DEFINED DUMMY PARAMS ARE ZEROED.

; ON CALL:

;	R3 POINTS TO FCN ENTRY

;	R1 POINTS TO FIRST ACTUAL ARG IN LIST

; ON EXIT:

;	R1 POINTS PAST CLOSING ')'

;	R0 POINTS PAST = SIGN FOR FCN DEF

;

;

; FNCLR:	SUBROUTINE TO RESET DATA ITEMS AFTER

; FUNCTION EVALUATION.

; REGISTERS USED: R2,R0

;

;	EVLS00 - EVALUATE A STRING EXPRESSION

;

; EVALS, EVLS00 - EVALUATE A STRING EXPRESSION OF THE FORM:

;	STRE1 + [STRE2 + ...]

;	WHERE:

;		STRE1 AND STRE2 ARE STRING ELEMENTS

;	A STRING ELEMENT MAY BE:

;		A STRING CONSTANT (IN DOUBLE QUOTES)

;		A STRING VARIABLE (ENDING IN '$')

;		A STRING FUNCTION (SYSTEM OR USER DEFINED)

; ON ENTRY:

;	R1 POINTS TO START OF PROGRAM TEXT WITH STRING EXPRESSION

;

; ON EXIT:

;	R1 POINTS TO FIRST CHAR PAST END OF EXPRESSION

;	R3 HAS ADDRESS (IN FREE SPACE)

;	R4 HAS LEN

;	'V' SET FOR BAD STRING EXPRESSION AND R1 AS ON ENTRY

;

;	GSTR00 - GET A STRING OPERAND

;

; GSTR, GSTR00 - GET A STRING OPERAND

;

;	AT END OF SUCCESSFUL COMPLETION OF THIS ROUTINE,

;	R3 CONTAINS AN INDIVIDUAL STRING ADDRESS AND R4

;	CONTAINS ITS LENGTH

;

;

;	GETHDR - GET ADDRESS OF DATA ELEMENT HEADER

;

; ON ENTRY:

;	R1 IS TEXT PTR WHICH SHOULD POINT TO VAR NAME START

;

; ON EXIT:

;	R1 POINTS TO CHAR FOLLOWING VAR NAME

;	R3 HAS HEADER ADDRESS

;	R0 HAS RETURN CODE (FOR USE AS JUMP INDEX)

;		0 = SINGLE PRECISION FLOATING

;		2 = INTEGER

;		4 = BYTE

;		6 = DOUBLE PRECISION

;		10= STRING VARIABLE

;	R4 HAS HEADER TYPE AND VAR NAME

;	'Z' SET IF VAR NOT DEFINED, CLEAR OTHERWISE

;		IN THIS CASE R3 IS NOT DEFINED

;	'V' SET IF NO VAR IN TEXT (SYNTAX ERROR)

;		IN THIS CASE R4, R3 AND R0 ARE NOT DEFINED

;

; NOTE: ON RETURN, 'V'-BIT SHOULD BE TESTED FIRST AND THEN 'Z' BIT

;

;

;	GTOFFS - GET OFFSET OF ACTUAL DATA

;

; ON ENTRY:

;	R3 POINTS TO DATA ITEM HEADER

;	R1 POINTS TO TEXT FOLLOWING VAR NAME (OPEN PAREN IF THERE)

;

; ON EXIT:

;	R3 AS ABOVE

;	R1 POINTS PAST CLOSE PAREN (IF SUBSCRIPT USED)

;	AC0 HAS:

;		1) FOR STANDARD VAR, OFFSET FROM STUDAT

;			(R0 HAS ABSOLUTE ADDRESS IN THIS CASE)

;		2) FOR VIRTUAL ARRAY, OFFSET FROM FILE START (IN BYTES)

;

; OTHER REGISTERS USED: R2, R0, R4, AC1, AC2

;

;

; SUBROUTINE VIRACC

; TO GET NEEDED DISK BLOCK IN MEMORY GIVEN AN

; OFFSET INTO THE FILE. THIS ROUTINE ALSO TAKES

; CARE OF WRITING OUT AN EXITSING BLOCK IF NECESSARY.

; ON ENTRY:

;	R3 IS PTR TO VIRTUAL DATA HEADER

;	AC0 HAS OFFSET (IN BYTES) FROM FILE START

;	R5 HAS WRITE FLAG (1 TO INDICATE WRITE TO BE PERFORMED

;		AFTER ACCESS)

;

; ON EXIT:

;	R3 AS ABOVE

;	AC0 AS ABOVE

;	R0 HAS ADDRESS OF BLOCK BUFFER

;	R2 HAS OFFSET INTO BLOCK (IN BYTES)

;

; OTHER REGISTERS USED: AC1, AC2

;

;

; SUBROUTINE WRITBK

; TO WRITE OUT BLOCK BUFFER ON BLOCK ACCESS FILE IF NEEDED

; AND SET UP FDB FOR BLOCK OPERATIONS

;

; ON ENTRY:

;	R3 POINTS TO BASIC FILE CONTROL BLOCK HEADER

; ON EXIT:

;	R0 POINTS TO FDB PROPER

;	F.BKDS AND F.BKDS+2 SET UP

;	F.BKVB(R0) SET UP WITH BLOCK NUMBER OF WRITTEN BLOCK IF WRITE DONE

;	14(R3) ZEROED (NO UNRECORDED WRITES TO THIS BLOCK)

;	F.ERR(R0) SET WITH IO STATUS CODE IF WRITE DONE

;	'C' SET IF ERROR IN WRITE, CLEAR OTHERWISE

; OTHER REGISTERS USED:R2

;

;	GETNUM - TO RETURN NUMBER -> AC0, GIVEN INFO FROM GTOFFS

;

; ON ENTRY:

;	R3 IS PTR TO DATA ITEM HEADER

;	AC0 HAS OFFSET

;		1) FOR STANDARD VAR, OFFSET FROM STUDAT

;		2) FOR VIRTUAL ARRAY, OFFSET FROM FILE START

; ON EXIT:

;	R3 AS ABOVE

;	AC0 HAS NUMERIC VALUE

; OTHER REGISTERS USED: R0, R2

;

;

;	NUMSUB: COMMON CODE FOR READ AND WRITE OF NUMERIC DATA

;

; ON ENTRY:

;	R3 IS PTR TO DATA ITEM HEADER

;	AC0 HAS OFFSET

;		1) FOR STANDARD VAR, OFFSET FROM STUDAT

;		2) FOR VIRTUAL ARRAY, OFFSET FROM FILE START

;	R5 HAS READ/WRITE FLAG (0=READ, 1=WRITE)

; ON EXIT:

;	R3,AC0,R5 AS ABOVE

;	R2 HAS ADDRESS OF DATA

;	R0 HAS TYPE INDICATOR

;		0 = FLOATING (REAL*4)

;		2 = INTEGER*2

;		4 = BYTE (LOGICAL*1)

;		6 = DOUBLE (REAL*8)

;

;

;	STONUM - STORE A NUMBER FROM AC0 INTO VARIABLE

;

; ON ENTRY:

;	R3 POINTS TO DATA ITEM HEADER

;	AC0 HAS VALUE

;	AC1 HAS OFFSET

;		1) FOR STANDARD VAR, OFFSET FROM STUDAT

;		2) FOR VIRTUAL ARRAY, OFFSET FROM FILE START

; ON EXIT:

;	R3 AS ABOVE

;	AC0 AS ABOVE

;	AC1 AS ABOVE

; OTHER REGISTERS USED: R0, R2

;

;	GETSTR - TO RETURN STRING ELEMENT DESCRIPTOR IN R3,R4

;

; ON ENTRY:

;	R0 IS PTR TO DATA ITEM HEADER

;	AC0 HAS OFFSET

;		1) FOR STANDARD VAR, OFFSET FROM STUDAT

;		2) FOR VIRTUAL ARRAY, OFFSET FROM FILE START

;

; ON EXIT:

;	R0, AC0 AS ABOVE

;	R3 HAS STRING ADDRESS

;	R4 HAS STRING LENGTH

;	'V' SET IF ERROR, CLEAR OTHERWISE

;

; OTHER REGISTERS USED:

;

;

;

;	STRADD: COMMON CODE FOR STRING READ AND WRITE

;

; ON ENTRY:

;	R0 IS PTR TO DATA ITEM HEADER

;	AC0 HAS OFFSET

;		1) FOR STANDARD VAR, OFFSET FROM STUDAT

;		2) FOR VIRTUAL ARRAY, OFFSET FROM FILE START

;	R5 HAS READ/WRITE FLAG (0=READ, 1=WRITE)

;

; ON EXIT:

;	R0,AC0,R5 AS ABOVE

;	R3 HAS ADDRESS OF STRING

;

; OTHER REGISTERS USED: R0,R2

;

;	STOSTR - STORE STRING IN MEMORY (REAL OR VIRTUAL)

;

; ON ENTRY:

;	R0 IS PTR TO DATA ITEM HEADER

;	AC0 HAS OFFSET

;		1) FOR STANDARD VAR, OFFSET FROM STUDAT

;		2) FOR VIRTUAL ARRAY, OFFSET FROM FILE START

;	R3 HAS STRING ADDRESS

;	R4 HAS STRING LENGTH

;

; ON EXIT:

;	R0, AC0 AS ABOVE

;	R3 HAS ADDRESS PAST END OF USED PORTION OF SOURCE STRING

;	R4 = 0

;

;

;

; FILSTR

; TO FILL IN A STRING VARIABLE

;

; ON ENTRY:

;	R3 HAS ADDRESS OF SOURCE

;	R4 HAS LEN OF SOURCE

;	R2 HAS ADDRESS OF VARIABLE

;	R5 HAS FILL COUNT

;	R0 HAS FILL CHAR

;

; ON EXIT:

;	R4 = 0

;	R5 = 0

;	R3 HAS ADDRESS PAST END OF USED SOURCE

;	R2 HAS ADDRESS PAST END OF FILLED VARIABLE

;	R0 HAS FILL CHAR

;

;	NEWVAR - TO CREATE A SIMPLE VARIABLE OF R0 TYPE

;

; ON ENTRY:

;	R0 HAS TYPE INDICATOR

;		0 = REAL*4

;		2 = INTEGER

;		4 = BYTE

;		6 = REAL*8

;		10= STRING

;	R4 HAS VARIABLE HEADER

;		LOWER 12 BITS HAVE VAR. NAME

;		TOP 3 BITS ARE DATA TYPE

;

; ON EXIT:

;	R0 AND R4 AS ABOVE

;	R3 POINTS TO DATA ITEM HEADER

;	ENUDAT UPDATED

;

; OTHER REGISTERS USED: R5

;

;	LET00 - LET STATEMENT

;  REGISTERS USED - ALL.

;

;

;	EVALC0 - EVALUATE LOGICAL CONDITION

;

; THIS CODE EVALUATES A COMPOUND CONDITION, ALLOWING FOUR LOGICAL

; OPERATORS: NOT, AND, XOR, OR.

;

; ON ENTRY:

;	R1 POINTS TO START OF CONDITION TEXT

; ON EXIT:

;	R1 POINTS TO NEXT CHAR OF TEXT

;	R0 HAS CONDITION -1 = TRUE, 0 = FALSE

;	'N' SET IF TRUE

;	'Z' SET IF FALSE

;	IF00 - IF STATEMENT

; REGISTERS USED - ALL.

;

;	PR00 - PRINT STATEMENT

;		REGISTERS USED 0,1,2,3,4-OR ALL FOR SHORT

;

;	INP00 - INPUT STATEMENT PROCESSOR

;

; STRCMP - STRING COMPARISON ROUTINE

; ON CALL:

;	R3 CONTAINS ADDRESS OF FIRST STRING

;	R2 CONTAINS ADDRESS OF SECOND STRING

;	R4 CONTAINS LEN FOR COMPARISON

;

; ON RETURN:

;	Z BIT CONDITION CODE CONTAINS RESULT

;	BEQ WILL BRANCH IF STRINGS ARE EQUAL

;

; REGISTERS USED : PROBABLY ALL

;

;

;

;	FILL00 - COMMON DATA ELEMENT FILL ROUTINE

;

; ON ENTRY:

;	R1 POINTS TO START OF DATA

;	LINELN HAS LENGTH OF DATA FOR BINARY OR INPUT LINE

;	STACK AS ON EXIT FROM COM00 PLUS TEXT PTR AND SUBROUTINE LINK

;

; ON EXIT:

;	R1 POINTS PAST END OF DATA USED (TO EOL CHAR IF ASCII)

;	LINELN = REMAINING CHAR COUNT (USUALLY 0) ON BINARY

;	'V' SET ON BAD DATA		(ASCII ONLY)

;	BGT TRUE IF TOO MUCH DATA	(ASCII ONLY)

;	BLT TRUE IF NOT ENOUGH DATA	(ASCII AND BINARY NUMERIC)

;

; NOTE: CONDITION CODES ONLY RELEVANT FOR ASCII DATA AND BINARY NUMERIC

;

; OTHER REGISTERS USED: R0 THROUGH R5, AC0 AND AC1

;

;

;	FILL1: INDIVIDUAL ITEM FILL PROCESSOR

;

; ON ENTRY:

;	R5 POINTS TO 4 WORD ITEM DESCRIPTOR ON STACK

;	R1 HAS START OF TEXT WITH INPUT INFO

;

; ON EXIT:

;	R5 AS ABOVE

;	R1 POINTS PAST END OF THIS ITEM'S TEXT

;

; OTHER REGISTERS USED:

;	COM00 - COMMON EXPRESSION HANDLER FOR READ AND INPUT

;

; ON ENTRY:

;	R1 IS TEXT PTR

; ON EXIT:

;	R1 POINTS PAST END OF STATEMENT

;	'V' SET IF ERROR IN VARIABLE LIST

; STACK ON EXIT:

;	DATA PTR	(1 WORD)

;	DATA TYPE	(1 WORD)

;	OFFSET		(2 WORDS)

;	DATA PTR

;	DATA TYPE

;	OFFSET

;	  .

;	  .

;	  .

;	NULL

; SP -> PTR TO START OF LIST

; OTHER REGISTERS USED: POTENTIALLY ALL (INCL FLOATING PT.)

;

;

;

; ROUTINE EOSCHK

; TO CHECK FOR LEGIT END OF STATEMENT

;

; ON ENTRY:

;	R2 HAS CHAR TO BE CHECKED

; ON EXIT:

;	'Z' SET IF END

;	'Z' CLEAR IF NOT END CHAR

; OTHER REGISTERS USED: NONE

;

;	READ00 - READ STATEMENT PROCESSOR

;	FOR00 - FOR STATEMENT

;  REGISTERS USED - ALL.

;

;	NEXT00 - NEXT STATEMENT TO TERMINATE FOR LOOP

;	REGISTERS USED - ALL.

;

;

;	SCHFOR: ROUTINE TO SEARCH FOR A GIVEN FOR/NEXT CTRL BLOCK

; ON ENTRY:

;	R4 HAS SIXBIT NAME

; ON EXIT:

;	R4 UNCHANGED

;	'Z' SET IF NOT FOUND, R3 = 0

;	'Z' CLEAR IF FOUND, R3 = ADDRESS OF CTRL BLOCK

; OTHER REGISTERS USED: NONE

;

;

;	DELFOR: ROUTINE TO DELETE A FOR/NEXT CTRL BLOCK

; ON ENTRY:

;	R3 POINTS TO CTRL BLOCK TO BE DELETED

; ON EXIT:

;	R3 UNDETERMINED

; OTHER REGISTERS USED: R0,R2

;

;

;	CHKFOR: ROUTINE TO CHECK ON FINISHED/NOT FINISHED CONDITION

; OF FOR/NEXT LOOP.

; ON ENTRY:

;	R3 POINTS TO FOR/NEXT CTRL BLOCK

; ON EXIT:

;	R3 UNCHANGED

;	'Z' SET IF CTRL VARIABLE AT END VALUE, CLEAR OTHERWISE

;	'N' SET IF CTRL VARIABLE PAST END VALUE, CLEAR OTHERWISE

;	R0 HAS ACTUAL STORAGE ADDRESS OF CONTROL VARIABLE

; REGISTERS USED:

;	AC0

;

;