SSTs and Fortran

---- --- -------

Author: Richard Neitzel

Process Control Applications

312 Laveta Pass

Golden, CO 80401

13-Apr-87

Programs sometimes go bump in the night and exit due some

nastiness, such as memory protection violations, odd addresses, etc.

Since the only notification of this behavior is given to the invoking

terminal, it is easy to not have any idea what happened, especially in

a production enviroment where the answer is run it again or reboot. We

have a large number of process control stations running in this

manner. Since we have redirected the console logger to a file via COT,

we wished to have termination notice given on the log.

The two code sections SST and F77SST do just this. SST is written

in Macro-11 to use the sstvec$ directive to remap the vector for the

following ssts:

odd address, memory protection violation, t bit trap, iot trap,

reserved instruction and non-RSX emt.

The F77 file F77SST contains a subroutine that of itself does

nothing. Instead there are entry points that correspond to the names

given the new sst vectors. When an sst occurs the code at that entry

point is executed. The code supplied does the following: get the task

name, put it in the correct message, set up a qio with write break

through, issue the qio to invoking terminal and console, then exit.

The qio can be a standard non-break through if desired, since to use

io.wbt requires that the task be built using /PR:0.

It is also possible to simply continue program execution if

desired. In this case a simple return from interrupt is used, for

example on an BPT:

bpt::	rti   OR   bpt::	qio$ (msg)

rti

Obviously since you need to use the rti instruction, this can only be

done from Macro.

It is likely that a real RSX wizard can take this one step

further and give back the approximate line of the problem, but that is

currently beyond my expertise. If someone does this, please contact

me:

Richard Neitzel

312 Laveta Pass

Golden, CO

80401

303 - 966 - 7203 or 303 - 279 - 1935