].rm 66

The Kermit protocol and the PDP-11

Brian Nelson   12-APR-1986 06:54

Abstract

This article will describe the author's implementation of the  Kermit

file  transfer protocol for the PDP-11 series under RSTS/E, RSX11M/M+,

P/OS RT11 and TSX+.  This protocol allows many (if not most) types  of

computer  systems to effect, at minimum, error free file transfer with

other systems and microcomputers over asynchronous lines.

The  first  obvious  use  of  any  program  or  protocol  designed  to

accomplish  file  transfer  is  to  be  able to provide the ability to

support file uploads and downloads from superminis such as the VAX and

PDP-11  to  remote  personal  computers,  such  as the PC and Rainbow.

Additionally, there is a need  to  provide  at  least  basic  terminal

emulation.   This  can  be  done  several  ways;  a Kermit program can

provide a simple 'glass' tty mode, or in  the  case  of  a  PC,  might

provide  an  intelligent  terminal  emulation,  most likely to be of a

VT100 series terminal  such  as  MacKermit  provides.   Some  mode  of

terminal  emulation  is  generally  required  since  the file transfer

operation of Kermit requires two Kermits to be  running;  one  on  the

LOCAL  system  (your  PC,  Rainbow,  ...),  and one on the HOST (often

called the REMOTE system) system.  Obviously, the simplest  method  of

achieving  this  is to use Kermit on the local system as a terminal to

log into the remote system and then invoke the host's Kermit program.

The first step one usually takes in starting a Kermit  session  is  to

establish  the  connection  between  the  two  systems.   When  we are

connecting two single user systems,  such  as  a  Rainbow  to  another

Rainbow,  we  would  simply connect the COM ports together with a NULL

modem cable and then invoke Kermit on each  system.   One  would  most

often  set  one  of  the PC's up as a Kermit 'server'; this allows the

other system to control the server and initiate file transfer requests

without the need to move between the two machines.

If we are on our  Rainbow  and  we  wish  to  transfer  files  from  a

multiuser  host,  such  as  a  VAX  or  PDP-11,  we would first make a

connection to the host with a modem or direct line.  Once the physical

link  has  been  made,  we would log into the remote system and invoke

Kermit on that system and run it as a server.  This is often  done  by

typing 'KERMIT SERVER' on the host.

(slide CONNECT1 goes here)

Note that the micro-computer Kermits assume the use of the systems COM

port  for  communications.   When a larger system, such as a PDP-11 or

VAX, is used to connect to a remote system, the  Kermit  program  must

usually  be  told  which  terminal  line to use.  For example, using a

PDP-11 to dial out to a remote  system  would  require  the  SET  LINE

Page 2

command,  where  the line given is the name of a terminal device, such

as TT27:  or TXA0:.  When this is done, there may be a need to control

some of the modem signals, most likely DTR (Data Terminal Ready).

(slide CONNECT2 goes Here)

Basic Kermit commands

Now that we have established the physical link, how do  we  get  files

transferred?   Well,  it's  quite  simple.   Assuming that we gave the

remote Kermit the SERVER command, we use:

Kermit-11>GET file.type         Request a file from the remote

Kermit-11>SEND file.type        Send a file to the remote system

Kermit-11>BYE                   Tell the remote Kermit to log out

These are the commands we need when talking to a Kermit server.  There

are, of course, many others which may support file manipulation on the

host, as well as obtaining HELP.  These commands are normally  thought

of  as  REMOTE  commands.   Indeed,  they  are  prefixed by the REMOTE

keyword, as in:

Kermit-11>REMOTE HELP

Kermit-11>REMOTE COPY FILE1.TYP FILE2.TYP

Kermit-11>REMOTE CWD [USERFILES]

Before continuing, note that not all  Kermit  programs  implement  the

same  level of support.  In general, the large system Kermits, such as

Kermit-32 (VMS), Kermit-20 (Tops-20) and Kermit-11  (PDP-11  and  PRO)

support  a large command set.  No single command is really required by

Kermit; the protocol specifies the transportation of  files,  not  the

command  interface.   At a minimum, however, a Kermit program requires

the SEND and RECEIVE commands to effect file transfer.  Server support

is  optional,  if  we don't have server support available then we must

use the SEND and RECEIVE commands and tell each Kermit such every time

we want to move a file.

What do we use Kermit for?

I routinely use Kermit for transferring  software  developed  for  the

PRO/350 on a RSTS/E 11/23+ host as well as using the PDP-11/44 and VAX

11/785 I run at the University of Toledo  for  dialing  out  to  other

systems,  such  as  the  LCG  Tops 20 system and the LDP public domain

library.  Considering that there exists a Kermit for  almost  any  DEC

configuration  one  can even use Kermit as a poor man's Decnet.  In my

case, I have a DMF32 modem port from the 11/785 and a DZ11  port  from

the 11/44 connected to the Gandalf PACX front end switch, which allows

me to connect either system to any of the other systems on  the  PACX,

which  includes an IBM 370 compatible system as well as connecting the

Page 3

VAX to the PDP-11.

With the knowledge that there  are  Kermit  implementations  for  most

personal computers in use it becomes apparent that the Kermit standard

is well worth looking in to.  A list of versions  running  on  Digital

hardware follows the article.

The Kermit protocol

The Kermit protocol is designed to operate  over  normal  asynchronous

terminal  lines.   All data and commands are transferred with a packet

oriented protocol, basically consisting of a start of packet character

(normally SOH), followed by length, control, data and checksum fields.

Communications is half duplex, in that  for  every  packet  sent,  the

sender  must  wait  for  either  an  acknowledgement packet (ACK) or a

negative acknowledgement packet (NAK).  Transmission is in ascii, with

no requirements needed for the transmission of eight bit characters or

control characters other than the choice of control-A for marking  the

start  of a packet.  All 'control' characters imbedded in the data are

prefixed to convert them to printable characters, the same applying to

eight  bit  characters if required by the characteristics of the line.

Since there are many different implementations of Kermit, the protocol

provides  a  mechanism  by  which  the  capabilities  of two connected

Kermits can be negotiated to allow for differences  in  the  level  of

protocol  support.  Examples of protocol features that not all Kermits

understand include data compression and transfer of file attributes.

How Kermit transfers a file

The means by which Kermit  transfers  a  file  is  quite  simple;  the

protocol includes a START OF HEADER (normally a SOH, control A), after

which follows a LENGTH field, then a SEQUENCE  character  and  a  TYPE

field.   After  this,  the  DATA follows, with a checksum trailing the

data segment.  The checksum can be one of three types, the first being

basically  an  additive  sum wrapped into 6 bits, the second being the

same but 12 bits in size, and the third type  a  CRC  based  checksum.

The sequence number increments for each packet sent, module 64.

(example slide goes here)

(checksum1 and Checksum2 files go here, if you feel its needed)

Rather than to go into more detail about the the Kermit protocol, the

reader  should  consult  the  references  listed  at  the  end of this

article.

The PDP-11 Kermit-11 implementation

The author's version of Kermit-11 is written in Macro-11 and can  run

on  RSTS/E, RSX11M, RSX11M Plus, P/OS and RT11.  The RSTS and RSX file

system interface is via RMS11 version  2,  while  the  RT11  interface

attempts  to  emulate the RMS11 subsystem.  The choice of Macro-11 for

the implementation language was made for several  reasons,  one  being

Page 4

the  availability  of  the  assembler on all systems and another being

speed and compactness of the code.

RMS11 was used for RSTS/E and RSX to provide a common  i/o  interface

to  the  host  file  system.  Additionally, Bob Denny of Alisa Systems

further extended the RMS interface to support remote file access  over

DECNET    with    Kermit,    allowing    commands    such    as   SEND

NODENAME::[BRIAN.FUBAR]FILE.TYPE and other remote file  accesses  over

DECNET.   RMS11  version  2  also  provides a very simple and powerful

means of doing wildcard searching, file renames and file deletion  via

the  $PARSE,  $SEARCH, $RENAME and $DELETE macros.  Points against RMS

basically amount to it's size, RMS is quite large even  if  overlayed.

This  is  helped by using the segmented RMSRES available on RSTS/E and

RSX11M Plus, though there is no remote file access for RMSRES  in  the

current  release  of  Kermit-11.  The other objection to RMS will come

from RSTS/E users, who are used to using files that normally lack file

attributes.   This  is  overcome  by  the  ability of RMS v2 to create

stream ascii files.

The RSTS/E Kermit, while it does 'run' under RSX emulation,  does  NOT

use  any  RSX  directives  (apart  from  GTSK$S)  to  interface to the

executive, as (one) the RSX directive emulation under RSTS/E is only a

small  subset of 'real' RSX and (two) there is no need to go though an

additional layer of overhead to make RSTS/E map RSX  calls  to  native

calls.   The  'multiple  private  delimiters' feature is used to avoid

losing read pass all (binary)  mode  on  read  timeouts,  as  well  as

setting  the  link  to  '8-bit'  mode to keep the terminal driver from

stripping the high bit from data received.

The RSX11M/M+ and P/OS versions of Kermit-11, like the RSTS/E  and  RT

versions,  receive  eight  bit data assuming no parity is used.  Where

parity is a must, Kermit-11 has to use a prefixing  scheme  for  eight

bit binary data.  Like the RSTS/E version, binary files are created as

FIXED no carriage control files such as used for  task  images.   Note

that  parity  generation  is  done by software in Kermit-11.  The P/OS

version runs under control of DCL.  The  next  release  of  Kermit-11,

which  will  be  3.50, will include support for the PRO TMS (Telephone

Management System) option.

The RT11 and  TSX+  version  of  Kermit-11  maintains  source  module

compatability  with  the  RSTS/E  and  RSX  versions.  Each version of

Kermit-11 has it's own source file to deal with the operating  system,

for  RSX  it  is  K11M41.MAC,  for  RSTS/E  they  are  K11E80.MAC  and

K1180S.MAC, and for RT11 they are called K11RT*.MAC.  Apart from these

specific files, all other source files are shared.  The RT11 Kermit-11

can use either the version 5.x XL and XC handler for high  throughput,

or  it  can  use multiple terminal service to do all its terminal i/o.

This  second  option  allows  the  use  of  any  interface  supported,

including  the  PDT150  modem  port,  DL/DLV11's  and DZ/DZV11's.  The

drawback is overhead, the RT11 MT service can't sustain  a  rate  much

past  1200  baud  at most.  This is not a problem for Kermit, however,

due to it's half duplex nature and the fact that no packet received is

ever  longer  than  the ring buffer size.  The only problem is in when

Kermit-11 is running  as  a  terminal  emulator  (the  Kermit  CONNECT

command) where the data coming from the remote host can easily overrun

the executive's buffer.  A SET RT11  [NO]FLOW  command  was  added  to

Page 5

force  Kermit-11  to send its own flow control to the host via XON and

XOFF.  TSX+ users can connect to CL:  for dialing out, the exact means

is documented in the Kermit-11 users guide.  The disk i/o emulates the

RSTS/E and RSX RMS11 version, and each  executive  directive  has  its

error codes mapped into an unique global error code, with the symbolic

names corresponding to the nearest RMS11 error name.   Wildcarding  is

handled,  of course, by non file-structured access to the directory on

the desired volume, and supports full RT11 wildcard filenames.

Transmission of file attributes

One of the optional features of the Kermit protocol is  the  ATTRIBUTE

packet.   The  attribute  packets  allow a Kermit program to send to a

receiving Kermit information regarding the  file  organization,  size,

cluster/retrieval  size,  protection  and  so  forth.  There is even a

system dependant attribute packet type that can be  used  to  transfer

things  like  the  RMS11  IFAB  (the  RMS/FCS attributes).  One of the

things that two Kermits  exchange  before  any  file  transfer  is  an

information  packet,  this  packet  tells  the  receiving Kermit about

itself.  The last field in this packet, the CAPAS mask,  tells  Kermit

if  the  other  one can process attribute packets.  If two Kermit-11's

are communicating, they will find that each can do so, and the  sender

of  a  file  will then send over attribute packets indicating the need

(or lack of) for binary transmission, based on the file  organization,

filetype  and  protection code (for RSTS/E).  If the sending Kermit-11

is running on RSTS/E, RSX11M/M+ or P/OS it will also send  a  copy  of

the  RMS/FCS attributes so the received file will be identical (to FCS

and/or  RMS)  to  the  copy  on  the  sender's  system.   Since  other

implementations  of  Kermit  may  use  this  special  system attribute

packet,  Kermit-11  always  sends  an  attribute  packet  telling  the

receiver what hardware and operating system it is running on, and thus

will only use such data if they are compatible.  Of course, there will

be  times  when a file may be binary and Kermit-11 can't tell so, many

Kermit's have a SET FILE BINARY and SET FILE ASCII to allow  the  user

to  override  defaults.   Kermit-11 also has a SET FILE AUTO/NOAUTO to

disable it from trying to determine a file's binary status.

Future directions

With  the  advent  of   packet   switched   networks   and   satellite

communications the Kermit protocol will likely be extended to increase

efficiency over such links.  The  main  problem  is  the  half  duplex

nature  of  Kermit, the packet acknowledgements can take up to several

seconds in transit thus drastically reducing  the  throughput.   There

are  several  possibilities  under discussion and a standard should be

appearing shortly.

Page 6

Summary

There are currently over 200  implementations  of  Kermit  available.

This  article describes only the PDP-11 Kermit-11 implementation, for

further reading see:

Kermit: A File-transfer Protocol for Universities

Frank da Cruz and Bill Catchings

BYTE Magazine, June/July 1984

The Kermit Protocol Manual, version 5

Frank da Cruz   April 1984

Columbia University Center for Computing Activities

(slide GETKERMIT should be here)

Digital hardware that Kermit is currently available for:

Operating  Program

Machine          System     Language     Contributor

DEC PDP-11       MUMPS-11   MUMPS-1982   Cornell U

DEC PDP-11       RSTS/E     Macro-11     U of Toledo

DEC PDP-11       RSX-11/M   Macro-11     U of Toledo

DEC PDP-11       RSX-11/M+  Macro-11     U of Toledo

DEC PDP-11       RT-11      Macro-11     U of Toledo

DEC PDP-11       RT-11      OMSI Pascal  U of Toronto

DEC PDP-11       TSX+       Macro-11     U of Toledo

DEC PDP-11       Unix 2xBSD C            Columbia U

DEC PDP-11, ...  Unix V7    C            Columbia U

DEC PDP-8        OS8, RTS8  PAL-8        R. Hippe

DEC Pro-3xx      P/OS       Bliss, Macro Stevens I.T.

DEC Pro-3xx      P/OS       Macro-11     U of Toledo

DEC Pro-3xx      Pro/RT     Macro-11     U of Toledo

DEC Pro-3xx      Venix V1   C            Columbia U

DEC Pro-3xx      Venix V2   C            Columbia U

DEC Rainbow      CPM86      ASM86        Columbia U

DEC Rainbow      MS-DOS     MASM         Columbia U

DEC Rainbow      QNX 1.x    C            Merrell-Dow

DEC VAX          Ultrix-32  C            Columbia U

DEC VAX          VMS        Bliss,Macro  Stevens I.T.

DEC VAX          VMS        C (VAX-11 C) Columbia U

DEC VAX          VMS        Pascal       U of Toronto

DEC VAX, ...     Unix 4xBSD C            Columbia U

DEC VT-180 Robin CPM80      Turbo Pascal Jeff Duncan

DEC VT-180 Robin CPM80 2.2  M80,LASM     ACC

Page 7

DECmate-II,III   CPM80 2.2  M80,LASM     ACC

DECsystem-10     TOPS-10    Bliss, Macro Stevens I.T.

DECSYSTEM-20     TOPS-20    MACRO-20     Columbia U