;     ------------------------------------------------------------------

;

;     Name: set BIT CouNTer

;

;     Computer:              LSI-11/23

;     Operating System:      RSX-11M V4.2 BL38C

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Writen:           2/17/87

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;

;     Description:

;           This routine counts the number of set bits in a word. The algorithm

;      used in based on the fact that the logical .and. of a number n and n-1

;      clears the lowest set bit in a word. The F77 call is:

;		CALL BITCNT (iword,icount)   where iword is the word to test and

;      icount is the returned set bit count.

;     ------------------------------------------------------------------

;

;     SUBROUTINE NAME: BYTe bit test

;

;     Computer:              LSI-11/23 & LSI-11/73

;     Operating System:      RSX-11M V4.2 BL38B

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Written:          11/3/86

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;

;     Description:

;           This subroutine determines if a bit is set or clear within a byte.

;        The F77 call is:

;    		CALL BYTTST (ibyte,ibit,istatus)

;

;        The Macro-11 call is:

;			a: 0,3

;			b: ibyte

;			c: ibit

;			d: istatus

;			mov  a,r5

;			call byttst (jsr  pc,byttst)

;                                              where ibyte is the byte to be

;        tested, ibit a word holding the bit location to be tested (0 - 7.)

;        and istatus is a word to hold the result of the test.

;

;           The returned status is identical to the bit, 0 if clear and 1 if

;        set, unless the bit position was greater then 7., which causes and

;        error return code of -1. Registers r0, r1, r2 and r5 are destroyed.

;

;     ------------------------------------------------------------------

;

;     Name: CHaRacter LENgth

;

;     Computer:              LSI-11/23

;     Operating System:      RSX-11M V4.2 BL38C

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Implemented:      2/4/87

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;     Description:

;	    This routine returns the actual number of characters in an ASCII

;	string. The asumption is made that the null character is the last

;	character in the string. The Fortran call is:

;			CALL CHRLEN (string, ilen)   where string is the string

;	to check and ilen is a integer*2 variable to hold the length.

;

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;

;     Name: DAY of week

;

;     Computer:              LSI-11/23

;     Operating System:      RSX-11M V4.2 BL38B

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Written:          2-Oct-86

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;

;     Description:

;           This module consists of two subroutines which share 90% common

;       code. The first is MACDAY, which returns the day of the week to

;       calling MACRO-11 routines. The second is F77DAY, which does the same

;       for Fortran. When MACDAY is called local event flag 24 is set, so user

;       programs must take care not to use this flag. Later in the common code

;       this flag is used to determine if the return should be through R0 or

;       the F77 OTS. Registers R1-R5 are saved, but must be restored by the

;       user if so desired. The calling sequence is:

;

;	MACRO-11         jsr	pc,macday       The result is returned in R0.

;

;	Fortran-77	 CALL F77DAY(cstring)   Where cstring is a 9 byte

;          variable.

.nlist bin

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;     Name: PCA RSX file open routines

;

;     Computer:              LSI-11/23

;     Operating System:      RSX-11M V4.2 BL38B

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Implemented:      10/8/86

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;                            R. Neitzel  10/27/86  Added conversion of file

;						   name and type to ASCII from

;						   RAD50 and device number and

;						   version to ASCII octal.

;

;     Description:

;           This set of subroutines allows the caller to do the following:

;         1> Open a file with a wildcard filespec. The following are legal

;		types - x.x;*   x.*;*   *.x;*  *.*;*

;	  2> Retrieve the name and version number of the currently open file.

;	  3> Move through a list of files by using the wildcard filespecs.

;	  4> Return to the first file of a wildcard set.

;

;           To use these routines the subroutine pcaini must be used first.

;      This subroutine creates the data set block used to open the file by name.

;      The F77 call is:

;			CALL PCAINI([d],[ds],[u],[us],[f],[fs])	where d is the

;      device name, ds the number of characters in the device name, u the UIC

;      (form:[xxx,xxx]), us the number of characters in u, f the file name and

;      fs the number of characters in f.

;

;           To open a file use the routine pcaopn. This routine is accessed via

;      the USEROPEN option within the F77 OPEN statement. pcaopn must be

;      declared as an EXTERNAL function. This will open the next file type that

;      matches the one used in pcaini. If this is the first use of pcaopn the

;      file with the lowest version number of the first file in the directory

;      that matches will be used. When coding the open statement you must give

;      some file name, however, this need not be that of an existing file (and

;      certainly not your true file if you are using wildcards). Further, the

;      status of the file must be 'OLD'.

;      The F77 call is:

;      		........

; 		EXTERNAL PCAOPN

;      		........

;      		........

;      		OPEN(UNIT=LUN,file='DUMMY',...,STATUS='OLD',USEROPEN=PCAOPN)

;

;      You may open a series of files simply by repeating the calling to pcaopn.

;      To determine when the end of the list is reached use the system routine

;      ERRSET to allow transfer of control to a specified ERR line when the

;      code IE.NSF is returned.

;

;           To return to the first file of a list call pcarst. The call is:

;      			CALL PCARST

;

;           To get the name and version number of the currently open file use

;      pcanam. This will give you the device name and number as well. This

;      routine must not be called until AFTER opening a file using pcaopn or you

;      will get garbage. The F77 call is:

;

;      			CALL PCANAM ([ds],[dn],[fn],[ft],[v])   where ds is a

;      2 byte buffer for the ASCII device name, dn a 3 byte buffer for the

;      ASCII device number (octal notation), fn an 9 byte buffer for the file

;      name, ft a 3 byte buffer for the file type and v a 3 byte buffer for

;      the ASCII version number (octal notation). Note that the old familar '.'

;      and ';' are not returned.

;

;           If you want the device and/or version numbers as numbers use pcanm2.

;      This will return the actual values. Follow the warning for pcanam. The

;      F77 call is:

;			CALL PCANM2 ([dn],[vn])  where dn is a word variable for

;      the device number and vn is a word variable for the version number.

;

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;     Name: FILe SIZe routine

;

;     Computer:              LSI-11/23

;     Operating System:      RSX-11M V4.2 BL38B

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Written:          12/3/86

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;     Description:

;	    This file contains two routines that allow determination of

;	various file characteristics. These are: record type (fixed, variable or

;       segmented), record size, number of bytes used and the number of records

; 	in the file. The first is for use with all files while the second is

;	for files using NOSPANBLOCKS.

;

;	    The first routine is called FILSIZ. No knowledge about the file's

;	structure is required to obtain the following: record type, record

;	size, number of bytes in file and number of records in file.

;	The F77 call is:

;

;			CALL FILSIZ (lun, itype, isize, ibytes, irec) where

;	lun is the file's LUN number, itype is the returned value of the

;	record type (1 for fixed, 2 for variable and 3 for segmented, however

;	see note below), isize is the size of the record in bytes, ibytes is

;       the size of the data area in bytes and irec is the number of records

;       in the file. All are integer*2 variables except for ibytes and irec,

;       which are integer*4 variables.

;

;	    This routine has been tested on direct access and sequential

;	files, with the latter using fixed and variable record lengths. It

;	must be noted that isize for a variable length file returns the size

;	of the largest record in the file, while the ibytes and irec values

;	are meaningless.

;

;	    Another interesting quirk is the fact that the number of bytes

;	returned for a fixed length file with an odd record size will not

;	match the predicted number. Instead the returned value will be greater

;	by the number of records in the file. This is due to padding to

;	preserve even byte starting addresses. This is why odd sizes

;	are rounded up.

;

;	    The second routine is called NOSPAN. The F77 call is:

;		CALL NOSPAN (lun, itype, isize, iblocks, ilast) where lun is

;	the LUN of the file, itype the record type (as described above), isize

;	the record size in bytes, iblocks the number of blocks in the file

;	minus one (since the last block may not be fully used) and ilast is

;	the last byte used in the last block. All are integer*2 variables

;	but iblocks which is integer*4.

;

;	    In using the results remember that files with odd record sizes

;	actually allocate one more byte per record then indicated. Again both

;	direct and sequential files have been tested, and the block count and

;	last byte for variable length files, while correct, are meaningless.

;

;   NOTE:   The FILSIZ routine may also be used for files that use NOSPANBLOCKS.

;	This is especially useful for opening a file when the attributes are

;	unknown. In this case the itype variable will be 9, 10 or 11, the

;	addition of 8 being the mark of the nospan attribute. In this case

;	ibytes will be the same as iblocks and irec as ilast in NOSPAN.

;	However, if the value of irec is greater then the the integer*2

;	maximum prior to the call you will get garbage, so if you are uncertain

;	about span/nospan clear irec first.

;

;     ------------------------------------------------------------------

;

;     Name: MAKE a i*4 variable

;

;     Computer:              LSI-11/23

;     Operating System:      RSX-11M V4.2 BL38C

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Writen:           2/17/87

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;     Description:

;           This routine takes two i*2 variables and converts them into an i*4

;       variable. The F77 call is:

;		CALL MAKE4 (i4,ilow,ihigh)   where i4 the resultant integer*4

;       variable, ilow is the low order word and ihigh is the high order word.

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;     Name: output MeSsage to TI:

;

;     Computer:              LSI-11/23

;     Operating System:      RSX-11M V4.2 BL38B

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Written:          11/26/86

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;     Description:

;           This subroutine prints a message to the TI: unit (lun 5) of a task.

;       The task's name is also printed. Message length is limited to one line

;       and the maximum number of characters printed is limited to @70. The F77

;       call is:

;			CALL MSG5 (string) whre string is any legal form of

;	character data.

;

;	Due to the use of $errlg, this routine cannot be called by other MACRO

;	routines unless they are also part of a Fortran program.

;

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;     Name: ODD to even

;

;     Computer:              LSI-11/23

;     Operating System:      RSX-11M V4.2 BL38B

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Written:          12/4/86

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;     Description:

;           This routine takes an integer and if it is odd makes it even. The

;	new value is either 1 greater or less then the origional. The F77 call

;	is:

;		CALL ODD(iselect, ivalue) where iselect is the value that

;	determines if addition or subtraction is performed (1 means add, 2

;	subtract, any other value is an error) and ivalue is the variable to be

;	worked on.

;

;     ------------------------------------------------------------------

;

;     Name: PUSH/POP routines

;

;     Computer:              LSI-11/23

;     Operating System:      RSX-11M V4.2 BL38C

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Written:	     3/17/87

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;

;     Description:

;	    This set of routines allows the user to push and pop variables

;	onto a user defined stack. Two versions are supplied, one for 2 byte

;	variables (PUSH/POP2) and one for four byte variables (PUSH/POP4). The

;	F77 call is:

;		CALL PUSHn (num, x, y, ...)

;		CALL POPn (num, x, y, ...)		where n is the correct

;	number indicating the number of bytes in the variables used, num is

;	the number of variables to be used and x and y are the variables

;	themselves. IT IS EXTREMELY IMPORTANT THAT YOU NOTE THAT POPPED

;	VARIABLES ARE RETURNED IN THE INVERSE ORDER IN WHICH THEY WERE PUSHED -

;		CALL PUSH2(2,A1,A2)	CALL POP2(2,A2,A1)

;

;	    Calls to the routines must be symetric, that is, a call to PUSH2

;	cannot be followed by a call to POP4, and the number of variables used

;	must be the same in the matching calls. The limit on the number of

;	variables that can be pushed is set by the size of the stack area. The

;	current limit is 100 word size variables or 50 long words.

;

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;     SUBROUTINE NAME: WoRD bit TeST

;

;     Computer:              LSI-11/23 & LSI-11/73

;     Operating System:      RSX-11M V4.2 BL38B

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Written:          11/3/86

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;

;     Description:

;           This subroutine determines if a bit is set or clear within a word.

;        The F77 call is:

;    		CALL WRDTST (iword,ibit,istatus)

;

;        The Macro-11 call is:

;			a: 0,3

;			b: iword

;			c: ibit

;			d: istatus

;			mov  a,r5

;			call wrdtst  (jsr  pc,wrdtst)

;                                              where iword is the word to be

;        tested, ibit a word holding the bit location to be tested (0 - 15.)

;        and istatus is a word to hold the result of the test.

;

;           The returned status is identical to the bit, 0 if clear and 1 if

;        set and -1 if the bit selection was illegal. Registers r0, r1, r2

;        and r5 are destroyed.

;

;       ---Executable statements begin here---

;

;     ------------------------------------------------------------------

;

;     SUBROUTINE NAME: WoRD bit SET

;

;     Computer:              LSI-11/23 & LSI-11/73

;     Operating System:      RSX-11M V4.2 BL38B

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Written:          11/3/86

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;

;     Description:

;           This subroutine determines sets a bit within a word.

;        The F77 call is:

;    		CALL WRDSET (iword,ibit,istatus)

;

;        The Macro-11 call is:

;			a: 0,3

;			b: iword

;			c: ibit

;			d: istatus

;			mov  a,r5

;			call wrdset  (jsr  pc,wrdset)

;                                              where iword is the word to be

;        used, ibit a word holding the bit location to be set (0 - 15.)

;        and istatus is a word to hold the result status.

;

;           The returned status is 1 if the operation is successful and -1 if

;        the bit selection was illegal. Registers r0, r1, r2

;        and r5 are destroyed.

;

;       ---Executable statements begin here---

;

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;     SUBROUTINE NAME: WoRD bit CLeaR

;

;     Computer:              LSI-11/23 & LSI-11/73

;     Operating System:      RSX-11M V4.2 BL38B

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Written:          11/3/86

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;

;     Description:

;           This subroutine determines clears a bit within a word.

;        The F77 call is:

;    		CALL WRDCLR (iword,ibit,istatus)

;

;        The Macro-11 call is:

;			a: 0,3

;			b: iword

;			c: ibit

;			d: istatus

;			mov  a,r5

;			call wrdclr  (jsr  pc,wrdclr)

;                                              where iword is the word to be

;        used, ibit a word holding the bit location to be cleared (0 - 15.)

;        and istatus is a word to hold the result status.

;

;           The returned status is 1 if the operation is successful and -1 if

;        the bit selection was illegal. Registers r0, r1, r2

;        and r5 are destroyed.

;

;     ------------------------------------------------------------------

;

;     Name: HIGH and LOW byte replacement

;

;     Computer:              LSI-11/23

;     Operating System:      RSX-11M V4.2 BL38B

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Written:          12/11/86

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;     Description:

;           These two routines put a byte into either the upper or lower half of

;	a word. The savings in code are 9 lines in lowb and 7 for highb over

; 	doing this via Fortran. The call is:

;		CALL {LOWB/HIGHB} (iword,byte)  where iword is the word to be

;	manipulated and byte is the byte value to use.

;

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;

;     Name: BIT REVersal routines

;

;     Computer:              LSI-11/23

;     Operating System:      RSX-11M V4.2 BL38B

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Written:          12/18/86

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;     Description:

;           This set of two routines allows the user to reverse the bit order

;	of either an Integer*2 or Integer*4 variable. The number of bits to be

;	reversed may also be specified. The integer*4 form is a subroutine

;	call with the form being:

;		CALL BITR4 (irev, ibits)   where irev is an integer*4 variable

;	to be reversed and ibits is the integer*2 number of bits to operated

;	on.

;

;	The integer*2 form is a function call of the form:

;		iresult = IBITR2 (irev, ibits)   where iresult is an integer*2

;	variable to recieve the function result, irev is the integer*2

;	variable to reverse and ibits is the integer*2 number of bits to

;	operate on.

;

;	    Note that if ibits is less then the maximum allowed value any

;	bits over that number are lost in the reversal. The maximum value for

;	BITR4 is 32. and 16. for IBITR2.

;

;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

;                                                                      ;

;                                                                      ;

;                       -----------------------                        ;

;                       this is file EVEN.MAC                          ;

;                       -----------------------                        ;

;                                                                      ;

;                                                                      ;

;     Compilation command line:                                        ;

;                                                                      ;

;        MCR>MAC EVEN=EVEN     !  normal command line                  ;

;                                                                      ;

;                                                                      ;

;     ------------------------------------------------------------------

;

;     Name: EVEN to odd

;

;     Computer:              LSI-11/23

;     Operating System:      RSX-11M V4.2 BL38C

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Written:          1/7/87

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;     Description:

;           This routine takes an integer and if it is even makes it odd. The

;	new value is either 1 greater or less then the original. The F77 call

;	is:

;		CALL even(iselect, ivalue) where iselect is the value that

;	determines if addition or subtraction is performed (1 means add, 2

;	subtract, any other value is an error) and ivalue is the variable to be

;	worked on.

;

;     ------------------------------------------------------------------

;

;     Name: GET the high or low BYTe from a word

;

;     Computer:              LSI-11/23 -11/73

;     Operating System:      RSX-11M V4.2 BL38c

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Written:          1/16/11/87

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;     Description:

;           These two routines get a byte from either the upper or lower half of

;	a word. The savings in code are 9 lines in lowb and 7 for highb over

; 	doing this via Fortran. The call is:

;		CALL {GETLOW/GETHI} (iword,byte)  where iword is the word to be

;	manipulated and byte is the byte value retruned.

;

;     ------------------------------------------------------------------

;

;     Name: upper/lower case routines

;

;     Computer:              LSI-11/23 -11/73

;     Operating System:      RSX-11M V4.2 BL38B

;     Language:              MACRO-11

;     Original Programmer:   R. E. K. Neitzel

;                            Rockwell International

;                            Rocky Flats Plant

;                            Process Control Applications

;                            Golden, CO 80401

;                            (303) 966-7171

;

;     Date Written:          1/12/87

;

;

;     Revisions:             Programmer    Date    Description

;                            ----------  --------  ---------------------

;

;

;     Description:

;           These subroutines allow the Macro and Fortran programmer to

;	convert letters in character strings from one case to the other.

;	Non-letter characters and those already in the desired case are not

;	effected. The routines uplow and lowup are Fortran routines to convert

;	upper to lower case and vice versa. The  Fortran calls are:

;		CALL UPLOW (string,length)

;		CALL LOWUP (string,length)     where string is the string to

;	process and length is the number of characters in the string to

;	process.

;

;           The routine $cvtlc is called by Macro programs and by the uplow

;	routine to covert upper to lower case. The Macro call is:

;		put starting address of input string in r0

;		put starting address of output string in r1

;		put number of characters in r2

;		call $cvtlc

;

;	Note that this is the same calling method used for the system library

;	routine $cvtuc. Also note that the Fortran routines always equate the

;	input to the output string.

;