CHAPTER 1

Terminal Driver Modifications

This chapter discusses the  modifications  to  the  RSX

terminal  driver  required  to  support  the security system

(SCLI) for remote dial-up lines.

1.1  Overview

The RSX-11 (M and M+) terminal  driver  unconditionally

forces  MCR  to  a terminal being answered on a remote line.

While intended to provide a consistent interface  to  remote

terminals,  this  functionality  breaks the security system,

which depends upon  SCLI  owning  any  protected  terminals.

Equally  unfortunate is the fact that TTDRV does not use the

normal SCLI$ mechanism to establish MCR, but instead  simply

hammers the executive data structures directly.  The exec is

thus unaware that a port's CLI has changed, and does not in-

form  SCLI of the action.  This prevents SCLI from re-taking

the port.

Fortunately, a very simple patch to the terminal driver

will  'correct'  this  situation.   The  remainder  of  this

chapter describes the modification,  and  includes  instruc-

tions  for making the patch either in the sources or via ZAP

to an existing system.

1.2  Modification Details

The relevant code is in subroutine MANS (modem answer),

in  module TTMOD.MAC.  This file can be found on the distri-

bution kits in [11,10].  MANS is called when a  remote  line

is  answered  (e.g.,  carrier  detected).  Its purpose is to

place the terminal in a known (and benign) state.  MANS mod-

ifies the terminal characteristics words, sets the buffer to

72., and forces the CLI to MCR.  It then initiates  autobaud

(if the terminal is set ABAUD), and returns.

The switch to MCR is accomplished by  one  of  two  in-

structions, depending upon certain SYSGEN conditionals.  For

MPLUS, this is a BIC instruction, while for M it  is  a  MOV

instruction.   Both  are  commented  as  "SET  MCR  AS CLI".

Terminal Driver Modifications                       Page 1-2

Comment out these instructions.

The driver also resets  the  lower-case  bit,  enabling

lower-to-upper   case   conversion.   Since  SCLI  is  case-

sensitive, we wish the terminal  to  be  left  in  lowercase

mode.   This is accomplished by locating the BIC instruction

containing "U2.LWC" in the first operand.  Remove the refer-

ence to "U2.LWC", leaving the other bits intact.

NOTE

Several other characteristics are set in this code

section.   Most  may  be  eliminated, if it is de-

sired.   Examples  include  U2.CRT,  U2.ESC,   and

S3.8BC.   It is also possible to alter the default

buffer size.  The buffer size must be at least 72.

for SCLI to operate properly.

1.3  Source Modification

The desired method of modifying the driver  is  at  the

source level.  UIC [1,101] contains the MPLUS files; [1,102]

those for M.  These uics contains the modified TTMOD.MAC, an

unmodified TTMOD.VGN, and TTMOD.COR.

Although M and M+ share the same source module, the up-

date  versions  are  different, requiring patch sets in both

[1,101] and [1,102].  The patches may have  to  be  manually

applied,  and  new correction files built, for later updates

or releases.   Both  patch  sets  contain  the  audit  trail

"JMB227".

The simplest way to apply these patches is to copy  the

modified  TTMOD  to [11,10] of the sysgen work disk prior to

SYSGEN.  Since TTDRV is always  re-assembled  and  built  by

SYSGEN,  this will incorporate the patches.  It is also pos-

sible to manually assemble TTMOD, and  rebuild  TTDRV  post-

SYSGEN.   Follow  the  instructions in the release notes for

rebuilding a driver.  The database doesn't change, so it  is

possible  to  use  VMR to UNLoad and LOAd the patched driver

without creating a new system image.

Terminal Driver Modifications                       Page 1-3

1.4  Patching TTDRV.TSK

It is also possible to apply these patches to TTDRV.TSK

using  ZAP,  or to a running system using OPEN.  In order to

do this, TTDRV.MAP, plus an assembly listing  of  TTMOD  are

required.   The  patches add no code, and are thus simple to

make.  Simply comment out the  "SET  MCR"  instruction  (all

three  words!),  and  alter the bitmask for the BIC instruc-

tion.

The unmodified  bitmask  is  currently  3047,  and  the

patched value 3046.

CONTENTS

1.  Terminal Driver Modifications  . . . . . . . . . . . 1-1

1.1  Overview  . . . . . . . . . . . . . . . . . . . 1-1

1.2  Modification Details  . . . . . . . . . . . . . 1-1

1.3  Source Modification . . . . . . . . . . . . . . 1-2

1.4  Patching TTDRV.TSK  . . . . . . . . . . . . . . 1-3