;	PBDRV.DOC

;

;	THIS FILE CONTAINS A SET OF FORTRAN CALLABLE SUBROUTINES TO USE

;	LK11 PUSHBUTTON BOX VIA THE PB: DRIVER.

;

;	CALLING PROCEDURES:

;

;		CALL PBATT(LUN,[IOST],[IDS])

;

;			ATTACHES THE PB: UNIT, SPECIFIED BY THE "LUN" ARGUMENT

;			LUN:  LOGICAL UNIT NUMBER

;			IOST: I/O STATUS IN LOW ORDER BYTE (OPTIONAL)

;			IDS:  DIRECTIVE STATUS (OPTIONAL)

;

;		CALL PBDET(LUN,[IOST],[IDS])

;

;			DETACHES THE PB: UNIT.

;			SAME ARGUMENTS AS IN PBATT.

;

;		CALL PBKIL(LUN,[IOST],[IDS])

;

;			KILLS OUTSTANDING I/O ON LOGICAL UNIT NUMBER "LUN".

;			SAME ARGUMENTS AS IN PBATT.

;

;		CALL PBSET(LUN,[IOST],[IDS])

;

;			SET UNIT, ASSIGNED TO "LUN".

;			SAME ARGUMENTS AS IN PBATT.

;

;		CALL PBCLR(LUN,[IOST],[IDS])

;

;			CLEARS UNIT, ASSIGNED TO "LUN".

;			SAME ARGUMENTS AS IN PBATT.

;

;		IF(PBSTAT(LUN,[IOST],[IDS])) GO TO 10

;		OR: CALL PBSTAT(LUN,[IOST],[IDS],[STAT])

;

;			"PBSTAT" IS A LOGICAL FUNCTION AND WILL REFLECT

;			THE STATUS OF THE UNIT SPECIFIED [ON/OFF].

;			IF THE SUBROUTINE IS USED, THE LOGICAL "STAT" WILL

;			CONTAIN THE STATUS.

;			SAME ARGUMENTS AS IN PBATT.

;

;		IF(PBRAL(LUN,[IOST],[IDS])) GO TO 10

;		OR: CALL PBRAL(LUN,[IOST],[IDS],[IVAL])

;

;			"PBRAL" IS AN INTEGER FUNCTION WHICH CONTAINS

;			THE 16 BIT STATUS OF THE LK11.

;			IF THE SUBROUTINE IS USED, "IVAL" REFLECTS THIS

;			STATUS.

;			SAME ARGUMENTS AS IN PBATT.

;

;		CALL PBAST(LUN,IEFN,[AST],[IDS])

;

;			A CALL TO THIS ROUTINE WILL INITIATE AN I/O REQUEST,

;			WHICH WILL BE COMPLETED BY PUSHING THE BUTTON, SPECIFIED

;			BY THE "LUN" ARGUMENT. AT COMPLETION THE EVENTFLAG WILL

;			BE SET AND AN USER SUPPLIED AST ROUTINE WILL BE ENTERED,

;			IF WANTED.

;			THE I/O STATUS WILL BE RETURNED AS ARGUMENT IN THE AST

;			ROUTINE.

;

;			LUN:  LOGICAL UNIT NUMBER

;			IEFN: EVENT FLAG NUMBER

;			AST:  ADDRESS OF AST ROUTINE (OPTIONAL)

;			IDS:  DIRECTIVE STATUS (OPTIONAL)

;