DCREM  -  DC removal and level scaling program

1.0  INTRODUCTION AND USE

This program has been provided to enable fast  scaling  and  DC  removal

from integer and floating point signal files so that they may be output in anal-

ogue form using D/A convertors.  Its main advantage is its  speed.   It  can  be

used  in  integer  mode  on  PDP 11's with EIS or EAE arithmetic units (11/40's,

11/20, 11/34).  Floating point mode only works at present on 11/40's.

To run the program type:

R DCREM

in response to Monitors ..  An asterisk is printed after the program title,  and

a filename and switches in standard RT 11 format should be typed in.  The DC re-

moval and scaling may be done in place or into a new file.   If  only  an  input

file  is provided the scaled signal overwrites that file, otherwise it is placed

in the output file.  If no errors occur then the length of the file  is  printed

for  information  purposes, and the first pass is then done.  This pass consists

of reading the input file to determine the maximum, minimum and DC levels of the

file.  These values are then printed on the terminal in the form:

File maximum aaa @ block bbb, minimum ccc @ block ddd, DC eee

In floating point mode some of these values (aaa, ccc, eee) may be  replaced  by

asterisks  if  the  actual value is outside the range 32767 to -32768 because of

the programs inability to print them.  The second pass then occurs which rereads

the  file  to  scale and remove DC as specified by the switches and write to the

output file.  If floating point input data is specified then a floating point to

integer conversion is done as well.  In all the calculations maximum accuracy is

maintained and rounding done where necessary (to the nearest integer).

2.0  SWITCHES

1.  If no switches are specified the input is assummed to be integer and is

scaled to +/-2047 with DC removal.

2.  If B switch is specified the output is packed in bytes.  For  this  op-

tion  a  value  for /S must be specified less than 256, and since /S is

specified, for DC removal /D must also be explicitly included.

3.  If H switch is specified, a help listing is typed by the program.

4.  If F switch is specified the program treats the input file as  floating

point numbers, otherwise it is always assummed to integer.

5.  If S switch is specified then the input file is scaled  to  either  the

switch value or +/-2047 if no value is specified.

6.  If D switch is specified the input DC level is altered  to  either  the

switch value or 0 if no value is specified.

PAGE 2

7.  If Z switch is specified then the last n blocks of the file are treated

as  zero  in the scaling and DC determining process, and put to zero in

the output file.  n is the switch value or 1 if no value specified.

3.0  ERROR MESSAGES

1.  No EIS or EAE, program won't run

No integer multiply hardware is available so the program cannot run.

2.  No FIS cannot do /F option

No floating point hardware is available so the program  cannot  do  the

floating point option.

3.  Input file driver not found

The device driver for the input file cannot be found by RT11.

4.  Output file driver not found

The device driver for the output file cannot be found by RT11.

5.  Chan in use

A software error has occurred , report to author.

6.  File not found

The input file cannot be found, check type in and reenter if necessary.

7.  No room for output file

From the length of the input file the program has found that  there  is

insufficient space for the output file on the device specified.

8.  Hardware read error

When reading the input file an error occurred,  the  program  is  aban-

doned.

9.  Hardware write error

When writing to the input file an error occurred, the program is  aban-

doned.

10. All zero, nothing done

The input file was found to consist only of zeroes.

11. Write end of file

A software error has occurred , report to author.

12. Write chan not open

A software error has occurred , report to author.

13. No output file name

An output file driver was specified without a filename, because of  the

danger of destroying data this is treated as an error.

14. Illegal switch

PAGE 3

A switch character was not recognised, retype  file  specification  and

switches line.

15. Out of range /Z switch value

Ther is a restriction on the size of the /Z switch value  that  can  be

dealt  with,  this depends on the amount of memory available for DCREM,

to over come this problem use program ZERO.

D.H.Walesby T63D 29/11/79

##################################################################################################################################################################################################################################################################################################################################################################################################