BMUX

Burroughs Polling Protocol

MULTIPLEXOR

SAGE

division of

GENERAL SCIENTIFIC CORPORATION

966 Hungerford Dr.

Rockville, Md. 20850

301-340-2773

TECHNICAL MANUAL

TABLE OF CONTENTS

BMUX USAGE INFORMATION

Startup                                               1-1

Console Commands                                      1-1

Initialization of Operations                          1-2

Status Information                                    1-2

Suspension of Operations                              1-2

Exit Procedure                                        1-2

Configuration of Multiplexor                          1-2

Console messages to a defined port                    1-4

Software Problems                                     1-4

BMUX CONSOLE COMMAND SUMMARY

PROCede                                               2-1

HALT                                                  2-1

MESSage##                                             2-1

STATistics                                            2-1

EXIT                                                  2-2

SOFTWARE PERFORMANCE REPORT

SPR                                                   3-1

HARDWARE

Processor option straping                             4-1

Ports 0 to 3 option strapping                         4-3

Ports 4 to 7 option strapping                         4-5

Ports 8 to 11 option strapping                        4-6

Ports 12 to 15 option strapping                       4-7

Ports 16 to 19 option strapping                       4-8

Ports 20 to 23 option strapping                       4-9

Disk Controller                                       4-10

Disk Drive                                            4-11

Cable Wiring                                          4-12

BMUX MAINLINE LOGIC DOCUMENTATION

Mainline Logic                                        5-1

Interrupt Service Logic                               5-2

Configuration Logic                                   5-2

SYSTEM SOFTWARE EQUATES

SF1 (Source File 1)                                   6-1

SUPPORT SUBROUTINE DOCUMENTATION

BTIME                                                 7-1

GBUFC                                                 7-1

SAVE                                                  7-1

STOR                                                  7-2

READ                                                  7-2

TABLE OF CONTENTS

WRITE                                                     7-2

FORCE                                                     7-2

FNDCOM                                                    7-2

JORSTA                                                    7-3

REBUF                                                     7-3

GPCBDA                                                    7-4

CONWRI                                                    7-4

PCBSET                                                    7-4

BOMB                                                      7-4

CONFDP                                                    7-4

CONCMD                                                    7-4

HLTPRO                                                    7-5

START                                                     7-5

STDISP                                                    7-5

MSG                                                       7-5

PROXIT                                                    7-5

DISTAT                                                    7-6

CONCPY                                                    7-6

Canned Messages                                           7-6

SF20 Module                                               8-1

SF30 Module                                               8-1

SF40 Module                                               8-2

SF50 Module                                               8-3

SF60 Module                                               9-1

BMUX DATA STRUCTURE DOCUMENTATION

CR1-CR5                                                  10-1

CR6                                                      10-1

CRTAB                                                    10-1

TRKSRP                                                   10-1

CONFLG                                                   10-1

TRAPA                                                    10-1

TRDQ2                                                    10-1

TSP-TEP                                                  10-1

PSP-PEP                                                  10-2

SPTR-PEN                                                 10-2

PRPTR-TWPTR                                              10-2

HALTFL                                                   10-2

INIFLG                                                   10-2

STDFLG                                                   10-2

SPFUN                                                    10-2

CCMD                                                     10-2

ISRTAB                                                   10-2

DA1-DA8                                                  10-2

TABLE OF CONTENTS

ISRSAV                                                10-2

CONSAV                                                10-3

RCSRA-XBUFA                                           10-3

RCSRI-XBUFI                                           10-3

WAIT1-NEWSEC                                          10-3

PAT                                                   10-3

CUMTAB                                                10-3

PCB                                                   10-3

POLTAB                                                10-3

TTYPE                                                 10-4

BRTAB                                                 10-4

PCBDAT                                                10-4

TWBPCB                                                10-4

RVITAB-JORST                                          10-4

BMUX USAGE INFORMATION

General Scientific Corporation

Rockville, Md.

1.   STARTUP

The  BMUX software is normally booted automatically when

the system is started by either a power-up  sequence  or  the

depression and lifting of the "RUN" switch on the LSI-11. If,

for some reason, the console terminal shows a period  (.)  on

the  current  line,  it is probably awaiting user response to

the RT-11 console prompt. in this case, enter:

RUN BMUX

and  depress  the  RETURN  key.  This  will  start  the  BMUX

software.  When the start sequence has run, regardless of the

cause of its initiation, the BMUX software will announce  its

readiness by displaying the message:

**** BMUX INITIALIZED TO HALT MODE ****

indicating that the console is functional and the software is

ready for operation.

2.   CONSOLE COMMANDS

The console terminal (Port 2) can asynchronously  access

both  the  Burroughs and the multiplexor's control interface.

If a console command  entry  is  desired,  the  user  of  the

console  terminal  must  start  the  current  line  with  two

Control-V's, created by simultaneously depressing the control

and  "V" keys. The command must immediately follow the second

control-V and be one of the following:

A. PROC        Initiates multiplexor operations

B. STAT        Generates a status  display  for  the  defined

trunk(s) and ports

C. MESS ##     Sends the remainder of the line to Port ##, if

defined

D. HALT        Temporarily suspends multiplexor operations by

preventing input by the ports

E.   EXIT      (must be preceeded by the HALT command) causes

the multiplexor software to release control to

RT-11 (generally used to run the configuration

program)

Invalid console command entry will  be  denoted  by  the

appearence  of  either  "!!!!" (invalid command) or "MESS ??"

(invalid port number). in the case where the  "EXIT"  command

is  entered  but not preceeded by the "HALT" command, it will

be ignored.

3.   INITIALIZATION OF OPERATIONS

Enter the "PROC" console command to begin communications

operations  between  the  port  terminals  and  the Burroughs

mainframe.

4.   STATUS INFORMATION

To examine the status of various ports and the trunk(s),

use  the  "STAT"  console  command. Remember that the display

values for traffic (TFC) and errors (ERR)  are  "wrap-around"

counts.  This  means that an accurate traffic and error count

over any timeframe is the  absolute  difference  between  the

last set of values and the current ones. This also applies to

the values displayed on the "LINK:" line at the  end  of  the

display.  refer  to  the  BMUX  console  command  summary for

display element descriptions.

5.   SUSPENSION OF OPERATIONS

If it is  desired  to  temporarily  block  all  terminal

access  to the Burroughs cpu, the "HALT" console command will

return the software to a pseudo-halt-mode  state  similar  to

that  immediately  after  startup.  In  this  state, the read

buffers are still active, but port  inputs  (except  for  the

console)  are  blocked. If a port terminal user attempts data

entry, the terminal will sound  a  bell  or  similar  audible

alarm.

6.   EXIT PROCEDURE

If  it  is  desired  that  the  software  be  completely

initialized to the startup state, use the "HALT"  and  "EXIT"

console  commands, in that order. the software will then exit

to  RT-11  control.  Refer  to  the  startup  procedures  for

restarting the BMUX software.

7.   CONFIGURATION OF MULTIPLEXOR

To  change the configuration of the multiplexor, use the

EXIT sequence  detailed  above.  when  under  RT-11  control,

enter:

RUN BCONF

and depress the RETURN key.

The configuration program will now run. The use of  this

portion of the software will result in the destruction of the

prior configuration. All ports must be defined, including the

trunk(s).  The  procedure  of  multiplexor configuration will

require verification that the prior configuration  is  to  be

destroyed.  The  user  responds  yes  (Y)  or  no  (N) to the

destruction message. A response of no (N) will cause an  exit

without distrubing the prior configuration.

If a yes (Y)  response  is  entered,  the  program  will

commence  prompting  for  the  trunk  and port parameters. At

least one trunk must be defined, or the software will  reject

definition  of  any  ports.  Port  2,  the  console,  must be

defined;  or  the  BMUX  software  cannot  be   started   and

operations cannot commence.

A trunk definition requires definition of the following:

A.   Two-character poll address

B.   6-character owner name

C.   5-character site name

D.   3-digit  decimal  line  buffer  size   (1-160.

characters)

A port definition requires definition of the  following:

A.   Two-character poll address

B.   6-character owner name

C.   5-character site name

D.   3-digit  decimal  line  buffer  size   (1-160.

characters)

E.   2-digit decimal carriage return  delay  (00-20

null characters)

F.   2-digit decimal form feed  delay  (00-20  null

characters)

G.   A yes (Y) or no (N) response  to  cause  input

characters  to  be  echoed  by the multiplexor

There are two  special  input  conditions  that  can  be

entered for the poll address prompt:

A.   Two   control-V   characters    (created    by

simultaneously  depressing  the  control and V

keys)  cause  the  current  port/trunk  to  be

"skipped",  with  definition proceeding to the

next port/trunk (if any).

B.   A  control-V  and  an "E" character terminates

the  configuration  process  by  setting   the

remaining ports to an undefined state.

After the configuration has been prepared, re-enter  the

BMUX  software  as  shown  in  the  startup information. This

process may be done as often as desired, but it disables  the

operational   capability   of   the  multiplexor  during  the

configuration procedure.

8.   CONSOLE MESSAGES TO A DEFINED PORT

If the user of the console terminal  wishes  to  send  a

message to the user of a defined port, the "MESS" command can

be utilized  (as  opposed  to  using  the  Burroughs  message

capability)  by  entering  the  message line in the following

format:

(control-V)(control-V)MESS ## (message as desired)

Where the ## is the port number assigned to the port  as

shown in the "STAT" display. this capability is convenient in

that it removes  the  processing  of  the  message  from  the

Burroughs  mainframe.  It  is  usable  only within the set of

ports defined by the "STAT" display.

9.   SOFTWARE PROBLEMS

If a software problem should occur,  the  logic  in  the

multiplexor  should  indicate  the  situation by displaying a

"**** BMUX ABORT ****"

message  upon  the console terminal. If this happens, certain

information should be acquired from the system and  the  BMUX

system  problem report (SPR) should be prepared as completely

as possible. When this type of situation occurs, the  console

terminal will then be under DEC "ODT" control. In this state,

the registers ($0, $1,  $2,  $3,  $4,  $5,  and  $6)  can  be

examined  by  entering  the  register  name followed by a "/"

character (ex: $0/ ). When the value has been copied, depress

the  return  key and go on to the next register. When all the

registers have been examined and their values entered on  the

spr  form,  enter  the $6 value, followed by a "/" character.

This will be some number between  0  and  1000  under  normal

abort conditions. when the value displayed by the initial "/"

character has been copied onto  the  SPR  form,  depress  the

line-feed  (LF)  key  to get the next value (displayed in the

form: address/value). repeat this process until  the  address

value  is  1000.  Refer  the form to the appropriate point of

contact for resolution of the cause of the  abort  condition.

BMUX CONSOLE COMMANDS SUMMARY

=============================

Console  commands  for  the BMUX software can be requested at

the console port (PORT NUMBER 02) only!! The format of all console

commands  is  as  follows:  (control-V) (control-V)command(RETURN)

where:

(Control-V)    is  the simultaneous depression of the control

and V keys

(RETURN)       is  the  depression of the return (or carriage

return) key

COMMAND        is  a  four-character  BMUX  command  from the

following table:

PROC           The processing of I/O will now begin

or resume

HALT           All inputs (except console commands)

will suspend

MESS ## (msg)  The   message  (msg)  (whatever  was

typed)  will  be  sent  to  port  ##

STAT           A table of information regarding all

ports and  trunks  defined  will  be

displayed on the console. The format

of the individual table line  is  as

follows:

##  PP  TT  OWNER  LOC  TFC  ERR  STS  RWQ

where:    ##        is the BMUX port number

PP        is the  port's  two-character  ASCII

poll address

TT        is the trunk  BMUX  port  number  to

which  the  port's  input and output

will 'flow' to  and  from  the  host

OWNER     is  the  six-character owner or user

name

LOC       is  the  five-character location for

the port

TFC       is  the count of lines sent/received

on the port

ERR       is the count of input errors for the

port or any errors in trunk  I/O  or

protocol handling

STS       is the port's status indication with

bit values:

1    Input complete

2    Output complete

4    Need an input buffer

8    Statistics count ready

16   Console command input

R    Appears  if  an input buffer is

active for the port

W    Appears  if an output buffer is

active for the port

Q    Appears  if there is port input

queued for the host

The  last  line of the display contains information

specific to the trunk line(s).  This  line  is  labeled:

LINK:

The values contained in the line are:

PRO ERR        Count    of   protocol   errors

detected

POL ADR        Count of invalid poll addresses

received

CONTENTION

1    Trunk  1  (port   #0)   is   in

contention

2    Trunk  2  (port   #2)   is   in

contention

EXIT      If preceeded by a 'HALT' command,  causes  the

BMUX program to exit to RT-11 control.

NOTE

If entered inadvertantly, type:

RUN BMUX(return)

to re-enter the BMUX program.

Please report any difficulties with  this  software  via  the

BMUX System Problem Report (SPR) form.

GENERAL SCIENTIFIC CORPORATION          BMUX PROBLEM REPORT

ODT ADDRESS:___________________       RELEASE:______    DATE:______

REPORTING: PERSON:_____________________  COMPANY:__________________

===================================================================

PROBLEM SYMPTOMS:__________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

===================================================================

REGISTERS:                    STACK:

R0_____________________     >>>>>>________________________

^

R1_____________________     ^     ________________________

^

R2_____________________     ^     ________________________

^

R3_____________________     ^     ________________________

^

R4_____________________     ^     ________________________

^

R5_____________________     ^     ________________________

^

R6_________________STACK>>> ^     ________________________

________________________

________________________

________________________

________________________

________________________

________________________

HARDWARE CONFIGURATION OF BMUX

The  BMUX  Multiplexor  is  designed  about  the  LSI-11

processor,    manufactured    by    Digital    Equipment

Corporation. The BMUX components are arranged in a 4 x 8

backplane arrangement as follows:

PROCESSOR LSI-11

-------------------------          -------------------------

PORTS 0,1,2,3    DLV11J                 DISK CONTROLLER

-------------------------          -------------------------

PORTS 4,5,6,7    DLV11J                 PORTS 8,9,10,11

-------------------------          -------------------------

PORTS 16,17,18,19                       PORTS 12,13,14,15

-------------------------          -------------------------

PORTS 20,21,22,23                       MEMORY

-------------------------          -------------------------

SPARE                                   SPARE

-------------------------          -------------------------

SPARE                                   SPARE

-------------------------          -------------------------

SPARE                                   SPARE

-------------------------          -------------------------

The placement of the component boards  is  critical  and

should  not be arbitrarily relocated. The jumpers on the

processor board should be configured as follows:

PROCESSOR OPTION JUMPERS

JUMPER    STATUS    FUNCTION

______    ______    ________________________________________________

W1        R         Resident memory bank 1 not selected

W2        R         Resident memory bank 2 not selected

W3        R         Event line (LTC) interrupt enabled

W4        I         Processor controlled memory refresh disabled

W5        R         Power up mode 0 selected

W6        R

W7        -         Factory configured bias voltage (do not change)

W8        -         Factory configured bias voltage (do not change)

W9        I         Disable reply from resident memory

W10       R         Disable reply from resident memory during refres

W11       R         Disable on board memory select

SERIAL LINE INTERFACE JUMPER OPTIONS    Lines 0 to 3

Label       Jumper         Function Implemented

_____       ___________    ________________________________________

A12         X to 1         This arrangement of jumpers A5-A12

A11         X to 1         implements the octal base device ad-

A10         X to 1         dress 17650X, which is the assigned

A9          X to 1         address for channel 0 RCSR. The least

A8          X to 0         significant digit is decoded on the mod-

A7          R              ule during operation to address one of

A6          I              four SLU device registers as follows

A5          X to 0             X=0,RCSR

X=2,RBUF

X=4,XCSR

X=6,XBUF

C1          X to 1         These jumpers are used to enable

C2          X to 1         channel 3 for console operation. Base

address must be 176500 (factory con-

figured), 176540, or 177500 for the

console.

BREAK       No Jumper      No response to break

V7          I              This arrangement of jumpers V5-V7

V6          I              implements the octal "base" vector of

V5          X to 0         300 with channel 3 at 60 and 64

CHANNEL 0

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 0         Parity generation and detection

CHANNEL 1

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 0         Parity generation and detection

CHANNEL 2

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 3

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

N0-3        X to 3         EIA RS-423 AND RS-232C

M0-3        X to 3         EIA RS-423 AND RS-232C

R10         22 kohm        Channels 0 and 1, slew rate of 2usec

R23         22 kohm        Channels 2 and 3, slew rate of 2usec

BAUD RATE

Port        Jump           Description

0           to N           Trunk 0 set to 9600 bps

1           to N           Trunk 1 set to 9600 bps

2                          selected remotely

3                          selected remotely

SERIAL LINE INTERFACE JUMPER OPTIONS    Lines 4 to 7

Label       Jumper         Function Implemented

_____       ___________    ________________________________________

A12         X to 1         This arrangement of jumpers A5-A12

A11         X to 1         implements the octal base device ad-

A10         X to 0         dress 17564X, which is the assigned

A9          X to 1         address for channel 0 RCSR. The least

A8          X to 1         significant digit is decoded on the mod-

A7          I              ule during operation to address one of

A6          R              four SLU device registers as follows

A5          X to 1             X=0,RCSR

X=2,RBUF

X=4,XCSR

X=6,XBUF

C1          X to 0         These jumpers are used to disable

C2          X to 0         channel 3 for console operation.

BREAK       No Jumper      No response to break

V7          I              This arrangement of jumpers V5-V7

V6          I              implements the octal "base" vector of

V5          X to 1         340

CHANNEL 0

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 1

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 2

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 3

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

N0-3        X to 3         EIA RS-423 AND RS-232C

M0-3        X to 3         EIA RS-423 AND RS-232C

R10         22 kohm        Channels 0 and 1, slew rate of 2usec

R23         22 kohm        Channels 2 and 3, slew rate of 2usec

SERIAL LINE INTERFACE JUMPER OPTIONS    Lines 8 to 11

Label       Jumper         Function Implemented

_____       ___________    ________________________________________

A12         X to 1         This arrangement of jumpers A5-A12

A11         X to 1         implements the octal base device ad-

A10         X to 0         dress 17570X, which is the assigned

A9          X to 1         address for channel 0 RCSR. The least

A8          X to 1         significant digit is decoded on the mod-

A7          I              ule durRng operation to address one of

A6          I              four SLU device registers as follows

A5          X to 0             X=0,RCSR

X=2,RBUF

X=4,XCSR

X=6,XBUF

C1          X to 0         These jumpers are used to disable

C2          X to 0         channel 3 for console operation.

BREAK       No Jumper      No response to break

V7          I              This arrangement of jumpers V5-V7

V6          I              implements the octal "base" vector of

V5          X to 1         340

CHANNEL 0

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 1

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 2

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 3

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

N0-3        X to 3         EIA RS-423 AND RS-232C

M0-3        X to 3         EIA RS-423 AND RS-232C

R10         22 kohm        Channels 0 and 1, slew rate of 2usec

R23         22 kohm        Channels 2 and 3, slew rate of 2usec

SERIAL LINE INTERFACE JUMPER OPTIONS    Lines 12 to 15

Label       Jumper         Function Implemented

_____       ___________    ________________________________________

A12         X to 1         This arrangement of jumpers A5-A12

A11         X to 1         implements the octal base device ad-

A10         X to 0         dress 17574X, which is the assigned

A9          X to 1         address for channel 0 RCSR. The least

A8          X to 1         significant digit is decoded on the mod-

A7          I              ule during operation to address one of

A6          I              four SLU device registers as follows

A5          X to 1             X=0,RCSR

X=2,RBUF

X=4,XCSR

X=6,XBUF

C1          X to 0         These jumpers are used to disable

C2          X to 0         channel 3 for console operation.

BREAK       No Jumper      No response to break

V7          I              This arrangement of jumpers V5-V7

V6          I              implements the octal "base" vector of

V5          X to 1         340

CHANNEL 0

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 1

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 2

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 3

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

N0-3        X to 3         EIA RS-423 AND RS-232C

M0-3        X to 3         EIA RS-423 AND RS-232C

R10         22 kohm        Channels 0 and 1, slew rate of 2usec

R23         22 kohm        Channels 2 and 3, slew rate of 2usec

SERIAL LINE INTERFACE JUMPER OPTIONS    Lines 16 to 19

Label       Jumper         Function Implemented

_____       ___________    ________________________________________

A12         X to 1         This arrangement of jumpers A5-A12

A11         X to 1         implements the octal base device ad-

A10         X to 1         dress 17600X, which is the assigned

A9          X to 0         address for channel 0 RCSR. The least

A8          X to 0         significant digit is decoded on the mod-

A7          R              ule during operation to address one of

A6          R              four SLU device registers as follows

A5          X to 0             X=0,RCSR

X=2,RBUF

X=4,XCSR

X=6,XBUF

C1          X to 0         These jumpers are used to enable

C2          X to 0         channel 3 for console operation.

BREAK       No Jumper      No response to break

V7          I              This arrangement of jumpers V5-V7

V6          I              implements the octal "base" vector of

V5          X to 1         340

CHANNEL 0

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 1

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 2

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 3

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

N0-3        X to 3         EIA RS-423 AND RS-232C

M0-3        X to 3         EIA RS-423 AND RS-232C

R10         22 kohm        Channels 0 and 1, slew rate of 2usec

R23         22 kohm        Channels 2 and 3, slew rate of 2usec

SERIAL LINE INTERFACE JUMPER OPTIONS    Lines 20 to 23

Label       Jumper         Function Implemented

_____       ___________    ________________________________________

A12         X to 1         This arrangement of jumpers A5-A12

A11         X to 1         implements the octal base device ad-

A10         X to 1         dress 17604X, which is the assigned

A9          X to 0         address for channel 0 RCSR. The least

A8          X to 0         significant digit is decoded on the mod-

A7          R              ule during operation to address one of

A6          R              four SLU device registers as follows

A5          X to 1             X=0,RCSR

X=2,RBUF

X=4,XCSR

X=6,XBUF

C1          X to 0         These jumpers are used to disable

C2          X to 0         channel 3 for console operation.

BREAK       No Jumper      No response to break

V7          I              This arrangement of jumpers V5-V7

V6          I              implements the octal "base" vector of

V5          X to 1         340

CHANNEL 0

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 1

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 2

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

CHANNEL 3

E           X to 1          Even parity

D           X to 1         8 data bits

S           X to 0         1 stop bit

P           X to 1         No Parity generation and detection

N0-3        X to 3         EIA RS-423 AND RS-232C

M0-3        X to 3         EIA RS-423 AND RS-232C

R10         22 kohm        Channels 0 and 1, slew rate of 2usec

R23         22 kohm        Channels 2 and 3, slew rate of 2usec

DISK CONTROLLER

The  MDC11  Disk  Controller  is a peripheral controller

intended for use in LSI-11 based computer systems. The option

straps are set as follows:

Label     Jumper           Function Implemented

-----     ------           ---------------------------------------

R0        I                Positioning step time period

R1        I                set at 40 milliseconds

Mx        None             1.6 second time out delay

RD        I                Refresh Disable

R13       I                Address 13

173       I                Enable address 173000

A15       R                Prom address bit 15

A14       R                Prom address bit 14

A13       I                Prom address bit 13

PD        I                Prom Disable

ER0       I                Enable Prom block 0

ER1       I                Enable Prom block 1

ER2       I                Enable Prom block 2

ER3       I                Enable prom block 3

A-B       I                A-B

C-D       I                C-D

E         R                E

12M       I                12 Volt supply

A11       R                Address 11 latched

5M        I                -5 Volt supply to ROMS

R-Q       R                R-Q

B12       R                Base address of minidisk

B11       R                controller located at 177200.

B10       R                  Register

B9        R                    Command-Status (CSR) 177200

B8        I                    Track (TR)           177202

B7        R                    Sector               177204

B6        I                    Data                 177206

B5        I

B4        I

B3        I

M1        R                Time out delay

M2        R                set at

M4        R                1.6 seconds after no user access

MD        R                Motor disable

DISK DRIVE

The disk utilized is the MPI 51/52 or the Shugart SA400.

They  are  physically  and  electrically interchangeable. The

option straps are as follows:

Label     Jumper           Function Implemented

-----     ------           ---------------------------------------

T1        I                Head solinoid with select

T2        I                Drive select 1 selected

T3        R                Drive select 2 not selected

T4        R                Drive select 3 not selected

T5        I                Drive always selected

T6        R                Drive 4 not selected

T7        I                Head solinoid with motor on

BMUX MAINLINE LOGIC DOCUMENTATION

The software is divided into three major portions:

A.   Mainline Logic

B.   Interrupt Service Logic

C.   Configuration Logic.

The  mainline   logic   is   divided   into   two   portions:

Initialization  and  Controller  Logic.  The  initialization logic

presets the operational conditions for the proper  functioning  of

the   controller  portion.  The  controller  handles  all  garbage

collection, I/O start/stop and control interface functions.

The initialization section does the following:

A.   Saves the current contents  and  presets  the  interrupt

vector address contents for the proper interrupt service

routine entry,

B.   Reads  the configuration information from disk to preset

the port- dependent operations  of  the  controller  and

interrupt service portions of the software,

C.   Prints an initialization  message  on  the  console  and

initiates the input process for the console, and

D.   Presets  flags  to  show  that  the   logic   has   been

initialized.

The controller section does the following:

A.   Starts all read/write I/O sequences for the trunk(s) and

ports,

B.   Processes   all   read/write  I/O  completions  for  the

trunk(s) and ports,

C.   De-queues  all traffic and messages for the trunk(s) and

ports,

D.   Acquires  and releases all buffers from/to the trunk and

port pools,

E.   Processes all console control interface commands,

F.   Maintains (when possible) a  double-read  condition  for

the trunk(s),

G.   Journals traffic and error statistics (by trunk/port) to

a cumulative table, and

H.   Handles the timer expiration situation for the trunk(s),

and   the  interrupt  service  logic  resides  with  the

mainline   logic   and   asychronously   services    the

transmission/receipt  of  characters by the trunk(s) and

ports. the trunk read ISR  (interrupt  service  routine)

works hand-in-hand with the controller section and trunk

write ISR to provide response to the Burroughs protocol.

The  port  read/write  ISR's process buffers to/from the

trunk(s) and ports. Messages from the  console  port  to

the  other  ports  is picked up by the command processor

segment of the controller section  and  is  massaged  to

look like a trunk read completion.

The buffers pools are separate and homogeneous  in  size  and

structure. Hence the same acquisition/release routines can be used

for both. The trunk read pool is used to supply read  buffers  for

the  trunk  read  ISR,  which  become  port write buffers when the

burroughs has sent a buffer to  a  port  in  a  'select'  protocol

sequence.  The  port  read pool is used to supply read buffers for

the port read ISR, which become trunk-bound write buffers. Console

messages  reside in the port read buffer, but are massaged to look

like a trunk read destined  for  the  specified  port.  These  are

dequeued  by  a  special segment of the mainline's traffic-routing

routines.

A  third  segment of the software is the configuration logic,

which shares and sets up defined port values in the data structure

segment (SF2) to enable the controller and interrupt service logic

to function as the user desires.

The  in-core  structure  of the multiplexor and configuration

portions of the software  (low-to-high  memory  adressing)  is  as

follows:

program stack            program stack

|                        |

data structure (SF2)          data structure (SF2)

|                        |

buffer pools (SF98, SF99)     configuration logic (SF60)

|

initialization and controller (SF10)

|

command processing routines (SF12)

|

trunk read ISR (SF20)

|

port write ISR (SF30)

|

trunk read ISR (SF40)

|

trunk write ISR (SF50)

|

canned messages (SF90)

These  two software entities cannot co-exist in memory at the

same time.

Buffer  request  and  release  are  controlled  by two common

routines having separate entry points for Trunk  and  Port  buffer

processing.  These routines maintain pointers for each buffer pool

which denote the start and end of the group of  buffers  currently

active  for  that  pool.  Traffic  scans  will  begin at the start

pointer value and stop when the end pointer value is reached. When

a  buffer  is  requested from the buffer pool, the search is first

made from the buffer immediately following the end  pointer  until

the  start  pointer  value  is reached. If none exist, a search is

made from the  start  pointer  to  the  end  pointer  looking  for

"embedded"  buffers  that  are available for use. When a buffer is

released to either buffer pool, the start and/or end pointers  are

updated  IF  the  buffer was being pointed to by either pointer. A

buffer is active if its Read Queue (RQ) byte is NOT a -1 value. If

the  Write Queue (WQ) byte is NOT a -1 value, the buffer is queued

for transfer to a Trunk or Port. Refer to  the  following  diagram

for  a  sample  condition of the two buffer pools, Trunk and Port.

In the example, Trunk buffers 4 and 5 are  active  for  Trunk

Read purposes. Due to the positional order of the two, buffer 4 is

probably active  and  buffer  5  is  waiting  for  use  (refer  to

references  to  the  "double-queued" buffers for Trunk reads). The

other Trunk buffers have completed the Trunk Read process and  are

awaiting  (or in the process of) transmission to the specified PCB

terminals: buffer 1 is queued for PCB 2, followed by buffer 3, and

buffer 2 is queued for PCB 3. In the Port buffer pool, buffers are

actively being used for Port Reads: buffer 6 is active for PCB  5,

buffer  11  for  PCB 3, and buffer 12 for PCB 4. Buffers 7-10 have

completed the Port Read process and are awaiting  transmission  to

the  Trunk.  Note that PCB 2 has NO buffer queued in the Port Read

buffer pool. It has probably JUST completed a Read process and the

mainline  has  NOT YET requested a new buffer for the Read process

for PCB 2.

In  the general flow of buffers, Port Read buffers "feed" the

Trunk Write process, just as Trunk Read buffers  "feed"  the  Port

Write  process.  The  "Traffic  Queues" for each Write process are

loosely held between the start and end pointers of the two  buffer

pools. Traffic dequeuing occurs when a Trunk or Port has completed

its last Write process and is ready to commence another. The Trunk

Write  dequeuing is controlled by the Burroughs requests, however.

It is probably obvious that the Trunk buffer pool stands the  best

chance  of  becoming  "staurated"  due to large "bursts" of output

from the Burroughs processor. This is purely  dependent  upon  the

ability  of  the defined Port PCB's to output the buffers received

from the  Trunk  fast  enough  to  keep  ahead  of  the  Burroughs

transmissions.   The   software   DOES   prohibit  further  buffer

transmittal from the Burroughs when the Trunk buffer pool  becomes

saturated  by  "throttling"  the  transmission  until at least one

Trunk buffer is available for the Read process.

.TITLE    BMUX -GENERAL SCIENTIFIC CORP.

.IDENT    /000001/  ;  GAITHERSBURG, MARYLAND

;

;    COPYRIGHT (C) 1979

;

; THIS SOFTWARE IS FURNISHED UNDER A LICENSE FOR USE ONLY ON A SINGLE

; COMPUTER SYSTEM AND MAY BE COPIED ONLY WITH THE INCLUSION OF THE ABOVE

; COPYRIGHT NOTICE AND THE EXPRESSED, WRITTEN PERMISSION OF GENERAL

; SCIENTIFIC CORPORATION.  TITLE TO AND OWNERSHIP OF THIS SOFTWARE SHALL

; REMAIN WITH GENERAL SCIENTIFIC CORPORATION.  COPIES OF THIS SOFTWARE

; MAY NOT BE PROVIDED TO OR OTHERWISE MADE AVAILABLE TO ANY OTHER PERSON

; EXCEPT FOR USE ON A GENERAL SCIENTIFIC CORPORATION SUPPLIED SYSTEM.

;

; THE INFORMATION IN THIS SOFTWARE IS SUBJECT TO CHANGE WITHOUT NOTICE AND

; SHOULD NOT BE CONSTRUED AS A COMMITMENT BY GENERAL SCIENTIFIC CORPORATION.

;

.LIST

;

;

;

.PAGE

.SBTTL    NOTES

;

;    THE FOLLOWING SET OF EQUATES EXPRESSES THE DESIGN PARAMETERS

;    FOR THE BMUX SOFTWARE.  NOT ALL OF THE EQUATES ARE USED IN THE

;    INITIAL VERSION OF THE SYSTEM.  SOME LOCAL EQUATES OCCUR IN SOME

;    OF THE MODULES.  AT A LATER TIME, THESE WILL BE MOVED TO THIS

;    SOURCE FILE.

;

;    ***** IMPORTANT NOTE: *****

;

;    THIS SOURCE FILE ** MUST ** BE ASSEMBLED WITH EVERY OTHER SOURCE

;    FILE, SF2-SF99.

;

;

.PAGE

.SBTTL    MACRO CALLS AND DEF'S

;

.MCALL    .SPND,.TWAIT,.REGDEF,..V2..

;

..V2..    ; VERSION 2 MACRO CALLS

.REGDEF   ;MAKE LIFE EASIER

;

.PAGE

.SBTTL    DLV11-J EQUATES

;    THIS SECTION DEFINES THE DLV-11J DEVICE BIT SETTINGS AND REGI

;    DEVICE HARDWARE REGISTER OFFSETS RELATIVE TO THE GIVEN

;    DEVICE'S RCSR ADDRESS.   REFER TO THE DEC LSI-11 PERIPHERALS

;    MANUAL FOR INFORMATION REGARDING THE OPERATIONS OF THE DLV-11J

;    VIA ITS HARDWARE REGISTERS.

;

;

;    RCSR:     READ CHANNEL CONTROL & STATUS REGISTER

;

RCSR=0         ;DEVICE ADR REG OFFSET

R.DONE=200     ;RCVR DONE BIT

R.IE=100  ;INTERRUPT ENABLE BIT

R.RE=1         ;RCVR ENABLE (CLEARS R.DONE)

R.KILL=101     ;RCVR STOP BITS

;

;    RBUF:     READ CHANNEL BUFFER REGISTER

;

RBUF=2         ;DEV ADR OFFSET

;

;    STATUS BIT VALUES FOR RBUF

;

R.ER=100000    ;ERROR BIT

R.OV=40000     ;OVER-RUN ERROR

R.FR=20000     ;FRAMING ERROR

R.PA=10000     ;PARITY ERROR

;

;    XCSR:     TRANSMIT CHANNEL CONTROL & STATUS REGISTER

;

XCSR=4    ;DEV ADR OFFSET

X.RDY=200 ;XMTR READY BIT

X.IE=100  ;INTERRUPT ENABLE BIT

X.BR=1         ;XMTR BREAK (SET TO BREAK XMIT)

;

;    XBUF:     TRANSMIT CHANNEL BUFFER REGISTER

;

XBUF=6         ;DEV ADR OFFSET

;    DATA IS TRANSMITTED BY MOVING A CHARACTER INTO THE LOWER BYTE

;

.PAGE

.SBTTL    SYSTEM EQUATES

;

;    THE FOLLOWING VALUES SET THE LIMITS OF THE MULTIPLEXOR'S

;    DESIGN AND DEFINE CERTAIN CRITICAL VALUES, SUCH AS THE PORT

;    CONTROL BLOCK (PCB) NUMBER FOR THE CONSOLE.

;

;

MAXPRT=24.     ;MAX # OF PORTS DEF'D

CONSOL=2  ;PORT # OF CONSOLE DEVICE

CONVEC=60 ;CONSOLE VECTOR

TRKA=0         ;PCB FOR TRUNK A

TRKB=1         ;PCB FOR TRUNK B

STA=MAXPRT     ;STATUS POLL FOR TRUNK A

STB=MAXPRT+1   ; SAME FOR B

STALL=MAXPRT+2 ;BOX STATISTICS POLL PCB #

TRAVEC=300     ;TRUNK A VECTOR

TRBVEC=310     ;TRUNK B VECTOR

CMDL=4         ;LEN OF ASCII CMD IN BYTES

NCMD=6         ;NUMBER OF CONSOLE COMMANDS

CTABL=6        ; LEN OF CMD TABLE ENTRY

TIMOUT=100.    ;10. SECOND MAX TIME-OUT

NUMBUF=54.     ;# BUFFERS IN EACH (OF TWO) POOLS

BUFSIZ=170.    ;SIZE OF DATA PORTION OF BUFFER

BUFLEN=BUFSIZ+6     ;LEN OF EACH BUFFER

.PAGE

.SBTTL    CHAR RECOGNITION EQUATES

;

;    THE FOLLOWING EQUATES DEFINE ASCII CHARACTER VALUES OF IMPORTANCE

;

;    TO THE PROCESSING OF THE TERMINAL PORTS, SUCH AS THE RETURN KEY.

;

;    TERMINAL ASCII:

;

ESC=33         ;ESCAPE

CR=15          ;CARRIAGE RETURN

LF=12          ;LINE-FEED

FF=14          ;FORM-FEED

NULL=0         ;NULL

BS=10          ;BACK-SPACE

DEL=177        ;LINE DELETE

SYNC=26        ;SYNC (CMD FLAG CHAR)

;

;

;    THE FOLLOWING VALUES ARE OF SPECIAL IMPORTANCE TO THE PROCESSING

;

;    OF THE BURROUGHS PROTOCOL BY THE MAINLINE AND TRUNK ISR'S, SUCH

;

;    AS THE POL OR SEL CHARACTERS (WHICH INDICATE 'POLL' AND 'SELECT'

;

;    REQUESTS ISSUED BY THE BURROUGHS' LINE CONTROL PROGRAM).

;

;    BURROUGHS ASCII:

;

ENQ=5          ;ENQUIRE

POL=160        ;PORT-TO-B SELECT

SEL=161        ;B-TO-PORT SELECT

CON=7          ;CONTENTION MODE (BELL)

BELL=7

EOT=4          ;END-OF-XMIT

ETX=3          ;END-OF-TEXT

ACK=6          ;ACKNOWLEDGE

NAK=25         ;NEGATIVE ACKNOWLEDGE

SOH=1          ;START OF HEADER

STX=2          ;START OF TEXT

;

RVI=40       ;REVERSE INTERRUPT USED TO STOP OUTPUT FROM HOST (NOT USED)

;

;

.PAGE

.SBTTL    PCB DEFINITION

;

;    THE FOLLOWING EQUATES DEFINE THE STRUCTURE AND SOME OF THE DATA

;    CONTENTS OF THE PORT CONTROL BLOCK (PCB).  CURRENTLY, THE P.BAUD

;    AND P.TYPE BYTES ARE NOT USED; DUE TO THE IMPLEMENTATION OF THE

;    DLV-11J DEVICES.

;

;

P.SOU=0        ;PCB INDEX TO POLTAB FOR THIS PORT (-1 IF NOT DEF'D)

P.DST=1   ;PCB INDEX TO POLTAB FOR DESTINATION FROM HERE (OR -1)

P.PST=2        ;PORT STATUS BYTE

;

;    P.PST BIT MEANINGS:

;

PS.RF=1        ;READ FINISHED BIT

PS.WF=2        ;WRITE FINISHED BIT

PS.RB=4        ;READ BUFFER NEEDED BIT

PS.JS=10  ;JOURNAL STATS HOLD WD TO BUFFER BIT

PS.CMD=20 ;PROCESS COMMAND FROM CONSOLE BIT

PS.TQ=40  ;TRAFFIC QUEUED FOR TRUNK BIT

;

P.DELF=3  ;DELAY FLAG (0=>INACTIVE)

;

;    P.DELF BIT MEANINGS:

;

PD.CR=1        ;CR DELAY FLAG

PD.FF=2        ;FF DELAY FLAG

;

P.BAUD=4  ;PORT BAUD RATE (INFO ONLY)

P.TYPE=5  ;PORT TERM TYPE (INFO ONLY)

P.LSIZ=6  ;LINE LENGTH (BYTES)

P.ECHO=7  ;ECHO FLAG  (0=>NO)

P.CRD=10  ;CR DELAY VALUE (0=NONE)

P.FFD=11  ;FF DELAY VALUE (0=NONE)

P.STAT=12 ;STATS FLAG (0=NONE)

;

;    P.STAT BIT MEANINGS:

;

PST.E=1        ;ERROR STATISTICS COLLECTION BIT

PST.T=2        ;TRAFFIC STATISTICS COLLECTION BIT

;

;

P.WIND=13 ;STATS WINDOW (.1 SEC 'TICKS')

P.ERR=14  ;ERROR STATS COUNTER

P.TFC=15  ;TFC STATS COUNTER

P.RST=16  ;READ STATUS BYTE

P.WST=17  ;WRITE STATUS BYTE

;

;    READ/WRITE STATUS BIT SETTINGS

;

PS.DOR=300     ;RD: DATA OVER-RUN ERROR

PS.FRA=240     ;RD: FRAMING ERROR

PS.PAR=220     ;RD: PARITY ERROR

PS.TMO=210     ;WR: TIME-OUT ERROR

PS.OK=1        ;R/W: SUCCESSFUL COMPLETION

;

;

P.RC=20        ;READ CHAR CNT

P.WC=21        ;WRITE CHAR CNT

P.RBFA=22 ;READ BUFFER ADR

P.RBP=24  ;READ BUFFER PTR (CURRENT)

P.WBFA=26 ;WRITE BUFFER ADR

P.WBP=30  ;WRITE BUFFER PTR (CURRENT)

P.SHLD=32 ;STATS HOLD WD (FOR JOURNALING TO CUMULATIVE TABLE)

P.STIM=34 ;STATS WINDOW TIMER

P.WTIM=35 ;WRITE I/O TIMER

P.DC=36        ;DELAY CNTR (0= NO DELAY, ELSE # OF NULL'S)

P.ECH=37  ;ECHO CHAR AWAITING OUTPUT

;

PCBLEN=40 ;LEN OF PCB IN BYTES

;

.PAGE

.SBTTL    BUFFER DEF'S

;

;    THE BUFFERS OF BOTH POOLS ARE HOMOGENEOUS IN SIZE AND FLAGS USAGE

;    IN THE F(IN THE FIRST 3 WDS OF EACH BUFFER).  THE FIRST BYTE IS A -1 IF IT

;    IS NOT QUEUED FOR USE AS A READ BUFFER.  THE SECOND BYTE IS A -1 IF

;    IT IS NOT QUEUED TO BE WRITTEN TO A SPECIFIED PORT (PCB NUMBER).

;    THE THIRD BYTE CONTAINS THE READ/WRITE CHARACTER SIZE (MAX 255.).

;    THE FOURTH BYTE CONTAINS A ZERO IF THE BUFFER IS NOT ACTIVELY BEING

;    USED, ELSE IT CONTAINS A 1.  THE FIFTH AND SIXTH BYTES ARE USED TO

;    COMPUTE THE BCC CHARACTER VALUE FOR BUFFERS DESTINED FOR TRANSMISSION

;    TO THE BURROUGHS VIA EITHER TRUNK.

;

;    PORT AND TRUNK READS BEGIN AT THE APPROPRIATE POINTS WITHIN THE BUFFER,

;    SINCE THE BURROUGHS HEADER VALUES ARE PLACED AT THE BEGINNING OF THE

;    PORT READ BUFFER PRIOR TO ITS TRANSMISSION.  BOTH THE ETX AND BCC

;    (COMPUTED) CHARACTERS ARE AFFIXED TO THE END OF THE PORT READ BUFFERS.

;

;    THE BUFFERS ARE MANAGED BY THE DATA LISTHEAD PTRS:

;

;    PST: START OF PORT ACTIVE BUFFERS

;    PEP: END OF PORT ACTIVE BUFFERS

;    TSP: START OF TRUNK ACTIVE BUFFERS

;    TEP: END OF TRUNK ACTIVE BUFFERS

;

;    THE BUFFER POOL(S) ARE COMPLETELY USED WHEN THE START PTR

;    VALUE IS THE SAME AS THE END PTR.

;

;

;

RQ=0 ;READ Q BYTE: -1=AVAIL,ELSE PORT #

WQ=1 ;WRITE Q BYTE: SAME AS RQ

SZ=2 ;R/W CHAR SIZE (MAX 146.)

AFL=3     ;ACTIVE FLAG (0=INACT, 1=ACT)

TDATA=6   ;START OF TRUNK READ

PDATA=10. ;START OF PORT READ DATA

;           IF XM# USED, PDATA SHOULD BE 11.

BCC=4          ;BCC COMPUTATION WD

BAD0=7    ;START OF TWO-BYTE POLL ADR IN HEADER

;         NOTE: AREA FROM TDATA TO PDATA CONTAINS BURROUGHS HDR

;

;

.LIST

BMUX Support Subroutine Documentation

1.   BTIME: Trunk Timeout Handler

This  subroutine  verifys  that  the  Trunk timeout

message byte is not as yet set. If so, the message  byte

is  set  to  trigger the Console message indicating that

this Trunk is not responding to communication  attempts.

2.   GBUFC: Buffer Request Handler

This subroutine has two entry points:

GTBUF:    To  request  a  Trunk  read  buffer,  and

GPBUF:    To request a Port read buffer.

The   common  routine  processes  the  buffer  pool

pointers as set by the entry point logic to  locate  and

allocate a buffer for the Trunk/Port which has requested

the action. If a  buffer  is  located,  its  address  is

placed  on  the  top  of  the stack upon return from the

subroutine. If no available buffers were found,  a  zero

will  be  returned.  This  routine  processes  the  pool

starting at the  first  buffer  after  the  current  end

pointer. If needed, the "search" pointer is reset to the

top of the buffer  pool  being  examined  (this  is  the

"wrap-around" case). If scanning from the end pointer to

the start pointer fails to provide an available  buffer,

the  scan  will  then  switch to the "embedded" case and

search from the start pointer to the end  pointer.  Once

the entire buffer pool is checked, a zero is returned to

the calling routine to indicate the pool  is  completely

used.  If a buffer islocated, the end pointer is the set

to the buffer's address, unless this was  the  "embedded

case",  to  set  the  limits  for the mainline's traffic

scan.

This is an important point, since the start and end

pointers are the limits for the mainline's dequeuing  of

traffic for the Trunk and Port write processes.

3.   SAVE:     Register Save Routine

When  the  routine  to  be  executed  will  destroy

register contents that are  needed  after  the  routines

execution,  this  routine is called to save the contents

of the registers, R0 through R5, on the stack (SP).  The

RSTOR  subroutine must be called at the logical point to

restore the registers to their saved values  just  prior

to exit from the calling routine.

4.   STOR:     Register Restoration Routine

This routine restores the registers, R0 through R5,

to the values saved  on  the  stack  (SP)  by  the  SAVE

routine.  See  usage  note for the SAVE routine for more

information regarding the application of  this  pair  of

subroutines.

5.   READ:     Trunk/Port Read Process Initiation Routine

When   the   mainline   processing   requires   the

initiation of a new read condition for a Trunk or  Port,

this   routine  is  called  to  preset  the  appropriate

pointers and counters  so  the  read  interrupt  service

routines  can  process the characters received and place

them in the Trunk/Port's read buffer. Just prior to exit

from  this  routine,  the  Interrupt  Enable bit for the

Trunk/Port is set in the appropriate Read Command/Status

Register (RCSR) to enable interrupt processing.

6.   WRITE:    Trunk/Port Write Process Initiation Routine

This routine has a special entry point, FORCE, used

to transmit the Trunk's  Contention  Break  message  (in

accordance  with  the  Burroughs  protocol).  The entire

routine functions similarly to the READ routine in  that

it  sets  up  the appropriate pointers to support output

character  interrupt  processing   by   the   Trunk/Port

interrupt  service  routines. In addition to setting the

Interrupt  Enable  bit  in  the  Trunk/Port's   Transmit

Command/Status  Register  (XCSR), the FIRST character to

be transmitted MUST be loaded into the  Transmit  Buffer

Register  (XBUF) to initiate the transmission process to

accomplish the Write function.

7.   FORCE:    Burroughs   Protocol   Contention  Break  to  Trunk

Routine

Refer   to   the   WRITE   routine  for  processing

information.  This  routine   is   entered   after   the

Contention  Break  message  has  been set up to initiate

communications  on  the  Trunk  line  NOW,  rather  than

waiting  for  the  next  cyclic  polling  process  to be

initiated by the Burroughs end of  the  link.  For  more

information  on  the conditions for initiation, refer to

the FNDCOM routine description.

8.   FNDCOM:   Traffic Dequeuing (Trunk/Port) Routine

This routine has two entry points:

FNDMSG:   Sets  up  Trunk  Write traffic scans, and

FNDPWB:   Sets up Port Write traffic scans.

The common logic scans the buffer pool specified by

the entry point, looking for buffers in the  pool  whose

Write  Queue  (WQ)  byte  contains the Trunk/Port's Port

Control Block (PCB) number as found in the PCB's  Source

(P.SOU) byte.

If a match  occurs,  the  buffer  address  will  be

loaded  into the Port's Write Buffer address word in the

PCB so the Write process can be initiated.

The  Trunk  is  processed  slightly differently. In

order for the buffer to be dequeued, the Read Queue (RQ)

byte  in the buffer MUST match the Dequeue byte (TRKRSP)

for this Trunk. Otherwise, scanning continues. If a Port

buffer  is  dequeued  for the Trunk, it is placed into a

holding array (TWBPCB)  ordered  in  PCB  sequence.  The

logic  then  determines  if a Break Contention is needed

for this the Trunk line to transmit the  buffer  to  the

Burroughs  processor.  If  not,  control  returns to the

calling routine, and the buffer will be  transmitted  in

its  proper  sequence.  If a Contention Break is needed,

the source PCB number from the buffer will be encoded in

ASCII  and  placed  in  the  Contention Break buffer and

control given to the FORCE  routine  to  "wake  up"  the

Burroughs.

9.   JORSTA:   Statistics Journaling Routine

This  routine  transfers  the  traffic  (P.TFC) and

error (P.ERR) byte values from the holding word (P.SHLD)

in  the  PCB into the "wrap-around" word accumulators in

the Statistics Cumulative array (CUMTAB), ordered in PCB

sequence.  This  array can be displayed using the 'STAT'

Console command.

10.  REBUF:    Buffer Release Routine

This routine returns buffers to  either  pool  upon

completion   of  the  Trunk/Port  Write  process.  After

return, it frees enables the dequeuing  of  traffic  for

the  Trunk  or Port by clearing the Write Buffer Address

(P.WBFA) word, which allows  the  traffic  dequeuing  to

resume. The header words of the buffer are reset and the

the buffer pool pointers (start/end)  are  appropriately

updated.

11.  GPCBDA:   Device Address Pointers Preset Routine

When  the mainline logic requires that the Read and

Transmit Device  Address  Registers  be  accessed,  this

routine  will  preset the pointer words according to the

PCB number contained in the PCB's source  byte  (P.SOU).

12.  CONWRI:   Console Message Output Routine

This routine copies a "canned" message into a Trunk

read  buffer  and queues it for the Console port's write

process. If the Trunk read  buffer  pool  is  full,  the

C-bit  will  be  set  upon  return,  indicating that the

routine failed to complete the requested action.

13.  PCBSET:   Configuration Preset Routine

During  initialization  of  the  multiplexor,  this

routine    is    called    to    find   and   copy   the

Configuration-dependent data structure from floppy  disk

to  core.  It  uses  standard RT-11 directives (.LOOKUP,

.READW  and  .CLOSE)  to  perform   this   action.   The

Configuration-dependent  area  consists  of  543.  words

beginning at label "PCB" in the data structure.

14.  BOMB:     Multiplexor Abort Exit Point

If   software   conditions   develop   which    the

multiplexor  cannot  handle,  this routine is entered to

indicate that the condition has occurred and to  provide

an  opportunity  to examine the software for information

about  the  condition.   This   routine   "undo's"   the

initialization  process, prints the abort message to the

Console, and enters DEC's standard SOFTWARE "ODT" module

to  provide  the  user  and  maintenance  personel  with

software tools to aid the examination.

15.  CONFDP: Special Message Dispatcher

This routine processes special message  conditions  such

as  Trunk timeout for both possible Trunks. The Trunk timeout

flag byte is examined to see if a message  is  needed.  Else,

the routine exits. If a message is needed, the routine copies

the appropriate Trunk's message into the LAST defined  buffer

in  the  Port  buffer  pool.  The  timeout  flag byte is then

incremented until it reaches a timed limit and is reset.  The

message copied is queued for the Console to indicate that the

named Trunk line is not supporting  communications.  At  some

later  point,  other  special  messages  could be added here.

16.  CONCMD: Console Command Decoding Routine

When a Console command is detected by the Port read ISR,

the mainline will route it to this routine for  decoding  and

dispatch.  The  input  buffer is checked against the existing

command dispatch table (CCMD)  and  the  appropriate  command

processing  routine is called. If the command is invalid, the

buffer will be echoed to the console with the console command

in  error replaced with an error indicator "!!!!" in place of

the command. Once the  processing  call  has  completed,  the

buffer will either be released or reused.

17.  HLTPRO: HALT Command Processing Routine

This  command  is  called  by  the CONCMD routine when a

'HALT' command is detected. It sets the HALT condition  flag,

HALTFL,  which  disables  input  from  the  Trunk  and  Ports

terminals (except for the Console).

18.  START: PROC Command Processing Routine

This routine is called by  the  CONCMD  routine  when  a

'PROC'  command  is  detected.  It  clears the HALT mode flag

(HALTFL), re-initializes the Read process for all Trunks  and

Ports   defined,   clears   the  Trunk  traffic-queued  table

(TWBPCB), and arbitrarily  sets  the  Trunk  Contention  mode

flag(s) to indicate a Contention condition.

19.  STDISP: STAT Command Display Header Routine

When  the  CONCMD routine detects a 'STAT' command, this

routine is called. It copies the display header into  a  Port

read  buffer  and  queues  it  for the Console. To preset the

remainder of the status display, a flag is set  (STDFLG)  and

the  PCB  counter  for the display line processor (DISTAT) is

set for PCB  0  (the  lowest-numbered  PCB  in  the  system).

Control is then returned to CONCMD.

20.  MSG: Console Message Dispatcher

When  the  CONCMD routine detects a 'MESS' command, this

routine is called. The PCB number (02-23)  is  validated  and

the  buffer  is  queued  for  that Port. If the PCB number is

invalid, it is replaced in the buffer by the  ASCII  constant

'??'  to signal an error. In this case, the buffer will begin

with 'MESS ??' and be echoed upon the Console.

21.  PROXIT: EXIT Command Processing Routine

Detection of an 'EXIT' command  by  the  CONCMD  routine

causes  this  routine to be called. If HALT mode is in effect

(HALTFL set), the routine  will  "un-do"  the  initialization

process,  print  an EXIT message upon the console, and return

Console and program control to RT-11. If the EXIT command  is

entered and the HALT mode flag is not set, it wil be IGNORED.

22.  DISTAT: STAT Display Line Processing Routine

When a 'STAT' command has been requested by the Console,

this routine is entered once for every COMPLETE scan  of  ALL

PCB's  by  the  mainline  logic.  If  the display flag is set

(STDFLG), the 'buffer-ready' flag (STRDY) is checked. If set,

the  logic  is awaiting an available Port read buffer. If the

flag is not set, the NEXT PCB to be  displayed  is  processed

into  an  intermediate buffer according to the format defined

by the header line and the  buffer-ready  flag  is  set.  The

CONCPY  routine  is  then  called  to copy the staging buffer

(CM16) into a Port read buffer. This process continues  until

ALL  PCB's  have  been  represented  in  a line of the status

display. Once this point has  been  reached,  the  Link  line

(representing  both  possible  Trunks)  is  formatted to show

protocol errors and conditions.  The  process  is  terminated

after  this  line  is queued for the Console by resetting the

display flag, STDFLG.

23.  CONCPY: Console Status Display Buffer Copy Routine

When a line of status display has been prepared  by  the

DISTAT routine, this routine is called to acquire a Port read

buffer, copy the formatted line into it from the staging area

(CM16),  and  queue  it for output upon the Console terminal.

24.  Canned Messages:

The system uses various "canned" messages, literal ASCII

strings used for  display  purposes,  etc.  The  strings  are

contained in the SF90 and SF12 source modules.

BMUX Port Interrupt Service Routines Documentation

25.  SF20 Module - Port Read Interrupt Service Processing  Routine

This routine has six entry points:

PR0: entry point for the Console;

PR1: entry point for PCB 3;

PR2: entry  point  for  PCB's  5,  9,  13,  17  and  21;

PR3:  entry  point  for  PCB's  6,  10,  14,  18 and 22;

PR4: entry point for PCB's 7, 11, 15,  19  and  23;  and

PR5:  entry  point  for  PCB's  4,  8,  12,  16  and 20.

The multiple entry points each establish  a  pointer  to

variable-length  table  containing  a LIST of pointers to the

various Ports' PCB and Device Address  entries.  Upon  entry,

the  ISR will scan ALL Ports defined in the list to see WHICH

Port caused the Interrupt to occur. This structured technique

permits the ISR to be data-driven and the actual logic exists

only ONCE but processes ALL Read  Interrupts  for  ANY  Port.

The   Read   ISR   routine   processes  input  character

interrupts from the terminal Ports, 2 through 23. It  handles

the following:

1.   Console command recognition for the Console,

2.   Blocks  input from ALL terminals EXCEPT the Console

during HALT mode,

3.   Backspace and line-deletion conditions,

4.   Blocks terminal input and sends a BELL character if

the   terminal's   Trunk   line  is  in  a  timeout

condition,

5.   Auto-echo   (if  defined)  of  a  terminal's  input

characters,

6.   End-of-line    conditions    by    formatting   the

appropriate header information in  the  buffer  for

transmission   of   the  buffer  to  the  Burroughs

processor, and 7.   Detects over-sized line

situations.

This routine computes a BCC value, the XOR-ed sum of ALL

input  characters  that  were  input  for transmission to the

Burroughs processor.  When  the  full  buffer  is  ready  for

transmission  to  the Trunk line, a completion status will be

set into the Read Status byte (P.RST) and a  Read  Completion

flag  (PS.RF)  set in the Port Status byte (P.PST) of the PCB

to trigger mainline completion handling.

26.  SF30 Module - Port Write Interrupt Service Processing Routine

This routine has six entry points:

PW0: entry point for the Console;

PW1: entry point for PCB 3;

PW2:  entry  point  for PCB's 5, 9, 13, 17 and

21;

PW3:  entry  point for PCB's 6, 10, 14, 18 and

22;

PW4:  entry  point for PCB's 7, 11, 15, 19 and

23; and

PW5:  entry  point  for PCB's 4, 8, 12, 16 and

20.

This    multiple-entry   ISR   routine   processes   the

identification of the Port which caused the Interrupt Entry in the

same  manner  as  the  SF20  Port  Read  ISR  logic.  Refer to the

description in that module's discussion for more details.

The ISR handles the Write buffer character processing to

complete the Write function for terminal Ports 2 through  23.

The following conditions are handled by this routine:

1.   Output character buffer processing;

2.   Carriage  return (CR) and form feed (FF) delays (if

defined) by sending the specified number  of  ASCII

NULL  characters  after the transmission of a CR or

FF; and

3.   Write completion handling.

When the  output  sequence  is  complete,  a  completion

status  is  placed  in  the Write Status byte (P.WST) and the

Write Completion flag is set in the Port Status byte  (P.PST)

of the PCB.

27.  SF40 Module - Trunk Read Interrupt Service Processing Routine

This routine has two entry points:

TR0: entry point for Trunk 0, and

TR1: entry point for Trunk 1.

The ISR entry points for this routine specify the  table

address  for  the  appropriate  Trunk  PCB and Device Address

entries. In this manner, the logic appears only ONCE but will

process Read Interrupts for either of the two Trunk lines (if

both are defined).

It is responsible for the following actions:

1.   Protocol interpretation and response for the  Trunk

lines,

2.   Traffic receipt from the Burroughs destined for the

Ports,

3.   Protocol verification,

4.   Poll address verification,

5.   Oversized line detection, and

6.   Burroughs-bound  traffic  control  by  coordination

with the Trunk Write ISR and  the  mainline  logic.

This  routine  is  the  sole  validator  of the protocol

messages  from  the  Burroughs.  Invalid  lengths,  bad  poll

addresses, etc will result in error conditions being noted by

this routine. If traffic is sent from  or  requested  by  the

Burroughs  processor,  this routine verifies the poll address

and request by the processor and attempts to fill  the  same.

Refer  to  information on the protocol for the Burroughs link

for specific details. The Burroughs protocol buffers accepted

by this routine are:

1.   Message buffers,

2.   POL requests for Burroughs-bound traffic,

3.   SEL requests for permission to send a buffer  to  a

terminal, and

4.   CON buffers indicating the Burroughs  is  declaring

the  line  to be in Contention mode (that is, "wake

me up if you have any traffic").

This  routine  also  handles completion of Trunk Writes,

since the buffers sent to the Byrroughs MUST be  acknowledged

by   the  processor  before  a  Trunk  Write  action  can  be

considered completed. Read Completions for the Trunk(s) occur

only  after a buffer has been received for one of the defined

terminal   Ports.   ALL   reads   on   the    Trunk(s)    are

"double-queued",  i.e. a second buffer MUST be waiting before

the receipt of a buffer destined for a Port  is  acknowledged

by  the  multiplexor. If the multiplexor is in HALT mode or a

second read buffer does NOT exist (due to Trunk  read  buffer

pool  usage),  SELect  requests  will NOT be acknowledged. As

part of the Trunk timeout processing, the receipt of a POL or

SEL request causes the Contention Break timer condition to be

canceled.

28.  SF50  Module  -  Trunk  Write  Interrupt  Service  Processing

Routine

This routine has two entry points:

TW0: entry point for Trunk 0, and

TW1: entry point for Trunk 1.

The ISR entry points specify  the  appropriate  PCB  and

Device Address entries for the Trunk to be processed. In this

fashion, the logic  to  process  either  Trunk  line's  Write

Interrupts occurs only ONCE.

This routine merely outputs ALL characters in the  Trunk

write  buffer and turns off the Interrupt Enable bit when the

last  character  has  been  transmitted  to   the   Burroughs

processor. It also clears the Trunk Write Buffer Address word

(P.WBFA)  in  the  PCB  to  allow   further   Trunk   traffic

processing.

BMUX Configuration Module Documentation

29.  SF60 Module - Configuration Generator

This routine uses standard RT-11 directives for terminal

and disk I/O to prompt the  user  for  a  definition  of  the

Trunk(s)  and  Port(s) he wishes to use. After the definition

has been completed, it is stored on a  mini-floppy  disk  for

subsequent  retrieval  by  the  multiplexor  PCBSET  routine.

The minimum configuration consists of a Trunk  (PCB's  0

or  1) and the Console Port (PCB 2). ALL others are optional.

The data structure module, SF2, is commonly used by both  the

Configuration  Generator  and  the  Multiplexor.  The user is

prompted, starting at PCB 0 and proceeding to PCB 23, for the

needed information. Refer to the user documentation for order

and limits upon the  various  prompts  by  the  Configuration

Generator.  If  a port is NOT defined, its Source (P.SOU) and

Destination (P.DST) bytes contain a -1 value. The  Statistics

are  preset  for  each  Trunk or Port defined to collect BOTH

Error and Traffic counts. All numeric inputs are  assumed  to

be DECIMAL values. All inputs are validated for size, but the

SITE and OWNER fields are  validated  for  length  ONLY,  and

stored AS ENTERED.

Entry of two Control-V's causes the PCB  to  be  skipped

UNLESS  it is PCB 2. If a Trunk is NOT defined, the Generator

will exit after the definition  of  the  first  Port  PCB.  A

Control-V  and  an ASCII 'E' causes the remaining PCB's to be

skipped and not defined.

BMUX Data Structure Documentation

30.  SF2 Module - Data Structure

The  following  data  elements  comprise  the functional

control set for both the  Configuration  Genertator  and  the

Multiplexor.  The  following table provides the item-specific

information:

Label:         Description:

CR1-CR5        These are canned responses used to handle

the Burroughs protocol

CR6            This buffer is sent  to  the  appropriate

Trunk  to  break the Contention condition

on that Trunk. The proper POLL ADDRESS is

first  inserted  after the EOT character.

CRTAB          This is a table of  the  canned  response

addresses.

TRKSRP         Each byte (0/1) is used by  the  mainline

to  scan  for traffic to be placed in the

Burroughs-bound table for the PCB  number

held in that Trunk's TRKRSP byte. This is

used only by the mainline.

CONFLG         Non-zero  values  in the respective bytes

(0/1)  indicate  that  the  corresponding

Trunk  line  is  in Contention mode. This

acts as  a  communications  mechanism  to

relay the receipt of a Contention message

by the Trunk Read ISR  to  the  mainline.

TRAPA          The  Trunk  Read  ISR uses the respective

bytes (0/1) to signal to the mainline the

PCB  from  which it wants traffic sent to

the Burroughs via the Trunk Write process

(in response to a POLL).

TRDQ2          The mainline stores the  address  of  the

second   buffer  of  the  "double-queued"

buffer pair in the respective words (0/1)

for  each  Trunk to signal the Trunk Read

ISR that it can accept buffers  from  the

Burroughs.  This  "double-queued"  buffer

process   prevents   "windows"   in   the

monitoring  of  the  Trunk communications

from the Burroughs processor.

TSP-TEP        These  two pointers hold the starting and

ending addresses  for  the  active  Trunk

buffer  pool  buffers. The buffer request

and release routines maintain the  values

of the pointers.

PSP-PEP        Used as TSP and TEP,  but  for  the  Port

buffer pool.

SPTR-PEN       These  scratch  areas  are  used  by  the

buffer  request  and  release routines to

examine the buffer pools.

PRPTR-TWPTR    These pointers are used to hold the entry

point table addresses for the  Trunk  and

Port Read/Write ISR's.

HALTFL         This byte, if  non-zero,  indicates  that

HALT mode is in effect.

INIFLG         This byte,  if  non-zero,  indicates  the

multiplexor is NOT yet initialized.

STDFLG         If  this  byte  is  non-zero,  the   STAT

command  is  processing  lines of display

information.

SPFUN          This  table is used to define the list of

special  subroutines  the  mainline  must

process between each complete scan of ALL

the defined PCB entries.

CCMD           The  Console  command  decoder  uses this

table to identify a valid Console command

and  execute  the proper routine for that

command.

ISRTAB         This  table is used during Initialization

to establish the Interrupt  Vector  entry

addresses  for  Interrupt processing. The

Vector  address,  Read  ISR  address  and

Write  ISR  address comprise a unit entry

in this table.

DA1-DA8        These  variable-length  tables consist of

PCB number/Device Read CSR address  pairs

for   the   Trunks/Ports   utilizing  the

specified Interrupt Vector  as  entry  to

the Read or Write ISR's. Refer to the ISR

module    documentation    for    further

information.

ISRSAV         This area is used by  the  Initialization

process  to save both the prior Interrupt

Vector  contents  and  the  prior  Device

Address  contents  before the multiplexor

"takes over" for its operations. In ABORT

or EXIT conditions, this table is used to

reset the Interrupt  Vectors  and  Device

Address  Registers  before giving control

back to RT-11.

CONSAV         The   Console   Device  Address  Register

contents are also saved in this area  for

abort reset.

RCSRA-XBUFA    This  set  of  Device  Address   Register

pointers  is  used SOLELY by the mainline

to access a given device  for  Read/Write

initiation.

RCSRI-XBUFI    This  set  of  Device  Address   Register

pointers  are used soley by the currently

active  ISR  routine   to   process   the

interrupt.

WAIT1-NEWSEC   RT-11 directive scratch areas.

PAT            Software  maintenance  scratch  area  for

absolute patches.

CUMTAB         Statistics  cumulative  accumulator table

(2 words/PCB).

PCB            Port  Control  Block  table.  This  table

defines  the  active   PCB's   that   the

multiplexor   must   process.  Being  the

Master   control   structure   for    the

multiplexor's  operation,  it is much too

difficult  to   describe   ALL   of   the

interactions   in  this  segment  of  the

documentation to any detail. In  general,

it  controls  traffic  flow, buffer flow,

ISR processing, traffic/error  statistics

collection,  Trunk timeout detection, and

status coordination between the ISR's and

the  mainline. Please refer to the System

Equates documentation for PCB

definitions.

POLTAB         This table contains the user-defined POLL

ADDRESS  for  each  defined  PCB, 2 ASCII

bytes in PCB  order.  Unused  areas  will

contain  a  -1  value,  and ASCII control

characters  are  not  permitted  by   the

Configuration  generator.  This  table is

used by the Trunk Read ISR to validate  a

POLL  ADDRESS  received  in  a POL or SEL

buffer from the Burroughs processor.  The

Port  Read  ISR uses this table to define

the POLL  ADDRESS  value  in  the  header

segment  of  the buffer to be sent to the

Burroughs CPU.

TTYPE          This  table  contains  the  6-byte  OWNER

value  entered  by  the  user   via   the

Configuration Generator. Undefined values

will contain a "******" value.

BRTAB          The  SITE  values entered by the user via

the Configuration  Generator  are  stored

here,  5 bytes per PCB in PCB order. If a

PCB is not defined, a "*****" value  will

appear.

PCBDAT         This table provides a cross-reference  of

PCB   numbers   and  their  corresponding

Device Address Register addreses.

TWBPCB         This table holds buffer addresses, in PCB

order, which are to be transmitted to the

Burroughs for processing. If no buffer is

queued   from   any   given   PCB,    the

corresponding  word will be a zero value.

RVITAB-JORST   Scratch areas for future expansion.

####################################################################################################################################################################################################################################################################################################################################################################################################################################