Pocket handbook

to

BONNER LAB RUNOFF

Runoff pocket guide - 22 Jan 86                                    - 2 -

RT-11 Running

RUNOFF can be started by typing

RUN DEV:RUNOFF.SAV

or

@COMMND.COM

where  DEV:   is  the  device on which the RUNOFF.SAV file is found, and

COMMND.COM is a command file that initiates RUNOFF and  might  or  might

not contain file specification information.

The  program  will  prompt you for a command string.  The command string

specifies the names of the input and output files and any  options  that

you  might select at run time.  The standard command string (for further

information look at the .CSIGEN  macro  in  the  Programmer's  Reference

Manual) should be entered in the following form:

output-file,toc-file=input-file,input-file,,,,input-file/options

where  output-file is the document to be produced by RUNOFF, toc-file is

a table of contents file that can be edited before it is reprocessed  by

RUNOFF to produce a table of contents, and input-file is a file contain-

ing text and RUNOFF commands to control the format  of  the  output-file

and the toc-file.

A valid file specification has the form:

DEV:FILNAM.EXT

Runoff pocket guide - 22 Jan 86                                    - 3 -

RT-11 Running

The  command  string can have one or more switch options associated with

it.  The valid switch options are:

/P:l:h    Output only the indicated page range.

/C:l:h    Output only the indicated chapter range.

/A:l:h    Output only the indicated appendix range.

/G        Force all text to be output in uppercase.

/G:N      Allow all text to be output as it is input.

/F        Use the standard ascii formfeed character.

/F:n      Simulate the formfeed character with multiple linefeeds.

/H        Hyphenate output.

/H:N      Do not hyphenate output.

/R:n      Right shift output n spaces.

/S:h:w    Set hardware page size.

/U:B      Underline with underline character and backspace.

/U:L      Underline with underline character and line overprint.

/U:S      Underline with hyphen on next line.

/U:N      Do not underline text.

/W        Wait for carriage return between pages.

/W:N      Do not wait for carriage return.

/I        Print available switch options on terminal.

/E        Output even pages.

/E:N      Do not output even pages.

/O        Output odd pages.

/O:N      Do not output odd pages.

/X        Exit program and return to operating system.

/2        Perform 2 passes.

/2:N      Perform 1 pass.

/M        Output warning messages.

/M:N      Suppress warning messages.

Default values are: /G:N/F/H/R:0./S:58.:60.

/U:L/W:N/E/O/M/2:N (print all)

Runoff pocket guide - 22 Jan 86                                    - 4 -

Special Characters

Flag characters

Below  is a list of RUNOFF's flag characters.  To cause a flag to appear

in the text as a normal character, it must be preceded by the underscore

character   "_"   (itself  a  special  character).   Since  the  special

characters can be changed by the .FLAGS  commands,  this  list  may  not

represent the actual usage for your document.

^ UPPERCASE the next character, or LOCK a function.

^^ No case conversion.

\ LOWERCASE the next character, or UNLOCK a function.

\\ Lowercase all text.

< CAPITALIZE word. *

^< capitalize all text. *

# SPACE character (non expandable or quoted space).

= HYPHENATE specification, or continue mark. *

& UNDERLINE the following character (also "^&,/&").

_ ACCEPT the following character.

% OVERSTRIKE the previous character by the next character. *

$ SUBSTITUTE defined text for abbreviation. *

> INDEX/SUBINDEX flags. *

| BREAK text with no spaces, or continue. *

+ PERIOD will add extra space if followed by space. *

{ EQUATION flag  1. * (Not in DSR)

} EQUATION flag  2. * (Not in DSR)

"tab" TAB flag character causes tabulation. (Not a flag in DSR)

-------- footnote --------

* Only becomes a special character if the FLAG is engaged.

Runoff pocket guide - 22 Jan 86                                    - 5 -

Special Characters

Terminal punctuation

.  :  ;  ?  !

These characters, if followed by a separator (space or tab), will appear

in the output with 1 extra space after them.  This  feature  works  only

when  both  .FILL  and  .PERIOD  are  engaged.   These characters can be

changed with the .PERIOD command.

Valid punctuation

/ \ . , " ( ) @

These   characters   are   valid   in  a  hyphenated  word.   Any  other

non-alphabetic character will prevent autohyphenation.  These characters

can be changed by the .AUTOHYPHENATE command.

Runoff pocket guide - 22 Jan 86                                    - 6 -

Runoff - List of commands

* Indicates the command causes a line BREAK.

$ Indicates a command forbidden during a FOOTNOTE.

D Indicates the default setting

~ Indicates not available in DSR

Command                                    Abbreviation    Opt.      _______                                    ____________    ____

APPENDIX [;][title]  -  -  -  -  -  -  -   AX   -  -  -  - *$

Start an appendix.

AUTOBREAK ["chars"]  -  -  -  -  -  -  -   AB   -  -  -  -  $ ~

Enable automatic breaking of lines.

AUTOHYPHENATE [n1,...n4][,"chars"]  -  -   AH   -  -  -  -  $D~

(n1=size, n2=begin, n3=end, n4=mode)

Enable automatic hyphenation until a NO AUTOHYPHENATE.

AUTOPARAGRAPH  -  -  -  -  -  -  -  -  -   AP   -  -  -  -  $

Begin paragraphs for lines with leading spaces,

tabs, or blank lines until a NO AUTOPARAGRAPH.

AUTOSUBTITLE   -  -  -  -  -  -  -  -  -   AST  -  -  -  -  $

Make subtitle the same as current section (header level)

until a NO AUTOSUBTITLE.

AUTOTABLE   -  -  -  -  -  -  -  -  -  -   AT

Enable automatic table format until NO AUTOTABLE.

AUTOTITLE   -  -  -  -  -  -  -  -  -  -   ATI  -  -  -  -  $D~

Title to be the same as current chapter/appendix.

BEGIN BAR   -  -  -  -  -  -  -  -  -  -   BB   -  -  -  -

Left hand margin will have a bar in it until .END BAR.

BLANK [-space] -  -  -  -  -  -  -  -  -   B -  -  -  -  - *

Skip lines independently of spacing, except at top of page.

BREAK -  -  -  -  -  -  -  -  -  -  -  -   BR   -  -  -  - *

Start a new line.

BREAK LINE  -  -  -  -  -  -  -  -  -  -   BRL  -  -  -  - *

Break at end of next line.

CENTER [LINE][+-n]   -  -  -  -  -  -  -   C [LINE]   -  - *

Center 1 line.

CENTER TEXT [+-n] -  -  -  -  -  -  -  -   C TEXT  -  -  - *  ~

Center a section until END CENTER.

CENTRE [+-n]   -  -  -  -  -  -  -  -  -   C -  -  -  -  - *

CHAPTER [;][title]   -  -  -  -  -  -  -   CH   -  -  -  - *$

Start a chapter.

CHAPTER LAYOUT [code][,spacing]  -  -  -   CHLO -  -  -  -  $ ~

Change the layout of first page of chapter.

COMMENT [comment text]  -  -  -  -  -  -   ; -  -  -  -  -

-  -   !

A comment that is not included in the output.

DEFINE COMMAND /command/text  -  -  -  -  -  -  -  -  -  -  $ ~

Define a new command.

DEFINE ESCAPE escape definition  -  -  -  -  -  -  -  -  -  $ ~

Define special characters to output control codes.

DEFINE ITEM /label/ [format],[+-n]  -  -  -  -  -  -  -  -    ~

The label is a number to appear in text.

Runoff pocket guide - 22 Jan 86                                    - 7 -

Runoff - List of commands

DEFINE NUMBER - Define a substitution label for a current "number".

DEFINE NUMBER APPENDIX /label/   -  -  -  -  -  -  -  -  -    ~

DEFINE NUMBER CHAPTER /label/ -  -  -  -  -  -  -  -  -  -    ~

DEFINE NUMBER ITEM /item/ /label/   -  -  -  -  -  -  -  -    ~

DEFINE NUMBER LEVEL /label/   -  -  -  -  -  -  -  -  -  -    ~

DEFINE NUMBER LIST /label/ -  -  -  -  -  -  -  -  -  -  -    ~

DEFINE NUMBER PAGE /label/ -  -  -  -  -  -  -  -  -  -  -    ~

DEFINE SUBSTITUTE /label/text -  -  -  -  -  -  -  -  -  -  $ ~

Define text to be substituted for label.

DEFINE SUBSCRIPT escape def   -  -  -  -  -  -  -  -  -  -    ~

Define subscript for equation formatting.

DEFINE SUPERSCRIPT escape def -  -  -  -  -  -  -  -  -  -    ~

Define superscript for equation formatting.

DEFINE VARIABLE SPACE escape def -  -  -  -  -  -  -  -  -    ~

Define how to do variable spacing.

DELETE INDEX "entry" -  -  -  -  -  -  -  -  -  -  -  -  -    ~

Remove an entry from index table.

DELETE SUBSTITUTE /label/  -  -  -  -  -  -  -  -  -  -  -    ~

Remove a substitution label from table of substitutions.

DISABLE - Turns off features turned on by an ENABLE.

DISABLE BAR -  -  -  -  -  -  -  -  -  -   DBB  -  -  -  -   D

DISABLE ESCAPE -  -  -  -  -  -  -  -  -   DES  -  -  -  -    ~

DISABLE COMMAND REPLACEMENT   -  -  -  -   $D~

DISABLE EQUATION  -  -  -  -  -  -  -  -   DEQ  -  -  -  -    ~

DISABLE FLAGS [flag] -  -  -  -  -  -  -   DFL  -  -  -  -   D~

DISABLE HYPHENATION  -  -  -  -  -  -  -   DHY  -  -  -  -

DISABLE INDEXING  -  -  -  -  -  -  -  -   DIX  -  -  -  -

DISABLE NUMBERING -  -  -  -  -  -  -  -   DNMPG   -  -  -  $ ~

DISABLE NUMBERING CHAPTER  -  -  -  -  -   DNMCH   -  -  -  $ ~

DISABLE ODD -  -  -  -  -  -  -  -  -  -   DOD  -  -  -  -   D~

DISABLE SUBSTITUTION -  -  -  -  -  -  -   DSST -  -  -  -

DISABLE PAGING -  -  -  -  -  -  -  -  -   DPAG -  -  -  -    ~

DISABLE TABS   -  -  -  -  -  -  -  -  -   DTB  -  -  -  -    ~

DISABLE TOC -  -  -  -  -  -  -  -  -  -   DTC  -  -  -  -

DISBLE TRAILING ZERO -  -  -  -  -  -  -    $ ~

DISABLE OVERSTRIKING -  -  -  -  -  -  -   DOV  -  -  -  -

DISABLE UNCONDITIONAL   -  -  -  -  -  -   DUNC -  -  -  - *$ ~

DISABLE UNDERLINING  -  -  -  -  -  -  -   DUL  -  -  -  -

DISPLAY APPENDIX ["pre",][form][,"post"]   DAX  -  -  -  - *$

DISPLAY - Change format of specified number.

(form = D,LU,LL,LM,RU,RL,RM)

DISPLAY CHAPTER ["pre",][form][,"post"]    DCH  -  -  -  - *$

DISPLAY ELEMENTS ["pre",][form][,"post"]   DLE  -  -  -  - *$

DISPLAY LEVELS [f1],...[f6]   -  -  -  -   DHL  -  -  -  - *$

DISPLAY LEVELS [level,]["pre",][format,]["post"]   -  -  - *  ~

DISPLAY NUMBER ["pre",][form][,"post"] -   DNM  -  -  -  - *$

DISPLAY SUBPAGE [form]  -  -  -  -  -  -   DSP  -  -  -  - *$

DISPLAY SUBTITLE ["pre"],["post"]   -  -  -  -  -  -  -  -    ~

DISPLAY TITLE ["pre"],["post"]   -  -  -  -  -  -  -  -  -    ~

DO INDEX [title]  -  -  -  -  -  -  -  -   DX   -  -  -  - *$

Print the index as an appendix.

Runoff pocket guide - 22 Jan 86                                    - 8 -

Runoff - List of commands

ELLIPSES -  -  -  -  -  -  -  -  -  -  -   ELL  -  -  -  -  $ ~

Unspecified tabs will be converted to ellipses.

ELSE aaaa

Follows an "IF aaaa" command.

ENABLE BAR n1,n2  -  -  -  -  -  -  -  -   EBB  -  -  -  -   D

Allow change bars in left margin.  n1,n2 are spaces before

and after bar.

ENABLE COMMAND REPLACEMENT -  -  -  -  -   $ ~

Allow user commands to replace permanent commands.

ENABLE ESCAPE  -  -  -  -  -  -  -  -  -   ESC  -  -  -  -   D~

Allow control code output.

ENABLE FLAGS [flag]  -  -  -  -  -  -  -   EFL  -  -  -  -   D~

Allow specified flag to be used.

ENABLE EQUATION n -  -  -  -  -  -  -  -   EEQ  -  -  -  -   D~

Allow equation formatting.

ENABLE HYPHENATION   -  -  -  -  -  -  -   EHY  -  -  -  -   D

Allow hyphenation.

ENABLE INDEXING   -  -  -  -  -  -  -  -   EIX  -  -  -  -   D

Entries will be made in index table.

ENABLE LEVELS [+-out],[+-toc] -  -  -  -  -  -  -  -  -  -  $ ~

Enable header levels for output and table of contents.

ENABLE NUMBERING  -  -  -  -  -  -  -  -   ENMPG   -  -  -  $ ~

Page numbers will be produced.

ENABLE NUMBERING CHAPTER   -  -  -  -  -   ENMCH   -  -  -  $ ~

Page numbers include chapter number (e.g., 1-2).

ENABLE ODD  -  -  -  -  -  -  -  -  -  -   EOD  -  -  -  -    ~

Chapters start only on odd page numbers.

ENABLE PAGING  -  -  -  -  -  -  -  -  -   EPAG -  -  -  -   D~

Pages will be formatted.

ENABLE TABS -  -  -  -  -  -  -  -  -  -   ETB  -  -  -  -   D~

Tabs will be converted to multiple spaces.

ENABLE TOC  -  -  -  -  -  -  -  -  -  -   ETC  -  -  -  -   D

Entries will be made in table of contents file.

ENABLE TRAILING ZERO -  -  -  -  -  -  -    $D~

Enable trailing zero for header level 1 (1.0...).

ENABLE OVERSTRIKING  -  -  -  -  -  -  -   EOV  -  -  -  -   D

The overstrike flag will be obeyed.

ENABLE SUBSTITUTION  -  -  -  -  -  -  -   ESST -  -  -  -   D

Text will be substituted for a defined label.

ENABLE UNCONDITIONAL -  -  -  -  -  -  -   ENUNC   -  -  - *$D~

Unconditional commands will be obeyed and text output.

ENABLE UNDERLINING   -  -  -  -  -  -  -   EUL  -  -  -  -   D

Text can be underlined.

END - Ends a feature.

END BAR  -  -  -  -  -  -  -  -  -  -  -   EB   -  -  -  -

END CENTER  -  -  -  -  -  -  -  -  -  -   ECN  -  -  -  -    ~

END FOOTNOTE   -  -  -  -  -  -  -  -  -   EFN  -  -  -  -

ENDIF aaaa

END LIST [space]  -  -  -  -  -  -  -  -   ELS  -  -  -  - *$

END LITERAL -  -  -  -  -  -  -  -  -  -   EL   -  -  -  - *$

END NOTE -  -  -  -  -  -  -  -  -  -  -   EN   -  -  -  - *$

END RIGHT   -  -  -  -  -  -  -  -  -  -   ER   -  -  -  - *  ~

END SEPARATED EQUATION  -  -  -  -  -  -   ESEQ -  -  -  -   D~

END SUBPAGE -  -  -  -  -  -  -  -  -  -   ES   -  -  -  - *$

Runoff pocket guide - 22 Jan 86                                    - 9 -

Runoff - List of commands

END TEXT -  -  -  -  -  -  -  -  -  -  -   ETX  -  -  -  - *$ ~

ENTRY index term  -  -  -  -  -  -  -  -   Y -  -  -  -  -

Defines index term without page number.

FIGURE [space] -  -  -  -  -  -  -  -  -   FG   -  -  -  - *$

Skip lines independent of spacing.  Works at top of page.

FIGURE DEFERRED [space] -  -  -  -  -  -   FGD  -  -  -  - *$

Skip lines independent of spacing, where space permits.

FILL     -  -  -  -  -  -  -  -  -  -  -   F -  -  -  -  -   D

Start fill mode until a NO FILL (normal mode of usage).

FIRST TITLE -  -  -  -  -  -  -  -  -  -   FT   -  -  -  -  $

Define title to appear on all pages including first.

FLAGS - Turns on and defines a special character to do something.

FLAGS ACCEPT [new flag] -  -  -  -  -  -   FL ACCEPT  -  -  $D

FLAGS [ALL] -  -  -  -  -  -  -  -  -  -   FL [ALL]   -  -  $

FLAGS BREAK [new flag]  -  -  -  -  -  -   FL BREAK   -  -  $

FLAGS CAPITALIZE [new flag]   -  -  -  -   FL CAPITALIZE -  $

FLAGS CONTROL [new flag]   -  -  -  -  -   FL CONTROL -  -  $D

FLAGS ESCAPE [new flag] -  -  -  -  -  -   FL ESCAPE  -  -  $ ~

FLAGS EQUATION -  -  -  -  -  -  -  -  -   FL EQUATION   -    ~

FLAGS HYPHENATE [new flag] -  -  -  -  -   FL HYPHENATE  -  $

FLAGS INDEX [new flag]  -  -  -  -  -  -   FL INDEX   -  -  $

FLAGS PERIOD [new flag] -  -  -  -  -  -   FL PERIOD  -  -  $

FLAGS LOWERCASE [new flag] -  -  -  -  -   FL LOWERCASE  -  $D

FLAGS OVERSTRIKE [new flag]   -  -  -  -   FL OVERSTRIKE -  $

FLAGS SUBINDEX [new flag]  -  -  -  -  -   FL SUBINDEX   -  $

FLAGS SUBSTITUTE [new flag]   -  -  -  -   FL SUBSTITUTE -  $

FLAGS TAB [new flag] -  -  -  -  -  -  -   FL TAB  -  -  -  $ ~

FLAGS UNDERLINE [new flag] -  -  -  -  -   FL UNDERLINE  -  $D

FLAGS UPPERCASE [new flag] -  -  -  -  -   FL UPPERCASE  -  $D

FLUSH -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  - *  ~

Flush out all deferred text.

FOOTNOTE -  -  -  -  -  -  -  -  -  -  -   FN   -  -  -  -  $

Start a footnote that ends with END FOOTNOTE.

HALF SPACING [spacing]  -  -  -  -  -  -  -  -  -  -  -  -    ~

Enable and define half spacing.

HEADERS [on]   -  -  -  -  -  -  -  -  -   HD [on] -  -  -  $D

Enable output of headers (title/subtitle) at top of page.

HEADER LEVEL [+-level][;][title] -  -  -   HL   -  -  -  - *$

Start a numbered section in document.

HEADERS LOWER  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  $

HEADERS MIXED  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  $D

HEADERS NO PAGE   -  -  -  -  -  -  -  -  -  -  -  -  -  -  $ ~

HEADERS PAGE   -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  $D~

HEADERS SPACING [spacing]  -  -  -  -  -   HDSP -  -  -  -    ~

Set spacing between title and first line of text.

HEADERS UPPER  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  $

IF aaaa [bbbb]...

If aaaa is "true", text following is output until an

ELSE or ENDIF.

Runoff pocket guide - 22 Jan 86                                   - 10 -

Runoff - List of commands

IF NOT aaaa [bbbb]...

If aaaa is "false", text following is not output until an

ELSE or ENDIF.

IMMEDIATE IF aaaa -  -  -  -  -  -  -  -   IIF  -  -  -  -    ~

Rest of line ignored if aaaa is false.

IMMEDIATE IF NOT aaaa   -  -  -  -  -  -   IIFNOT  -  -  -    ~

Rest of line ignored if aaaa is true.

IMMEDIATE TEST PAGE [n] -  -  -  -  -  -   ITP  -  -  -  -  $ ~

Start new page if fewer than n line remain on page.

IMMEDIATE TEST TEXT [n] -  -  -  -  -  -   ITT  -  -  -  -  $ ~

Start new page if fewer than n lines of text can be put on page.

INDENT [-spaces]  -  -  -  -  -  -  -  -   I -  -  -  -  - *

Indent line from left margin.

INDENT LEVELS [-l],[-r] -  -  -  -  -  -  -  -  -  -  -  -  $ ~

Specify left, right indentation for header levels (sections).

INDEX entry -  -  -  -  -  -  -  -  -  -   X -  -  -  -  -

Specify term to be indexed with page number.

JUSTIFY  -  -  -  -  -  -  -  -  -  -  -   J -  -  -  -  -   D

Output text will be left and right justified.

KEEP [LINES]   -  -  -  -  -  -  -  -  -   K -  -  -  -  -

Blank lines in input will also appear in output in NO FILL mode.

KEEP TABS   -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -    ~

Tabs at beginning of lines are kept and not removed in FILL mode.

LAYOUT [code] [,spacing]   -  -  -  -  -   LO   -  -  -  - *$

Change overall page layout of title/subtite and page number.

(0=all at top 1-15=Header top,number bottom)

( 1=C,C    2=LR,C  3=L,C   4=L,R  5=C,R   6=C,LR)

( 7=LR,LR  8=L,LR  9=LR,R 10=C,L 11=LR,L 12=L,L)

(13=R,L   14=R,C  15=R,LR 16=R,R)

LEFT MARGIN [+-n] -  -  -  -  -  -  -  -   LM   -  -  -  - *

Set the left margin.

LIST [lines] ["char"]   -  -  -  -  -  -   LS   -  -  -  - *$

Begin a numbered list of items separated by blank lines until END

LIST.

LIST ELEMENT [;]text -  -  -  -  -  -  -   LE   -  -  -  - *$

Print current number in list.

LITERAL  -  -  -  -  -  -  -  -  -  -  -   LT   -  -  -  - *

Text following appears in output exactly as typed until END

LITERAL.

LOCK  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  $ ~

Lock some parameters so they can not be changed.

LOWER CASE  -  -  -  -  -  -  -  -  -  -   LC   -  -  -  -

All text will be in same case as input.

NO - turns off selected features.

NO AUTOBREAK ["chars"]  -  -  -  -  -  -   NAB  -  -  -  -  $D~

NO AUTOHYPHENATE "chars"   -  -  -  -  -   NAH  -  -  -  -  $

NO AUTOPARAGRAPH  -  -  -  -  -  -  -  -   NAP  -  -  -  -  $D

NO AUTOSUBTITLE   -  -  -  -  -  -  -  -   NAST -  -  -  -  $D

NO AUTOTABLE   -  -  -  -  -  -  -  -  -   NAT

NO AUTOTITLE   -  -  -  -  -  -  -  -  -   NATI -  -  -  -  $ ~

NO ELLIPSES -  -  -  -  -  -  -  -  -  -   NELL -  -  -  -  $D~

Runoff pocket guide - 22 Jan 86                                   - 11 -

Runoff - List of commands

NO FILL  -  -  -  -  -  -  -  -  -  -  -   NF   -  -  -  - *

NO FLAGS [flag name] -  -  -  -  -  -  -   NFL  -  -  -  -  $

NO HALF SPACING   -  -  -  -  -  -  -  -  -  -  -  -  -  -   D~

NO HEADERS [on]   -  -  -  -  -  -  -  -   NHD

NO JUSTIFY  -  -  -  -  -  -  -  -  -  -   NJ   -  -  -  -

NO KEEP  -  -  -  -  -  -  -  -  -  -  -   NK   -  -  -  -   D

NO NUMBER   -  -  -  -  -  -  -  -  -  -   NNM  -  -  -  -  $

NO NUMBER CHAPTER -  -  -  -  -  -  -  -  -  -  -  -  -  -  $ ~

NO PAGING   -  -  -  -  -  -  -  -  -  -   NPA  -  -  -  -  $

NO PERIOD ["chars"]  -  -  -  -  -  -  -   NPR

NO SPACE -  -  -  -  -  -  -  -  -  -  -   NSP  -  -  -  -

NO TITLE -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  $ ~

NO UNDERLINE "characters"  -  -  -  -  -   NUN  -  -  -  -    ~

NO VARIANT aaaa [bbbb]...  -  -  -  -  -  -  -  -  -  -  -    ~

NOTE [;title]  -  -  -  -  -  -  -  -  -   N -  -  -  -  - *$

Start an indented note until END NOTE.

NUMBER - Sets the new number.

NUMBER APPENDIX [+-n]   -  -  -  -  -  -   NMAX -  -  -  -  $

NUMBER CHAPTER [+-n] -  -  -  -  -  -  -   NMCH -  -  -  -  $D

NUMBER ITEM [+-n] -  -  -  -  -  -  -  -   NMIT -  -  -  -    ~

NUMBER LEVEL [+-n1],....,[+-n6]  -  -  -   NMLV -  -  -  -  $

NUMBER LIST [+-n] -  -  -  -  -  -  -  -   NPLS -  -  -  -  $

NUMBER [PAGE] [+-n]  -  -  -  -  -  -  -   NMPG -  -  -  -  $D

NUMBER SUBPAGE [+-n] -  -  -  -  -  -  -   NMSPG   -  -  -  $

PAGE  -  -  -  -  -  -  -  -  -  -  -  -   PG   -  -  -  - *$

Start a new page if paging enabled.

PAGE EVEN   -  -  -  -  -  -  -  -  -  -   PGE  -  -  -  - *$ ~

Start a new even numbered page.

PAGE ODD -  -  -  -  -  -  -  -  -  -  -   PGO  -  -  -  - *$ ~

Start a new odd numbered page.

PAGE SIZE [+-h],[+-w],[+-l],[+-t],[s]  -   PS   -  -  -  -  $

Set page size, permanent margins, sets PAGING.

PAGING   -  -  -  -  -  -  -  -  -  -  -   PA   -  -  -  - * D

Text will be divided into pages.

PAPER SIZE  -  -  -  -  -  -  -  -  -  -   PS   -  -  -  -  $

Synonym for page size.

PARAGRAPH [-n],[v],[t],[b] -  -  -  -  -   P -  -  -  -  - *

Start a new paragraph until a command causing break.

(n=indent, v=skip, t=test, b=bottom test)

PERIOD "characters"  -  -  -  -  -  -  -   PR   -  -  -  -  $D

Enable specified characters to end sentence (produce 2 spaces).

PERMANENT FOOTNOTE   -  -  -  -  -  -  -   PFN  -  -  -  -  $ ~

Start a section of text to begin each footnote until END FOOTNOTE.

PRINT INDEX "entry"  -  -  -  -  -  -  -   PX   -  -  -  - *$

Print the index and clear it.

RESET - Remove all entries in a table.

RESET ESCAPE   -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  $ ~

RESET INDEX -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -    ~

RESET SUBSTITUTE  -  -  -  -  -  -  -  -  -  -  -  -  -  -  $ ~

RESTORE STATUS -  -  -  -  -  -  -  -  -  -  -  -  -  -  -    ~

Restore status saved by SAVE STATUS.

Runoff pocket guide - 22 Jan 86                                   - 12 -

Runoff - List of commands

REQUIRE 'file spec'  -  -  -  -  -  -  -   REQ  -  -  -  -  $

Take input from specified file.

REQUIRE BINARY [spacing ]'file-spec'   -   REQ BIN -  -  - *  ~

The specified file is transferred to output file.

RIGHT [-indent]   -  -  -  -  -  -  -  -   R -  -  -  -  - *$

Line is right, but not left, justified.

RIGHT MARGIN [+-n]   -  -  -  -  -  -  -   RM   -  -  -  - *

Set right margin.

RIGHT TEXT [-indent] -  -  -  -  -  -  -   R TEXT  -  -  - *  ~

Section of text is right, but not left, justified until END RIGHT.

SAVE CHAPTER   -  -  -  -  -  -  -  -  -  -  -  -  -  -  -    ~

Current status is restored at each CHAPTER command.

SAVE HEADER -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -    ~

Current status is restored at each page header.

SAVE LEVEL  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -    ~

Current status is restored at each HEADER LEVEL command.

SAVE STATUS -  -  -  -  -  -  -  -  -  -  -  -  -  -  -  -    ~

Current status is restored at each RESTORE command.

SEND PAGE TOC [n;]text  -  -  -  -  -  -  -  -  -  -  -  -    ~

Text followd by page number will appear in table of contents.

SEND TOC [;]text  -  -  -  -  -  -  -  -   STC  -  -  -  -

Text will appear in table of contents.

SEPARATED EQUATION   -  -  -  -  -  -  -   SEQ  -  -  -  -    ~

Begin separated equation mode until END SEPARATED EQUATION.

SET PARAGRAPH [-n],[v],[t],[b]   -  -  -   STP  -  -  -  -  $

Set paragraph parameters.

(n=indent, v=skip, t=test, b=bottom test)

SETUP escape sequence   -  -  -  -  -  -  -  -  -  -  -  -    ~

Setup printer before text.

SKIP [lines]   -  -  -  -  -  -  -  -  -   S -  -  -  -  - *

Skip lines depending on spacing except at top of page.

SPACING [s] -  -  -  -  -  -  -  -  -  -   SP   -  -  -  -  $

Set line spacing.

STYLE CHAPTER [n1],...[n5] -  -  -  -  -   STCH -  -  -  -  $ ~

Change style of first chapter page.

(1-3 = spacing, 4,5=indent (-1 centered,-2 right just)

STYLE HEADERS [n1],...[n10]   -  -  -  -   STHL -  -  -  - *$

Change style of header levels (sections).

(line,CAPS,Cap,number,center,skip1,skip2,test,indent,[n.m])

SUBPAGE  -  -  -  -  -  -  -  -  -  -  -   SPG  -  -  -  - *$

Start a subpage until END SUBPAGE.

SUBTITLE [subtitle]  -  -  -  -  -  -  -   ST   -  -  -  -  $

Set line of text to appear in subtitle.

TAB LEFT -  -  -  -  -  -  -  -  -  -  -   TL   -  -  -  -  $D~

Unspecified tabs left justify.

TAB PROPORTIONAL n,["ell"][R/C/L][+-n1],.. TABP -  -  -  -  $ ~

Set up proportionally spaced tab stops.

TAB RIGHT   -  -  -  -  -  -  -  -  -  -   TR   -  -  -  -  $ ~

Unspecified tabs right justify.

TAB STOPS ["ell"][R/C/L]+n1,...  -  -  -   TS   -  -  -  -  $

Set up ellipses, justification and location of tab stops.

TEST PAGE [n]  -  -  -  -  -  -  -  -  -   TP   -  -  -  - *$

Start new page if fewer than n lines remain on this page.

Runoff pocket guide - 22 Jan 86                                   - 13 -

Runoff - List of commands

TEST TEXT [n]  -  -  -  -  -  -  -  -  -   TT   -  -  -  - *$ ~

New page if fewer than n lines can be output on this page.

TEXT  -  -  -  -  -  -  -  -  -  -  -  -   TX   -  -  -  - *$ ~

Start a block of text that cannot be split between pages until

TEXT, END TEXT, or PAGE.

TEXT DEFERRED  -  -  -  -  -  -  -  -  -   TXD  -  -  -  - *$ ~

Start a block of text to appear on a later page until

TEXT, END TEXT, or PAGE.

TITLE [title]  -  -  -  -  -  -  -  -  -   T -  -  -  -  -  $D

Set line of text to appear as page title.

TOP MARGIN [+-n]  -  -  -  -  -  -  -  -   TM   -  -  -  - *$ ~

Set top margin for the next page.

UNDERLINE "characters"  -  -  -  -  -  -   UN   -  -  -  -    ~

The specified characters can be underlined.

UNDERLINE SPACES  -  -  -  -  -  -  -  -  -  -  -  -  -  -    ~

Spaces can be underlined.

UNDERLINE NO SPACES  -  -  -  -  -  -  -  -  -  -  -  -  -   D~

Disable underlining of spaces.

UPPER CASE  -  -  -  -  -  -  -  -  -  -   UC   -  -  -  -   D

All text will be output in upper case.

VARIABLE SPACING  -  -  -  -  -  -  -  -   VARSP   -  -  - *  ~

Enable variable spacing.

VARIANT aaaa   -  -  -  -  -  -  -  -  -  -  -  -  -  -  -    ~

Set aaaa to be true until NO VARIANT.

###############################################################################################################################################################################################################