Graphics on the Letterprinter 100

Using "LA100G"

Graphics on the Letterprinter 100

using "LA100G"

by

Rob Hamilton

Multiware, Inc.

Davis, California

May 1984

1.0  Introduction

LA100G is a program that can read a  file  of  formatted  generic

graphics   instructions,   translate   the  instructions  into  a

disk-based "bitmap" file, and then create a hardcopy image on one

of Digital's Letterprinter or Letterwriter 100 serial printers.

The program features a dot resolution of  up  to  1024  by  1024,

simulated  hardware  character generation, dotted and dashed line

generation, and a flexible means  for  scaling  and  rotating  an

image.

The   input   file   format   is   easily    user-generated    by

"post-processing" device or metafile output from most any popular

graphics package.  It can be read with standard FORTRAN READ  and

FORMAT statements.

Graphics on the Letterprinter 100                          PAGE 2

Using "LA100G"

2.0  How it works

LA100G works in two stages.  In the first stage, the user's input

file   is  read  and  processed.   Vectors  are  "rasterized"  or

converted to "1" dots (bits) in a zeroed background bitmap.  Text

characters  are converted to suitable dot patterns and positioned

in the bitmap.

In the second stage, the bitmap is accessed  in  a  top-to-bottom

direction,  and  rows  of  dots are converted to a format that is

compatible  with  the  LA100  printers.   The  serial  output  is

directed to a logical device called "LA:" until all of the bitmap

has been transferred.

2.1  The Input File

LA100G reads graphics input from  a  user-specified  file.   This

file should be formatted as rows (ASCII records) of three numeric

values each.  The spacing or width of the numeric fields within a

record   is   quite  flexible  because  a  "free-formatted"  READ

statement is used to get the values.

The first number in each record is  an  INTEGER  operation  code.

This  code  defines  the nature of the instruction.  Examples are

"1"  for  MOVE,  "2"  for  DRAW,  "3"  for  TEXT  and   "4"   for

SET_LINE_STYLE.   The  second  and  third  values in a record are

operand arguments, such as X and Y coordinate values  or  a  line

style  code.   A  special  additional record is used to list text

information.

Records are read by LA100G  one-by-one  until  an  END_OF_PICTURE

record or an end-of-file condition is encountered.

2.2  The Raster-Bitmap File

In order to store a rectangular bitmap  with  1024  points  along

each side, it is necessary to maintain an array of 1,048,576 bits

(1MBit)  or  131,072  eight-bit  bytes  (128K-bytes).   This   is

accomplished  by  storing the array on a direct-access disk file.

Only those parts being updated at a particular  moment  are  read

into memory.

Furthermore, it is convenient to divide the bitmap  into  smaller

pieces  or  "cells".  Each cell is designed to occupy exactly one

disk  block,  so  it  can  be  accessed  with  relatively  little

overhead.   Cells  represent square areas of sixty-four bits on a

side.  Thus, there are sixteen cells across the top  and  sixteen

cells down the side of the bitmap, for a total of two-hundred and

fifty-six cells.

Graphics on the Letterprinter 100                          PAGE 3

Using "LA100G"

It is interesting to note that in many instances, only a  portion

of   the   bitmap's  cells  are  actually  filled  with  non-zero

information.  This is because much of the graphics information we

create  contains  large areas of blank space.  LA100G anticipates

this condition by storing a mapped file  of  cells  that  contain

vectors  or  text.   Cells  that are not used in an image are not

represented in the bitmap file.  The first block  of  the  bitmap

file  is  dedicated to holding two-hundred and fifty-six pointers

to respective cells.  A pointer whose  value  is  zero  indicates

that the corresponding cell need not be represented.

2.3  Disk-Cache of Cells

LA100G maintains a list of bitmap cells  that  are  currently  in

memory.   To  maximize  the  speed  at which vectors and text are

rasterized, the 16-bit versions keep thirty-two cells resident or

"cached." When a new cell is needed and all available cell arrays

are full, the cached cell list is examined, and  the  cell  which

has  been  used  least  recently  is  swapped to the bitmap file.

Thus, cells that are used  most  frequently  are  swapped  least.

Once  the input file has been fully processed, the bitmap file is

closed and saved for future access.

2.4  Printer Formatting

The LA100 printers utilize a six-hammer  vertical  impact  column

that  moves  horizontally.   For  best scan-to-scan registration,

graphics mode limits actual printing to  print-head  sweeps  from

left-to-right.  A specially defined protocol allows graphics data

to be passed to the printer in dense groups of  six-bit  columns.

The  LA100G program reads the bitmap file blocks (cells) and then

constructs a row of six-bit columns as an array of  ASCII  bytes.

The array is optimized by using the printer's "repeat introducer"

sequence.   Multiple  occurances  of  adjacent   and   equivalent

characters  can  be  replaced  by  the repeat introducer, a count

value, and the character value.

Each six-bit-high row is processed in a different  way  than  the

one  immediately  above  or below it.  This is because the bitmap

file is organized as eight-bit bytes.  Four methods of extracting

six-bits columns are utilized;

1. Get lowest six bits of cell row

2. Combine  highest  two  bits of cell row with lowest four

bits of next cell row

3. Combine highest four  bits  of cell  row with lowest two

bits of next cell row

4. Get highest six bits of cell row

Graphics on the Letterprinter 100                          PAGE 4

Using "LA100G"

A rotation of the four methods is used  to  create  adjacent  and

contiguous six-bit-column rows.

The printer places 132 horizontal dots per inch and 72  dot  rows

per  vertical  inch.   To  best  approximate  a one-to-one aspect

ratio, two horizontal dots are drawn for each  one  specified  in

the  raster  bitmap  file.   Thus, the true aspect ratio of plots

drawn is 66x72 or 0.91667.

3.0  System Requirements

Under the RT-11 operating system,  the  LA100G  program  requires

about  21K  words of memory.  The raster bitmap file (RASFIL.TMP)

is created  with  a  maximum  initial  size  of  two-hundred  and

fifty-eight blocks.  Space for this file must be available on the

default device or directory.  The output file in  LA100  graphics

format  is directed to a logical device called "LA:".  You should

use the ASSIGN command to  connect  this  device  with  either  a

disk/directory or an LA100/LS100 printer.

4.0  Special LA100 Setup Requirements

The Letterprinter 100 should be set up such that its margins  are

at   the   extreme   left   and   right  positions.   Of  course,

communications parameters such as BAUD RATE, PARITY,  and  NUMBER

OF  BITS  must be set in the same manner as for alphanumeric use.

The printer should be ON-LINE and WIDE (14-7/8")  paper  inserted

at the TOP-OF-FORM position.

Keep in mind that once the program has begun to transmit graphics

data  to  the printer, the printer is in its "graphics" mode.  If

the program is aborted prematurely, it will  probably  leave  the

printer  in this mode, causing strange behavior when alphanumeric

data is transmitted.  For this reason, it  is  best  to  let  the

program  finish  normally, whereupon the printer mode is reset to

alphanumeric.  Of course, you can always reset the printer's mode

by turning its power off, and then on again.

5.0  Input File Format

As  discussed  in   section   2.1,   input   files   consist   of

FORTRAN-readable  formatted  records,  with  three numeric values

each.  The first value must be an INTEGER operation code  between

1  and  12.   Each operation code value denotes a unique graphics

function.  The second and third values are used to hold  argument

values.   Note  that some operation codes do not require specific

argument values.  However, dummy arguments must  be  supplied  to

Graphics on the Letterprinter 100                          PAGE 5

Using "LA100G"

satisfy  the  FORTRAN sequential free-format READ.  The value "0"

is usually employed for this purpose.

The following table lists graphics  operation  codes,  and  their

respective operand arguments.

Input File Op Codes and Operands

Op Code       Op Code        Argument        Argument

Function       Value            1               2

MOVE            1           X value         Y value

DRAW            2           X value         Y value

TEXT            3         No. of Chars.        0

SET_LINE_STYLE   4          Style Code          0

SET_LIMIT        6           X value         Y value

SET_OFFSET       7           X value         Y value

SET_LA100_LIMIT  8           X value         Y value

END_OF_PICTURE   9              0               0

SET_TEXT_PATH   10           degrees            0

SET_TEXT_SIZE   11           X value         Y value

SET_ROTATE      12           0 or 1             0

5.1  The MOVE command

The  MOVE  command  is   implemented   as   operation   code   1.

Accompanying  X  and  Y  argument  values  direct  a new "current

position" to be established at the coordinate (X,Y).

5.2  The DRAW command

The DRAW command is implemented as operation code 2.  A vector is

drawn  between the previous "current position" and the coordinate

specified by the accompanying arguments.  This coordinate becomes

the new "current position".

Graphics on the Letterprinter 100                          PAGE 6

Using "LA100G"

5.3  The TEXT command

The TEXT command is implemented as operation code 3.   The  first

argument  specifies  the number of characters to be written.  The

text string should be placed in the next input record,  beginning

in column 1.  Text records should not contain any other values or

arguments.

5.4  The SET_LINE_STYLE command

The SET_LINE_STYLE command is implemented as  operation  code  4.

One  of eight different styles (dot-dash patterns) of vectors can

be selected by using this command.   The  first  argument  should

hold  an  INTEGER value between 1 and 8.  Line-style 1 (solid) is

the default.  The others are described here:

Line Styles

1 . . . . . . . . . . Solid Line

2 . . . . . . . . . . Dot-Dash

3 . . . . . . . . . . Course Dots

4 . . . . . . . . . . Fine Dots

5 . . . . . . . . . . Spaced Dots

6 . . . . . . . . . . Widely Spaced Dots

7 . . . . . . . . . . Dash-Dash-Gap

8 . . . . . . . . . . Dashed Line

5.5  The SET_TEXT_PATH command

The SET_TEXT_PATH command is implemented as  operation  code  10.

Text can be written in one of four directions.  The first operand

argument is used to  select  the  desired  direction  in  angular

degrees:

0 . . . . . . . . . . left-to-right (default)

90 . . . . . . . . . . bottom-to-top

180 . . . . . . . . . . upside-down

270 . . . . . . . . . . top-to-bottom

5.6  The SET_TEXT_SIZE command

The SET_TEXT_SIZE command is implemented as  operation  code  11.

This  command  allows you to select one of seven character sizes.

An INTEGER character size code is specified in the first argument

position.   The  default size is 2, which creates characters that

Graphics on the Letterprinter 100                          PAGE 7

Using "LA100G"

are fourteen dots high and ten dots wide.  This size  is  closest

in  proportion  to  most popular graphics CRT text sizes.  Size 1

makes the smallest text characters at 5x7  dots,  and  all  other

sizes are integer multiples of size 1.

5.7  The SET_LIMITS command

The SET_LIMITS command is implemented as operation code 6.   This

command  allows  you  to  adjust  the  range  of the input file's

coordinate system.  The maximum expected X and Y values should be

specified  as  the  operand  arguments.   If  this command is not

given, a coordinate system that ranges from 0 to 4095 in X, and 0

to 3071 in Y is assumed.

5.8  The SET_LA100_OFFSETS command

The SET_LA100_OFFSETS command is implemented as operation code 7.

This  command  allows you to displace an image on the page by the

specified number of X and Y LA100 dot units.  Positive  X  and  Y

values  will  cause the image to be placed above and to the right

of the default (0,0) origin.

5.9  The SET_LA100_LIMITS command

The SET_LA100_OFFSETS command is implemented as operation code 8.

This  command  allows  you  to  place an upper limit to the LA100

coordinates to which the input  file  is  scaled.   (Note:   This

feature  implements  a  WINDOW;   not a VIEWPORT.  Information is

scaled and not clipped.) The default LA100 limits are 958 wide by

720 high.

5.10  The SET_ROTATED command

The SET_ROTATED command is  implemented  as  operation  code  12.

Graphics  images  can be rotated 90 degrees by using this command

at the beginning of  the  input  file.   When  a  non-zero  first

argument is passed, graphics will be rotated.  If the argument is

zero, graphics images will be displayed normally.

Graphics on the Letterprinter 100                          PAGE 8

Using "LA100G"

5.11  The END_OF_PICTURE command

The END_OF_PICTURE command is implemented as  operation  code  9.

This  command defines the end of graphics information in an input

file.   No  other  information  in  the  file  will  be  read  or

interpretted.

5.12  Example Input File

Following is a simple  input  file  that  draws  a  rectangle,  a

diagonal  line through the rectangle, the eight line types, and a

text string "Testing ONE TWO".

12 1 0         ! rotate graphics

1 10 10

2 2000, 10

2 2000, 1000

2 10, 1000

2 10 10

1 1000, 300

3 15 0

Testing ONE TWO

4 1 0

1 100 900

2 500 900

4 2 0

1 100 850

2 500 850

4 3 0

1 100 800

2 500 800

4 4 0

1 100 750

2 500 750

4 5 0

1 100 700

2 500 700

4 6 0

1 100 650

2 500 650

4 7 0

1 100 600

2 500 600

4 8 0

1 100 350

2 500 350

4 1 0

1 10 10

2 2000, 1000

9 0 0         ! end picture

Graphics on the Letterprinter 100                          PAGE 9

Using "LA100G"

6.0  Installation of LA100G

The  program  as  distributed  for  RT-11  and  RSX-11M  systems,

consists of six FORTRAN source files:

LA100G.FOR . . . . Main module

LA100D.FOR . . . . Input file decoder

LA100B.FOR . . . . Bitmap Management and Vector Rasterizer

LA100C.FOR . . . . Dot-Matrix Character Generation

LA100P.FOR . . . . Bitmap to LA100 conversion

LA100S.FOR . . . . LA100 format and output processor

Also provided is a command file for  compiling  and  linking  the

program.  This command file is called LA100G.COM.

7.0  Sample Run

You can run the  LA100G  program  by  typing  your  system's  RUN

command.    The   program   will   ask  you  for  an  input  file

specification.  After the file specification has been typed,  the

bitmap  file  will  be  opened  on  your default device.  If this

fails, an error diagnostic message will be printed.   You  should

make  certain  that  there  is  space available for a file of 258

blocks in length before trying again.

.RUN LA100G

Name of input file: TEST1.ARG

Processing input record  128

Beginning LA100 Graphics Printing.

Another copy? NO

.

Note that the program creates but  does  not  delete  the  raster

bitmap file RASFIL.TMP.  This file is kept so that more copies of

the image can be made, without repeating the rasterization  step.

If  it  is  no  longer  needed,  you can use your system's DELETE

command to reclaim the disk space.

To make more copies from an existing raster bitmap file, you  can

simply  press  <RETURN>  in response to the "Name of input file:"

prompt.  A message will appear that says

Attempt to print from existing bitmap file...

Graphics on the Letterprinter 100                         PAGE 10

Using "LA100G"

7.1  Error Diagnostic Messages

?LA100P-F-LA100 output file failed to open

The  logical  device  "LA:"  does  not

exist.    Use   your  system's  ASSIGN

statement  to  connect  a  device   or

directory with this name.

?LA100G-F-Named input file not found.

The input file you specified does  not

exist   on  the  directory  or  device

specified.   Make  certain  that   you

typed its name correctly.

?LA100G-F-Intermediate bitmap file (RASFIL.TMP) not found.

No input file specification was given,

so   LA100G   attempted   to  open  an

existing bitmap file.  This  file  did

not  exist  or was "locked".  Check to

see whether the file  is  present  and

that it is ready for access.

?LA100G-F-Raster file failed on open

The system tried to  open  the  raster

bitmap file RASFIL.TMP on your default

directory and failed, probably due  to

lack  of space.  This file requires an

initial  allocation  of  258   blocks.

Once  it  has been created and closed,

it will probably occupy less space.

########################################################################################################################################################################################################################################################################################################