FORWARD          _______

The software described herein is intended to be  used  as  a

utility  to  assist  in  the  configuration control of RT-11

software.  It was  written  in  PASCAL  to  demonstrate  the

versatility  and  power  of  that  language.   Some  of  the

features used are dependant  upon  a  particular  compiler:-

that  of  Oregon Software Inc., and hence is not immediately

transportable.

COPYWRITE          _________

This manual and the associated software exits in the  Public

Domain  and  may  be used or reproduced without the explicit

permission of the author.

SUPPORT          _______

Support for this Program may Be obtained from

Stephen Michell

Dept of National Defence

Ottawa, Ontario

Canada K1A 0K2

Attn.  DAAVSE 5

DISCLAIMER          __________

Neither The author nor the Department  of  National  Defence

shall  be  liable  for  any direct, indirect, incidental, or

specific damages of any kind or from any cause arising  from

the use or performance of this program.

ACKNOWLEDGEMENTS          ________________

I would like to  give  special  thanks  to  Greg  Adams  who

motivated this project and critiqued and assisted throughout

the development.

DEVICE SCANNER - VERSION 1.0                                       2

GENERAL DESCRIPTION          _______ ___________

DSC is a disk scanning and bulk  file  comparer  for  RT-11.

DSC  may be used to compare any two random access devices in

RT-11 format.  It accepts commands  in  standard  RT-11  CSI

format  and produces 3 lists, one of the files unique to the

first device, one of the files unique to the second  device,

plus  one  of  all files common to both devices, including a

report for each file as to whether it is identical or not.

Various switches are available to shorten  (time  or  report

length ) or tailor the generated report to the user's needs.

CALLING AND USING DSC          _______ ___ _____ ___

To call DSC, respond to the dot(.) by typing :

R DSC (ret)

DSC responds by printing the asterisk  (*)  and  awaiting  a

response to the command string interpreter.  Enter a command

in the format:

[Outputfile=]firstdevice:[/options],seconddevice:[/options]

Notes:

This utility works on the file name.   This  means  that  no

attempt  has  been made to address the situation where files

of different names may be identical.  The user must  use  or

develop other utilities for this.

Output defaults to TT:  unless an output file is  specified,

or /P invoked.

Output file format is "DEV:[FILNAM.EXT]"

Input File format is "DEV:".  Note the ":" is mandatory.

eg.  DY:DYDK.DIF=DY:,DK:

The default output extension is ".DIF".

On errors DSC notifies the user and reprompts.

Exit is acheived by typing C to the prompt.

OUTPUT          ______

DEVICE SCANNER - VERSION 1.0                                       3

DSC will output a list of  all  files  on  the  devices

selected, sorted alpha-numerically or by type into three (3)

categories:- unique to  the  first  device,  unique  to  the

second  device  and  common  to both devices.  Some of these

lists may not be present, depending upon  selected  potions.

With  the  common  list,  SCAN  will report upon whether the

files are identical or  not,  again  depending  upon  option

selection.

THEORY OF OPERATION DSC calls the standard RT-11 CSI routine          ______ __ _________

for  its  commands.   It  then  opens  the directory of each

device and decodes the valid entry, linking each one into  a

sorted  linked  list.   The  sorted  lists  are compared and

another, common list built.  The  Unique  files  and  Common

files are then reported and, if applicable, common files are

compared and reported on.

SCAN OPTIONS          ____ _______

The  options  available  permit  you  to  control   the

information  reported  by the utility and to inhibit certain

actions,as described below.  If you include no  options  and

do  not  specify  an  output  file,  SCAN will report on all

unique files, and common  files,  sorted  alpha-numerically,

and  will  compare  each  pair  of  similarly named files to

detect differences and will send output to TT:.

DEVICE SCANNER - VERSION 1.0                                       4

SCAN OPTIONS

OPTION    EXPLANATION

/D

Uses the creation DATE as a pre-

judgement as to whether two files

differ or not.If the dates differ,

SCAN assumes the files differ:

if the dates are identical, it

compares them.

/N:SCA

/N[:COM]

Selectively disables the scan

(reporting on unique files) (/N:SCA),

or disabling the file comparisons

(/N). The second mode still reports

on common files, but doesn't compare

them.

eg. *RKDK=RK:,DK:/N

Will report on all unique files,plus

common files, but won't verify the

sameness of any common files.

*DL1:,DX:/N:SCA

won't report on any unique files,

but will report on and verify all

common files.

/P

This  option  causes  output  to be

directed   to   LP: vice  TT:.

eg. *RK:,RK2:/P

An alternate method is to type:

LP:=firstdevice,second-device.

/S[:ALP]

/S:TYP

This causes the  information  being

displayed   to   be   sorted alpha-

numerically (default) or by typing

(/S:TYP).EG.

*DL:,DL1: or *DK:,DY:/S

Sorts alpha-numerically,but

*DX:,VM:/S:TYP

DEVICE SCANNER - VERSION 1.0                                       5

sorts each list by type,then alpha-

numerically within each type.

/U

This option may be put on either

input file. The purpose is to cause

SCAN to report on the unique files

to that device only, plus common

files. eg.

*DX1:/U,DL1:

would report on all unique files on

DX1: and on all common files, but

woud not report on any unique files

on DL1:

EXAMPLES

To get a complete list of DX  and  DY  to  the  lineprinter,

type:

*DX:,DY:/P

To compare DX:  and  DX1:,  listing  unique  files  on  DX1:

only,  and no verification of common files on DX:  and DX1:,

type:

*DX1:/U,DX:/N

To list only common files on SY:  and DK:

*SY:,DK:/N:SCA

NOTES ON USE          _____ __ ___

This utility reads many files on two disks,so it can be

slow  (for  those  of us using floppies, we're use to that).

Use the options to speed  it  up  if  you  don't  need  file

compares,  or if you are confident that files with different

dates truly are different.  Turning off the unique  listings

won't speed it up (unless you are at a 30 BAUD terminal) but

it will cut out unneeded data.

The scanner is an excellent verifier for backup  disks,etc.,

especially  after  a  copy/device.   Once  you have used it,

you'll agree that no RT-11 shop should be without one.

DEVICE SCANNER - VERSION 1.0                                       6

INSTALLATION NOTES               ____________ _____

Two versions of  the  program  are  included  in  this  kit.

DSC.SAV is the normal, EIS version,

DSC.SIM is the "no extra hardware" version.

Three command files, DSCCMP,DSCMAC, and DSCLNK are  provided

to  regenerate  the modules .  All source files are included

in this  distribution  kit  to  permt  regeneration  of  the

program.

#################################################################################################################################################################################################################################################################################################################################################################################################################################