.

NET-I (INCI) Users Manual

-------------------------

Eckart Meyer

Institut fuer Nachrichtentechnik

TU Braunschweig, FRG

March 1982

CONTENTS                                                          (a)

1.0            Introduction

2.0            Generation and Installation

2.1            Distributed files

2.2            System generation

2.3            System start

3.0            Using the system (some examples)

4.0            GTRDTM - GeT Remote Date and TiMe

NET-I  MK 4.2                                                     PAGE 2

1.0  Introduction

NET-I (INCI - IFN Computer Intercommunication)  is  a  simple

network  for  PDP11 computers running under RT11.  It is ideal for

sharing resources like backup floppy, magtape or line printer.  It

is easy to use and transparent to the user.

NET-I currently uses standard serial lines (RS 232C)  and  is

tested up to 9600 baud on a PDP11/03 and PDP11/23 link.

NET-I works under RT11 V4.0 F/B.

NET-I  MK 4.2                                                     PAGE 3

2.0  Generation and Installation

2.1  Distributed files

- Documentation

NETI.DOC                Users Manual

- Source files:

CIDF.MAC                Definition file

CI12.MAC                Conditional file for node 1 (example)

CI21.MAC                Conditional file for node 2 (example)

CI.MAC                  Driver source

CIROOT.MAC              Root segment for spooler job

CIPOLL.MAC              Poll overlay       "

CISPOL.MAC              Function execute overlay for spooler job

SYSJOB.MAC              Main control program for spooler job

SYSJOB.OBJ

- Command files:

CIASM.COM               assembly command file

CILNK.COM               link file for spooler job

- prebuild system:

CI12.SYS                Driver for node 1

CI21.SYS                Driver for node 2

CISP12.REL              Spooler job for node 1

CISP21.REL              Spooler job for node 2

CIAS12.COM              assign file for node 1 (example)

CIAS21.COM              assign file for node 2 (example)

NET-I  MK 4.2                                                     PAGE 4

- Program GTRDTM:

GTRDTM.MAC

GTRDTM.SAV

GTRDTM.COM

IOCO.OBJ                IOCS I/O subroutine package library

additionaly the following file is needed:

SYCND.MAC               RT11 SYSGENed option file.

This file is not needed, if the

RT11 FB baseline System (without

system job support) is used.

NET-I  MK 4.2                                                     PAGE 5

2.2  System generation

You can use the prebuild system, if:

- you have a FB monitor with(!) system job support,

- the node numbers are node 1 and node 2,

- the driver name is CI:.

Continue with chapter 'System start'.  The prebuild system is gen-

erated with distributed CI12.MAC and CI21.MAC.

For system generation all distributed  (the  prebuild  system

needs not) files must reside on the default device DK:.

For generating one link you have  to  repeat  the  generation

twice.   First  select  a unique node number for each RT11 system.

For each line select a unique two character device  name  for  the

line  driver  (e.g.   'CI' for the line between node 1 and 2, 'CJ'

for the line between node 2 and 3).  Now  create  the  conditional

files CIsd.MAC like the distributed examples CI12.MAC and CI21.MAC

where 's' is the local node number and 'd' the remote.   For  each

line  two  files have to be created.  Insert the local (SNODE) and

remote (DNODE) node number and the driver name in RAD50  (CINAME).

If  possible  insert the CSR address and the vector.  If you omitt

them the default values 176500 for CSR and 300 for the vector  are

used.  Note, that CSR and vector can be simply changed by the mon-

itor SET command (see 'System start').

Now build the two systems for one line:  say, a line  between

node 2 and 3 is to be build.  Copy CI23.MAC to CICND.MAC and start

the assembly command file:

@CIASM

The object files CI.OBJ, CIROOT.OBJ, CIPOLL.OBJ and CISPOL.OBJ are

created.  Now link the spooler job:

@CILNK

The file CISP.REL is created.  Rename it to CISPsd.REL, where  's'

is  the local node number and 'd' the remote.  In our example this

is CISP23.REL.  Now link the driver:

LINK/EXE:ddsd.SYS CI

where 'dd' is the two letter device name and 'sd' like above.   In

the example CI23.SYS.

Now build the system for the other node, node 3 in the  exam-

ple.  Copy CI32.MAC to CICND.MAC and repeat with renaming CISP.REL

to CISP32.REL and linking CI.OBJ to CI32.SYS.

Transfer each system to the appropriate RT11 system.

NET-I  MK 4.2                                                     PAGE 6

2.3  System start

Copy the driver and the spooler job on both  systems  to  the

system device:

COPY/SYS ddsd.SYS SY:dd

COPY CISPsd.REL SY:

where dd is the driver device name for the  line,  's'  the  local

node  number  and 'd' the remote.  If, for example, a line between

node 2 and 3 with the driver name 'CJ' is to build:

COPY/SYS CJ23.SYS SY:CY

COPY CISP23.REL SY:

Do the same on node 3 with '32' instead of '23'.

If the device CSR and vector must be changed, use the monitor

SET command:

SET CJ: CSR=XXXXXX

SET CJ: VECTOR=XXX

These changes will be permanent (the driver CJ.SYS is patched).

Install and load(!) the driver on both nodes:

INS CJ

LO CJ

Start the spooler jobs:

SRUN SY:CISP23.REL      on node 2

and     SRUN SY:CISP32.REL      on node 3

or, if you have no system job support:

FRUN SY:CISP23          on node 2

and     FRUN SY:CISP32          on node 3

On both nodes, a system identification message should be displayed

on the terminal.

NOTE

Never unload the driver before the spooler job is aborted!

Unloading it will cause a system crash.

Now assign on the local node the remote devices to  the  line

driver.   If,  for example, there is no local line printer but the

remote has one, assign LP:  to the driver (the driver name of  the

NET-I  MK 4.2                                                     PAGE 7

above example is used):

ASS CJ: LP:

All output to LP:  (including the PRINT command) will  go  to  the

line.   The  remote  spooler  (remote control job) will direct all

transfer to LP:  on the remote node, which can either be the  real

line  printer  or  a line driver to an other node, if LP:  on this

node is assigned to it.

If you want to have access to the remote default device,  the

following assignments can be done (with the above example):

ASS CJ: DK3:            on node 2

ASS DL1: DK3:           on node 3

if the default device (DK:) on node 3 is DL1:.

In summary:  all routing is done through logical  device  as-

signments.   The  only limit is the size of the RT11 device table.

This table can be increased only during RT11 system generation.

NOTE

Because a foreground-/systemjob  cannot  .FETCH  a  driver

into  memory,  all  devices, which should be used by other

nodes, must be loaded!

An example of the assignments is  given  in  the  distributed

files CIAS12.COM and CIAS21.COM.

If you want to remove the line (or may be, the line seems  to

be destroyed...) first(!) abort the spooler job:

CTRL X

JOB>CISPsd

CISPsd>CTRL C  CTRL C

or, if you have no system job support:

CTRL F

F>CTRL C  CTRL C

and then unload the driver:

UNL dd:

For starting again, load the driver and start the spooler job.

If you want only one node to be  the  active  part  (this  is

'your'  node, where you sit and type), the spooler job is not nec-

cassary on this node.

NET-I  MK 4.2                                                     PAGE 8

3.0  Using the system (some examples)

Suppose, you are on node 1 and there a lines to  node  2  and

from node 2 to node 3.  All nodes have two RL01/02 disks, DL0:  is

the system device SY:  and DL1:  the default  device  DK:.   There

are  two  lineprinters, may be a fast one on node 1 and a slow but

high quality one on node 2.  There is a backup floppy disk RX01/02

DX:   and  DX1:  on node 2.  All other devices are local and there

is no access from other nodes.  In this configuration the  assign-

ment files can look like this:

node 1:

AS DL0: SY1:            assign own system device to SY1:

AS DL1: DK1:

AS LP:  LP1:

- line to node 2 (driver name 'CI'):

AS CI:  SY2:

AS CI:  DK2:

AS CI:  SY3:

AS CI:  DK3:

AS CI:  DX:

AS CI:  DX1:

AS CI:  LP2:

node 2:

AS DL0: SY2:

AS DL1: DK2:

AS LP:  LP2:

- line to node 1 (driver name 'CI'):

AS CI:  SY1:

AS CI:  DK1:

AS CI:  LP1:

- line to node 3 (driver name 'CJ'):

AS CJ:  SY3:

AS CJ:  DK3:

node 3:

AS DL0: SY3:

AS DL1: DK3:

- line to node 2 (driver name 'CJ'):

AS CJ:  SY1:

AS CJ:  DK1:

AS CJ:  SY2:

NET-I  MK 4.2                                                     PAGE 9

AS CJ:  DK2:

AS CJ:  LP1:

AS CJ:  LP2:

AS CJ:  LP:             use LP: on node 2 = LP2:

AS CJ:  DX:

AS CJ:  DX1:

NET-I  MK 4.2                                                    PAGE 10

Now you may type the following commands (on node 1):

DIR SY2:

DIR DK3:*.MAC

DIR/OUT:LP2: DX:CI*.COM   ! DX:, not DX0:

COP SY2:STARTF.COM SY:

COP *.FOR DK2:/QUE

COP LETTER.LST LP2:       ! use high quality printer

PRI PROG                  ! print PROG.LST to LP:=LP1:

not possible:    COP DX:*.DOC DK:  ! no wildcards with remote

devices

TECO is possible with remote devices but  deletes  old  file.

KED displays a mysterious error.  Linking is possible.

All programed request exept .RENAME are possible with  remote

devices.

PIP wildcard operations are not possible with remote devices,

because the driver is a 'special device driver'.

NET-I  MK 4.2                                                    PAGE 11

4.0  GTRDTM - GeT Remote Date and TiMe

A very usefull program for RT11 users (and  for  the  system  man-

agers),  who usually crash the system several times a day and then

have to type in date in time every  time  the  system  is  booted.

This  little  program gets date time from one remote system if the

spooler job is active there, otherwise a  timeout  occurs.   Place

the command

RUN SY:GTRDTM dd:

into the startup file before starting the own spooler  job.   'dd'

is  the  line  driver device name.  The program opens a channel to

the driver and issues a .SPFUN.  On  return,  a  buffer  containes

date and time in RT11 format.  Build the program with:

@GTRDTM

and copy GTRDTM.SAV to the system device.

##########################################